3GPP TSG RAN WG1 Meeting #105-e	R1- 210xxxx

e-Meeting, 10th – 27th May 2021

Title:	Summary of NR UE Power Saving

Agenda item:	7.2.7

Source:	CATT

Document for:	Discussion

Final Summary of Email Discussions and Agreements

Email Discussion [104b-e-NR_UE_Pow_Sav_01]

Email Discussion during Preparation[104b-e-Prep_NR_UE_Pow_Sav]

| Company | Supporting Issues and draft CR | Comments |
| --- | --- | --- |
| ZTE, Sanechips | Issue 1, 3 | Okay to discuss issue 1, 3. Regarding issue 2, we share the same view with moderator that it’s not needed to re-open the discussion, the conclusion/discussion in the last meeting is clear. |

Summary of Open Issues

- Issue 1:  The proposed CR for applicable K2min in TS 38.214 when SUL is configured [1]

If SUL is configured, both UL and SUL belongs to the same serving cell with separate BWP configurations.  K2min is also configured separately for UL and SUL. ,  K2min needs to be clarified when DCI schedules a PUSCH on either UL or SUL.

Prospoal for email discussion with the draft CR.

- 6.1.2.1 Resource allocation in time domain ======skipped part======= When the UE is configured with minimumSchedulingOffsetK2 in an active UL BWP it applies a minimum scheduling offset restriction indicated by the 'Minimum applicable scheduling offset indicator' field in DCI format 0_1 or DCI format 1_1 if the same field is available. When the UE configured with minimumSchedulingOffsetK2 in an active UL BWP and it has not received 'Minimum applicable scheduling offset indicator' field in DCI format 0_1 or 1_1, the UE shall apply a minimum scheduling offset restriction indicated based on 'Minimum applicable scheduling offset indicator' value '0'. When the minimum scheduling offset restriction is applied the UE is not expected to be scheduled with a DCI in slot n to transmit a PUSCH scheduled with C-RNTI, CS-RNTI, MCS-C-RNTI or SP-CSI-RNTI with K2 smaller than K2min⋅2μ'2μ, where K2min and μ are the applied minimum scheduling offset restriction and the numerology of the active UL BWP of the scheduled cell when receiving the DCI in slot n, respectively, and μ' is the numerology of the new active UL BWP in case of active UL BWP change in the scheduled cell and is equal to μ, otherwise. If the UE is configured with supplementaryUplink in ServingCellConfig in the cell, K2min and μ are the applied minimum scheduling offset restriction and the numerology of the active UL BWP of the scheduled UL or the scheduled SUL carrier for the PUSCH transmission when receiving the DCI in slot n, respectively, and μ' is the numerology of the new active UL BWP in case of active UL BWP change in the scheduled UL or SUL carrier of the scheduled cell and is equal to μ, otherwise. The minimum scheduling offset restriction is not applied when PUSCH transmission is scheduled by RAR UL grant or fallbackRAR UL grant for RACH procedure, or when PUSCH is scheduled with TC-RNTI. The application delay of the change of the minimum scheduling offset restriction is determined in Clause 5.3.1. ======skipped part=======

Issue 2:  Restrict A-CSI-RS trigger by SRS request field for non-codebook based MIMO without cross-slot scheduling. [2]

The issue of A-CSI-RS trigger for Non-codebook based UL MIMO and the cross slot scheduling with K > 0 was discussed in RAN1#104b-e with no consensus in change the specification.

Proposal:  No further discussion in RAN1#105e

Issue 3:  The condition of default A-CSI-RS offset value 0 is “or” or “and” when the UE is not configured with minimumSchedulingOffsetK0 for any DL BWP  or/and minimumSchedulingOffsetK2 for any UL BWP [3]

- Proposal:  Email discussion the condition is “or” or “and” for minimumSchedulingOffsetK0, minimumSchedulingOffsetK2

TP for Clause 5.2.1.5.1 of TS38.214 <omit unchanged text> When aperiodic CSI-RS is used with aperiodic reporting, the CSI-RS offset is configured per resource set by the higher layer parameter aperiodicTriggeringOffset or aperiodicTriggeringOffset-r16. The CSI-RS triggering offset has the values of {0, 1, 2, 3, 4, 5, 6, …, 15, 16, 24} slots. If the UE is not configured with minimumSchedulingOffsetK0 for any DL BWP or and minimumSchedulingOffsetK2 for any UL BWP and if all the associated trigger states do not have the higher layer parameter qcl-Type set to 'typeD' in the corresponding TCI states, the CSI-RS triggering offset is fixed to zero. <omit unchanged text>

Contributions summary and proposals

| Huawei, HiSilicon [1] | - Proposal 1: The proposed CR for applicable K2min in TS 38.214 when SUL is configured. |
| --- | --- |
| Apple [2] | - Proposal 1: Explicitly specify that A-CSI-RS is located in the same slot as the SRS request field to the case where cross-slot scheduling is not configured, or when current applicable K0min = 0. |
| Ericsson[3] | - Proposal : Adopt below TP for 38.214-g50, subclause 5.2.1.5.1 when A-CSI-RS for non-codebook-based MIMO and cross-slot scheduling is configured |

Reference

- R1-2104254	Remaining issues for Rel-16 UE power saving	Huawei, 	HiSilicon

- R1-2105086	Maintenace of UE power saving for NR	Apple

- R1-2105788	Maintenance for Rel-16 UE power savings		Ericsson