3GPP TSG RAN meeting #92e										RP-21xxxx

Electronic Meeting, June 14-18, 2021

Status Report to TSG

Agenda item:			9.8.8

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | NR_MBS |  |  |  |  |
| Unique ID | 860048 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-201038 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 03/2022 | Performance part: N/A | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 50% | Performance Part: N/A | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | TSG RAN WG2 |
| --- | --- | --- |
| Rapporteur | Name | Zhenzhen CAO |
|  | Company | Huawei |
|  | Email | caozhenzhen@huawei.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

RAN1#104bis-e agreements

Mechanisms to support group scheduling for RRC_CONNECTED UEs

Agreement:

For group-common PDCCH of Rel-17 MBS, support at least two DCI formats.

- DCI format 1_0 is used as the baseline for the first DCI format with CRC scrambled with G-RNTI.

- DCI format 1_1 or 1_2 is used as the baseline for the second DCI format with CRC scrambled with G-RNTI

  - FFS: Which of DCI format 1_1 or 1_2 is used as the baseline

- FFS: Details of the reuse (or not) of DCI format 1_0, 1_1 or 1_2 fields

Agreement:

The same HARQ process ID and NDI are used for PTM scheme 1 (re)transmissions and PTP retransmissions of the same TB.

Agreement:

At least support the following cases for PDSCH reception for MBS in a slot based on UE capability for RRC_CONNECTED UEs

- Case 1: support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC

  - FFS: the value(s) of M

- Case 2: support TDM among N (N>1) group-common PDSCHs in a slot per CC

  - FFS: the value(s) of N

- Case 3: support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC

  - FFS: the value(s) of K and L

Agreement:

If a CFR is configured for multicast in RRC-CONNECTED state and confined within a dedicated unicast BWP, further study the following options.

- Option 1: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP can be used for multicast transmission if the CORESET is fully contained in the CFR in frequency domain, and the CORESET configured in PDCCH-config for MBS in the CFR can be used for unicast transmission.

- l  Option 2: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP cannot be used for multicast transmission even if the CORESET is fully contained in the CFR in frequency domain, and the CORESET configured in PDCCH-config for MBS in the CFR cannot be used for unicast transmission.

- l  Option 3: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP can be used for multicast transmission if the CORESET is fully contained in the CFR in frequency domain, but the CORESET configured in PDCCH-config for MBS in the CFR cannot be used for unicast transmission.

- l  Option 4: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP cannot be used for multicast transmission even if the CORESET is fully contained in the CFR in frequency domain, but the CORESET configured in PDCCH-config for MBS in the CFR can be used for unicast transmission.

Agreement:

One CFR is supported per dedicated unicast BWP for multicast of RRC-CONNECTED UEs.

- FFS: Whether more than one CFR is supported per dedicated unicast BWP

- FFS: Whether multicast can be supported or not in a dedicated unicast BWP when no CFR is configured for that BWP

Agreement:

The retransmission scheme for a given SPS group-common PDSCH can be either PTM scheme 1 or PTP.

- FFS: Whether PTM scheme 1 retransmission and PTP retransmission can be used simultaneously for different UEs in the same MBS group

Agreement:

Define G-CS-RNTI at least for SPS group-common PDSCH and activation/deactivation of SPS group-common PDSCH, different from CS-RNTI for unicast SPS PDSCH.

- G-CS-RNTI is used for PTM scheme 1 based dynamic retransmission of SPS group-common PDSCH

- FFS: Whether CS-RNTI can be used for PTP retransmission of SPS group-common PDSCH.

- FFS: Number of G-CS-RNTI.

Conclusion:

The maximum number of HARQ processes per cell, currently supported for unicast, is kept unchanged for UE to support multicast reception.

- How to allocate HARQ processes between unicast and multicast is up to gNB.

Agreement:

Send an LS to RAN2 regarding at least the following questions:

- Whether RAN1 should take into account the case of UE supporting multiple G-RNTIs?

Agreement:

Include the following in the LS to RAN2:

- Whether RAN1 should consider the case of UE supporting multiple G-CS-RNTIs?

- The agreements related to SPS will also be included in the LS for information

Agreement:

For CSS of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, down-select from the following alternatives (to be decided in RAN1#105):

- Alt 1: support Type-3 CSS

  - The monitoring priority of Type-3 CSS for group-common PDCCH is the same as existing Rel-15/16 CSS, regardless of which DCI format of group-common PDCCH is configured in Type-3 CSS

- Alt 2: support a new Type-x CSS

  - The monitoring priority of new Type-x CSS is determined based on the search space set indexes of the new Type-x CSS set and USS sets, regardless of which DCI format of group-common PDCCH is configured in the new Type-x CSS.

- Alt 3: support both Alt 1 and Alt 2

Agreement:

The down-selection of Option 2A and Option 2B for CFR for multicast of RRC-CONNECTED UEs will be made before the end of RAN1#105-e.

Conclusion:

It is based on gNB implementation to schedule unicast on the frequency resources covered by CFR configured for multicast.

Agreement:

For RRC_CONNECTED UE supporting MBS, support up to 8 configured SPS configurations in a BWP of a serving cell for unicast and MBS in total.

- It is up to gNB implementation to configure the SPS configuration indexes for unicast and MBS, respectively.

Agreement:

Confirm the working assumption:

For activation/deactivation of SPS group-common PDSCH for MBS in RRC_CONNECTED state,

- At least group-common PDCCH is supported

  - FFS: Whether and how to address the missed activation and deactivation

- FFS: Whether UE-specific PDCCH is supported for activation/deactivation

Mechanisms to improve reliability for RRC_CONNECTED UEs

Agreement:

Support NACK-only based HARQ-ACK feedback for RRC_CONNECTED UEs receiving multicast.

Agreement:

Two priority indexes are introduced for multicast, with

- Index 0 meaning low priority and index 1 meaning high priority.

- Priority index can be included in DCI formats scheduling the group-common PDSCH.

  - FFS details for DCI formats.

- FFS: the priority comparison between multicast and unicast with the same priority index.

Agreement:

For a separate PUCCH-ConfigurationList for multicast that is optionally configured, at least for ACK/NACK based HARQ-ACK feedback,

- The separate PUCCH-ConfigurationList for multicast configuration can be a list which includes up to 2 PUCCH-Config configurations corresponding low priority codebook and high priority codebook, respectively.

- FFS other configurations

Agreement:

For Type-2 HARQ-ACK codebook concatenation to be multiplexed in the same PUCCH resource,

- The first Type-2 HARQ-ACK sub-codebook for unicast precedes the second Type-2 HARQ-ACK sub-codebook for multicast.

- FFS: The number of Type-2 HARQ-ACK sub-codebooks for multicast.

- Note: The case of SPS PDSCH will be discussed separately.

Agreement:

For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to, down-select the following alternatives:

- Alt.1: the last DCI for unicast;

- Alt.2: the last DCI across unicast and multicast;

RAN1#105-e agreements

Mechanisms to support group scheduling for RRC_CONNECTED UEs

Agreement:

For CSS of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, Alt 2 is supported:

- Alt 2: support a Type-x CSS

  - The monitoring priority of Type-x CSS is determined based on the search space set indexes of the Type-x CSS set and USS sets, regardless of which DCI format of group-common PDCCH is configured in the Type-x CSS.

- FFS: Whether the Type-x CSS is a Type-3 CSS

Agreement:

For PTP retransmission of SPS group-common PDSCH, CS-RNTI is used for CRC scrambling of PDCCH with the NDI bit set to 1.

Agreement:

As a baseline, reuse existing fields in DCI format 1_0 with CRC scrambled by C-RNTI for the fields of first DCI format with CRC scrambled with G-RNTI.

- FFS: how to determine the bitlength of FDRA field.

- FFS: Whether ‘Identifier for DCI formats’, ‘TPC command for scheduled PUCCH’ are needed.

- FFS: How to perform DCI size alignment

- FFS: Whether to include new DCI fields

- Note: All of the fields may not be reused and the size of the fields may not be the same

Working assumption:

Option 2B for CFR associated with UE active BWP other than initial BWP is supported at least for multicast of RRC-CONNECTED UEs.

- FFS: CFR associated with initial BWP

- FFS: CFR larger than initial BWP

Agreement:

For multicast of RRC_CONNECTED UEs, further study

- How the LBRM (Limited buffer rate-matching) for GC-PDSCH TBS is determined.

- how the xOverhead for GC-PDSCH TBS determination is configured.

- whether MAC-CE over GC-PDSCH is needed for activation/deactivation of semi-persistent ZP CSI-RS resource set if the semi-persistent ZP CSI-RS resource set is configured in PDSCH-Config in CFR.

Agreement:

Confirm the working assumption:

Keep the “3+1” DCI size budget defined in Rel-15 for Rel-17 MBS.

- FFS: Whether the G-RNTI is counted as “C-RNTI” or as “other RNTI” when considering the “3+1” DCI size budget rule for group-common PDCCH.

Agreement:

For Rel-17 MBS UE, the UE maximum number of TDMed PDSCH receptions capability in a slot per CC is kept as for Rel-15/Rel-16, i.e., {2/4/7} based on UE FG5-11/5-11a/5-11b.

- Note:   Group-common PDSCH(s) are counted as unicast PDSCH(s).

Agreement:

For reliability of the group-common PDCCH activation of SPS group-common PDSCH, support at least one of the following alternatives.

- Alt 1: retransmit the activation command via group-common PDCCH.

- Alt 2: retransmit the activation command via UE-specific PDCCH.

- Alt 3: retransmit the activation command via MAC-CE.

- FFS other details.

- Note: Down-selection can take into account the HARQ-ACK feedback scheme for SPS activation

Working assumption:

The maximum number of CORESETs per BWP is not increased for support of MBS, and the number of CORESETs configured within the CFR is left to gNB implementation.

Agreement:

As a baseline, reuse existing fields in DCI format 1_1 for the fields of the second DCI format with CRC scrambled with G-RNTI.

- FFS: whether ‘Identifier for DCI formats’, ‘TPC command for scheduled PUCCH’, ‘Carrier indicator’ and ‘Bandwidth part indicator’ are needed.

- FFS: How to perform DCI size alignment

- FFS: Whether to include new DCI fields for the second DCI format

- Note: All of the fields may not be reused and the size of the fields may not be the same

Agreement:

For HARQ process management, further study whether/how to differentiate the HARQ process ID used for PTP (re)transmission for unicast and PTP retransmission for multicast.

Mechanisms to improve reliability for RRC_CONNECTED UEs

Agreement:

The signaling for URLLC feature can be reused to configure separate codebooks for unicast and multicast, respectively, at least for the case of different priorities, at least for Type-2 HARQ codebook

- FFS: The case for the same priority.

- FFS: The case of Type-1 HARQ codebook

- FFS: Whether this applies to separate PUCCH transmissions only

Agreement:

Support PUCCH format 0 and format 1 for NACK-only based HARQ-ACK feedback for multicast.

Agreement:

Support NACK-only based HARQ-ACK feedback at least for multicast SPS PDSCH without PDCCH scheduling.

- FFS for SPS activation/deactivation.

Agreement:

The priority of multicast is the same as the priority of unicast for the same priority index of HARQ-ACK at least for ACK/NACK based feedback.

Agreement:

NR supports at least the following cases for UE supporting multicast:

- UE supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast with different priorities in a slot subject to UE capability.

- UE supports two non-overlapping slot-based PUCCHs for ACK/NACK based HARQ-ACK feedback for multicast and unicast with different priorities, respectively, in a slot subject to UE capability.

Agreement:

For Type-1 HARQ-ACK codebook construction for FDM-ed unicast and multicast with the same priority from the same TRP, support

- Opt 4: HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for unicast, precede, HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for multicast. (This is similar to the joint Type-1 codebook for mTRP).

- FFS: If UE reports the capability of supporting the FDM-ed unicast and multicast in the same slot, UE can be indicated semi-statically to generate Type-1 HARQ-ACK codebook as FDM-ed manner (i.e., Opt 4).

  - Otherwise, UE does not expect unicast and multicast are to be scheduled in FDM-ed.

Conclusion:

PUCCH resource for NACK-only can be shared by UEs transmitting the NACK-only based HARQ-ACK feedback.

Agreement:

For ACK/NACK based HARQ-ACK feedback for multicast, the multiplexing/prioritizing rule between the HARQ-ACK for multicast and SR/CSI can reuse Rel-16 multiplexing/ prioritizing rule between the HARQ-ACK for unicast and SR/CSI.

Agreement:

For support of ACK/NACK based HARQ-ACK feedback for SPS multicast,

- the HARQ-ACK codebook index corresponding the HARQ-ACK codebook for SPS PDSCH is included in the configuration for SPS multicast.

  - UE determines a priority index from the HARQ-ACK codebook index

- UE can be optionally configured a separate SPS-PUCCH-AN-List for all SPS multicast configurations. Otherwise, a common SPS-PUCCH-AN-List applies to all SPS unicast and SPS multicast configurations.

Agreement:

For TDM-ed unicast and multicast, for Type-1 HARQ-ACK codebook construction for ACK/NACK-based unicast and multicast to be multiplexed in the same PUCCH resource, determining PDSCH reception candidate occasions is based on down-selecting one of the two alternatives as follows:

- Alt 1:

  - for slot timing values K1 in the intersection of K1 set for unicast (termed set A) and K1 set for multicast (termed set B), based on union of the PDSCH TDRA sets,

  - for slot timing values K1 in set A but not in set B, based on PDSCH TDRA set for unicast, and

  - for slot timing values K1 in set B but not in set A, based on PDSCH TDRA set for multicast.

- Alt 2: for slot timing values K1 in the union of K1 set for unicast and K1 set for multicast, based on the union of the PDSCH TDRA sets.

- Companies are encouraged to continue discussion of pros and cons for each alternative for further down-selection in the next meeting.

Working assumption:

For enabling/disabling ACK/NACK-based HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast via dynamic group-common PDSCH:

- RRC signaling configures the enabling/ disabling function of group-common DCI indicating the enabling /disabling ACK/NACK based HARQ-ACK feedback.

  - If RRC signaling configures the function of group-common DCI based indication, group-common DCI indicates (explicitly or implicitly) whether ACK/NACK based HARQ-ACK feedback is enabled/disabled

  - Otherwise, enabling/disabling ACK/NACK based HARQ-ACK feedback is configured by RRC signaling.

  - FFS details on RRC signaling and group-common DCI indicating.

- FFS whether/how this option is extended to apply to NACK-only based feedback and multiple G-RNTI cases.

- FFS the relation to the HARQ-ACK codebook types and HARQ-ACK codebook construction.

- FFS the relation to the enabling/disabling ACK/NACK based HARQ-ACK feedback for retransmission.

- FFS whether/how to allow UE not to react to the DCI signaling, but instead follow UE-specific RRC configuration for HARQ feedback.

- FFS whether/how to apply it to SPS group-common PDSCH.

Basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs

Agreement:

For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, both searchSpace#0 and common search space other than searchSpace#0 can be configured for GC-PDCCH scheduling MCCH.

Agreement:

For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, DCI format 1_0 is used as baseline for GC-PDCCH of MCCH and MTCH.

- FFS details of FDRA.

Agreement:

For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, RAN1 confirms the following assumptions made by RAN2

- RAN2 assumes, in case searchSpace#0 is configured for MCCH (if allowed, pending RAN1 decision), the mapping between PDCCH occasions and SSBs is the same as for SIB1.

- RAN2 assumes that if common search space other than searchSpace#0 is configured for MCCH (if allowed, pending RAN1 decision), the PDCCH monitoring occasions for MCCH message which are not overlapping with UL symbols are sequentially numbered from one in the MCCH transmission window and mapped to SSBs using the similar rule as defined for OSI in TS 38.331.

Agreement:

For broadcast reception, RRC_IDLE/RRC_INACTIVE UEs support the same CSS type for MCCH and MTCH.

- FFS support of different CSS types for MCCH and MTCH channels for broadcast reception.

Agreement:

For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, study the following alternatives for MCCH change notification indication due to session start:

- Alt 1: Define a dedicated RNTI to scramble the CRC of a DCI indicating a MCCH change notification;

- Alt 2: Use of a field in a DCI format scheduling a MCCH without a dedicated RNTI for MCCH change notification;

Other solutions are not precluded and it is also not precluded whether to support both Alt1 and Alt2.

Conclusion:

It is up to RAN2 to decide the specific contents of the MCCH change notification, e.g, whether notification only informs about session start, whether or not notification also informs about session modification/stop or whether or not the notification informs about any other information.

Agreement:

For broadcast reception, RRC_IDLE/RRC_INACTIVE UEs can use a configured/defined CFR with the same size as the initial BWP, where the initial BWP has the same frequency resources as CORESET0 (i.e., Case A), to receive GC-PDCCH/PDSCH carrying MCCH.

- Note: GC-PDCCH/PDSCH transmission within a narrower portion of the Initial BWP (where the initial BWP has the same frequency resources as CORESET0) is possible by implementation via appropriate scheduling.

Agreement:

For broadcast reception, RRC_IDLE/RRC_INACTIVE UEs can use a configured/defined CFR with the same size as the initial BWP, where the initial BWP has the same frequency resources as CORESET0 (i.e., Case A), to receive GC-PDCCH/PDSCH carrying MTCH.

- Note: GC-PDCCH/PDSCH transmission within a narrower portion of the Initial BWP (where the initial BWP has the same frequency resources as CORESET0) is possible by implementation via appropriate scheduling.

Agreement:

For RRC_IDLE/RRC_INACTIVE UEs, the CORESET index can be the same for GC-PDCCH of MCCH and MTCH.

Agreement:

For RRC_IDLE/RRC_INACTIVE UEs, for broadcast reception, the same beam can be used for group-common PDCCH and the corresponding scheduled group-common PDSCH for carrying MCCH or MTCH.

- UE may assume that DMRS ports of the group-common PDCCH/PDSCH for MCCH is QCL’d with SSB.

- UE may assume that DMRS ports of the group-common PDCCH/PDSCH for MTCH is QCL’d with SSB.

- FFS: group-common PDCCH/PDSCH for MTCH is QCL’d with periodic TRS if configured

Agreement:

For Rel-17, for broadcast reception, RRC_IDLE/RRC_INACTIVE UEs do not exceed the maximum number of CORESETs mandatorily (in the minimum capability) supported for Rel-15/Rel-16 UEs, i.e., 2 CORESETs.

- If the CFR has the same frequency range as the initial BWP, where the initial BWP has the same frequency resources as CORESET0 or where the initial BWP has the frequency resources configured by SIB1, RRC_IDLE/RRC_INACTIVE UEs can be configured with the following options:

  - CORESET#0 (default option if CFR is the initial BWP and CORESET is not configured); or

  - CORESET configured by commonControlResourceSet; or

  - CORESET#0 and CORESET configured by commonControlResourceSet.

2.1.2	Remaining Open issues

- Group scheduling mechanism for MBS in RRC_CONNECTED state, including enhancements to enable simultaneous operation with unicast reception

- Reliability improvement mechanisms for MBS in RRC_CONNECTED state

- Required changes for MBS in RRC_IDLE/ RRC_INACTIVE states

2.2	RAN2

2.2.1	Agreements

RAN2#113bis-e agreements

Stage-2 and Multicast activation

RAN2 will prioritize Active Multicast support in RRC Connected mode in Rel-17. If time permits Multicast support for RRC Inactive can be considered later (once connected mode Multicast solution, and Broadcast solution has become more mature).

There is Support to have group notification for multicast for MBS supporting nodes (e.g. paging)

Support group notification for multicast for MBS supporting nodes

For delivery mode 1 UE is not expected to monitor Group notification channel in RRC_CONNECTED

It is FFS whether RAN2 needs to handle PRACH capacity issues due to group notifications

Use same group notification identity for both RRC_IDLE and RRC_INACTIVE states

For non-supporting nodes, using MBS session ID will not work as it would impact non-MBS nodes. Unicast paging would work.

For supporting nodes, using MBS session ID is feasible.

Reliability

For a given UE, if the MRB’s QoS requirements are not met via PTM, switching to PTP with RLC-AM shall be supported.

Dynamic PTM/PTP switch is supported for a split MRB bearer (type) with a common (single) PDCP entity.

As a baseline, no new UE based signalling is introduced to support gNB switch decision (e.g. PDCP SR for high reliability is still TBD)

Assuming a split-MRB (as agreed during the online session) configured with a PTM leg and PTP leg, the usage of the PTP leg cannot be deactivated (i.e. the UE needs to always monitor C-RNTI) after the necessary split-MRB configuration.

Assuming a split-MRB (as agreed during the online session) configured with a PTM leg and PTP leg, it is FFS whether the usage of the PTM leg of the split-MRB may be subject to activation or deactivation and the details of such.

Idle and Inactive UEs

The MCCH transmission window is defined by MCCH repetition period, MCCH window duration and radio frame/slot offset.

New RNTI is defined for scheduling MCCH.

The concept of MCCH transmission window, similar to the one used for LTE SC-PTM, is used for NR MCCH scheduling. The exact parameters to define the window are FFS (discussed in the following proposals).

Common search space is needed for MCCH scheduling. RAN2 should request RAN1 to discuss the details of CSS for MCCH.

R2 assumes PDCCH occasions for MCCH search space are associated with SSBs in a pre-defined manner so that the UE can receive MCCH scheduling on PDCCH occasions according to its detected SSB.

R2 assumes, In case searchSpace#0 is configured for MCCH (if allowed, pending RAN1 decision), the mapping between PDCCH occasions and SSBs is the same as for SIB1.

R2 assumes that If common search space other than searchSpace#0 is configured for MCCH (if allowed, pending RAN1 decision), the PDCCH monitoring occasions for MCCH message which are not overlapping with UL symbols are sequentially numbered from one in the MCCH transmission window and mapped to SSBs using the similar rule as defined for OSI in TS 38.331.

Request RAN1 to discuss the details of the configuration of the bandwidth for MCCH reception.

The modification period is defined for NR MCCH and NR MCCH contents are only allowed to be modified at each modification period boundary.

The updated MCCH message should be sent in the same MCCH modification period where the change notification is sent.

UE in RRC IDLE/INACTIVE should be able to monitor/read both MCCH channel and SI/Paging without BWP switch. It is up to RAN1 to decide how this is ensured.

It is up to RAN1 to to decide about the RNTI and DCI format used for MCCH change notifications.

FFS whether to support multiple MCCH, e.g. to support different service types.

RAN2 will discuss and down-select from the following two options for the UE to get aware of session stop/modification:

- Reading MCCH once per each MCCH modification period when receiving an ongoing broadcast session

- DCI used for MCCH notification indicates the change of an ongoing broadcast session

RAN2#114-e agreements

Stage-2 and Multicast activation

Use PCCH for Multicast activation notification (also for MBS supporting nodes).

Confirm that we convey the MBS session ID in the notification.

Use of paging in all (legacy) PO with PRNTI is the baseline assumption (can still discuss other variants)

Reliability

RLC-AM is not supported for PTM (for MBS R17 WI).

Group scheduling and others

One-to-one mapping between G-RNTI and MBS session is supported in NR MBS. Other mappings FFS

One-to-one mapping between G-CS-RNTI and MBS session is supported in NR MBS. Other mappings FFS.

A UE can support multiple G-RNTIs/G-CS-RNTIs, It is FFS whether this depends on UE capability. Inform RAN1 of this agreement.

Multiple MBS QoS flows corresponding to the same MBS session can be mapped to one or more than one MBS radio bearers.

MCCH is mapped to the DL-SCH for NR MBS delivery mode 2.

MTCH is specified for PTM transmission of NR MBS.

MTCH is mapped to the DL-SCH.

DTCH is reused for PTP transmission of NR MBS.

FFS if there is a need to have specific LCID spaces for the used channels.

Multiplexing/de-multiplexing of different logical channels associated with the same G-RNTI is supported for NR MBS.

FFS if Multiplexing/de-multiplexing of different logical channels associated with the same G-CS-RNTI is supported for NR MBS.

Multiplexing/de-multiplexing of different logical channels associated with the C-RNTI is supported for NR MBS.

For NR MBS delivery mode 2, LTE SC-PTM DRX scheme is used as baseline.

FFS whether For PTM transmission of NR MBS, DRX scheme is independent of DRX for unicast transmission, e.g. supported on a per G-RNTI basis

FFS whether For PTP transmission, DRX operation for unicast transmission is reused.

Idle and Inactive UEs

MBS specific SIB is defined to carry MCCH configuration.

MCCH contents should include information about broadcast sessions such as G-RNTI, MBS session ID as well as scheduling information for MTCH (e.g. search space, DRX). L1 parameters that need to be included in MCCH are pending further RAN1 progress and input.

Postpone the discussion on whether dedicated MCCH configuration is required until RAN1 makes progress on BWP/CFR for MCCH.

Indication of an MCCH change due to modification of an ongoing session’s configuration (including session stop) is provided with an explicit notification from the network  (provided that RAN1 confirms a separate bit for this purpose can be accommodated in the MCCH change notification DCI, in addition to a bit for session start notification). FFS on whether this notification can be reused for modification of other information carried by MCCH, if any.

FFS whether the possibility of UE missing an MCCH change notification needs to be addressed or can be left to UE implementation.

At least in case RAN1 decides to utilize RNTI other than MCCH-RNTI for MCCH change notification, MCCH change notification is sent in the first MCCH monitoring occasion of each MCCH repetition period.

We support single MCCH (in this release)

2.2.2	Remaining Open issues

- Remaining issues for multicast, including but not limited to:

  - Details of group paging for multicast activation;

  - Dynamic switch remaining issues (e.g. activation/deactivation of the PTM leg);

  - Use cases of PDCP status reporting and retransmission;

  - Remaining issues for mobility support, including mobility between MBS-supporting gNBs, as well as between MBS-supporting and non-MBS-supporting gNBs.

- Remaining issues for broadcast, including but not limited to:

  - Service continuity (including MBS Interest Indication, prioritization of cell reselection, and etc.)

  - MCCH content;

  - Remaining issues for MCCH change notification.

- RAN2 aspects of group scheduling, including but not limited to:

  - DRX;

  - SPS;

  - BWP

2.3	RAN3

2.3.1	Agreements

RAN3#112-e agreements

General

- Work plan noted.

- Baseline CR endorsed.

Session Management

- For gNBs not supporting MBS, group paging using MBS session ID is not supported, only legacy per-UE paging is applicable.

- Introduce a new class 2 procedure for multicast MBS Group Paging. Name and content FFS.

- 5GC enables both options, multicast and unicast NG-U/N3 transport for NR MBS, but RAN decides, i.e. the RAN either provides for unicast transport the DL TEID or requests the 5GC to provide IP multicast address; St3 details are FFS.

- A supporting gNB indicates in PDU Session Resource SMF containers for associated PDU Sessions that it support MBS (i.e., effectively the functional support of MBS Session related information). FFS whether this is needed in all containers.

- Acknowledge that MBS related information within the associated PDU Session Resource Context may not include associated QoS flow information if interworking with non-supporting RAN nodes is not required; stage3 details are FFS.

- In case of 5GC individual MBS traffic delivery, if a PDU Session is associated with more than one MBS Session, if applicable, MBS traffic for all MBS Sessions is delivered (concurrently) via the one NG-U/N3 tunnel established for the associated PDU session.

Dynamic Change Between PTP and PTM for UEs in RRC_CONNECTED State

- WA: For the RAN2 agreed split MRB bearer with a common PDCP: the decision of using PTP (RLC leg) or PTM (RLC leg) is made by the gNB-DU. Applicability of current flow control is FFS.

Bearer Management over F1/E1

- WA: Standard shall enable a one to one mapping between an MRB and a shared F1-U tunnel.

- WA: F1-U multicast transport is not supported.

- Use non-UE associated F1/E1 procedures to set up the MBS context and shared F1-U tunnel(s) for a broadcast session (MBS context is used in analogy to UE context).

- For broadcast, an MBS context ID may be associated to one or more MRB IDs, to be included in the non-UE-associated F1AP procedure (procedure and IEs are FFS).

- Flow control should be enabled for an MRB established for a broadcast MBS session.

- WA: For broadcast session, agree to introduce the following gNB-CU-CP triggered F1AP procedures: MBS Context Setup, MBS Context Modification, MBS Context Release. Message name, scope, association with other F1AP procedures and potential alignment with multicast F1AP procedures are FFS.

- WA: For broadcast session, agree to introduce the following gNB-CU-CP triggered E1AP procedures: MBS Bearer Setup, MBS Bearer Modification, MBS Bearer Release. Message name, scope, association with other E1AP procedures and potential alignment with multicast E1AP procedures are FFS.

Service Area

- A list of cell IDs and/or tracking area IDs may be included in the NGAP MBS session source related signaling to indicate MBS service area information for local multicast session and local broadcast session.

- Area session ID may be included in the NGAP MBS session resource related signaling to indicate MBS service area information for local dependent multicast session(s) (FFS for local dependent broadcast session).

- Discuss whether MBS service area identity (SAI) can be used as MBS service area information.

Mobility Between MBS Supporting Nodes

- No agreement achieved.

Mobility Between MBS Supporting and non-MBS Supporting Nodes

- One TP agreed in R3-212908

- WA: Standards shall provide means whereby the SMF knows when receiving a Path Switch Request when a target NG-RAN node does not support MBS and means for SMF to then switch from shared delivery to individual delivery.

- WA: MBS support Indicator is included in Path Switch Request Transfer sent by an MBS supporting node to indicate support.

- MBS traffic delivery resources will be set up at target side using the information provided in the associated PDU session resource context in HO Request (for both Xn and NG mobility)

- Standards support data forwarding to minimize data loss during handover from MBS-supporting nodes to non-MBS supporting nodes.

- If data forwarding is used from MBS-supporting nodes to non-MBS supporting nodes, the source NG-RAN node should include in forwarded packets the unicast (flow) QFI mapped from the received MBS (flow) QFI.

Others:

- Support service continuity for broadcast service.

  - Support of MBMS interesting indication (RAN2 has agreed)

    - FFS: the cell lists to be transferred over the NG interface include the cells in both the current and the neighbor gNBs.

  - Support of MBMS frequency layer prioritization

    - Pending to RAN2 progress

    - FFS for SAI/ group ID

  - Neighbor cell broadcasting status for ongoing services,

    - Pending to RAN2 progress

- No need to exchange target/neighbor cell MTCH configuration of a Broadcast Service via Xn interface.

- Relationship between the state of MBS session and UEs:

  - There is no need to discuss the relationship between the state of the MBS session and that of per UE in RAN3 at this stage.

- Exchange of per cell MBS configuration

  - There is no need to transfer the per cell MBS configuration over Xn, F1 and/or E1 interfaces for coordination.

2.3.2	Remaining Open issues

- Session management over NG.

- MBS Bearer management over F1 and E1.

- Mobility with Service Continuity for UEs in RRC_CONNECTED State.

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1#104bis-e contributions:

- R1-2102899	Updated NR MBS work plan CMCC

- R1-2102318	Resource configuration and group scheduling for RRC_CONNECTED UEs	Huawei, HiSilicon

- R1-2102414	Group scheduling for NR Multicast and Broadcast Services	OPPO

- R1-2102469	Discussion on MBS group scheduling for RRC_CONNECTED UEs	Spreadtrum Communications

- R1-2102501	Discussion on Mechanisms to Support Group Scheduling for RRC_CONNECTED UEs	ZTE

- R1-2102542	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UEs	vivo

- R1-2102609	Discussion on group scheduling mechanism for RRC_CONNECTED UEs in MBS	CATT

- R1-2102655	Group Scheduling Mechanisms to Support 5G Multicast / Broadcast Services for RRC_CONNECTED Ues	Nokia, Nokia Shanghai Bell

- R1-2102702	Discussion on NR MBS group scheduling for RRC_CONNECTED UEs	MediaTek Inc.

- R1-2102782	Discussion on aspects for group scheduling	FUTUREWEI

- R1-2102844	Discussion on common frequency resource configuration for MBS	ETRI

- R1-2102900	Discussion on group scheduling mechanisms	CMCC

- R1-2103050	NR-MBS Group Scheduling for RRC_CONNECTED UEs	Intel Corporation

- R1-2103124	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	Apple

- R1-2103186	Views on group scheduling for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

- R1-2103260	On mechanisms to support group scheduling for RRC_CONNECTED UEs	Samsung

- R1-2103316	Considerations on MBS group scheduling for RRC_CONNECTED UEs	Sony

- R1-2103357	Support of group scheduling for RRC_CONNECTED UEs	LG Electronics

- R1-2103362	Further discussion on group scheduling for RRC_CONNECTED UEs	Chengdu TD Tech, TD Tech

- R1-2103419	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	Convida Wireless

- R1-2103546	Discussion on group scheduling mechanism for NR MBS	Lenovo, Motorola Mobility

- R1-2103594	Discussion on group scheduling mechanism for RRC_CONNECTED Ues	NTT DOCOMO, INC.

- R1-2103658	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UEs	ASUSTeK

- R1-2103738	Mechanisms to support MBS group scheduling for RRC_CONNECTED UEs	Ericsson

- R1-2102909	Summary#1 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS	Moderator (CMCC)

- R1-2103831	Summary#2 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS	Moderator (CMCC)

- R1-2103877	Summary#3 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS	Moderator (CMCC)

- R1-2103901	Summary#4 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS	Moderator (CMCC)

- R1-2103923	Summary#5 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS	Moderator (CMCC)

- R1-2103961	Summary#6 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS	Moderator (CMCC)

- R1-2104013	Summary#7 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS	Moderator (CMCC)

- R1-2104044	Summary#8 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS	Moderator (CMCC)

- R1-2104045	LS on G-RNTI and G-CS-RNTI for MBS	RAN1, CMCC

- R1-2104080	Summary#9 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS	Moderator (CMCC)

- R1-2104097	Summary#10 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS	Moderator (CMCC)

- R1-2102319	Mechanisms to improve reliability for RRC_CONNECTED UEs	Huawei, HiSilicon

- R1-2102415	UL feedback for RRC-CONNECTED UEs in MBS	OPPO

- R1-2102470	Mechanisms to improve reliability for RRC_CONNECTED UEs	Spreadtrum Communications

- R1-2102502	Discussion on Mechanisms to Improve Reliability for RRC_CONNECTED UEs	ZTE

- R1-2102543	Discussion on mechanisms to improve reliability for RRC_CONNECTED UEs	vivo

- R1-2102552	Discussion on MBS reliability for RRC_CONNECTED UEs	Google Inc.

- R1-2102610	Discussion on reliability improvement mechanism for RRC_CONNECTED UEs in MBS	CATT, CBN

- R1-2102656	Reliability Improvements for RRC_CONNECTED UEs	Nokia, Nokia Shanghai Bell

- R1-2102703	Discussion on mechanisms to improve reliability for RRC_CONNECTED Ues	MediaTek Inc.

- R1-2102783	Discussion on improving reliability for RRC_CONNECTED UEs	FUTUREWEI

- R1-2102845	Discussion on HARQ-ACK feedback method for MBS	ETRI

- R1-2102875	Reliability improvement for RRC_CONNECTED UEs in MBS	Potevio Company Limited

- R1-2102901	Discussion on reliability improvement	CMCC

- R1-2103051	Mechanisms to Improve Reliability of NR-MBS for RRC_CONNECTED UEs	Intel Corporation

- R1-2103125	Discussion on MBS reliability improvement for RRC_CONNECTED UEs	Apple

- R1-2103187	Views on UE feedback for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

- R1-2103261	On mechanisms to improve reliability for RRC_CONNECTED UEs	Samsung

- R1-2103358	Mechanisms to improve reliability of Broadcast/Multicast service	LG Electronics

- R1-2103363	Further discussion on reliability for RRC_CONNECTED UEs	Chengdu TD TECH, TD Tech

- R1-2103420	Discussion on reliability enhancement for RRC_CONNECTED UEs	Convida Wireless

- R1-2103547	Discussion on reliability improvement for RRC-CONNECTED UEs	Lenovo, Motorola Mobility

- R1-2103595	Discussion on HARQ-ACK feedback for multicast for RRC_CONNECTED Ues	NTT DOCOMO, INC.

- R1-2103739	Discussion on reliability mechanisms for NR MBS	Ericsson

- R1-2103834	FL summary#1 on improving reliability for MBS for RRC_CONNECTED UEs	Moderator (Huawei)

- R1-2103900	FL summary#2 on improving reliability for MBS for RRC_CONNECTED UEs	Moderator (Huawei)

- R1-2103929	FL summary#3 on improving reliability for MBS for RRC_CONNECTED UEs	Moderator (Huawei)

- R1-2104031	FL summary#4 on improving reliability for MBS for RRC_CONNECTED UEs	Moderator (Huawei)

- R1-2104098	FL summary#5 on improving reliability for MBS for RRC_CONNECTED UEs	Moderator (Huawei)

RAN1#105-e contributions:

- R1-2104195	Group Scheduling Aspects for Connected UEs	FUTUREWEI

- R1-2104248	Resource configuration and group scheduling for RRC_CONNECTED UEs	Huawei, HiSilicon, CBN

- R1-2104336	Discussion on Mechanisms to Support Group Scheduling for RRC_CONNECTED UEs	ZTE

- R1-2104387	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UEs	vivo

- R1-2104442	Discussion on MBS group scheduling for RRC_CONNECTED UEs	Spreadtrum Communications

- R1-2104491	Discussion on group scheduling mechanism for RRC_CONNECTED UEs in MBS	CATT

- R1-2104550	Group Scheduling Mechanisms to Support 5G Multicast / Broadcast Services for RRC_CONNECTED Ues	Nokia, Nokia Shanghai Bell

- R1-2104632	Discussion on group scheduling mechanisms	CMCC

- R1-2104695	Views on group scheduling for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

- R1-2104759	Group scheduling for NR Multicast and Broadcast Services	OPPO

- R1-2104865	On group scheduling mechanism for NR MBS	Lenovo, Motorola Mobility

- R1-2104928	NR-MBS Group Scheduling for RRC_CONNECTED UEs	Intel Corporation

- R1-2105069	Discussion on Group Scheduling and Simultaneous MBS and Unicast Reception 	TCL Communication Ltd.

- R1-2105128	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	Apple

- R1-2105179	Considerations on MBS group scheduling for RRC_CONNECTED UEs	Sony

- R1-2105336	Support of group scheduling for RRC_CONNECTED Ues	Samsung

- R1-2105381	Discussion on NR MBS group scheduling for RRC_CONNECTED UEs	MediaTek Inc.

- R1-2105437	Support of group scheduling for RRC_CONNECTED UEs	LG Electronics

- R1-2105600	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	Convida Wireless

- R1-2105647	Discussion on common frequency resource for multicast of RRC_CONNECTED UEs	ETRI

- R1-2105670	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	Google Inc.

- R1-2105720	Discussion on group scheduling mechanism for RRC_CONNECTED UEs	NTT DOCOMO, INC.

- R1-2105838	Discussion on group scheduling for RRC_CONNECTED UEs	CHENGDU TD TECH LTD.

- R1-2105844	Discussion on mechanisms to support group scheduling for RRC_CONNECTED UEs	ASUSTeK

- R1-2105914	Mechanisms to support MBS group scheduling for RRC_CONNECTED Ues	Ericsson

- R1-2105973	Summary#1 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS Moderator (CMCC)

- R1-2106039	Summary#2 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS Moderator (CMCC)

- R1-2106093	Summary#3 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS Moderator (CMCC)

- R1-2106126	Summary#4 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS Moderator (CMCC)

- R1-2106169	Summary#5 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS Moderator (CMCC)

- R1-2106197	Summary#6 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS Moderator (CMCC)

- R1-2106304	Summary#7 on mechanisms to support group scheduling for RRC_CONNECTED UEs for NR MBS Moderator (CMCC)

- R1-2104196	Further Discussions on Reliability for RRC_CONNECTED UEs	FUTUREWEI

- R1-2104249	Mechanisms to improve reliability for RRC_CONNECTED UEs	Huawei, HiSilicon

- R1-2104337	Discussion on Mechanisms to Improve Reliability for RRC_CONNECTED UEs	ZTE

- R1-2104388	Discussion on mechanisms to improve reliability for RRC_CONNECTED UEs	vivo

- R1-2104443	Mechanisms to improve MBS reliability for RRC_CONNECTED UEs	Spreadtrum Communications

- R1-2104492	Discussion on reliability improvement mechanism for RRC_CONNECTED UEs in MBS	CATT, CBN

- R1-2104551	Reliability Improvements for RRC_CONNECTED UEs	Nokia, Nokia Shanghai Bell

- R1-2104633	Discussion on reliability improvement	CMCC

- R1-2104696	Views on UE feedback for Multicast RRC_CONNECTED UEs	Qualcomm Incorporated

- R1-2104760	UL feedback for RRC-CONNECTED UEs in MBS	OPPO

- R1-2104866	On reliability improvement for RRC-CONNECTED UEs	Lenovo, Motorola Mobility

- R1-2104929	Mechanisms to Improve Reliability of NR-MBS for RRC_CONNECTED UEs	Intel Corporation

- R1-2105129	Discussion on MBS reliability improvement for RRC_CONNECTED UEs	Apple

- R1-2105337	On mechanisms to improve reliability for RRC_CONNECTED UEs	Samsung

- R1-2105382	Discussion on mechanisms to improve reliability for RRC_CONNECTED UEs	MediaTek Inc.

- R1-2105438	Mechanisms to improve reliability of Broadcast/Multicast service	LG Electronics

- R1-2105601	Discussion on reliability enhancement for RRC_CONNECTED UEs	Convida Wireless

- R1-2105648	Discussion on HARQ-ACK feedback for multicast of RRC_CONNECTED UEs	ETRI

- R1-2105721	Discussion on HARQ-ACK feedback for multicast for RRC_CONNECTED UEs	NTT DOCOMO, INC.

- R1-2105843	Discussion on reliability for RRC_CONNECTED UEs	CHENGDU TD TECH LTD.

- R1-2105915	Mechanisms to improve reliability for RRC_CONNECTED Ues	Ericsson

- R1-2106012	FL summary#1 on improving reliability for MBS for RRC_CONNECTED UEs	Moderator (Huawei)

- R1-2106064	FL summary#2 on improving reliability for MBS for RRC_CONNECTED UEs	Moderator (Huawei)

- R1-2106113	FL summary#3 on improving reliability for MBS for RRC_CONNECTED UEs	Moderator (Huawei)

- R1-2106198	FL summary#4 on improving reliability for MBS for RRC_CONNECTED UEs	Moderator (Huawei)

- R1-2106324	FL summary#5 on improving reliability for MBS for RRC_CONNECTED UEs	Moderator (Huawei)

- R1-2106330	FL summary#6 on improving reliability for MBS for RRC_CONNECTED UEs	Moderator (Huawei)

- R1-2104197	MBS Support for RRC IDLE/INACTIVE UEs	FUTUREWEI

- R1-2104250	Discussion on UE receiving broadcast in RRC IDLE/INACTIVE state	Huawei, HiSilicon, CBN

- R1-2104338	Discussion on basic Functions for Broadcast or Multicast for RRC_IDLE or RRC_INACTIVE UEs		ZTE

- R1-2104389	Discussion on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs			vivo

- R1-2104444	Basic Functions for Broadcast or Multicast for RRC_IDLE or RRC_INACTIVE UEs	Spreadtrum Communications

- R1-2104493	Discussion on basic functions for MBS for RRC_IDLEINACTIVE UEs	CATT, CBN

- R1-2104552	Basic Functions for Broadcast / Multicast for  RRC_IDLE / RRC_INACTIVE Ues	Nokia, Nokia Shanghai Bell

- R1-2104634	Discussion on NR MBS in RRC_IDLE/ RRC_INACTIVE states	CMCC

- R1-2104697	Views on group scheduling for Multicast RRC_IDLE/INACTIVE UEs	Qualcomm Incorporated

- R1-2104761	Discussion on support for IDLE and INACTIVE state Ues	OPPO

- R1-2104867	Basic functions for broadcast/multicast in idle/inactive states	Lenovo, Motorola Mobility

- R1-2104930	NR-MBS for RRC_IDLE/INACTIVE UEs	Intel Corporation

- R1-2105130	Discussion on MBS for RRC_IDLE/RRC_INACTIVE UEs	Apple

- R1-2105180	Considerations on MBS functions for RRC_IDLE/RRC_INACTIVE UEs	Sony

- R1-2105338	On basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs	Samsung

- R1-2105383	Discussion on broadcast or multicast for RRC_IDLE or INACTIVE UEs	MediaTek Inc.

- R1-2105439	Basic function for broadcast/multicast	LG Electronics

- R1-2105602	Discussion on MBS for RRC_IDLE/RRC_INACTIVE UEs	Convida Wireless

- R1-2105673	Discussion on MBS for RRC_IDLE/INACTIVE UE	Google Inc.

- R1-2105722	Discussion on basic functions for broadcast/multicast for RRC_IDLE/RRC_INACTIVE UEs			NTT DOCOMO, INC.

- R1-2105849	Basic functions for MBS for RRC_IDLE/RRC_INACTIVE UEs	CHENGDU TD TECH LTD.

- R1-2105916	Support for NR multicast reception in RRC Inactive/Idle	Ericsson

- R1-2105993	Feature Lead summary #1 on RAN basic functions for broadcast/multicast for UEs in RRC_IDLE/RRC_INACTIVE states	Moderator (BBC)

- R1-2105994	Feature lead summary #2 on RAN basic functions for broadcast/multicast for UEs in RRC_IDLE/ RRC_INACTIVE states	Moderator (BBC)

- R1-2105995	Feature lead summary #3 on RAN basic functions for broadcast/multicast for UEs in RRC_IDLE/ RRC_INACTIVE states	Moderator (BBC)

- R1-2106217	Feature lead summary #4 on RAN basic functions for broadcast/multicast for UEs in RRC_IDLE/ RRC_INACTIVE states	Moderator (BBC)

- R1-2106218	Feature lead summary #5 on RAN basic functions for broadcast/multicast for UEs in RRC_IDLE/ RRC_INACTIVE states	Moderator (BBC)

- R1-2104339	Consideration on performance enhancement for RRC_IDLE/INACTIVE UEs	ZTE

- R1-2104390	Other issues for Rel-17 MBS	vivo

- R1-2104494	Discussion on other issues in Rel-17 MBS	CATT

- R1-2104762	PUCCH resource allocation for  NACK-only based HARQ-ACK feedback in MBMS	OPPO

- R1-2105440	Other aspects for MBS	LG Electronics

- R1-2105526	Impact from MCCH and MTCH on broadcast reception	Huawei, HiSilicon

- R1-2105855	MBS related network planning for two delivery modes 	CHENGDU TD TECH LTD.

- R1-2105917	Assumptions for performance evaluations of NR-MBS	Ericsson

RAN2#113bis-e contributions:

- R2-2102635	Reply LS on 5MBS progress and issues to address (R3-211296; contact: Huawei)	RAN3

- R2-2102666	Reply LS on 5MBS progress and issues to address (S2-2102077; contact: Huawei)	SA2

- R2-2102670	Reply LS on 5MBS progress and issues to address (S3-211313; contact: Huawei)	SA3

- R2-2102716	Discussion on Multicast in Idle and Inactive Mode	CATT, CBN

- R2-2102717	Reliability Improvement for PTM Transmission	CATT

- R2-2102718	Dynamic PTM/PTP Switch	CATT

- R2-2102719	Discussion on Group Scheduling	CATT

- R2-2102720	Further Discussion on Delivery Mode 2	CATT, CBN

- R2-2102765	Discussion on deactivation of MBS reception	Shanghai Jiao Tong University

- R2-2102766	Consideration on Group Scheduling for NR MBS	Shanghai Jiao Tong University

- R2-2102767	Discussion on dynamic PTM and PTP switching	Shanghai Jiao Tong University

- R2-2102782	MBS UP architecture	MediaTek Inc.

- R2-2102783	Dynamic Switch for NR MBS	MediaTek Inc.

- R2-2102784	RAN2 aspects of Group Scheduling for NR MBS	MediaTek Inc.

- R2-2102785	L1 HARQ operation for PTM transmission	MediaTek Inc.

- R2-2102838	Discussion on SA2 LS and multicast session activation	Intel Corporation

- R2-2102839	MBS MAC Layer and Group Scheduling Aspects	Intel Corporation

- R2-2102893	Discussion on beam sweeping transmission for delivery mode 2	OPPO

- R2-2102894	Discussion on MBS interesting indication and service continuity for delivery mode 2	OPPO

- R2-2102895	Discussion on group based scheduling for MBS	OPPO

- R2-2102896	RRC state control for MBS reception	OPPO

- R2-2102934	On Group Scheduling and Multiplexing Aspects	Samsung

- R2-2102937	On MBS DRX, Data-Inactivity & BWP aspects	Samsung

- R2-2102938	NR MBS operation in Idle/Inactive mode	Samsung

- R2-2102945	MBS Reliability	Nokia, Nokia Shanghai Bell

- R2-2102946	Miscellaneous Aspects of MBS Provisioning	Nokia, Nokia Shanghai Bell

- R2-2103118	Considerations on the SA2 questions about session activation	vivo

- R2-2103119	Dynamic PTM PTP switch for RRC Connected UE	vivo

- R2-2103120	Updated summary for MBS Group Scheduling	vivo

- R2-2103121	Group Scheduling for MBS	vivo

- R2-2103122	MBS in Idle and Inactive Mode	vivo

- R2-2103152	Configuration and scheduling in MBS delivery mode 2	Futurewei

- R2-2103163	PTP_PTM dynamic switch	NEC

- R2-2103167	Discussion on Beam Sweeping Configuration for Flexible MBS Control Plane Scheduling	TCL Communication Ltd.

- R2-2103178	NR MBS control signalling aspects for UEs in different RRC states	Qualcomm Inc

- R2-2103179	NR Multicast group paging aspects	Qualcomm Inc

- R2-2103180	NR Multicast and Broadcast Radio Bearer Architecture aspects	Qualcomm Inc

- R2-2103188	NR Multicast PTM bearer RLC AM mode operation	Qualcomm Inc, FirstNet,UIC, Kyocera, AT&T

- R2-2103200	Split MRB Protocol Architecture and Logical Channel Aggregation	Futurewei

- R2-2103201	ARQ of PTM with Logical Channel Aggregation	Futurewei

- R2-2103202	Service Continuity during Dynamic PTM/PTP Switch with Logical Channel Aggregation	Futurewei

- R2-2103254	Discussion on MBS session activation/deactivation and UAC	Spreadtrum Communications

- R2-2103255	Discussion on dynamic PTM PTP switching	Spreadtrum Communications

- R2-2103256	Discussion issues on delivery mode2	Spreadtrum Communications

- R2-2103267	HARQ modelling for supporting retransmission in MBS	Asia Pacific Telecom co. Ltd, FGI

- R2-2103277	MBS Idle	Nokia, Nokia Shanghai Bell

- R2-2103278	MBS session activation and group paging	Nokia, Nokia Shanghai Bell

- R2-2103358	Dynamic PTM PTP switching	LG Electronics Inc.

- R2-2103359	MBS configuration for RRC_CONNECTED	LG Electronics Inc.

- R2-2103360	MBS in IDLEINACTIVE	LG Electronics Inc.

- R2-2103372	Further consideration of control plane aspects for NR MBS	Kyocera

- R2-2103373	Consideration of dynamic PTM - PTP switching with service continuity for NR MBS	Kyocera

- R2-2103374	Consideration of possible solutions for L2 reliability in NR MBS	Kyocera

- R2-2103413	Issues on MBS reliability	Lenovo, Motorola Mobility

- R2-2103414	MBS dynamic switch between PTP and PTM with service continuity	Lenovo, Motorola Mobility

- R2-2103415	Discussion on MBS delivery modes	Lenovo, Motorola Mobility

- R2-2103416	MBS Group Scheduling Aspects	Lenovo, Motorola Mobility

- R2-2103450	UE stay in RRC_CONNECTED when no MBS data ongoing	ASUSTeK

- R2-2103471	draft LS about deployment scenarios of NR Broadcast	ZTE, Sanechips

- R2-2103472	NR Broadcast deployment scenarios	ZTE, Sanechips

- R2-2103473	Consideration on MBS reliability guarantee	ZTE, Sanechips

- R2-2103474	Mode switching signaling of NR MBS	ZTE, Sanechips

- R2-2103475	Discussion on Group scheduling for NR MBS	ZTE, Sanechips

- R2-2103476	Idle and Inactive mode UEs support of NR MBS	ZTE, Sanechips

- R2-2103507	Discussion on two delivery modes for NR MBS	CHENGDU TD TECH LTD.

- R2-2103508	Further discussion on reliability for RRC_CONNECTED state	CHENGDU TD TECH LTD.

- R2-2103512	Procedure for dynamic PTM/PTP switch	CHENGDU TD TECH LTD.

- R2-2103513	Further discussion on delivery mode 2 for NR MBS	CHENGDU TD TECH LTD.

- R2-2103515	Architecture aspects for NR MBS	Ericsson

- R2-2103516	Reliability and Dynamic Switch for MBS	Ericsson

- R2-2103517	Aspects of Group Scheduling	Ericsson

- R2-2103518	Email discussion report [Post113-e][054]PTP/PTM dynamic switch and MRB type change	Ericsson (Rapporteur)

- R2-2103523	Updated NR MBS workplan	Huawei, CMCC, HiSilicon

- R2-2103524	PTP/PTM dynamic switch and MRB initialization	Huawei, CBN, HiSilicon

- R2-2103525	RAN2 aspects of group scheduling	Huawei, CBN, HiSilicon

- R2-2103543	MRB and DRB configuration	Sony Europe B.V.

- R2-2103649	Discussion on MRB	Samsung

- R2-2103650	Layer-2 for MBS	Samsung

- R2-2103670	Considerations on measurements for NR MBS in idle/inactive	Lenovo, Motorola Mobility

- R2-2103679	Way forward on UP architecture for MBS	InterDigital Inc., ZTE, Sanechips, MediaTek Inc., Huawei, HiSilicon, Ericsson, LG Electronics Inc., Samsung Telecommunications, Fujitsu, Sharp, CATT, CBN, Spreadtrum Communications, Xiaomi Communications, Asia Pacific Telecom co. Ltd., OPPO, Lenovo, Motor

- R2-2103680	PTM/PTP mode switching	InterDigital

- R2-2103703	Discussion on group scheduling for MBS	CMCC

- R2-2103704	Discussion on delivery mode 2 remaining issues	CMCC

- R2-2103705	Discussion on delivery mode 2 transmission	CMCC

- R2-2103706	LS on delivery mode 2 transmission	CMCC

- R2-2103728	Discussion on SA2 Reply LS on 5G MBS	CMCC

- R2-2103729	Draft reply LS on Group Paging	CMCC

- R2-2103775	Multicast in Idle and Inactive	Ericsson, MediaTek, FirstNet, CBN

- R2-2103776	Open issues for UEs in idle or inactive mode	Ericsson

- R2-2103871	Consideration on the MBS transmission reliability	Apple

- R2-2103872	MBS PTP/PTM  switching	Apple

- R2-2103873	MBS reception in CONNECTED state	Apple

- R2-2103874	MBS reception in IDLE/INACTIVE state	Apple

- R2-2103905	Discussion on group notification for multicast session activation	Huawei, HiSilicon

- R2-2103906	Reply LS on 5MBS progress and issues to address	Huawei, HiSilicon

- R2-2103907	Multicast session reception in RRC INACTIVE	Huawei, HiSilicon

- R2-2103908	Service continuity aspects of delivery mode 2	Huawei, HiSilicon, CBN

- R2-2103909	Report of e-mail discussion: [Post113-e][053][MBS17] MCCH scheduling and MCCH change notification (Huawei)	Huawei, HiSilicon

- R2-2103946	On NR multicast and broadcast for RRC_IDLE/RRC_INACTIVE UEs	Convida Wireless

- R2-2103947	NR MBS Configuration Information	Convida Wireless

- R2-2103949	PTM Reliability Considerations	Convida Wireless

- R2-2103963	Way forward on UP architecture for MBS	InterDigital Inc., ZTE, Sanechips, MediaTek Inc., Huawei, HiSilicon, Ericsson, LG Electronics Inc., Samsung Telecommunications, Fujitsu, Sharp, CATT, CBN, Spreadtrum Communications, Xiaomi Communications, Asia Pacific Telecom co. Ltd., OPPO, Lenovo, Motor

- R2-2104088	Support of PDCP status reporting for PTM-PTP switching	SHARP Corporation

- R2-2104089	L2 architecture for delivery mode 2	SHARP Corporation

- R2-2104118	Dynamic switch between PTM and PTP for service continuity	Intel Corporation

- R2-2104119	MBS support for delivery mode 2	Intel Corporation

- R2-2104150	Comparison of L2 Reliability Solutions for MRB with dynamic PTM/PTP Switch	Futurewei, Qualcomm Inc, Intel

- R2-2104161	Discussion on reliability improvement and UL feedback in NR multicast	LG Electronics Inc.

- R2-2104162	Discussion on RAN2 aspects of group scheduling	LG Electronics Inc.

- R2-2104207	Support of dynamic switch between PTP and PTM	SHARP Corporation

- R2-2104226	Clarification on the PDCP-anchored MRB	Xiaomi Communications

- R2-2104227	MBS impacts on PDCP	Xiaomi Communications

- R2-2104228	Discussion on group scheduling	Xiaomi Communications

- R2-2104229	Remaining issues of MCCH and MCCH change notification	Xiaomi Communications

- R2-2104230	Service continuity for delivery mode 2	Xiaomi Communications

- R2-2104284	Performance improvement for delivery mode 2	CHENGDU TD TECH LTD.

- R2-2104494	Updated summary for MBS Group Scheduling	vivo

- R2-2104501	Summary of A.I. 8.1.2.1 Reliability	LG Electronics Inc.

- R2-2104509	Report of e-mail discussion: [Post113-e][053][MBS17] MCCH scheduling and MCCH change notification (Huawei)	Huawei, HiSilicon

- R2-2104577	[AT113bis-e][031][MBS17] MBS session activation (Nokia)	Rapporteur(Nokia)

- R2-2104588	Email discussion report for [Post113-e][054][MBS17] PTP/PTM dynamic switch and MRB type change	Ericsson

- R2-2104629	Report of offline discussion: [AT113bis-e][032][MBS17] MCCH scheduling and Change notification (Huawei)	Huawei, HiSilicon

- R2-2104630	[DRAFT] LS on broadcast session delivery and MCCH design	Huawei

- R2-2104639	LS on broadcast session delivery and MCCH design	RAN2

- R2-2104646	Reply LS on 5MBS progress and issues to address	RAN2

- R2-2104655	Reply LS on 5MBS progress and issues to address	RAN2

RAN2#114-e contributions:

- R2-2104710	LS on G-RNTI and G-CS-RNTI for MBS (R1-2104045; contact: CMCC)	RAN1

- R2-2104754	Reliability Improvement for PTM Transmission	CATT

- R2-2104755	Open Issues on Mobility with Service Continuity	CATT, CBN

- R2-2104756	Discussion on Group Scheduling	CATT

- R2-2104757	Further Discussion on delivery mode 2	CATT, CBN

- R2-2104758	Discussion on Multicast Session Activation	CATT, CBN

- R2-2104820	draft LS about deployment scenarios of NR Broadcast	ZTE, Sanechips

- R2-2104821	NR Broadcast deployment scenarios	ZTE, Sanechips

- R2-2104822	Consideration on MBS reliability guarantee	ZTE, Sanechips

- R2-2104823	Lossless handover support for NR MBS	ZTE, Sanechips

- R2-2104824	Discussion on Group scheduling for NR MBS	ZTE, Sanechips

- R2-2104825	Idle and Inactive mode UEs support of NR MBS	ZTE, Sanechips

- R2-2104875	Group notification and RACH congestion	Intel Corporation

- R2-2104876	MBS MAC layer and Group scheduling aspects	Intel Corporation

- R2-2104936	Discussion on beam sweeping transmission for delivery mode 2	OPPO

- R2-2104937	Discussion on MBS interesting indication and service continuity for delivery mode 2	OPPO

- R2-2104938	Discussion on group based scheduling for MBS	OPPO

- R2-2104939	Service continuity for MBS	OPPO

- R2-2104940	Group notification and unicast paging for MBS activation	OPPO

- R2-2104947	MCCH based Group Notification	MediaTek Inc.

- R2-2104948	Reliability Mechanism for MBS	MediaTek Inc.

- R2-2104949	Mobility and Service continuity for NR Multicast	MediaTek Inc.

- R2-2104950	RAN2 aspects of Group Scheduling for NR MBS	MediaTek Inc.

- R2-2104951	L1 HARQ operation for PTM transmission	MediaTek Inc.

- R2-2104969	Discussion on HARQ process for MBS data reception	Asia Pacific Telecom, FGI

- R2-2104984	On NR MBS operation in Idle/Inactive mode	Samsung

- R2-2104993	Considerations on Multiplexing & Scheduling Aspects	Samsung

- R2-2104995	Considerations on Mobility and Service Continuity	Samsung

- R2-2105007	MCCH Configuration and messaging in MBS delivery mode 2	Futurewei

- R2-2105008	Discussion on the remaining issues with MBS group notification	Futurewei

- R2-2105009	Handling MBS during UE mobility	Futurewei

- R2-2105013	NR MBS control signalling aspects for UEs in different RRC states	Qualcomm Inc

- R2-2105015	NR Multicast and Broadcast Radio Bearer Architecture aspects	Qualcomm Inc

- R2-2105018	NR Multicast group paging aspects	Qualcomm Inc

- R2-2105019	NR Multicast Broadcast mobility enhancements with service continuity	Qualcomm Inc

- R2-2105020	NR Multicast PTM bearer RLC AM mode operation	Qualcomm Inc, FirstNet,UIC, Kyocera, AT&T

- R2-2105028	ARQ of PTM with Logical Channel Aggregation	Futurewei

- R2-2105096	Discussion on the MBS transmission reliabilty	Apple

- R2-2105097	Mobility with MBS service continuity	Apple

- R2-2105098	MBS reception in CONNECTED state	Apple

- R2-2105099	Access Control for the MBS Service Reception	Apple

- R2-2105265	MBS Reliability	Nokia, Nokia Shanghai Bell

- R2-2105266	Miscellaneous Aspects of MBS Provisioning	Nokia, Nokia Shanghai Bell

- R2-2105284	Consideration on Group Notification	vivo

- R2-2105285	PTP PTM switch and service continuity	vivo

- R2-2105286	Service Continuity for Connected UE	vivo

- R2-2105287	Group Scheduling for MBS	vivo

- R2-2105288	Open Issues for Delivery mode 2	vivo

- R2-2105310	Dynamic PTM and PTP switching	Shanghai Jiao Tong University

- R2-2105311	Group Scheduling for NR MBS	Shanghai Jiao Tong University

- R2-2105313	Deactivation of MBS reception	Shanghai Jiao Tong University

- R2-2105365	Discussion on two delivery modes for NR MBS	CHENGDU TD TECH LTD.

- R2-2105366	Performance improvement for delivery mode 2	TD TECH LTD.

- R2-2105370	Discussion on reliability for RRC_CONNECTED state	CHENGDU TD TECH LTD.

- R2-2105373	UE stay in RRC_CONNECTED when no MBS data ongoing	ASUSTeK

- R2-2105386	Discussion on mobility with service continuity for NR MBS	CHENGDU TD TECH LTD.

- R2-2105387	Discussion on delivery mode 2 for NR MBS	CHENGDU TD TECH LTD.

- R2-2105439	Discussion on Multicast Control Channel Scheduling Configurations for Delivery Mode 2	TCL Communication Ltd.

- R2-2105511	Control plane aspects for delivery mode 2 in NR MBS	Kyocera

- R2-2105512	Consideration of dynamic PTM - PTP switching with service continuity for NR MBS	Kyocera

- R2-2105513	Group notification for Delivery mode 1 in NR MBS	Kyocera

- R2-2105514	Consideration of possible solutions for L2 reliability in NR MBS	Kyocera

- R2-2105550	Discussion on MBS session activation/reactivation	Spreadtrum Communications

- R2-2105551	Discussion on service continuity during mobility	Spreadtrum Communications

- R2-2105552	Discussion issues on delivery mode2	Spreadtrum Communications

- R2-2105572	RAN2 aspects of group scheduling	TCL Communication Ltd.

- R2-2105573	Dynamic PTM PTP Switch	TCL Communication Ltd.

- R2-2105577	Support of group notification	Huawei, CBN, HiSilicon

- R2-2105578	MBS support for delivery mode 2	Huawei, CBN, HiSilicon

- R2-2105579	Service continuity during inter-cell mobility	Huawei, CBN, HiSilicon

- R2-2105596	PTP_PTM dynamic switch	NEC

- R2-2105653	Open issues broadcast	Ericsson

- R2-2105654	Open issues group scheduling	Ericsson

- R2-2105655	Open issues multicast	Ericsson

- R2-2105668	MCCH design details	Nokia, Nokia Shanghai Bell

- R2-2105669	MBS group notification	Nokia, Nokia Shanghai Bell

- R2-2105680	MRB and DRB configuration	Sony

- R2-2105681	MBS BWP UE capability and MBS resources	Sony

- R2-2105726	Discussion on MBS support on MRDC	Xiaomi Communications

- R2-2105727	MBS impacts on PDCP	Xiaomi Communications

- R2-2105728	Service continuity for delivery mode 2	Xiaomi Communications

- R2-2105729	Remaining issues of MCCH and MCCH change notification	Xiaomi Communications

- R2-2105730	Discussion on the MBS paging for delivery mode 1	Xiaomi Communications

- R2-2105756	Architecture aspects for NR MBS	Ericsson

- R2-2105757	Reliability for MBS	Ericsson

- R2-2105764	Discussion on MRB Architecture	Samsung

- R2-2105765	SDAP/PDCP/RLC Aspects for MBS	Samsung

- R2-2105795	Way forward on UP architecture for MBS	InterDigital Inc., ZTE, Sanechips, MediaTek Inc., Huawei, HiSilicon, Ericsson, LG Electronics Inc., Samsung Telecommunications, Fujitsu, Sharp, CATT, CBN, Spreadtrum Communications, Xiaomi Communications, Asia Pacific Telecom co. Ltd., OPPO, Lenovo, Motor

- R2-2105796	PTM/PTP mode switching	InterDigital

- R2-2105832	Issues on MBS reliability	Lenovo, Motorola Mobility

- R2-2105833	Connected Mode Mobility with Service Continuity	Lenovo, Motorola Mobility

- R2-2105834	MBS Group Scheduling Aspects	Lenovo, Motorola Mobility

- R2-2105835	Discussion on Idle and Inactive mode UEs	Lenovo, Motorola Mobility

- R2-2105914	MBS support for RRC_IDLE/INACTIVE	Intel Corporation

- R2-2106008	L2 ARQ of PTM Transmission with Dynamic PTM/PTP Switch	Futurewei, Qualcomm Inc., Intel, UIC, Kyocera

- R2-2106009	Protocol Architecture of MRB with Dynamic PTM/PTP Switch	Futurewei

- R2-2106112	On RLC mode for PTM transmission	SHARP Corporation

- R2-2106113	Support of PDCP status reporting for PTM-PTP switching	SHARP Corporation

- R2-2106114	L2 architecture for delivery mode 2	SHARP Corporation

- R2-2106205	Activation/Deactivation of PTM	Sharp

- R2-2106238	Discussion on MBS L2 Structure	cmcc

- R2-2106239	Discussion on MBS UP remaining issues	cmcc

- R2-2106240	Discussion on Mobility with Service Continuity	cmcc

- R2-2106241	Discussion on group scheduling for MBS	cmcc

- R2-2106242	Discussion on delivery mode 2 remaining issues	cmcc

- R2-2106248	38.300 Running CR for MBS in NR	CMCC

- R2-2106282	Multicast and Broadcast transport channels	Huawei, CBN, HiSilicon

- R2-2106283	RAN2 aspects of group scheduling	Huawei, CBN, HiSilicon

- R2-2106334	MBS L2 architecture for PTP-PTM switching	Intel Corporation

- R2-2106335	MBS service continuity in mobility	Intel Corporation

- R2-2106345	Group notification for multicast session	LG Electronics Inc.

- R2-2106350	MBS in IDLE/INACTIVE	LG Electronics Inc.

- R2-2106352	MBS Mobility with Service Continuity	Nokia, Nokia Shanghai Bell

- R2-2106356	Activation and Deactivation of PTM/PTP leg	Convida Wireless

- R2-2106357	On NR multicast and broadcast for RRC_IDLE/RRC_INACTIVE UEs	Convida Wireless

- R2-2106361	NR MBS Configuration Information	Convida Wireless

- R2-2106365	PTM Reliability Considerations	Convida Wireless

- R2-2106417	Discussion on overall architecture of MBS traffic delivery	LG Electronics Deutschland

- R2-2106419	[Pre114-e][001][MBS] Summary 8.1.2.1 MBS Connected mode Reliability (LGE)	LG Electronics Inc.

- R2-2106422	Discussion on RAN2 aspects of group scheduling and DRX	LG Electronics Deutschland

- R2-2106423	Discussion on MBS Reliability	LG Electronics Deutschland

- R2-2106483	[Pre114-e][002][MBS] Summary on MBS scheduling	vivo

- R2-2106687	Reply LS on G-RNTI and G-CS-RNTI for MBS	RAN2

- R2-2106730	Report of offline discussion: [AT114-e][039][MBS] MCCH and MCCH change notification (Huawei)	Huawei, HiSilicon

RAN3#112-e contributions:

- R3-211426	Reply LS on 5MBS progress and issues to address	RAN2

- R3-211453	Reply LS on 5MBS progress and issues to address	SA2

- R3-211471	Introduction of MBS(BL CR for 38.463)	CATT

- R3-211472	Introduction of NR MBS	Lenovo, Motorola Mobility

- R3-211478	MBS BL CR for TS38.410	ZTE

- R3-211479	Introduction of NR MBS	Huawei, CMCC

- R3-211481	Introduction of NR MBS	Samsung

- R3-211482	BL CR for NR MBS for 38.413	Qualcomm Incorporated

- R3-211484	Introduction of NR MBS	LG Electronics

- R3-211485	Introduction of NR Multicast and Broadcast Services	Ericsson

- R3-211515	Reply LS on 5MBS progress and issues to address	RAN2

- R3-211542	MBS session management and TP to 38.410 38.413 BL CR	ZTE

- R3-211543	TP to 38.414 MBS BL CR on IP multicast	ZTE

- R3-211544	Mode switching for NR MBS	ZTE

- R3-211545	MBS bearer management over F1 and E1 and TPs to 38.470 38.473 38.460 38.463 BL CR	ZTE

- R3-211546	MBS service area	ZTE

- R3-211547	Discussion on lossless Mobility in NR MBS	ZTE

- R3-211548	Discussion on MBS mobility procedure	ZTE

- R3-211549	Discussion on UE mobility between an MBS-supporting gNB and a non-MBS-supporting gNB	ZTE

- R3-211550	Discussion on Broadcast service continuity in NR MBS	ZTE

- R3-211656	(TP for 38.300 & 38.410) Stage 2 for Multicast and Broadcast	Nokia, Nokia Shanghai Bell

- R3-211657	(TP for 38.413) Setup of MBS Context and UE MBS Context	Nokia, Nokia Shanghai Bell

- R3-211658	(TP for 38.413) Management of User Plane Shared Delivery Tunnel	Nokia, Nokia Shanghai Bell

- R3-211659	(TP for 38.300 & 38.401 & 38.470) Stage 2 for PTP-PTM Switching	Nokia, Nokia Shanghai Bell

- R3-211660	(TP for 38.401) MBS F1, E1 Bearer Management	Nokia, Nokia Shanghai Bell

- R3-211661	(TP for 38.415) Seamless Mobility between two MBS Supporting Nodes	Nokia, Nokia Shanghai Bell

- R3-211662	(TP for 38.300) MBS Stage 2 for Mobility Management	Nokia, Nokia Shanghai Bell

- R3-211663	Realization of Data Forwarding to minimize packet loss	Nokia, Nokia Shanghai Bell

- R3-211751	NR Multicast Session Management Procedure	Qualcomm Incorporated

- R3-211752	Data forwarding in multicast handover	Qualcomm Incorporated

- R3-211753	MBS Session Context Transfer for mobility and SFN	Qualcomm Incorporated

- R3-211866	Discussion on dynamic change between PTM and PTP	CATT

- R3-211867	Discussion on multicast context establishment during Xn-based handover	CATT

- R3-211868	Possible solutions on PDCP sync while keeping MRB mapping flexible	CATT

- R3-211869	Draft LS on aligning MRB PDCP Count among multiple gNBs	CATT

- R3-211870	TP on TS 38.300 on aligning MRB PDCP Count among multiple gNBs	CATT

- R3-211872	MBS reception of Idle and In-active UEs	CATT

- R3-211873	Further Consideration on MBS context management over F1 and E1	CATT

- R3-211874	TP for 38.463 on MBS Bearer Management for broadcast at E1 interface	CATT

- R3-211875	TP for 38.473 on MBS Context Management for Broadcast at F1 interface	CATT

- R3-211876	TP for 38.413 on session management for broadcast service	CATT,CBN, Huawei

- R3-211877	Discussion on MBS session management for multicast	CATT

- R3-211971	TP for MBS BLCR for 38.413-Session management for MBS over NG	Samsung

- R3-211972	Coordinated assignment of PDCP SN	Samsung

- R3-211973	Data forwarding for mobility between MBS supporting nodes	Samsung

- R3-211974	TP for TS38.473 BL: Introduction of NR MBS	Samsung

- R3-211975	TP for TS38.463 BL: Dynamic Change Between PTP and PTM	Samsung

- R3-212092	Discussion on the current status in SA2 and RAN2	Ericsson

- R3-212093	[DRAFT] Reply LS on on 5MBS progress and issues to address	Ericsson

- R3-212101	[TP for BL CR for 38.300 and TS 38.410] Session Management over NG and its application on Xn	Ericsson

- R3-212102	[TP for BL CR for 38.413 and TS 38.423] on MBS Session Management	Ericsson

- R3-212103	On PTP and PTM Switch	Ericsson

- R3-212104	[TP for BL CR for 38.473 and TS 38.463] On F1 and E1 protocol aspects for NR MBS	Ericsson

- R3-212105	On Support of mobility between gNBs supporting MBS	Ericsson

- R3-212106	[TP for BL CR TS 38.300] HO between supporting gNBs	Ericsson

- R3-212107	[TP for BL CR TS 38.401 and TS 38.463] HO between supporting gNBs	Ericsson

- R3-212108	On support of mobility between MBS supporting and non-MBS supporting gNBs	Ericsson

- R3-212181	Multicast Session Management over NG	Lenovo, Motorola Mobility

- R3-212182	(TP for BL CR 38.401) Dynamic switch between PTP and PTM	Lenovo, Motorola Mobility

- R3-212183	Bearer Management over F1/E1 for Multicast Session	Lenovo, Motorola Mobility

- R3-212184	User Plane Protocol Aspects of F1-U for NR MBS	Lenovo, Motorola Mobility

- R3-212185	MBS Service Area Management	Lenovo, Motorola Mobility

- R3-212186	Enhancements to support loss-less handover for NR multicast	Lenovo, Motorola Mobility

- R3-212187	Mobility between MBS Supporting and non-Supporting nodes	Lenovo, Motorola Mobility

- R3-212385	Discussion on a group notification towards NG-RAN supporting MBS	LG Electronics

- R3-212386	Issues on dynamic change between PTP and PTM	LG Electronics

- R3-212388	Handling on Mobility between MBS Supporting Nodes	LG Electronics

- R3-212421	Updated Rel-17 NR MBS work plan	Huawei, CMCC

- R3-212422	Discussion about LSs on 5MBS progress and issues to address	Huawei

- R3-212423	[DRAFT]  Reply LS on 5MBS progress and issues to address	Huawei

- R3-212424	(TP to TS 38.410 BL CRs) Multicast Session Management	Huawei, CBN

- R3-212425	(TP to TS 38.413 BL CRs) Multicast Session Management	Huawei, CBN

- R3-212426	(TP to TS 38.401 BL CR) Decision on PTP and PTM	Huawei, CBN

- R3-212427	(TP to TS 38.300 BL CR) Mobility between MBS supporting nodes	Huawei, CBN, CMCC

- R3-212428	(TP to TS 38.401 BL CR) Mobility between MBS Supporting Nodes	Huawei, CBN

- R3-212429	(TP to TS38.300 BL CR) Consideration on DL PDCP Synchronization	Huawei, CBN, China Unicom, CMCC

- R3-212430	(TP to TS38.300 BL CR) Data forwarding between MBS supporting nodes	Huawei, CBN, China Unicom, China Telecom

- R3-212431	(TP to TS38.300 BL CR) Mobility between MBS supporting and non-supporting nodes	Huawei, CBN, China Telecom, China Unicom

- R3-212436	(TP to TS 38.401 BL CR) Bearer management over F1 and E1	Huawei, CBN

- R3-212437	(TP to TS 38.470 and 38.460 BL CRs) Bearer management over F1 and E1 interfaces	Huawei, CBN

- R3-212438	(TP to TS 38.415 BL CR) Support of NR MBS data transmission	Huawei, CBN, China Telecom, China Unicom

- R3-212439	(TP to TS 38.300 BL CR) Service reception continuity for broadcast service	Huawei, CBN

- R3-212479	Introduction of MBS	CMCC

- R3-212480	Discussion on SA2 reply LS on 5MBS progress and issues to address	CMCC

- R3-212481	[Draft]Reply LS to SA2 on 5MBS Progress and Issues to address	CMCC

- R3-212482	MBS Session management over NG	CMCC

- R3-212483	(TP to TS 38.410 ) MBS session management over NG	CMCC

- R3-212484	(TP to TS 38.300 ) MBS session management over NG	CMCC, Huawei

- R3-212485	Discussion on dynamic change between PTP and PTM	CMCC

- R3-212486	Discussion on MBS Bearer Management	CMCC

- R3-212487	Discussion on Mobility with Service Continuity	CMCC

- R3-212488	Discussion on Mobility from non-MBS-supporting node to MBS-supporting node	CMCC

- R3-212489	Discussion on Mobility from MBS-supporting node to non-MBS-supporting node	CMCC

- R3-212593	Effect of NR MBS related network planning on NG-RAN architecture	Chengdu TD Tech, TD Tech

- R3-212702	CB: # 97_MBS_General - Summary of email discussion	Huawei - moderator

- R3-212704	CB: # 100_MBS_NGsessMgmt - Summary of email discussion	Ericsson - moderator

- R3-212705	CB: # 101_MBS_PTP-PTMdynChg - Summary of email discussion	Huawei - moderator

- R3-212707	CB: # 43_MBS_ServiceArea - Summary of email discussion	Lenovo - Moderator

- R3-212708	CB: # 103_MBS_MobilitySupporting - Summary of email discussion	Qualcomm - moderator

- R3-212709	CB: # 104_MBS_MobilityNonSupporting - Summary of email discussion	Nokia - moderator

- R3-212710	CB: # 99_MBS_Others - Summary of email discussion	Huawei - moderator

- R3-212713	CB: # 50_eNBarchEvo_AP - Summary of email discussion	Huawei - moderator

- R3-212791	CB: # 102_MBS_F1-E1bearerMgmt - Summary of email discussion	CATT - moderator

- R3-212908	(TP for 38.300) Mobility with non-MBS supporting NG-RAN nodes	Nokia, Nokia Shanghai Bell

- R3-212982	Introduction of NR MBS	Nokia, Nokia Shanghai Bell

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template