3GPP TSG-RAN WG1 Meeting #105-e	Tdoc R1- 2106045

E-meeting, May 10th – 27th, 2021

Agenda Item:	8.3.2

Source:	Moderator (Ericsson)

Title:	Summary#2 - Enhancements for IIOT/URLLC on Unlicensed Band

Document for:	Discussion, Decision

# 1	Introduction

This document that is updated version of R1-2106044, summarizes the discussion during RAN1#105-e under the following email thread assigned by Mr. Chairman:

[105-e-NR-R17-IIoT-URLLC-03] Email discussion on enhancements for unlicensed band URLLC/IIoT – Sorour (Ericsson)

- 1st check point: 5/21

- 2nd check point: 5/25

- Final check: 5/27

# 2	Discussion topics

## 2.1	Harmonization w.r.t NR-U and URLLC CG

The topic of CG harmonization of NR-U and URLLC features developed in Rel-16 (excluding PUSCH repetition) has been discussed in previous meetings where the companies view in RAN1 have been categorized by 4 options as listed in the agreement in RAN1#103-e. Meanwhile RAN2 made some agreements on this topic with a disclaimer that the corresponding agreement is from RAN2 perspective.

The discussions in RAN1 in the last meeting concluded in down-selecting to two options, i.e. Option 1 and Option 2a.

Agreements (RAN1#103-e): Down-select one of the following options (target RAN1#104-e): - Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16. - Option 2-a: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and cg-RetransmissionTimer-r16, respectively. - Option 2-b: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and new parameter Y, respectively, where X and Y are different from cg-RetransmissionTimer-r16. - Option 3: CG-UCI based procedures are supported for unlicensed. CG-DFI based procedures are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16 - Note: Procedures based on CG-UCI rely on UE including CG-UCI in CG PUSCH at least as in Rel-16 where the values of the respective fields of CG-UCI are decided by UE. - Note: Procedures based on CG-DFI rely on automatic re-transmission on CG configuration and reception of CG downlink feedback information (DFI) in DCI for re-transmissions. Agreement (RAN1#104bis-e): - Option 2-b and option 3 are not considered further for the agreement in RAN1#103-e regarding CG harmonization

Agreements (RAN2#112-e):

From RAN2 perspective

1 	It is assumed that LBT failures only happen infrequently in UCE (unlicensed controlled environment).  A formal definition of UCE and its relationship to semi-static or dynamic access mode is not necessary in RAN2 specifications.

2	cg-RetransmissionTimer can be configured optionally for shared spectrum

3	When cg-RetransmissionTimer is configured, Rel-16 NR-U mechanism is used for HARQ process ID and RV selection.

4	When cg-RetransmissionTimer is not configured, Rel-16 URLLC mechanism may be used for HARQ process ID and RV selection.

5	As a baseline, HARQ processes sharing between multiple CGs are allowed when cg-RetransmissionTimer is configured as in Rel-16 NR-U.

6	HARQ processes sharing between multiple CGs are not allowed when cg-RetransmissionTimer is not configured.

7	FFS if LCH based prioritization can be configured with cg-RetransmissionTimer

8	The assumption for Rel-16 is that the network will not configure autonomousTx and cg-RetransmissionTimer simultaneously per cell.  No optimizations will be pursued to allow the two features be configured together in Rel-16.  No CR is needed for this for now.

9	If a configured grant is deprioritized and/or gNB didn’t get it (e.g. LBT failure and/or tx failure) then we should be able to autonomously re-transmit it.  FFS how to achieve it (using existing mechanisms should be considered as baseline)

Table 1 shows the comparison between these two options with respect to the corresponding RRC parameters where Case#2 and Case#3 are the cases that the two options differ.

- Case#2:

  - Companies in favor of Option 1 argue that Case#2 is not well motivated and creates complications with respect to specifications and corresponding UE features. Companies in favor of Case#2 argue that CG-DFI is used to provide HARQ-ACK feedback and is thus more closely related to the CG retransmission timer. Therefore, CG-UCI and CG-DFI should be configured independently. On the other hand, CG-DFI and CG retransmission timer are complimentary and can be configured together.

    - Moderator observes that few proponents of Option 2a discard Case#2 and only consider Case#1,Case#3&Case#4 (Apple, Intel).

- Case#3:

  - Companies in favor of Option 1 argue that Case#3 is not well motivated and creates complications with respect to specifications and corresponding UE features. They also argue delay reduction claims in case of misalignment on traffic arrival and CG configurations, can be handled by e.g. multiple CG configuration toa LCH or adjusting periodicity. Also, the COT sharing information is not well motivated in controlled environments. Proponents of Option 2a, argue that Case#3 reduces mis-alignment delay and even LBT delay due to UE’s freedom of choosing HARQ-ID and RV, or sharing a gNB COT in a transmission opportunity without waiting for a later transmission opportunity, and hence being more efficient and less complex than other solutions such as multiple CG configurations. They also argue that in controlled environments LBT failures may not be often, but still present and important to be handled for URLLC application and in that regard, COT information in CG-UCI is useful.

Table 1: Combination of features based on configurations of corresponding RRC parameters in respective options. Note: 0 means the feature is disabled/not configured, 1 means enabled/configured)

| Combination cases | Applicable options | CG-UCI | CG-DFI | New RRC parameter X | cg-RetransmissionTimer-r16 |
| --- | --- | --- | --- | --- | --- |
| Case#1 | Option 1 & Option 2a | 0 | 0 | 0 | 0 |
| Case#2 | Option 2a | 0 | 1 | 0 | 1 |
| Case#3 | Option 2a | 1 | 0 | 1 | 0 |
| Case#4 | Option 1 & Option 2a | 1 | 1 | 1 | 1 |

Moderator observations:

- Reviewing the contributions shows that the companies’ preferences with respect to Option 1 and 2a has remained similar to previous meeting, as well as the arguments.

- It seems each group has different perspectives on complexity and gain assessments. At least, the arguments to support or NOT to support Case#3 have not been changed.

- It seems from Option 1 proponents’ perspective, Case#2 is the most problematic case. Meanwhile some of Option 2a proponents discard Case#2 as well.

## 2.1.1	Discussion – 1st round

Proposal and summary of views:

Proposal 1-1

Agreement:

Down-select one of the following options (target RAN1#104-e):

- Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16.

- Option 2-a: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and cg-RetransmissionTimer-r16, respectively.

- Note: Procedures based on CG-UCI rely on UE including CG-UCI in CG PUSCH at least as in Rel-16 where the values of the respective fields of CG-UCI are decided by UE.

- Note: Procedures based on CG-DFI rely on automatic re-transmission on CG configuration and reception of CG downlink feedback information (DFI) in DCI for re-transmissions

Companies’ preferences on Proposal 1-1 are summarized below:

- Option 1:

  - Nokia/NSB, vivo, HW/HiSi, Ericsson, DCM, NEC, LG, CATT, Sharp, ZTE, MTK, ETRI, Spreadtrum, OPPO

- Option 2a:

  - QC, Intel, Samsung, Apple, IDC, Panasonic, Sony

| Questions: - Q1: Companies are kindly requested to provide any update/correction of their position with respect to the Option 1 and Option 2a. - Q2: Do the proponents of Option 2a prefer to add the sub-bullet below to exclude Case#2 in Table 1 from Option 2a? - Q3: Do the proponents of Option 1 support Option 2a if Case#2 in Table 1 is excluded by adding the sub-bullet below? - Q4: Please provide any other comments if needed. Proposal 1-1(updated) Agreement: Down-select one of the following options (target RAN1#104-e): - Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16. - Option 2-a: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and cg-RetransmissionTimer-r16, respectively. - [If cg-RetransmissionTimer-r16 is configured, “CG-UCI based procedures” should also be enabled by X.] - Note: Procedures based on CG-UCI rely on UE including CG-UCI in CG PUSCH at least as in Rel-16 where the values of the respective fields of CG-UCI are decided by UE. - Note: Procedures based on CG-DFI rely on automatic re-transmission on CG configuration and reception of CG downlink feedback information (DFI) in DCI for re-transmissions |  |
| --- | --- |
| Company | Comment |
| Sony | Q1: Still support Option 2a. Q2: The original proposal in Option 2a already described the new sub-bullet. The original proposal did say that ”CG-UCI procedures” and ”CG-DFI procedures” are independently enabled/disabled by X and cgRetransmissionTimer-r16 respectively. The new sub-bullet can be a note, but it does not offer any more information than the original proposal. |
| Intel | Q1: We still support Alt. 2a Q2: The additional sub-bullet is fine for us if the intention is to preclude Case #2. This is inline with our understanding that when the cg-RetransmissionTimer-r16 is enabled disabling the CG-UCI procedure does not add any technical value |
| HW/HiSi | Q1: We still support Option 1 Q2: We cannot agree with introducing a new parameter X and impact all the current RAN1/2 spec depending on the CG-RetransmissionTimer just to allow one new combination out of 4, especially that the benefit of this new combination (CG-UCI without CG-DFI and CG ReTX) is not evident while the benefits of early termination and saving dynamic overhead of the rescheduling UL grants are lost. In that new combination, - If only COT sharing indication is desired while the remaining HARQ ID/RV/NDI are still tied to the CG-RetransmissionTimer, it is not justified to incur the dynamic overhead of CG-UCI in every PUSCH to achieve COT sharing while operating an unlicensed controlled environment. - If HARQ ID/RV/NDI are made to be tied to the X parameter, then the only benefit seems to multiplexing of TBs in a period (without early termination and CG ReTx) which in our view can be matched by configuring shorter CG periodicity or using multiple CGs. Q3: No, as explained in Q2, we do not see the motivation to enable this new combination given the drawbacks, spec impact and misalignment with RAN2 understanding |
| Nokia/NSB | Q1: We still support Option 1 Q3: No. The benefits of this combination are questionable and do not justify the added complexity |
| Apple | Q2: We support adding the sub-bullet for Option 2a. Case #2 should be excluded because CG retransmission does not work properly if CG-UCI is not enabled. This concern on case #2 has been raised by many companies against Option 2a. As one of proponents for Option 2a, we do not intend to support case #2. |
| IDC | Q2: We are fine with adding the sub bullet to preclude Case #2. |
| CATT | Q1: We still support Option 1 Q3: No, the motivation of this combination need be clarified and for case 3, the benefit isn’t clear to us One more comment for case 3, when CG-UCI based procedures are enabled but CG-DFI based procedures are disabled, NR-U autonomous ReTX does not apply since cg-retransmissionTimer is not configured. UE can’t be scheduled to retransmit the TB if decoding of CG-UCI is failure because gNB doesn’t know the HARQ process ID selected by the UE. Anyway, both case 2 and case3 should be excluded in option2a so we should go with option 1. |
| vivo | Q1: We still support Option 1 Q3: No. The main usage of CG-UCI is to inform gNB about the NDI, HARQ ID and RV for autonomous retransmission. As seen in RAN2’s agreement, the CG-UCI is used when cg-RetransmissionTimer is configured. RAN2 agreements 3 When cg-RetransmissionTimer is configured, Rel-16 NR-U mechanism is used for HARQ process ID and RV selection. 4 When cg-RetransmissionTimer is not configured, Rel-16 URLLC mechanism may be used for HARQ process ID and RV selection. 5 As a baseline, HARQ processes sharing between multiple CGs are allowed when cg-RetransmissionTimer is configured as in Rel-16 NR-U. |
| LG | Q1: We still support Option 1 Q3: No. We share the same view with other companies. In case where CG-DFI and CG-ReTx-timer are disabled, we don’t think CG-UCI alone could work well, especially in terms of NDI signaling to differentiate initial TX and retransmission for a given HPN. |
| ZTE | Q1: We still support Option 1 Q3: Yes, we are also fine with the updated Option2a |
| ETRI | Q1: We still support Option 1 Q3: No. We share the view with other companies. |
| QC | Q1: Still support Option 2a. Q2: We are fine with the sub-bullet adding to preclude Case #2. It is not helpful to disable the CG-UCI procedure when the cg-RetransmissionTimer-r16 is enabled. |
| Samsung | Q1: We still support Option 2a Q2: OK to add the sub-bullet to exclude case #2. |
| Sharp | Q1: Support Option 1. |
| Asia Pacific | Q1: We support option 1. Q3: No. Case 3 should also be excluded. |
| Panasonic | Q2: We support adding the sub-bullet for Option 2a. |
| DCM | Q3: We don’t agree with the updated Option 2a as it is. However, if the enabled CG-UCI includes COT sharing information only when cg-RetransmissionTimer is not configured, we support the proposal. In this case, Rel-16 URLLC mechanism is used for HARQ process ID and RV selection, which is the same as Option 1. Indication of COT sharing information is useful irrespective of the configuration of cg-RetransmissionTimer. |
| Spreadtrum | Q3: NO. as same as other companies, we still think cg-retransmissionTimer enable CG-DFI but disable CGUCI cannot work well. |
| Moderator | Companies’ preferences on Updated Proposal 1-1 are summarized below: - Option 1: - Nokia/NSB, vivo, HW/HiSi, Ericsson, DCM, NEC, LG, CATT, Sharp, ZTE, MTK, ETRI, Spreadtrum, OPPO, Asia Pacific - Option 2a: - QC, Intel, Samsung, Apple, IDC, Panasonic, Sony @DCM: It seems to me that DCM is interested to have a case that only COT sharing information in CG-UCI is transmitted, under Option 1. It is not clear to me why the sub-bullet under Option 2a causes issue for the intended application. Please let me know if that I misunderstood. @All: It seems companies’ positions have not changed. Proponents of Option 1 considers that Option 2a, even with excluding Case#2, is in conflict with RAN2 agreements, beside other arguments. @All: Considering the clarification above to DCM, let’s consider the updated Proposal 1-1 below and focus on concluding this proposal. Proposal 1-1(updated) Agreement: Down-select one of the following options (target RAN1#104-e): - Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16. - Option 2-a: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and cg-RetransmissionTimer-r16, respectively. - If cg-RetransmissionTimer-r16 is configured, “CG-UCI based procedures” should also be enabled by X. - Note: Procedures based on CG-UCI rely on UE including CG-UCI in CG PUSCH at least as in Rel-16 where the values of the respective fields of CG-UCI are decided by UE. - Note: Procedures based on CG-DFI rely on automatic re-transmission on CG configuration and reception of CG downlink feedback information (DFI) in DCI for re-transmissions |

## 2.1.2	Discussion – 2nd round

Companies’ preferences on Updated Proposal 1-1 are summarized below:

- Option 1:

  - Nokia/NSB, vivo, HW/HiSi, Ericsson, DCM, NEC, LG, CATT, Sharp, ZTE, MTK, ETRI, Spreadtrum, OPPO, Asia Pacific

- Option 2a:

  - QC, Intel, Samsung, Apple, IDC, Panasonic, Sony

Proposal 1-1(updated)

Agreement:

Down-select one of the following options (target RAN1#104-e):

- Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16.

- Option 2-a: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and cg-RetransmissionTimer-r16, respectively.

  - If cg-RetransmissionTimer-r16 is configured, “CG-UCI based procedures” should also be enabled by X.

- Note: Procedures based on CG-UCI rely on UE including CG-UCI in CG PUSCH at least as in Rel-16 where the values of the respective fields of CG-UCI are decided by UE.

- Note: Procedures based on CG-DFI rely on automatic re-transmission on CG configuration and reception of CG downlink feedback information (DFI) in DCI for re-transmissions

Moderator observation/suggestion:

- Companies’ position has not changed. Considering the observations/clarifications by Moderator in the previous section, let’s consider the updated Proposal 1-1 below and focus on concluding this proposal.

| Questions: - Q1: Please provide any comments, including follow-up of discussion in previous if needed, or changes the status or suggestions to help to reach a consensus. |  |
| --- | --- |
| Company | Comment |

## 2.2	COT-initiator determination for configured UL transmissions

During the last meeting, determination of ownership of a COT for a configured UL transmission was extensively discussed. In case of configured UL transmission, the determination rule is already agreed for transmission opportunities within a UE-FFP that occur after the UE-FFP. However, the determination rule for the transmission opportunities aligned with UE-FFP boundaries are yet to be decided. The discussion in the previous meeting resulted in narrowing down the options to two Alt-a and Alt-b as described in the agreement below:

Agreement:

In semi-static channel access mode when a UE can operate as UE-initiated COT,

- Select one of the following alternatives to determine whether a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, is based on UE-initiated COT or sharing a gNB-initiated COT:

  - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT

  - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT.

On determination of COT initiator

- Alt-a: Companies arguments in favour of Alt-a include at least the following:

  - More flexibility and efficiency

  - Possibility for no LBT as apposed to Alt-b (always 9us LBT)

  - Protecting gNB idle period

  - Similar procedures as Rel-16

  - …

- Alt-b: Companies arguments in favour of Alt-b include at least the following:

  - UE power saving

  - More robust to (DL) mis-detection

  - Less dependency on DL for UL transmissions

  - More suitable for CG PUSCH w.r.t. UE knowledge of traffic arrival

  - No LBT not possible (due to implementation?)

  - …

- Alt-a/Alt-b:

  - for large UE-FFP periodicities (e.g., {4, 5, 10} ms) adopt Alt-a, and for small UE-FFP periodicities (e.g., {1, 2, 2.5} ms) adopt Alt-b.

    - Len/MOT

  - RRC configure between Alt-a/Alt-b

    - ETRI

Moderator observations:

- Reviewing the contributions shows that the companies’ preferences with respect to Alt-a and Alt-b has remained similar to previous meeting, as well as the arguments.

- It seems each group has different perspectives on complexity and gain assessments.

- A company (MOT/Len) suggested compromised solution based on FFP duration configuration to choose between Alt-a and Alt-b.

- Irrespective of decision for Alt-a or Alt-b, a separate discussion is needed to clarify the following since moderator observes different interpretations among the companies:

Does the gNB need to terminate its ownership of a COT that is initiated in gNB FFP when a UE initiated a COT within gNB FFP? Please note that is different from colliding DL/UL transmission

  - No: Ericsson, DCM, ETRI, Samsung, vivo (support design restriction)

  - Yes: Speadtrum, Intel?, FW?

## 2.2.1	Discussion – 1st round

Proposals and summary of views:

Proposal 2-1:

Agreement:

In semi-static channel access mode when a UE can operate as UE-initiated COT,

- Select one of the following alternatives to determine whether a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, is based on UE-initiated COT or sharing a gNB-initiated COT:

  - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT

  - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT.

Companies’ preferences on Proposal 2-1 are summarized below:

- Alt-a:

  - Nokia/NSB, Intel, HW/HiSi, DCM, Futurewei, IDC, LG, WILUS, Panasonic, Sony, ZTE, MTK, Spreadtrum, OPPO, Sharp(2nd), ETRI(2nd)

- Alt-b:

  - QC, vivo, Ericsson, Samsung, Apple, CATT, Sharp(1st), ETRI(1st)

- Alt-a/Alt-b:

  - Lenovo/MOT: For large UE FFP, Alt-a, otherwise Alt-b

  - ETRI: RRC configure between Alt-a/Alt-b

| Questions: - Q1: Companies are kindly requested to provide any update/correction of their position with respect to Proposal 2-1. - Q2: What is your view on the following question? Does the gNB by regulation need to terminate its ownership of a COT that has initiated for a gNB FFP, when a UE initiates a COT within that gNB FFP? - Please note that is different from colliding DL/UL transmissions. - In order to establish a common understanding on regulation, please share your understanding assuming there is no collision between UL and DL transmission. Also, assume all signalling aspects are handled. - Q3: Please share any other comments if needed. |  |
| --- | --- |
| Company | Comment |
| Intel | Q1: We support Alt. a Q2: The regulatory requirements do not provide any restrictions on whether a device should terminate the ownership of a COT once this is acquired: in fact, as it was pointed out a device could potentially operate at anytime as an initiating device, a responding device, or both. However, for the specific case of CG operation, if the UE initiates a COT and shares it with the gNB (i.e., CG-UCI contains information that the UE intends to share its COT), in this case the gNB must terminate its ownership at least for the duration of the u-FFP. |
| HW/HiSi | Q1: We still support Alt-a. Q2: No, by regulations, the two are different initiating devices that can continue transmitting until their respective idle periods and one device can transmit in the idle period of the other inline with the conclusion made in the previous meeting Conclusion: - In semi-static channel access mode, a UE as an initiating device, is allowed to transmit during the idle period of any FFP associated with the serving gNB if the UE transmission is based on UE initiated COT ○ Note: the gNB may disallow UL transmission during symbols of the idle period by configuring them either as semi-static DL symbols, or indicating them as DL with SFI. |
| FW | Q1: We still support Alt-a, the arguments presented so far were not convincing that an UE needs to start its own COT rather than remain in the COT of gNB, together with all other UEs. Q2: First I would like to thank the moderator for her efforts in creating the summary and the NWM document. Regarding the Question 2, we would like to understand the reasons for this question. There are few things confusing. By ”terminate its ownership” should we understand that gNB terminates the COT prior to UE initiating its own COT? Our understanding is that this question is related to Proposal 2-1, in this case the question is a little bit misleading because it assumes that Alt-b is already accepted and UE follows it. Maybe the question should be rephrased to include the situation when gNB initiates a COT, sends a grant to UE for sharing the COT, then UE sends its scheduled transmission in gNB COT, and then UE has a UL configured transmission 5 during the same gNB COT. Should UE start its own COT in the first place? As the group already decided that gNB is in control of scheduling and decisions, the gNB should have priority over UEs and UEs should defer their COT to gNB. We think that a more relevant question related to the Alt-b is the following: For Alt-b, please clarify how the gNB during its initiated COT acknowledges (HARQ) an UL transmission from UE in a possible UE initiated COT when gNB COT and UE COT overlaps in time. Or equivalently how two initiating COT devices can communicate back and forth with each other during their overlapping COTs? |
| Apple | Q2: No, the gNB can continue to use its own COT even if a UE initiates a COT within the gNB’s COT. From regulation perspective, gNB and UE are considered as two devices. They can each initiate its own COT, and the COT can overlap. The mechanism to ensure the coexistence (no collision) is that each device needs to do the LBT before performing another transmission after stopping the transmission for a while. So there is no issue for two initiating devices to communicate back and forth, to answer the question raised by Futurewei. Regarding Intel’ comment that the gNB must terminate its COT if the UE initiates a COT and shares it with the gNB, we don’t know why this has to be the case. In our view, the gNB can continue to use its own COT (i.e. not sharing the UE’s COT) if it prefers. |
| Len/Mot | Q2: No. We think the COT association is per transmission, and we think the question is related to both configured grant and scheduled UL transmissions. BTW, not clear to us what “assume all signalling aspects are handled” means. |
| Nokia/NSB | Q1: We still support Alt-a Q2: No. The regulations wrt COT are written from a single device perspective, and nothing forbids gNB and UE from initiating overlapping COTs. |
| CATT | Q1: We support Alt.b and we are also fine with supporting both Alt.a and Alt.b which can address different scenarios and is configurable Q2: Yes, because it isn’t clear whether gNB transmission belongs to gNB initiated COT or sharing UE-initiated COT from UE perspective and from gNB perspective, it isn’t clear that UE transmission belongs to UE initiated COT or sharing gNB-initiated COT. |
| LG | Q1: We still support Alt-a Q2: Before providing the opinion, I’d like to clarify the same question is also being asked for the opposite case, i.e., the UE by regulation need to terminate its ownership of a COT that has initiated for a UE FFP, when the gNB initiates a COT within that UE FFP. Our view is that, even though there is no such restriction in the regulations, we could design in such way so that one of UE and gNB becomes COT initiator while the other becomes COT responder at a same timing as have designed so far, to avoid potential complication. |
| ETRI | Q1: We prefer Alt-b, or RRC configuration between Alt-a/Alt-b. We are also okay with Alt-a. Q2: No, COT overlapping between two nodes is not forbidden based on ETSI regulation. |
| QC | Q1: Still support Alt-b. |
| Samsung | Q1: We still support Alt-b Q2: No. We share the similar view with Intel that the regulation supports device could potentially operate at any time as an initiating device, a responding device, or both, which means it can change per transmission. A COT initiated by UE can be overlapped with another COT initiated by gNB. We don’t agree with Intel that gNB must terminate its COT if the UE initiates a COT and shares it with the gNB. gNB has freedom to use UE’s COT or use gNB’s COT, similar as LBE. |
| ZTE | Q1: We still support Alt-a Q2: NO. There is no regulation forbidding gNB and UE to initiate overlapping COTs. |
| Asia Pacific | Q1: We support Alt-a. Q2: No. Regulation does not require gNB to terminate its own COT. |
| OPPO | Q1: we support Alt-a |
| DCM | Q2: No, a node is allowed to use the initiated COT until the idle period of the FFP |
| Panasonic | Q2: No. In our view, in this case, both gNB and UE would be ”initiating device” (”initiating device” is the device that initiates COT in the definition of the regulation). According to the regulation (ETSI EN 301 893 section 4.2.7.3.1), there seems to be no description that a COT of initiating device is terminated by other devices. Then, we think that gNB does not need to terminate its ownership of a COT. |
| vivo | Q2: No, by regulations, there is no restrictions to terminate gNB’s ownership of a COT when a UE initiates a COT within that gNB FFP. Q3: We would like to propose one compromised solution. Hope companies can consider it for progress. Since the main motivation for the objective of supporting UE initiated COT is for CG transmission, as the UE should more knowledge about the buffer status. So we would like to at least give the UE a chance to decide whether to share gNB’s COT or initiate its own COT. Compromised solution: Alta + Altb - When a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, if CG-UCI is configured, UE can decide and inform gNB whether the CG transmission uses gNB initiated COT or UE initiated COT; - else if CG-UCI is not configured, Alt-a should be used. |
| Sony | Q1: Still support Alt-a. Q2: Yes. Otherwise, if both UE & gNB are COT initiator, it isn’t clear whether the UE can transmit in gNB’s FFP Idle Period and vice-versa. Q3: On vivo’s compromise solution, does this mean the UE still needs to detect whether gNB has initiated a COT? If gNB did not initiate a COT, is there a need for UE to send this CG-UCI indication? |
| Sharp | Q2: No. I think this issue has been addressed in RAN1 104b-e meeting. Q3: We can support the compromised solution proposed by vivo. If CG-UCI is configured, via CG-UCI reporting the understandings on COT association between gNB and UE can be aligned. The potential misaligned understandings are considered as a drawback of Alt-a. |
| Moderator | Companies’ preferences on Proposal 2-1 are summarized below: - Alt-a: - Nokia/NSB, Intel, HW/HiSi, DCM, Futurewei, IDC, LG, WILUS, Panasonic, Sony, ZTE, MTK, Spreadtrum, OPPO, Asia Pacific, Sharp(2nd), ETRI(2nd) - Alt-b: - QC, vivo, Ericsson, Samsung, Apple, CATT, Sharp(1st), ETRI(1st) - Alt-a/Alt-b: - Lenovo/MOT: For large UE FFP, Alt-a, otherwise Alt-b - ETRI: RRC configure between Alt-a/Alt-b - Vivo: For UL with CG-UCI UE decides between Alta/Altb, otherwise Alta. - Support: vivo, Sharp Companies’ view on Q2: - No: Intel, HW/Hi, Apple, Len/MOT, Nokia/NSB, LG, ETRI, Samsung, ZTE, Asia Pacific, DCM, Panasonic, vivo, Sharp - Yes: [CATT, Sony] - Moderator: It seems the view is due to signalling aspects, not regulations. @CATT/Sony: In my understanding what you refer to, is the signalling aspects that are part of the design. Please read also the explanations to others, e.g. FW. I hope it is clear now that from “regulation” point of view there is no restriction, but the eventual design may lead to restrictions. @LG: To clarify, regulation is transparent, and the question is also applicable to the other side as you rightly pointed out. The reason for formulation like that was based on the descriptions in contributions. And completely agree that for design, we have to consider how gNB and UE properly communicate via implicit/explicit signalling. @Intel: Interesting question. It makes me think that if the UE indicates to gNB to share the COT in CG-UCI, should the gNB follow UE or have the possibility to ignore? We need to discuss these signalling aspects. @FW: Thank you for the good questions. The reason for question was not to be biased towards Alt-b, but rather to have a common understanding on regulations for the decision we make. I do agree that depending on the design and related signalling (implicit/explicit) between gNB and UE, the operation would be different from what regulation allows. Your questions (copied below) are [signalling] design related issue: For Alt-b, please clarify how the gNB during its initiated COT acknowledges (HARQ) an UL transmission from UE in a possible UE initiated COT when gNB COT and UE COT overlaps in time. Or equivalently how two initiating COT devices can communicate back and forth with each other during their overlapping COTs? I assume with overlapping COT, you don’t mean there is collision. My understanding of Alt-b is that if gNB detects UL transmission at the FFP boundary, it assumes UE initiated COT. For Alt-a, if gNB has already initiated COT, by detection of UL at UE FFP boundary within its FFP, assumes UEs shares its COT, otherwise assumes UE initiates its COT. And similarly, at the UE. @Sony: Moderator’s understanding of the proposal is that for the case UE is configured with CG-UCI, UE would transmit anyway CG-UCI with PUSCH. If it doesn’t detect gNB initiated a COT, in that transmission can only indicate that UE initiated COT is used. @All: With clarifications above on Q2, I hope all have the same understanding for the “regulations”. Please note that the communication between UE/gNB with explicit/implicit signalling is part of the 3gpp design as the task on our hands, which we continue to do so. @All: On Alt-a and Alt-b Moderator observation is that the status has not changed. However, I would like to ask your view in the next round on vivo’s compromised proposal. I would like to mention that is seems the compromised proposal is only applicable to CG-PUSCH if CG-UCI is configured. For other configured UL transmission such as SR, PUCCH, as well as CG PUSCH without CG-UCI, Alt-a is applicable. Please vivo clarify if my understanding is incorrect. Proposal 2-1: Agreement: In semi-static channel access mode when a UE can operate as UE-initiated COT, - Select one of the following alternatives to determine whether a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, is based on UE-initiated COT or sharing a gNB-initiated COT: - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT. Vivo’s compromise Proposal 2-1: In semi-static channel access mode when a UE can operate as UE-initiated COT, - When a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, - if CG-UCI is configured, UE can decide and inform gNB whether the CG transmission uses gNB initiated COT or UE initiated COT; - else if CG-UCI is not configured, Alt-a should be used. |

## 2.2.2	Discussion – 2nd round

Companies’ preferences on Proposal 2-1 are summarized below:

- Alt-a:

  - Nokia/NSB, Intel, HW/HiSi, DCM, Futurewei, IDC, LG, WILUS, Panasonic, Sony, ZTE, MTK, Spreadtrum, OPPO, Asia Pacific, Sharp(2nd), ETRI(2nd)

- Alt-b:

  - QC, vivo, Ericsson, Samsung, Apple, CATT, Sharp(1st), ETRI(1st)

- Alt-a/Alt-b:

  - Lenovo/MOT: For large UE FFP, Alt-a, otherwise Alt-b

  - ETRI: RRC configure between Alt-a/Alt-b

  - Vivo: For UL with CG-UCI UE decides between Alta/Altb, otherwise Alta.

    - Support: vivo, Sharp

Proposal 2-1:

Agreement:

In semi-static channel access mode when a UE can operate as UE-initiated COT,

- Select one of the following alternatives to determine whether a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, is based on UE-initiated COT or sharing a gNB-initiated COT:

  - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT

  - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT.

Vivo’s compromise Proposal 2-1:

In semi-static channel access mode when a UE can operate as UE-initiated COT,

- When a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP,

  - if CG-UCI is configured, UE can decide and inform gNB whether the CG transmission uses gNB initiated COT or UE initiated COT;

  - else if CG-UCI is not configured, Alt-a should be used.

Moderator observation/suggestions:

- There is a common understanding on regulations where two devices can simultaneous be COT initiators.

- It is well understood that as part of design, explicit/implicit signalling is needed to communicate COT ownership between UE and gNB.

- The status has not changed, except vivo’s compromised Proposal 2-1.

| Questions: - Q1: Please provide your view on Vivo’s compromised Proposal 2-1. - Q2: Please provide any comments, including follow-up of discussion in previous if needed, or changes the status or suggestions to help to reach a consensus. |  |
| --- | --- |
| Company | Comment |

## 2.3	COT-initiator determination for scheduled UL transmissions

During the last meeting, determination of ownership of a COT for a scheduled UL transmission was extensively discussed and the following agreement was made with reduction of possible options to two main alternatives i.e. Alt-a and Alt-b. The discussion also led to the question whether the channel access field in Rel-16 DCI 0_1 and 1_1, should be extended to Rel-17 DCI 0_2 and 1_2, respectively.

On cross-FFP related discussions, the moderator would like to clarify the following:

- In the contributions for this meeting, few companies shared their views on the preferred alternatives Alt-a or Alt-b, along with the corresponding motivations, also addressed the cross-FFP scheduling, which is an FFS for DCI based method, i.e. Alt-a. However, in previous meeting, the FL summary had captured the discussions related to cross-FFP scheduling in section 2.8 of R1-21003960, which is not prioritized by Chair for this meeting (see section 1 above). Therefore, the discussion on this topic would be limited to clarifications that may be needed in order to progress and conclude the topics in this section unless companies prefer to discuss fully the cross-FFP remaining issues.

On determination of COT initiator

- Alt-a: Companies arguments in favour of Alt-a (DCI based COT determination) include at least the following:

  - Flexibility and efficiency

  - Simplification at UE due to following DCI rather than pre-determined rules

  - Reusing the already existing fields in DCI

  - Unified framework for scheduled uplink for both LBE and FBE

  - …

- Alt-b: Companies arguments in favour of Alt-a (DCI based COT determination) include at least the following:

  - LBT failure impacts/invalidates already indicated COT initiation assumptions in Alt-a:

    - Reduced flexibility and efficiency

    - The need for rule as in Alt-b when operating with Alt-a

  - Flexible since timing flexibility already involved in scheduling and COT determination in Alt-b

  - Unified framework for scheduled and configured UL transmissions in FBE.

  - …

On absence of COT initiator indication in DCI if Alt-a is supported

- Always present in DCI:

  - Similar approach as LBE and FBE in Rel-16

- Can be absent in DCI

  - DCI overhead, optionality and configurability to 0-bit filed for compact DCI

On cross-FFP scheduling

- Few options proposed:

  - Use a combination of Alt-a/Alt-b with consideration to UE processing timeline

  - Always assume UE initiated COT irrespective of indication for UL scheduled outside a COT initiated by gNB

On extension of Rel-16 channel access fields to DCI X_2

- Extend

  - Enable same functionality as DCI X_1

- Do Not extend

  - Optionality and configurability to 0-bit filed for compact DCI

## 2.3.1	Discussion – 1st round

Proposals and summary of views:

Proposal 3-1:

Agreement:

In semi-static channel access mode when a UE can operate as initiating device,

- Select one of the following alternatives to determine whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT:

- Alt-a: Determination based on the content in the scheduling DCI

  - FFS on whether the corresponding field(s) can be absent in DCI

    - If absent, determination based on the rules applied for configured UL transmissions is applied

  - FFS whether/how to handle the case when the gNB schedules an UL transmission in the next gNB’s FFP period

  - Alt-b: Determination based on the rules applied for a configured UL transmission

Companies’ preferences on Proposal 3-1 are summarized below:

- Alt-a (Based on DCI content)

  - QC, Nokia/NSB, Ericsson, Samsung, Apple, Futurewei, IDC, CATT, WILUS, Panasonic, Sony, Sharp, Len/MOT, ETRI, Spreadtrum, OPPO, Intel, [DCM]

- Alt-b (Based on config. UL rule)

  - Vivo, HW/HiSi, LG, ZTE, MTK, [DCM]

- In case Alt-a is supported:

  - Corresponding COT determination field(s) always present in DCI

    - Nokia/NSB, Ericsson, Futurewei, Sony, Len/MOT, Apple, Intel

  - Corresponding COT determination field(s) can be absent in DCI

    - DCM, CATT, Panasonic, ETRI, Spreadtrum

Proposal 3-2:

Select one of the following:

- Option 1: Support the channel access fields in Rel-16 DCI 0_1 and 1_1 to be included in Rel-17 DCI 0_2 and 1_2, respectively.

- Option 2: Study whether to support the channel access fields in Rel-16 DCI 0_1 and 1_1 to be included in Rel-17 DCI 0_2 and 1_2, respectively

Companies’ preferences on Proposal 3-2 are summarized below:

- Option 1: Nokia, Ericsson, Intel, Sharp, ETRI, DCM (can be absent), Intel, Sony, Apple, FW

- Option 2: vivo, HW/HiSi

| Questions: - Q1: Companies are kindly requested to provide any update/correction of their positions with respect to Proposal 3-1 and Proposal 3-2 - Q2: Please provide any other comments if needed. |  |
| --- | --- |
| Company | Comment |
| Sony | Proposal 3-1: We still prefer Alt-a. And the new indicator in the DCI is always present for UE configured to initiate its own COT. Proposal 3-2: We support Option 1. |
| Intel | Many thanks for the moderator effort in summarizing the view from all companies and for the proposal. However, our view is missing for proposal 3.1. We support Alt. a, and our view is that the field carrying information related to the COT initiator should be always carried in both fall-back and non fall-back DCI. As for proposal 3.2, we support 1, and we believe that the extension of unlicensed functionalities (and in this case, channel access & COT initiator information) in the compact DCIs X_2 is quite important to support the reliability targets imposed in Rel.16 for URLLC. |
| Apple | For propose 3-2, we support Option 1. |
| Len/MOT | We think we can continue the high-level designs and decide on fields of DCI x_2 afterwards |
| HW/HiSi | Proposal 3-1: We still support Alt-b. We cannot agree with Alt-a as it is sufficient to address the cases in which the gNB schedules the UL transmission and indicates it is associated to a forthcoming UE/gNB COT. In such cases, if the indicated COT has not been initiated, either a rule similar to that of Alt-b has be applied disregarding the DCI indication or the additional dynamic signaling has to be used to cancel the scheduled UL. This in fact defeats the purposes of simplicity at UE (by just following DCI), providing more control/flexibility to gNB, and efficiency (when dynamic cancellation need to be signaled) Proposal 3-2: We are OK with Option 2 For the question on whether the DCI filed is always/can be present in the scheduling DCI, it is related to the outcome of Propsal 3-2 and the study in its Option 2. That is, if the fields in DCI formats x_0 and x_1 are not agreed to be extended to DCI formats X_2, the field can be absent. |
| FW | We still support Alt-a for Proposal 3.1 , and Option 1 for Proposal 3.2 |
| CATT | Proposal 3-1: we support Alt.-a Proposal 3-2: we prefer to Option 1. |
| vivo | For Proposal 3.1, we support Alt-b. We think it may be better to first discuss whether the COT determination field(s) always exist in all the scheduling DCI formats: 0-0, 0-1 and 0-2. As we know, the channel access fields in Rel-16 is mainly for LBE mode. It is not necessary for FBE mode. To support URLLC and ensure the reliability, compact DCI is one important feature. We did not find strong reason why the channel access fields mainly for LBE mode is mandated to be always there in all the UL scheduling DCI formats. |
| LG | P3-1: We support Alt-b for unified solution to address cross-FFP scheduling as well as multi-carrier transmission, without additional arguments/efforts on various solutions. P3-2: We prefer to Option 2, but not prefer to additional enhancement on the field itself including configurable absence of the field in DCI. |
| ETRI | Proposal 3-1: We still prefer Alt-a. Proposal 3-2: We support Option 1. |
| ZTE | Proposal 3-1: We still prefer Alt-b. Proposal 3-2: We support Option 2. FBE only support a channel sensing of 9us and the channel access filed in Rel-16 DCI 0_1 and 1_1 may not be mandatory in Rel-17 compact DCI 0_2 and 1_2. |
| Samsung | For Proposal 3-1: We still prefer Alt-a. For proposal 3-2, we support option 1. Regarding the channel access fields in Rel-16, we think LBT type and CP extension is also important for FBE mode, e.g. it helps to reduce LBT overhead and reduce LBT failure, which is also beneficial for URLLC. |
| Panasonic | For Proposal 3-1, we still prefer Alt-a. For Alt-a, we also supportive to the condition that the corresponding field cannot be absent in the scheduling DCI. For Proposal 3-2, we support Option 1. |
| QC | Proposal 3-1: We still prefer Alt-a. Proposal 3-2: We support Option 1. |
| Sharp | Proposal 3-1: support Alt-a Proposal 3-2: support Option 1 |
| Asia Pacific | Proposal 3-1: support Alt-a Proposal 3-2: support Option 1 |
| Spreadtrum | Proposal 3-1: Alt-a. Proposal 3-2: We support Option 1. |
| OPPO | Proposal 3-1: We still prefer Alt-a. Proposal 3-2: We support Option 1. |
| Moderator | Companies’ preferences on Proposal 3-1 are summarized below: - Alt-a (Based on DCI content) - QC, Nokia/NSB, Ericsson, Samsung, Apple, Futurewei, IDC, CATT, WILUS, Panasonic, Sony, Sharp, Len/MOT, ETRI, Spreadtrum, OPPO, Intel, Asia Pacific, [DCM] - Alt-b (Based on config. UL rule) - Vivo, HW/HiSi, LG, ZTE, MTK, [DCM] - In case Alt-a is supported: - Corresponding COT determination field(s) always present in DCI - Nokia/NSB, Ericsson, Futurewei, Sony, Len/MOT, Apple, Intel, Panasonic, Intel - Corresponding COT determination field(s) can be absent in DCI - DCM, CATT, Panasonic, ETRI, Spreadtrum Companies’ preferences on Proposal 3-2 are summarized below: - Option 1: Nokia, Ericsson, Intel, Sharp, ETRI, DCM (can be absent), Intel, Sony, Apple, FW, CATT, LG, Samsung, QC, Asia Pacific, Spreadtrum, OPPO - Option 2: vivo, HW/HiSi, ZTE @All: On Proposal 3-1, the arguments and the status with respect to Alt-a/Alt-b in Proposal 3-1 have not changed, with majority of companies see the benefit of Alt-a. With respect to flexibility, the proponents of Alt-b consider that following rule-based approach would provide enough flexibility. It is also argued that Alt-a limits the flexibility. @All: On Proposal 3-2, majority of the companies are supportive of Option 1. However, one of the arguments from the proponents of Option 2 is the lack of configurability to 0-bits field. It is not clear how reliability of DCI is at risk if the corresponding field is not present. @All: The motivations for Alt-a/Alt-b seems originate from different perspective on design. Moderator suggests to focus on concluding this proposal. |

## 2.3.2	Discussion – 2nd round

Companies’ preferences on Proposal 3-1 are summarized below:

- Alt-a (Based on DCI content)

  - QC, Nokia/NSB, Ericsson, Samsung, Apple, Futurewei, IDC, CATT, WILUS, Panasonic, Sony, Sharp, Len/MOT, ETRI, Spreadtrum, OPPO, Intel, Asia Pacific, [DCM]

- Alt-b (Based on config. UL rule)

  - Vivo, HW/HiSi, LG, ZTE, MTK, [DCM]

- In case Alt-a is supported:

  - Corresponding COT determination field(s) always present in DCI

    - Nokia/NSB, Ericsson, Futurewei, Sony, Len/MOT, Apple, Intel, Panasonic, Intel

  - Corresponding COT determination field(s) can be absent in DCI

    - DCM, CATT, Panasonic, ETRI, Spreadtrum

Companies’ preferences on Proposal 3-2 are summarized below:

- Option 1: Nokia, Ericsson, Intel, Sharp, ETRI, DCM (can be absent), Intel, Sony, Apple, FW, CATT, LG, Samsung, QC, Asia Pacific, Spreadtrum, OPPO

- Option 2: vivo, HW/HiSi, ZTE

Proposal 3-1:

Agreement:

In semi-static channel access mode when a UE can operate as initiating device,

- Select one of the following alternatives to determine whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT:

- Alt-a: Determination based on the content in the scheduling DCI

  - FFS on whether the corresponding field(s) can be absent in DCI

    - If absent, determination based on the rules applied for configured UL transmissions is applied

  - FFS whether/how to handle the case when the gNB schedules an UL transmission in the next gNB’s FFP period

  - Alt-b: Determination based on the rules applied for a configured UL transmission

Proposal 3-2:

Select one of the following:

- Option 1: Support the channel access fields in Rel-16 DCI 0_1 and 1_1 to be included in Rel-17 DCI 0_2 and 1_2, respectively.

- Option 2: Study whether to support the channel access fields in Rel-16 DCI 0_1 and 1_1 to be included in Rel-17 DCI 0_2 and 1_2, respectively

Moderator observation/suggestion:

- Considering the observations/clarifications by Moderator in the previous section, let’s focus on concluding Proposal 3-1 at least.

| Questions: - Q1: Please provide any comments, including follow-up of discussion in previous if needed, or changes the status or suggestions to help to reach a consensus. |  |
| --- | --- |
| Company | Comment |

## 2.4	Other issues

Companies discussed other issues mainly related to scheduled UL transmissions with respect to at least the following topics:

Single DCI scheduling multiple PUSCHs

Proposal 4-1:

- In semi-static channel access mode when a UE can operate as initiating device, all the PUSCH transmissions scheduled by a single DCI is based on UE-initiated COT or sharing a gNB-initiated COT based on the same content in the scheduling DCI.

  - Support: CATT

On Early termination of FFP

Proposal 4-2

- In semi-static channel access mode, early termination or cancellation of a FFP is enabled by allowing the gNB to overwrite through scheduling DCI any prior decision regarding the initiator of the COT.

  - Support: Intel

## 2.4.1	Discussion – 1st round

| Questions: - Please share your view on Proposal 4-1 and 4-2. |  |
| --- | --- |
| Company | Comment |
| Sony | Proposal 4-1: It isn’t clear what it means by ”based on the same content in the scheduling DCI.” Proposal 4-2: Support. |
| Intel | Our suggestion is to come back to these proposals once we have progressed in the topic of the COT initiation. |
| Apple | The two proposals are not very clear to us, but we can come back to these proposals later. |
| Len/MOT | We can discuss these aspects after the decision is taken on the question in section 2.2. |
| HW/HiSi | We think the two proposlas need further clarification |
| CATT | For proposal 4-1, we would like to clarify it. Based on agreement in RAN1#104b-e meeting as below, single DCI scheduling multiple PUSCHs is allowed. Agreement: - In semi-static channel access mode, the gNB can schedule by a DCI UL transmission(s) in a later g-FFP that is different from the g-FFP that carries the scheduling DCI. - The UL transmission can occur only if the corresponding channel access requirements are met. - FFS on details. In this case, we need further consider whether these scheduled UL transmissions are based on UE-initiated COT or sharing a gNB-initiated COT based on the content in the scheduling DCI or not. Of course, we need also consider the decision in section 2.3. For proposal 4-2, the motivation need be clarified. |
| vivo | Further clarification is needed for the two proposals. |
| LG | P4-1: It seems better to discuss later since a specific assumption which is not agreed is used. P4-2: It seems better to discuss later with further clarification as other companies commented. |
| ETRI | Proposal 4-1: This issue is related to Proposal 3-1. Can be discussed together with Proposal 3-1, or can be discussed later after single PUSCH case is clear. Either approach is fine for us. Proposal 4-2: The proposal is not clear to us. |
| DCM | We can come back once further progress is made in Sections 2.2 and 2.3 |
| ZTE | The two proposals can be discussed after the proposals in other sections are clarified. |
| Samsung | we can come back later, if time allows. |
| QC | More further discussion is needed for the two proposals. |
| Moderator | @All: Moderator suggests deprioritizing this section for further discussions at this meeting. Needless to mention that companies are welcome to provide more comments if they wish. |

# 3	Conclusion

TBD

# 4	References

| 1 | R1-2104200 | UE initiated COT for semi-static channel access | FUTUREWEI |
| --- | --- | --- | --- |
| 2 | R1-2104219 | Enhancements for IIoT/URLLC on Unlicensed Band | Ericsson |
| 3 | R1-2104265 | Uplink enhancements for URLLC in unlicensed controlled environments | Huawei, HiSilicon |
| 4 | R1-2104328 | Discussion on unlicensed band URLLC/IIoT | ZTE |
| 5 | R1-2104355 | Enhancements for unlicensed band URLLC/IIoT | vivo |
| 6 | R1-2104422 | Discussion on enhancements for unlicensed band URLLC/IIoT | Spreadtrum Communications |
| 7 | R1-2104514 | Enhancements for unlicensed band URLLC/IIoT | CATT |
| 8 | R1-2104665 | Uplink enhancements for URLLC in unlicensed controlled environments | Qualcomm Incorporated |
| 9 | R1-2104804 | Enhancements for unlicensed band URLLC/IIoT | OPPO |
| 10 | R1-2104901 | On the Harmonization of UL CG between NR-U and URLLC and COT-initiator Determination | Intel Corporation |
| 11 | R1-2105099 | URLLC uplink enhancements for unlicensed spectrum | Apple |
| 12 | R1-2105142 | Further UL enhancements for IIoT/URLLC in unlicensed controlled environment | Nokia, Nokia Shanghai Bell |
| 13 | R1-2105143 | Enhancements for unlicensed band URLLC/IIoT | Panasonic Corporation |
| 14 | R1-2105162 | Considerations on unlicensed URLLC | Sony |
| 15 | R1-2105213 | Enhancements for unlicensed band URLLC/IIoT | ETRI |
| 16 | R1-2105304 | Enhancements for unlicensed band URLLC/IIoT | Samsung |
| 17 | R1-2105400 | Enhancements for unlicensed band URLLC/IIoT | InterDigital, Inc. |
| 18 | R1-2105409 | Enhancements for unlicensed band URLLC/IIoT | NEC |
| 19 | R1-2105427 | Discussion on unlicensed band URLLC/IIOT | LG Electronics |
| 20 | R1-2105632 | Enhancements for unlicensed band URLLC/IIoT | Sharp |
| 21 | R1-2105695 | Discussion on enhancements for unlicensed band URLLC | NTT DOCOMO, INC. |
| 22 | R1-2105734 | On the enhancements for unlicensed band URLLC/IIoT | MediaTek Inc. |
| 23 | R1-2105768 | Enhancements for unlicensed band URLLC/IIoT | Lenovo, Motorola Mobility |
| 24 | R1-2105873 | Discussion on enhancement for unlicensed URLLC/IIoT | WILUS Inc. |

# 5	Appendix

## 5.1	List of agreements

### 5.1.1	Agreements in RAN1#102-e

Agreements:

- For semi-static channel access mode,

- If sensing is needed, it is performed immediately before the configured/scheduled transmission opportunity.

- For operation with semi-static channel access, the Rel-16 random starting offsets for UL configured grants with Full BW allocation when UE initiates a COT, is not supported.

Agreements:

- For semi-static channel access mode,

  - When gNB operates as an initiating device

    - The gNB is not allowed to transmit during the idle period of any FFP associated with the gNB in which the gNB initates a COT

  - When a UE operates as an initiating device

    - The UE is not allowed to transmit during the idle period of any FFP associated with the UE in which the UE initates a COT

  - When a UE shares a COT initiated by the gNB during an FFP associated with the gNB

    - The UE is not allowed to transmit during the idle period of that FFP in which the UE shares the COT initiated by the gNB

  - When the gNB shares a COT initiated by a UE during an FFP associated with the UE

    - The gNB is not allowed to transmit during the idle period of that the FFP in which the gNB shares the COT initiated by the UE

  - FFS whether/how to support additional restrictions to the idle period

Agreements:

- For semi-static channel access mode, support using the transmission of any scheduled/configured UL channel/signal to initiate a COT by a UE in RRC_CONNECTED mode

  - FFS the case when the UE is IDLE/INACTIVE mode

Agreements:

- A UE initiates a COT in an FFP associated with the UE, if the UE transmits a UL transmission burst starting at the beginning of the FFP and ending at any symbol before the FFP’s idle period after a successful CCA of 9us immediately before the UL transmission burst.

Update on 8/26

Agreements:

- At least for FBE, configuration of (cg-RetransmissionTimer) should not be mandated when configured grant Type 1 or Type 2 are configured on unlicensed spectrum.

Conclusion:

Further study and decide how to harmonize the CG features for Rel-16 URLLC and Rel-16 NR-U. Table 1 in R1-2005376 can be used as a starting point for the corresponding discussion and decision.

Agreements:

- Conditions on the channel access procedures with respect to sensing duration and transmission gap for UE-initiated COT with UE-to-gNB COT sharing is similar as those for gNB initiated COT and gNB-to-UE COT sharing in Rel-16 by exchanging UE and gNB roles.

Agreements:

- UE-to- gNB COT sharing in semi-static channel access mode is supported.

  - The gNB determines a COT in an FFP associated to a UE, that is initiated by the UE, if the gNB detects a UL transmission from the UE starting from the beginning of the FFP and ending before the idle period of the FFP.

    - FFS details

  - When the gNB determines a UE has initiated a COT in an FFP associated to the UE, the gNB can transmit within the FFP and before the idle period corresponding to the FFP.

    - FFS whether/how UE to gNB COT sharing when the gap is >16us

Update from 8/28 GTW

Agreements:

For semi-static channel access mode,

o    Start of FFP for UE-initiated COT can be different from the start of FFP for gNB-initiated COT.

o    FFS: FFP Periodicity for UE-initiated COT can be different from the FFP periodicity for gNB-initiated COT.

Agreements:

- For semi-static channel access mode,

- FFP parameters for UE-initiated COT can be provided to the UE by at least dedicated RRC signaling.

  - FFS on to be provided by SIB-1

- FFS whether the UE FFP periodicity is explicitly configured, or implicitly determined based on other higher layer parameters

### 5.1.2	Agreements in RAN1#103-e

Agreements:

- In semi-static channel access mode, a single FFP (periodicity and offset) is associated to an initiating device (gNB or UE) at a given time which can be used for the purpose of channel occupancy. The FFP configuration that is used for initiating channel occupancy purposes, is such that it shall not be changed for at least 200ms

Conclusion:

- For operation on unlicensed channels and irrespective of the adopted LBT mechanism (LBE or FBE), all transmissions in DL and UL are controlled by gNB similarly to licensed channels, and potential collisions or blocking are controlled/mitigated by gNB.

Agreements:

  - UE-to-gNB COT sharing in semi-static channel access mode with a gap > 16us is supported

Conclusion:

If a device X at a given time is initiating a COT, the applicable FFP for the device X is the FFP associated with X.

If a device X at a given time is sharing a COT initiated by a device Y, the applicable FFP for the device X is the FFP associated with Y.

Note 1: One of the devices X and Y is a UE and the other is its serving gNB.

Note 2: Whether or not there is additional restriction on idle period is still FFS.

Agreements:

Down-select one of the following options (target RAN1#104-e):

- Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16.

- Option 2-a: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and cg-RetransmissionTimer-r16, respectively.

- Option 2-b: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and new parameter Y, respectively, where X and Y are different from cg-RetransmissionTimer-r16.

- Option 3: CG-UCI based procedures are supported for unlicensed. CG-DFI based procedures are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16

- Note: Procedures based on CG-UCI rely on UE including CG-UCI in CG PUSCH at least as in Rel-16 where the values of the respective fields of CG-UCI are decided by UE.

- Note: Procedures based on CG-DFI rely on automatic re-transmission on CG configuration and reception of CG downlink feedback information (DFI) in DCI for re-transmissions.

Agreements:

- The gNB configures a UE to initiate semi-static CO in an unlicensed channel(s) only if the gNB configures the UE also with the higher layer parameters of the gNB’s initiating semi-static CO in the same channel(s).

  - Note: UE initiated FBE configuration is configured per serving cell

Agreements:

In semi-static channel access mode, FFP Period for UE-initiated COT is separately provided from FFP period for gNB-initiated COT.

o    Note: Any value for the period, shall be at least 1ms and at most 10ms.

o    Note: Aim for low complexity operation to handle gNB and UE COT interactions

Agreements:

In semi-static channel access mode, a UE should be able to determine whether a scheduled UL transmission should be transmitted according to shared gNB COT or UE-initiated COT.

- UE determines the initiator of a COT based on at least one of the following alternatives:

  - Alt 1: Introduce additional bit field in the scheduling DCI

  - Alt 2: Based on ChannelAccess-CPext field in DCI

  - Alt. 3: Based on a predetermined rule(s)

  - Alt. 4: Based on RRC signalling

  - Alt. 5: Based on MAC CE

  - FFS other alternatives

- FFS on overriding possibility and/or the assumption

- Note: A scheduled UL transmission cannot be transmitted according to both shared gNB COT and UE-initiated COT.

Agreements:

In semi-static channel access mode:

- When a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE, down-select one of the following:

  - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT

  - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT.

  - Alt-c: The UE assumption on whether the configured UL transmission is allowed to correspond to UE-initiated COT is based on gNB configuration.

- When a configured UL transmission starts after a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE:

  - If the UE has already initiated the UE FFP, then UE assumes that the configured UL transmission corresponds to UE-initiated COT

  - Otherwise, If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and if the UE has already determined that gNB has initiated that gNB FFP, then UE assumes that the configured UL transmission corresponds to gNB-initiated COT.

- FFS on other conditions for determining the corresponding UE or gNB initiated COT

- Note: A configured UL transmission cannot be transmitted according to both shared gNB COT and UE-initiated COT.

### 5.1.3	Agreements in RAN1#104-e

Agreement:

- PUSCH repetition Type B is supported for unlicensed band operation when using NR IIoT Rel-16 based CG

  - FFS whether/how to enhance

Agreement:

- In semi-static channel access mode, UE FFP periodicity is chosen from the following set of values in ms: {1, 2, 2.5, 4, 5,10}.

  - FFS on other values

Agreement:

- In semi-static channel access mode:

  - An FFP period for UE-initiated COT is configured as the same, integer multiple of, or inter-factor of the FFP period configured for gNB-initiated COT

  - FFP period for UE-initiated COT can be configured independently from FFP period of gNB-initiated COT, if the UE indicates the corresponding capability

  - FFP offset for UE-initiated COT is the starting point of first UE FFP relative to the radio frame X boundary.

    - The offset value range is 0 ≤ offset ＜FFP period of UE-initiated COT

      - FFS on X (e.g. X=0, or X= even index number)

Agreement:

In semi-static channel access mode when a UE can operate as initiating device,

- Select one of the following alternatives to determine whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT:

- Alt-a: Determination based on the content in the scheduling DCI

  - FFS on whether the corresponding field(s) can be absent in DCI

    - If absent, determination based on the rules applied for configured UL transmissions is applied

  - FFS whether/how to handle the case when the gNB schedules an UL transmission in the next gNB’s FFP period

  - Alt-b: Determination based on the rules applied for a configured UL transmission

Agreement:

In semi-static channel access mode when a UE can operate as UE-initiated COT,

- Select one of the following alternatives to determine whether a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, is based on UE-initiated COT or sharing a gNB-initiated COT:

  - Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT

  - Alt-b: The UE assumes that the configured UL transmission corresponds to UE-initiated COT.

Agreement:

- In semi-static channel access mode, sharing a UE initiated COT through the gNB to other intra-cell UEs for UL transmissions, is not supported.

### 5.1.3	Agreements in RAN1#104bis-e

Agreements:

- Support explicit RRC configuration for the UE-FFP parameters including period and offset in RRC connected mode.

Agreements:

- For semi-static channel access mode, the offset value for configuration of a UE-FFP for a serving cell has a symbol level granularity.

The following agreements were made during the GTW on 16th:

Agreement:

- For semi-static channel access mode, in addition to the agreed set of period values for configuration of a UE-FFP for a serving cell:

  - Do not support any additional period value

Agreement:

- For semi-static channel access mode, the starting point of first UE FFP for a serving cell

  - is relative to the boundary of the radio frame of even index number (i.e. X=even indexed number in RAN1#104-e agreement).

Agreement:

- In semi-static channel access mode, the gNB can schedule by a DCI UL transmission(s) in a later g-FFP that is different from the g-FFP that carries the scheduling DCI.

  - The UL transmission can occur only if the corresponding channel access requirements are met.

    - FFS on details.

Agreement:

- In semi-static channel access mode, the gNB can schedule by a DCI DL transmission(s) in a later g-FFP that is different from the g-FFP that carries the scheduling DCI.

  - The DL transmission can occur only if the corresponding channel access requirements are met.

    - FFS on details.

Agreement:

- Select one of the following options (aiming for RAN1#105-e):

  - Option 1: Do not support PUSCH repetition Type Bwhen using based on NR-U Rel-16 based CG for unlicensed band operation.

  - Option 2: Support enhancements of PUSCH repetition Type B when using based on NR-U Rel-16based CG for unlicensed band operation. FFS whether/how to enhance

Agreements

- For PUSCH repetition Type B enhancements on unlicensed spectrum, further study whether PUSCH segmentation should take into account the idle period of an FFP.

  - FFS on details

Agreements

- For PUSCH repetition Type B enhancements on unlicensed spectrum, further study whether orphan symbol(s) are transmitted if they are between two actual repetitions that are transmitted. FFS on details

Conclusion:

- In semi-static channel access mode, a UE as an initiating device, is allowed to transmit during the idle period of any FFP associated with the serving gNB if the UE transmission is based on UE initiated COT

  - Note: the gNB may disallow UL transmission during symbols of the idle period by configuring them either as semi-static DL symbols, or indicating them as DL with SFI.

Agreement:

- Option 2-b and option 3 are not considered further for the agreement in RAN1#103-e regarding CG harmonization

## 5.2	List of observations and proposals in contributions

### R1-2104665  Qualcomm Incorporated	Uplink enhancements for URLLC in unlicensed controlled environments

Proposal 1: gNB indicates UE to initiate a COT based on ChannelAccess-CPext field in DCI.

Proposal 2: The UE assumption that the configured UL transmission is allowed to correspond to UE-initiated COT when the configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP.

Proposal 3: Study ED thresholds selection when UE share its COT to gNB.

Observation 1: Either supporting UE initiated COT in IDLE/INACTIVE mode or allowing PRACH transmission in idle period can provide more chances for the UE to send PRACH.

Proposal 4: Study the following alternatives for PRACH transmission in idle mode:

Alt.1: Supporting UE initiated COT by PRACH transmission in idle mode;

Alt.2: Allowing PRACH transmission in idle period of an FFP.

Proposal 5: Study the following two alternatives for SSB to PRACH mapping:

Alt.1 Divide PRACH occasions into two groups and SSB is mapped to PRACH occasion per group;

Alt.2: Introduce two PRACH configurations and SSB is mapped to PRACH occasions per PRACH configuration.

Proposal 6: Study the following alternatives for MsgA transmission in idle mode:

Alt.1: Supporting UE initiated COT by MsgA transmission in idle mode;

Alt.2: Allowing MsgA transmission in idle period of an FFP.

Proposal 7: Study the following for RO-to-PO mapping:

Alt.1: Divide PUSCH occasions into two groups and PRACH occasion is mapped to PUSCH occasion per group;

Alt.2: Introduce two sets of PUSCH configurations and each PUSCH configuration is associated with one PRACH configuration.

Proposal 8: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and cg-RetransmissionTimer-r16, respectively.

Proposal 9: When cg-RetransmissionTimer is not configured, CG-CUI can be configured including at least HARQ process ID, RV ID and COT sharing information.

Proposal 10: For LBE, configuration of (cg-RetransmissionTimer) should be mandated when configured grant Type 1 or Type 2 are configured on unlicensed spectrum.

Proposal 11: NR-U CG-PUSCH shall support type A PUSCH repetition introduced in Rel.16 URLLC by reinterpreting the # of repetitions in consecutive slots as the # of repetitions in consecutive transmission occasions.

Proposal 12: NR-U CG-PUSCH shall support type B PUSCH repetition introduced in Rel.16 URLLC with the proposal in this contribution.

Proposal 13: NR-U CG-PUSCH shall support type B PUSCH repetition introduced in Rel.16 URLLC with the proposal in this contribution.

Proposal 14: CP extension can be used to handle the non-transmission of orphan symbol for Tpye B PUSCH repetition

Proposal 6: gNB can send a dynamic indicator in a DCI for UE to initiate COT by LBT, while gNB maintains

### R1-2105142  Nokia, Nokia Shanghai Bell 	Further UL enhancements for IIoT URLLC in unlicensed controlled environment

Observation 1: Back-to-back PUSCH repetitions are already supported with NR-U as part of Type A repetitions.

Proposal 1: Non-back-to-back Type A repetitions are not supported in unlicensed band.

Proposal 2: The use of PUSCH repetition Type B together with NR-U based multi-slot allocations should not be considered.

Proposal 3: For PUSCH repetition type B, symbols overlapping with an idle period of an FFP where the UE is not allowed to transmit should be specified as invalid symbol.

Proposal 4: A 1-symbol cyclic prefix extension (i.e. repetition of the following symbol) can be transmitted for filling the orphan symbol(s) gap that is between two actual repetitions that are transmitted, in order to avoid missing a COT during a type B repetition to other initiating device.

Proposal 5: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter (Option 1  from RAN1#103-e), i.e. when cg-RetransmissionTimer is not configured, NR-U specific CG features such as CG-UCI, COT sharing indication, UE-selected HARQ process ID and RV, autonomous retransmission, consecutive allocations for different TB and CG-DFI are not supported.

Proposal 6: PHY multiplexing/prioritization introduced in Rel-16 is supported also with NR-U CG. Interaction of CG-UCI and HARQ-ACK codebooks of different priorities is FFS.

On the support for UE-initiated COT for FBE:

Proposal 7: Alt-a is used for determination of the COT initiator for intra-FFP scheduled UL transmissions.

Proposal 8: The field(s) indicating the COT initiator in the scheduling DCI cannot be absent. Channel access related fields in Rel-16 DCI 0_1 and 1_1 are included in Rel-17 DCI 0_2 and 1_2, respectively and supported with both dynamic as well as semi-static channel access.

Proposal 9: Use a combination of Alt-a and Alt-b for determination of the COT initiator for cross-FFP scheduled UL transmissions. The UE processing delay for gNB COT detection should also be taken into account. Details are FFS.

Proposal 10: Alt-a is chosen for determination of the COT initiator for configured grant UL transmissions.

Observation 2: With semi-static channel occupancy, the mechanism used by a legacy (Rel-16) UE to detect and share a serving gNB COT needs further clarifications if UE-to-gNB COT sharing is supported in Rel-17.

### R1-2104901  Intel Corporation	On the Harmonization of UL CG between NR-U and URLLC and COT-initiator Determination

Proposal 1: If a gNB operates as an initiating device and schedules an UL transmission outside of its FFP, then the UE must assume that the scheduled UL transmission would need to be performed as if the UE is the initiating device irrespectively from any explicit indication provided by the gNB within the scheduled DCI or any implicit assumptions that the UE may be able to make.

Proposal 2: If a gNB operates as an initiating device and schedules via a DCI a DL transmission outside of its FFP, no special considerations are needed in terms of channel access requirements or COT initiator.

Proposal 3: In semi-static channel access mode, a UE performing scheduled UL transmissions determines the initiator of the COT based on the content of the scheduling DCI (Alt-a). In particular the bitfield ChannelAccess-CPext jointly indicates not only the channel access type and CP extension to use, but also whether a UE should operate as initiating or responding device.

Proposal 4: In semi-static channel access mode, early termination or cancellation of a FFP is enabled by allowing the gNB to overwrite through scheduling DCI any prior decision regarding the initiator of the COT.

Proposal 5: When a configured UL transmission is aligned with a u-FFP boundary and ends before the idle period of that u-FFP, if the transmission is confined within a g-FFP before the idle period of that g-FFP, and the UE has already determined that gNB has initiated that g-FFP, then the UE assumes that the configured UL transmission corresponds to gNB-initiated COT (Alt-a). Otherwise, the UE assumes that the configured UL transmission corresponds to UE-initiated COT.

Proposal 6: When operating on multiple carriers, the assumptions regarding the COT initiator are aligned across all carriers/ LBT BWs. In this case, a UE could assume to operate:

as an initiating device over all RBs if for at least one LBT BW i) the UE assesses that it shall operate as initiating in that LBT BW or ii) the UE has received indication to the gNB that it shall operate as an initiating device; or

as a responding device over all RBs, if for each LBT BW i) the UE assesses that it shall operate as a responding device or ii) the UE has received indication from the gNB that it shall operate as responding device.

Proposal 7: The “CG-UCI based procedures” is enabled or disabled through a new RRC parameter, while the “CG-DFI based procedures” is enabled or disabled based on the cg-RetransmissionTimer-r16 (Option 2-a).

### R1-2104355  vivo	Enhancements for unlicensed band URLLC IIoT

Observation 1: No LBT for CG UL transmission within the gNB-initiated COT is not feasible.

Observation 2: Prioritizing the idle period of UE’s FFP is not necessary. gNB can transmit during this idle period if it does not share the UE-initiated COT.

Observation 3: With Alt-a, all the CG PUSCHs not confined within the gNB-initiated COT will not be transmitted if they arrive after a first CG PUSCH which is aligned with the UE’s PPF boundary and confined within the gNB-initiated COT

Observation 4: Alt-a will cause misunderstanding between the gNB and the UE, and additional effort is needed to avoid the confusion.

Observation 5: Indicating the COT sharing scheme in advance will reduce the flexibility and transmission efficiency for FBE in unlicensed band due to the uncertainty of the channel availability.A

Proposal 1: The equipment can be either an initiating device or responding device per active FFP.

Proposal 2: UE assumes that the configured UL transmissions corresponds to UE-initiated COT if it is aligned with the UE FFP boundary.

Proposal 3: Predetermined rules should be used for UE to determine whether a scheduled UL transmission should be transmitted according to shared gNB COT or UE-initiated COT

Proposal 4: Support option 1 as the framework for harmonization of UL Configured Grant (CG).

Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16.

Proposal 5: Do not support PUSCH repetition Type B when using NR-U Rel-16 based CG for unlicensed band operation.

### R1-2104265  Huawei, HiSilicon	Uplink enhancements for URLLC in unlicensed controlled environments

Proposal 1: For determining the COT initiator for configured UL transmissions in semi-static channel access, support Alt-a in the agreement of RAN1#104-e

Observation 1: Given that gNB can control the timing of the scheduled UL transmission which is already involved in the determination of the COT initiator, and given the above cases in which the indication would be disregarded, in addition to the resulting inconsistency of UE behavior across configured and scheduled UL, there is no need to introduce a dynamic indication and increasing the dynamic overhead for the gNB to cancel the UL transmission if the corresponding channel access conditions are not met.

Proposal 2: For determining the COT initiator for scheduled UL transmissions in semi-static channel access, support Alt-b (Determination based on the rules applied for a configured UL transmission depending on whether or not the scheduled UL is confined within an initiated gNB COT) in the agreement of RAN1#104-e.

For Harmonization of CG enhancements:

Proposal 3: Support Option 1, i.e., both ‘CG-UCI based procedures’ and ‘CG-DFI based procedures’ are enabled/disabled using the cg-RetransmissionTimer-r16.

### R1-2104219  Ericsson	Enhancements for IIoT URLLC on Unlicensed Band

Proposal 1: When a first DL transmission at a gNB FFP boundary is assumed to initiate a gNB-COT for that gNB FFP, if a UL transmission at a UE FFP boundary is assumed to initiate a UE COT for that UE FFP and the UL transmission is confined within that gNB FFP, a second DL transmission in that gNB FFP after the UL transmission may be associated to gNB initiated COT.

Proposal 2: When a first UL transmission at a UE FFP boundary is assumed to initiate a UE-COT for that UE FFP, if a DL transmission at a gNB FFP boundary is assumed to initiate a gNB COT for that gNB FFP and the DL transmission is confined within that UE FFP, a second UL transmission in that UE FFP after the DL transmission may be associated to UE initiated COT.

Proposal 3: In semi-static channel access mode, when a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE, the UE assumes that the configured UL transmission corresponds to UE-initiated COT (Alt-b)

Proposal 4: Support inclusion of channel access fields ChannelAccess-CPext-CAPC and ChannelAccess-CPext in DCI formats 0_2 and 1_2, respectively with baseline functionality as in Rel-16. Further study additional enhancements

Proposal 5: In semi-static channel access mode, for a scheduled UL transmission the content in the scheduling DCI determines whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT (Alt-a).  The corresponding fields are assumed to be present in DCI

Proposal 6: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16 (i.e. Option 1).

### R1-2105695  NTT DOCOMO, INC.	Discussion on enhancements for unlicensed band URLLC

Proposal 1:

When a first DL transmission at a gNB FFP boundary is assumed to initiate a gNB-COT for that gNB FFP, if a UL transmission at a UE FFP boundary is assumed to initiate a UE COT for that UE FFP and the UL transmission is confined within that gNB FFP, a second DL transmission in that gNB FFP after the UL transmission may be associated to gNB initiated COT.

When a first UL transmission at a UE FFP boundary is assumed to initiate a UE-COT for that UE FFP, if a DL transmission at a gNB FFP boundary is assumed to initiate a gNB COT for that gNB FFP and the DL transmission is confined within that UE FFP, a second UL transmission in that UE FFP after the DL transmission may be associated to UE initiated COT.

Proposal 2:

When a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP,

If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT.

Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT

Proposal 3:

For a scheduled UL transmission, support one of the following alternatives for the COT-initiator determination

Alt-a: Determination based on the content in the scheduling DCI

The corresponding field(s) can be absent in DCI. If absent, determination based on the rules applied for configured UL transmissions is applied

Alt-b: Determination based on the rules applied for a configured UL transmission

Proposal 4:

Support the channel access fields in Rel-16 DCI 0_1 and 1_1 to be included in Rel-17 DCI 0_2 and 1_2, respectively

The field can be absent in DCI

Proposal 5:

Support Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16.

Indication of COT sharing information included in CG-UCI is separately enabled/disabled from the configuration of cg-RetransmissionTimer-r16

### R1-2105304  Samsung	Enhancements for unlicensed band URLLC IIoT

Proposal 1: When a configured UL transmission aligns with a UE FFP boundary and ends before the idle period of that UE FFP, the UE assumes that the configured UL transmission corresponds to UE-initiated COT (Alt-b).

Proposal 2: For a scheduled UL transmission, the UE determines whether the UL transmission should be transmitted according to shared gNB COT or UE-initiated COT according to LBT indication by existing bit field ChannelAccess-CPext in the scheduling DCI (Alt-a).

Proposal 3: Support option 2-a for IIOT over unlicensed band:

Option 2-a: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and cg-RetransmissionTimer-r16, respectively.

### R1-2105099  Apple	URLLC uplink enhancements for unlicensed spectrum

Proposal 1-1: When a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE, adopt Alt-b, i.e., the UE assumes that the configured UL transmission corresponds to UE-initiated COT.

Proposal 1-2: When a configured UL transmission overlaps with the UE FFP’s idle period,

If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the UL transmission corresponds to gNB-initiated COT using Cat-2 LBT.

Otherwise, it is not transmitted.

Proposal 1-3: For a scheduled UL transmission, the scheduling DCI always indicates whether the UE initiates its own COT or shares gNB’s COT (Alt-a), and the corresponding LBT scheme (Cat-1 or Cat-2 with proper CP extension).

Proposal 2-1: Adopt Option 2-a, i.e., “CG-UCI based procedures” are enabled or disabled for unlicensed using a new RRC parameter, and “CG-DFI based procedures” are enabled or disabled for unlicensed using cg-RetransmissionTimer-r16.

If cg-RetransmissionTimer-r16 is configured, “CG-UCI based procedures” should also be enabled.

### R1-2105409  NEC	Enhancements for unlicensed band URLLC IIoT

Proposal 1: Predetermined rules are supported for both configured transmission and scheduled transmission.

Proposal 2: gNB may send a PDCCH to cancel a low priority UE’s transmission and release the corresponding UE initiated COT in order to support high priority URLLC transmission of another UE.

Proposal 3: Once a UE initiated COT is released by gNB, the UE may not initiate another COT for the same transmission/service until gNB reschedules its UL transmission.

Proposal 4: Support both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16.

### R1-2104200  FUTUREWEI	UE initiated COT for semi-static channel access

Observation 1: gNB controls UEs channel access, potential collisions and blocking, therefore UEs always should defer the channel access and control to gNB.

Observation 2: Alt-a may lead to a better spectrum utilization, while Alt-b may save some energy when UE is not required to determine if gNB initiated a COT.

Observation 3: Alt-b requires further clarifications if and how two initiating COT devices can communicate to each other during their COTs overlap.

Observation 4: In Alt-b when there are multiple UEs initiating COTs transmissions in UL to a gNB initiating COT and gNB responds with multiple DL transmissions, further clarifications on the COT gNB use are necessary.

Observation 5: Alt-b cannot be considered without the required clarifications mentioned in the above observations.

Proposal 1:  Support Alt-a for configured UL transmissions that are aligned with UE FFP boundary.

Observation 6: With an explicit DCI indication of the COT type (gNB/UE) the gNB can control exactly each UE behavior for each UL UE transmission in each gNB FFP.

Proposal 2: Support determination based on the content in the scheduling DCI whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT (Alt-a). The corresponding field(s) cannot be absent in DCI.

Proposal 3: When the gNB schedules an UL transmission in the next gNB’s FFP period support determination based on the content in the scheduling DCI whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT (Alt-a).

### R1-2105400  InterDigital, Inc.	Enhancements for unlicensed band URLLC IIoT

Proposal 1: Use Alt-a for configured UL transmissions occurring at the beginning of a UE FFP.

Proposal 2: Use Alt-a for scheduled UL transmissions.

Proposal 3: For an UL transmission scheduled in a subsequent g-FFP, the UE operates based on the rules applied for a configured UL transmission.

Proposal 4: CG-UCI based procedures and CG-DFI based procedures are independently enabled or disabled for unlicensed (Option 2a)

### R1-2105427  LG Electronics	Discussion on unlicensed band URLLC IIOT

Proposal #1: Alt-a is preferred for the configured UL transmission aligned with UE FFP boundary, with consideration of LBT burden at UE and COT management at gNB.

Proposal #2: Alt-b is preferred for a scheduled UL transmission, for unified/consistent UE behaviour on any UL transmission including multi-carrier transmission and cross-FFP scheduling.

Proposal #3: Option 1 is preferred for the harmonization of NR-U CG and URLLC CG, with consideration of UE complexity and specification impacts.

### R1-2104514  CATT	Enhancements for unlicensed band URLLC IIoT

Proposal 1: In semi-static channel access mode when a UE can operate as initiating device, UE determines whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT based on the content in the scheduling DCI if present and based on the rules applied for a configured UL transmission otherwise.

Proposal 2: In semi-static channel access mode when a UE can operate as initiating device, all the PUSCH transmissions scheduled by a single DCI is based on UE-initiated COT or sharing a gNB-initiated COT based on the same content in the scheduling DCI.

Proposal 3: In semi-static channel access mode when a UE can operate as UE-initiated COT, UE assumes that a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP corresponds to UE-initiated COT.

Proposal 4: Support Option 1 “Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16”.

### R1-2105873  WILUS Inc.	Discussion on enhancement for unlicensed URLLC IIoT

Proposal 1: We support an explicit or implicit signaling mechanism based on the content in the scheduling DCI to determine whether a scheduled UL transmission should be transmitted according to UE-initiated COT or gNB-shared COT.

Proposal 2: We support Alt-a with the same UE behavior on determination of gNB and UE’s COT for the configured UL transmission regardless of whether or not the configured UL transmission is starting from being aligned with a UE’s FFP boundary.

Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT.

Proposal 3: It should be further discussed whether or not to possibly transmit configured-grant PUSCH with repetition at candidate SS/PBCH block positions for the same SS/PBCH block index after the detection of the SS/PBCH block index.

Proposal 4: To enhance PUSCH repetition Type-B for URLLC/IIoT in the unlicensed band, it should be further discussed how to handle on LBT gap/switching gap between segmented transmissions of nominal repetition by slot boundary or between non-contiguous PUSCH Type-B repetitions by DL reception (e.g., candidate SS/PBCH blocks, or others).

### R1-2105143	Panasonic Corporation	Enhancements for unlicensed band URLLC IIoT

Proposal 1: In semi-static channel access mode, when a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE, if the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT.

Proposal 2: In semi-static channel access mode, when a UE can operate as an initiating device, whether a scheduled UL transmission is based on a UE-initiated COT or sharing a gNB-initiated COT is determined based on the content in the scheduling DCI. If the corresponding field(s) is absent, the determination is based on the rules applied for configured UL transmission.

Proposal 3: When configured grant Type 1 or Type 2 are configured on unlicensed spectrum, it would be useful that at least functions related to autonomous retransmission on CG and COT sharing are separately enabled/disabled by the configuration.

Proposal 4: The functions related to autonomous transmission on CG (e.g., DFI, indication of NDI and RV on CG-UCI) are enabled or disabled for unlicensed using one RRC parameter i.e., cg-RetransmissionTimer-r16.

Proposal 5: The functions related to COT sharing information in CG-UCI is independently enabled or disabled.

### R1-2105162	Sony	Considerations on unlicensed URLLC

Observation 1: If the UE always assumes that it is the COT initiator for a configured uplink transmission that starts at the UE’s FFP (i.e. Alt-b), the gNB would have to refrain from transmitting during the UE’s CCA despite not knowing whether the UE would perform an uplink transmission.  This limits the gNB scheduling flexibility.

Observation 2: The UE power saving in not detecting whether gNB has initiated a COT in the case where UE always assumes it is the COT initiator for a configured uplink transmission that starts at the UE’s FFP (i.e. Alt-b), is marginal.

We therefore propose the following:

Proposal 1: The UE determines the COT initiator based on an indication in the scheduling DCI.  A UE that is configured to enable COT initiation is also configured with a COT ownership indicator in the scheduling DCI (Alt-a).

Proposal 2: The COT ownership indicator should not be absent for a UE configured for COT initiation.

Proposal 3: For a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, the COT initiator is determined as follows:

If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that the gNB had initiated a COT in that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT (Alt-a)

Proposal 4: “CG-UCI based procedures” and “CG-DFI based procedures” are independently enabled or disabled for unlicensed using respective RRC parameter, i.e. new parameter X and cg-RetransmissionTimer-r16, respectively. (i.e. Option 2-a)

### R1-2105632	Sharp	Enhancements for unlicensed band URLLC IIoT

Proposal 1: When a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE, the UE assumes that the configured UL transmission corresponds to UE-initiated COT (Alt-b as the first preference). Alt-a can be supported as the second preference.

Proposal 2: Whether a UE-initiated COT is initiated to transmit a dynamic scheduled PUSCH is indicated by the DCI format scheduling the PUSCH, reusing ChannelAccess-CPext/ChannelAccess-CPext-CAPC field in the DCI format.

Proposal 3: In Rel-17 DCI 0_2 and 1_2, include ChannelAccess-CPext/ChannelAccess-CPext-CAPC field.

Proposal 4: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using cg-RetransmissionTimer-r16. (Option 1 can be supported).

### R1-2105734  MediaTek Inc.	On the enhancements for unlicensed band URLLC IIoT

Proposal 1: Support Alt-a with further reduction to the transmission confining interval to take UE processing time into consideration.

Proposal 2: Support Alt-b to determine whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT.

- Alt-b: Determination based on the rules applied for a configured UL transmission

Proposal 3: Support Option 1:

Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16.

### R1-2105768	Lenovo, Motorola Mobility	Enhancements for unlicensed band URLLC IIoT

Proposal 1: For configured UL transmissions,

for large UE-FFP periodicities (e.g., {4, 5, 10} ms) adopt Alt-a, and

for small  UE-FFP periodicities (e.g., { 1, 2, 2.5} ms) adopt Alt-b.

Proposal 2: In semi-static channel access mode when a UE can operate as initiating device, the UE determines if a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT according to the scheduling DCI (i.e., adopt Alt-a).

The COT initiator field is always present

The UE would not transmit the scheduled UL transmission if the indicated COT initiation cannot be achieved.

### R1-2104328	ZTE	Discussion on unlicensed band URLLC IIoT

Proposal 1: When a scheduled/configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP:

If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the scheduled/configured UL transmission corresponds to gNB-initiated COT.

Otherwise, UE assumes that the scheduled/configured UL transmission corresponds to UE-initiated COT.

Proposal 2: Option 1 is supported, i.e., both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16.

### R1-2105213	ETRI	Enhancements for unlicensed band URLLC IIoT

Observation 1: Alt-b may provide a larger flexibility of choosing a COT-initiator (i.e., choosing whether or not to utilize gNB FFP idle period for transmission) for subsequent UL transmissions.

Proposal 2: For COT-initiator determination for configured UL, support one of the followings:

Down-select to Alt-b. (1st preference)

Support RRC configuration between Alt-a and Alt-b. (1st preference)

Down-select to Alt-a. (2nd preference)

Proposal 3: Support the channel access fields in Rel-16 DCI 0_1 and 1_1 to be included in Rel-17 DCI 0_2 and 1_2, respectively.

Proposal 4: For COT-initiator determination for scheduled UL, support Alt-a. If the field is absent in DCI, the determination is based on the rules applied for configured UL transmissions.

Proposal 5: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16 (Option 1).

### R1-2104422	Spreadtrum Communications	Discussion on enhancements for unlicensed band URLLC IIoT

Proposal 1: For scheduled UL transmission, the initiator of a COT can be:

Alt-a: Determination based on the content in the scheduling DCI

the corresponding field(s) can be absent in DCI

If absent, determination based on the rules applied for configured UL transmissions is applied

Proposal 2: When a configured UL transmission is aligned with a UE FFP boundary and ends before the idle period of that UE FFP associated to the UE:

Alt-a: If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT

Proposal 3: Support Option 1 for CG-PUSCH harmonization.

Option 1: Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16.

### R1-2104804	OPPO	Enhancements for unlicensed band URLLC IIoT

Proposal 1: UE should determine the initiator of a COT based on content in DCI.

Proposal 2: Alt-a should be adopted for UE to determine the initiator of a COT for configured grant uplink transmission.

Proposal 3: Option 1 is preferred that CG-UCI procedure and CG-DFI procedure follow R16 NR-U mechanism.

Proposal 4:  cg-RetransmissionTimer can be configured for each configured grant independently.