3GPP TSG RAN WG1 #105-e			      R1-210XXXX

e-Meeting, May 10th – May 27th, 2021

Agenda Item: 8.7.1.1

Source: Moderator (MediaTek)

Title: Summary of Paging Enhancements

Document for: Discussion and Decision

# Introduction

In RAN1#104-bis-e meeting [1][2], there agreed observations for coexistence, performance and resource overhead from companies’ contributions.

Agreement: Observation 1a: For the evaluation and comparison of PEI candidate designs, the following observations for coexistence with legacy PDSCH are identified: - For coexistence with legacy PDSCH, semi-static resouce sharing by configuring RB-symbol-level or RE-level rate-matching patterns covering PEI REs is supported for all PEI candidate designs. - For coexistence with legacy PDSCH, dynamic resource sharing can be realized for all PEI candidates if PDSCH is scheduled by DCI format 1_1 - For PDCCH based PEI, CORESET-level rate matching can be realized for the PDSCH as per mandatory capability - For SSS-based PEI, CORESET-level rate matching may be realized for the PDSCH as per mandatory capability, depending on the design of SSS-based PEI and UE capability regarding number of supported CORESETs - For TRS/CSI-RS based PEI, RE-level rate matching can be realized for the PDSCH as per mandatory capability - When PDSCH is not scheduled by DCI format 1_1, it is up to gNB implementation whether and how PEI is transmitted in PDSCH resource

Agreement: Observation 2a: For the evaluation and comparison of PEI candidate designs, the following summarize the identified configurations of PEI candidate designs, including pairs of the minimum required resource and maximum UE (sub)group indication capacity per PEI, that can comply with the mandatory performance metrics agreed in RAN1 #104-e: - If Behv-A is assumed, (Please refer to Section 8.7.1.1 in [1] for the summary table) - If Behv-B is assumed, (Please refer to Section 8.7.1.1 in [1] for the summary table)

Agreement: Observation 3a: For the evaluation and comparison of PEI candidate designs, the following summarize average resource overheads per PO for PEI candidate designs, considering the configurations identified from performance observation. - The average overhead results are based on PO settings without impact from UE sub-grouping indication within the PO. - Note: For comparison purpose, single-beam transmission for PEI is assumed, and results with multi-beam transmission for PEI is scaled. This doesn’t preclude any beam-forming related design for PEI. - If Behv-A is assumed: (Please refer to Section 8.7.1.1 in [1] for the summary table) - If Behv-B is assumed: (Please refer to Section 8.7.1.1 in [1] for the summary table)

After RAN1 #104-bis-e meeting, RAN2 also sent a reply LS on UE subgrouping for paging enhancement

1. Overall Description: RAN2 would like to thank RAN1 for their LS on UE sub-grouping for paging enhancements. RAN2 discussed this topic and agreed to the following: - We adopt network controlled subgrouping (based on individual UE characteristics, not specified or limited to paging probability as EUTRA, possibly with additional randomization). - If the network chooses to not provide specific subgrouping information, there will be configuration option where subgrouping can be supported by randomization (by UE-ID). - RAN2 also briefly discussed the number of UE subgroups per PO, and, considering it depends on which and how many UE characteristics above network controlled subgrouping will support, RAN2 could only provide an indication that the maximum number of UE subgroups per PO should be at least 8. However, RAN2 understands that it should also depend on observed power saving gain and the signalling method for indicating the subgroups (PEI and/or paging DCI) and so, potentially on the PEI design itself, therefore RAN2 leaves it to RAN1 to decide the final value. 2. Actions: To RAN1: RAN2 respectfully asks RAN1 to take the above information into account for RAN1’s future work.

Since there will be THREE RAN1 meetings left for specification after this meeting, the following targets are suggested for this meeting:

- Specify each of PEI physical-layer candidate designs, facilitating companies’ understanding and decision

  - Specific conclusion for coexistence and resource sharing design

  - Conclude PEI and PO mapping design as determination of PEI monitoring occasions (MOs)

  - Specify support of Behv-A and/or Behv-B for each of PEI physical-layer candidates

  - Subgroups indication design

    - Specify how to carry subgroups indication with each of PEI physical-layer candidates

    - Decide maximum number of subgroups per PO

    - Whether subgroups indication is supported in RAN if PEI is not configured

  - Whether to include additional indications in PEI

  - Collection of remaining details for further decisions:

    - PDCCH-based PEI: DCI format design alternatives

    - TRS-based PEI: Sequence generation alternatives

    - SSS-based PEI: New multi-symbol format, sequence generation, avoidance of impact to neighbor cell initial access, etc.

- Discuss and capture power saving gain observations

  - Clarify difference in companies’ evaluation assumption

  - Capture observations for UE power saving gains

- Decision of PEI physical-layer candidate: PDCCH-based, TRS-based. SSS-based

We hope proponents of each PEI physical-layer candidate can help to clarify the design for other companies so that every company can best understand each design and select their best candidate for the final PEI physical-layer design.

To assist “dialogues” between companies, we will use NWM for companies to raise questions and provide responses “in parallel”, according to a “tag” naming rule. Please go to https://nwm-trial.etsi.org/#/documents search the following title: RAN1-105-e-NWM-NR-R17-PowSav-01, and provide your valuable feedbacks to FL or any other company’ question(s) or response(s).

# Characteristics of PEI Physical-Layer Candidate Designs from 
RAN1 #104-bis-e Observations

Based on the observations in RAN1 #104-bis-e, specific characterization/specification on the following topics for each of PEI physical-layer candidate designs will be discussed and decided:

- Sub-section 2.1: Coexistence and resource sharing/overhead

- Sub-section 2.2: PEI and PO mapping

- Sub-section 2.3: Support of Behv-A and/or Behv-B

Finally sub-section 2.4 collects companies further comparisons not related to UE subgrouping enhancement. Since UE power saving observation is missing in previous meeting, we can aim for a power saving observation if feasible.

## Coexistence and Resource Sharing/Overhead

In Table 1, companies’ views related to PEI coexistence with legacy UEs and resource sharing/overhead are collected:

Table 1: Companies’ views related to PEI coexistence with legacy UEs and resource sharing/overhead

| Company | Company Views |
| --- | --- |
| ZTE, Sanechips | TS 38.306 multipleCORESET Indicates whether the UE supports configuration of up to two PDCCH CORESETs per BWP in addition to the CORESET with CORESET-ID 0 in the BWP. If this is not supported, the UE supports one PDCCH CORESET per BWP in addition to the CORESET with CORESET-ID 0 in the BWP. It is mandatory with capability signaling for FR2 and optional for FR1.. - Observation 9: It is problematic to configure a dedicated CORESET for SSS-like PEI to co-exist with legacy PDSCH by implementation, considering supporting more than 2 CORESET is an optional UE capability for FR1, and the different resource mapping/assignment between SSS-like PEI and CORESET, etc. - Observation 10: It is expected that there shall be no available ZP-CSI-RS-ResourceSet to handle the coexistence between TRS/CSI-RS based PEI and legacy PDSCH. - Proposal 2: For SSS-like PEI and TRS-like PEI, semi-static RB- symbol-level rate matching pattern should be used to coexistence with legacy PDSCH. - Observation 11: For SSS-like PEI and TRS-like PEI, the UE behavior is not defined in Rel-15 specification for the coexistence with PDCCH/SSB. - Observation 12: SSS-like PEI may impact the initial access procedure and neighbor cell measurement of the legacy UEs. - Observation 13: TRS-like PEI is not good for the power consumption of idle/inactive mode UE due to the large bandwidth. - Observation 14: To co-exist with other signals/channels, the mechanisms in the current specifications can be reused for DCI-based PEI. However, new mechanisms are needed for sequence-based PEI to resolve the resource collision with other signals/channels. Table 3 Summary of the co-existence of PEI candidate design with legacy UEs PEI candidate; Coexistence with PDSCH; Coexistence with PDCCH; Coexistence with SSB. DCI based PEI; - Semi-static rate-matching; - Dynamic rate-matching: - Based on UE capability #5-27a which is mandatory; - Scheduled by DCI format 1_1/1_2;; Transparent to Rel-15 UEs by virtue of mandatory support of blind decoding; Rel-15 UEs is not required to monitor the PDCCH candidate colliding with SSB. TRS/CSI-RS like PEI; - Semi-static rate-matching; - Dynamic rate-matching: RE-level rate matching. - Scheduled by DCI format 1_1/1_2; - Note: Whether the ZP-CSI-RS- ResourceSet is available for TRS/ CSI-RS like PEI should be considered.; The UE behavior is not defined in Rel-15/16 specification. Note: The large bandwidth to transmit TRS/CSI-RS like PEI is not good for the power consumption of idle/inactive mode UE. SSS-like PEI; - Semi-static rate-matching; - Dynamic rate-matching: CORESET level rate matching. - Scheduled by DCI format 1_1/1_2; - Note: The dedicated CORESET configuration is required, which is problematic due to the optional UE capability.; The UE behavior is not defined in Rel-15/16 specification. Note: If the SSS-like PEI is transmitted in the same frequency band as the SSS in SSB, the PEI may be taken as an SSS by other UEs, which may impact the initial access procedure and neighbor cell measurement of the legacy UEs.. |
| Huawei, HiSilicon | Observation 1: DCI based PEI can fully support coexistence with the legacy downlink physical layer channels/signals of legacy UE based on Rel-15 specifications. Observation 2: Semi-statically configured rate matching pattern can be used to multiplex SSS-based PEI and PDSCH(s) of legacy UE. Observation 3: Using dedicated CORESET for dynamic rate matching of SSS-based PEI restricts the legacy UE from using UE specific CORESET in connected mode in FR1, and restricts multi-beam operation of downlink traffic for FR2. Observation 4: CORESET0 with additional search space set would be used for dynamic rate matching of SSS-based PEI, which may consume the whole CORESET0 resource if the SSS based PEI needs to be rate matched. Figure 1 Configuring a shared CORESET for SSS-based PEI would block the AL2~AL8 candidates (The RBs with the same color corresponds to the same PDCCH candidate) Figure 2. One ZP-CSI-RS resource set contains multiple ZP-CSI-RS resources to cover multiple TRS-based PEI occasions in one slot Observation 5: At most one aperiodic ZP-CSI-RS resource set can be utilized on legacy UE for rate matching purpose of PEI considering Rel-15 only supports up to 3 aperiodic ZP-CSI-RS resource sets per BWP. Observation 6: The ZP-CSI-RS resources for the TRS-based PEI occasions associated with different sub-groups or different POs in the same SS burst period have to be configured within the same ZP-CSI-RS resource set. Figure 4. SSS-based PEI might impact the legacy cell search procedure Observation 17: The SSS-based PEI has impact on the legacy cell search procedure. |
| vivo | Observation 9: Sequence-based PEI can be well co-existed with legacy signals/channels. |
| Spreadtrum | Observation 4: For Behv-A, DCI-based PEI has comparable resource overhead compared to sequence-based PEI. For Behv-B, DCI-based PEI has lower resource overhead than sequence-based PEI. For collision handling between PEI and PDCCH of legacy UE, gNB should hold the PDCCH transmission to avoid the collision, and the legacy UE needs to detect the legacy PDCCH “absent” under risk of increase of false alarm rate. In the discussion of collision handing between SSS-based PEI and PDCCH of legacy UE in RAN1#104bis-e, it was argued that SSS-based PEI can be mapped to CORESET#0 like a PDCCH candidate. However, CCE-to-REG bundle mapping in CORESET#0 is in interleaving manner, so SSS-based PEI is scattered in different PDCCH candidates, and then the risk of false alarm rate is higher. For resource sharing between PEI and PDSCH of legacy UE, gNB should inform the legacy UE to perform PDSCH rate matching around PEI and gNB should schedule the PDSCH with DCI format 1_1. It is still restrictive. Observation 5: Semi-static resource sharing between PEI and legacy signals/channels is better, and Behv-B is suitable. Observation 7: For Behv-B and DCI-based PE, PEI can be semi-statically configured to avoid collision with SS burst. Observation 8: Behv-B is more robust for resource sharing or collision handling between PEI and other signals/channels. |
| CATT | Observation 4: DCI-based PEI or sequence-based PEI can coexist with dynamic resource sharing with PDSCH used by legacy UE if it is scheduled by DCI format 1_1 or Rel-17 UEs. Observation 5: PDCCH-based PEI with dynamic resource sharing of CORESET used by legacy UE increases the PDCCH blocking probability for legacy PDCCH. Observation 6: Resource sharing of TRS used by legacy UE and TRS-based PEI allows TRS-based PEI without additional resources. Proposal 2: The sequence-based PEI can co-exist well with existing channels and signals. Table 3: Resource overhead evaluation for PEI without assuming the support of multiple POs and sub-grouping PEI candidate designs; Resource overhead(semi-static); Resource overhead(dynamic). PDCCH-based PEI(12 bits); 288REs; 28.8 – 288 REs (overheads still count when CCEs reserved for PEI are not used by other UEs or by PDSCH when no paging). PDCCH-based PEI(41 bits); 576REs; 57.6 – 576 REs (overheads still count when CCEs reserved for PEI not are used by other UEs or by PDSCH when no paging). TRS-based PEI; 0 or 288REs(0.8dB better than 12 bits PDCCH-based PEI); 0 or 28.8REs(0.8dB better than 12 bits PDCCH-based PEI). Observation 7: The resource overhead of sequence-based PEI is not greater than that of DCI-based PEI when sub-grouping is not considered for one PO associated with one PEI. |
| CMCC |  |
| Qualcomm | Observation 8: Depending on network implementation and UE capability - For higher layer configured or SPS CSI-RS, resources of configured PEI occasions are semi-statically not available - For legacy PDCCH or aperiodic CSI-RS, resources of configured PEI occasions can be dynamically not available if the PDCCH or CSI-RS is not transmitted when it collides with a transmitted PEI. Table 6 is the lower bound of amount of unavailable resources Table 5: Resource overhead for PEI that is semi-statically not available to legacy channels/signals PEI design; No. of UE sub-groups = 2; No. of UE sub-groups = 4; No. of UE sub-groups = 6; No. of UE sub-groups = 8. PDCCH; 576; 576; 576; 576. CSI-RS; 300; 600; 900; 1200. SSS; 288; 576; 864; 1152. Table 6: Resource overhead for PEI that is dynamically not available to legacy channels/signals PEI design; No. of UE sub-groups = 2; No. of UE sub-groups = 4; No. of UE sub-groups = 6; No. of UE sub-groups = 8. PDCCH; 57.6; 57.6; 57.6; 57.6. CSI-RS; 30; 30.8; 31.1; 31.2. SSS; 28.8; 29.6; 29.8; 29.9. |
| OPPO | Observation 3: The beam sweeping of PEI transmission requires large resource overhead. Observation 4: Reusing legacy PDCCH CSS set for PEI delivery has no backward compatibility issue. Proposal 2: Legacy PDCCH CSS set can be reused for paging early indication delivery to reduce resource overhead. |
| Intel | Observation 4: Both TRS and PDCCH-based PEI could meet the MDR requirements however TRS-based PEI provides significant resource overhead advantage over PDCCH-based PEI. - For example, to meet SNR -7.79dB at 1% BLER for PDSCH with TB scaling 1 and CFO 0ppm, 24 RBs (144 REs) suffices for TRS whereas AL8 (576 REs) are needed for PDCCH-based PEI. Observation 6: AL 8+ is necessary for PDCCH-based PEI in most cases. Observation 9: TRS-based PEI achieves most efficient rate matching when coexisting with PDSCH. Observation 10: At a given instance, the CORESET where PDCCH based PEI is transmitted may not have other PDCCH candidates transmitted, resulting in much larger overhead than resource occupied by PDCCH based PEI when dynamic CORESET-level rate matching is considered. Observation 11: TRS can coexist with CORESET and SSB according to Rel-15 design. |
| Apple | If we assume total system bandwidth of 100 MHz, the worst case that there are 4 POs in every paging frame (10ms), 10% group paging rate with Behv-A, and PEI using PDCCH AL 8 (576 REs), the PEI overhead for the system is ~0.05%. Note that this is a very extreme case. From the calculation, we can see that the overhead is quite small. In addition, if we compare PDCCH-based vs sequence-based solution, the overhead difference should be quite minor when similar performance is considered. Therefore, we do not see the overhead as the main decision factor to choose between the two types of solutions. |
| Sony |  |
| Samsung | Observation 1: TRS/CSI-RS based PEI has best coexistence performance with legacy signal/channel of Rel-15 UEs due to RE-level rate matching pattern. Table 2: Average resource overhead per PO (REs) for different PEI designs for Behav-A Alternatives; Dynamic rate-matching; Semi-static rate matching. PDCCH-based; 28.8 -57.6; 288.0 – 576.0. SSS-based; 25.4 - 34.0; 254.0 - 437.0. TRS/CSI-RS based; 14.4 – 30.0; 168.0 - 300.0. Table 3: Average resource overhead per PO (REs) for different PEI designs for Behav-B Alternatives; Dynamic rate-matching; Semi-static rate matching. PDCCH-based; 518.4 (always transmitted). SSS-based; 228.6; 254.0 - 561.0. TRS/CSI-RS based; 259.2; 168.0 - 279.0. Observation 6: The resource overhead for different PEI designs has the following order: - PDCCH based > SSS-based > TRS/CSI-RS based Proposal 6: Support TRS/CSI-RS based PEI for low resource overhead. |
| MediaTek | Observation 1: It is optional UE capability for Rel-15 UE to support more than one CORESET in addition to CORESET 0 in FR1, as specified by the IE, multipleCORESET, in TS 38.306 (Release 15) Table 1: UE capability IE related to supported number of CORESETs in TS 38.306 Definitions for parameters; Per; M; FDD-TDD DIFF; FR1-FR2 DIFF. multipleCORESET Indicates whether the UE supports configuration of up to two PDCCH CORESETs per BWP in addition to the CORESET with CORESET-ID 0 in the BWP. If this is not supported, the UE supports one PDCCH CORESET per BWP in addition to the CORESET with CORESET-ID 0 in the BWP. It is mandatory with capability signaling for FR2 and optional for FR1.; UE; CY; No; Yes. Observation 2: Since the frequency span of the CORESET in addition to CORESET 0 can exceed idle-mode UE reception bandwidth, SSS-based PEI can only utilize CORESET 0 to guarantee dynamically sharing PDSCH of Rel-15 UE. Observation 3: For SSS-based PEI, CORESET0 has at least the following limitations: - Multi-symbol format is not always available (as the example quoted in Table 2) - Minimum RE number in CORESET 0 is at least 576 REs for FR1, which in turn bounds the minimum resource overhead for dynamically sharing PDSCH of REl-15 UE. Table 2: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {30, 30} kHz for frequency bands with minimum channel bandwidth 5 MHz or 10 MHz Index; SS/PBCH block and CORESET multiplexing pattern; Number of RBs; Number of Symbols; Offset (RBs). 0; 1; 24; 2; 0. 1; 1; 24; 2; 1. 2; 1; 24; 2; 2. 3; 1; 24; 2; 3. 4; 1; 24; 2; 4. 5; 1; 24; 3; 0. 6; 1; 24; 3; 1. 7; 1; 24; 3; 2. 8; 1; 24; 3; 3. 9; 1; 24; 3; 4. 10; 1; 48; 1; 12. 11; 1; 48; 1; 14. 12; 1; 48; 1; 16. 13; 1; 48; 2; 12. 14; 1; 48; 2; 14. 15; 1; 48; 2; 16. Proposal 1: For SSS-based PEI, semi-static sharing PDSCH of Rel-15 UE should be assumed as baseline. - Dynamically sharing PDSCH of Rel-15 UE based on CORESET 0, if available, will occupy at least 576 REs when a SSS-based PEI is transmitted. Proposal 2: For PDCCH-based PEI, dynamically sharing CORESET 0 of Rel-15 UE is supported. Proposal 3: For PDCCH-based PEI, Behv-A or Behv-B is configurable by gNB for prioritization on PEI or legacy PDCCH, accordingly. Observation 4: For TRS/CSI-RS-based PEI, dynamic sharing PDSCH of Rel-15 UE is conditioned on: - There can configure at least one aperiodic ZP-CSI-RS rate-matching resource set to all Rel-15 connected-mode UEs out of the maximum of three resource sets supported by each Rel-15 UE. - gNB can manage the performance impact to UEs receiving PDSCH scheduled by DCI format 1_0 or PDSCHs with SPS activated by DCI format 1_0 since those UEs will assume the PEI occasion can be used for PDSCH even when PEI is transmitted. Proposal 4: For TRS/CSI-RS-based PEI, semi-static sharing PDSCH of Rel-15 should be assumed as baseline. - Dynamically sharing PDSCH of Rel-15 UE, if available, should utilize at most one aperiodic ZP-CSI-RS rate-matching resource set for the minimum impact to Rel-15 network. |
| LG | It seems not possible to support resource overhead dynamic adaptation by the gNB when sequence based PEI is adopted. Note that DMRS for the legacy PDCCH should be guaranteed and should not be interfered by other reference signal. However, if sequence based PEI overlaps with the CORESET for the legacy PDCCH transmission, there is no way to avoid collision between DMRS for the legacy PDCCH and PEI sequence. Observation 7: PDCCH based PEI can support coexistence with the legacy PDCCH. |
| Panasonic | Observation 2: To check the average resource overhead in terms of REs for PEI design candidates, the metric should be RE numbers per bit of indication. |
| Nokia | Observation: PDCCH-based EPI multiplexing with Connected Mode UEs is most straight forward, while with different mechanisms multiplexing of TRS-EPI and SSS-EPI with Connected mode UEs can be achieved. PDCCH-EPI would affect PDCCH capacity, while TRS- and SSS-EPI would impact to the PDSCH capacity. Resource reservation for TRS and SSS-based EPI would depend on the number of sub-groups supported per single SSS-sequence or TRS resource set. Observation: In TRS- and SSS-based EPI design sufficient sequence space would need to be reserved to avoid collisions between neighbouring cells. In asynchronous deployments, only frequency domain and sequence domain would needed to be used ensure the orthogonality. |
| Xiaomi | Observation 5: Sequence-based PEI, especially TRS/CSI-RS, can easily encounter resource conflicting issue with legacy channels/signals. Observation 6: SSS-based PEI may be mistaken as SSS for cell identification/synchronization by legacy UEs. |
| DOCOMO | In the last meeting, we discussed how PEI coexist with legacy signals/channels. Regarding SSS-based PEI, we understand that CORESET-level rate matching may be realized for the PDSCH if a dedicated CORESET and Search Space set are configured for a rate matching. However, there may be impact for legacy UE since it causes additional power comsumption to do the blind detection in a dedicated CORESET and Search Space set. a dedicated CORESET used only for the rate matching wastes the configurable number of CORESET. When it comes to coexsistance SSS-based PEI with PDCCH, the current specification defines the UE behavior for coexistence only based on SSB provided by SIB1 [section 10 in TS38.213] . Observation 5: There are some restrictions in SSS-based PEI to avoid to impact legacy UE. |
| InterDigital |  |
| Lenovo |  |
| Ericsson | - Observation 5 In multi-beam deployments for which PEI needs to be swept in multiple occasions, a single symbol PDCCH based PEI is more efficient both in terms of UE/NW power consumption and system resource utilization than multi-symbol TRS/SSS based PEI. - Puncture other user traffic - This would have high traffic/KPI impact (both on legacy and new UEs) especially at higher paging rates and/or if multiple sequences are transmitted addressing multiple groups with multiple information elements. - Rate-match around TRS/SSS using semi-static resource sharing - Locking up resources (semi-)statically for the sake of potential PEI transmission is too wasteful. It shall further be noted that such resources cannot be communicated to UEs in idle/inactive mode and would therefore limit the occasions used for broadcast transmissions. - Rate-match around TRS/SSS using dynamic resource sharing - For SSS, creating a fake Coreset to make UEs rate-match around it is cumbersome. This is also infeasible in some cases where legacy UEs support limited number of Coresets and both Coresets (with much different characteristics than fake Coreset) are used for regular scheduling. - For TRS, when it comes to dynamic resource reservation with RE granularity, only up to 2 bits are available for dynamic indication through DCI in each of those (“Rate matching indicator”/” ZP CSI-RS trigger”). We think that these bits are needed to convey information about reserved resources used for typically higher prioritized usage cases (rate-matching around connected mode TRS/CSI-RS transmissions, neighbor cell signals, etc.). It would hence not be wise to spend these bits on PEI. - Rate match other user traffic around these sequences/signals (TRS/SSS) - This would mean that all UEs in connected mode, including pre-Rel-17, would have to be aware of the resources reserved for potential PEI transmission, which creates unnecessary signaling overhead for both NW and legacy UEs. There would still be negative impact on user traffic when multiple sequences are potentially transmitted, and considerable NW configuration effort would be necessary with respect to reserved resources coupled to paging configuration. - Observation 5 Dynamic resource reservation through “Rate matching indicator” in DCI is an optional UE capability and can therefore not be assumed for legacy UEs. - Observation 6 Reserved (semi-)static resources used for sequenced-based PEI for the sake of potential PEI transmission locks up resources even though no paging occurs. Furthermore, these reserved resources cannot be communicated to UEs in idle/inactive mode and would therefore limit the occasions used for broadcast transmission. |
| Nordic | Observation-1: All candidate signals/channel may coexist with PDSCH, but only TRS and PDCCH can be multiplexed with signal/channel of its own kind. Observation-2: Based on agreed Observation 2 and 3: - PDCCH can carry the largest payloads and payload size is scalable by means of padding. - Overhead per bit is the smallest for PDCCH Furthermore, it has been agreed in idle TRS sub-AI that TRS are explicitly indicated, and blind detection is not intended: Agreements: For a cell with TRS/CSI-RS occasions configured for IDLE/Inactive UEs, IDLE/Inactive UE’s assumption on the availability of TRS/CSI-RS at the configured occasion(s) is informed to the idle/inactive UE based on explicit indication. - FFS details (e.g., the signalling, detailed information for the TRS/CSI-RS, etc.) - There is no intended blind detection of the presence/absence of TRS/CSI-RS at the UE side in this feature. That is, the UE assumes TRS/CSI-RS is not present if the network does not indicate it is available (or indicates it is unavailable).. Observation-3: There was no support for TRS as TRS for idle UEs to be blind detected, it is unclear why TRS as PEI should be treated differently. |

In RAN1 #104-bis-e, coexistence with legacy UEs is discussed with high-level observation captured. From companies’ inputs to this meeting, the following facts will impact the feasibility of CORESET based dynamic sharing of PDSCH of legacy UEs:

- It is optional UE capability in FR1 for Rel-15 UEs to support more than one additional CORESET to CORESET 0, according the following IE of TS 38.306:

multipleCORESET Indicates whether the UE supports configuration of up to two PDCCH CORESETs per BWP in addition to the CORESET with CORESET-ID 0 in the BWP. If this is not supported, the UE supports one PDCCH CORESET per BWP in addition to the CORESET with CORESET-ID 0 in the BWP. It is mandatory with capability signaling for FR2 and optional for FR1.

- CORESET 0 applies interleaved resource allocation, and localized SSS-based PEI will extensively occupy CORESET 0

By 1, both PDCCH-based PEI and SSS-based PEI needs to share CORESET 0 since the other CORESET in FR1 can exceed idle-mode UE reception bandwidth (up to 20 MHz). By 2, for SSS-based PEI cannot share existing CORESET 0 and instead requires exclusive occupation of a dedicated CORESET 0 of 576 or 1152 REs.

Since coexistence with legacy UEs is the fundamental design metric for paging enhancement, we suggest to make specific conclusions on the feasible coexistence schemes for each of PEI physical-layer candidate designs. Since coexistence has network impact, we also categorize the available schemes according to whether additional gNB DCI indication to legacy UEs is required or not.

FL- [Q2.1-1]

Conclusion:

If PDCCH-based PEI is selected for Rel-17 paging enhancement, either of the following two schemes for coexistence with legacy Rel-15/16 UEs can be utilized as per network configuration:

- Without additional gNB DCI indication to legacy UEs:

  - PDCCH-based PEI can share existing CORESET 0 as a Rel-15 PDCCH

  - Network can configure Behv-A or Behv-B to Rel-17 idle/inactive mode UEs and prioritize PEI or legacy PDCCH, respectively

- With additional gNB DCI indication to legacy UEs:

  - PDCCH-based PEI can dynamically share an additional CORESET 0 provided by additional search space set(s) associated with CORESET 0 and set to rateMatchPatternToAddModList in PDSCH-Config or ServingCellConfigCommon for legacy UEs

  - When PDCCH-based PEI is not transmitted, network can indicate the REs of the additional CORESET 0 are available for PDSCH scheduled by DCI format 1_1

  - For PDSCH scheduled by other DCI formats, UE assumes the REs of the additional CORESET 0 are not available for the PDSCH

- DCI payload size carried by PDCCH-based PEI should be different from DCI format 1_0

FL- [Q2.1-2]

Conclusion:

If SSS-based PEI is selected for Rel-17 paging enhancement, either of the following two schemes for coexistence with legacy Rel-15/16 UEs can be utilized as per network configuration:

- Without additional gNB DCI indication to legacy UEs,

  - SSS-based PEI can semi-statically share PDSCH REs of Rel-15/16 UEs by configuring at least one RB-level RateMatchPattern set to ateMatchPatternToAddModList in PDSCH-Config or ServingCellConfigCommon for legacy UEs

- With additional gNB DCI indication to legacy UEs,

  - SSS-based PEI can dynamically and exclusively occupy an additional CORESET 0 provided by additional search space set(s) associated with CORESET 0 and set to rateMatchPatternToAddModList in PDSCH-Config or ServingCellConfigCommon for legacy UEs.

  - When SSS-based PEI is not transmitted, network can indicate the REs of the additional CORESET 0 are available for PDSCH scheduled by DCI format 1_1

  - For PDSCH scheduled by other DCI formats, UE assumes the REs of the additional CORESET 0 are not available for the PDSCH.

- FFS: Signal design that can avoid impact to initial access of neighbour cell UEs

FL- [Q2.1-3]

Conclusion:

If TRS/CSI-RS-based PEI is selected for Rel-17 paging enhancement, either of the following two schemes for coexistence with legacy Rel-15/16 UEs can be utilized as per network configuration:

- Without additional gNB DCI indication to Rel-15/16 UEs,

  - TRS/CSI-based PEI can semi-statically share PDSCH REs of legacy UEs by configuring one sp-ZP-CSI-RS-ResourceSetsToAddModList or p-ZP-CSI-RS-ResourceSet in PDSCH-Config for legacy UEs

- With additional gNB DCI indication to Rel-15/16 UEs,

  - TRS/CSI-based PEI can dynamically share PDSCH resources of legacy UEs by configuring one aperiodic-ZP-CSI-RS-ResourceSetsToAddModList in PDSCH-Config for legacy UEs

  - When TRS/CSI-based PEI is not transmitted, network can indicate the resources corresponding to the aperiodic-ZP-CSI-RS-ResourceSetsToAddModList are available for PDSCH scheduled by DCI format 1_1

  - For PDSCH scheduled by other DCI formats, UE assumes the REs of the additional CORESET 0 are always available for the PDSCH

## PEI and PO Mapping

In Table 2, Companies’ views related to PEI and PO mapping related PEI and PO mapping are collected

Table 2: Companies’ views related to PEI and PO mapping

| Company | Company Views |
| --- | --- |
| ZTE, Sanechips | - Observation 19: For DCI-based PEI, one PEI associated with multiple POs will not degrade the UE power saving gain. - Observation 21: For sequence based PEI associated with multiple POs, - if PEI is generated by different sequences, the performance of sequence-based PEI will be deteriorated with the increased blind detection； - if a group common PEI is used, the power saving gain of the UE will decrease due to the unnecessary wake-up, especially when the group paging rate is large. - if TDM or FDM technique is used for sequence-based PEI, the resource overhead will be multiplied. Table 4 UE power saving gain for PEI associated with different number of POs Group paging rate; PEI associated with one PO; PEI associated with four POs. DCI/sequence-based PEI; DCI-based PEI; Sequence-based PEI (w/ group common PEI). RG = 10%; 22.5%; 22.5%; 21.6%. RG = 40%; 14.4%; 14.4%; 9.1%. RG = 60%; 9.3%; 9.3%; 2.9%. Figure 8 MDR performance when multiple PEI sequences occupy the same resource |
| Huawei, HiSilicon | Observation 7: It is common issue for all three PEI candidates to associate PEI transmission(s) near a SS burst to the different POs during a SS burst period after the PEI transmission. In our opinion, the paging delay would not be a problem for multiple PO association. Take the case in Figure 3 , for example, it is assumed that each PO is consist of eight monitoring occasions and each slot contains two monitoring occasions. The time offset between the first monitoring occasion of the first PO and the first monitoring occasion of the last PO associated with the same PEI is 12-1x8/2=44slots=22ms for 30kHz SCS. Assuming that one PEI is transmitted at the time of T, if the paging message for UEs monitoring the last PO arrives in the time period (T,T+22ms], the paging message will be delayed to the next DRX cycle. Compared to the typical idle DRX cycle length, which is 1280ms, it means that only 221280=1.72% paging message arrives in that time period, the delay is negligible. Figure 3 One PEI associating with 12 POs Observation 8: One PEI associating with multiple POs can save the resource overhead for PEI and does not introduce additional paging delay or power saving consumption. |
| vivo |  |
| Spreadtrum |  |
| CATT | Observation 12: When one PEI indicates multiple POs, no matter behavior-A or behavior-B, the TRS-based PEI could maintain the same performance but the DCI-based PEI detection performance is degraded. Proposal 7: Whether to support multiple POs associated with one PEI need further study, taking into account the following factors: - PEI overheads. - PEI detection performance. - Power saving gains between different POs associated with one PEI. - The benefit of multiple POs associated with one PEI. Proposal 8：One-to-one mapping between PEI and PO is taken as baseline. Table 8: Paging rate for different PO numbers from network perspective Case; The number of POs paged; Calculate formula; Paging rate. Case 1; 0; (1-0.1)^4; 65.61%. Case 2; 1; C41*0.1*(1-0.1)^3; 29.16%. Case 3; 2; C42*0.1^2*(1-0.1)^2; 4.86%. Case 4; 3; C43*0.1^3*(1-0.1); 0.36%. Case 5; 4; C44*0.1^4; 0.01%. Observation 13: For supporting sequence-based PEI associated with multiple POs and sub-grouping, if no more than one sequence is transmitted in a resource at a given time, the false alarm probability is acceptable. |
| CMCC | (A) (B) Figure 1. Illustration of association with PEI and POs In a summary, the association between PEI and PO can be summarised as the following: - One PEI associate with multiple sub-groups of one PO - One PEI associate with one/multiple POs of one/multiple PFs For case 1), one PEI can provide the wake-up information of each sub-groups in one PO, which is much suitable for the low system PO density scenario. For case 2), one PEI can provide the wake-up information of one/multiple POs of one/multiple PFs, which is much suitable for high system PO density scenario. Proposal 6. Support one-to-one and one-to-many mapping between PEI and PO. |
| Qualcomm | Proposal 5: PEI design should be transparent to one PEI per PO and one PEI per N>1 POs design. Potential discussion on one PEI for multiple POs can be postponed after the basic PEI design is complete. Figure 8: One PEI to multiple POs mapping |
| OPPO |  |
| Intel | Observation 8: Spanning sub-groups over multiple PO causes following issues: - Gap between PEI monitoring occasion and PO for a given UE can be large which can increase paging latency and also feasibility/practicality of such a-priori determination by gNB much ahead of a PO is questionable. - It makes the UE sub-grouping process significantly more complex. |
| Apple |  |
| Sony |  |
| Samsung | Observation 2: Flexible UE grouping for paging reception is already supported based on the configuration of a number of PFs/POs per DRX cycle. A PEI associated with one PO with potential UE subgrouping within a PO is sufficient. Proposal 1: Support 1:1 mapping between a PEI and associated PO. - PEI allocated right before PO w/ small time offset, O - PEI w/ large time offset, O, and additional SSBs between PEI and associated PO Figure 1: Illustration of PEI MOs and UE processing timeline in idle/inactive mode Proposal 2: Support configuration of a time offset, O, between start of PEI monitoring occasions and start of an associated PO without need for additional SSBs between PEI and associated PO. Proposal 3: Support N>=1 PEI monitoring occasions per PEI transmission, where each monitoring occasion is associated with a QCL information. |
| MediaTek | Observation 5: From the identified configurations based on evaluation results from more than one company, PDCCH-based PEI achieves the best resource overhead when UE subgrouping is not incorporated. Being able to indicate multiple POs with 1 PEI is the distinguishing factor. Proposal 5: For PDCCH-based PEI, exploiting DCI payload bits for indicating multiple UE (sub)groups is supported. - Note: From UE monitoring perspective, there is only one PEI occasion for a targeted PO. |
| LG |  |
| Panasonic |  |
| Nokia |  |
| Xiaomi |  |
| DOCOMO |  |
| InterDigital |  |
| Lenovo |  |
| Ericsson | - Observation 10 Irrespective of an average assumed group paging rate, during periods of extensive paging load, the PEI transmissions lead to unacceptable blockage of control channel especially in multi-beam deployments where PEI needs to be swept. - Observation 11 A one-to-many mapping scheme between a PEI and multiple POs can provide multiplexing gain and reduce system overhead compared to a one-to-one mapping scheme. - Proposal 7 PEI design supports associating one PEI DCI with multiple POs and/or paging groups. |
| Nordic |  |

From UE point of view, 1-to-1 or 1-to-multiple PEI and PO mapping is a network implementation if UE only needs to decide whether to monitor a PO from one detected PEI. In this regard, the following proposal is suggested to move forward, and PEI-PO mapping design is translated to how to determine PEI monitoring occasion(s) and PEI indication. Note that the concern on the impact to paging preparation timing can also be addressed by setting a proper range between PEI MO(s) and PEI.

FL- [Q2.2-1]

Proposal:

- For a target PO, UE is indicated to monitor the PO from one detected PEI

  - FFS: Determination of PEI monitoring occasion(s) (MO(s)), including the range on the time gap between PEI MO(s) and PO,  and how UE interprets the detected PEI

## Support of Behv-A and/or Behv-B

In Table 3, companies’ views related to support of Behv-A and/or Behv-B are collected:

Table 3: Companies’ views related to support of Behv-A and/or Behv-B

| Company | Company Views |
| --- | --- |
| ZTE, Sanechips | - Observation 1: The advantages and disadvantages of Behv-A and Behv-B is summarized in Table 1. Table 1 Summary of the advantages and disadvantages of Behv-A and Behv-B Advantages; Disadvantages. Behv-A; Resource overhead of PEI is relatively small when paging rate is low.; If UE misses the PEI with wake-up indication, - both network and UEs associated with the same PEI or UE group consume more energy; - Cost more resources to re-transmit the PEI, paging DCI and paging PDSCH; - lead to the information loss and increase the latency of delivery of paging message; - resource overhead of PEI is significantly large when paging rate is high.. Behv-B; - Resource overhead of PEI is relatively small when paging rate is high. - In an extreme case, the resource overhead can be reduced to, for example, 0. - Not impact on the delivery of the paging message in the case of resource collision and PEI miss detection.; Resource overhead of PEI is high when paging rate is low.. - Proposal 1: Behv-B should be supported for PEI design. Table 2 Performance comparison between DCI-based PEI, TRS-like PEI and SSS-like PEI TB scaling =1; TB scaling =0.5. Behv-A; Behv-B; Behv-A; Behv-B. DCI based PEI with SCL decoding; AL=4; Yes; Yes; No; No. AL=8; Yes; Yes; Yes; Yes. DCI based PEI with ML decoding; AL=2; Yes; Yes; No; No. AL=4; Yes; Yes; Yes; Yes. TRS-like PEI with two symbs; FAR=1%; Yes; No; No; No. FAR=0.1%; /; Yes; No; No. SSS-like PEI with two symbs; FAR=1%; Yes; No; No; No. FAR=0.1%; /; Yes; No; No. |
| Huawei, HiSilicon | From the gNB side view, the Behv-A always asks for PEI transmission once a PO/sub-group is paged, if a large number of UEs (e.g. Redcap UEs) camped in the cell, there is a risk that the downlink traffic will be blocked by PEI transmission in addition to paging transmission. While the Behv-B provides the flexibility for the network to balance the power consumption of UE and the congestion rate of other downlink transmission by transmitting or not transmitting the PEI channel/signal. For dense camped UE case, if larger number of UEs are to be paged, the gNB can choose to not transmit the PEI signal/channel to wake up all the UEs associating with that PEI. Although the power saving gain might be a loss, the downlink traffic is guaranteed. Therefore it would better to support both Behv-A and Behv-B, and the gNB can configure which UE behavior is used for current network. Proposal 1: Both Behv-A and Behv-B are supported by Rel-17, which can be configurable by the network. Observation 19: TRS based PEI cannot be used for time/frequency tracking for Behv-B. |
| vivo | Table 3: Comparison of Behavior A and Behavior B PEI design; Resource overhead [4]; Power saving gain (* compared to baseline paging scheme and based on the same resource overhead of Behavior A). 1 to 1 [1]; 4 to 1 [2]; Cases; Gap[3] =3ms; Gap =0. Behavior A; Network needs to transmit PEI in only 10% cases; Network needs to transmit PEI in only 34.4% cases; Low SINR; 24.43%; 27.80%. Medium SINR; 13.93%; 17.84%. High SINR; 1.89%; 8.06%. Behavior B; Network needs to transmit PEI in 90% cases; Network needs to transmit PEI in 99% cases; Low SINR; 1 to 1; -6.97%; -3.60%. 4 to 1; 2.61%; 5.98%. Medium SINR; 1 to 1; -9.72%; -5.81%. 4 to 1; -2.51%; 1.40%. High SINR; 1 to 1; -12.02%; -5.84%. 4 to 1; -7.78%; -1.60%. Notes: [1] Each PO associated with one PEI. [2] Four POs associated with one PEI. [3] The gap between PEI and the nearest SSB. [4] Dynamic rate-matching in PDCCH or PDSCH to co-exist with legacy signals/channels is assumed.. Observation 6: Behavior B will consume 64.6%~80% more PEI resources than Behavior A when assuming the same power saving gain achieved by Behavior A/B. Observation 7: Behavior B will result in more power consumption than Behavior A, and it is even worse compared with baseline paging scheme, when assuming its resource overhead is the same as that of Behavior A. Table 4: The critical probability of sending PEI with no paging indication (i.e. the value of X) for Behavior B to achieve the same total power consumption with baseline paging scheme Different cases; Average power consumption per slot; The value of X, i.e. the probability of sending PEI to indicate go to sleep by network. Baseline i.e., no PEI is configured go to sleep (90%) ¦ wake up (10%); PEI scheme PEI with go to sleep indication (X) ¦ PEI with wake-up indication or without sending PEI (1-X); Paging rate = 10%; Paging rate = 20%; Paging rate = 40%. Gap between PEI and the nearest SSB is 3ms; Low SINR; 3.035; 3.254; 2.19; 3.39; 28%; 26%; 22%. Medium SINR; 2.613; 2.832; 2.19; 2.969; 43%; 40%; 35%. High SINR; 1.663; 1.718; 1.608; 1.898; 79%; 77%; 74%. Gap between PEI and the nearest SSB is 0; Low SINR; 3.035; 3.254; 2.087; 3.287; 19%; 18%; 14%. Medium SINR; 2.613; 2.832; 2.087; 2.866; 30%; 27%; 21%. High SINR; 1.663; 1.718; 1.505; 1.795; 44%; 42%; 38%. Observation 8: For Behavior B, the network shall guarantee the probability of sending PEI with go to sleep indication is larger than 79%, otherwise there is no power saving benefit to configure PEI for idle/inactive UE. Proposal 5: Behavior A should be adopted for UE behavior of PEI detection. |
| Spreadtrum | Observation 8: Behv-B is more robust for resource sharing or collision handling between PEI and other signals/channels. Proposal 1: Behv-B is prioritized. |
| CATT | Observation 1: For behavior-B, UE expects to receive PEI at every PO to determine whether to wake up or go back to sleep. If PEI is not detected by UE, UE falls back to proceed with the legacy paging procedure. Proposal 1: For the evaluation and comparison of PEI candidate designs based on PDCCH, TRS/CSI-RS and SSS, Behv-A is supported. - Behv-A: - PEI indicates UE should monitor a PO if UE’s group/subgroup is paged - UE is not required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO |
| CMCC | Proposal 3. Behv-A is preferred which UE is not required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO. |
| Qualcomm | Proposal 2: Regarding the following Behv-A and Behv-B in RAN1 #104-e agreements, it should be clarified that the UE is allowed to choose a subset of PEI occasions to detect the PEI based on the wording in red color - Behv-A: - PEI indicates UE should monitor a PO if UE’s group/subgroup is paged - UE is not required to monitor a PO if UE does not detect PEI at all PEI occasion(s) chosen by the UE for PEI detection for the PO - Behv-B: - PEI indicates whether or not UE should monitor a PO - UE is required to monitor a PO if UE does not detect PEI at all PEI occasion(s) chosen by the UE for PEI detection for the PO - Note: It is up to UE implementation which PEI occasion(s) is chosen by the UE for PEI detection. Figure 1: PEI collision with legacy channel Observation 2: Regarding Behv-A and Behv-B for PEI - When group paging rate for each PO is below 50% (e.g., 10%), Behv-A allows network to less often transmit PEI (10% of the time), and Behv-B requires network to more often transmit the PEI (90% of the time) - In a heavily loaded network, if network has to often drop PEI transmissions due to the lack of resources, network should better just disable PEI - NB-IoT has assumed Behv-A type of wake-up signal design - In Rel-16 connected mode WUS design, network can configure which behavior the UE should assume. This unnecessarily increases the UE implementation complexity. Proposal 3: Only support Behv-A for Rel-17 PEI design from the two UE behaviors identified in RAN1# 104-e. |
| OPPO | gNB; UE; delay. Normal status (10% group paging rate per PO); Behv-A has the lower power consumption than Behv-B; The same power consumption for Behv-A and Behv-B; -. Miss detection happens; Behv-A has extra power consumption; Behv-B has extra power consumption; Behv-A has extra delay. False detection happens (which is with little probability); Behv-B has extra power consumption; Behv-A has extra power consumption; Behv-B has extra delay. The transmission of PEI is mainly for UE power saving purpose, which is at the cost of network resource overhead and energy efficiency. With considering resource overhead and the flexibility of network, it will be reasonable to allow PEI discontinuous transmission (DTX) for the case when there is no UE will be paged in a PO. Thus, Behv-A is essential and more suitable. Proposal 6: Behv-A should be considered in PEI designs. |
| Intel | Observation 3: For 1 PEI to 1 PO association, Behv-B could cause more signalling overhead for PEI transmission and/or increased UE power consumption compared to Behv-A. This makes benefits of Behv-B over Behv-A questionable. Proposal 2: Support Behv-A only as PEI functionality. - Behv-A: - PEI indicates UE should monitor a PO if UE’s group/subgroup is paged - UE is not required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO |
| Apple | As the calculation showed above, the overhead is rather small, so it should not have much impact to require the gNB to always transmit positive PEI. Proposal 2: Adopt Behv-A, i.e., - PEI indicates UE should monitor a PO if UE’s group/subgroup is paged. - UE is not required to monitor a PO if UE does not detect PEI at all PEI occasion(s) for the PO. |
| Sony | Observation 6 – Resource overhead in Behv-A is lower than the one in Behv-B since the PEI does not need to always be transmitted. Proposal 4 – Use an SSS-based PEI based on Behv-A, i.e., use a PEI where the indicator is only transmitted when there is a paging available for the target UE or sub-group of UEs. |
| Samsung | Observation 13: Behav-B requires much large (9x larger w/ group paging rate =10%) resource overhead than Behav-A. Proposal 10: Support only Behav-A for PEI reception for the benefit of low resource overhead. |
| MediaTek | Proposal 3: For PDCCH-based PEI, Behv-A or Behv-B is configurable by gNB for prioritization on PEI or legacy PEI, accordingly. |
| LG |  |
| Panasonic | Observation 1: If UE does not detect any PEI, the supported UE behavior can largely impact the system overhead caused by PEI transmission under different GPR assumptions. Proposal 1: Behv-A should be supported. Proposal 2: When UE is certain about the SIB configuration of PEI, if UE does not detect PEI, UE is not required to continue monitoring paging PDCCH in the PO after PEI. Proposal 3: When UE is not certain about the SIB configuration of PEI, e.g. during SI modification period or before obtaining SIB configuration related to PEI and/or TRS/CSI-RS for time/frequency tracking availability status, UE should continue monitoring paging PDCCH in the PO after PEI. |
| Nokia | Observation: Behv-A should be assumed as a baseline operation for EPI. Proposal: Enable support of both Behv-A and Behv-B based on network configuration. Details would be for RAN2. |
| Xiaomi |  |
| DOCOMO | Thus, it is beneficial that NW can choose the default behavior between Behv-A and B, and Behv-A/B should be configurable, e.g., via SIB and/or Paging DCI. Proposal 2: Behavior A or Behavior B should be configurable. |
| InterDigital |  |
| Lenovo |  |
| Ericsson | - Observation 10 Irrespective of an average assumed group paging rate, during periods of extensive paging load, the PEI transmissions lead to unacceptable blockage of control channel especially in multi-beam deployments where PEI needs to be swept. - Proposal 8 PEI design supports higher-layer configuration of UE behavior wrt PEI detection/absence of PEI, i.e. whether UE follows Behv-A or Behv-B. |
| Nordic |  |

For the companies’ inputs, we can observe the following statistics. Comparing support of “Behv-A only” and “Behv-A and Behv-B”, the supporting levels are similar (9 companies vs. 8 companies).

|  | Support PDCCH-based PEI | Support Seq-based PEI |
| --- | --- | --- |
| Support Behv-A only (9) | CMCC, OPPO, Apple | Vivo, CATT, QC, Intel, Sony, SS, |
| Support Behv-A and Behv-B (8) | ZTE, HW, Spreadtrum, MTK, Nokia, DoCoMo, Ericsson | Panasonic |

To make progress, we suggest the following compromised version, i.e., Behv-A is supported and, conditioned on that PDCCH-based PEI is selected for Rel-17 paging enhancement, Behv-B can be configured by network. We hope this can gain maximum supports from both sides:

FL- [Q2.3-1]

Proposal:

- A PEI monitoring occasion (MO) is defined as the REs in a slot where UE monitors one PEI for a target PO.

- UE is not required to monitor the target PO if UE does not detect any PEI from all associated PEI MO(s) for the target PO.

- If PDCCH-based PEI is selected for Rel-17 paging enhancement, network can configure that UE is required to monitor the target PO if UE does not detect any PEI from all associated PEI MO(s) for the target PO.

## Further PEI Characteristics/Comparisons without UE Sub-Grouping

In Table 4, companies’ inputs on further PEI characteristics/comparisons without UE sub-grouping are collected:

Table 4: Companies’ inputs on further PEI characteristics/comparisons without UE sub-grouping

| Company | Company Views |
| --- | --- |
| ZTE, Sanechips | - Observation 2: For Behv-A, a two-symbol TRS-like PEI or SSS-like PEI with FAR=1% fulfills the performance required by the paging PDSCH without scaling. - Observation 3: For Behv-B, a two-symbol TRS-like PEI or SSS-like PEI with FAR=0.1% fulfills the performance required by the paging PDSCH without scaling. - Observation 4: For both Behv-A and Behv-B, sequence-based PEI (both TRS-like PEI and SSS-like PEI) with more than two symbols are needed to fulfill the JMDR performance requirement when the paging PDSCH is configured with TB scaling 0.5. (a) Behv-A (b) Behv-B Figure 1 JMDR performance of DCI-based PEI (SCL decoding) and paging PDCCH - Observation 5: For Behv-A/B and SCL decoding, DCI-based PEI with AL 4 fulfills the performance required by paging PDSCH without TB scaling; DCI-based PEI with AL 8 fulfills the performance required by paging PDSCH with TB scaling=0.5. (a) Behv-A (b) Behv-B Figure 2 JMDR performance of DCI based-PEI (ML decoding) and paging PDCCH - Observation 7: With ML decoding, DCI-based PEI with AL 2 fulfills the performance requirement of paging PDSCH with TB scaling 1 and DCI-based PEI with AL 4 fulfills the performance requirement of paging PDSCH with TB scaling 0.5 in the case of Behv-A and Behv-B. Figure 3 Performance of DCI-based PEI (no more than 12 information bits) and two-symbol TRS-like PEI - Observation 8: When the information bits of the DCI based PEI are reduced, the MDR performance of DCI based PEI is further improved. The MDR performance of DCI-based PEI with AL 2 carrying no more than 6 information bits is better than that of the two-symbol TRS-like PEI. Table 2 Performance comparison between DCI-based PEI, TRS-like PEI and SSS-like PEI TB scaling =1; TB scaling =0.5. Behv-A; Behv-B; Behv-A; Behv-B. DCI based PEI with SCL decoding; AL=4; Yes; Yes; No; No. AL=8; Yes; Yes; Yes; Yes. DCI based PEI with ML decoding; AL=2; Yes; Yes; No; No. AL=4; Yes; Yes; Yes; Yes. TRS-like PEI with two symbs; FAR=1%; Yes; No; No; No. FAR=0.1%; /; Yes; No; No. SSS-like PEI with two symbs; FAR=1%; Yes; No; No; No. FAR=0.1%; /; Yes; No; No. To sum up, DCI-based PEI can fulfill performance requirements by flexibly adjusting the aggregation levels, while sequence-based PEI requires more standardization efforts to improve the performance. Figure 4 UE processing timeline - Observation 15: Based on the simulation results in Figure 5, it can be seen that - the power saving gains obtained by the PEI located in location 1 and location 2 are basically the same; - the PEI located near the first SSB provides the maximum power saving gain; - for PEI without sub-grouping, the power saving gain brought by PEI only will obviously decrease when group paging rate increases. Figure 5 Power saving gain provided by PEI - Observation 23: The power saving gain of DCI-based PEI and sequence-based PEI is almost the same even the capacity limitation of sequence-based PEI is not considered and sequence-based PEI with synchronization function is assumed. Figure 9 Power saving gain from DCI-based PEI and sequence-based PEI - Observation 24: Whether the sequence-based PEI can provide the synchronization function and replace SSB or not is questionable considering the following factors. - If gNB does not send the sequence-based PEI but FAR occurs, UE would take noise as PEI for synchronization; - The modules for PEI processing and paging DCI/PDSCH can be different in the implementation. The corrected synchronization accuracy may not be sufficient for paging DCI and PDSCH decoding. Observation 25: If the on-demand (instead of always-on) sequence-based PEI is used for synchronization without justification, it will impact the correction of synchronization and paging performance, and increase more UE energy. - Observation 27: DCI based-PEI occupies less resources for both Behv-A and Behv-B. Table 11 Power saving gain and resource overhead of PEI without sub-grouping PEI without sub-grouping; Resource overhead per PO - Behv-A; Resource overhead per PO - Behv-B; Power saving gain. Paging rate; Paging rate; Paging rate. 10%; 40%; 60%; 10%; 40%; 60%; 10%; 40%; 60%. DCI-based PEI (associated with 12 POs, AL=4) Dynamic resource sharing; 17.2; 24; 24; 24; 24; 24; 22.5%; 14.4%; 9.3%. DCI-based PEI (associated with 4 POs, AL=4) Dynamic resource sharing; 24.8; 62.7; 70.2; 72; 70.2; 62.7. TRS-like PEI (associated with 1 PO) Dynamic or semi-static resource sharing; 28.8 or 288; 115.2 or 288; 172.8 or 288; 259.2 or 288; 172.8 or 288; 115.2 or 288. SSS-like PEI (associated with 1 PO) Dynamic or semi-static resource sharing; 26.4 or 264; 105.6 or 264; 158.4 or 264; 237.6 or 264; 158.4 or 264; 105.6 or 264. |
| Huawei, HiSilicon | Observation 18: The false alarmed TRS-based PEI would introduce more time and frequency drift before the next paging cycle if TRS-based PEI is used for time and frequency tracking. |
| vivo |  |
| Spreadtrum | Observation 1: Without UE subgrouping, the power saving gain of DCI-based PEI and sequence-based PEI are similar. Observation 2: With UE subgrouping, the power saving gain of DCI-based PEI and sequence-based PEI are similar. Observation 3: In NR, RRM measurement for serving cell cannot be relaxed, so the power saving gain of sequence-based PEI may not be higher than that of DCI-based PEI. For DCI-based PEI, the coherent receiver is realized in frequency domain. The coherent receiver will perform channel estimation, equalization, QPSK demodulation and decoding. The receiver is robust for the residual frequency error (possibly plus the residual timing error) with protection of equalization, QPSK demodulation and decoding. For SSS-based PEI, the non-coherent receiver is usually realized in time domain. The non-coherent receiver can try multiple hypotheses for the residual frequency error (possibly plus the residual timing error) to improve performance. In our view, the overall complexity of the non-coherent receiver for SSS-based PEI is similar to that of the coherent receiver for DCI-based PEI. For TRS/CSI-RS based PEI, the non-coherent receiver may be realized in frequency domain. The residual frequency error (possibly plus the residual timing error) is hard to be overcome in frequency domain if equalization is not applied. Hence, the non-coherent receiver for TRS/CSI-RS based PEI has higher complexity. Observation 9: The non-coherent receiver for TRS/CSI-RS based PEI has higher complexity. |
| CATT | Figure 2: Illustration of paging reception with TRS/CSI-RS assisted PDCCH-based paging indication Figure 3: Illustration of paging reception with sequence-based PEI and channel tracking Table 4: Comparison of PEI candidate designs without assuming the support of multiple POs and sub-grouping PEI candidate designs; Detection performance(the SNR at JMDR=1% without paging performance influence); Power saving gain; Coexistence; Resource overhead. DCI-based PEI(12 bits); -7.5dB(AL=4 for Alt 1); 4.41%~10.07%; PDSCH, PDCCH; 288 REs(semi-static) 28.8 - 288 REs(dynamic). DCI-based PEI(41 bits); -7.9dB(AL=8 for Alt 1); 4.41%~10.07%; PDSCH, PDCCH; 576 REs(semi-static) 57.6 - 576 REs(dynamic). TRS-based PEI; -8.3dB(Alt 1); 23.11%~51.59%; PDSCH, TRS/CSI-RS; 0 or 288 REs(semi-static) 0 or 28.8 REs (dynamic). Proposal 3: The sequence-based PEI should be adopted in Rel-17 for UE in IDLE/Inactive mode for UE power saving. |
| CMCC |  |
| Qualcomm | Observation 3: PEI can provide essential power saving gain for idle and inactive mode UEs - Optimal location of PEI transmission can be different for different SINR conditions - Optimal location of PEI transmission can be different for PDCCH-based and RS/sequence-based PEI due to potential different requirement of time and frequency synchronization and AGC accuracy. PDCCH-based PEI may have lower power saving gain than RS/sequence-based PEI - When UE sub-grouping is adopted, it provides about 2% additional power saving gain. - Table 3: Power and power saving gain vs. number of UE subgroups at different locations SINR condition; Low; Medium; High with 4ms PO; High with 1ms PO. Baseline power; 3.238; 2.816; 1.875; 1.743. Early paging indication without UE sub-grouping power (gain); Location 1: 2.66 (17.8%) Location 2: 2.728 (15.7%) Location 3: 3.053 (5.7%) Location 4: 3.095 (4.4%) Location 5: 3.095 (4.4%) Location 6: 3.095 (4.4%); Location 1: 2.655 (5.7%) Location 2: 2.651 (5.8%) Location 3: 2.639 (6.3%) Location 4: 2.673 (5%) Location 5: 2.673 (5%) Location 6: 2.673 (5%); Location 1: 1.93 (-2.9%) Location 2: 1.93 (-2.9%) Location 3: 1.954 (-4.2%) Location 4: 1.917 (-2.2%) Location 5: 1.579 (15.7%) Location 6: 1.578 (15.8%); Location 1: 1.90 (-9.1%) Location 2: 1.90 (-9.1%) Location 3: 1.93 (-10.5%) Location 4: 1.89 (-8.3%) Location 5: 1.55 (11.04%) Location 6: 1.55 (11.09%). Early paging indication with UE sub-grouping with 8 sub-groups (gain); Location 1: 2.583 (20.2%) Location 2: 2.658 (17.9%) Location 3: 3.015 (6.9%) Location 4: 3.061 (5.5%) Location 5: 3.061 (5.5%) Location 6: 3.061 (5.5%); Location 1: 2.582 (8.3%) Location 2: 2.582 (8.3%) Location 3: 2.601 (7.7%) Location 4: 2.639 (6.3%) Location 5: 2.639 (6.3%) Location 6: 2.639 (6.3%); Location 1: 1.881 (-0.3%) Location 2: 1.881 (-0.3%) Location 3: 1.904 (-1.6%) Location 4: 1.867 (0.4%) Location 5: 1.529 (18.4%) Location 6: 1.529 (18.4%); Location 1: 1.877 (-7.7%) Location 2: 1.877 (-7.7%) Location 3: 1.901 (-9.0%) Location 4: 1.864 (-6.9%) Location 5: 1.526 (12.5%) Location 6: 1.526 (12.5%). |
| OPPO |  |
| Intel |  |
| Apple | For low SINR case, the baseline PEI case (i.e., PEI is not used for time/frequency tracking, and the UE still needs to monitor 3 SSBs for PO reception) provides 26.3% UE power saving gain. If we assume PEI can replace one SSB for time/frequency tracking (i.e., the UE only needs to monitor two SSBs plus PEI for PO reception), the UE power saving gain is 27.9%. The difference is only 1.6%, which is small. Also note that the sequence detection is not 100% reliable, and false alarm event may cause the tracking loop at the UE to be adjusted based on wrong signal, which may take some time to recover. Putting these two aspects together, even though using sequence-based PEI may provide additional power saving compared to PDCCH-based PEI, the difference appears to be small. So this should not be considered as the main reason to favor sequence-based solution. |
| Sony | - In a DCI-based design, the UE needs to be fully synchronized to the network to be able to decode the DCI-based PEI. This means that the UE needs to decode SSB prior to its PEI decoding/detection. Even if the DCI-based PEI is placed as close as possible to an SSB signal, the UE still needs to receive and decode both SSB and DCI-based PEI before it decides whether or not there is paging. This means a DCI-based PEI design does not fulfil the condition of proposal 1 as it needs to receive synchronization in advance for its detection, leading to extra unnecessary power consumption. - A TRS/CSI-RS based design requires a longer time and wider bandwidth than the other two designs to accommodate the signal content/information. This means longer time and higher received power consumption and consequently relatively higher power consumption for its detection, compared to the other candidates. - By using a sequence-based PEI design, having similar characteristics as SSS, the UE does not need to do any synchronization prior to its PEI detection. The UE proceeds to read SSB and PO only if the SSS-based PEI is detected. This basically means that the SSS-based PEI scheme fulfils the condition in proposal one, resulting in higher power saving gain compared to a DCI-based solution. Observation 3 – Sequence-based (e.g. SSS-based) early paging indication fulfils the condition that paging enhancement schemes should avoid the UE unnecessarily transitioning from/to deep sleep and from/to synchronization states, when there is no paging for target UE. Observation 8 – The use of sequence-based early paging indicator is also beneficial for the FR2 case since the UE does not need to monitor a wider bandwidth for its paging detection. Proposal 7 - Use sequence-based early paging indicator with sub-grouping for paging enhancement for FR2 operation. |
| Samsung | - PEI allocated right before PO w/ small time offset, O - PEI w/ large time offset, O, and additional SSBs between PEI and associated PO Figure 1: Illustration of PEI MOs and UE processing timeline in idle/inactive mode First of all, we evaluate average power consumption for the following four cases. - Case #0: baseline without PEI - Case #1: PDCCH-based PEI with monitoring occasions after last SSBs burst used for synchronization, where time offset between PEI and PO is small (8ms) as illustrated in Figure 1(a). - Case #2: PDCCH-based PEI with monitoring occasions after first SSBs burst used for synchronization, where time offset between PEI and PO is large (8ms, 28ms, 48ms for N_SSBs = 1, 2, 3, respectively) as illustrated in Figure 1(b). - Case #3: RS-based PEI with monitoring occasions after last SSBs burst used for synchronization, where time offset between PEI and PO is small (8ms) as illustrated in Figure 1(a). In this case, UE use SSBs for intra-frequency or inter-frequency RRM measurement only, while the RS-based PEI can be used for synchronization or loop convergence before PO. The processing timeline is illustrated in Figure 1 in Section 2.2.1. For PDCCH-based PEI, we assume the duration of PEI MOs are same as PO, which is 4ms. For RS-based solution, the duration of PEI is assumed to be same as SSBs, which is 2ms. In practice, UE may monitor only partial configured MOs for paging PDCCH and PEI. So, we evaluate the power saving gain for two scenarios. One scenario is that UE monitors all configured MOs, while the other scenario UE only monitor one MO out from the multiple configured MOs for both paging PDCCH and PEI. The power saving gain for different L1 signal/channel design of PEI are computed relative to the baseline in Table 1. We consider different channel conditions, where a number of NSSBs =1, 2, and 3 SSB bursts are used for synchronization corresponds to high, medium, and low SINR, respectively. Table 1: Power saving gain for different L1 signal/channel design for PEI with group paging rate of 10% PEI configurations; Power saving gain. UE monitors all configured MOs for paging PDCCH/PEI; UE monitors one MO for paging PDCCH/PEI. High SINR (N_SSBs = 1); Case #1; 8.95%; 7.94%. Case #2; 8.95%; 7.94%. Case #3; 15.40%; 10.91%. Medium SINR (N_SSBs = 2); Case #1; 2.16%; 1.29%. Case #2; 15.64%; 15.52%. Case #3; 20.49%; 18.71%. Low SINR (N_SSBs = 3); Case #1; 1.88%; 1.11%. Case #2; 25.33%; 25.54%. Case #3; 30.84%; 29.64%. Observation 4: PDCCH based PEI achieves 1.11% to 25.54% power saving gain, which is very sensitive to the monitoring occasions of PEI. The high power saving gain comes at low SINR based on very large time gap between PEI and PO. Observation 5: RS-based PEI achieves 10.91% to 30.84% power saving gain, and achieves about 3% to 5% more power saving gain than PDCCH-based PEI regardless of PEI monitoring occasions and SINR. Proposal 4: Support RS-based PEI for better power saving performance. |
| MediaTek | Observation 7: A sequence-based PEI provides similar synchronization resource amount as a SSB, and one can assume up to one less SSB (burst) processing between PEI and PO when UE is indicated to monitor PO Observation 8: Figure 1 and Figure 2 illustrate UE processing timelines with PDCCH-based PEI and TRS/SSS-based PEI, respectively. Figure 1: UE processing timeline with PDCCH-based PEI Figure 2: UE processing timeline with TRS-based PEI or SSS-based PEI Observation 11: When UE subgrouping enhancement is not applied and PO paging rate is low, assumed 10%, comparison of PEI candidate designs shows (with details in Table 6) - With PDCCH-based PEI: - Power saving gain of 14.8% - 25.3% where the baseline assumes 1 - 3 SS bursts for synchronization before PO reception - Average resource overhead per PO of 20.5 REs – 41.0 REs where 1 PEI can indicate up to 8 POs - With TRS/CSI-RS-based PEI: - Power saving gain of 14.8% - 26.6% where the baseline assumes 1 - 3 SS bursts for synchronization before PO reception - Average resource overhead per PO of 30 REs – 300 REs where 1 PEI indicates 1 PO - With SSS-based PEI: - Power saving gain of 14.8% - 26.6% where the baseline assumes 1 - 3 SS bursts for synchronization before PO reception - Average resource overhead per PO of 57.6 REs – 264 REs where 1 PEI indicates 1 PO Table 6: Comparison of PEI candidate designs with 10% PO paging rate and without UE subgrouping enhancement PEI indication; Average resource overhead (#REs) per PO; Power saving gain. #SSB before PO; Gain (%). PDCCH-based PEI - AL4 ∼AL8 with 12-bit payload and 1 bit per PO - Share CORESET 0 with legacy UEs; Up to 8 POs; 20.5 ∼ 41.0; 1; 14.8%. 2; 19.5%. 3; 25.3%. TRS/CSI-RS-based PEI - 50-RB-and-2-symbol format with 1 PEI for 1 PO - Semi-static resource sharing: One sp ZP-CSI-RS rate-matching resource set of 1 TRS pattern - (Dynamic resource sharing: One aperiodic ZP-CSI-RS rate-matching pattern set of 1 TRS patterns) †; 1 PO; 300 ∼ (30); 1; 14.8%. 2; 20.7%. 3; 26.6%. SSS-based PEI - 11-RB-and-2-symbol format with 1 PEI for 1 PO - Semi-static resource sharing: RB-symbol rate-matching pattern with 11 RBs and 2 symbols - (Dynamic resource sharing: CORESET 0 of 576 REs) †; 1 PO; 264 ∼ (57.6); 1; 14.8%. 2; 20.7%. 3; 26.6%. †: Whether dynamic resource sharing is available is not guaranteed (network dependent). |
| LG |  |
| Panasonic |  |
| Nokia | Figure 1 Processing timeline for DCI-based EPI in low SINR. UE is paged. Figure 2 Processing timeline for TRS-based EPI when an SSB is received prior to the EPI in low SINR. UE is paged. Figure 3 Processing timeline for SSS-based EPI in low SINR. UE is paged. Observation: DCI-based EPI consistently provides the highest energy saving potential for low/medium/high SINR UEs as compared to SSS- and TRS-based EPI. |
| Xiaomi | Observation 1: DCI-based PEI and sequence-based PEI has similar power saving gain when the two forms of PEI have similar relative locations to the target PO. Observation 2: Compared to sequence-based PEI, DCI-based PEI can carry more information such as subgrouping indication/ TRS/CSI-RS for idle UE indication/ paging type. Observation 3: DCI-based PEI can reuse legacy DCI format and PDCCH search space/CORESET configuration while sequence-based PEI needs extra specification design. Observation 4: DCI-based PEI can adjust its coverage flexibly by shifting AL while sequence-based PEI can hardly do. |
| DOCOMO |  |
| InterDigital | At MDR of 10-2, there is no meaningful difference between TRS and 2-symbol SSS and these two sequence-based schemes are about 1 dB better than PDCCH AL8. The performance of 1-symbol SSS and PDCCH AL4 are worse by 2-to-3 dB and cannot meet the joint MDR requirements. Figure 2-1 MDR comparison of PDCCH, SSS, TRS for PEI Table 1 Overehead comparison Number of simultaneously paged UE subgroups; Overhead (REs). PDCCH AL8; PDCCH AL16; TRS; 2-symbol SSS. 1; 576; 1152; 288; 264. 2; 576; 1152; 576; 528. 3; 576; 1152; 864; 792. 4; 576; 1152; 1152; 1054. It is concluded that for two subgroups, the overhead of PDCCH AL8, TRS and 2-sybmol SSS are very close to each other. However, it should be remembered that the MDR of PDCCH AL8 is about 1 dB worse than the sequence-based methods. PDCCH AL16 can provide better MDR but doubles the overhead. Figure 2-5 Probability of paging n subgroups |
| Lenovo |  |
| Ericsson |  |
| Nordic |  |

Since power saving observation is not captured in last meeting, we suggest to compile a power saving observation for the case without UE sub-grouping.

To compare companies’ results based on a common ground, we assume 1 SS burst is required before PEI detection in UE processing timeline. The reason is because companies already checked PEI performances based on a common large residue CFO, e.g., with [-0.5 0.5] ppm, before PEI detection, and assuming a different requirement on SS burst number before PEI detection looks inconsistent. With this rule in mind, Table 5 collects the power saving gain results. As there are more than two companies’ results only in the case of PO paging rate of 10%, the following observation is therefore proposed (analogous to previous RAN1 observation sent to RAN2):

FL-[Q2.4-1]

Observation:

For NR idle/inactive-mode UEs with PO paging rate of 10%, there can achieve the following power saving gains w.r.t Rel-16 by utilizing

- PDCCH-based PEI:

  - 8.95% –14.8% where the baseline assumes 1 SS burst for synchronization before PO reception

  - 19.5% –22.5% where the baseline assumes 2 SS bursts for synchronization before PO reception

  - 25.3% –37.0% where the baseline assumes 3 SS bursts for synchronization before PO reception

- TRS/CSI-RS-based PEI:

  - 14.8% – 15.4% where the baseline assumes 1 SS burst for synchronization before PO reception

  - 20.7% – 22.5% where the baseline assumes 2 SS bursts for synchronization before PO reception

  - 26.6% – 30.2% where the baseline assumes 3 SS bursts for synchronization before PO reception

- SSS-based PEI:

  - 14.8% – 15.8% where the baseline assumes 1 SS burst for synchronization before PO reception

  - 5.00% –20.7% where the baseline assumes 2 SS bursts for synchronization before PO reception

  - 15.7% – 27.9% where the baseline assumes 3 SS bursts for synchronization before PO reception

- Note: It is assumed one SS burst can be received for time-frequency synchronization before detecting PEI

Table 5: Companies’ results of UE power saving gain without UE sub-grouping and with up to 1 SS burst before PEI detection

| Sync setting | 3 SS bursts before PO |  |  | 2 SS bursts before PO |  |  | 1 SS burst before PO |  |  | Note |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PO paging rate | 10% | 20-35% | 40-60% | 10% | 20-35% | 40-60% | 10% | 20-35% | 40-60% |  |
| PDCCH-based PEI |  |  |  |  |  |  |  |  |  |  |
| ZTE |  |  |  | 22.50% |  | 9.30% -14.40% |  |  |  |  |
| Samsung |  |  |  |  |  |  | 8.95% | 7.48% |  |  |
| DOCOMO | 29.60% |  | 12.50% |  |  |  |  |  |  |  |
| Apple | 26.3% |  |  |  |  |  |  |  |  |  |
| E/// | 33.00%-37.00% |  |  |  |  |  |  |  |  |  |
| MTK | 25.3% |  |  | 19.5% |  |  | 14.8% |  |  |  |
| TRS/CSI-RS-based PEI |  |  |  |  |  |  |  |  |  |  |
| ZTE |  |  |  | 22.50% |  | 9.30%-14.40% |  |  |  |  |
| CATT | 30.19% |  |  |  |  |  |  |  |  | Paging cycle: 128rf |
| Samsung |  |  |  |  |  |  | 15.40% | 13.53% |  |  |
| Apple | 27.9% |  |  |  |  |  |  |  |  |  |
| MTK | 26.6% |  |  | 20.7% |  |  | 14.8% |  |  |  |
| SSS-based PEI |  |  |  |  |  |  |  |  |  |  |
| ZTE |  |  |  | 22.50% |  | 9.30%-14.40% |  |  |  |  |
| QC | 15.7%-17.8% |  |  | 5 %-6.30% |  |  | 15.7%-15.8% |  |  | Location (1, 2)/(3, 4)/(5, 6) for #SS bursts before PO = 3/2/1 |
| Apple | 27.9% |  |  |  |  |  |  |  |  |  |
| MTK | 26.6% |  |  | 20.7% |  |  | 14.8% |  |  |  |

# Physical-Layer Designs and Comparison with UE Subgrouping

In this Section, physical-layer designs and comparison with UE sub-grouping will be discussed and decided.

## Subgroups Indication Design

In Table 6, companies’ views related to subgroups indication design are collected:

Table 6: Companies’ views related to subgroups indication design

| Company | Company Views |
| --- | --- |
| ZTE, Sanechips | - Observation 16: When discussing the paging enhancement scheme, the larger paging rate (e.g., 20%-60%) should also be considered. Figure 6 Power saving gain provided by PEI with sub-grouping - Observation 17: PEI with sub-grouping can provide higher power saving gain. When the number of sub-groups exceeds a certain range, the additional power saving gain obtained by increasing the number of sub-groups becomes smaller. 8 sub-groups can achieve the best tradeoff between the signaling overhead of PEI and power saving gain. - Proposal 3: Supporting the agreements of RAN2 LS, at least 8 sub-groups per PO should be considered. - Observation 18: If the sub-grouping information is carried on the paging PDCCH, the power saving gain is negligible. - Proposal 4: The combination of PEI and sub-grouping indication should be supported. - Observation 21: For sequence based PEI associated with multiple POs, - if PEI is generated by different sequences, the performance of sequence-based PEI will be deteriorated with the increased blind detection； - if a group common PEI is used, the power saving gain of the UE will decrease due to the unnecessary wake-up, especially when the group paging rate is large. - if TDM or FDM technique is used for sequence-based PEI, the resource overhead will be multiplied. - Observation 22: For sequence-based PEI associated with multiple POs or sub-groups, Alt 3 (TDM or FDM technique) are considered. Table 4 UE power saving gain for PEI associated with different number of POs Group paging rate; PEI associated with one PO; PEI associated with four POs. DCI/sequence-based PEI; DCI-based PEI; Sequence-based PEI (w/ group common PEI). RG = 10%; 22.5%; 22.5%; 21.6%. RG = 40%; 14.4%; 14.4%; 9.1%. RG = 60%; 9.3%; 9.3%; 2.9%. Figure 8 MDR performance when multiple PEI sequences occupy the same resource |
| Huawei, HiSilicon | Observation 9: PDCCH-based PEI can support to indicate at least 8 sub-groups per PO, and the payload can be used for sub-group indication and associated with multiple POs flexibly. Observation 9: 8 FDMed or TDMed PEI resources for SSS based PEI or TRS based PEI are needed to support 8 sub-groups indication in PEI, according to the observations in RAN1#104bis. Observation 18: The false alarmed TRS-based PEI would introduce more time and frequency drift before the next paging cycle if TRS-based PEI is used for time and frequency tracking. Table 1 PEI configuration PDCCH-based; TRS-based; SSS-based. Single PEI configuration; AL4 12bit payload or AL8 or 32bits payload; 1-slot 48-RB TRS, occupying 288 REs (48 RB x 3 REs per RB x 2 symbols); 2-symbol SSS, occupying 264 REs (11 RB x 2 symbols). Case; Case1; Case1; Case2; Case1; Case2. PEI indication; 8 sub-groups; 8 sub-groups; PO-wise; 8 sub-groups; No need. Paging DCI indication; No need; No need; 8 sub-groups; No need; 8 sub-groups. Co-existence method; Dynamic sharing PDCCH resource; Dynamic sharing PDSCH resource by CORESET0 with multiple MOs ( 48RB*2symbol*2); Dynamic sharing PDSCH using one ZP-CSI-RS resource.; Dynamic sharing PDSCH resource by CORESET0 with multiple MOs (48RB*2symbol*2); Dynamic sharing PDSCH resource using CORESET0 (24RB*2symbol). |
| vivo | Table 1: Grouping method adopted in Rel-16 NB-IoT for sub-grouping indication carried by sequence-based PEI-only. Characteristics for different cases; Sequence index; Probability. >= 2 sub-groups need to be woke up; 0; 0.0046. Only to wake up sub-group 1; 1; 0.0119. Only to wake up sub-group 2; 2; 0.0119. Only to wake up sub-group 3; 3; 0.0119. Only to wake up sub-group 4; 4; 0.0119. Only to wake up sub-group 5; 5; 0.0119. Only to wake up sub-group 6; 6; 0.0119. Only to wake up sub-group 7; 7; 0.0119. Only to wake up sub-group 8; 8; 0.0119. All sub-groups go to sleep; 9; 0.9 (1 - per PO paging rate). Observation 1: Sub-grouping method introduced in Rel-16 NB-IoT can save the number of candidate sequences by defining a “common sequence” representing the case that no less than two sub-groups need to be paged. Observation 2: The additional false alarm rate and power consumption caused by the common sequence is marginal. Observation 3: Based on the sub-grouping method introduced in Rel-16 NB-IoT, the network only needs to transmit one certain sequence for each PEI occasion to indicate its associated PO(s). Proposal 1: Adopt the sequence-based grouping method introduced in Rel-16 NB-IoT as described in Table 1. With this method, UE only needs to detect the following two sequences of PEI per PEI occasion, if sub-groups are configured. - The sub-group specific sequence, to indicate only the sub-group which the UE belongs to receive paging, and - The common sequence, to indicate no less than two sub-groups to receive paging. Observation 10: Performance of sequence-based PEI will not degrade with the increasing of number of subgroups, if a common sequence is introduced to wake up no less than two sub-groups. Figure 1: The trend of additional power saving gain with the increasement of the number of sub-groups. Table 2: Additional power saving gain by introducing sub-grouping paging indication carried by PEI compared with the scheme of PEI without sub-grouping. Additional power saving gain; The number of sub-groups; For Low SINR case with 3 SSBs before PO; For High SINR case w/o RRM relax. Per PO paging rate=10%; 2; 2.80%; 0.90%. 4; 4.26%; 1.37%. 8; 5.01%; 1.62%. 16; 5.35%; 1.73%. Per PO paging rate=20%; 2; 5.11%; 1.71%. 4; 7.94%; 2.66%. 8; 9.36%; 3.14%. 16; 10.12%; 3.39%. Per PO paging rate=40%; 2; 8.59%; 3.08%. 4; 13.75%; 4.93%. 8; 16.59%; 5.95%. 16; 18.09%; 6.48%. Observation 4: Up to 5.35% or 10.12% additional power saving gain can be achieved by introducing the sub-grouping paging indication carried by PEI-only when the per PO paging rate is 10% or 20% respectively. Observation 5: Adopting more than 8 sub-groups per PO will bring a marginal increase of power saving gain. Proposal 2: Sub-grouping indication carried in sequence-based PEI-only should be supported for paging enhancement. Proposal 3: 4 or 8 sub-groups configured per PO is preferred. Proposal 4: The sub-grouping indication by using paging PDCCH should be excluded. And reply the LS sending from RAN2 as follow: - From RAN1 perspective, the sub-grouping indication by using paging PDCCH should not be supported. |
| Spreadtrum |  |
| CATT | Figure 10: Additional power saving gain of sequence-based PEI with sub-grouping Observation 8: Introducing sub-grouping in sequence-based PEI can bring 30.19%~31.73% and 1.2%~2.21% power saving gain compare to Rel-16 paging and sequence-based PEI respectively. Observation 9: With the increase of the number of sub-group, the gain of power saving gain is tending flat. Figure 11: Power saving gain of option 1 compare to option 2 Observation 10: To support sequence-based PEI with sub-grouping, the power saving gain of option 1 relative to option 2 is negligible. - Option 1: Multiple sub-grouping PEI can be transmitted in a resource. - Option 2: Single sequence is transmitted in a resource. Proposal 5: The sequence-based PEI with sub-grouping does not provide much power saving gain over that without sub-grouping and require large additional overhead. Supporting sub-grouping should be further study. Proposal 6: If the sequence-based PEI with sub-grouping is supported, the number of sub-groups should be up to 8 sub-groups considering the trade-off between power saving gain and system overhead although the sequence-based subgrouping could support up to 222 code points. Assume that the paging rate for a PO is 10% and one PEI indicates 4 POs. The paging rates of 0, 1, 2, 3, 4 POs from network perspective are calculated in Table 8. As given in this Table, the probability of more than one PO being paged is 5.23%. If more than one PO are paged, common sequence-based PEI is transmitted, some POs would be false alarmed and waste unnecessary power to decode PDCCH and PDSCH. But the false alarm rate of the POs not paged is less than 5.23% in the worst case, which is acceptable. For supporting indicate multiple POs and sub-grouping by one sequence-based PEI, this design only causes acceptable false alarm but the system resource overhead is not increased Table 8: Paging rate for different PO numbers from network perspective Case; The number of POs paged; Calculate formula; Paging rate. Case 1; 0; (1-0.1)^4; 65.61%. Case 2; 1; C41*0.1*(1-0.1)^3; 29.16%. Case 3; 2; C42*0.1^2*(1-0.1)^2; 4.86%. Case 4; 3; C43*0.1^3*(1-0.1); 0.36%. Case 5; 4; C44*0.1^4; 0.01%. Observation 13: For supporting sequence-based PEI associated with multiple POs and sub-grouping, if no more than one sequence is transmitted in a resource at a given time, the false alarm probability is acceptable. |
| CMCC | Proposal 2. Support using PDCCH-based PEI to provide UE subgrouping indication. Table 2. Power saving gain of PEI based UE subgrouping Paging rate of one PO; 10%; 25%. Subgroup number; 2; 4; 8; 12; 2; 4; 8; 12. Power saving gain; low SINR; 21.75%; 22.35%; 22.65%; 22.75%; 20.70%; 22.27%; 23.08%; 23.35%. high SINR; 11.39%; 11.73%; 11.90%; 11.96%; 10.69%; 11.58%; 12.04%; 12.20%. Proposal 4. The maximum number of subgroups of one PO can be 8. |
| Qualcomm | Observation 1: UE sub-groups can be indicated by - Paging PDCCH - Unused bits and/or reserved bits of the DCI, this includes cross-slot scheduling based paging PDCCH as PEI - PDCCH based PEI - DCI field bits - RS or sequence-based PEI - Different sequences, these sequences can be transmitted in different sets of RBs and symbols. Proposal 1: Support UE sub-group indication carried by narrowband sequence-based (e.g., SSS) PEI for Rel-17 idle/inactive mode power saving. Observation 3: PEI can provide essential power saving gain for idle and inactive mode UEs - Optimal location of PEI transmission can be different for different SINR conditions - Optimal location of PEI transmission can be different for PDCCH-based and RS/sequence-based PEI due to potential different requirement of time and frequency synchronization and AGC accuracy. PDCCH-based PEI may have lower power saving gain than RS/sequence-based PEI - When UE sub-grouping is adopted, it provides about 2% additional power saving gain. Figure 3: Power saving gain vs. number of UE subgroups at optimal location |
| OPPO | For DCI-based PEI, almost all companies evaluated the performance assumed the payload size is 12 bits, the number 12 is the minimum DCI payload size w/o zero padding, and the aggregation level is assumed as 4(288 REs)/8(576 REs). For sequence-based PEI, the performance evaluation of all companies shows the UE (sub)group indication capacity of sequence-based PEI is worse than DCI-based PEI. Observation 1: DCI-based PEI can support larger number of subgroups indication. In the RAN1#103-e meeting, sub-grouping on Paging DCI is also studied, it is clearly subgrouping on Paging DCI can provide marginal power saving gain about 0.3 - 1.1%. It is not necessary to further study the subgrouping on Paging DCI. Proposal 4: The sub-grouping indication is supported by PEI, while sub-grouping indication by paging PDCCH is not supported. |
| Intel | Observation 5: TRS-based PEI may include UE subgrouping information for at least 8 sub-groups where TRS BW can be as low as 24 RBs Observation 12: If N sub-groups are indicated via sequence transmission, UE needs to check 2^(N-1) sequences, i.e., only the sequences that would wake up UE’s subgroup. Table 3: Power consumption (average value/slot) of different UE sub-grouping indication methods. 1 SSB is monitored before PEI. No additional SSB monitoring is assumed between PEI and PO. GPR 10% and 1.28S cycle are assumed. # subgroups; PEI indication (number of sub-groups/sets); PO (number of sub-groups per set); Power Consumption - No Cross-Slot Sch.; Power Consumption – w/ Cross-Slot Sch.. 8; 8; 1; 1.4085; 1.4081. 2; 4; 1.4142; 1.4126. 4; 2; 1.4104; 1.4096. 16; 16; 1; 1.4074; 1.4072. 2; 8; 1.4141; 1.4125. 8; 2; 1.4083; 1.4079. 4; 4; 1.4102; 1.4094. Observation 13: Although PDCCH-based PEI may potentially include information of a larger number of UE sub-groups, this does not seem to offer critical PS advantage compared to the case, when both PEI and Paging PDCCH jointly indicate the UE sub-groups. Proposal 4: Both PEI and paging DCI may jointly indicate UE sub-grouping information, especially when number of sub-groups is large and PEI is sequence based. Proposal 5: Signalling design for UE sub-grouping can be postponed until after PEI signal/channel design is confirmed. |
| Apple |  |
| Sony | Observation 4 – Among the three sub-grouping candidates, UE sub-grouping using sequenced-based/SSS-based early paging indication fulfils the condition that paging enhancement schemes should avoid the UE unnecessarily transitioning from/to deep sleep and from/to synchronization states, when there is no paging for a target UE. Table 2 - Summary of power saving gains for different UE subgrouping techniques in percentage UE sub-grouping technique; Low SINR/bad coverage; High SINR/good coverage. N=10/paging rate per PO = 10%; N=100/paging rate per PO = 60%; N=10/paging rate per PO = 10%; N=100/paging rate per PO = 60%. Cross-slot scheduling; 0.32; 3.0; 0.1; 0.99. SSS-based PEI carries UE subgrouping info.; 12.66; 13.67; 27.56; 25.02. SSS-based PEI + UE subgroup using paging DCI; 12.58; 9.93; 26.28; 11.77. Observation 5 – The amount of power saving gain for high paging rate per PO is much higher when UE grouping is carried in a sequence-based early paging indicator than when UE sub-grouping is done using paging DCI. Proposal 3 – Use sequence-based early paging indicator with sub-grouping for paging enhancement and reducing unnecessary paging reception. |
| Samsung | Proposal 7: Support at least paging PDCCH only for UE subgrouping indication without additional resource overhead. Proposal 8: Support CDM and/or FDM/TDM for UE subgroup indication based RS based PEI. Observation 11: The minimum required RE for CSI-RS/TRS based PEI to achieve target reliability based on Behav-A are as follows: - TRS only without CDM requires 36 RBs* 6REs/RB = 216 REs - CSI-RS with CDM8-FD2-TD4 requires 21 RB*8REs/RB = 168 REs Observation 12: The minimum required RE for CSI-RS/TRS based PEI to achieve target reliability based on Behav-B are as follows: - TRS only without CDM requires 27 RBs* 6REs/RB = 162 REs - CSI-RS with CDM8-FD2-TD4 requires 15 RB*8REs/RB = 120 REs |
| MediaTek | Observation 6: From the identified configurations based on evaluation results from more than one company, - UE (sub)group indication capacity per PDCCH-based PEI of AL4/AL8 is 12 bits - AL4 or AL8 can be selected by network according paging PDSCH TB scaling setting - UE (sub)group indication capability per PDCCH-based PEI of AL8/AL16 can be increased to 48 bits since AL8/AL16 of 48-bit payload and 24-bit CRC has the same code rate as AL/4AL8 of 12-bit payload and 24-bit CRC - UE (sub)group indication capacity per TRS/CSI-RS-based PEI of 50-RB-and-1-slot format is 1 bit - UE (sub)group indication capacity per SSS-based PEI is of 11-RB-and-2-symbol format is 1 bit - Note: By X bits, it means there can provide 2^X states with the resource of an identified PEI physical-layer configuration. For X > 1 bit, there is additional design flexibility in mapping the states to the paging combination of associated UE (sub)groups. For ease of comparison, we assume one bit is mapped to one UE (sub)group in later analyses. Proposal 6: RAN1 physical-layer design should maximize the flexibility for RAN2 UE subgrouping enhancement. In particular, maximum number of UE subgroups per PO should be at least 8, and more PO/group paging rate cases, e.g., 10%, 20% - 60% should be considered (as was done in RAN1 LS to RAN2. Proposal 7: For PDCCH-based PEI, subgroup indications for a PO are carried by PEI. Proposal 8: For TRS/CSI-RS-based PEI, PO indication is carried by PEI while subgroup indications for a PO are carried by a paging DCI. - In case of multiple POs in a SS burst period, multiple PEIs are FDMed in the resource configured by a common sp/periodic or aperiodic ZP-CSI-RS rate-matching resource set for semi-static or dynamic resource sharing with PDSCH of legacy UE, respectively. Proposal 9: For SSS -based PEI, PO indication is carried by PEI while subgroup indications for a PO are carried by a paging DCI. - In case of multiple POs in a SS burst period, multiple PEIs are multiplexed in the resource configured by semi-static RB-symbol rate-matching pattern(s) or CORESET 0 for semi-static or dynamic resource sharing with PDSCH of legacy UE, respectively. Proposal 10: PDCCH-based PEI is adopted to support UE subgrouping enhancement - UE subgrouping enhancement is not applied if PEI is not configured |
| LG | Observation 1: The UE sub-group indication using PEI outperforms UE sub-group indication within a PO. Observation 2: Compared to the case where only UE_ID based sub-grouping is supported, higher UE sub-grouping efficiency can be achieved when network controlled sub-grouping is configured. Proposal 2: The maximum number of UE sub-groups per PO should be at least 8 It should be considered that paging rate of normal NR UE would be much higher than MTC/NB-IoT UE. In MTC/NB-IoT, common WUS can be used to wake up more than one UE groups at the same time. The common WUS would be transmitted more frequently when UEs have higher paging rate, and it causes degradation of the benefit from UE group indication. M |
| Panasonic | Proposal 4: Sub-grouping information could be carried in the PEI. Proposal 5: Which sub-grouping UE belongs are informed from the higher layer. Proposal 6: Sub-grouping information can also be carried in the paging DCI. When PEI is configured, more refined sub-grouping indication is achieved. When PEI is not configured, just sub-grouping indication within paging DCI can also serve the function. Table 1. Example of configurable subgrouping indication by PEI and paging DCI Channel carrying subgrouping index; Number of bits of the subgrouping index carried by each channel.. PEI; 0; 1; 2. Paging DCI; 0 or 1; 0; 0, 1 or 2. |
| Nokia | Observation: Supporting sufficient number of sub-groups enables to obtain comparable power saving gains with random subgrouping as with deterministic sub-group assignment. Observation: Based on RAN2 feedback RAN1 PEI design needs to support sub-grouping, considering both network controlled grouping and UE-ID based grouping. Observation: Based on RAN2 feedback the maximum number of supported sub-groups needs to be at least 8. Proposal: The EPI design supports at least 8 sub-groups per PO as suggested by RAN2. Actual number used should be configurable by network. |
| Xiaomi | Proposal 3: Sub-grouping methods by 1) reserved bits in legacy paging DCI and 2) DCI-based PEI should be further studied. |
| DOCOMO | Observation 3: DCI-based PEI that can notify a lot of information is desirable for subgroups. Observation 4: The power saving gain can be obtained at any grouping rate by combining PEI with subgroup indication and/or TRS/CSI-RS indication. |
| InterDigital |  |
| Lenovo | Observation 1: Sub-group based PEI (i.e. sub-grouping of UEs of the same PO) may degrade detection/decoding performance of PDCCH based PEI due to an increased DCI size. Observation 2: Sub-group based indication in paging DCI to skip decoding of a paging message is beneficial for UE power saving (e.g. reduction of paging PDSCH decoding rate from 28% to 6%). Proposal 6: Support sub-group based indication in paging DCI to skip decoding of a paging PDSCH. |
| Ericsson | - Observation 1 Based on input from RAN2, PEI is meant to carry paging sub-grouping information for addressing at least 8 sub-groups for one PO. - Proposal 1 In deployments of high paging rate, in order to avoid excessive false paging during simultaneous paging of more than one sub-group, PEI supports addressing individual sub-group invocation (i.e. 8 bits, one per each of 8 subgroups in a PO). - Proposal 2 In deployments of high paging rate, where one-to-many PEI configuration is a necessary tool for the NW for avoiding excessive resource waste, PEI supports addressing sub-groups of at least up to 4 POs (i.e. in addition to the 8 bits used for sub-groups). - Proposal 3 In order to not waste bits unnecessarily in deployments where low paging rate is expected (few sub-groups/one-to-one configured), the sub-group/PO addressing bits shall be configurable (both number of bits and their meaning). |
| Nordic | In Figure 1 paging group size is shown to be dependent on number of UEs in the tracking area. In the Figure, each curve illustrates different number of paging groups (1 to 1024) available. If there are for example 1000 UEs in the same tracking area monitoring paging divided in 8 paging groups, there are about 125 UEs per paging group monitoring paging as can be read in grey colored curve in Figure 1. Figure 1 Number of UEs per paging group dependent on number of UE in tracing area and number of paging groups Observation-6: The paging group size depends on the DRX cycle parameters and the number of the UEs in the tracking area. Observation-7: The larger the paging groups are the more power is consumed by the UEs in the group Figure 2 below illustrates an example of wake-up probability from individual UE point of view as a function of paging group size for different individual paging probabilities. The same paging probability is assumed for each UE. It can be seen from the figure that when the number of UEs per group increases, the probability to wake up increases. The linearity of the increase depends on the UE paging rate. Higher the UE paging rate is more concave the dependency is. This means that group paging rate reduction does not saturate with number of sub-groups, but contrary. Therefore, we think that NR should support at least 8 subgroups to cover scenarios with many eMBB/RedCap UE operating in wide NR cells, where UE groups could be of 200-300 size. Proposal-2: Support at least 8 sub-groups as recommended by RAN2. Figure 2 UE wake up probability as a function of UEs per paging group for different UE paging probabilities |

One important goal of evaluating UE sub-grouping is to reply RAN2 LS where maximum #subgroups per PO can be at least 8 is indicated. Since RAN2 leaves the final decision to RAN1, and from companies inputs, 8 looks the value all companies can be fine. In this regard, the following proposal is suggested:

| Maximum #subgroups per PO | Supporting companies |
| --- | --- |
| ≥8 | ZTE, HW, Intel, MTK, LG, Nokia, Ericsson, Nordic (8 companies) |
| 8 | CMCC (1 company) |
| ≤8 | Vivo, CATT (2 companies) |

Proposal 7:

For UE subgroups indication in physical layer, maximum of 8 subgroups per PO is supported.

Next, we further identify the subgroups indication design for each PEI physical layer candidates, considering the following:

- From Observation 2a agreed in RAN1 #104-bis-e for Behv-A, as quoted below, UE (sub)group indication capacity per PEI with majority can be identified as

  - PDCCH-based PEI: ≥12 bits (supported by 13 out of 13 companies)

  - SSS-based PEI: 1 bit (supported 6 out of 9 companies)

  - TRS/CSI-RS-based PEI: 1 bit (supported 7 out of 10 companies)

| Paging Setting | PEI candidate design | Physical-layer configuration and resource | UE (sub)group indication capacity | Number of companies providing performance results |
| --- | --- | --- | --- | --- |
| PDSCH: MCS0, TB scaling 1.0 PDCCH: AL8, 41-bit payload | PDCCH-based PEI | AL4 PDCCH with 12-bit payload, occupying 288 REs | 12 bits | 5 (HW/HiSi, OPPO, ZTE, CATT, MTK) |
|  |  | AL8 PDCCH with 12-bit payload, occupying 576 REs | 12 bits | 7 (Xiaomi, Intel, QC, Samsung, IDCC, Ericsson, vivo) |
|  |  | AL8 PDCCH with 41-bit payload, occupies 576 REs | 41 bits | 1 (CATT) |
|  | SSS-based PEI | 1-symbol SSS, occupying 132 REs (11 RB x 1 symbol) | 3 bits | 1 (IDCC) |
|  |  | 2-symbol SSS, occupying 264 REs (11 RB x 2 symbols) | 1 bit | 6 (HW/HiSi, vivo, ZTE, CATT, QC, Samsung) |
|  |  |  | 3 bits | 1 (IDCC) |
|  |  | 3-symbol SSS, occupying 396 REs (11 RB x 3 symbols) | 4 bits | 1 (MTK) |
|  | TRS/CSI-RS-based PEI | 1-slot 24-RB TRS, occupying 144 REs (24 RB x 3 REs per RB x 2 symbols) | ≥ 8 bits | 1 (Intel) |
|  |  | 1-slot 28-RB TRS, occupying 168 REs (28 RB x 3 REs per RB x 2 symbols) | 1 bit | 1 (HW/HiSi) |
|  |  | 1-slot 36-RB TRS, occupying 216 REs (36 RB x 3 REs per RB x 2 symbols) | 1 bit | 1 (Samsung) |
|  |  | 1-slot 48-RB TRS, occupying 288 REs (48 RB x 3 REs per RB x 2 symbols) | 1 bit | 3 (vivo, ZTE, Ericsson) |
|  |  |  | 6 bits | 1 (CATT) |
|  |  | 1-slot 50-RB TRS, occupying 300 REs (50 RB x 3 REs per RB x 2 symbols) | 1 bit | 2 (OPPO, QC) |
|  |  |  | 4 bits | 1 (MTK) |

- Bit-to-(sub)group mapping or code-point-based mapping?

  - By identifying the possible UE (sub)group indication capacity that can comply with the performance requirement, there is additional freedom in designing the mapping between the available code-points (of size 2^#bits) and all the paging combinations of associated POs or UE subgroups.

  - Code-point-based mapping can bring performance benefit by reducing the required number of sequence realizations for sequence-based PEI or number of DCI payload bits for PDCCH-based PEI. On the other hand, as noted by LG and ZTE, there is increased false paging rate for the use case with higher PO paging rate. Example can be checked as below with N (=4) POs and PO paging rate of P:

| PO paging rate, P | Prob. UE is paged (bit-to-PO mapping): (1-P) | Prob. UE is paged (code-point mapping with 1 common code-point/seq. for the case of two or more POs paged): 1 – (Prob. no PO paged) – (Prob. one other PO is paged) = 1-1-PN-C1N-1P1-PN-1 |
| --- | --- | --- |
| 10% | 10% | 12.5% ( +2.5%) |
| 20% | 20% | 28.3% ( +8.3%) |
| 30% | 30% | 45.1% (+15.1%) |
| 40% | 40% | 61.1% (+21.1%) |

  - Since the false paging rate increment can be confined if applying code-point-based mapping to subgroups in a PO, one way forward is to first support simple bit-to-group mapping and FFS code-point based mapping

By the above the following proposal is suggested as the way forward:

FL-[Q3.1-1]

Proposal:

- If PDCCH-based PEI is selected for Rel-17 paging enhancement,

  - Subgroups in a PO are indicated by one PDCCH-based PEI

  - One bit in the DCI payload indicating one UE subgroup is supported

- If TRS/CSI-RS-based PEI or SSS-based PEI is selected for Rel-17 paging enhancement,

  - One PEI indicating one PO or one UE subgroup is supported

- FFS: Whether and how code-point based mapping is utilized for paging indication to subgroups in a PO

## PEI Comparisons with UE Sub-grouping Enhancement

In Table 7, companies’ inputs related to PEI comparisons with UE sub-grouping enhancement are collected:

Table 7: Companies’ inputs related to PEI comparisons with UE sub-grouping enhancement

| Company | Company Views |
| --- | --- |
| ZTE, Sanechips | - Observation 26: When sub-grouping is considered, the DCI-based PEI can provide larger power saving gain than sequence-based PEI. Table 8 Power saving gain and resource overhead of PEI with 1 PO, 8 sub-groups PEI associated with 1 PO; Resource overhead per PO - Behv-A; Resource overhead per PO - Behv-B; Power saving gain. Paging rate; Paging rate; Paging rate. 10%; 40%; 60%; 10%; 40%; 60%; 10%; 40%; 60%. DCI-based PEI (with AL=4) Dynamic resource sharing; 28.8; 115.2; 172.8; 259.2; 172.8; 115.2; 25.4%; 25.7%; 25.4%. TRS-like PEI Dynamic or semi-static resource sharing 48 RB and 2 symbols; No sub-grouping; 28.8 or 288; 115.2 or 288; 172.8 or 288; 259.2 or 288; 172.8 or 288; 115.2 or 288; 22.5%; 14.4%; 9.3%. Sub-grouping information carried by PEI; 30.0 or 2304; 142.8 or 2304; 248.8 or 2304; 2274 or 2304; 2161 or 2304; 2055 or 2304; 25.4%; 25.7%; 25.4%. Sub-grouping information carried by paging DCI; 28.8 or 288; 115.2 or 288; 172.8 or 288; 259.2 or 288; 172.8 or 288; 115.2 or 288; 23.5%; 18.4%; 15.0%. SSS-like PEI Dynamic or semi-static resource sharing 11 RB and 2 symbols; No sub-grouping; 26.4 or 264; 105.6 or 264; 158.4 or 264; 237.6 or 264; 158.4 or 264; 105.6 or 264; 22.5%; 14.4%; 9.3%. Sub-grouping information carried by PEI; 27.5 or 2112; 130.9 or 2112; 228.1 or 2112; 2085 or 2112; 1981 or 2112; 1884 or 2112; 25.4%; 25.7%; 25.4%. Sub-grouping information carried by paging DCI; 26.4 or 264; 105.6 or 264; 158.4 or 264; 237.6 or 264; 158.4 or 264; 105.6 or 264; 23.5%; 18.4%; 15.0%. Table 9 Power saving gain and resource overhead of PEI with 2 POs, 8 sub-groups PEI associated with 2 POs; Resource overhead per PO - Behv-A; Resource overhead per PO - Behv-B; Power saving gain. Paging rate; Paging rate; Paging rate. 10%; 40%; 60%; 10%; 40%; 60%; 10%; 40%; 60%. DCI-based PEI (with AL=4) Dynamic resource sharing; 27.4; 92.2; 121; 142.6; 121; 92.2; 25.4%; 25.7%; 25.4%. TRS-like PEI Dynamic or semi-static resource sharing 48 RB and 2 symbols; No sub-grouping; 28.8 or 288; 115.2 or 288; 172.8 or 288; 259.2 or 288; 172.8 or 288; 115.2 or 288; 22.5%; 14.4%; 9.3%. Sub-grouping information carried by PEI; 30.0 or 2304; 142.8 or 2304; 248.8 or 2304; 2274 or 2304; 2161 or 2304; 2055 or 2304; 25.4%; 25.7%; 25.4%. Sub-grouping information carried by paging DCI; 28.8 or 288; 115.2 or 288; 172.8 or 288; 259.2 or 288; 172.8 or 288; 115.2 or 288; 23.5%; 18.4%; 15.0%. SSS-like PEI Dynamic or semi-static resource sharing 11 RB and 2 symbols; No sub-grouping; 26.4 or 264; 105.6 or 264; 158.4 or 264; 237.6 or 264; 158.4 or 264; 105.6 or 264; 22.5%; 14.4%; 9.3%. Sub-grouping information carried on PEI; 27.5 or 2112; 130.9 or 2112; 228.1 or 2112; 2085 or 2112; 1981 or 2112; 1884 or 2112; 25.4%; 25.7%; 25.4%. Sub-grouping information carried on paging DCI; 26.4 or 264; 105.6 or 264; 158.4 or 264; 237.6 or 264; 158.4 or 264; 105.6 or 264; 23.5%; 18.4%; 15.0%. Table 10 Power saving gain and resource overhead of PEI with 4 POs, 8 sub-groups PEI associated with 4 POs; Resource overhead per PO - Behv-A; Resource overhead per PO - Behv-B; Power saving gain. Paging rate; Paging rate; Paging rate. 10%; 40%; 60%; 10%; 40%; 60%; 10%; 40%; 60%. DCI-based PEI (with AL=8) Dynamic resource sharing; 49.6; 125.4; 140.4; 144; 140.4; 125.4; 25.4%; 25.7%; 25.4%. TRS-like PEI Dynamic or semi-static resource sharing 48 RB and 2 symbols; No sub-grouping; 28.8 or 288; 115.2 or 288; 172.8 or 288; 259.2 or 288; 172.8 or 288; 115.2 or 288; 22.5%; 14.4%; 9.3%. Sub-grouping information carried on PEI; 30.0 or 2304; 142.8 or 2304; 248.8 or 2304; 2274 or 2304; 2161 or 2304; 2055 or 2304; 25.4%; 25.7%; 25.4%. Sub-grouping information carried on paging DCI; 28.8 or 288; 115.2 or 288; 172.8 or 288; 259.2 or 288; 172.8 or 288; 115.2 or 288; 23.5%; 18.4%; 15.0%. SSS-like PEI Dynamic or semi-static resource sharing 11 RB and 2 symbols; No sub-grouping; 26.4 or 264; 105.6 or 264; 158.4 or 264; 237.6 or 264; 158.4 or 264; 105.6 or 264; 22.5%; 14.4%; 9.3%. Sub-grouping information carried on PEI; 27.5 or 2112; 130.9 or 2112; 228.1 or 2112; 2085 or 2112; 1981 or 2112; 1884 or 2112; 25.4%; 25.7%; 25.4%. Sub-grouping information carried on paging DCI; 26.4 or 264; 105.6 or 264; 158.4 or 264; 237.6 or 264; 158.4 or 264; 105.6 or 264; 23.5%; 18.4%; 15.0%. |
| Huawei, HiSilicon | Observation 11: PDCCH-based PEI carrying 8 sub-group indication can provide the most efficient resource utilization with comparable power saving gain compared to TRS based PEI. Observation 12: PDCCH-based PEI carrying 8 sub-group indication can provide better power saving performance with smaller resource overhead compared to SSS based PEI. Table 2 Comparison of PEI designs with respect to resource overhead and power saving gain. - Evaluation results with one PO per SS burst period for Behv-A (2 SS bursts needed before PO for baseline) PDCCH-based; TRS-based; SSS-based. PEI indication; 8 sub-groups; 8 sub-groups; PO-wise; 8 sub-groups; PO-wise. Paging DCI indication; No need; No need; 8 sub-groups; No need; 8 sub-groups. Co-existence method; Dynamic sharing PDCCH resource; Dynamic sharing PDSCH resource by CORESET0 with multiple MOs (48RB*2symbol*2); Dynamic sharing PDSCH using ZP-CSI-RS resource; Dynamic sharing PDSCH resource by CORESET0 with multiple MOs ( 48RB*2symbol*2); Dynamic sharing PDSCH resource by CORESET0(24RB* 2 symbol). Resource overhead @ PO paging rate (REs per PO); 10%; 28.8; 230.4; 28.8; 230.4; 57.6. 20%; 57.6; 460.8; 57.6; 460.8; 115.2. 40%; 115.2; 921.6; 115.2; 921.6; 230.4. 60%; 170.2; 1382.4; 172.8; 1382.4; 345.6. Power saving gain without interFrequency measurement @PO paging rate Power saving gain with interFrequency measurement @ PO paging rate; 10%; 31.70% 20.36%; 31.88% 20.51%; 30.66% 19.99%; 31.73% 20.39%; 28.90% 18.56%. 20%; 31.95% 20.71%; 32.31% 21.01%; 29.79% 19.93%; 31.95% 20.71%; 26.18% 16.98%. 40%; 31.64% 20.84%; 32.69% 21.70%; 27.93% 19.66%; 31.64% 20.84%; 20.77% 13.79%. 60%; 31.02% 20.73%; 32.85% 22.24%; 26.11% 19.34%; 31.02% 20.73%; 15.60% 10.70%. - Evaluation results with four POs per SS burst period for Behv-A (2 SS bursts needed before PO for baseline) PDCCH-based; TRS-based; SSS-based. PEI indication; 8 sub-groups per PO, 4POs; 8 sub-groups per PO, 4POs by FDM and TDM; PO-wise; 8 sub-groups per PO, 4POs by FDM and TDM; PO-wise. Paging DCI indication; No need; No need; 8 sub-groups; No need; 8 sub-groups. Co-existence method; Dynamic sharing PDCCH resource; Not available by using CORESET0 with multiple MOs (re quire 48RB*2symbol*2*4); Dynamic sharing PDSCH using four ZP-CSI-RS resource in one ZP CSI-RS resource set; Not available by using CORESET0 with multiple MOs (re quire 48RB*2symbol*2*4); Dynamic sharing PDSCH resource by CORESET0 (48RB*2symbols). Resource overhead @ PO paging rate (REs per PO); 10%; 49.5; N/A; 99.0; N/A; 99.0. 20%; 85.0; N/A; 170.0; N/A; 170.0. 40%; 125.3; N/A; 250.7; N/A; 250.7. 60%; 140.3; N/A; 280.6; N/A; 280.6. Power saving gain without interFrequency measurement @PO paging rate Power saving gain with interFrequency measurement @ PO paging rate; 10%; 31.70% 20.36%; N/A; 30.66% 19.99%; N/A; 28.90% 18.56%. 20%; 31.95% 20.71%; N/A; 29.79% 19.93%; N/A; 26.18% 16.98%. 40%; 31.64% 20.84%; N/A; 27.93% 19.66%; N/A; 20.77% 13.79%. 60%; 31.02% 20.73%; N/A; 26.11% 19.34%; N/A; 15.60% 10.70%. - Evaluation results with one PO per SS burst period for Behv-A (3 SS bursts needed before PO for baseline) PDCCH-based; TRS-based; SSS-based. PEI indication; 8 sub-groups; 8 sub-groups; PO-wise; 8 sub-groups; PO-wise. Paging DCI indication; No need; No need; 8 sub-groups; No need; 8 sub-groups. Co-existence method; Dynamic sharing PDCCH resource; Dynamic sharing PDSCH resource by CORESET0 with multiple MOs (48RB*2symbol*2); Dynamic sharing PDSCH using ZP-CSI-RS resource; Dynamic sharing PDSCH resource by a CORESET of 96RB*2symbol; Dynamic sharing PDSCH resource by CORESET0(24RB* 2 symbol). Resource overhead @ PO paging rate (REs per PO); 10%; 28.8; 230.4; 28.8; 230.4; 57.6. 20%; 57.6; 460.8; 57.6; 460.8; 115.2. 40%; 115.2; 921.6; 115.2; 921.6; 230.4. 60%; 170.2; 1382.4; 172.8; 1382.4; 345.6. Power saving gain without interFrequency measurement @PO paging rate Power saving gain with interFrequency measurement @ PO paging rate; 10%; 42.19% 30.65%; 42.34% 30.78%; 39.73% 29.05%; 42.19% 30.65%; 38.11% 27.69%. 20%; 42.12% 30.72%; 42.43% 30.97%; 37.54% 27.72%; 42.12% 30.72%; 34.49% 25.15%. 40%; 41.11% 30.20%; 42.00% 30.95%; 32.78% 24.79%; 41.11% 30.20%; 26.70% 19.67%. 60%; 39.77% 29.42%; 41.33% 30.74%; 28.11% 21.90%; 39.77% 29.42%; 19.11% 14.30%. - Evaluation results with four PO per SS burst period for Behv-A (3 SS bursts needed before PO for baseline) PDCCH-based; TRS-based; SSS-based. PEI indication; 8 sub-groups per PO, 4POs; 8 sub-groups per PO, 4POs by FDM and TDM; PO-wise; 8 sub-groups per PO, 4POs; 8 sub-groups per PO, 4POs by FDM and TDM. Paging DCI indication; No need; No need; 8 sub-groups; No need; No need. Co-existence method; Dynamic sharing PDCCH resource; Not available by using CORESET0 with multiple MOs (re quire 48RB*2symbol*2*4); Dynamic sharing PDSCH using four ZP-CSI-RS resource in one ZP CSI-RS resource set; Dynamic sharing PDCCH resource; Not available by using CORESET0 with multiple MOs (re quire 48RB*2symbol*2*4). Resource overhead @ PO paging rate (REs per PO); 10%; 49.5; N/A; 99.0; N/A; 99.0. 20%; 85.0; N/A; 170.0; N/A; 170.0. 40%; 125.3; N/A; 250.7; N/A; 250.7. 60%; 140.3; N/A; 280.6; N/A; 280.6. Power saving gain without interFrequency measurement @PO paging rate Power saving gain with interFrequency measurement @ PO paging rate; 10%; 42.19% 30.65%; N/A; 39.73% 29.05%; N/A; 38.11% 27.69%. 20%; 42.12% 30.72%; N/A; 37.54% 27.72%; N/A; 34.49% 25.15%. 40%; 41.11% 30.20%; N/A; 32.78% 24.79%; N/A; 26.70% 19.67%. 60%; 39.77% 29.42%; N/A; 28.11% 21.90%; N/A; 19.11% 14.30%. |
| vivo | For the evaluation of resource overhead with PEI candidate designs based on PDCCH, TRS/CSI-RS and SSS, we provide the evaluation results for PEI candidate designs based on the following assumptions: - Assumption of Behavior A/B; - Required #REs from performance evaluation described in section 3.1; - Per PO paging rate is 10%; - Dynamic rate-matching in PDCCH or PDSCH to co-exist with legacy signals/channels; - PO and PEI can be one to one mapping or four to one mapping; - As identified and analyzed in section 2.1, 8 UE sub-groups are configured with PEI-only for each PO and sequence-based grouping method introduced in Rel-16 NB-IoT is adopted; Table 6: Resource overhead of each PEI occasion with different PEI candidate designs PEI design; Parameters; Baseline: Resource overhead for per PEI occasion and per PO (unit: RE) PO and PEI mapping manner: 1 to 1 for sequence-based PEI; 1 to 1 for DCI based PEI;; Resource overhead for per PEI occasion and per PO (unit: RE) PO and PEI mapping manner: 1 to 1 for sequence-based PEI; 2 to 1 for DCI based PEI;. Behavior A; Behavior B; Behavior A; Behavior B. SSS like sequence; - Length: 127 - Sequence: same as SSS - Number of symbols (symbol): 2 or 4; 25.4/50.8 for 2/4 symbols; 228.6/457.2 for 2/4 symbols; 25.4/50.8 for 2/4 symbols; 228.6/457.2 for 2/4 symbols. TRS like sequence; - Number of RBs: 48 - Number of symbols (symbol): - 2 symbols in a slot - 4 symbols in 2 slots - Density: 3 REs/RB; 28.8/57.6 for 2/4 symbols; 259.2/518.4 for 2/4 symbols; 28.8/57.6 for 2/4 symbols; 259.2/518.4 for 2/4 symbols. DCI; - Aggregation level (AL): 8 or 16 - DCI size: 12 (minimum) or 40 bits (align with DCI 1-0); 57.6/115.2 for AL 8/16; 518.4/1036.8 for AL 8/16; 54.72/109.4 for AL 8/16; 466.56/933.12 for AL 8/16. Observation 13: In term of resource overhead for per PEI occasion and per PO, SSS-like sequence-based PEI will consume the least resource, TRS-like sequence-based PEI followed and DCI-based PEI comes last. |
| Spreadtrum |  |
| CATT | Table 7: Resource overhead evaluation for PEI with sub-grouping assuming multiple POs is not supported PEI candidate designs; Resource overhead(semi-static); Resource overhead(dynamic). PDCCH-based PEI(12 bits, 8 sub-groups); 36REs; 3.6 – 36 REs (depending whether the CCEs are used by other UEs or PDSCH when no paging). PDCCH-based PEI(41 bits, 8 sub-groups); 72REs; 7.2 – 72 REs (depending whether the CCEs are used by other UEs or PDSCH when no paging). TRS-based PEI(8 sub-groups); 0 or 36REs(0.8dB better than 12 bits PDCCH-based PEI); 0 or 3.6REs(0.8dB better than 12 bits PDCCH-based PEI). Observation 11: The resource overhead of sequence-based PEI is not greater than that of DCI-based PEI when sub-grouping is supported. Table 10: Comparison of PEI candidate designs with sub-grouping assuming that multiple POs is supported PEI candidate designs; Detection performance(the SNR at JMDR=1% without paging performance influence); Power saving gain; Coexistence; Sub-group numbers in evaluation; Resource overhead. DCI-based PEI(12 bits); -7.5dB(AL=4 for Alt 1); 4.41% ~10.07%; PDSCH, PDCCH; 8; 36 REs(semi-static) 3.6 REs – 36 REs (dynamic). DCI-based PEI(41 bits); -7.9dB(AL=8 for Alt 1); 4.41% ~10.07%; PDSCH, PDCCH; 32; 18 REs(semi-static) 6.19 REs – 18 REs (dynamic). TRS-based PEI; -8.3dB(Alt 1); 23.11% ~51.59%; PDSCH, TRS/CSI-RS; 32; 0 or 9REs (semi-static) 0 or 3.10REs (dynamic). Observation 14: The resource overhead of sequence-based PEI is smaller than that of DCI-based PEI when sub-grouping and multiple POs associated with one sequence-based PEI are supported. Observation 15: If sub-grouping is supported, the sequence-based PEI is better than DCI-based PEI from detection performance, power saving gain, coexistence and resource overhead views. |
| CMCC |  |
| Qualcomm | Table 5: Resource overhead for PEI that is semi-statically not available to legacy channels/signals PEI design; No. of UE sub-groups = 2; No. of UE sub-groups = 4; No. of UE sub-groups = 6; No. of UE sub-groups = 8. PDCCH; 576; 576; 576; 576. CSI-RS; 300; 600; 900; 1200. SSS; 288; 576; 864; 1152. Table 6: Resource overhead for PEI that is dynamically not available to legacy channels/signals PEI design; No. of UE sub-groups = 2; No. of UE sub-groups = 4; No. of UE sub-groups = 6; No. of UE sub-groups = 8. PDCCH; 57.6; 57.6; 57.6; 57.6. CSI-RS; 30; 30.8; 31.1; 31.2. SSS; 28.8; 29.6; 29.8; 29.9. Observation 7: The number of REs reserved by the PEI at a configured PEI occasion is determined by whether the resource is semi-statically or dynamic not available to legacy channels and signals - If resource for the configured PEI occasion is semi-statically not available to the other channels and signals, PDCCH based PEI has lower resource overhead than CSI-RS and SSS based PEI. - If resource for the configured PEI occasion is dynamically not available to the other channels and signals, PDCCH based PEI has higher resource overhead than CSI-RS and SSS based PEI. |
| OPPO |  |
| Intel | Table1: Power consumption of TRS and PDCCH based PEI with different number of sub-groups Number of sub-groups indicated by PEI; Seq based PEI (1 SSB before PEI, 0 SSB between PEI and PO) (values in % in indicate power consumption w.r.t. Seq-based PEI and a single sub-group); PDCCH PEI is placed far from PO, with 1 SSB in front, and 1 or more SSBs between PEI and PO (example 1) (values in % indicate power consumption w.r.t. Seq-based PEI); PDCCH PEI is placed closed to PO, with 1 or more SSBs in front (example 2) (values in % indicate power consumption w.r.t. Seq-based PEI). PDCCH based PEI (1 SSB before PEI, 1 SSB between PEI and PO); PDCCH based PEI (1 SSB before PEI, 2 SSBs between PEI and PO); PDCCH based PEI (1 SSB before PEI, 3 SSBs between PEI and PO); PDCCH based PEI (2 SSBs before PEI, 0 SSB between PEI and PO); PDCCH based PEI (3 SSBs before PEI, 0 SSB between PEI and PO). 1; 1.4243 (0.00%); 1.4477 (1.64% more); 1.4712 (3.29% more); 1.4946 (~ 5% more); 1.6541 (16.13% more); 1.885 (32.34% more). 4; 1.4108 (1.0% less); 1.4166 (0.41% more); 1.4225 (0.83% more); 1.4283 (1.24% more); 1.6451 (16.6% more); 1.8795 (33.22% more). 8; 1.4085 (1.12% less); 1.4114 (0.21% more); 1.4144 (0.42% more); 1.4173 (0.63% more); 1.6429 (16.64% more); 1.8773 (33.28% more). 12; 1.4078 (1.15% less); 1.4097 (0.14% more); 1.4117 (0.27% more); 1.4137 (0.42% more); 1.6421 (16.64% more); 1.8765 (33.29% more). 16; 1.4074 (1.19% less); 1.4089 (0.1% more); 1.4103 (0.21% more); 1.4118 (0.32% more); 1.6418 (16.66% more); 1.8761 (33.30% more). Observation 1: For the agreed evaluation assumptions, TRS based PEI results in more power saving gain than PDCCH based PEI Observation 2: For the agreed evaluation assumptions, relative PS gain diminishes as number of sub-groups is increased. - The PS gains saturate from 4 sub-groups onwards. Proposal 1: A maximum of 4 sub-groups per PO is preferred. Considering feedback from RAN2, at most 8 sub-groups may be supported per PO. |
| Apple |  |
| Sony |  |
| Samsung | Table 5: Power saving gain for UE subgrouping w/ group paging rate = 10.01% Alternatives; K=1; K=4; K=8; K=16; K=32. PEI; PDCCH based; 8.95%; 10.96%; 11.31%; 11.49%; 11.58%. RS based; 15.40%; 17.68%; 18.08%; 18.28%; 18.38%. Paging PDCCH; 0%; 0.86%; 1.02%; 1.09%; 1.13%. Note: Behav-A, UE monitors all MOs for PEI and paging PDCCH reception Table 6: Power saving gain for UE subgrouping w/ group paging rate = 19.02% Alternatives; K=1; K=4; K=8; K=16; K=32. PEI; PDCCH based; 7.48%; 11.21%; 11.89%; 12.24%; 12.41%. RS based; 13.53%; 17.76%; 18.54%; 18.93%; 19.13%. Paging PDCCH; 0%; 1.60%; 1.90%; 2.04%; 2.12%. Note: Behav-A, UE monitors all MOs for PEI and paging PDCCH reception Observation 8: UE sub-grouping on paging PDCCH achieves small (up to 2.12%) power saving gain, where the gain depends on group paging rate. Observation 9: UE sub-grouping on PEI achieves up to 11.58% - 19.13% power saving gain, depending on group paging rate, where RS based PEI achieves 1.5x power saving gain than PDCCH based PEI. Observation 10: The power saving gain converges when number of UE subgroups is large. No need to support UE sub-grouping larger than 8. Proposal 9: Support UE subgroup size up to 8 for PEI only. Observation 11: The minimum required RE for CSI-RS/TRS based PEI to achieve target reliability based on Behav-A are as follows: - TRS only without CDM requires 36 RBs* 6REs/RB = 216 REs - CSI-RS with CDM8-FD2-TD4 requires 21 RB*8REs/RB = 168 REs Observation 12: The minimum required RE for CSI-RS/TRS based PEI to achieve target reliability based on Behav-B are as follows: - TRS only without CDM requires 27 RBs* 6REs/RB = 162 REs - CSI-RS with CDM8-FD2-TD4 requires 15 RB*8REs/RB = 120 REs Table 7: Resource overhead (averaged RE) for PEI with UE subgrouping (Behav-A, UE group paging rate = 10%) K=4; K=8; K=16; K=32. PDCCH-based; per MO; 57.6; 57.6; 115.3; 115.3. per PO; 460.8; 460.8; 922.4; 922.4. CSI-RS/TRS-based; Per MO; 22.5 (TRS); 16.8 (CSI-RS); 16.8; 17.3; 17.5. Pre PO; 90 (TRS); 67.2 (CSI-RS); 67.2; 69.2; 70. Note: PDCCH-based PEI has 8 MOs, RS-based PEI has 4 MOs Table 8: Resource overhead (averaged RE) for PEI with UE subgrouping (Behav-B, UE group paging rate = 10%) K=4; K=8; K=16; K=32. PDCCH-based; per MO; 518.4; 518.4; 1036.7; 1036.7. per PO; 4147.2; 4147.2; 8293.6; 8293.6. CSI-RS/TRS-based; Per MO; 631.1 (TRS); 108.0 (CSI-RS); 108.0; 227.7; 467.5. Pre PO; 2524.4 (TRS); 432 (CSI-RS); 432; 910.8; 1870. Note: PDCCH-based PEI has 8 MOs, RS-based PEI has 4 MOs Observation 14: PDCCH-based PEI requires 3.4x to 6.6x more resource overheads than CSI-RS/TRS-based PEI for indicating UE subgroups for paging. |
| MediaTek | Proposal 7: For PDCCH-based PEI, subgroup indications for a PO are carried by PEI. Proposal 8: For TRS/CSI-RS-based PEI, PO indication is carried by PEI while subgroup indications for a PO are carried by a paging DCI. - In case of multiple POs in a SS burst period, multiple PEIs are FDMed in the resource configured by a common sp/periodic or aperiodic ZP-CSI-RS rate-matching resource set for semi-static or dynamic resource sharing with PDSCH of legacy UE, respectively. Proposal 9: For SSS -based PEI, PO indication is carried by PEI while subgroup indications for a PO are carried by a paging DCI. - In case of multiple POs in a SS burst period, multiple PEIs are multiplexed in the resource configured by semi-static RB-symbol rate-matching pattern(s) or CORESET 0 for semi-static or dynamic resource sharing with PDSCH of legacy UE, respectively. Observation 10: For NR idle/inactive-mode UEs, UE sub-grouping enhancement with paging early indication and 8 subgroups per PO can provide the following power saving gains w.r.t Rel-16 (details in Tables 3 -5): - With PDCCH-based PEI: - 18.0% –20.0% where the baseline assumes 1 SS burst for synchronization before PO reception - 22.4% –23.2% where the baseline assumes 2 SS bursts for synchronization before PO reception - 28.7% –29.1% where the baseline assumes 3 SS bursts for synchronization before PO reception - With TRS/CSI-RS-based PEI or SSS-based PEI: - 7.6% –15.8% where the baseline assumes 1 SS burst for synchronization before PO reception - 17.5% –21.4% where the baseline assumes 2 SS bursts for synchronization before PO reception - 20.3% –27.2% where the baseline assumes 3 SS bursts for synchronization before PO reception - The PO paging rate considered ranges from 10% to 60% Observation 10: For NR idle/inactive-mode UEs, UE sub-grouping enhancement with paging early indication and 8 subgroups per PO requires the following worst-case average resource overhead per PO w.r.t Rel-16 (details in Tables 3 -5): - With PDCCH-based PEI: - 57.6 REs – 99.0 REs for PO paging rate of 10% - 115.2 REs – 170.0 REs for PO paging rate of 20% - 230.4 REs – 250.7 REs for PO paging rate of 40% - 280.6 REs – 345.6 REs for PO paging rate of 60% - With TRS/CSI-RS-based PEI: - 300 REs for PO paging rate ranging from 10% to 60% - With SSS-based PEI: - 264 REs for PO paging rate ranging from 10% to 40% - 264 REs – 280.6 REs for PO paging rate of 60% - The PO number in a SS burst period considered ranges from 1 to 4 Table 3: Comparison of PEI candidate designs with number of POs in a SS burst period no larger than 1 and number of subgroups per PO set to 8 PEI indication; Paging DCI indication; Average resource overhead (#REs) per PO @ PO paging rate of; Power saving gain @ PO paging rate of. 10%; 20%; 40%; 60%; #SSB before PO; 10%; 20%; 40%; 60%. UE subgrouping without PEI; N/A; 8 sub-groups; N/A; 1; 0.9%; 1.9%; 3.7%; 5.4%. 2; 0.6%; 1.3%; 2.5%; 3.7%. 3; 0.6%; 1.2%; 2.3%; 3.4%. PDCCH-based PEI - AL4 ∼ AL8 with 12-bit payload and 1 bit per subgroup - Share CORESET 0 with legacy UEs; 8 sub-groups; No need; 28.8 ∼ 57.6; 57.6 ∼ 115.2; 115.2 ∼ 230.4; 172.8 ∼ 345.6; 1; 18.0%; 18.4%; 19.2%; 20.0%. 2; 22.4%; 22.5%; 22.8%; 23.2%. 3; 28.7%; 28.8%; 29.0%; 29.1%. TRS/CSI-RS-based PEI - 50-RB-and-2-symbol format with 1 PEI for 1 PO - Semi-static resource sharing: One sp ZP-CSI-RS rate-matching resource set of 1 TRS pattern - (Dynamic resource sharing: One aperiodic ZP-CSI-RS rate-matching pattern set of 1 TRS patterns) †; 1 PEI for 1 PO; 8 sub-groups; 300 ∼ (30); 300 ∼ (60); 300 ∼ (120); 300 ∼ (180); 1; 15.8%; 14.1%; 10.8%; 7.6%. 2; 21.4%; 20.6%; 19.0%; 17.5%. 3; 27.2%; 25.8%; 23.0%; 20.3%. SSS-based PEI - 11-RB-and-2-symbol format with 1 PEI for 1 PO - Semi-static resource sharing: RB-symbol rate-matching pattern with 11 RBs and 2 symbols - (Dynamic resource sharing: CORESET 0 of 576 REs) †; 1 PEI for 1 PO; 8 sub-groups; 264 ∼ (57.6); 264 ∼ (115.2); 264 ∼ (230.4); 264 ∼ (345.6); 1; 15.8%; 14.1%; 10.8%; 7.6%. 2; 21.4%; 20.6%; 19.0%; 17.5%. 3; 27.2%; 25.8%; 23.0%; 20.3%. † : Whether dynamic resource sharing is available is not guaranteed (network dependent). Table 4: Comparison of PEI candidate designs with number of POs in a SS burst period being set to 2 and number of subgroups per PO set to 8 PEI indication; Paging DCI indication; Average resource overhead (#REs) per PO @ PO paging rate of; Power saving gain @ PO paging rate of. 10%; 20%; 40%; 60%; #SSB before PO; 10%; 20%; 40%; 60%. PDCCH-based PEI - 2 PEIs, each of AL4∼AL8 with 12-bit payload and 1 bit per subgroup - Share CORESET 0 with legacy UEs; 8 sub-groups per PEI × 2 PEIs; No need; 28.8 ∼ 57.6; 57.6 ∼ 115.2; 115.2 ∼ 230.4; 172.8 ∼ 345.6; 1; 18.1%; 18.6%; 19.6%; 20.6%. 2; 22.4%; 22.6%; 23.0%; 23.4%. 3; 28.7%; 28.7%; 28.8%; 28.9%. TRS/CSI-RS-based PEI - 2 PEIs each of 50-RB-and-2-symbol format with 1 PEI for 1 PO - Semi-static resource sharing: One sp ZP-CSI-RS rate-matching resource set of 2 TRS patterns - (Dynamic resource sharing: One aperiodic ZP-CSI-RS rate-matching resource set of 2 TRS patterns) †; 2 POs indicated by 2 PEIs; 8 sub-groups; 300 ∼ (57); 300 ∼ (108); 300 ∼ (192); 300 ∼ (252); 1; 15.8%; 14.1%; 10.8%; 7.6%. 2; 21.4%; 20.6%; 19.0%; 17.5%. 3; 27.2%; 25.8%; 23.0%; 20.3%. SSS-based PEI - 2 PEIs, each of 11-RB-and-2-symbol format with 1 PEI for 1 PO - Semi-static resource sharing: RB-symbol rate-matching pattern with 22 RBs and 2 symbols - (Dynamic resource sharing: CORESET 0 of 576 REs) †; 2 POs indicated by 2 PEIs; 8 sub-groups; 264 ∼ (54.7); 264 ∼ (103.7); 264 ∼ (184.3); 264 ∼ (241.9); 1; 15.8%; 14.1%; 10.8%; 7.6%. 2; 21.4%; 20.6%; 19.0%; 17.5%. 3; 27.2%; 25.8%; 23.0%; 20.3%. †: Whether dynamic resource sharing is available is not guaranteed (network dependent. Table 5: Comparison of PEI candidate designs with number of POs in a SS burst period being set to 4 and number of subgroups per PO set to 8 PEI indication; Paging DCI indication; Average resource overhead (#REs) per PO @ PO paging rate of; Power saving gain @ PO paging rate of. 10%; 20%; 40%; 60%; #SSB before PO; 10%; 20%; 40%; 60%. PDCCH-based PEI - AL8 ∼AL16 with 32-bit payload - 1 bit per subgroup; 32 sub-groups indicated by 1 PEI; No need; 49.5 ∼ 99.0; 85.0 ∼ 170.0; 125.3 ∼ 250.7; 140.3 ∼ 280.6; 1; 18.0%; 18.4%; 19.2%; 20.0%. 2; 22.4%; 22.5%; 22.8%; 23.2%. 3; 28.7%; 28.8%; 29.0%; 29.1%. TRS/CSI-RS-based PEI - 4 PEIs, each of 50-RB-and-2-symbol format with 1 PEI for 1 PO - Semi-static resource sharing: One sp ZP-CSI-RS rate-matching resource set of 4 TRS patterns - (Dynamic resource sharing: One aperiodic ZP-CSI-RS rate-matching resource set of 4 TRS patterns) †; 4 POs indicated by 4 PEIs; 8 sub-groups; 300 ∼ (103); 300 ∼ (177.1); 300 ∼ (261.1); 300 ∼ (292.3); 1; 15.8%; 14.1%; 10.8%; 7.6%. 2; 21.4%; 20.6%; 19.0%; 17.5%. 3; 27.2%; 25.8%; 23.0%; 20.3%. SSS-based PEI - 4 PEIs, each of 11-RB-and-2-symbol format with 1 PEI for 1 PO - Semi-static resource sharing: RB-symbol rate-matching pattern with 44 RBs and 2 symbols - (Dynamic resource sharing: CORESET 0 of 1152 REs) †; 4 POs indicated by 4 PEIs; 8 sub-groups; 264 ∼ (99.0); 264 ∼ (170.0); 264 ∼ (250.7); 264 ∼ (280.6); 1; 15.8%; 14.1%; 10.8%; 7.6%. 2; 21.4%; 20.6%; 19.0%; 17.5%. 3; 27.2%; 25.8%; 23.0%; 20.3%. †: Whether dynamic resource sharing is available is not guaranteed (network dependent). |
| LG |  |
| Panasonic |  |
| Nokia | Figure 1 Processing timeline for DCI-based EPI in low SINR. UE is paged. Figure 2 Processing timeline for TRS-based EPI when an SSB is received prior to the EPI in low SINR. UE is paged. Figure 3 Processing timeline for SSS-based EPI in low SINR. UE is paged. Observation: DCI-based EPI with 4-8 sub-groups provide reasonable energy saving gain compared to DCI-based EPI without groups, while larger number of sub-groups (16) only marginally improve the energy saving potential. Table 1 Energy saving gain vs DCI-based EPI without subgrouping. All numbers in %. #Subgroups\Group paging rate; 10 %; 20 %; 30 %; 40 %. Low SINR; 2; 3.1; 5.8; 8.2; 10.3. 3; 4.1; 7.7; 10.9; 13.8. 4; 4.6; 8.6; 12.3; 15.5. 8; 5.4; 10.1; 14.3; 18.1. 16; 5.7; 10.8; 15.3; 19.4. Medium SINR; 2; 2.1; 4.0; 5.7; 7.3. 3; 2.8; 5.3; 7.6; 9.8. 4; 3.1; 5.9; 8.6; 11.0. 8; 3.6; 6.9; 10.0; 12.9. 16; 3.9; 7.4; 10.7; 13.8. High SINR; 2; 1.3; 2.6; 3.7; 4.9. 3; 1.8; 3.4; 5.0; 6.5. 4; 2.0; 3.8; 5.6; 7.3. 8; 2.3; 4.5; 6.6; 8.5. 16; 2.5; 4.8; 7.0; 9.1. Table 2 Energy consumption for DCI-based EPI relative to 10 % group paging rate. All numbers in %. Group paging rate; Low SINR; Medium SINR; High SINR. 5 %; 96.9; 97.9; 98.7. 1 %; 94.5; 96.3; 97.6. 0.1 %; 94.0; 95.9; 97.4. Observation: Supporting sufficient number of sub-groups enables to obtain comparable power saving gains with random subgrouping as with deterministic sub-group assignment. |
| Xiaomi |  |
| DOCOMO |  |
| InterDigital |  |
| Lenovo |  |
| Ericsson |  |
| Nordic |  |

Since the suggested maximum subgroups number per PO is 8, we next discuss and decide how to carry subgroups indication with PEI and/or paging DCI. From companies’ inputs, there are the following possible indication methods:

| PEI candidate | PEI indication | Paging DCI indication | Companies suggesting the subgroups indication design with a given PEI candidate |
| --- | --- | --- | --- |
| PDCCH-based PEI | All associated subgroups to one PEI | N/A | 7 companies: ZTE, HW, vivo, CATT, QC, MTK, Nokia |
| TRS/CSI-RS based PEI | PO paging indication | Subgroups in a PO | 3 companies: MTK, ZTE (if lower resource overhead prioritized), HW (if lower resource overhead prioritized) |
|  | All associated subgroups to one PEI | N/A | 6 companies: CATT, QC, Intel, Samsung, ZTE (if lower UE power prioritized), HW (if lower UE power prioritized) |
| SSS-based PEI | PO paging indication | Subgroups in a PO | 3 companies: MTK, ZTE (if lower resource overhead prioritized), HW (if lower resource overhead prioritized) |
|  | All associated subgroups to one PEI | N/A | 3 companies: QC, ZTE (if lower UE power prioritized), HW (if lower UE power prioritized) |

By the above, carrying subgroups indication only in PEI looks a good way forward, as LTE UE-sub-grouping enhancement which is based on idle-mode WUS. The following proposal is therefore suggested:

FL-[Q3.2-1]

Proposal:

- For Rel-17 UE subgrouping for paging enhancement, subgroups indication is by PEI only.

- Subgroups indication is not provided by physical layer if PEI is not configured.

For capturing observations related to power saving and/or resource overhead with UE sub-grouping, we suggest to do it after phase 1 discussions and decisions.

# Further Specification for PEI candidates based on PDCCH, TRS/CSI-RS and SSS

In Table 8, companies’ inputs on further design details are collected:

Table 8: Companies’ inputs on further design details

| Company | Company Views |
| --- | --- |
| ZTE, Sanechips | - Proposal 5: The system information change and availability indication of periodic TRS can be conveyed by PEI. - Proposal 7: Associating a PEI with multiple POs should be supported. - Observation 29: It needs to be discussed whether the number of POs associated with one PEI is fixed in specification or configurable. Figure 10 Bit structure of DCI-based PEI - Proposal 8: For DCI-based PEI, bitmap can be used to indicate the sub-grouping information. - Observation 30: Multi-beam can be considered for PEI, and the QCL information of the PEI can be associated with a SSB. - Proposal 9: The PEI reception window is used to determine the PEI occasion, and a time offset should be configured between the start of PEI reception window and the associated PO. Figure 11 Mapping between PEI and POs |
| Huawei, HiSilicon | - PEI locates behind the nearest SS burst before PO. - PEI locates behind the third nearest SS burst before PO which can save two SS burst reception for bad coverage UEs - PEI locates behind the third nearest SS burst before PO which introduces a short time deep/light sleep and power state transition for good coverage UE - Multiple PEI transmissions for one PO Figure 5. Idle mode UE procedures with respect to different PEI locations. Observation 20: Considering there are different UEs in a cell, there can be multiple PEI occasions indicating the same PO. Figure 6. Monitoring occasion for PEI. Observation 21: A monitoring window and a small offset between the SS burst and the monitoring window can be specified for the PEI design. Figure 7 Reuse DCI format 2_6 for PEI transmission Observation 22: DCI format 2_6 with appropriate modification can be used to transmit PEI for idle/inactive mode UEs. |
| vivo | Figure 6: The configuration for PEI occasion. Proposal 7: The configuration of PEI occasion should satisfy that the gap between PEI and the first indicated PO contains M SSB bursts, where the value of M can be 1, 2, 3 etc. Observation 14: TRS/CSI-RS availability indication through PEI is not unified solution since PEI and TRS/CSI-RS for idle/inactive UEs are decoupled features for UE power saving. Observation 15: Feasibility of TRS/CSI-RS availability indication through PEI also depends on the signal/channel design of PEI, and it can be discussed after the details are settled. |
| Spreadtrum | Proposal 3: If DCI-based PEI is supported in R17 with Behv-A, new DCI format is a starting point. Proposal 4: If DCI-based PEI is supported in R17 with Behv-B, extending DCI format 2_6 is a starting point. |
| CATT | Figure 9: TRS-based PEI with Multi-segment orthogonal cover Proposal 4: The number of code points generated from multi-segment orthogonal cover is up to 222 to indicate either or all paging subgroup/subgroup combination, paging occasions/occasions combination, and TRS/CSI-RS resource availability indication with the same detection performance. |
| CMCC | Proposal 5. For UE-ID based subgrouping, the subgroup index for one UE can be calculated as floor[UE_ID/(N*Ns)] mod n, where n is the number of subgroups in one PO. Figure 2. Potential PDCCH based PEI time domain resource allocations Figure 3. PDCCH based PEI time domain resource allocations when SSB and PEI are in the same slots Proposal 7. PDCCH-based PEI can be behind or within the slots of first SS burst. Proposal 8. A new search space can be defined for PDCCH-based PEI monitoring. |
| Qualcomm |  |
| OPPO | Proposal 3: Reuse the existing DCI format or specify a new DCI format for paging early indication, if DCI-based indication is considered. Figure 8 PEI monitoring time before PO Proposal 5: Time offset parameters are configured for UE to determine a time duration before target PO where the UE starts and stop monitoring PDCCH for detecting a PEI. |
| Intel | Specification Impact TRS based PEI design can be based on Rel-15 legacy TRS pattern, whereas PDCCH based PEI will likely require introduction of a new DCI format. |
| Apple | - If we consider the support of sub-grouping, the idea is that we can have a high group paging rate but a much lower sub-group paging rate, so that each UE belonging to a certain sub-group needs to wake up only when the sub-group is paged. However, if a PO has a mix of legacy and new UEs, even though the new UEs has a reduced sub-group paging rate, the legacy UEs still need to receive the paging PDSCH according to the much higher group paging rate, which results in more power consumption for the legacy UEs. If we allow the legacy and new UEs to be separated, the sub-grouping feature can be fully utilized for the new UEs, without any negative impact on the legacy UEs. - If a PO has legacy UEs only, there is no need for a network to transmit PEI. This is in contrast to the case where each PO has a mix of legacy and new UEs, and PEI always needs to be transmitted. This further reduces the unnecessary overhead. This can be achieved by providing separate parameters for the number of paging frames and offset for the new UEs. Proposal 3: Consider the support of separate PO configurations for legacy UEs and new UEs. |
| Sony | Observation 7 - There is a certain probability that PEI transmission may potentially collide with DL transmission to a legacy UE or DL transmission of existing signals/channels to other UEs. Proposal 5 – A window for PEI transmission/reception prior to PO is supported to avoid any blocking when other DL transmissions coincide with PEI. Proposal 6 – Signaling aspects on conveying the configuration of the PEI transmission/reception window and UE/network behavior on PEI reception/transmission are FFS. |
| Samsung |  |
| MediaTek | Proposal 12: PEI occasion determination is based on - Broadcast of periodic resources for potential PEI occasions - PEI occasion as the nearest period of resources before a specified time gap from UE’s PO Proposal 13: UE monitors one PEI occasion for its assigned PO and subgroup. Whether UE needs to monitor PO is according to Behv-A/B configured or specified. Figure 3: Illustration of PEI occasion association Proposal 14: For PDCCH-based PEI, the following indications are additionally supported: - SI update and ETWS indication - TRS availability indication - Note: UE is expected to support all idle-mode features since gNB cannot distinguish different UEs |
| LG | Observation 3: Once the SI change indication is transmitted, repetitions of SI change indication may occur within preceding modification period. Observation 4: Informing short messages over the PEI avoids unnecessary UE wake ups at PO. Proposal 1: PEI should at least convey the information on UE sub-group indication and short message – FFS: TRS/CSI-RS availability indication and/or UE group indication via PEI – FFS: UE sub-group indication at a PO with same slot scheduling Observation 5: A PEI occasion may be consist of multiple monitoring occasions to support multi-beam operation. Observation 6: When one PEI is associated with multiple POs, using the first PO within the SS burst period as a reference point for the PEI occasion would be beneficial from power saving perspective. Proposal 4: Introduce a new compact DCI format that conveys PEI information with smaller DCI bits than paging DCI. - PEI monitoring occasion can be configured with the offset to the PO |
| Panasonic |  |
| Nokia | Proposal: Network flexibility to choose in which cells/beams paging is sent, should be maintained and applied also to EPI. Proposal: Network should be able to configure the EPI to only sub-set of SSB/‘broadcast’ beams. Proposal: A single EPI should be able to address multiple POs to reduce EPI (PDCCH) indication overhead. Proposal: The monitoring occasions defined for PDCCH-EPI are defined by search space configuration. The paging search space (‘pagingSearchSpace’) configuration could be re-used for EPI. Proposal: Define the reference location for EPI monitoring, EPI frame (EPI-F), based on offset to PF. Offset could be defined in radio frames. Proposal: Define a PO specific offset (EPI-O) in relation to EPI monitoring reference location (EPI-F). Offset could be defined in symbols. Proposal: Determine the valid PDCCH-EPI monitoring occasions from the search space configuration (e.g. ‘pagingSearchSpace’) based on monitoring occasion timing indicated by EPI-F and EPI-O and number of actually transmitted SSBs. Proposal: To enable/disable broadcast beam specific PEI, bit map could be used to indicate the SSBs to which the EPI is active. Proposal: The PDCCH-PEI configuration needs to provide for each PO the location of the sub-grouping field. The size of the sub-grouping field could be common for all EPIs in the cell. Proposal: The TRS occasion availability indication in PDCCH-EPI can be configured in cell specific manner, providing the location and size of the information field. Proposal: Neither Short Message Indicator nor Short Message are included to EPI. |
| Xiaomi | Proposal 2: DCI-based PEI can be carried in paging search space. For target UE group A’s PO for paging, its PEI can be located in another UE group B’s PO which is earlier in time domain. |
| DOCOMO | Observation 1: If UE needs to monitor PO for SI change and/or ETWS in the case of configuring PEI, it causes additional UE power consumption. Proposal 1: It should be considered that ETWS and SI update is indicated via PEI. Observation 2: Availability indication of TRS/CSI-RS for idle/inactive mode UE to acquire ACG and synchronization can be informed by DCI-based PEI. Proposal 4: The following candidates can be considered as the information notified by PEI: - Whether or not UE wakes up at PO - Subgroup Information - Availability of TRS/CSI-RS for idle/inactive mode - legacy indication (ETWS, SI update) |
| InterDigital |  |
| Lenovo | Proposal 1: Support repetition of PEI with multiple beams, where each PEI occasion is QCLed with one SSB of transmitted SSBs. Proposal 2: A non-zero gap between the PEI and the corresponding PO or MO is configured for sequence based NR PEI. Proposal 3: For PDCCH based PEI, Paging Power Saving (PPS)-PDCCH search space configuration can be signaled in SIB1 or in an RRC release message. Proposal 4: A DCI size for PDCCH based PEI is limited up to 120-140 bits to guarantee similar coverage and detection performance as DCI format 2_6. Proposal 5: A PDCCH carrying PEI is intended to a group of UEs associated with a set of paging frames. A size of the set of paging frames may be dependent on selected paging configuration parameter values. |
| Ericsson | - Observation 7 Use of reserved bits in paging DCI (as a PDCCH-PEI) in one PO as paging early indication for UEs in one or more groups in other POs can further reduce PEI signaling overhead. - Proposal 5 In order to facilitate flexible content in PEI, the number of information bits conveyed by PEI is configurable between 1 and a maximum value PEImax (FFS on PEImax). - Proposal 6 For the PEI DCI, the RNTI used for CRC masking is configured via higher layers. - Observation 8 As PEI is only monitored in RRC Idle/inactive states where only fallback DCI is used, a DCI based PEI does not impact DCI size budget of up to 4 for a cell. - Observation 9 PEI transmissions should not be restricted to be in conjunction/adjacent to other transmission. - Proposal 9 RAN1 to discuss UE behavior w.r.t. PO PDCCH monitoring (e.g. to acquire ETWS/SI updates) even when UE determines that PEI indicates no paging in corresponding PO. - Observation 12 Irrespective of PEI format, from the UE power consumption perspective, at average 10% paging rate, the UE can at 90% of the time in idle mode immediately go back to deep sleep after PEI decoding regardless of PEI location with respect to PO. - Proposal 10 Search space for PEI PDCCH monitoring can be configured separately from or can be same as one of the existing search spaces configured for PDCCH monitoring in Idle/inactive. - Proposal 11 PO-specific configuration of the PEI includes an offset from PO ranging at least up to 3 SSBs prior to PO and includes a window of PEI monitoring occasions during which the UE searches for PEI. - Proposal 12 PEI design should allow the use of reserved bits in paging DCI in one PO as paging early indication for UEs in one or more groups in other POs. |
| Nordic | Proposal-3: If TRS are validated in PEI, a UE expects that TRS are present between PO and PEI in time domain. Observation-8: There is always 6bits reserved in a paging DCI. Furthermore, in case - a Short message is not transmitted in the DCI, 8 more bits become reserved. - a paging PDSCH is not scheduled by the DCI, 21 more bits become reserved. Proposal-4: A paging PDCCH for group N contains in addition to R16 content for group M: - One bit to indicate availability/unavailability of TRS for UE-group N, - up to at least 8 sub-groups of UE-group N using 4 or 8 (when available) bits to indicate which sub-groups of group N are to wake up for upcoming PO - where location of paging occasion of UE group M is [x] frame before PO of group N. Observation-9: By leveraging reserved bits in Paging DCI, and assuming that gNB must be prepared for transmitting one DCI per PO, - PDCCH blocking is not increased. - UE needs to support only single DCI format in Idle mode. |

From companies’ inputs, the following topics can be discussed and decided in this meeting:

- Determination of PEI monitoring occasions (MOs)

- Whether to include additional indications in PEI

- Collection of remaining details for further decisions:

  - PDCCH-based PEI: DCI format design alternatives

  - TRS-based PEI: Sequence generation alternatives

  - SSS-based PEI: New multi-symbol format, sequence generation, avoidance of impact to neighbor cell initial access, etc.

For the first topic, it is commonly expected PEI MO(s) should be earlier than the associated PO by a time gap. Also “how near” to a SS burst is one important design consideration. This effectively brings the concept of “monitoring window”. Consequently, the following proposal is suggested:

FL-[Q4-1]

Proposal:

UE determination of PEI monitoring occasion(s) (MO(s)) for a target PO is specified by both of the following:

- Possible PEI MOs broadcasted by gNB via system information

- A monitoring time window before the target PO

  - FFS: Description on the time window, e.g., including minimum time gap between PEI and the target PO and whether and how to specify time offset from a SS burst.

For the second topic, additional indication looks a unique characteristics of PDCCH-based PEI where some spared payload bits (e.g., 4 spared bits in PDCCH-based PEI with 12-bit payload to indicate 8 subgroups in a PO). In particular, TRS availability indication and SI update/ETWS indication are of support, and the following proposals are suggested:

FL-[Q4-2]

Proposal:

If PDCCH-based PEI is selected for Rel-17 paging enhancement, the following additional indications are supported:

- TRS information for idle/inactive-mode UEs

  - Up to [2] bits and indication detail should be designed in Agenda Item 8.7.1.2

- SI update indication and/or ETWS indication

  - Up to [2] bits and FFS detailed content design

For the 3rd topic, we can separate more specific proposals for different PEI physical-layer candidates:

FL-[Q4-3]

Proposal:

If PDCCH-based PEI is selected for Rel-17 paging enhancement, DCI payload design is based on

- Alt 1: Extending DCI format 2_6 with PS_RNTI

  - Note: Coexistence with connected-mode UE utilizing DCI format 2_6 should be ensured

- Alt 2: Extending DCI format 1_0 with P_RNTI for paging early indication

- Alt 3: Create a new DCI format with P-RNTI for Rel-17 paging enhancement

FL-[Q4-4]

Proposal:

If TRS/CSI-based PEI or SSS-based PEI is selected for Rel-17 paging enhancement, FFS sequence generation with P-RNTI and subgroups indication, subject to no impact to legacy serving-cell and neighbour-cell UEs.

# Summary

In Table 9, companies’ views on overall recommendation are collected:

Table 9: Companies’ views on overall recommendation

| Company | Company Views |
| --- | --- |
| ZTE, Sanechips | - Observation 6: DCI-based PEI with AL adaptation provides more flexibility and better coverage. - Observation 28: Compared with sequence-based PEI, DCI-based PEI has less workload. Table 12 Comparison between DCI-based PEI and SSS/TRS-like PEI DCI-based PEI; SSS/TRS-like PEI. Performance; Better performance and coverage. - Performance can be further improved with ML decoding; SSS-like PEI based on legacy pattern CANNOT meet the performance requirement; Two-symbol TRS-like PEI meet the performance requirement of PDSCH with TB scaling 1;. Power saving gain; Almost the same.. Resource overhead; DCI-based PEI occupies less resources for both Behv-A and Behv-B.. Flexibility /capacity; High flexibility. DCI-based PEI can flexibly adapt to different number of sub-groups, POs, and paging PDSCH TBS scaling factors.; Low flexibility. Carrying information corresponding to multiple POs and sub-groups at the resource overhead. System impact; 1)Capable of carrying more information, such as, - SI change and ETWS, - TRS availability indication. 2) No impact on the existing signals and channels. 3) Smaller standardization workload.; 1) Limited information can be carried if the resource load is not increased. 2) Impact on the functionalities of existing signals, e.g., SSS. 3) Heavier standardization workload, such as, - CDM, TDM, or FDM, if sub-grouping and multiple POs are supported - sequence generation, - resource mapping, - Co-existence with legacy signals/channels. - Proposal 6: Adopt DCI-based PEI to reduce the paging reception for RRC idle/inactive state UE. |
| Huawei, HiSilicon | Proposal 2: Adopt DCI carried by PDCCH as the physical layer channel for PEI indication in Rel-17. |
| vivo | Table 7: Summary of different PEI candidate designs PEI designs Metrics; SSS-like sequence-based PEI; TRS-like sequence-based PEI; DCI-based PEI. Performance (Alt1-behavior A); SSS 1 symbol; TRS 1 symbol; PDCCH AL 8 (40bits); PDCCH AL 4 (12 or 40 bits). PDCCH AL 8 (12bits). SSS 2, 4 symbols; TRS 2, 4 symbols; PDCCH AL 16 (12 or 40 bits). Performance (Alt2-behavior A); SSS 1 symbol; TRS 1 symbol; PDCCH AL 16 (40bits) PDCCH AL 8 (12 or 40bits); PDCCH AL 4 (12 or 40 bits). SSS 2 symbols; TRS 2 symbols; PDCCH AL 16 (12bits). SSS 4 symbols; TRS 4 symbols. Resource overhead (behavior A); 25.4/50.8 for 2/4 symbols for 1 PO to 1 PEI mapping; 28.8/57.6 for 2/4 symbols for 1 PO to 1 PEI mapping; 57.6/115.2 for AL 8/16 for 1 PO to 1 PEI mapping;. 54.72/109.4 for AL 8/16 for 2 POs to 1 PEI mapping. Coexistence; Semi-static or dynamic PDSCH occupation; Semi-static or dynamic PDSCH occupation; Semi-static or dynamic PDCCH occupation. Proposal 6: Sequence (i.e., SSS-like or TRS-like) should be adopted for PEI design. |
| Spreadtrum | Proposal 2: If two PEI candidate schemes should be down selected, DCI-based PEI and SSS-based PEI are selected. Proposal 3: DCI-based PEI is supported in R17. |
| CATT |  |
| CMCC | Table 1. Comparison of three PEI candidate designs PDCCH-based; SSS-based; TRS/CSI-RS-based. Coexistence complexity; Easy, UE’s mandatory capability to support both semi-static and dynamic rate matching; Harder, gNB should configure a fake CORESET for dynamic rate matching; Easy, UE’s mandatory capability to support both semi-static and dynamic rate matching. UE (sub)group indication capacity; Largest; Small; Small. Average resource overhead (Behv-A); AL=4: Smallest AL=8: Larger than others but comparable; Medium; Medium. Average resource overhead (Behv-B); AL=4: Smallest AL=8: Larger than others but comparable; Large; Large. Proposal 1. PDCCH-based PEI is preferred. |
| Qualcomm | Proposal 4: Rel-17 PEI design is based on narrowband sequence such as the SSS - Availability of TRS/CSI-RS at configured occasion(s) is indicated by paging PDCCH - How paging PDCCH and PDSCH is transmitted follows legacy rules but is not impacted by PEI. |
| OPPO | Observation 2: The SSS or TRS/CSI-RS based PEI have the cons of higher resource overhead, higher complexity and less UE sub-group information carriage. Proposal 1: DCI-based PEI is preferred for paging early indication. |
| Intel | Overall comparison To summarize the above discussion, in Table 2, we compare the overall comparison between TRS-like sequence-based PEI and PDCCH based PEI based on MDR performance, resource OH, coexistence considerations, and specification efforts, under the agreed evaluation methodology and assumptions. Table 2: Comparison of PEI designs PEI designs Metrics; TRS-like sequence-based PEI; DCI-based PEI. MDR Performance; Winner. Resource overhead; Winner. Coexistence; Winner. Spec effort; Tie; Tie. Proposal 3: Support sequence-based PEI (TRS/CSI-RS or SSS based) in Rel-17 for idle/inactive mode UE. |
| Apple | PDCCH-based solution has advantages in some other aspects: - Relatively speaking, it has less specification impact, and less gNB/UE implementation impact. - It is easier to use PDCCH to carry other information, such as sub-grouping information, the TRS/CSI-RS availability indication. Proposal 1: Use PDCCH to carry paging early indication. |
| Sony | Observation 1 – The cost of transition from/to deep sleep as well as synchronization cost are dominant sources of power/energy consumption. Proposal 1 – Paging enhancement schemes should avoid the UE unnecessarily transitioning from/to sleep states and from/to synchronization states, when there is no paging for a target UE. Proposal 2 – Use sequence-based early paging indication as a paging enhancement scheme to reduce idle PO monitoring cost at the UE. |
| Samsung |  |
| MediaTek | Proposal 11: PDCCH-based PEI is adopted for the best trade-off in power saving gain and average resource overhead per PO, with and without UE subgrouping enhancement Table 7: Initial specification for PEI candidate designs PDCCH-based PEI; TRS/CSI-RS based PEI; SSS-based PEI. Channel/signal specification; Reuse Rel-15 PDCCH FFS: DCI format and content; Reuse Rel-15 TRS/CSI-RS FFS: Modification on sequence generation, e.g., incorporating P-RNTI; New multi-symbol SSS format needs to be defined FFS: Sequence generation. Support of Behv-A and/or Behv-B; gNB configuration on either Behv-A or Behv-B; Behv-A only; Behv-A only. Mapping to PO(s) and UE subgroup(s); - 1 PEI indicates all UE subgroups in a PO as well as multiple POs per available payload bit number. - One bit per UE (sub)group as baseline - PEI-PO mapping is specified by PEI occasion for a PO - Subgroup indication is not supported if PEI is not configured; - 1 PEI indicates 1 PO - Subgroup indication by paging DCI - FFS how to utilize reserved bits - Subgroup indication is not supported if PEI is not configured; - 1 PEI indicates 1 PO - Subgroup indication by paging DCI - FFS how to utilize reserved bits - Subgroup indication is not supported if PEI is not configured. PEI occasion; Periodic resources for potential PEI occasion is specified by CORESET 0 and a broadcasted search space set setting. - Start of each period should be close and at/after a SS burst - FFS: How close to a SS burst For a PO, PEI occasion corresponds to the nearest period of resources subject to a non-negative time gap before the start of the PO - FFS: Time gap range; Periodic resources for potential PEI occasion is specified by a set of periodic TRS patterns. - Start of each period should be close and at/after a SS burst - FFS: How close to a SS burst For a PO, PEI occasion corresponds to the nearest period of resource subject to a non-negative time gap before the start of the PO - FFS: Time gap range; Periodic resources for potential PEI occasion is specified by a set of periodic RB-symbol patterns. - Start of each period should be close and at/after a SS burst - FFS: How close to a SS burst For a PO, PEI occasion corresponds to the nearest period of resource subject to a non-negative time gap before the start of the PO - FFS: Time gap range. UE procedure with PEI; UE monitors one PEI occasion for its assigned PO and subgroup. Whether UE needs to monitor PO is according to Behv-A/B configured.; UE monitors one PEI occasion for its assigned PO and subgroup. Whether UE needs to monitor PO follows Behv-A.; UE monitors one PEI occasion for its assigned PO and subgroup. Whether UE needs to monitor PO follows Behv-A.. Additional indication; - SI update and ETWS indication - TRS availability indication (also related to TRS agenda); No; No. |
| LG | Proposal 3: Conveying PEI information using PDCCH is supported |
| Panasonic | Proposal 7: To agree on functionalities of PEI and the number of bits supported by PEI, before agreeing on PDCCH, SSS or TRS/CSI-RS based design. |
| Nokia | Observation: PDCCH based EPI offers the best flexibility in terms of information payload support. Observation: In terms of specification RAN1 effort different EPI designs are somewhat comparable, while PDCCH based design would be most straight forward. Observation: For TRS-EPI and SSS-EPI some receiver changes may be needed, depending on the final EPI design. Use of TRS-EPI for time/frequency synchronisation for paging reception or in general would not be feasible in all cases and in all behaviours. Observation: For TRS-EPI there would be a need for new performance requirements for missed detection. For SSS-EPI new requirements maybe needed as current cell detection requirements cover joint detection of PSS and SSS. For PDCCH-EPI existing requirements can be re-used, similarly as for paging. Proposal: Base the EPI design on PDCCH/DCI. |
| Xiaomi | The observations agreed in last RAN1 meeting are similar as the above observations, so we think at least SSS-based PEI has more impact than the others from the coexistence with legacy PDSCH and UE aspects, and it carried the least information regardless of the Behv-A or B as in the second observation. Proposal 1: Based on the above observations, DCI-based PEI and TRS/CSI-RS based PEI can be for further discussion based on each physical layer design details. We prefer the DCI-based PEI Proposal 4: Cross-slot scheduling with extended k0 is not supported. |
| DOCOMO | Table1．PEI design characteristic PEI Design; Advantages; Disadvantages. DCI-based; - Low false alarm due to CRC - Information other than Wake UP notifications can be easily notified; - sensitive to synchronization error. RS/Sequence-based; - Not sensitive to synchronization error; - Trade-off between false alarm rate and miss detection rate. As explained in section 2.1-2.4, DCI-based PEI is better from several points of view and should be supported. Compared with RS/sequence-based PEI, DCI-based PEI has less system impact and can provide more benefits by indicating additional information. Proposal 3: DCI-based PEI should be supported. |
| InterDigital | From the above results and discussion, we can conclude that TRS and 2-sybmol SSS offer better MDR than PDCCH AL 8 with almost the same overhead. Although the capacity of PDCCH is larger, this capacity is not needed for common paging rates. In addition, detection of sequence-based PEI is more power efficient, and sequences can also be used for other purposes such as AGC, time/frequency offset correction, providing further power saving gains. Therefore, we propose the following. Proposal 1: Sequence-based paging early indication is adopted for Rel-17. |
| Lenovo |  |
| Ericsson | If a DCI-based design is used for PEI, compared to the sequence-based counterpart, it: - readily supports link adaptation with flexible payload size support and resource usage, - is easy to support and configure multiple contents, - has lower standardization effort (e.g. reuse existing WUS and PDCCH framework, including the ps-Wakeup functionality to support both BehvA and BehvB functionalities (agreed in RAN1#104-e meeting)), - has less impact on UE and the NW both in terms of power, complexity, and consumed resources when multiple groups are addressed simultaneously and/or including various information elements, - has minimum impact on other ongoing/legacy user data traffic as PDCCH is easily multiplexed and/or rate-matched around (discussed further below), - can readily support flexible one PEI-to-many PO mapping - is possible to extend in the future. - Observation 2 A DCI-based PEI, compared to the sequence-based, can conveniently carry more information, is future extendable, brings configurable contents, has lower standardization effort. - Observation 3 A DCI-based PEI, compared to the sequence-based, has less impact on UE and the NW both in terms of power, complexity, and consumed resources when multiple groups are addressed simultaneously and/or including various information elements. |
| Nordic | Proposal-1: Support PDCCH as PEI channel. |

The final proposal is a down-selection proposal for online decision. We hope the above information is sufficient for every company to select the best PEI physical-layer design to move forward.

FL-[Q5-1]

Proposal:

The following is selected as PEI physical-layer design:

- [PDCCH-based PEI]

- [TRS-based PEI]

- [SSS-based PEI]

# Reference

- “Draft Report of 3GPP TSG RAN WG1 #104-bis-e v0.1.0”, MCC Support, RAN1 #104-bis-e, online available @ https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_104b-e/Report

- R1-2103848, “Summary of Paging Enhancements”, Moderator (MediaTek), RAN1 #104-bis-e

- R2-2104356, “Reply LS on UE Sub-grouping for Paging Enhancement,” RAN2, MediaTek, RAN2 #113-bis-e; as R1-2104229 for RAN1 #105-e

- R1-2009801, “LS on Paging Enhancement”, RAN1 to RAN2, RAN1 #103-e

- R1-2104221, “Discussion on power saving enhancements for paging	”, ZTE, Sanechips

- R1-2104251, “Paging enhancements for UE power saving in IDLE/inactive mode”, Huawei, HiSilicon

- R1-2104371, “Paging enhancements for idle/inactive mode UE power saving”, vivo

- R1-2104432, “Discussion on potential paging enhancements for UE Power Saving”, Spreadtrum Communications

- R1-2104532, “Paging enhancement for UE power saving”, CATT

- R1-2104622, “Discussion on paging early indication design”, CMCC

- R1-2104682, “Paging enhancements for idle/inactive UE power saving”, Qualcomm Incorporated

- R1-2104787, “Further discussion on Paging enhancements for power saving”, OPPO

- R1-2104916, “On Paging Enhancements for UE Power Saving”, Intel Corporation

- R1-2105116, “Paging early indication for idle/inactive-mode UE”, Apple

- R1-2105174, “Considerations on potential paging enhancements”, Sony

- R1-2105321, “Discussion on paging enhancements”, Samsung

- R1-2105386, “On Paging Enhancements for Idle/Inactive Mode UE Power Saving”, MediaTek Inc.

- R1-2105434, “Discussion on potential paging enhancements”, LG Electronics

- R1-2105474, “On paging enhancement”, Panasonic

- R1-2105504, “On paging enhancements for UE power saving”, Nokia, Nokia Shanghai Bell

- R1-2105573, “Paging enhancements for power saving”, Xiaomi

- R1-2105708, “Discussion on paging enhancements”, NTT DOCOMO, INC.

- R1-2105742, “Paging enhancements for UE power saving”, InterDigital, Inc.

- R1-2105770, “Paging enhancement for UE power saving”, Lenovo, Motorola Mobility

- R1-2105791, “Design of Paging Enhancements”, Ericsson

- R1-2105886, “On  paging early indication”, Nordic Semiconductor ASA