3GPP TSG RAN WG1 Meeting #106-e	R1- 2106490

E-meeting, August 16th – 27th, 2021

Agenda Item:	8.3.1.1

Source:	Huawei, HiSilicon

Title:	UE feedback enhancements for HARQ-ACK

Document for:	Discussion and Decision

Introduction

This contribution provides our views on the details of HARQ-ACK feedback enhancements in Rel-17, including SPS HARQ-ACK dropping for TDD systems, Type 1 HARQ-ACK codebook for sub-slot configuration, PUCCH repetition over sub-slots, PUCCH carrier switching, and retransmission of cancelled HARQ-ACK.

Feedback enhancements for HARQ-ACK

2.1 Enhancements for HARQ-ACK dropping for DL SPS

In the RAN1#104b-e meeting, the following agreements about deferral of dropped SPS HARQ-ACK were achieved [1].

Agreements: For SPS HARQ-ACK deferral, for the determination of valid symbols in the target slot/sub-slot a collision with semi-static DL symbols, SSB and CORESET#0 is regarded as ‘invalid’ or ‘no symbols for UL transmission’. Agreements: For SPS HARQ-ACK deferral, support a limit on the maximum deferral of SPS HARQ in terms of k1def or k1+ k1def - FFS: limitation given by a maximum value of k1def or a maximum of k1eff =k1+ k1def - FFS how the limitation is determined (e.g. by K1 set(s) or RRC configured limit) Agreements: For SPS HARQ-ACK deferral, there is no lower limit defined for k1def Agreement: For SPS HARQ-ACK deferral, the limit on the maximum deferral of SPS HARQ is defined in terms of k1+ k1def Agreement: Restrict the further discussions on the initial slot handling for SPS HARQ-ACK deferral to the identified alternatives Alt. 1, Alt. 1A and 2. Agreement: For SPS HARQ-ACK deferral, the initial HARQ-ACK transmission occasion is considered to determine the out-of-order HARQ condition

Maximum deferral value for SPS HARQ-ACK deferral

As shown in the agreement, the first open issue is how to define the limitation on the maximum deferral of SPS HARQ-ACK. It was agreed that the limitation on the maximum deferral of SPS HARQ is defined in terms of k1+k1def  which represents the slot number between the slot of SPS PDSCH reception and the slot where the first available PUCCH resource is determined. Therefore the definition of k1+k1def  is similar as the definition of k1 value configured in dl-DataToUL-ACK. Thus, the maximum k1 value configured in dl-DataToUL-ACK can be reused as the maximum limitation for SPS HARQ-ACK deferral. There is also a benefit for reusing the current HARQ codebook if k1+k1def is limited with the maximum K1 value, details of which is described in Section 2.1.3.

Proposal 1: The maximum deferral k1+ k1def of the SPS HARQ-ACK should be the maximum k1 value in dl-DataToUL-ACK.

SPS HARQ-ACK deferral condition

The second open issue is the SPS deferral condition. In the previous meeting, there were 3 candidate options for the deferral conditions from/within the initial slot:

- Alt. 1:  “If SPS HARQ-ACK is multiplexed with any other UCI / dynamic PUCCH resource then it cannot be deferred!”

- Alt. 1A “Defer SPS HARQ even if multiplexing & transmission based on PRI in initial slot would be possible”

- Alt. 2 – “Consider intra-slot deferral before inter-slot deferral”

In the following we would analyze the three alternatives in the perspectives of latency, reliability, and standard efforts.

In terms of latency, Alt.1 and Alt.2 can improve the latency by making use of earlier PUCCH resources other than the SPS PUCCH in a later slot, while the time slot of the postponed SPS HARQ-ACK for Alt.1A would be fixed depending on the DL/UL configurations.

In terms of reliability, Alt. 1A is more robust to be immune to the case of DCI missing as compared to Alt.1. As shown in the following Figure 1, the DL Slot#1 is not available for SPS#0 HARQ-ACK due to the collision with DL, and the SPS#1 HARQ-ACK is supposed to be transmitted on the UL Slot#2. The gNB sends a DG DCI scheduling a DG PUCCH#1 at Slot#1. Under Alt.1, the gNB expects the UE to multiplex the DG HARQ-ACK and SPS#0 HARQ-ACK at PUCCH#1, and expects to receive 1 bit SPS#1 HARQ-ACK (BPSK) at PUCCH#2. If the UE misses the DG DCI, it will postpone the SPS#0 HARQ-ACK to Slot#2 and multiplex it with SPS#1 HARQ-ACK on the SPS PUCCH resource, PUCCH#2, with the payload of 2 bits (QPSK). This mismatch will result in the incorrect reception at the gNB. Under Alt.1A, in contrast, the UE will always postpone to Slot#2 regardless of the DG DCI so that the mismatch can be avoided.

Figure 1 DCI missing issue for Alt.1.

In terms of standard efforts, both Alt.1 and Alt.1A basically reuse the PUCCH resource determination principle. Alt.2, on the other hand, needs more discussions on the detailed rule of determining the intra-slot PUCCH resources for deferral. E.g., whether the extra intra-slot PUCCH is extra configured, or selected from the DG/SPS PUCCH set; whether and how it is needed to ensure the predefined/configured intra-slot PUCCH resource will not conflict with DL symbols, etc. In addition, if the target PUCCH is determined based on a semi-statically predefined/configured PUCCH, this PUCCH format may not be suitable to guarantee the SPS HARQ-ACK latency/reliability. To alleviate the efforts of standard discussion, we recommend not to further discuss Alt.2 and to adopt Alt.1 or Alt.1A as the deferral method.

Proposal 2：Down select between Alt.1 and Alt.1A as the SPS deferral condition:

- Alt. 1:  “If SPS HARQ-ACK is multiplexed with any other UCI / dynamic PUCCH resource then it cannot be deferred!”

- Alt. 1A “Defer SPS HARQ even if multiplexing & transmission based on PRI in initial slot would be possible”.

SPS HARQ-ACK Codebook construction

For codebook construction for the deferred SPS HARQ-CAK in the target slot, there could be three cases e.g., SPS only codebook (CB), Type 1 CB and Type 2 CB. For SPS only CB and Type 2 CB, it is simple to treat the deferred SPS HARQ-ACK as a normal SPS HARQ-ACK, i.e., the Rel-16 rules can be reused to get the position in the CB for deferred SPS HARQ-ACK(s). For Type 1 CB, in our understanding, deferred SPS HARQ-ACK with the eventual k1+ k1def included in the K1 set can be mapped in the Type 1 CB as specified by the legacy spec; for the deferred SPS HARQ-ACK(s) with the eventual k1+ k1def not included in the K1 set, it makes sense to append them at the end of the Type 1 CB.

Proposal 3: For codebook construction for deferred SPS HARQ-ACK in the target slot,

- For Type 1 CB, deferred SPS HARQ-ACK with k1+ k1def included in the K1 set can be mapped in the Type 1 CB following the Rel-15/Rel-16 rule, while those with k1+ k1def not included in the K1 set can be appended.

- For SPS only CB and type 2 CB, Rel-16 rules can be reused.

Moreover, there is a concern raised in RAN1#104b-e that for a candidate slot/sub-slot, there could be deferred SPS HARQ-ACKs taking it as a candidate target slot, and also a non-deferral SPS HARQ-ACK taking it as an initial slot/sub-slot. The slot may be available for the non-deferral SPS HARQ-ACK only which corresponds to a small PUCCH resource located within the available UL symbols, but may become unavailable after the deferred SPS HARQ-ACK(s) are multiplexed and thereby corresponding to a larger PUCCH resource which overlap with DL symbols. In our understanding, the procedure for checking the validity of a candidate target slot/sub-slot is first to multiplex all HARQ-ACKs including those originally pointed to this slot/sub-slot and those deferred to this slot/sub-slot, then to determine the PUCCH resource based on the total payload of the multiplexed HARQ-ACKs, and finally to check the validity by comparing whether the available UL symbols can hold the determined PUCCH. Therefore, the non-deferral SPS HARQ-ACK should be multiplexed with the deferred SPS HARQ-ACKs and keep deferring to the next slot/sub-slot instead of being transmitted standalone on the candidate slot/sub-slot for the above case.

Proposal 4: For a candidate target slot/sub-slot, if the resulting PUCCH resource is unavailable, both the SPS HARQ-ACK(s) which have been deferred to this slot/sub-slot and the SPS HARQ-ACK originally pointed to this slot/sub-slot should be deferred in together to the next candidate slot/sub-slot.

2.2 Type 1 HARQ-ACK codebook construction based on sub-slot configuration

In the 104b-e meeting, the following agreement was made for sub-slot based Type 1 HARQ-ACK codebook.

Agreement: Support Type-1 HARQ-ACK codebook for sub-slot based PUCCH configuration in Rel-17. - The properties of the Type-1 HARQ-ACK codebook for sub-slot PUCCH at least includes that a PDSCH TDRA is associated with a UL /PUCCH sub-slot if the end of the PDSCH overlaps with the associated sub-slot determined by a k1 in the set of sub-slot timing values K1. - FFS: whether the PDSCH TDRA grouping is performed per DL slot or sub-slot - Decide between PDSCH TDRA grouping per DL slot and sub-slot during RAN1#105-e

The remaining issue is how to perform the TDRA grouping. For the current Rel-15/Rel-16 specification, the slot based Type 1 CB construction consists of three steps:

- Step 1: For a UL slot where the UE has to transmit HARQ-ACK information, loop the PDSCH-to-HARQ-timing value k1 in timing set K1 to find the candidate DL slots which may be potentially associated with the UL slot with respect to k1.

- Step 2: For each candidate DL slot, prune the SLIVs that is conflict with the UL symbol regarding to the DL/UL configuration from the TDRA table.

- Step 3: For the rest of the SLIVs, perform the SLIV splitting to generate the TDRA groups, each group of which is associated with the HARQ-ACK bit field.

In Rel-17, there could be two alternatives for TDRA grouping of the sub-slot based Type 1 CB: sub-slot based grouping and slot based grouping. Sub-slot based TDRA grouping loops the K1 set in Step 1, removes the conflict SLIVs in Step 2, and performs the SLIV splitting in Step 3 per sub-slot level. This may incur redundancy of HARQ-ACK bit number in Step 3 if the TDRA table includes SLIVs crossing sub-slots.

As a comparison, slot based TDRA grouping loops the K1 set in Step 1 per slot level, prunes the SLIVs conflict with the UL symbol as well as the SLIVs in the DL sub-slot which is not associated with the UL sub-slot in Step 2 based on K1 set, and performs the SLIV splitting jointly per slot level in Step 3. As the SLIV splitting is performed jointly in Step 3, the redundancy due to the overlap of SLIVs crossing sub-slots can be avoided, and therefore the HARQ-ACK payload can be minimized.

As an example shown in Figure 2 below, assuming there are 8 SLIVs in the Slot#1 which is composed of 7 sub-slots each with 2OS length. The sub-slot level K1 set is configured as k1={2,3,5,6} subs-lots, thus the DL sub-slots associated with the UL sub-slot are sub-slot#2, sub-slot#3, sub-slot#5 and sub-slot#6, respectively, while sub-slot#1/#4/#7 are not associated with the UL sub-slot.

If per sub-slot based TDRA grouping is applied, 4-bit HARQ-ACK CB is generated with each bit respectively associated with group {#2} in sub-slot#2, group {#3} in sub-slot#3, group {#5} in sub-slot#5 and group {#6} in sub-slot#6. Considering only one PDSCH between the overlapping SLIV#3 and SLIV#4 or between SLIV#5 and SLIV#6 would be actually transmitted, reporting 4 bits results in redundancy.

On the other hand, if per slot based TDRA grouping is applied, SLIV#1/#4/#7/#8 in sub-slot#1/#4/#7 are pruned from the TDRA table in Step 2, and the remaining SLIV#2/#3/#5/#6 are jointly applied with the slot based TDRA grouping, resulting group {#2, #3} and group {#5, #6}. Thus only 2-bit HARQ-ACK CB is generated, saving 2 bits in contrast with per sub-slot based grouping.

Figure 2 Comparison between per slot grouping and per sub-slot grouping

From the perspective of standard impact, for per slot based grouping, the impact on Step 1 is to find the DL slot based on the UL sub-slot and the sub-slot level k1 value(s) in K1 set, where the DL slot consists at least one DL sub-slot associated with the UL sub-slot. The impact on Step 2 is, in the DL slot, pruning the SLIVs whose ending symbols overlap with the DL sub-slots that are not associated to the determined UL sub-slot based on K1 set. The impact on Step 3 is to split the SLIVs based on slot level. To this end, per slot based grouping for sub-slot type 1 CB does not change the principle of the legacy Type 1 CB generation procedure, and has limited changes on the specification.

From the perspective of reduced payload by slot based TDRA grouping, it could save up to 3 bits for per slot in contrast with sub-slot based TDRA grouping under the 2OS sub-slot pattern as shown in the following figure where the overlapping of SLIVs occurs in each sub-slot. Therefore, we think per slot based TDRA grouping is beneficial for HARQ-ACK payload reduction which could enhance the UL transmission reliability of the URLLC service.

Figure 3 First example for per slot grouping /per sub-slot grouping for 2OS sub-slot

Observation 1: For sub-slot based Type 1 codebook, per slot based grouping has limited changes for codebook generation procedure on top of the legacy specification.

Observation 2: For sub-slot based Type 1 codebook, per slot based grouping can reduce non-negligible bit number of HARQ-ACK feedback compared to per sub-slot based grouping.

Proposal 5：Support TDRA grouping performed per slot for sub-slot based Type 1 CB,

- Step 1: Determine DL slots consisting of DL sub-slots associated to the determined UL sub-slot

- Step 2: In each determined DL slot, prune the PDSCH SLIVs whose ending symbols overlap with DL sub-slots that not associated to the determined UL sub-slot based on K1 set.

- Step 3: Perform per slot SLIV splitting among the remaining SLIVs for each slot to generate the TDRA groups, each group of which is associated with the HARQ-ACK bit field.

2.3 PUCCH repetition over sub-slots

In RAN1#104-e, sub-slot based PUCCH repetition for HARQ-ACK was agreed. Also, dynamic indication of the repetition number is supported.

Agreements: Support sub-slot based PUCCH repetition for HARQ-ACK based on the Rel-16 PUCCH procedure for slot-based PUCCH applied to sub-slot based PUCCH - Note: the intention is to take the Rel-16 slot-based PUCCH by replacing with “sub-slot” appropriately, without further optimization unless necessary - FFS whether or not there is any restriction for the applicability of sub-slot based PUCCH repetition for HARQ-ACK - Dynamic repetition indication is supported also for sub-slot based PUCCH in Rel-17 - FFS: if the method to be specified in Cov. Enh WI for slot-based PUCCH repetition can be directly applied to sub-slot PUCCH or if changes are needed Agreements: Support PUCCH repetition for PUCCH formats 0 and 2 at least for sub-slot based PUCCH repetition. - FFS: Support for slot-based PUCCH repetition

In the Coverage Enhancements WI, two options for the repetition factor indication were agreed. In Option 1 the PUCCH repetition factor is indicated in DCI by reusing the legacy field. For this method, the PUCCH resource and the PUCCH repetition number are jointly encoded, and a legacy field such as PRI is utilized to indicate both information. While for Option 2, an independent PUCCH repetition factor is explicitly indicated by the DCI. In the RAN1#105 meeting, it has made a Working Assumption in Coverage Enhancements WI, in which the PUCCH repetition factor should be configured per PUCCH resource, and indicated by PRI, as it can provide more flexibility and reduce the DCI overhead. The Working Assumption should be confirmed as discussed in our companion paper [2].

Proposal 6: Confirm the WA in the Coverage Enhancements WI where the repetition factor is dynamically indicated by the PRI field, and the PUCCH repetition factor is configured per PUCCH resource.

Moreover, as the slot-based PUCCH repetition for Format 0 and Format 2 has been agreed to be supported in Multi-TRP, it would be better to align with the discussion in Multi-TRP.

Proposal 7: Support slot-based PUCCH repetition for PUCCH Format 0 and Format 2.

2.4 PUCCH carrier switching for HARQ-ACK feedback

It was agreed in RAN1#105-e that both dynamic indication and semi-static configured pattern are supported for PUCCH carrier switching [3]. Some details about these two method are discussed in this section.

Agreement: Support PUCCH carrier switching based on dynamic indication in DCI scheduling a PUCCH and semi-static configuration - Details are FFS (including applicability of dynamic and/or semi-static means) - Aim for minimum specification impact - Dynamic indication and/or semi-static configuration are subject to separate UE capabilities - The semi-static PUCCH carrier switching configuration operation is based on RRC configured PUCCH cell timing pattern of applicable PUCCH cells and supports PUCCH carrier switching across cells with different numerologies. - FFS whether additional rules are needed to support PUCCH carrier switching across cells with different numerologies - FFS the maximum number of PUCCH cells - FFS whether and how to support joint operation of dynamic and semi-static carrier switching for a UE - FFS whether and how to support joint operation of PUCCH carrier switching and SPS HARQ-ACK deferral Agreement: For PUCCH carrier switching, the PUCCH resource configuration is per UL BWP (i.e. per candidate cell and UL BWP of that specific candidate cell). Agreement: For PUCCH carrier switching based on dynamic indication in DCI scheduling a PUCCH (i.e. Alt. 1), the PDSCH to HARQ-ACK offset k1 is interpreted based on the numerology of the dynamically indicated target PUCCH cell.

PUCCH carrier switching based on dynamic indication

PUCCH carrier indication

For PUCCH carrier switching based on dynamic indication, the first open issue is how the dynamic indication is designed in the triggering DCI. There are three alternatives summarized in the FL summary which are legacy PRI, extended PRI and adding a new DCI field. For more flexibility, adding a new field is preferred which is also the simplest option among the alternatives.

For determining the target carrier, gNB could configure a carrier set consisting of the carrier index of candidate carriers for the PUCCH transmission. The total number of candidate carriers included in carrier set should be limited based on UE capability for supporting UL carriers.

Proposal 8: For dynamic indicating the target PUCCH carrier, consider to introduce a new field in DCI and a configured carrier set consisting of candidate carrier indices.

HARQ codebook construction

For the interpretation of the PDSCH to HARQ-ACK offset k1, in last meeting it was agreed that the PUCCH resources can be configured per UL BWP, thus individual K1 sets and individual PUCCH resources can be configured for per UL BWP. And it was also agreed that k1 is interpreted based on the numerology of the dynamically indicated target PUCCH carrier. Therefore, it is straightforward to also interpret the value of k1 based on the K1 set configured in the PUCCH-config of the indicated carrier.

Proposal 9: For interpreting the PDSCH to HARQ-ACK offset k1 under dynamic carrier indication, the K1 set configured for the indicated carrier should be used.

For codebook construction, we will describe our view as shown in Figure 4 below. Assume there are 4 DCIs scheduling 4 PDSCHs. Based on what we described above, the associated HARQ-ACK of each PDSCH will be transmitted on the indicated carrier according to the carrier indication in DCI. Then the PDSCH to HARQ-ACK k1 in each DCI will be interpreted on the K1 set configured for the indicated carrier. Therefore, the HARQ-ACKs of PDSCHs in red will be transmitted in Slot#3 on SCell#1 and these HARQ-ACKs will be constructed as a CB. Then the associated PUCCH resource will be determined following the PRI in the last received DCI among DCIs scheduling those PDSCHs in red. The same procedure will be applied to the PDSCHs in black as well to construct the CB in Slot#4 on PCell.

Figure. 4 Example for Codebook construction in dynamic carrier switching

Proposal 10: For the codebook construction in case of PUCCH carrier switching, HARQ-ACKs with same priority index pointed to the same slot/sub-slot on the same carrier will be constructed as one codebook.

Rule clarification for out of order

In Rel-15 the rule of OoO limits the timing order of the PUCCHs on the PCell. In particular, if the HARQ-ACK of an earlier PDSCH and the PUCCH of a later PDSCH are scheduled to be transmitted on the same slot/sub-slot, they will be constructed into one codebook, and such case will not be regarded as OoO. On the other hand, if they are in different slots and the PUCCH of the later PDSCH is earlier than the PUCCH of the earlier PDSCH then it is regarded as OoO and is not allowed in Rel-15/Rel-16.

For PUCCH carrier switching in Rel-17, the PUCCH can be transmitted on PCell or SCell, and the SCS of PCell may be different with SCell, so a new case may occur where the two PUCCHs belong to the same slot/sub-slot on PCell but belong to different slots/subslots on SCell or vice versa. Take Figure 5 as an example, PUCCH 1 and PUCCH 2 are in the same Slot#2 of PCell with 15kHz but in different Slot#4 and Slot#5, respectively, of SCell with 30 kHz. Therefore, it needs to identify the reference numerology/carrier for assessing OoO in Rel-17. E.g., if OoO rule is based on 15 kHz SCS, they are in the same slot and will not be considered as OoO; if OoO rule is based on 30 kHz SCS, they are in different slots and will be considered as OoO. In our understanding, OoO rule should be applied based on the larger SCS, so that Figure 5 should be viewed as an OoO scheduling which needs to be avoided.

Figure. 5 Example of OoO scheduling for different SCS

Proposal 11: For PUCCH carrier switching across different numerologies, the OoO rule between the carriers with PDSCH transmission and the carrier with PUCCH transmission should be applied based on the largest SCS.

PUCCH carrier switching based on RRC configured PUCCH carrier timing pattern

Reference timing

For PUCCH carrier switching based on a RRC configured PUCCH carrier timing pattern, the first open issue is what the reference numerology is for the time domain pattern. It should be considered to use the SCS of PCell as the reference numerology of the timing pattern. The granularity of the timing pattern for indicating the available carrier index for UL should be based on the numerology of PCell. In addition, the PDSCH to HARQ-ACK offset k1 in DCI should be interpreted also based on the K1 set configured for PCell. Based on above description, the length of the timing pattern naturally is aligned with the TDD time domain pattern of PCell.

Proposal 12: For configuration of the RRC configured PUCCH carrier timing pattern,

- The reference numerology is the SCS of PCell and the granularity of the timing pattern is based on the SCS of PCell.

- K1 interpretation is based on K1 set configured for PCell.

Rule for different numerologies among carriers

As shown in the agreement, another open issue for configured timing pattern is whether to support additional rules for PUCCH carrier switching, especially for the case of switching across carriers with different numerologies. From the perspective of flexibility, the SCS of candidate carriers for PUCCH carrier switching should not be restricted. The SCS of the target carrier can be different from the reference numerology, i.e., the SCS of the PCell, in which case some additional rules are needed for determining the PUCCH resource on the target carrier.

For example, as shown in Figure 6, assume the PUCCH carrier timing pattern takes PCell numerology as the reference granularity. For PDSCH#1 in Slot#1 on PCell, the PUCCH carrier timing pattern indicates the available target carrier index for Slot#2 (of PCell numerology) is SCell#1. The SCS of SCell#1 is 15 kHz which is smaller than the SCS of PCell with 30kHz. In this case, the target slot for transmitting PUCCH#1 on SCell#1 is Slot#1 which overlaps with the Slot#2 of PCell.

As another example, for PDSCH#2 in Slot#3 on PCell, the PUCCH carrier timing pattern indicates the available target carrier index for Slot#4 (of PCell numerology) is SCell#2. The SCS of SCell#2 is 60 kHz which is larger than the SCS of PCell with 30kHz. In this case, the target slot for transmitting PUCCH#2 on SCell#2 is the Slot#7 which is the first available slot among the slots overlapping with the Slot#4 of PCell.

Figure. 6 Example for PUCCH carrier switching across different SCS

Proposal 13: For PUCCH carrier switching across different numerologies based on RRC configured timing pattern, the target slot on the target carrier for PUCCH transmission is the first available slot overlapping with the reference slot of the PUCCH carrier timing pattern.

Rule for PUCCH repetition

In current spec only PCell can be used for PUCCH transmission, i.e. all PUCCH repetitions transmission are in the same carrier. In Rel-17 for PUCCH carrier switching, the PUCCH can be transmitted in PCell or SCell, thus for PUCCH repetitions transmission, the target carriers of different PUCCH repetitions can be different based on the availability of the carrier. An example is shown in the figure below assuming PUCCH repetition number is 4, where the first PUCCH repetition is in Slot#1 of SCell 2, the second PUCCH repetition is in Slot#2 of SCell 1, the third PUCCH repetition is in Slot#3 of PCell and the fourth PUCCH repetition is in Slot#4 of SCell 2.

Figure. 7 Example of PUCCH repetitions transmission for PUCCH carrier switching

Proposal 14: For PUCCH carrier switching, consider to determine the target carrier for each PUCCH repetition individually based on the availability carrier of the slot.

Interaction of PUCCH carrier switching and SPS HARQ-ACK deferral

As another remaining FFS issue of the last meeting, it should be discussed whether and how to support joint operation of PUCCH carrier switching and SPS HARQ-ACK deferral. E.g., when both features are enabled, the UE will defer the SPS HARQ-ACK to an available SCell in case the PCell is not available for SPS HARQ-ACK transmission. In our understanding, it is beneficial to support PUCCH carrier switching for SPS HARQ-ACK transmission to reduce the latency.

As shown in the Figure 8, the target carriers of the PUCCH carrier timing pattern for Slot#1/#2/#3/#4#5 are SCell#1/SCell#1/SCell#2/SCell#1/PCell, respectively. As the UE is not able to transmit the SPS HARQ-ACK on SCell#1 at Slot#2 due to the conflict with DL symbols, it will then perform the SPS HARQ-ACK deferral and check the validity of the next slot. In particular, it will check the validity in each slot on the associated target carrier, e.g., Slot#3 on SCell#2, then Slot#4 on SCell#1, then Slot#5 on PCell until an available PUCCH resource is determined which does not overlap with invalid symbols. If PUCCH carrying SPS HARQ-ACK overlaps with some invalid symbols in the determined target slot and both deferral and carrier switch are configured, SPS HARQ-ACK can be deferred according to the RRC timing pattern.

Figure 8 Example of joint operation of semi-static carrier switching and SPS HARQ-ACK deferral

Proposal 15: Support joint operation of PUCCH carrier switching and SPS HARQ-ACK deferral.

- For each candidate target slot/sub-slot, the UE will check its validity on its associated target carrier based on the PUCCH carrier switching pattern, until an available PUCCH resource is identified to carry the deferred SPS HARQ-ACK.

Case when dynamic PUCCH cell indication cannot be used

As shown in the agreements, it was agreed that dynamic indication and semi-static configuration are subjected to separate UE capabilities. It is straightforward that the dynamically scheduled HARQ-ACK should follow the dynamic indication in the DCI, while the semi-static CSI/SR/SPS HARQ-ACK will follow the semi-static carrier switching pattern.

For dynamic carrier switching, a straightforward way is to introduce a new field in DCI to indicate the target carrier. However, as the field in fallback DCI with DCI format 1_0 can’t be extended, dynamic carrier switching based on new indicator field doesn’t work for a HARQ-ACK scheduled by fallback DCI. In this case, the semi-static carrier switching should be applied, or PUCCH cell is always assumed as PCell for DCI format 1_0 scheduling.

Proposal 16: For dynamic HARQ-ACK scheduled by fallback DCI with DCI format 1_0,

- If semi-static carrier switching is configured, it will be transmitted based on semi-static carrier switching.

- Otherwise, it should be transmitted on PCell.

2.5 Retransmission of cancelled HARQ

In the RAN1 #105-e meeting [4], the following agreements have been achieved.

Working Assumption: For at least HARQ-ACK re-transmission: - Support at least one enhanced Type 3 HARQ-ACK CB with smaller size (compared to Rel-16) in Rel-17 - Definition of enhanced Type 3 CB: - The codebook size of a single triggered enhanced Type 3 HARQ-ACK codebook at least determined by RRC configuration - The codebook construction uses HARQ processes as a bases (i.e. ordered according to HARQ-IDs and serving cells) - Support one-shot triggering (by a DL assignment) of HARQ-ACK re-transmission on a PUCCH resource other than enhanced Type 2 or (enhanced) Type 3 HARQ-ACK CB (i.e. Alt. 3) in Rel-17 - Details are FFS - Enhanced Type 3 HARQ-ACK CB and/or one-shot triggering (by a DL assignment) of HARQ-ACK re-transmission on a PUCCH resource other than enhanced Type 2 or (enhanced) Type 3 HARQ-ACK CB are subject to separate UE capabilities

The working assumption shows that two methods would be supported in Rel-17 for retransmission of cancelled HARQ-ACK. And enabling/disabling mechanism for the method should rely on UE capability report/configuration. However, we want to emphasize again that the motivation and benefit to retransmit cancelled HARQ-ACK is not clear.

According to the discussion in the RAN1#102-e meeting [3], two cases were raised for the retransmission of the cancelled HARQ-ACK(s). One is the retransmission of cancelled low priority (LP) HARQ-ACK(s) due to the collision with the high priority (HP) UCI, and the other is the retransmission of cancelled HP HARQ-ACK(s) due to CI. From our perspective, the retransmission of cancelled LP HARQ-ACK(s) due to a collision with HP UCI is not necessary in this stage, because the benefit is not clear since RAN1 already has the objective to perform HP/LP multiplexing in agenda 8.3.3. By enabling the HP/LP multiplexing, the potential cases for the re-transmission of the cancelled/dropped HARQ-ACK may be very rare. For the retransmission of cancelled HP HARQ-ACK, the benefit may be marginal since it would probably exceed the delay budget. In addition, since it is HP HARQ-ACK the gNB would try to indicate a proper K1 to avoid the cancellation, thus the case of cancelling HP HARQ-ACK is rare.

For the solution of enhanced type 3 HARQ-ACK CB with smaller size, a lot of details have been discussed in past few meetings. Since the motivation or benefit is not clear, we should avoid spending too much effort or time on optimizing it if it really needs to be supported. Therefore, we would prefer to determine the codebook size based on RRC configuration and no dynamic signaling involved.  In addition, the codebook construction can be simply based on HARQ processes.

Proposal 17：If supporting Type 3 CB(s) with smaller size (compared to Rel-16) in Rel-17, the codebook size is determined only by RRC configuration, i.e. no any additional dynamic signaling involved.

Conclusions

According to the discussion, following proposals and observations are provided:

Observation 1: For sub-slot based Type 1 codebook, per slot based grouping has limited changes for codebook generation procedure on top of the legacy specification.

Observation 2: For sub-slot based Type 1 codebook, per slot based grouping can reduce non-negligible bit number of HARQ-ACK feedback compared to per sub-slot based grouping.

Proposal 1: The maximum deferral k1+ k1def of the SPS HARQ-ACK should be the maximum k1 value in dl-DataToUL-ACK.

Proposal 2：Down select between Alt.1 and Alt.1A as the SPS deferral condition:

- Alt. 1:  “If SPS HARQ-ACK is multiplexed with any other UCI / dynamic PUCCH resource then it cannot be deferred!”

- Alt. 1A “Defer SPS HARQ even if multiplexing & transmission based on PRI in initial slot would be possible”.

Proposal 3: For codebook construction for deferred SPS HARQ-ACK in the target slot,

- For Type 1 CB, deferred SPS HARQ-ACK with k1+ k1def included in the K1 set can be mapped in the Type 1 CB following the Rel-15/Rel-16 rule, while those with k1+ k1def not included in the K1 set can be appended.

- For SPS only CB and type 2 CB, Rel-16 rules can be reused.

Proposal 4: For a candidate target slot/sub-slot, if the resulting PUCCH resource is unavailable, both the SPS HARQ-ACK(s) which have been deferred to this slot/sub-slot and the SPS HARQ-ACK originally pointed to this slot/sub-slot should be deferred in together to the next candidate slot/sub-slot.

Proposal 5：Support TDRA grouping performed per slot for sub-slot based Type 1 CB,

- Step 1: Determine DL slots consisting of DL sub-slots associated to the determined UL sub-slot

- Step 2: In each determined DL slot, prune the PDSCH SLIVs whose ending symbols overlap with DL sub-slots that not associated to the determined UL sub-slot based on K1 set.

- Step 3: Perform per slot SLIV splitting among the remaining SLIVs for each slot to generate the TDRA groups, each group of which is associated with the HARQ-ACK bit field.

Proposal 6: Confirm the WA in the Coverage Enhancements WI where the repetition factor is dynamically indicated by the PRI field, and the PUCCH repetition factor is configured per PUCCH resource.

Proposal 7: Support slot-based PUCCH repetition for PUCCH Format 0 and Format 2.

Proposal 8: For dynamic indicating the target PUCCH carrier, consider to introduce a new field in DCI and a configured carrier set consisting of candidate carrier indices.

Proposal 9: For interpreting the PDSCH to HARQ-ACK offset k1 under dynamic carrier indication, the K1 set configured for the indicated carrier should be used.

Proposal 10: For the codebook construction in case of PUCCH carrier switching, HARQ-ACKs with same priority index pointed to the same slot/sub-slot on the same carrier will be constructed as one codebook.

Proposal 11: For PUCCH carrier switching across different numerologies, the OoO rule between the carriers with PDSCH transmission and the carrier with PUCCH transmission should be applied based on the largest SCS.

Proposal 12: For configuration of the RRC configured PUCCH carrier timing pattern,

- The reference numerology is the SCS of PCell and the granularity of the timing pattern is based on the SCS of PCell.

- K1 interpretation is based on K1 set configured for PCell.

Proposal 13: For PUCCH carrier switching across different numerologies based on RRC configured timing pattern, the target slot on the target carrier for PUCCH transmission is the first available slot overlapping with the reference slot of the PUCCH carrier timing pattern.

Proposal 14: For PUCCH carrier switching, consider to determine the target carrier for each PUCCH repetition individually based on the availability carrier of the slot.

Proposal 15: Support joint operation of PUCCH carrier switching and SPS HARQ-ACK deferral.

- For each candidate target slot/sub-slot, the UE will check its validity on its associated target carrier based on the PUCCH carrier switching pattern, until an available PUCCH resource is identified to carry the deferred SPS HARQ-ACK.

Proposal 16: For dynamic HARQ-ACK scheduled by fallback DCI with DCI format 1_0,

- If semi-static carrier switching is configured, it will be transmitted based on semi-static carrier switching.

- Otherwise, it should be transmitted on PCell.

Proposal 17：If supporting Type 3 CB(s) with smaller size (compared to Rel-16) in Rel-17, the codebook size is determined only by RRC configuration, i.e. no any additional dynamic signaling involved.

- References

- RAN1 Chairman’s Notes, RAN1#104b-e, e-Meeting, 12th April – 20th April, 2021

- R1-2106489, “Discussion on PUCCH coverage enhancement”, Huawei, HiSilicon, 16th August – 27th August, 2021

- RAN1 Chairman’s Notes, RAN1#105e, e-Meeting, 10th May – 27th May, 2021.

- RAN1 Chairman’s notes, RAN1#102e, e-Meeting, August 2020.