3GPP TSG RAN WG1 #106-e                                               R1-2106550

e-Meeting, August 16th – 27th, 2021

Source:         ZTE

Title:            Accuracy improvement for UL-AoA positioning solutions

Agenda item:    8.5.2

Document for:	Discussion and Decision

Introduction

According to the agreed WID [1], a number of agreements have been approved to improve positioning accuracy for UL-AOA method over the past several meetings [2]. In this contribution, some remaining issues and other promising approaches for accuracy improvements of UL-AOA are analyzed and proposed.

Multiple antenna reference points for a TRP

Rel-16 specification supports different DL PRS resources being transmitted through different antenna reference points (i.e. different geographical coordinates) from the same TRP. As defined in NRPPa protocol, the following Table 1 in section 9.2.47 of TS 38.455 [3] defines the geographical coordinates of the antenna reference points (ARP) for the DL-PRS Resources of the same TRP. In other words, as depicted in FIG.1, the antenna configuration of one TRP may include multiple antenna groups (i.e. ARP) located in slightly different geographical coordinates, where each of the ARP may have their own processing unit to virtualize different Rx and Tx beam directions or detect timings.

Table.1 Coordinates of DL-PRS Resources

FIG.1 Antenna configuration of one TRP

Furthermore, for UL reception, each measurement result can be associated with a DL PRS resource or an SSB in Measurement Beam Information as shown in Table 2, which indicates the receiving beam information when measuring UL signals. Therefore, the ARP information is implicitly indicated for the UL measurement when DL PRS resource is included in a measurement report as requested by LMF. In addition, whether the Measurement Beam Information should be reported or not is configured by LMF in Measurement Beam Information Request.

Table.2 Measurement Beam Information

However, at least in the following cases, current specification doesn’t support gNB/TRP to report ARP information of UL measurement results,

- Measurement Beam Information is not requested by LMF. However, the gNB/TRP still expects to report measurement results from multiple ARPs.

- The gNB/TRP only supports UL based positioning, so there is no DL PRS resources configured.

- The gNB/TRP is a reception point (RP) attached with multiple ARPs, where the RP only supports UL reception.

From technical perspective, the derived positioning accuracy improves along with the increase of antenna reference points. Therefore, to facilitate UL measurement from multiple ARPs, Rel-17 gNB should be able to report ARP information of UL measurement results at least for the above cases. For example, to enhance UL-AOA measurement from multiple ARPs, each UL-AOA measurement result should be associated with ARP information (e.g. ARP ID and corresponding geographical coordinate) when forwarding measurement report to LMF.

Observation 1: At least in the following cases, current specification doesn’t support gNB/TRP to report ARP information of UL measurement results,

- Measurement Beam Information is not requested by LMF. However, the gNB/TRP still expects to report measurement results from multiple ARPs.

- The gNB/TRP only supports UL based positioning, so there is no DL PRS resources configured.

- The gNB/TRP is a reception point (RP) attached with multiple ARPs, where the RP only supports UL reception.

Proposal 1: In order to improve positioning accuracy, Rel-17 should support reporting UL-AOA measurement results being associated with ARP (Antenna Reference Point) information

- FFS: whether above enhancement can be also applicable to UL-RTOA, gNB Rx-Tx time difference and UL SRS-RSRP measurement

Number of reported UL-AOA values for the first arrival path

Agreement in RAN1#104e: - NR supports reporting of M > 1 UL-AOA (AoA/ZoA) measurement values by gNB to the LMF at least for the first arrival path - FFS: Supporting of UL-AOA measurements for additional paths - FFS: Supporting of N >= 1 UL-AOA values per path for additional paths - FFS: Whether the multiple values can correspond to the same time stamp. - FFS: Further details of measurement and reporting Agreement in RAN1#104b-e: Reporting to LMF of M > 1 UL-AOA (AoA/ZoA) measurement values associated with the first arrival path and corresponding to the same timestamp is supported. Agreement in RAN1#105e: NR supports gNB reporting of the first arrival path UL-AOA/ZOA measurement per SRS for positioning resource and SRS for MIMO resource - Note: The use of SRS for MIMO resource is transparent to the UE

According to the above agreements, gNB can report M > 1 UL-AOA (AoA/ZoA) measurement values with the first arrival path per SRS resource to LMF, where the M > 1 UL-AOA (AoA/ZoA) measurement values are corresponding to the same time stamp. One remaining issue is the value of M. As we discussed in previous meetings, the intention to support more than one UL-AOA values with the first arrival path is to solve the angle ambiguities caused by NLOS, larger than half wavelength of antenna element spacing or strong sidelobes. Therefore, a bunch of UL-AOA values, e.g. 8 may help LMF to select the real LOS direction or main lobe out of reported M values.

Proposal 2: LMF indicates maximum number of UL-AOA values (pair of AOA & ZOA values) for the first arrival path corresponding to the same time stamp is up to 8.

Conclusions

In this contribution, we provide our views to reduce enhance UL-AOA positioning accuracy, the following observations and proposals are proposed,

Observation 1: At least in the following cases, current specification doesn’t support gNB/TRP to report ARP information of UL measurement results,

- Measurement Beam Information is not requested by LMF. However, the gNB/TRP still expects to report measurement results from multiple ARPs.

- The gNB/TRP only supports UL based positioning, so there is no DL PRS resources configured.

- The gNB/TRP is a reception point (RP) attached with multiple ARPs, where the RP only supports UL reception.

Proposal 1: In order to improve positioning accuracy, Rel-17 should support reporting UL-AOA measurement results being associated with ARP (Antenna Reference Point) information

- FFS: whether above enhancement can be also applicable to UL-RTOA, gNB Rx-Tx time difference and UL SRS-RSRP

Proposal 2: LMF indicates maximum number of UL-AOA values (pair of AOA & ZOA values) for the first arrival path corresponding to the same time stamp is up to 8.

References

- RP-210903 Revised WID on NR Positioning Enhancements, Intel Corporation, CATT

- Draft Report of 3GPP TSG RAN WG1 #105-e v0.1.0

- 3GPP TS 38.455, NR Positioning Protocol A (NRPPa) (Release 16)