3GPP TSG RAN WG1 #106-e		R1-2106574

e-Meeting, August 16th –  27th, 2021

Source:	vivo

Title:	Further discussion on MTRP multibeam enhancement

Agenda Item:	8.1.2.3

Document for:	Discussion and Decision

Beam reporting for MTRP

In RAN1#105-e meeting [1], following agreement on beam reporting for MTRP was achieved.

Agreement For CMR configuration for option 2, adopt - Alt-1: “set” Agreement The bitwidth of each SSBRI/CRI is determined based on the number of SSB/CSI-RS resources in the associated CMR resource set - FFS: specify the association between SSBRIs/CRIs in a reported group and CMR resource sets Agreement - For beam measurement/reporting option 2, the maximum number of beam groups in a single CSI-report is a UE capability and may take value from Nmax = {1,2,3,4} in Rel.17. - FFS: If UCI payload reduction for Nmax>=2 is needed and if so, how - The number of beam groups (N) reported in a single CSI-report - Alt1: The value of N is configured by RRC signaling

In Rel-16, both ideal backhaul and non-ideal backhaul are considered for MTRP with the same beam reporting framework as Rel-15. The MTRP transmission defined in Rel-16 such as NCJT, URLLC SDM, FDM schemes requires simultaneous reception by the UE. And in these use cases above, MTRP based beam management is beneficial for traffic offload between TRPs, where the network can configure UE to report the beams from different TRPs which can be transmitted simultaneously.  Some commercial UEs have the capability to activate multiple Rx panels indicating that the UE can connect to and receive from different TRPs with multiple Rx panels simultaneously. Beam reporting for MTRP should take all the above into account.

Rel-17 MTRP beam reporting enhancement should take into account both ideal backhaul and non-ideal backhaul scenarios to support simultaneous transmission at the network side and simultaneous reception at the UE side.

Beam reporting scheme in ideal backhaul MTRPs

In RAN1#104-e meeting[2], Option 2, that is a single CSI-report consisting of N beams pairs/groups and M (M=2) beams per pair/group, and different beams within a pair/group can be received simultaneously was agreed. In ideal backhaul MTRPs, backhaul delay between TRPs can be negligible so that the TRP receiving the reported beam information can forward it to the other TRP. In that case, Option 2 can effectively achieve simultaneous transmission on the TRP side and simultaneous reception on the user side. Some progress on Option 2 had been achieved after discussion in the last two meetings: two CSI-RS/SSB resource sets have been agreed to be configured and measured, and then X beam pairs that can be received simultaneously could be reported, where X is no more than 4 and is configured by the network based on UE capability. However, some remaining issues need to be further determined and resolved for Option2 to work well. Detailed discussion on the issues are as follows:

UCI design

- UCI design mainly consists of two parts: CMR indices and its corresponding measured value of L1-RSRP. For the CMR indices, the bitwidth of each SSBRI/CRI had been agreed to be determined based on the number of SSB/CSI-RS resources in the associated CMR resource set in the last meeting. To facilitate gNB to distinguish which TRP the reported CMR indices belong to, some extra indications are necessary to be carried in the beam report. Two main indication methods are listed as follows:

Option 1: The paring relationship is fixed, and the mapping rule of set/TRP and CRIs/SBBRIs within each beam pair is the same. In this case, one bit is added to indicate the order of CRIs/SBBRIs within each pair or the location of the reference RS with the largest measured value of L1-RSRP within the first beam pair.

Option 2: The field of CRI/SSBRI is extended by 1 bit to indicate the TRP the CRI/SSBRI belongs to.

- Compared with Option 1, Option 2 is more direct and simpler but may cause larger UCI payload size.

- As for the measured value of L1-RSRP, to reduce UCI payload size, differential L1-RSRP based reporting that the largest measured value of L1-RSRP is quantized to a 7-bit value in the range [-140, -44] dBm, and the differential L1-RSRP referred to the largest measured value of L1-RSRP is quantized to a 4-bit value, had been specified in Rel-15. For enhanced beam report in MTRP scenarios in Rel-17, L1-RSRP based beam reporting could be reused, but some details should be reconsidered, such as the number of reference RS(s).  Three differential L1-RSRP based beam reporting mechanisms, including the legacy mechanism,  are listed in Table 1. In terms of payload size, Alt-1that global differential and only one reference RS is the best.

differential L1-RSRP based beam reporting mechanisms

| - Differential L1-RSRP based beam reporting mechanism | - Alt-1 global differential - The RS corresponding to the largest measured value is determined as the reference RS | - Alt-2 local differential within set - The RS corresponding to the largest measured value in each CMR set is determined as the reference RS | Alt-3 local differential within pair The RS corresponding to the largest measured value in each beam pair is determined as the reference RS |
| --- | --- | --- | --- |
| - Payload size | - 35bits | - 38bits | - 44bits |

UCI design for beam report in MTRP scenario

- Combined with the two parts mentioned above, we would like to propose a UCI design scheme shown in Figure 1. In the proposed scheme, the relationship between the reference RS and set/TRP is fixed and is the same in all reported beam pairs. The reported information of the beam pair containing the largest measured value of L1-RSRP would be put first in the corresponding sequence of CRI/SSBRI and the measured value of L1-RSRP respectively, e.g. (CMR#1, CMR#2), and the specific location of the CRI/SSBRI with the largest measured value of L1-RSRP within the beam pair is indicated by one extra bit as shown in Figure 1. For example, if the bit is 0, the reference RS is CMR#1; otherwise, the reference RS is CMR#2. From the perspective of UCI payload size, the proposed scheme only increases one bit on UCI payload size compared with the legacy beam report mechanism, which makes less specification impact.

- In addition to designing the order of CRIs/SSBRIs and the differential format of the measured value of L1-RSRP to reduce UCI payload size, we think the reported beam information could be divided into two parts, like CSI,  with part 1 only reporting a subset of the reported beam information and indicating whether there is the second subset of beam information that needs to report in part 2, that is another valid method to reduce UCI payload size and achieve reliable transmission, especially when the channel resources for different objectives collide and available resource is not enough to load all reported beam information.

For UCI design,

- Support differential report of L1-RSRP with the RS with the maximum RSRP as the reference

- Support one bit to indicate the order of CRI/SSBRI or the location of the reference RS

- Consider UCI payload reduction for N>2, e.g. two parts with part 1 only reporting a subset of beam pairs and indicating whether there is the second subset of beam pairs that need to be reported

Activation of two semi-persist or aperiodic CSI-RS resource sets

- In Rel-15, only one semi-persist CSI-RS resource set can be activated by a MAC CE activation command as shown in Figure 2, and only one aperiodic CSI-RS resource set per report can be triggered by a DCI. Therefore, how to activate two CSI-RS resource sets is a potential issue to be considered.  For semi-persist CSI-RS resources, two CSI-RS resource sets can be activated by one activation command together or activated by two activation commands separately. Compared with the former, the latter can reuse the format of the MAC CE activation command in Rel-15. But for some specific cases, such as one of the activation commands can not be received by UE or both activation commands can not be received simultaneously, UE behavior needs to be clarified.  As for the former, two CSI-RS resource sets can be activated simultaneously, but the shortcoming is the format of the MAC CE activation command need to be redesigned. For aperiodic CSI-RS resources, if allowed to be measured and reported based on Option 2, the RRC signal may need to be enhanced to support activating two sets by one DCI. And the most straightforward way is to add another CSI-RS set ID and TCI state list in the high layer parameter CSI-AssociatedReport ConfigInfo.

SP CSI-RS/CSI-IM Resource Set Activation MAC CE

Support enhancement on activation of two semi-persist or aperiodic CSI-RS resource sets simultaneously

Default beam of aperiodic CSI-RS

- In Rel-16, the default beam of aperiodic CSI-RS in single-DCI-based MTRP and multi-DCI-based MTRP scenarios had been specified. For multi-DCI-based MTRP,  if the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the aperiodic CSI-RS resources in a NZP-CSI-RS-ResourceSet is smaller than the UE reported threshold beamSwitchTiming/beamSwitchTiming-r16, the UE applies the QCL assumption of the other DL signals satisfying following conditions to receive the aperiodic CSI-RS:

- overlap fully in symbols with the aperiodic CSI-RS

- associate with the same CORESETPoolindex as the aperiodic CSI-RS

- If there is no DL signal meeting the requirements, UE will apply the QCL parameter(s) of the CORSET associated with a monitored search space with the lowest controlResourceSetId among CORESETs, which are configured with the same value of coresetPoolIndex as the PDCCH triggering that aperiodic CSI-RS. However, due to the agreed two CSI-RS resource sets for beam measurement and report in MTRP scenarios, it means for aperiodic CSI-RS resource, a DCI would trigger two resource sets. In this case, directly reusing the default beam define in Rel-16 may be ambiguous, as a result of both sets may associate the same CORESETPoolindex. Therefore, the default beam of aperiodic CSI-RS for beam management in multi-DCI-based MTRP scenarios needs to be clarified. In our view, defining the default beam per TRP is the most straightforward.

Default beam of aperiodic CSI-RS resource should be clarified when a DCI trigger two CSI-RS resource sets

Beam reporting in non-ideal backhaul MTRPs

In RAN1#104-e meeting, three options for beam measurement/reporting enhancement to facilitate inter-TRP beam pairing were discussed and Option 2 was agreed. Option 1 and Option 2 are proposed based on antenna group-based reporting and beam group-based reporting respectively which were discussed in Rel-15. Different beams in different pairs/groups can be received simultaneously in Option1, while different beams in a pair/group can be received simultaneously in Option 2. Option 3 is implemented by associating different CSI reports to achieve simultaneous reception, which realizes the function of group-based reporting in another way. We prefer Option 2 only in ideal backhaul scenarios and some details have been discussed above. As for non-ideal backhaul MTRPs, three options are analyzed from the following aspects:

Scheduling flexibility

First of all, we investigate the scheduling flexibility of three options in non-ideal backhaul scenarios, which can be implied by the number of candidate beam pairs with the same UCI payload. For non-ideal backhaul MTRPs, it is desired that each TRP can receive the beam report from the UE separately. In this case, UE may report the same beam information to both of TRPs repeatedly in Option 1 and Option 2. Under this assumption, the numbers of candidate beam pairs of three options are listed in Table 2 respectively. For example, when UCI payload size is 83 bits/report, the performances of three options are as follows:

Option 1: 2 groups in each report, 4 beam indices in each group, and the total number of candidate beam pairs is 16.

Option 2: 4 groups in each report, 2 beam indices per group, and the total number of candidate beam pairs is 4.

Option 3: 8 beam indices in each report and the total number of candidate beam pairs is 64.

With the same UCI payload size, the more candidate beam pairs, the higher scheduling flexibility the network will have. Therefore, considering the scheduling flexibility, Option 3 has much advantage over Option 1 and Option 2 in non-ideal backhaul MTRPs.

Number of candidate beam pairs in non-ideal backhaul MTRPs

| UCI payload size (bits/report) | Option 1 | Option 2 | Option 3 |
| --- | --- | --- | --- |
| 23 | 1 | 1 | 4 |
| 43 | 4 | 2 | 16 |
| 63 | 9 | 3 | 36 |
| 83 | 16 | 4 | 64 |

Option 3 provides the highest scheduling flexibility with the same UCI payload size per PUCCH/PUSCH resource for non-ideal backhaul scenarios.

Throughput performance in non-ideal backhaul MTRPs

For non-ideal backhaul MTRPs, in order to achieve simultaneous transmission, only after all TRPs receiving beam report can the network synchronize PDSCH transmission with the reported beam pair, otherwise mismatched Tx beams would probably lead to the multiple Rx beams that cannot be received by the UE simultaneously. In Option 1 and Option 2, two possible ways to implement simultaneous transmission are: a) UE reports the same beam information to all TRPs separately which has been mentioned in the analysis of UCI payload size and it will cause large overhead; b) UE reports beam information to one TRP and then the TRP forwards to another with backhaul delay. As for Option 3, separate reports to different TRPs can guarantee each TRP to obtain their own beam information, and the delay for the separate beam information taking effect is up to network implementation. Therefore, we think Option 3 is more suitable in non-ideal backhaul scenarios. To verify the above argument, we evaluate the throughput performance of three options as follows.

Table 3 to Table 4 show the UPT loss of Option 1 and Option 2 compared to the baseline of Option 3, with different traffic models. More simulation parameters can be found in Appendix A.

Option 1 and Option 2: UE reports the optimal beam pair to one TRP and then the TRP forwards to another with backhaul delay.

Option 3 (baseline): UE reports partial beam information of the optimal beam pair in separate beam reports to different TRPs simultaneously.

We provide UPT comparison for Full Buffer and FTP model 1 with RU for Option 3 (baseline) setting to 9%, 22% and 58%, backhaul delay setting to 50ms, and UE rotation speed setting to 50rpm for FR2 Indoor Hotspot.

MTRP with non-ideal backhaul, FR2 InH (full buffer)

| Backhaul delay(ms) | Option | Mean | 5% | 50% |
| --- | --- | --- | --- | --- |
| - | 3 | 0.00% | 0.00% | 0.00% |
| 50 | 1&2 | -8% | -46.91% | -13.14% |

MTRP with non-ideal backhaul, FR2 InH

| RU | Backhaul delay(ms) | Option | Mean | 5% | 50% |
| --- | --- | --- | --- | --- | --- |
| 9% | - | 3 | 0.00% | 0.00% | 0.00% |
|  | 50 | 1&2 | -3.68% | -6.13% | 0.00% |
| 22% | - | 3 | 0.00% | 0.00% | 0.00% |
|  | 50 | 1&2 | -7.01% | -10.66% | -9.88% |
| 58% | - | 3 | 0.00% | 0.00% | 0.00% |
|  | 50 | 1&2 | -19.83% | -47.49% | -23.68% |

From the above tables, we observe that Option 1 and Option 2 have obvious performance losses compared to Option 3, especially for the UE at the TRP boundary. And with RU increasing, the performance loss is more significant. The reason for the above phenomena in Option 1 and Option 2 is that the scheduled beam pair cannot keep pace with the time-varying channel due to backhaul delay, especially when UE rotates.

For non-ideal backhaul cases, some throughput gain can be achieved by Option 3, while outdated beam pair applied due to backhaul delay for Option 1 and Option 2 when forwarding the beam report from one TRP to the other TRP.

Support Option 3 for multi-TRP beam report enhancement in non-ideal backhaul scenarios.

Interference measurement and report metric

In previous meetings, solutions for this issue were discussed but no agreement was reached. In our opinion, when the UE measures the CSI-RS/SSB resources, the result of L1-SINR may reflect the inter-beam interference, but it can’t reflect the quality of the simultaneous reception. For example, in the NCJT scenario, the interference between different beams does not necessarily correspond to the effective post-SINR if simultaneously received by two Rx panels. L1-SINR value measured as strong interference between two beams may also imply that the two beams contribute higher spatial diversity to the MIMO capacity. Generally, the factors such as received signal power, spatial diversity, and spatial correlation determine the MIMO performance of NCJT, which cannot be reflected by the L1-SINR computation based on current specification.

We analyze the throughput performance of reported beams based on L1-SINR criterion and L1-RSRP criterion respectively. In the simulation, we assume one panel on the TRP side and two panels (back to back) on the UE side.  Each panel contains one TXRUs with dual-polarization. While sweeping beams, UE only selects and reports one optimal beam pair based on L1-RSRP or L1-SINR. More simulation parameters can be found in Appendix A.

The methods of selecting beam pair for an MTRP UE are given as below:

L1-RSRP criterion: a UE selects one beam with the highest L1-RSRP per Rx panel, each corresponding to a different TRP. Independent spatial domain filters per Rx panel are assumed, implying any beam pairs can be received simultaneously by different spatial domain filters.

L1-SINR criterion: a UE selects one beam pair with the highest capacity based on L1-SINR regarding the CMR resources from another TRP as interference. Assuming beam 1 is for by Rx panel 1 and beam 2 is for Rx panel 2, then the leakage of beam 1 received by Rx panel 2 is deemed as the interference, and so is beam 2. The capacity of a beam pair is calculated as follows:

capacity=log2(1+SINR1)+log2(1+SINR2)

where SINR1 and SINR2 are L1-SINR of the beam pairs for two Rx panels respectively, given by

SINR1=P1P2'+I1

SINR2=P2P1'+I2

where P1, P2 are signal power of beam 1, beam 2 received by Rx panel 1 and Rx panel 2 respectively, P2', P1' are interference power of beam 2, beam 1 measured by Rx panel 1 and Rx panel 2 respectively. I1 and I2 are interference outside the two TRPs.

We provide UPT performance and comparison for FTP model 1 with RU for baseline L1-RSRP setting to 4%, 16% and 61% for FR2 Indoor Hotspot, where UE is not allowed to rotate and backhaul delay is negligible between TRPs. We set the same packet arrival rate (λ) for the cases with the same value of RU. Considerable UPT loss can be observed at 5% and 50% UPT, and mean UPT as well.

UPT with different measurement and report metrics in MTRP, FR2 InH

| RU | Report metric | Mean | 5% | 50% |
| --- | --- | --- | --- | --- |
| 4% | L1-RSRP | 519 MHz | 291 MHz | 444 MHz |
|  | L1-SINR | 473 MHz | 263 MHz | 444 MHz |
|  | Loss | -8.86% | -9.62% | 0.00% |
| 16% | L1-RSRP | 434 MHz | 147 MHz | 434 MHz |
|  | L1-SINR | 363 MHz | 117 MHz | 405 MHz |
|  | Loss | -16.36% | -20.41% | -6.68% |
| 61% | L1-RSRP | 334 MHz | 75.1 MHz | 337 MHz |
|  | L1-SINR | 217 MHz | 33.3 MHz | 188 MHz |
|  | Loss | -35.03% | -55.66% | -44.21% |

From the above simulation results, we observe that beam pair selected on L1-SINR criterion have obvious performance loss compared to that of L1-RSRP criterion, especially for the user at the cell boundary. The reason for the performance degradation of L1-SINR criterion is that regarding the CMR resources from another TRP as interference directly may ignore some signal information, especially for NCJT.

For MTRP, performance loss is observed for L1-SINR-based beam pair report with interference calculated between the reported beam pair.

Compared with L1-SINR, we think the CQI for MTRP CSI can precisely reflect inter-beam interference and determine whether two beams are suitable for grouping. However, feeding back the CQI per beam pair could increase too much calculation complexity at UE to implement. Thus, L1-RSRP discussion should be prioritized in Rel-17.

Do not support L1-SINR report with interference calculated between the reported beam pair.

Beam failure recovery for MTRP

In RAN1#104bis-e meeting [1], following agreement on BFR for MTRP was achieved.

Agreement Select one of the following alternatives with possible modification in RAN1#106-e - Alt 2.5.2 A: - On PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured, there is no consensus to adopt alt-1 or alt-2. PUCCH-SR resource selection is up to UE implementation. - Alt 2.5.2 B: - On the PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured, and at most one BFD RS set fails per CC, adopt alt 2 if all failed BFD RS sets cross CCs are associated with the same PUCCH SR resource, else PUCCH-SR resource selection is up to UE implementation. - Alt 2.5.2 C: - On the PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured, and at most one BFD RS set fails per CC, adopt alt 1 if all failed BFD RS sets cross CCs are associated with the same PUCCH SR resource, else PUCCH-SR resource selection is up to UE implementation. - Alt 2.5.2 D: - Revert the past agreement on supporting configuration of up to 2 PUCCH-SR resources. A UE can be configured up to 1 PUCCH-SR resource in a cell group.

In the last two meetings, the issue that the configuration of cell-specific BFR and TRP-specific BFR was discussed, and support simultaneous configuration of cell-specific BFR and TRP-specific BFR in different CCs was agreed and simultaneous configuration of cell-specific BFR and TRP-specific BFR in the same CC still needs to be further studied. However, in our view, the issues of simultaneous configuration are complicated without the basic design and common understanding of how TRP-specific BFR procedure works. So we should discuss and make a consensus on TRP-specific BFR procedure firstly, especially when only one TRP fails, and then discuss other cases.

The case of TRP-specific BFR procedure with only one TRP failure in one CC should be prioritized for discussion over more complicated cases, e.g., simultaneous failure of multiple TRPs and simultaneous configuration of cell-specific BFR and TRP-specific BFR cases.

TRP specific BFR for single-DCI-based MTRP

TRP-specific BFD, TRP-specific new candidate beam identification, and TRP-specific BFRQ have been agreed in RAN1#103-e meeting and some details, such as the number of BFD-RS set, have been determined in RAN1#104-e meeting. We think both single-DCI-based MTRP and multi-DCI-based MTRP should be considered for TRP-specific BFR enhancement. Even though single-DCI-based MTRP is designed to conquer the possible blockage happened in FR2 by transmitting different layers or frequency resources of a PDSCH or PDSCH transmission occasions from two TRPs, the network would achieve more reliable PDCCH transmission or better PDSCH/PUSCH transmission scheme if the network was able to know beam failure of the radio link between either TRP and the UE in time.

In multi-DCI-based MTRP operation, TRP can be identified by the configured CORESETPoolIndex. However, there is no explicit TRP index in single-DCI-based MTRP scenario, so that UE can not distinguish TRPs and reset beam correctly. Additionally, considering multi-DCI has higher priority than single-DCI agreed in RAN#1 104-e meeting, we think the procedure of TRP-specific BFR in single-DCI-based MTRP scenarios can be simplified as that UE only indicate the failure event in BFR MAC CE based on the explicitly configured BFD-RS, and no TRP -specific NBI-RS configuration and beam resetting.  In this case, UE only reports the beam failure event and does not need to reset beam for the failed TRP and thus reuses the procedure defined for multi-DCI-based MTRP BFR procedure as much as possible.

Simply the procedure of TRP-specific BFR in Single-DCI-based MTRP scenarios due to low priority

- Support indication of failure event in BFR MAC CE based on the explicitly configured BFD-RS

- No further enhancement on the configuration of TRP-specific NBI-RS and beam resetting

TRP specific BFR for multi-DCI-based MTRP

Configuration of TRP-specific BFD-RS

Independent BFD-RS configuration per-TRP has been agreed, where each TRP is associated with a BFD-RS set and the total number of BFD-RS set per BWP is 2. No matter it is single-DCI-based MTRP or multi-DCI-based MTRP, two sets of BFD-RS, e.g., q0,0, q0,1, can be explicitly configured by the higher layer parameter failureDetectionResources, each set consisting of the BFD-RS(s) associating with one TRP. For multi-DCI-based MTRP, a straightforward independent BFD-RS configuration is that a number of P-CSI-RS indexes with QCL-typeD for PDCCH monitoring associating with different values of CORESETPoolIndex can be implicitly determined as two sets of BFD-RSs.

As for the number of resources, an agreement that the total number of RSs in two BFD-RS sets per DL BWP is a UE capability was reached and two alternatives on the maximum number of RS per BFD-RS set were discussed in the last meeting. Compared with a fixed value of 2, we would like to regard it as UE capability to guarantee the generality of the specification implementation. Therefore, we prefer to determine the maximum number of RS(s) per BFD-RS set.

For TRP-specific BFD-RS configuration,

Two sets of BFD-RS can be explicitly configured for both multi-DCI-based MTRP and single-DCI-based MTRP.

Two sets of BFD-RS can be implicitly configured for multi-DCI-based MTRP, each including P-CSI-RS indexes with QCL-typeD for PDCCH monitoring associating with one of the two values of CORESETPoolIndex.

Support the maximum number of RS(s) per BFD-RS set as UE capability.

Configuration of TRP-specific NBI-RS sets

As the independent configuration of the NBI-RS set per TRP and the 1-to-1 association between each BFD-RS set and an NBI-RS set has been supported, new candidate beam selection based on the TRP in beam failure is enabled if TRP-specific NBI-RS set is configured. However, for multi-DCI-based MTRP, we don’t think the configuration of TRP-specific NBI-RS set(s) for TRP-specific BFR is a necessity. For example, if one TRP fails and its corresponding TRP-specific NBI-RS set is not configured, an aperiodic CSI report can be triggered by the TRP in a good radio link to obtain one or more new beams to recover the failed TRP. Compared with periodic measurement of NBI-RS resources, aperiodic beam measurement consumes less resource, which is beneficial for the network to schedule various services of users within the limited UE capability flexibly.

If NBI-RS resources are configured, to fully leverage the benefit of MTRP operation, UE should be able to freely select new candidate beams of other TRPs but not limit to the TRP in beam failure, so that any TRP providing good radio links can be selected to serve the UE. Therefore, new beams of more other TRPs should be configured in the candidate beam set. For example, as illustrated in Figure 3, two NBI-RS sets can be configured, each associating with a BFD-RS set and including the new beams of the associating TRP and other TRPs that the UE can select. In this way, the UE can select one new beam out of the new beam set corresponding to the beam link pair of the TRP in failure and of other TRPs except the one in good radio link, rather than limiting to the beam of the TRP in beam failure. Otherwise, restricting the new candidate beam of the TRP in beam failure will achieve inferior performance to the free selection case.

Example of NBI-RS set configuration

However, whether the new candidate beam can be the beam with the TRP still in good radio link other than the one still in use needs further study. If it is allowed, the network would fall back to single TRP transmission once the new candidate beam is selected as the beam of the TRP in a good radio link.

Support to optionally configure TRP-specific NBI-RS for multi-DCI-based MTRP.

In RAN1#104-e meeting, the 1-to-1 association between each BFD-RS set and an NBI-RS set was agreed, but we think the premise for the 1-to-1 association is that two NBI-RS set are configured. As for the association details, in multi-DCI-based-MTRP, associating NBI-RS set and BFD-RS set with CORESETPoolindex respectively, and an NBI-RS set associates with a BFD-RS set with the same value of CORESETPoolindex.

For the 1-to-1 association between each NBI-RS set and a BFD-RS set for multi-DCI-based MTRP

The association only exists when two NBI-RS sets are configured.

The association can be indicated by BFD-RS set ID or CORESETPoolindex.

TRP-specific BFRQ

One of the benefits of TRP-specific BFR is that a UE connecting with multiple TRPs can help itself avoid the break of all radio links. This can also be reflected by the beam failure recovery request reliability. The BFRQ can be transmitted through the good radio link to the TRP not in beam failure. One or two dedicated PUCCH-SR resources in a cell group had been supported in previous meetings.

For the case of two dedicated PUCCH-SR resources, the configuration of two PUCCH-SR resources was discussed but no consensus was reached in the last meeting. Considering high-layer parameters including sr-ProhibitTimer, sr-TransMax and SR_COUNTER are configured per SR configuration, we would like to configure two PUCCH-SR resources under different SR configuration to make both PUCCH-SR resources work well.

As for the spatial relation per PUCCH-SR resource, when a UE is configured with two PUCCH-SR resources in a cell group, each associated with a TRP/BFD-RS set and a spatial filter for the other TRP, the UE can select the appropriate PUCCH-SR resource associated with the TRP in beam failure to request a UL grant for MAC CE with BFRQ. With TRP-specific PUCCH-SR resources, the network is able to know the TRP in beam failure and may not schedule the failed TRP to transmit. However, when a UE is configured with only one PUCCH-SR resource in a cell group, the PUCCH-SR resource may be configured with one or two spatial relations. And for the case of two spatial relations, we think it is used as agreed for PUCCH repetition to facilitate reliability, not for selecting one spatial relation towards a certain TRP to transmit PUCCH.

For the dedicated PUCCH-SR resources for TRP-specific BFRQ,

For the case when two PUCCH-SR resources are configured for TRP-specific BFRQ in a cell group,

Two PUCCH-SR resources are under different SR configurations respectively.

Each PUCCH-SR resource associated with a BFD-RS set of one TRP can be configured with the spatial filter towards the other TRP in FR2.

For the case when one PUCCH-SR resource is configured for TRP-specific BFRQ in a cell group,

The PUCCH-SR resource can be associated with one or two spatial relations.

The case of two spatial relations is used as agreed for PUCCH repetition.

For the cased of BFR of one TRP, if two dedicated PUCCH-SR resources are configured in a cell group or/and two UL grants are available, where each PUCCH-SR resource/UL grant associated with one BFD-RS set, how does UE select one of them to transmit SR or BFR MAC CE?  In the above case, the selection rule should be considered. In the last meeting, three alternatives for the case of configuring two dedicated PUCCH-SR resources in a cell group were discussed and they are listed as follows:

Alt-1: PUCCH-SR resource associated with other/non-failed BFD-RS set, association details FFS

Alt-2: PUCCH-SR resource associated with failed BFD-RS set, association details FFS

Alt-3: Leave it up to UE implementation

According to different use cases, we think three alternatives can be divided into two categories. For the case that one TRP fails in a cell where two PUCCH-SR resources are configured, Alt1 and Alt2 are appropriate to be used.   Compared Alt1 and Alt2, we think the only difference between them is the spatial relation, where the spatial relation of PUCCH-SR resource in Alt1 towards the TRP which the PUCCH-SR resource associates with, while the spatial relation of PUCCH-SR resource in Alt2 towards the TRP which the other PUCCH-SR resource associates with. Therefore, we are fine with Alt1 and Alt2. As for the case of one TRP fails in a Cell, while no TRP fails in another cell where both PUCCH-SR resources are configured, it is up to UE implementation to select a PUCCH-SR resource to transmit.

For the cased of BFR of both TRPs in SpCell or SCell, it should be discussed case by case according to whether to simultaneously configure TRP-specific BFR and cell-specific BFR. So we propose to discuss the issue after the procedure of TRP-specific BFR is determined.

For TRP-specific BFRQ,

For the case of BFR of one TRP,

When one TRP fails in a cell where two PUCCH-SR resources are configured, any one of Alt1 and Alt2 can be used to select one PUCCH-SR resource to be transmitted.

When one TRP fails in a cell, while no TRP fails in another cell where two PUCCH-SR resources are configured, the PUCCH-SR resource selected to be transmitted is up to UE implementation.

When more than one UL grants are available to transmit BFR MAC CE, the selection rule should also be considered.

For the case of BFR of both TRPs in SpCell or SCell, it needs to be further studied.

Some information carried in BFR MAC CE has been agreed in RAN1#104-e meeting, including failed CC indices (if applicable), one new candidate beam index for the failed TRP/CC(if found and NBI-RS(s) is configured for the failed TRP), and an indication whether new beam(s) is found if NBI-RS(s) is configured for the failed TRP. Except for the above information, we think it is necessary to indicate the failed TRP index in BFR MAC CE. For example, assuming there are two UL grants available for UE to convey BFR MAC CE, but if no new candidate beam is found and failed TRP index is not included in BFR MAC CE, the network can’t distinguish which TRP fails based on the BFR MAC CE. Therefore, considering the reliability of TRP-specific BFR, an indication of failed TRP is a necessity. As for the indication of failed TRP, if TRP-specific BFR is applied for both single-DCI-based MTRP and multi-DCI-based MTRP, the index(es) of BFD-RS set is better to be used for the index information of failed TRP.

Following information can be provided in BFR MAC CE based on configuration

Index of the BFD-RS set associated with the failed TRP

CC index (if applicable)

New candidate beam index(es) (if found) if NBI-RS(s) is configured for the failed TRP

Indication whether new beam(s) is found if NBI-RS(s) is configured for the failed TRP

gNB response enhancement

For the case of BFR of one TRP, after the UE transmits the BFRQ carrying the new beam with a first PUSCH MAC CE, it can monitor PDCCH for scheduling a PUSCH transmission with the same HARQ process number as for the transmission of the first PUSCH and having a toggled NDI field value like legacy procedure designed for SCell beam failure. As another option, allowing a quick beam link re-establishment, the network can directly indicate new beams through MAC CE activation command for the CORESET(s) related to the BFD-RS set in beam failure by the TRP in a good radio link, or re-configure the CORESETs by RRC signaling after the network detects a MAC CE with BFRQ. In addition, when one TRP is in failure and no new beam has been found and reported based on the corresponding NBI-RS  resource(s), the network can deactivate the failed TRP through MAC CE deactivation command to achieve the switch of transmission hypothesis from MTRP to STRP. Except for the above all, when the network receives the BFRQ, in which neither the indication of whether a new beam is found nor the indication of a new beam for the failed TRP is included, it may trigger a beam measurement and reporting procedure to the UE by the good radio link. This will enable fast beam searching to re-establish the failed beam link as soon as possible. When UE detects both beam links to two TRPs fail, legacy process on receiving the network response can be applied if only one new beam has been reported in the BFRQ.

Support to deactivate TRP through MAC CE if there is no new beam found.

For the case of BFR of one TRP, a UE can receive the following signaling as BFRR in addition to legacy BFRR:

a MAC CE activation command to update the TCI states for the CORESET(s) related to the TRP/BFD-RS set in beam failure.

a MAC CE deactivation command to deactivate the failed TRP.

a PDCCH to trigger a beam measurement and reporting procedure for the failed TRP.

UE behavior upon receiving BFRR and not receiving BFRR

UE behavior upon receiving BFRR should consider the MTRP mode, the number of beam links in failure, and the number of new beams. As for UE behavior, beam reset, TRP deactivation, and beam training may be conducted optionally in different cases, detailed analyses are as follows.

For a UE operating with multi-DCI-based MTRP, and in the case of BFR of one TRP, if it has found and reported the new beam, the UE may reset the beams to the new beam for PDCCH and PDSCH reception, associating with the CORESETPoolIndex which has been declared beam failure until the UE receives a new MAC CE activation command or RRC reconfiguration. If BFRR is a MAC CE activation command to update the TCI states for the CORESET(s) related to the TRP/BFD-RS set in beam failure, UE may reset the beam according to the beam indication in the MAC CE. If no new beam has been found and the received BFRR is a MAC CE deactivation command for the failed TRP, UE may terminate reception and measurement of any resources associated with the failed TRP and then fall back to single TRP reception.  If UE receives a PDCCH triggering an aperiodic beam report, it will perform an aperiodic beam training and report new beam(s) to the network. If the UE has not received any BFRR during a time window it can turn the receiving beam for the CORESET of the PDCCH monitoring back to the new beam automatically. This is useful to save another UL grant with the same HARQ process with NDI toggled.

When both TRPs are in beam failure and only a single new candidate beam is found and reported, the UE can fall back to single TRP reception assuming the PDCCH and PDSCH reception QCLed with the reported new beam starting from a certain time after receiving a MAC CE deactivation command or RRC reconfiguration command. When multi-DCI-based MTRP operation falls back to single TRP transmission, one of the two CORESETPoolIndexes can survive, and all CORESETs associating with the surviving CORESETPoolIndex apply the QCL of the new beam.

For a UE operating with multi-DCI-based MTRP,

For the case of BFR of one TRP, if it has reported the new beam,

UE may reset the beam to the new beam for the PDCCH and PDSCH associating with the CORESETPoolIndex which has been declared beam failure.

UE may reset the beam according to the beam indication in the MAC CE when BFRR is a MAC CE activation command to update the TCI states.

For the case of BFR of one TRP but no new beam has been found and reported, UE does not reset the beam.

UE may fall back to single TRP reception when BFRR is a MAC CE deactivation command for the failed TRP.

UE may perform an aperiodic beam training and report new beam(s) to the network when receiving a PDCCH including the field of CSI request.

For the case of both TRPs in beam failure and only a single new beam, the UE may fall back to single TRP reception when receiving a MAC CE deactivation command or RRC reconfiguration command.

Conclusion

In this contribution, we analyze enhancements of beam management for MTRP in Rel-17, including beam reporting and TRP-specific BFR. To summarize, we have following observations and proposals.

- Option 3 provides the highest Scheduling flexibility with the same UCI payload size per PUCCH/PUSCH resource for non-ideal backhaul scenarios.

For non-ideal backhaul cases, some throughput gain can be achieved by Option 3, while outdated beam pair applied due to backhaul delay for Option 1 and Option 2 when forwarding the beam report from one TRP to the other TRP.

For MTRP, performance loss is observed for L1-SINR-based beam pair report with interference calculated between the reported beam pair.

- Rel-17 MTRP beam reporting enhancement should take into account both ideal backhaul and non-ideal backhaul scenarios to support simultaneous transmission at the network side and simultaneous reception at the UE side.

For UCI design,

- Support differential report of L1-RSRP with the RS with the maximum RSRP as the reference

- Support one bit to indicate the order of CRI/SSBRI or the location of the reference RS

- Consider UCI payload reduction for N>2, e.g. two parts with part 1 only reporting a subset of beam pairs and indicating whether there is a second subset of beam pairs that need to be reported

Support enhancement on activation of two semi-persist or aperiodic CSI-RS resource sets simultaneously

Default beam of aperiodic CSI-RS resource should be clarified when a DCI trigger two CSI-RS resource sets

Support Option 3 for multi-TRP beam report enhancement in non-ideal backhaul scenarios.

Do not support L1-SINR report with interference calculated between the reported beam pair.

The case of TRP-specific BFR procedure with only one TRP failure in one CC should be prioritized for discussion over more complicated cases, e.g., simultaneous failure of multiple TRPs and simultaneous configuration of cell-specific BFR and TRP-specific BFR cases.

Simply the procedure of TRP-specific BFR in Single-DCI-based MTRP scenarios due to low priority

- Support indication of failure event in BFR MAC CE based on the explicitly configured BFD-RS

- No further enhancement on the configuration of TRP-specific NBI-RS and beam resetting

For TRP-specific BFD-RS configuration,

Two sets of BFD-RS can be explicitly configured for both multi-DCI-based MTRP and single-DCI-based MTRP.

Two sets of BFD-RS can be implicitly configured for multi-DCI-based MTRP, each including P-CSI-RS indexes with QCL-typeD for PDCCH monitoring associating with one of the two values of CORESETPoolIndex.

Support to report the maximum number of RS per BFD-RS set as UE capability.

Support to optionally configure TRP-specific NBI-RS for multi-DCI-based MTRP.

For the 1-to-1 association between each NBI-RS set and a BFD-RS set for multi-DCI-based MTRP

The association only exists when two NBI-RS sets are configured.

The association can be indicated by BFD-RS set ID or CORESETPoolindex.

For the dedicated PUCCH-SR resources for TRP-specific BFRQ,

For the case when two PUCCH-SR resources are configured for TRP-specific BFRQ in a cell group,

Two PUCCH-SR resources are under different SR configurations respectively.

Each PUCCH-SR resource associated with a BFD-RS set of one TRP can be configured with the spatial filter towards the other TRP in FR2.

For the case when one PUCCH-SR resource is configured for TRP-specific BFRQ in a cell group,

The PUCCH-SR resource can be associated with one or two spatial relations.

The case of two spatial relations is used as agreed for PUCCH repetition.

For TRP-specific BFRQ,

For the case of BFR of one TRP,

When one TRP fails in a cell where two PUCCH-SR resources are configured, any one of Alt1 and Alt2 can be used to select one PUCCH-SR resource to be transmitted.

When one TRP fails in a cell, while no TRP fails in another cell where two PUCCH-SR resources are configured, the PUCCH-SR resource selected to be transmitted is up to UE implementation.

When more than one UL grants are available to transmit BFR MAC CE, the selection rule should also be considered.

For the case of BFR of both TRPs in SpCell or SCell, it needs to be further studied.

Following information can be provided in BFR MAC CE based on configuration

Index of the BFD-RS set associated with the failed TRP

CC index (if applicable)

New candidate beam index(es) (if found) if NBI-RS(s) is configured for the failed TRP

Indication whether new beam(s) is found if NBI-RS(s) is configured for the failed TRP

Support to deactivate TRP through MAC CE if there is no new beam found.

For the case of BFR of one TRP, a UE can receive the following signaling as BFRR in addition to legacy BFRR:

a MAC CE activation command to update the TCI states for the CORESET(s) related to the TRP/BFD-RS set in beam failure.

a MAC CE deactivation command to de-activate the failed TRP so that to achieve the switch of transmission hypothesis from MTRP to STRP.

a PDCCH to trigger a beam measurement and reporting procedure for the failed TRP.

For a UE operating with multi-DCI-based MTRP,

For the case of BFR of one TRP, if it has reported the new beam,

UE may reset the beam to the new beam for the PDCCH and PDSCH associating with the CORESETPoolIndex which has been declared beam failure.

UE may reset the beam according to the beam indication in the MAC CE when BFRR is a MAC CE activation command to update the TCI states.

For the case of BFR of one TRP but no new beam has been found and reported, UE does not reset the beam.

UE may fall back to single TRP reception when BFRR is a MAC CE deactivation command for the failed TRP.

UE may perform an aperiodic beam training and report new beam(s) to the network when receiving a PDCCH including the field of CSI request.

For the case of both TRPs in beam failure and only a single new beam, the UE may fall back to single TRP reception when receiving a MAC CE deactivation command or RRC reconfiguration command.

References

Chairman’s notes, RAN1#105-e.

Chairman’s notes, RAN1#104-e.

Appendix A: Simulation parameters

SLS assumption for MTRP beam report enhancement

| Parameters |  | Value |
| --- | --- | --- |
| Duplex, Waveform |  | FDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Indoor hotspot (InH) |
| Frequency Range |  | FR2, 30GHz |
| Inter-BS distance |  | 20m |
| Channel model |  | According to the TR 38.901 |
| Antenna setup and port layouts at TRP |  | 2 Tx ports: (M, N, P, Mg, Ng) = (4,4,2,1,1) |
| Antenna setup and port layouts at UE |  | 4Rx Port: (M, N, P, Mg, Ng, Mp,Np) = (2,4,2,1,2,1,2) (dH,dV) = (0.5, 0.5)λ |
| BS Tx power |  | 23dBm |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 120kHz |
| Number of RBs |  | 52 |
| Simulation bandwidth |  | 80 MHz |
| Frame structure |  | Slot Format 0 (all downlink) for all slots |
| Configuration for MTRP | Cluster | 4 neighboring TRPs |
|  | Maximal number of coordinating TRPs | 2 |
|  | Backhaul assumption | non-ideal |
| Feedback assumption |  | Beam and CSI feedback periodicity: 5 ms Scheduling delay (from CSI/beam feedback to time to apply in scheduling): 4 ms Subband PMI, subband CQI Rank 1 or rank 2 per TRP |
| UE receiver |  | MMSE-IRC |
| Channel estimation |  | Ideal |