3GPP TSG RAN WG1 #106-e	                                R1-2106590

e-Meeting, August 16th – 27th, 2021

Source: 	vivo

Title:  	Discussion on propagation delay compensation enhancements

Agenda Item:	8.3.4

Document for:	Discussion and Decision

Introduction

In RAN1#104b e-meeting, downlink frame timing detection error is discussed and a LS is sent to RAN4 for clarification on downlink frame timing detection error and Te. According to reply LS from RAN4, it is the common understanding that downlink frame timing detection error is already included in UE transmit timing error[1].

On the other hand, the equations on TA-based method are discussed and the following agreements are achieved[2].

Working assumption:

Take the following two alternatives as the equation for evaluation of the overall time synchronization error for TA based propagation delay compensation:

- Alt. 1:

errortotal, TAbased≤

errorBS, DL, TX+errorUE, DL,RX+errorBS, DL,TX+errorUE, DL,RX +errorUE, UL, TX +errorBS, UL,RX+errorTAindication2

  - Either option 1 or option 2 below will be applied based on the RAN4 reply to RAN1 LS R1-2102245:

    - Option 1: errorUE, DL,RX+errorUE, UL, TX <= Te

    - Option 2: errorUE, UL, TX = Te and errorUE, DL,RX is equal to a value separate from Te

- Alt. 2:

errortotal, TAbased≤

[12*]errorBS, DL, TX+errorUE, DL,RX +errorUE, UL, TX +errorBS, UL,RX+errorTAindication2

  - Either option 1 or option 2 below will be applied based on the RAN4 reply to RAN1 LS R1-2102245:

    - Option 1: errorUE, DL,RX+errorUE, UL, TX <= Te

    - Option 2: errorUE, UL, TX = Te and errorUE, DL,RX is equal to a value separate from Te

  - [Note: Alt.2 assumes that the time of PD estimation is close to the time of PD compensation, in which case the DL frame timing error and BS transmit timing error for propagation delay estimation is correlated to that for the transmission of RRC signaling carrying the reference time clock]

According to LS from RAN2, the suggested single Uu interface budgets are listed in the following table [3]

| Scenario | Single Uu interface Budget |
| --- | --- |
| Control-to-Control | ±145ns to ±275ns |
| Smart Grid | ±795ns to ±845ns |

In this contribution, we share our views on propagation delay compensation enhancements.

Parameters for propagation delay compensation evaluation

According to agreement in the previous RAN1 meeting, parameters for propagation delay evaluation are discussed. The parameter on BS transmit timing error is still open.

BS transmit timing error for smart grid use case

The maximum gNB transmit time alignment error is defined in 38.104 as shown below.

- 6.5.3.2 Minimum requirement for BS type 1-C and BS type 1-H For MIMO transmission, at each carrier frequency, TAE shall not exceed 65 ns. For intra-band contiguous carrier aggregation, with or without MIMO, TAE shall not exceed 260ns. For intra-band non-contiguous carrier aggregation, with or without MIMO, TAE shall not exceed 3µs. For inter-band carrier aggregation, with or without MIMO, TAE shall not exceed 3µs. The time alignment error requirements for NB-IoT are specified in TS 36.104 [13] clause 6.5.3.

According to the description, if 65ns is used to represent the maximum BS frame transmission error, ±32.5 ns can be interpreted as a single gNB antenna port transmit timing error for the control-to-control scenario at least for analysis of propagation delay compensation.

For smart grid use case, a single cell with MIMO transmission is typical scenario. In this contribution the same TAE value for both control-to-control and smart grid use case is assumed, i.e., TAE=32.5 ns.

Proposal 1: For both control-to-control and smart grid use case, ±32.5 ns is assumed for BS transmit timing error.

TA-based timing error evaluation

In the following section, the propagation delay budget is evaluated for TA-based method.

Figure 1. TA-based method for PDC

When a UE receives referenceTimeInfo-r16, the UE obtains TBS indicated by referenceTimeInfo-r16.

TUE=TBS+PDL

After UE does the propagation delay compensation, the estimated time clock at the UE side is

TUE+errorUE,DL,RX=TBS+errorBS, DL, TX +PDL+errorPDL

TUE=TBS+PDL+(errorBS, DL, TX+errorPDL-errorUE,DL,RX)

The total error is

errortotal, TA=errorBS, DL, TX+errorPDL-errorUE,DL,RX

It should be noted that the ‘minus’ is to avoid the double calculation for the same component in the subsequent equation.

Since the downlink propagation delay PDL is gotten from the following equation sets:

TA=PDL+PUL

Assuming PDL=PUL, the downlink propagation delay PDL is calculated as:

PDL=TA2

PDL+errorPDL=(TA+errorTA)2

Then the error of the downlink propagation delay PDL is:

errorPDL=errorTA2

The error of TA estimation shown in Fig.1 is

errorTA=errorUE,DL,RX+errorTA-Granularity+errorUE, UL, TX+errorBS, UL, RX

UE gets the reference time from the network, then it can use the current TA to determine the time clock based on the same RS set on a similar channel condition. Thus, the correlated error source can be assumed. For errorUE,DL,RX in sending reference time and TA procedure, the same positive or negative sign is assumed.

The total error with TA estimation is

errortotal, TA=errorBS, DL, TX+errorUE,DL,RX+errorUE,UL,TX+errorBS, UL, RX+errorTA-Granularity 2-errorUE,DL,RX

errortotal, TA=errorBS, DL, TX+-errorUE,DL,RX+errorUE,UL,TX+errorBS, UL, RX+errorTA-Granularity 2

Since the value for each error component can be positive or negative, i.e. ± X, thus the maximum error should be the sum of all positive values.

Summarily,

errortotal, TA=errorBS, DL, TX+Te+errorBS, UL, RX+errorTA-Granularity 2                        (1)

Parameters and results are shown in Table 1 for both control-to-control and smart grid use case.

Table 1: TA-based timing error in clock synchronisation

| Factors | Timing error type | Requirement for different SCS (kHz) (unit: ns) |  |
| --- | --- | --- | --- |
|  |  | 15 | 30 |
| 1.1 | Frame alignment error of gNB transmitter (errorBS, DL, TX) | 32.5 | 32.5 |
| 1.2 | Inaccuracy of UE transmitting (Te) | 390 | 260 |
| 1.3 | Inaccuracy of gNB detection (errorBS, UL, RX) | 100 | 100 |
| 1.4 | Inaccuracy caused by TA indication (errorTA-Granularity). | 260 | 130 |
| 1.5 | Inaccuracy caused by propagation delay when propagation delay is decided from TA. (1.2+1.3+1.4)/2 | 375 | 245 |
| Total error over a single Uu interface for control to control use case | 1.1+1.5 | 407.5 | 277.5 |
| Total error over a single Uu interface for smart grid use case | 1.1+1.5 | 407.5 | 277.5 |

Proposal 2: Equation (1) is used for TA-based PDC, i.e.,

errortotal, TA=errorBS, DL, TX+Te+errorBS, UL, RX+errorTA-Granularity 2

According to the result in the Table 1, the inaccuracy of 15/30KHz SCS after PDC cannot fulfil the TSN clock synchronization requirements for control-to-control use case.

For control-to-control use case, it can be improved by enhancements on accuracy of some parameters, e.g., to reduce the Te and TA granularity error. For the maximum budget with ±275ns in 15 kHz SCS, 192.5ns is left for the sum of reduced Te and TA granularity error. Possible improvements for the reduced Te and TA granularity error is shown in Table 2.

Table 2 The improvements on the reduced Te and TA command indication granularity for TA-based method

| SCS | Budget | The sum of Reduced Te + Reduced TA command indication granularity | Reduced Te | Reduced TA command indication granularity | Assumptions: - Equation: Equation (1) - Value for BS transmit timing error: ±32.5ns |
| --- | --- | --- | --- | --- | --- |
| 15kHz | ±275ns | 377ns | Reduced to (4/5)* Te:312ns | Reduced to (1/4)*TAG: 65ns |  |
| 30kHz | ±275ns | 357.5ns | No change: 260ns | Reduced to (3/4)*TAG: 97.5 ns |  |

Observation 1: For single Uu interface budget with ±275ns, assuming BS transmit timing error with±32.5 ns, 192.5ns is left for potential improvements to meet the accuracy requirement for TA-based PDC.

Proposal 3: For TA-based PDC method, Table 2 can be reference for potential improvements on the reduced Te and TA command indication error.

RTT-based method

Details for RTT-based method

TA-based method has been studied and applied widely in NR and previous system. For RTT-based solution, more details need discussion. At least the following aspects should be clarified.

- Exchange of time information

For RTT-based method, gNB does not know the transmitting time and receiving time of reference signal at UE side and vice versa. In order to perform propagation delay compensation at UE or gNB side, the exchange of time information is needed between gNB and UE.

- The signaling overhead

Obtaining the required precision for external clock may need quite frequent time information updates over Uu interface. Thus, the signaling overhead caused by triggering RTT-based delay measurement may be huge in order to guarantee the synchronization error is always less than synchronicity budget requirement.

- Resource allocation of reference signal.

For RTT-based method, the large bandwidth of reference signal is used to guarantee accuracy. In IIOT scenario, the larger bandwidth of reference signal is required for all UEs if high accuracy delay compensation is needed. Thus, the overhead of reference signal is larger, especially for UE specific reference signal.

Currently, RTT-based method is widely used for positioning procedure. It should be clarified whether and how the procedure/signalling for positioning can be reused for propagation delay compensation. On the other hand, if procedure/signaling for positioning is reused, it is also needed to discuss whether URLLC UEs with propagation delay compensation capability must support positioning capability.

Observation 2: For RTT-based method, more details need clarify, e.g. exchange of time information, the signaling overhead, resource allocation of reference signal, and so on.

RTT-based timing error evaluation

For RTT-based method, propagation delay compensation at gNB side or at UE side have no obvious difference. For compensation at gNB side, UE reports Rx-Tx time difference to gNB and gNB executes PDC, which may have the potential RAN3 impact. For compensation at UE side, UE receives signalling of Rx-Tx time difference from gNB and UE executes PDC. The compensation at UE side has no RAN 3 impact compared to compensation at gNB side.

In this section, taken RTT-based gNB compensation as an example, the total error for RTT-based PDC is

errortotal, RTT=errorBS, DL, TX+errorPDL,  RTT-errorUE, DL, RX

Similar with TA-based method, the ‘minus’ in equation is only to avoid the double calculation for the same component. The relation of transmitting and receiving signals for gNB and UE is shown in Fig.2.

Figure 2 The relation of transmitting and receiving signals between gNB and UE

Double of propagation delay equal to ∆t1+∆t2, where,

∆t1=TUE,Rx+errorUE,DL,Rx-(TBS,Tx+errorBS,DL,Tx)

and

∆t2=TBS,Rx+errorBS,UL,Rx-(TUE,Tx-errorUE,UL,Tx)

Therefore,

∆t1+∆t2 =TBS,Rx-TBS,Tx)+(TUE,Rx-TUE,Tx+errorUE,DL,Rx-errorBS,DL,Tx+ errorBS,UL,Rx+errorUE,UL,Tx

When using d=(∆t1+∆t2)2 to estimate propagation delay, the uncertainties for PL are

errorPDL,  RTT=-errorBS, DL, Tx+errorUE,DL,Rx+errorUE, UL, Tx+errorBS, UL, Rx+errorBS,RxTxDiff+errorUE, RxTxDiff+errorRxTxDiff, report+errorindication2

Where errorBS,RxTxDiffand errorUE, RxTxDiff express Rx-Tx time difference inaccuracy at gNB and UE side, respectively. errorRxTxDiff, reportis the error caused by the reporting granularity and errorindicationis the error caused by the indication granularity.

Then, the total uncertainties for RTT-based method can be derived by:

errortotal, RTT=errorBS, DL, Tx+-errorBS, DL, Tx+errorUE,DL,Rx+errorUE, UL, Tx+errorBS, UL, Rx+errorBS,RxTxDiff+errorUE, RxTxDiff+errorRxTxDiff, report+errorindication 2-errorUE,DL,Rx

errortotal, RTT=

errorBS, DL, Tx-errorUE,DL,Rx+errorUE, UL, Tx+errorBS, UL, Rx+errorBS,RxTxDiff+errorUE, RxTxDiff+errorRxTxDiff, report++errorindication 2          (2)

Since the value for each error component can be positive or negative, i.e. ± X, the maximum error should be the sum of all positive values.

errortotal, RTT=

errorBS, DL, Tx+errorUE,DL,Rx+errorUE, UL, Tx+errorBS, UL, Rx+errorBS,RxTxDiff+errorUE, RxTxDiff+errorRxTxDiff, report+errorindication 2                                                                                                                                                (3)

According to the conclusion in RAN 4[4], ±[137+]ns is applied for errorUE, RxTxDiff for 15kHz in fading channel based on TDL-A (30 ns delay spread, 5Hz) channel for FR1. We assume the PDC evaluation can base on FR1 firstly. However, for FR1, the value is not determined for 30kHz case. Even for 15kHz, the  value is still open.

For errorBS,RxTxDiff, only the accuracy requirements for AWGN propagation conditions are provided for both 15kHz and 30kHz. There are no available values for fading channel.

For errorRxTxDiff, report, the error caused by the reporting granularity is from Tc to 32Tc according to TS38.133. Here we use 32Tc as errorRxTxDiff, report for evaluation. The inaccuracy due to the granularity of reporting the Rx-Tx time difference = 32*Tc = 16 ns. For errorindication in evaluation, 5ns is assumed.

Parameters for RTT-based method are shown in Table 3 for both control-to-control and smart grid use case.

Table 3: RTT-based timing error in clock synchronisation

| Factors | Timing error type | Requirement for different SCS (kHz) (unit: ns) |  |
| --- | --- | --- | --- |
|  |  | 15 kHz | 30 kHz |
| 1.1 | Frame alignment error of gNB transmitter (errorBS, DL, Tx) | 32.5 | 32.5 |
| 1.2 | Inaccuracy caused by UE detection (errorUE,DL,Rx) | 100 | 100 |
| 1.3 | Difference between Rx and Tx at UE side (errorUE,RxTx diff) | [137+], is FFS | TBD in fading channel |
| 1.4 | Difference between Rx and Tx at gNB side (errorBS,RxTx diff) | TBD in fading channel | TBD in fading channel |
| 1.5 | Inaccuracy of UE transmitter errorUE,UL,Tx | TBD | TBD |
| 1.6 | Inaccuracy of reporting errorRxTxDiff, report | 16 | 16 |
| 1.7 | Inaccuracy of gNB detection (errorBS, UL, Rx) | 100 | 100 |
| 1.8 | Inaccuracy of errorindication | 5 | 5 |

Before evaluation of RTT-based PDC, the following parameters should be determined:

  - errorUE,RxTx diff for 30kHz and  value for 15kHz

  - errorBS,RxTx diff for fading channel

  - errorUE,UL,Tx

Observation 3: Before evaluation of RTT-based PDC, the following parameters should be determined:

  - errorUE,RxTx diff for 30kHz and  value for 15kHz

  - errorBS,RxTx diff for fading channel

  - errorUE,UL,Tx

Conclusions

Based on the analysis given above, we have the following Observations and Proposals:

Observation 1: For single Uu interface budget with ±275ns, assuming BS transmit timing error with±32.5 ns, 192.5ns is left for potential improvements to meet the accuracy requirement for TA-based PDC.

Observation 2: For RTT-based method, more details need clarify, e.g. exchange of time information, the signaling overhead, resource allocation of reference signal, and so on.

Observation 3: Before evaluation of RTT-based PDC, the following parameters should be determined:

  - errorUE,RxTx diff for 30kHz and  value for 15kHz

  - errorBS,RxTx diff for fading channel

  - errorUE,UL,Tx

Proposal 1: For both control-to-control and smart grid use case, ±32.5 ns is assumed for BS transmit timing error.

Proposal 2: Equation (1) is used for TA-based PDC, i.e.,

errortotal, TA=errorBS, DL, TX+Te+errorBS, UL, RX+errorTA-Granularity 2

Proposal 3: For TA-based PDC method, Table 2 can be reference for potential improvements on the reduced Te and TA command indication error.

Table 2 The improvements on the reduced Te and TA command indication granularity for TA-based method

| SCS | Budget | The sum of Reduced Te + Reduced TA command indication granularity | Reduced Te | Reduced TA command indication granularity | Assumptions: - Equation: Equation (1) - Value for BS transmit timing error: ±32.5ns |
| --- | --- | --- | --- | --- | --- |
| 15kHz | ±275ns | 377ns | Reduced to (4/5)* Te:312ns | Reduced to (1/4)*TAG: 65ns |  |
| 30kHz | ±275ns | 357.5ns | No change: 260ns | Reduced to (3/4)*TAG: 97.5 ns |  |

Reference

[1] R1-2104171(R4-2105850) Reply LS on UE transmit timing error

[2] Chairman notes RAN1#104b e-meeting

[3] R2-2010837 Reply LS on propagation delay compensation enhancement

[4] R4-2108300 Big CR 38133-NR positioning RRM perf_R16