3GPP TSG RAN WG1#106-e		R1-2106609

e-Meeting, August 16th – 27th, 2021

Source:	vivo

Title:	Additional UE Power Model for Downlink Reception

Agenda Item:	8.7.3

Document for:	Discussion and Decision

- Background

The current UE power model is defined in TR38.840 from Rel-16(see annex 1). The power consumption for deep sleep is regarded as ‘1’, and other state power consumption is relative to this. However, the current power model is based on the assumption that UE turn on the transceiver/PA/modem, it does not consider low power reception, such as RF-front-end receiver or even envelope detector. These new receiver architectures are useful for power saving perspective and should be addressed in the future.

- Discussion

  - model of low power modes

R1-1811281discusses the depth of sleep mode. In principle, to minimize power consumption, when a hardware module that consumes power is not currently being used, it should be turned off or at least put into low power state.

The following categories are described in R1-1811281 for different depth of sleep applicable to the hardware modules in a UE modem. In the last row, PDCCH-only monitoring is added for comparison

Table 1 usage of hardware modules

| Hardware subsystem | Oscillator | RF / front-end | Baseband modem | Control processor, DDR memory | Relative Power |
| --- | --- | --- | --- | --- | --- |
| Deep sleep | Off | Off | Off | Low power (D) | 1 |
| Light sleep | On (A) | Off | On (low-speed process) (C1) | Active (E) | 20 |
| Microsleep | On (A) | Off (partial) | On (low-speed process) (C1) | Active (E) | 45 |
| PDCCH-only | On (A) | On (B) | On (high-speed process) (C2) | Active (E) | 100 |

The relative power of these low power modes is captured in TR38.840.

  - additional low power modes and models

It is expected that in the future discussion, more low power receivers can be considered. The hardware module that depending on the assumptions for the low power receiver.

[3][4] provides some low power receivers for wake-up signal monitoring, which can be universal for downlink reception. For example,

- RF-front-end receiver: a RF front-end receiver may not require digital receivers or digitize the RF signal directly, without down-conversion to an IF.

- Envelope detector, an envelope detector can shut down the power consumption 100x-1000x times, i.e., 0.01~0.1 milliwatts. This is achieved by OOK waveform which only require simple energy accumulation and comparator as well as other simplified designs. High accuracy oscillator is not required to be operated. Thus, the power consumption is able to be reduced to an extreme low level which can be applied to many battery-driven cases. However, envelope detection may cause some sensitivity degradation, e.g., down to 60 dB~80dB, and how to deal with the sensitivity degradation can be studied further. The ultra-low power receiver for receiving wake-up signal can be separated with main radio, such that the main radio can be turned off.

The following table is a description of the possible hardware modules statue for -	RF-front-end receiver and envelope detector receiver.

Table 2

| Hardware subsystem | Oscillator | RF / front-end | Baseband modem | Control processor, DDR memory | Relative Power | Note: |
| --- | --- | --- | --- | --- | --- | --- |
| RF front-end receiver | On (A) | On (B) | Off | Off or low power (D) | X |  |
| Envelope detector | Off | Off | Off | Off | Y |  |

  - Power model for additional low power receiver

In order to infer X and Y value, we made the following assumptions,

Based on some test of UE, for example, for a PDCCH-only reception, the power consumption for modem/processor part is 1-2 times than the RF part, which means

C2 + E = 1.5 (A+B)																(1)

And based on relative power for PDCCH-only reception,

C2 + E  + A + B = 100																(2)

Then by jointly considering (1) and (2), A + B  = 40

Also, D can be inferred as 1. Then the total relative power for RF front-end receiver X = A + B + D = 41

For envelope detector, based on the reference document in [3], the overall power consumption can be less than 0.01 – 0.1 mW.

| receivers | Relative Power | Power consumption (mW) |
| --- | --- | --- |
| NR IDLE (1280ms) | 1.48[1] | 40mW[2] |
| NR IDLE (320ms) | 2.93[1] | 68mW[2] |
| Almost-zero power (AZP) wake-up receiver: Envelope detector (external receiver) | Y | 0.01 - 0.1 mW[3] |

Note:

[1] see annex 3: Calculation of the power consumption based on TR 38.840

[2] see annex 2: Test of NR IDLE model power consumption.

[3] see reference in [3]

Comparing the relative power and tested power consumption for IDLE state, we can infer

- Y = 0.0004 ~ 0.004

  - update for TR38.840

The follow update is proposed for TR38.840 as the additional power model for low power receiver.

------------------------------------TR38.840 Section 8.1.1------------------------------------

…

Table 18: UE power consumption model for FR1

| Power State | Characteristics | Relative Power |
| --- | --- | --- |
| Deep Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not be maintained. | 1 (Optional: 0.5) |
| Light Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. | 20 |
| Micro sleep | Immediate transition is assumed for power saving study purpose from or to a non-sleep state | 45 |
| PDCCH-only | No PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot. | 100 |
| SSB or CSI-RS proc. | SSB can be used for fine time-frequency sync. and RSRP measurement of the serving/camping cell. TRS is the considered CSI-RS for sync. FFS the power scaling for processing other configurations of CSI-RS. | 100 |
| PDCCH + PDSCH | PDCCH + PDSCH. ACK/NACK in long PUCCH is modeled by UL power state. | 300 |
| UL | Long PUCCH or PUSCH. | 250 (0 dBm) 700 (23 dBm) |

Table 18-a: additional UE power consumption model for low power receiver for FR1

| Power State | Characteristics | Relative Power |
| --- | --- | --- |
| RF-front-end Rx | Using RF-front-end receiver for, e.g., sequence detection. FFS how much time for UE switch from RF-front-end Rx to normal Rx state(defined in Table 18) | 41 |
| Envelope detector Rx | Using an external envelope detector receiver for, e.g., sequence detection. - When the main radio is switch off, FFS how much time for UE turn on the main radio for normal Rx (defined in Table 18) - When the main is not switch off, the normal Rx (defined in Table 18) is not impacted. | Typical 0.001 (0.0004 ~ 0.004) |

…

- Conclusion

Additional power models for low power downlink reception is proposed as follows. TR38.840 is proposed to be updated to include this.

| Power State | Characteristics | Relative Power |
| --- | --- | --- |
| RF-front-end Rx | Using RF-front-end receiver for, e.g., sequence detection. FFS how much time for UE switch from RF-front-end Rx to normal Rx state(defined in Table 18) | 41 |
| Envelope detector Rx | Using an external envelope detector receiver for, e.g., sequence detection. - When the main radio is switch off, FFS how much time for UE turn on the main radio for normal Rx (defined in Table 18) - When the main is not switch off, the normal Rx (defined in Table 18) is not impacted. | Typical 0.001 (0.0004 ~ 0.004) |

References

- 3GPP TR 38.840 Study on UE power saving

- R1-1811281, UE Power Saving Evaluation Methodology, Qualcomm Incorporated

- RWS-210168, Motivation for new study item on ultra-low power wake up signal in Rel-18, vivo, Spreadtrum, Guangdong Genius

- RWS-210169, New SID: Study on ultra-low power wake up signal in Rel-18, vivo

Annex 1 TR38.840 UE power consumption model

Table 1: UE power consumption model for FR1

| Power State | Characteristics | Relative Power |
| --- | --- | --- |
| Deep Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. Accurate timing may not be maintained. | 1 (Optional: 0.5) |
| Light Sleep | Time interval for the sleep should be larger than the total transition time entering and leaving this state. | 20 |
| Micro sleep | Immediate transition is assumed for power saving study purpose from or to a non-sleep state | 45 |
| PDCCH-only | No PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot. | 100 |
| SSB or CSI-RS proc. | SSB can be used for fine time-frequency sync. and RSRP measurement of the serving/camping cell. TRS is the considered CSI-RS for sync. FFS the power scaling for processing other configurations of CSI-RS. | 100 |
| PDCCH + PDSCH | PDCCH + PDSCH. ACK/NACK in long PUCCH is modeled by UL power state. | 300 |
| UL | Long PUCCH or PUSCH. | 250 (0 dBm) 700 (23 dBm) |

Table 2: UE power consumption model for FR1

| Power State | Characteristics | Relative Power |  |
| --- | --- | --- | --- |
|  |  | FR1 | FR2 |
| PDCCH-only | No PDSCH and same-slot scheduling; this includes time for PDCCH decoding and any micro-sleep within the slot. | 100 | 175 |
| SSB or CSI-RS proc. | SSB can be used for fine time-frequency sync. and RSRP measurement of the serving/camping cell.. TRS is the considered CSI-RS for sync. FFS the power scaling for processing other configurations of CSI-RS. (Note 2 SSBs in a slot for the ref. config.) | 100 | 175 |
| PDCCH + PDSCH | PDCCH + PDSCH. ACK/NACK in long PUCCH is modeled by UL power state. | 300 | 350 |
| UL | Long PUCCH or PUSCH. | 250 (0 dBm) 700 (23 dBm) | 350 (FFS Tx power level) |

-	Sleep states power level, energy overhead, and transitions are same as FR1.

Annex 2 Test of NR IDLE model power consumption

(a) NR UE power consumption, RF + baseband @ RRC IDLE : Band N41, 20MHz, DRX cycle 1.28s

(b) zoom in: around each paging reception

Figure 1. UE power consumption (x-axis; Time(s), y-axis, current(mA))

Annex 3 Calculation of the power consumption for paging

For PDCCH and SSB monitoring based on power model defined in TR38.840

- T_PDCCH = 0.5 ms * N_SSB, P_PDCCH = 100

- T_SSB = 0.5 ms* N_SSB / 2, P_SSB = 100

- T_micro_sleep = 2 ms, P_micro_sleep = 45

- T_deep_sleep_transition = 20 ms, P_transition = 450

For one paging cycle 320ms, assuming 2ms offset between SSB and PO,

- T_deep_sleep = 297 ms, P_deep_sleep = 1

Then the average power

= TPDCCH×PPDCCH+TSSB×PSSB+Tmicro_sleep×Pmicro_sleep+Ptransition+Tdeep_sleep×Pdeep_sleepTPDCCH+TSSB+Tmicro_sleep+Ttransition+Tdeep_sleep

= 2.93

For one paging cycle 1280ms, assuming 2ms offset between SSB and PO,

- T_deep_sleep = 1257 ms, P_deep_sleep = 1

Then the average power

= TPDCCH×PPDCCH+TSSB×PSSB+Tmicro_sleep×Pmicro_sleep+Ptransition+Tdeep_sleep×Pdeep_sleepTPDCCH+TSSB+Tmicro_sleep+Ttransition+Tdeep_sleep

= 1.48