3GPP TSG RAN WG1 #106-e		R1-2106612

e-Meeting, August 16th – 27th, 2021

Source:	vivo

Title:	Discussion on PUSCH TB processing over multiple slots

Agenda Item:	8.8.1.2

Document for:	Discussion and Decision

- Introduction

In previous RAN1 meetings, TB processing over multiple slots (TBoMS) was discussed, and several agreements were made. We will further discuss on the detailed solutions for TBoMS in this contribution.

- Definition of TBoMS and RV rate-matching

In previous meetings, the definition of TBoMS, including definition of transmission occasion of TBoMS, and RV mapping metholds were discussed, and following agreements were made.

Agreement: - The structure of TBoMS will be according to only one of these two options (to be down-selected in RAN1#106-e) - Option 3, if a design based on single RV is adopted. - Option 4, if a design based on different RVs is adopted. - FFS: other details, e.g., rate-matching, TBS determination, collision handling, etc. - The single RV is not constrained to have only the same coded bits in each slot or in each TOT - The concept of TOT as per the corresponding Working assumption is used to define Option 3 and Option 4 and may or may not be used to design other details, e.g., rate-matching, TBS determination, collision handling and so on. Agreement: The following three options for rate-matching for TBoMS are considered for down-selection during RAN1 #106-e, where only one option will be selected: - Option a: Rate-matching is performed per slot; - Option b: Rate matching is performed continuously across all the allocated slot(s) per TOT; - Option c: Rate matching is performed continuously across all the allocated slots/TOTs for TBoMS Note: “rate-matching is performed per X” means that the time unit for the bit selection and bit interleaving is X. Note2: the above 3 options imply that the UL resource in the time unit may or may not be consecutive (depending on the given option)

The major difference between TBoMS designs is how the coded bits is rate matched in resources for TBoMS, rather single RV or different RVs over the multiple TOTs.

In the three options for rate-matching, rate-matching performed per slot, i.e., Option-a results in similar channel structure as rel-16 PUSCH, the differences maybe only reflected in TB size determination with additional scaling. If this method is adopted, the TBoMS over multiple slots can be regarded as slot based type-A PUSCH repetition, and definition of ToT is no longer necessary.

RV rate-matching in Option-b and Option-C has been extensively discussed in previous meetings. The core issue is whether RV could rate matching across non-consecutive resources. Option-b is similar to the mechanism for a normal PUSCH generation, which is further extended to TBoMS, and RV can be refreshed per TOT instead of refreshing per slot. For a different TOT, a new RV can be transmitted. Thus, repetition + TBoMS can be implicitly supported. For Option-c, RV is rate matched continuously across multiple TOTs which could be non-consecutive. It means UE need to map RV continuously across non-consecutive slots, and the UE has to memorize segment of the TB after mapping partial TB on slot n until next transmission occasion on slot n+x where x can be few slots. From implementation wise it is extra burden on UE. Note that, even for type-B PUSCH repetition, continuous RV mapping across non-consecutive transmission is not required, and RV is refreshed based on RV cycling mechanism after each segement.

Although the Option-b may result in higher code rate for a TBoMS, and the TB would be not decodable within a single TOT if the TB size is determined based on slots in multiple TOTs, NW can properly set the RV sequence and the TB size to make sure the TB can be decoded over the multiple TOTs. Even in Option-C, UE has to receive multiple TOTs to decode the TBoMS. Therefore, there is no big difference between these Option-b and Option-c, from NW receiver perspective. While Option-b is less complicated from UE transmitter perspective.

Observation 1: Option-C, i.e., RV mapping across non-consecutive slots leads to extra burden in UE implementation.

To support TBoMS with lower complexity, Option-b can be considered as baseline rate-matching method for TBoMS. Option-c can also be supported by higher capability UEs, UE can report capability on which rate-matching method is supported

Another issue is whether a single RV (Option-3) or different RV (Option-4) are used across multiple TOTs. For rate-matching Option-b, RV is refreshed per TOT, same RV across TOTs can be achieved by proper RV sequence configuration. It is not necessary to limit the RV across TOTs to be same, hence option-4 is preferred. For rate-matching Option-c, RV is rate matched continuously across multiple TOTs. It is natural that the same RV should be used across the TOTs within a TBoMS. Different RVs can be supported across different repetitions, if repetition on top of TBoMS is supported. Hence, Option-3 and Option-4 should be supported together with rate-matching Option-c and Option-b, respectively. The illustration of {Option-3 + Option-c} and {Option-4 + Option-b} are shown in Figure 1.

Figure 1. Potential channel structure for TBoMS

Proposal 1: Both rate-matching per TOT (Option-b) and rate matching across TOTs (Option-c) can be supported based on UE capability reporting.

- Rate matching per TOT (Option-b) can be considered as baseline capability for TBoMS.

Proposal 2: TBoMS definition Option-3 is supported together with rate-matching method Option-c, and TBoMS definition Option-4 is supported together with rate-matching method Option-c.

- Time domain resources for TBoMS

In RAN1#105e, the following agreements are made on time domain resources for TBoMS.

Agreement: Time domain resource determination for TBoMS can be performed only via PUSCH repetition Type A like TDRA. - FFS: details - FFS: whether or not optimizations for time domain resource determination are necessary for allocating resource in the S slots (for the unpaired spectrum case) Agreement: Number of slots allocated for TBoMS is determined by using a row index of a TDRA list, configured via RRC. - FFS: details. Working assumption A transmission occasion for TBoMS (TOT) is constituted of at least one slot or multiple consecutive physical slots for UL transmission - FFS: whether the concept of TOT will be used for designing aspects related to signal generation, e.g., rate-matching, power control, etc. - FFS: whether such concept will be specified or not.

In this section, we will discuss the details of time domain resource determination for TBoMS.

  - TDRA table for TBoMS

In Rel-16 TDRA table, the K2, S, L and repetition number are configured to determine the time domain resources for type-A PUSCH repetition. For TBoMS, at least K2, S, L can be kept to indicate the time domain resource within slot which constructs TBoMS.

For a TBoMS, number of slots for a TOT and number of TOTs should be additionally indicated to UE to construct a TBoMS. These two parameters can be configured within entries in TDRA table. Multiple values can be configured in TDRA table and one can be indicated by DCI. While with too many parameters linked with TDRA table will limit the flexibility of resource allocation within a slot, i.e., options for {K2, S, L} will be further limited.

Between the two parameters, the number of slots in a TOT has finer granularity compared with number of TOTs in a TBoMS. NW can dynamically determine the number of slots for a TOT in slot level if this parameter is included in TDRA table. While a single value can be configured for the number of TOTs for a TBoMS, which is a separate parameter from TDRA in PUSCH configuration. An example of modified TDRA table is shown in Table 1.

Table 1. Example of TDRA table for TBoMS

| Row index | PUSCH mapping type | K2 | S | L | Number of slots for TOT(X) | Number of repetitions(Y) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Type-A | j | 0 | 14 | X1 | Y1 |
| 2 | Type-B | j | 2 | 12 | X2 | Y2 |
| …… | …… | …… | …… | …… | …… | …… |

Proposal 3: Number of slots in a TOT is associated with entries in the TDRA table, and a single value for number of TOTs for a TBoMS is separately configured.

Repetition for TBoMS can be kept in the TDRA table for TBoMS, as discussed in section 3.2.

  - Repetition on top of TBoMS

In previous meetings, whether to support repetition on top of TBoMS was also discussed. In our understandings, TBoMS with repetition should be supported. Otherwise, TBoMS cannot achieve better coverage compared with legacy PUSCH repetition schemes.

The repetition number can be indicated in the TDRA table, as in rel-16. Since a single TBoMS can be composed of M TOTs, the number of repetitions for TBoMS can be regarded as repetition times for the M TOTs. In other words, if TBoMS contains X TOTs and there are M repetitions allocated for TBoMS, TBoMS is transmitted over M*X TOTs.

Proposal 4: Repetition on top of TBoMS is supported, and the repetition number M is indicated in TDRA table.

- Where M is the repetition times of X TOTs which composes the TBoMS.

  - Definition of TOT

In current PUSCH channel design, transmission occasion concept is used in physical channel generation, although no explicit wording of ‘transmission occasion’ was found in specification. For example, in Type-A PUSCH repetition, the transmission occasion can be regarded as the contiguous L symbols within a slot, which is used for TB size determination and RV refreshing. Specifically, TB size is determined based on resources in a Tx occasion, and RV is refreshed across different transmission occasions.

Observation 2: Concept of transmission occasion is used in current mechanism for PUSCH channel generation.

- TB size is determined based on resources in a Tx occasion, and

- RV is refreshed across transmission occasions.

Similarly, TBS size determination, RV determinatin and refreshing should also be considered for TBoMS, as discussed in previous section, and section 4 and section 5, definition of TOTs useful at least for RV rate-matching, TB size determination and UCI multiplexing.

Proposal 5: Concept of TOT should be specified at least for the following purposes

- RV refreshing;

- UCI multiplexing;

- TB size determination.

- TB size determination for TBoMS

In RAN1#105e, following agreements are made on TB size determination for TBoMS.

Agreement: The following approach is used to calculate NInfo for TBoMS: - Approach 2: Based on the number of REs determined in the first L symbols over which the TBoMS transmission is allocated, scaled by K≥1. - FFS: the definition of K. L is the number of symbols determined using the SLIV of PUSCH indicated via TDRA FFS: impacts and further details if repetitions of TBoMS is supported. FFS: whether the symbols over which the TBoMS transmission is allocated are the same or can be different from the symbols over which the TBoMS transmission is performed, and details on how to handle such scenarios. Working assumption: Agreement: For TBS determination of TBoMS: - NohPRB is configured by xOverhead and represents the overhead per slot. - NohPRB is assumed to be the same for all the slots over which the TBoMS transmission is allocated. Note: xOverhead configuration is as per Rel-15/16.

In last meeting, it has been agreed that the NInfo is calculate by scaling the value determined from L symbols indicated by TDRA table. The remaining issue is determining the definition of K. K should be number of slots considering only type-A repetition like TDRA is supported, and the number of symbols in each slot is the same. For TBoMS rate-matching per TOT, K can be number of slots of a TOT to avoid high code rate within a TOT. While for TBoMS rate-matching continuously across multiple TOTs, K can be number of slots for multiple TOTs which construct a TBoMS, and larger TB size can be supported.

Proposal 6: For definition of scaling factor K, it can be

- Number of slots in a TOT, if rate matching is performed per TOT;

- Number of slots of multiple TOTs which construct a TBoMS, if rate-matching is performed across the multiple TOTs.

Based on the discussion above, the procedure for TB size determination is summarized as follows,

Step 1: A UE first determines the number of REs allocated for PUSCH within a PRB (NRE') by NRE'=NscRB⋅Nsymbsh-NDMRSPRB-NohPRB

Step 2: A UE determines the total number of REs allocated for PUSCH (NRE) by NRE=min156,NRE'⋅nPRB, where nPRB is the total number of allocated PRBs for the UE.

Step 3: Unquantized intermediate variable (Ninfo) is obtained by Ninfo=K·NRE·R·Qm·υ

    - Where K is the number of slots for a TOT or multiple TOTs which construct a TBoMS.

Step 4: Calculate TB size based on Ninfo as that in Rel-16.

- UCI multiplexing on TBoMS

In Rel-16, transmission parameter of PUSCH can be changed if overlapping with PUCCH. For UCI multiplexing on TBoMS, the following issue should be discussed.

- Whether the multiplexing timeline is check per slot or per TOT/TBoMS

- How to determine the number of resources in TBoMS for the multiplexed UCI.

For UCI multiplexing on legacy slot based PUSCH, timeline should be fulfilled if UCI is multiplexed on the PUSCH. The timeline is checked relative to S0, which is the start of PUSCH or PUCCH. While for TBoMS, the time domain unit of a RV is TOT or multiple TOTs which construct a TBoMS. It is natural that the timeline for UCI multiplexing should be checked Per TOT/TBoMS, instead of checking timeline per slot for legacy PUSCH. The symbol used for determining S0 is the starting symbol of a TOT or a TBoMS. The granularity to determine the starting symbol for time domain resources for TBoMS can be different for rate-matching method Option-b and Option-C. If the RV is rate-matched per TOT, symbol used for determine S0 is the starting symbol of a TOT. While if RV is rate-matched continuously across multiple TOTs of a TBoMS, the symbol used for determine S0 is the starting symbol of the multiple TOTs for a TBoMS.

Proposal 7: For UCI multiplexing on PUSCH with TB processing over multiple slots, the starting symbol for TBoMS used for determining S0 is the starting symbol of a TOT or a TBoMS.

In Rel-15/16, the resources on PUSCH for UCI multiplexing is derived based on RRC parameter beta-offset, scaling (α) and PUSCH length. The number of symbols for UCI multiplexing on a PUSCH is derived based on the following equation.

Q'ACK=minOACK+LACK⋅βoffsetPUSCH⋅l=0Nsymb,allPUSCH-1MscUCIlr=0CUL-SCH-1Kr,α⋅l=l0Nsymb,allPUSCH-1MscUCIl

where Nsymb,allPUSCH is the total number of OFDM symbols of the PUSCH, including all OFDM symbols used for DMRS. While for TBoMS, the number of symbols for a PUSCH transmission is far more than legacy PUSCH occasion limited within a slot, which may lead to increased number of symbols for UCI multiplexed in TBoMS. Thus, the reliability for the UCI multiplexed on the PUSCH over multiple slots would be higher compared with UCI multiplexed on PUSCH which is limited within a slot, if the same set of beta-offset and scaling (α) for slot based PUSCH is reused for TBoMS. However, higher performance of UCI on the PUSCH is not required even if it is piggybacked on a TBoMS, and the target performance of UCI should be independent of number of symbols for TBoMS. Therefore, excessive occupation of the resources in TBoMS should be avoided.

For the amount of resources in TBoMS, the number of resources can be scaled by 1/K, where K is the number of slots for a TOT/TBoMS. The scaling of amount of UCI resources can be realized by re-defining the parameter Nsymb,allPUSCH as number of symbols per slot allocated for TBoMS. Another option is Nsymb,allPUSCH is still number of all symbols for PUSCH-TBoMS, i.e., total number of symbols across K slots for a TOT/TBoMS, while beta-offset and scaling(α) for slot based PUSCH should be scaled by 1/K. With these modifications applied for TBoMS, the amount of resources for UCI in TBoMS can be almost the same as that multiplexed in a single slot PUSCH.

Proposal 8: For UCI multiplexing on TBoMS, the number of modulated symbols in the TBoMS for UCI should be same/close to that multiplexed in a single slot PUSCH, following options can be considered

- Opt-1: Re-define the parameter Nsymb,allPUSCH as number of symbols per slot allocated for TBoMS;

- Opt-2: BetaOffset and scaling (α) is scaled by 1/K, where K is the number of slots for a TOT or TBoMS.

- MIMO layers for TBoMS

Both PUSCH repetition and PUSCH with TB processing over multiple slots are motivated for improved reliability, and for PUSCH repetition Type A, in case number of repetitions K>1, the PUSCH is limited to a single transmission layer in Rel-16. Multi-layer PUSCH is not a typical use case for PUSCH-TBoMS, which is motivated for coverage enhancement rather than higher data rate. Therefore, PUSCH with TB processing over multiple slots should also be limited to single transmission layer.

Proposal 9: PUSCH with TB processing over multiple slots should be limited to single transmission layer.

- Conclusion

In this contribution, we discussed the potential issues for PUSCH with TB processing over multiple slots. Based on the discussion in previous sections, and we have the following observations and proposals:

Observation 1: Option-C, i.e., RV mapping across non-consecutive slots leads to extra burden in UE implementation.

Proposal 1: Both rate-matching per TOT (Option-b) and rate matching across TOTs (Option-c) can be supported based on UE capability reporting.

- Rate matching per TOT (Option-b) can be considered as baseline capability for TBoMS.

Proposal 2: TBoMS definition Option-3 is supported together with rate-matching method Option-c, and TBoMS definition Option-4 is supported together with rate-matching method Option-c.

Proposal 3: Number of slots in a TOT is associated with entries in the TDRA table, and a single value for number of TOTs for a TBoMS is separately configured.

Proposal 4: Repetition on top of TBoMS is supported, and the repetition number M is indicated in TDRA table.

- Where M is the repetition times of X TOTs which composes the TBoMS.

Observation 2: Concept of transmission occasion is used in current mechanism for PUSCH channel generation.

- TB size is determined based on resources in a Tx occasion, and

- RV is refreshed across transmission occasions.

Proposal 5: Concept of TOT should be specified at least for the following purposes

- RV refreshing;

- UCI multiplexing;

- TB size determination.

Proposal 6: For definition of scaling factor K, it can be

- Number of slots in a TOT, if rate matching is performed per TOT;

- Number of slots of multiple TOTs which construct a TBoMS, if rate-matching is performed across the multiple TOTs.

Proposal 7: For UCI multiplexing on PUSCH with TB processing over multiple slots, the starting symbol for TBoMS used for determining S0 is the starting symbol of a TOT or a TBoMS.

Proposal 8: For UCI multiplexing on TBoMS, the number of modulated symbols in the TBoMS for UCI should be same/close to that multiplexed in a single slot PUSCH, following options can be considered

- Opt-1: Re-define the parameter Nsymb,allPUSCH as number of symbols per slot allocated for TBoMS;

- Opt-2: BetaOffset and scaling (α) is scaled by 1/K, where K is the number of slots for a TOT or TBoMS.

Proposal 9: PUSCH with TB processing over multiple slots should be limited to single transmission layer.

- References

- RP-202928, New WID on NR coverage enhancements, RAN#90.

- R1-2102241, FL summary of TB processing over multi-slot PUSCH (AI 8.8.1.2)