3GPP TSG RAN WG1 #106-e		R1-2106697

e-Meeting, August 16th – 27th, 2021

Agenda item:	8.3.1.1

Source: 	Spreadtrum Communications

Title: 	Discussion on HARQ-ACK feedback enhancements for Rel-17 URLLC

Document for:	Discussion and decision

Introduction

During RAN 88e meeting, a revised WID of Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR was approved. The first objective is about whether some physical layer feedback enhancements should be studied and specified for URLLC, including HARQ-ACK and CSI feedback enhancements. The objective is showed below.

- Study, identify and specify if needed, required Physical Layer feedback enhancements for meeting URLLC requirements covering

    - UE feedback enhancements for HARQ-ACK [RAN1]

    - CSI feedback enhancements to allow for more accurate MCS selection [RAN1]

Note: DMRS-based CSI feedback is not in scope of this WI

This contribution provides initial thoughts on Physical Layer UE feedback enhancements for HARQ-ACK [RAN1].

Discussion

Support avoiding SPS HARQ-ACK dropping for TDD

- Limit on the maximum deferral

In RAN1#104b-e meeting, it has been agreed to discuss HARQ-ACK feedback enhancements for TDD DL SPS with high priority, and following agreements have been made:

Agreements: For SPS HARQ-ACK deferral, support a limit on the maximum deferral of SPS HARQ in terms of k1def  or k1+ k1def

  - FFS: limitation given by a maximum value of k1def or a maximum of k1eff =k1+ k1def

  - FFS how the limitation is determined (e.g. by K1 set(s) or RRC configured limit)

Agreement: For SPS HARQ-ACK deferral, the limit on the maximum deferral of SPS HARQ is defined in terms of k1eff =k1+ k1def.

Regarding the second FFS of the first agreement, the maximum deferring value can be configured by RRC signalling. Note that multiple traffic types exist in URLLC/IIOT systems, which may have different QoS and survival time requirements, gNB can configure different SPS indexes for each traffic type, and it is natural that RRC can configure different maximum deferring values for different SPS indexes. In addition, if RRC signalling does not exist, the maximum value of K1 set can be used as a maximum deferring value.

Proposal 1. For SPS HARQ-ACK deferral, a maximum deferring value can be configured by RRC signalling. If the RRC signalling does not exist, the maximum value of K1 set can be used as a maximum deferring value.

- Initial slot handling for SPS HARQ-ACK deferral

In last meeting, it has been agreed to further discuss the initial slot handling issue, and following agreements have been made:

Agreement: Restrict the further discussions on the initial slot handling for SPS HARQ-ACK deferral to the identified alternatives Alt. 1, Alt. 1A and 2.

The alternatives are shown below:

- Alt. 1: Deferral only, if the SPS HARQ-ACK in the initial slot/sub-slot cannot be transmitted as the resulting PUCCH resource for transmission using the PUCCH by SPS-PUCCH-AN-List-r16 or n1PUCCH-AN is not valid

- Alt. 1A: Deferral only, if the PUCCH resource configured by SPS-PUCCH-AN-List-r16 or n1PUCCH-AN for the HARQ-ACK transmission assuming SPS HARQ-ACK only is not valid in the initial slot/sub-slot

- Alt. 2: Deferral only, if the SPS HARQ-ACK in the initial slot/sub-slot cannot be transmitted as the resulting PUCCH resource for transmission using SPS-PUCCH-AN-List-r16, n1PUCCH-AN or other configured PUCCH resource(s)  is not valid

For Alt.1, if a DCI scheduling a dynamic PUCCH is miss detected, UE will not multiplex the SPS HARQ-ACK on current slot but delay it to a later slot, it will cause ambiguous between gNB and UE and great performance loss. Regarding Alt.1A, it changes the Rel-16 UCI multiplexing rule, and the latency is also high. While for Alt.2, it can maintain the latency since other configured PUCCH resources can also be applied if SPS-PUCCH-AN-List-r16 is not valid. In addition, Alt.2 can also effectively handle the DCI miss detection issue due to that gNB can do blind detection on the configured PUCCH resources chosen by UE via certain rules.  Overall, we prefer Alt. 2.

According to Alt. 2, if the SPS PUCCH resource is not available at the slot/sub-slot indicated in activation DCI and a PUCCH HARQ-ACK for dynamic PDSCH is transmitted on a same slot/sub-slot, the SPS HARQ-ACK can be multiplexed with dynamic PDSCH HARQ-ACK on a PUCCH resource configured in PUCCH-ResourceSets. Otherwise, if no dynamic PDSCH HARQ-ACK is configured to transmit on the same slot/sub-slot or the dynamic DCI is miss detected, the other configured PUCCH resources (e.g., PUCCH-ResourceSet, multi-CSI-PUCCH-ResourceList) that do not overlap with invalid symbols can also be used to transmit SPS HARQ-ACK. For all the configured PUCCH resources, a default rule can be used to choose one resource, e.g., the one with smallest resource index or the one with earliest starting symbol. In this way, the SPS delay can be reduced due to more available PUCCH resources and DCI miss detection issue can also be better handled.

Proposal 2. To handle Initial slot issue for SPS HARQ-ACK deferral, Alt. 2 is supported.

Proposal 3. For all the other configured PUCCH resources, a default rule can be used to choose one resource, e.g., the one with smallest resource index or the one with earliest starting symbol.

- SPS HARQ-ACK deferral configuration

In RAN1#104-e meeting, it has been agreed to do down-select between the following two options for SPS HARQ-ACK deferral:

Further down-select between the following two options for SPS HARQ-ACK deferral:

- Option 1: Joint RRC configuration of the SPS HARQ-ACK deferral per PUCCH cell group

  - Note: any SPS HARQ-ACK within a PUCCH cell group in principle is subject to deferral

- Option 2: The SPS HARQ-ACK deferral is configured per SPS configuration

  - Note: part of sps-config, only HARQ-ACK of SPS PDSCH configurations configured for deferral is in principle subject to deferral

In general, Option 1 is simpler to implement, while Option 2 is more flexible. Considering that some traffics transmit frequently and have very short period, they may not need retransmission to some extent. Delaying the HARQ-ACK feedback to a later slot may waste PUCCH resources and outperform packet survival time. So SPS configurations related to such traffics do not need to configure SPS HARQ-ACK deferral function. We prefer Option 2 here.

Proposal 4. The SPS HARQ-ACK deferral is configured per SPS configuration

- PUCCH repetition enhancement

Since the repetition for PUCCH formats 0 and 2 has been supported for sub-slot based PUCCH repetition, it is natural to also support slot-based PUCCH repetition for reliability enhancement. In addition, considering the latency issue, at least repetition number of 2 should be supported, which is also in accordance with Rel-17 eMIMO characters for multi-TRP transmission. Thus, we have a following agreement:

Proposal 5.  Support slot-based PUCCH repetition for PUCCH formats 0 and 2.

Retransmission of cancelled HARQ

Regarding how to how to indicate an enhanced Type 3 CB to the UE, using DCI is more general and more flexible than applying RRC signalling. Besides, using different RNTIs has a limitation when a HARQ process is shared between SPS PDSCH and dynamic PDSCH.  In addition, the triggering DCI can also schedule PDSCH at the same time, which is in accordance with Rel-16 URLLC. So we have a following proposal:

Proposal 6. Regarding how to how to indicate an enhanced Type 3 CB to the UE, using dynamic indication in the DCI is supported. The triggering DCI can also schedule PDSCH at the same time.

SPS HARQ skipping & payload size reduction

In the #104b-e meetings, a following conclusion has been made with respect to SPS HARQ-ACK skipping:

Conclusion:

No support for dynamic indication of skipped SPS PDSCH occasions in Rel-17 as part of this WI.

For NACK skipping, both skipped and non-skipped SPS PDSCH can apply this scheme if all feedback bits in the PUCCH are NACK. This is because gNB is aware of the SPS PDSCH is actually sent out or not, and it can distinguish between false alarm and PDSCH detection failure if no PUCCH is received (NACK). So there would be no ambiguous between gNB and UE.

The NACK skipping scheme can be configured by RRC signalling for all configured SPSs. That is because if NACK skipping scheme is configured per SPS or per group of SPS, e.g., SPS index 1 is configured with NACK skipping but SPS index 2 is not, and these two SPS PDSCHs are all NACK and are scheduled to feedback together. In this case, NACK skipping is performed or not is a question, and may cause confusions. So configuring NACK skipping function for all configured SPSs by higher layer is an effective and simple solution.

Proposal 7. NACK skipping should be supported, and it can be applied by both skipped and non-skipped SPS PDSCH.

Proposal 8. NACK skipping scheme can be configured by higher layer signalling for all configured SPSs.

To reduce the SPS HARQ payload size, ACK skipping should be supported at least since it can be applied for both skipped and non-skipped SPS PDSCH as mentioned before. Besides Alt.2, considering the reliability requirement of URLLC systems are generally high for non-skipped SPS PDSCH, decoding results are ACK for most of the cases, ACK skipping can also be considered to simplify the UE encoding complexity and reduce UL interferences.

Proposal 9. ACK skipping scheme can be considered for SPS HARQ payload size reduction of non-skipped SPS PDSCH.

Type 1 HARQ-ACK codebook based on sub-slot PUCCH configuration

In RAN1#104b-e meeting, it has been agreed to support Type-1 HARQ-ACK codebook for sub-slot based PUCCH configuration, and a following agreement has been made:

Agreement: Support Type-1 HARQ-ACK codebook for sub-slot based PUCCH configuration in Rel-17.

- The properties of the Type-1 HARQ-ACK codebook for sub-slot PUCCH at least includes that a PDSCH TDRA is associated with a UL /PUCCH sub-slot if the end of the PDSCH overlaps with the associated sub-slot determined by a k1 in the set of sub-slot timing values K1.

- FFS: whether the PDSCH TDRA grouping is performed per DL slot or sub-slot

  - Decide between PDSCH TDRA grouping per DL slot and sub-slot during RAN1#105-e

Regarding the FFS, firstly, the TDRA subset selection and dropping due to invalid symbol should be based on each sub-slot. Secondly, if the HARQ-ACK reporting window indicated by K1 set cannot contain a whole DL slot, sub-slot based TDRA grouping should still be applied since TDRA grouping based on a total DL slot will bring extra HARQ-ACK bit overhead. Considering to have a unified solution and less Spec. impact, we choose to use PDSCH TDRA grouping per sub-slot.

Proposal 10. For Type-1 HARQ-ACK codebook for sub-slot based PUCCH configuration, support PDSCH TDRA grouping per sub-slot.

2.41 Type 1 HARQ-ACK codebook generation procedure

A sub-slot based type 1 codebook generation can follow a similar process as Rel-15 slot based type 1 codebook, which can be given as below:

- For a UCI to be sent in sub-slot n, determine the union set of K1 values in unit of sub-slot according to the DCI formats the UE is configured to monitor PDCCH.

- Determine the union set of row indexed of TDRAs for DCI formats the UE is configured to monitor PDCCH

  - At sub-slot n-K1 with the given value K1, all the PDSCH occasions (end symbols are whining sub-slot n-K1) indicated in the TDRA tables configured by higher layers are considered to determine the codebook size.

  - If PDCCH starting symbol as the reference of SLIV is supported, the corresponding SLIVs with starting symbol S replaced by S+S0 should also be added into candidate PDSCH occasion sets.

- The PDSCH occasions that conflict with TDD DL/UL configuration are removed first. The remaining PDSCH occasions selection for determining the codebook size is given as  the procedure below:

- Select T to be smallest end symbol index of all the available SLIVs in sub-slot n-K1.

- Move the corresponding SLIV with ending symbol T into the chosen SLIV set ∅.

- Cancel the remaining SLIVs that starts no later than T.

- Go back to step 1) until all the SLIVs ending in sub-slot n-K1 are looped and get the final SLIV set ∅ to generate HARQ-ACK bits.

Conclusion

Proposal 1. For SPS HARQ-ACK deferral, a maximum deferring value can be configured by RRC signalling. If the RRC signalling does not exist, the maximum value of K1 set can be used as ae maximum deferring value.

Proposal 2. To handle Initial slot issue for SPS HARQ-ACK deferral, Alt. 2 is supported.

Proposal 3. For all the other configured PUCCH resources, a default rule can be used to choose one resource, e.g., the one with small resource index or the one with earliest starting symbol.

Proposal 4. The SPS HARQ-ACK deferral is configured per SPS configuration

Proposal 5.  Support slot-based PUCCH repetition for PUCCH formats 0 and 2.

Proposal 6. Regarding how to how to indicate an enhanced Type 3 CB to the UE, using dynamic indication in the DCI is supported. The triggering DCI can also schedule PDSCH at the same time.

Proposal 7. NACK skipping should be supported, and it can be applied by both skipped and non-skipped SPS PDSCH.

Proposal 8. NACK skipping scheme can be configured by higher layer signalling for all configured SPSs.

Proposal 9. ACK skipping scheme can be considered for SPS HARQ payload size reduction of non-skipped SPS PDSCH.

Proposal 10. For Type-1 HARQ-ACK codebook for sub-slot based PUCCH configuration, support PDSCH TDRA grouping per sub-slot.

References

RP-201310, Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR

Draft Report of 3GPP TSG RAN WG1 #104b-e.

Draft Report of 3GPP TSG RAN WG1 #105-e.