3GPP TSG RAN WG1 #106-e		R1-2106816

e-Meeting, August 16th – 27th, 2021

Source: 	Sony

Title:	Considerations on TRS/CSI-RS occasion(s) for idle/inactive UEs

Agenda Item:	8.7.1.2

Document for:	Discussion & Decision

# Introduction

One of the objectives of the power saving enhancement Rel-17 work item is shown below [1]:

- Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1] - Study and specify paging enhancement(s) to reduce unnecessary UE paging receptions, subject to no impact to legacy UEs [RAN2, RAN1] - NOTE: RAN1 to check and update, if needed, evaluation methodology in RAN1 #100-2 meeting - Specify means to provide potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs, minimizing system overhead impact [RAN1] - NOTE: Always-on TRS/CSI-RS transmission by gNodeB is not required

In the RAN1#105-e meeting [2], the following group of agreements related to potential TRS/CSI-RS occasion(s) were made:

Agreements related to TRS/CSI-RS availability:

- Confirm the following working assumption: - Support at least L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs. - FFS details, including paging DCI and/or PEI for L1 based signaling - FFS SIB-based signaling/configuration - Note: It is RAN1 understanding that existing SI update procedure is used for SIB based signalling - For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, support availability/unavailability information for configured RS resources using a bitmap or codepoint - e.g. using bitmap, where each bit is associated with at least one resource/configuration or a set/group of resources - e.g. a codepoint to indicate a state of availability/unavailability for all or some of configured RS resources - FFS maximum number of configured RS resources per physical layer availability indication to support. - FFS whether availability/unavailability information is for all or some of configured RS resources - Further study supporting SIB based signalling for availability information of TRS/CSI-RS occasions for idle/inactive UEs at least based on the presence/absence of the configuration of the TRS/CSI-RS occasion in SIB_X in case L1 based availability indication is not configured. - FFS whether and how SIB based signalling and L1 based signalling can be configured simultaneously Working assumption: Support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs. Support PEI based availability indication of TRS/CSI-RS occasions for idle/inactive UEs at least if PDCCH-based PEI is down-selected. - FFS whether and how to enable/disable L1 based availability indication configurable by SIB

Agreements related to TRS/CSI-RS configuration:

- Support applicable values for the following configuration parameters as below. - powerControlOffsetSS: {-3, 0, 3, 6}dB - scramblingID: 0 to 1023 - firstOFDMSymbolInTimeDomain: 0 to 9 - firstOFDMSymbolInTimeDomain indicates first symbol in a slot, a second symbol in the same slot can be derived implicitly with symbol index as firstOFDMSymbolInTimeDomain+4 - startingRB: 0 to 274 - nrofRBs: 24 to 276 - The QCL information of TRS/CSI-RS occasion(s) for idle/inactive UEs is indicated as a SSB index in range of 0 to 63. - FFS: how the QCL information can be configured, e.g. per RS resource set or per configuration - FFS: QCL type, which is predetermined - Configuration of TRS/CSI-RS occasion(s) for idle/inactive UEs include: - periodicityAndOffset {10, 20, 40, 80} ms - frequencyDomainAllocation for row1 with applicable values from {0, 1, 2, 3} to indicate the offset of the first RE to RE#0 in a RB - FFS Configuration index - details, - E.g. Per resource or resource set or group of resource sets - E.g. explicit or implicit indication based on QCL source

In this contribution, we discuss aspects related to how to provide availability information and configuration of TRS/CSI-RS. Hence, the UE can use TRS/CSI-RS when the UE is in idle/inactive mode.

# Discussions

## Availability Information of TRS/CSI-RS for idle/inactive UEs

An explicit indication of availability information of TRS/CSI-RS in a TRS/CSI-RS occasion is provided to the UE. This will be beneficial for the UE since blind detection operation to detect whether TRS/CSI-RS is transmitted or is no longer required. We also need to understand some potential limitations from the network side, for example how often the availability of TRS transmission is potentially changed. Frequent changes whether TRS is transmitted or not may require a lot of signaling from the gNB to the UEs. We support providing TRS/CSI-RS availability information to the UE while considering network limitation / preference. Acquiring TRS availability information by the UE in idle/inactive mode is quite challenging as there is limited possible communication exchange between the gNB and UE.

In legacy NR operation, the UE operating in idle/inactive mode can only acquire explicit information from the gNB by receiving system information (SIB) or by paging reception. In order to provide the availability information for idle/inactive UEs, we propose a UE can be explicitly informed via paging DCI. In the legacy NR system, the paging DCI contains a short-message information element that is used to inform the UE when there is a SIB update. This information element is allocated with 8 bits of which 6 bits are still reserved bits. The availability information can be explicitly informed via paging DCI using one or some of these reserved bits.

In RAN1#104bis-e, L1 based signaling for the availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs is supported. It is for further studies whether it is done through paging DCI and/or Paging Early Indication (PEI). We consider the usage of TRS/CSI-RS for idle/inactive UEs and paging enhancements in rel-17 are two different features. Depending on the progress of the WI, each of the features can be as a mandatory or optional feature.

Observation 1: The usage of TRS/CSI-RS for idle/inactive UEs and paging enhancements are two different features in rel-17. Depending on the progress of the WI, each of the features can be as a mandatory or optional feature.

Considering the above observation, we should only use paging DCI to provide availability information of TRS/CSI-RS occasions for idle/inactive UEs.

Proposal 1: Confirm the working assumption on support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs.

Proposal 2: The availability indication can be explicitly informed using one or some of these reserved bits in paging DCI.

We need to further discuss the definition of availability indication. The simplest option is that the availability information only contains information on whether TRS/CSI-RS for idle/inactive UEs is transmitted or not. Hence, it may only require 1 bit (e.g. “1” is available, “0” is not available). Availability information can also be extended by providing information, such as the duration or validity time of the availability of the TRS/CSI-RS for idle/inactive UEs. Another possibility is to extend availability information by providing information on which TRS/CSI-RS are currently active. This is particularly applicable to the case where a gNB can be configured with multiple TRS/CSI-RS configurations.

Proposal 3: Support to provide additional availability information (e.g. availability duration, which active TRS/CSI-RS are currently available).

In RAN1#104bis-e, two alternatives were identified describing the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs. It is still unclear on the fundamental different between these two alternatives. It may be related to the detailed of signaling implementation. We are fine to support alternative 1 in which availability/unavailability information for all or some of configured RS resources using a bitmap or codepoint. A bitmap/codepoint can represent the resource(s) associated with a TRS/CSI-RS configuration

Proposal 4: For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, use a bitmap / codepoint to indicate availability/unavailability information for all or some of configured RS resources.

In RAN1#104bis-e, it was also discussed whether and how to indicate the ‘availability’ in beam selective manner. We consider we should use the legacy approach as in NR Rel-15, where there is an association of TRS/CSI-RS beam and SSB beam. Hence, the transmission of TRS/CSI-RS should refer to SSB. Particularly in this case where the UE is in idle/inactive mode. The gNB is not aware of UE position/direction when the UE is in idle/inactive mode.

Proposal 5: Indication the ‘availability’ of TRS/CSI-RS occasions in beam selective manner is not supported.

## Configuration of TRS/CSI-RS for idle/inactive UEs

According to the WID, idle/inactive mode UEs can re-use TRS/CSI-RS occasion(s) available in connected mode (of other UEs). The TRS/CSI-RS configuration for connected mode UEs is provided via UE specific RRC messaging. Hence, in practice, two or more connected mode UEs can receive different TRS/CSI-RS configurations or basically the gNB transmits multiple TRS/CSI-RS with different configurations. It would be beneficial if idle/inactive mode UE know the existence of the multiple TRS/CSI-RS configurations. The UE can select the TRS/CSI-RS that suits with UE condition/scenario. Multiple configurations can also be beneficial for the gNB to have flexibility. For example, allocating TRS/CSI-RS in different resources. The gNB can select/activate at least an active configuration at a given period of time.

Proposal 6: Support providing multiple TRS/CSI-RS configurations to idle/inactive UEs.

Further consideration related to TRS/CSI-RS configuration is on where to transmit the configuration to the UEs in idle/inactive mode. As discussed above, at least SIB can be used. The legacy SIB structure is divided into two parts, minimum system information (i.e PBCH and SIB1) and Other System Information (OSI). TRS/CSI-RS configuration is an enhancement feature, and the size of the configuration can be relatively large. Hence, minimum system information may not be suitable for carrying this configuration information. Another alternative for transmitting the configuration information is in the other system information (OSI). A UE that requires OSI can send a request to the gNB and then later the gNB provides it to the UE. This type of mechanism is suitable for a UE that has entered a new cell in idle mode.

A UE that was in connected mode (and therefore has acquired TRS/CSI-RS for connected mode) may not necessarily acquire the TRS/CSI-RS configuration by requesting OSI containing TRS/CSI-RS configuration. This is particularly the case where the UE in idle mode can continue to use the same TRS/CSI-RS configuration that it had when it was in connected mode. There can be, for example, a flag in a dedicated RRC, such as RRC release message indicating that once the UE enters idle/inactive mode, the UE can re-use the same TRS/CSI-RS. Here, we basically propose that the TRS/CSI-RS configuration is signaled in a dedicated RRC/RRC release message. This was partly discussed in RAN1#103e resulting in the following agreement.

- SIB signalling provides the configuration of TRS/CSI-RS occasion(s) for idle/inactive UE(s). - Up to RAN2 to decide which SIB is to be used. - Whether or not to additionally support other high-layer signalling methods (e.g., dedicated RRC, RRC release message, etc.) is up to RAN2

Proposal 7: In addition to SIB, support TRS/CSI-RS configuration via other high-layer signaling (e.g., dedicated RRC, RRC release message, etc.).

# Conclusion

In this contribution, we discuss power saving enhancement for UEs in IDLE or INACTIVE mode through providing TRS / CSI-RS. Our observations and proposals are listed below:

Observation 1: The usage of TRS/CSI-RS for idle/inactive UEs and paging enhancements in rel-17 are two different features. Depending on the progress of the WI, each of the features can be as a mandatory or optional feature.

Proposal 1: Confirm the working assumption on support paging PDCCH based availability indication of TRS/CSI-RS occasions for idle/inactive UEs.

Proposal 2: The availability indication can be explicitly informed using one or some of these reserved bits in paging DCI.

Proposal 3: Support to provide additional availability information (e.g. availability duration, which active TRS/CSI-RS are currently available).

Proposal 4: For the information provided by a physical layer availability indication of TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs, use a bitmap / codepoint to indicate availability/unavailability information for all or some of configured RS resources.

Proposal 5: Indication the ‘availability’ of TRS/CSI-RS occasions in beam selective manner is not supported.

Proposal 6: Support providing multiple TRS/CSI-RS configurations to idle/inactive UEs.

Proposal 7: In addition to SIB, support TRS/CSI-RS configuration via other high-layer signaling (e.g., dedicated RRC, RRC release message, etc.).

# References

- RP-193239, New WID: UE Power Saving Enhancements.

- RAN1-105e Chairman’s notes