3GPP TSG-RAN WG1 Meeting#106-e                                                 R1-2106851

E-meeting, August 16th – 27th, 2021

Title:	[Draft] LS Reply to RAN4 on synchronous operation between Uu and SL in TDD band n79

Response to:	R1-2106430/R4-2108114

Release:	Rel-17

Work Item:	NR_SL_enh-Core

Source:	RAN1

To:	RAN4

Cc:

Contact Person:

Name:	Emad Farag

E-mail Address:	e.farag@samsung.com

# Overall description

RAN1 would like to thank RAN4 for the LS on “synchronous operation between Uu and SL in TDD band”. In the LS, RAN4 presents two options under discussion in RAN4 for alignment between SL and Uu to mitigate interference between SL and UL:

Option 1: To follow the Rel-16 agreement to align SL transmission timing with DL timing.

Option 2: To reconsider SL transmission timing to align with UL timing to mitigate the interference between Uu and SL, i.e.

- For sidelink transmissions,

  - SL transmission timing is aligned with Uplink timing when Uu and sidelink is TDMed/FDMed coexistence in the same band, including TDM coexistence within the same carrier or different carriers.

  - Otherwise, SL transmission timing is aligned with Downlink timing.

RAN4 is further soliciting RAN1’s perspective on supporting option 2:

RAN4 respectfully ask RAN1 to clarify that is it feasible that RAN4 consider option 2 from RAN1 perspective to define SL transmission timing to align with UL timing when SL is synchronized to a network?

RAN1 has discussed this issue and came to the following answer and conclusion:

RAN1 prefers Option 1 as this already follows the Rel-16 RAN1 agreements. On the other hand, Option 2 requires additional spec impacts to align UL based timing between sidelink UEs. This could further lead to backward compatibility issues between UEs supporting option 1 (i.e. Rel-16) and UEs supporting option 2 in the same network.

# Actions

To: RAN4

ACTION: 	RAN1 kindly asks RAN4 to take the above into consideration.

# Dates of next TSG WG1 meetings

RAN1#106bis-e	October 11th – 19th, 2021	E-meeting

RAN1#107-e	Novermber 11th – November 19th, 2021	E-meeting