3GPP TSG RAN WG1 #106-e	R1-2106857

e-Meeting, August 16th – 27th, 2021‎

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | - | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on counting PDCCH candidates in overbooking procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_L1enh_URLLC-Core |  |  |  |  |  | Date: |  |  | 2021-08-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The procedure to resolve PDCCH overbooking counts a total number of PDCCH candidates, instead of a total number of decoding operations for DCI formats, per CSS/USS set. The two variables differ when a PDCCH candidate is associated with multiple sizes of DCI formats. As a result, a required UE processing may be under-counted and specifications may be interpreted as requiring PDCCH monitoring for more USS sets than the UE is capable of. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that, for the overbooking procedure, the counting is for the total number of decoding operations for DCI formats per search space set instead of the total number of PDCCH candidates per search space set. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous specifications. A UE capability may be over-counted. A UE may drop search space sets when not expected to do so based on specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: If the gNB applies this change but the UE does not, there is no impact as the gNB assumes a more conservative capability for the UE. If the UE applies this change but the gNB does not, gNB and UE may have a different understanding for PDCCH monitoring in some search space sets. The gNB may assume that the UE monitors PDCCH when the UE does not. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 10.1	UE procedure for determining physical downlink control channel assignment

< Unchanged part is omitted >

Denote by , , the number of counted PDCCH candidates for monitoring for CSS set  and by , , the number of counted PDCCH candidates for monitoring for USS set , for CCE aggregation level L.

For the CSS sets, a UE monitors MPDCCHcss=i=0Icss-1d(i)∙LMScss(i)(L)PDCCH candidates requiring a total of  non-overlapping CCEs in a slot or in a span, where d(i) is a number of sizes for DCI formats for CSS set i after alignment of DCI format sizes [5, 38.212].

< Unchanged part is omitted >

Denote by  the set of non-overlapping CCEs for search space set  and by  the cardinality of  where the non-overlapping CCEs for search space set  are determined considering the allocated PDCCH candidates for monitoring for the CSS sets and the allocated PDCCH candidates for monitoring for all search space sets , .

Set

Set

Set d(j) to a number of sizes for DCI formats for USS set j after alignment of DCI format sizes

Set

while d(j)∙ AND

allocate d(j)∙ PDCCH candidates for monitoring to USS set

MPDCCHuss=MPDCCHuss-d(j)∙LMSuss(j)(L);

;

;

end while

< Unchanged part is omitted >