3GPP TSG WG1 Meeting #106-e	R1-2106859

# e-Meeting, August 16th – 27th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.214 | CR |  | rev |  | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on interpretation of frequency resource indication value for NR sidelink |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  | Date: |  |  | 2021-05-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | If TRIV indicates N < sl-MaxNumPerReserve, the starting sub-channel indexes corresponding to sl-MaxNumPerReserve minus N last resources are ambiguous in the calculation of FRIV. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define the starting sub-channel indexes corresponding to sl-MaxNumPerReserve minus N last resources as zero |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Undefined parameter value for the starting sub-channel indexes corresponding to sl-MaxNumPerReserve minus N last resources in specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Unchanged text is omitted ***

8.1.5	UE procedure for determining slots and resource blocks for PSSCH transmission associated with an SCI format 1-A

The set of slots and resource blocks for PSSCH transmission is determined by the resource used for the PSCCH transmission containing the associated SCI format 1-A, and fields 'Frequency resource assignment', 'Time resource assignment' of the associated SCI format 1-A as described below.

'Time resource assignment' carries logical slot offset indication of N = 1 or 2 actual resources when sl-MaxNumPerReserve is 2, and N = 1 or 2 or 3 actual resources when sl_MaxNumPerReserve is 3, in a form of time RIV (TRIV) field which is determined as follows:

if N=1

TRIV=0

elseif N=2

TRIV=t1

else

if t2-t1-1≤15

TRIV=30t2-t1-1+t1+31

else

TRIV=3031-t2+t1+62-t1

end if

end if

where the first resource is in the slot where SCI format 1-A was received, and ti denotes i-th resource time offset in logical slots of a resource pool with respect to the first resource where for N = 2, 1≤t1≤31; and for N = 3, 1≤t1≤30, t1<t2≤31.

The starting sub-channel nsubCH,0start of the first resource is determined according to clause 8.1.2.2. The number of contiguously allocated sub-channels for each of the N resources LsubCH≥1 and the starting sub-channel indexes of resources indicated by the received SCI format 1-A, except the resource in the slot where SCI format 1-A was received, are determined from "Frequency resource assignment" which is equal to a frequency RIV (FRIV) where.

If sl-MaxNumPerReserve is 2 then

FRIV=nsubCH,1start+i=1LsubCH-1N subchannel SL+1-i

If sl-MaxNumPerReserve is 3 then

FRIV=nsubCH,1start+nsubCH,2start⋅N subchannel SL+1-LsubCH+i=1LsubCH-1N subchannel SL+1-i2

where

-	nsubCH,1start denotes the starting sub-channel index for the second resource

-	nsubCH,2start denotes the starting sub-channel index for the third resource

-	N subchannel SL is the number of sub-channels in a resource pool provided according to the higher layer parameter sl-NumSubchannel

If TRIV indicates N < sl-MaxNumPerReserve, the starting sub-channel indexes corresponding to sl-MaxNumPerReserve minus N last resources are not used and set to zero.

The number of slots in one set of the time and frequency resources for transmission opportunities of PSSCH is given by Cresel where Cresel= 10*SL_RESOURCE_RESELECTION_COUNTER [10, TS 38.321] if configured else Cresel is set to 1.

If a set of sub-channels in slot t'mSL is determined as the time and frequency resource for PSSCH transmission corresponding to the selected sidelink grant (described in [10, TS 38.321]), the same set of sub-channels in slots t'm+j×Prsvp_TX'SL are also determined for PSSCH transmissions corresponding to the same sidelink grant where j=1, 2,…, Cresel-1, Prsvp_TX, if provided, is converted from units of msec to units of logical slots, resulting in Prsvp_TX' according to clause 8.1.7, and t'0SL, t'1SL, t'2SL,… is determined by Clause 8. Here, Prsvp_TX is the resource reservation interval indicated by higher layers.

*** Unchanged text is omitted ***