3GPP TSG RAN WG1 Meeting #106-e                               R1-2106972

e-Meeting, August 16th – 27th, 2021

Source:	CATT

Title:	Discussion on enhancements for UL-AoA positioning method

Agenda item:	8.5.2

Document for:	Discussion and Decision

Introduction

In this contribution, we further discuss the remaining issues related to the enhancements of UL-AoA (AoA/ZoA) measurement reporting for UL-AoA positioning.

UL-AoA measurements enhancements

The following agreements relating to the UL-AoA (AoA/ZoA) measurement of the first arrival path and additional path were made in RAN1#104-e [1] and RAN1#104b-e[2]:

Agreement: (RAN1#104-e) - NR supports reporting of M > 1 UL-AOA (AoA/ZoA) measurement values by gNB to the LMF at least for the first arrival path - FFS: Supporting of UL-AOA measurements for additional paths - FFS:Supporting of N >= 1 UL-AOA values per path for additional paths - FFS: Whether the multiple values can correspond to the same time stamp. - FFS: Further details of measurement and reporting - Note: The reporting by gNB to the LMF is optional Agreement: (RAN1#104b-e) Reporting to LMF of M > 1 UL-AOA (AoA/ZoA) measurement values associated with the first arrival path and corresponding to the same timestamp is supported.

According to the above agreements, the reporting of M>1 UL-AoA measurement values will be supported for the first arrival path in Rel-17. The remaining issue is what is the value of M. The issue was discussed in RAN1#105-e without conclusion [3][4]. In addition, the reporting of UL-AoA measurement values of the additional paths were also discussed in RAN1#105-e without conclusion [3][4]. In this section, we present our views on these two issues.

For the reporting of M>1 UL-AoA measurement values for the first arrival path, the latest FL proposal discussed was as follows [4]:

Proposal 3.7-4 - LMF can request maximum number of UL-AOAs values (pair of AOA & ZOA values, AOA values or ZOA values) to be reported per SRS resource for the first arrival path corresponding to the same timestamp - The maximum number is selected from the set {1, 2, [X1], [X2]} - X1 ≥ 4, X2 ≥ 8 - FFS specific values for X1 and X2

The main reason that no consensus was reached in the previous meeting was that companies have different views on whether it is meaningful to report 4 or more UL-AoA measurement values for the first arrival path, and more than one UL-AoA measurement value for the additional arrival path.

For UL-AoA measurement values for the first arrival path, in our view it may not have a practical value for gNB to report more argument for UL-AoA measurement values based on the following considerations:

- The scenario of the signal of the same SRS resource arrives at one TRP from 4 or more different angles at the same time of the first arrival path was not considered during the discussion of the R17 simulation assumption, and this scenario may not be common in a practical deployment environment either;

- It is also a question on how the LMF can effectively use 4 or more UL-AoA measurements for the first arrival path in the calculation of the UE position. At least, the complexity of the LMF algorithm will be significantly increased for dealing with such scenarios.

Based on the above analysis, we do not see the practical value for requesting gNB to report 4 or more UL-AoAs values per SRS resource for the first arrival path to the LMF.

- Proposal 1: In Rel-17, support gNB to report up to 2 UL-AoAs values (pair of AoA & ZoA values, AoA values or ZoA values) per SRS resource for the first arrival path.

For the reporting of UL-AoA measurements of additional paths, the intention is to allow LMF to use more advanced algorithms to potentially achieve a higher positioning accuracy. There are two options to support the UL-AoA report for the additional path:

- Option 1: NR supports reporting to LMF of N = 1 UL-AoA measurement values per additional path for the same timestamp.

- Option 2: NR supports reporting to LMF of N ≥ 1 UL-AoA measurement values per additional path for the same timestamp.

The performance gain of Option 2 is so far unclear. In our opinion, although one may use advanced algorithms, such as AI/ML, to process N ≥ 1 UL-AoA measurement values per additional path for improving the positioning accuracy, the effectiveness and implementation issues of these AI/ML algorithms for positioning are under very early stage of investigation and further study is needed in future releases. In reality, it may be difficult to implement high-resolution AI/ML algorithms that require the great efforts in training and modelling of AI/MI algorithms. In addition, reporting multiple UL-AoA measurements per additional path increases the implementation complexity in both gNB and LMF, and also the reporting overhead. Furthermore, it would be difficult to define the measurement performance and testing requirements for supporting it for RAN4. Thus, we prefer only to support N=1 UL-AoA measurement value per additional path in Rel-17. Whether to support N>1 UL-AoA measurement value per additional path can be discussed in future releases.

- Proposal 2: In Rel-17, support gNB to report one UL-AoAs value (pair of AoA & ZoA values, AoA values or ZoA values) per SRS resource per additional arrival path.

Conclusions

In this contribution, we discussed the remaining issues related to the enhancements for UL-AoA positioning with the following proposal.

- Proposal 1: In Rel-17, support gNB to report up to 2 UL-AoAs values (pair of AoA & ZoA values, AoA values or ZoA values) per SRS resource for the first arrival path.

- Proposal 2: In Rel-17, support gNB to report one UL-AoAs value (pair of AoA & ZoA values, AoA values or ZoA values) per SRS resource per additional arrival path.

References

- 3GPP TSG RAN WG1 Meeting #104-e, RAN1 Chairman’s Notes.

- 3GPP TSG RAN WG1 Meeting #104b-e, RAN1 Chairman’s Notes.

- 3GPP TSG RAN WG1 Meeting #105-e, RAN1 Chairman’s Notes.

- R1-2106334, “Feature Lead Summary#3 for E-mail Discussion [105-e-NR-ePos-02]”, Moderator (Intel Corporation)

- R1-2104521, Discussion on accuracy improvements for UL-AoA positioning solutions, CATT