3GPP TSG RAN WG1 #106-e			R1-2106986

e-Meeting, August 16th – 27th, 2021

# Source:             CATT

Title:                  PDCCH monitoring adaptation

Agenda Item:    8.7.2

Document for:  Discussion and Decision

Introduction

As the agreement of RAN1#105e [1], the detailed design for the two candidate schemes, i.e. Search space set group switching and PDCCH skipping, was agreed to be both supported for PDCCH monitoring adaptation. However, one of the two detailed implement schemes would be decided in RAN1#106e meeting.

Agreement: - PDCCH schedules data and also indicates PDCCH monitoring adaptation by SSSG switching and PDCCH skipping for a duration is supported. - At least DCI format(s) 1-1, 0-1, 1-2 and 0-2 can be used for the indication(s) Agreement: - At least one of Alt 1 and Alt 2 is supported, to be decided in RAN1#106, - Alt 1: Supporting SSSG switching to emulate PDCCH skipping functionality, - Alt 1-1: by an ‘empty’ SSSG which no SS set(s) is configured for the ‘empty’ SSSG, UE does not monitoring PDCCH on the ‘empty’ SSSG, - Alt1-2: by a ‘dormant SSSG’ which may have associated SS sets, and monitored conditionally (e.g., depending on HARQ NACK or RTT/ReTx timers) - Alt 2: PDCCH schedules data and also indicates PDCCH monitoring adaptation by PDCCH skipping for a duration is supported. - FFS details, including - e.g., joint / separate indication of SSSG switching and PDCCH skipping - Determination of the duration(s) for PDCCH skipping, e.g., - by RRC signaling, - by DCI indication - Implicitly, to the end of C-DRX active time Agreement: At least SSSG#0 and SSSG#1 switching is supported for Rel-17 SSSG switching indicated by PDCCH scheduling data and/or timer. - FFS: support of more than 2 SSSGs

In this contribution, we firstly analyze the drawbacks of the Search Space Set Group (SSSG) switching including its enhancement scheme in Section 2. The advantages and detailed DCI design for the PDCCH skipping would be provided in Section 3 and Section 4, respectively. Finally, the power saving gain of the SSSG switching and PDCCH skipping are provided in Section 5.

The drawbacks of the Search Space Set Group switching and its enhancement

As the discussion in RAN1#104 E-meeting and RAN1#105 E-meeting, it was proposed that the PDCCH skipping could be realized by SSSG switching [2][3][4], in which the “dormant SSSG” or “empty SSSG” is used to emulate the PDCCH skipping functionality. However, there are several non-negligible problems for the SSSG switching scheme in emulation of PDCCH skipping function as follows,

- The additional application delay applied in both trigger mechanisms [5], i.e. explicit trigger by DCI format 2_0 and implicit switching, would affect the power saving gain. The SSSG switching as similar as the BWP switching, UE would start to monitor the PDCCH in the target SSSG at the first slot at least P_switch symbols after the last symbol of the PDCCH with the DCI format 2_0 or after a slot where the timer expires. The value of P_switch is up to 25 symbols. The application delay would set the constraint of gNB scheduling and reduce the efficiency of the PDCCH monitoring reduction during the period of application delay. When the traffic arrival is robust and unpredictable, the PDCCH monitoring periods would be changed quite frequently. The application delay would be accumulated for each SSSG switching. The frequent SSSG switching will increase UE power consumption. Based on the power saving gain analysis, the application delay can affect the power saving gain for longer preparation duration [2]. The SSSG switching would not dynamically adapt the PDCCH monitoring reduction in achieving UE power saving.

- For SSSG switching, the PDCCH monitoring reduction depends on the RRC pre-configuration of the different search spaces in the SSSGs. If the parameters of the search spaces change, it needs RRC reconfiguration or more search space sets to achieve the desired power saving. Furthermore, in order to realize the flexible PDCCH adaptation, the SSSG switching would need large Search Space Set overhead.

- For the SSSG switching enhancement using the UE-specific indication, e.g. scheduling DCI, the catastrophe could be created when DCI is miss-detected and additional signaling overhead. There would also be some non-negligible drawbacks for SSSG switching. An example of UE-specific scheduling DCI based SSSG switching is shown in Figure 1. The UE-specific search space of the scheduling DCI with the search space switching trigger flag expects to receive HARQ-ACK. The HARQ-ACK to acknowledge the UE-specific search space switching is additional overhead of UL resource and additional power consumption by UE. If the HARQ-ACK sent by UE is miss-detected by gNB, the gNB would continue to indicate the PDCCH monitoring based on the current search space set, which is different to the UE assumption on PDCCH monitoring indicated by new search space after search space switching UE still consume power in PDCCH decoding and could not decode any subsequent scheduling DCI.

- For the SSSG switching enhancement, the switching would be performed after the acknowledge of HARQ-ACK for scheduling DCI, the additional latency for HARQ-ACK feedback would also affect the power saving gain, which has been evaluated in Section 5.

Figure 1.The ambiguous understanding between the gNB and UE
 for scheduling DCI based SSSG switching miss-detected

According to the analysis above, the SSSG switching is sensitive to the DCI miss-detection and has the constraint of application delay. The SSSG switching cannot create the exact function of the PDCCH skipping to achieve UE power saving.

Observation 1: SSSG switching has the additional application delay during search space switching, which can decrease the power saving gain.

Observation 2: For SSSG switching, the flexible PDCCH adaptation would need large Search Space Set overhead.

Observation 3: Although the SSSG switching is enhanced by using UE-specific indication, e.g. scheduling DCI, the catastrophe could be created when DCI is miss-detected and large search space set overhead.

Proposal 1: The SSSG switching cannot create the exact function of the PDCCH skipping to achieve UE power saving for its non-negligible drawbacks, especially for the additional switching delay, miss detection of HARQ-ACK and large search space set overhead.

The advantages of PDCCH skipping scheme

The PDCCH skipping uses L1 signaling to indicate UE to skip the number of  PDCCH monitoring occasion based on the data in the gNB buffer without any additional delay. When the PDCCH skipping is indicated, the skipping steps could be maintained until the next new skipping step indicated by the received L1 signaling. The skipped monitoring occasions are the subset of those in the configured Search Space Set. The monitoring occasions always belong to the same search space set and there is not any switching delay for performing PDCCH skipping. Even if the DCI based indication is miss-detected, UE would not be ambiguous for PDCCH monitoring but a few latency until next monitoring occasion. Furthermore, not only the scheduling DCI but also the non-scheduling DCI can be used for PDCCH skipping indication. For the non-scheduling DCI based PDCCH skipping indication could also inform UE which monitoring occasion to be detected without the constraint of the HARQ-ACK, whose detailed designs are provided in Section 4.

Thus, the PDCCH skipping should be individually indicated by DCI but not emulated by SSSG switching or its any enhancement. The Alt 2: PDCCH schedules data and also indicates PDCCH monitoring adaptation by PDCCH skipping for a duration should be supported.

Proposal 2: The Alt 2: PDCCH schedules data and also indicates PDCCH monitoring adaptation by PDCCH skipping for a duration should be supported.

DCI design for PDCCH Skipping

The PDCCH monitoring adaptation for PCell can be applied by the enhanced DCI format during the DRX active time, which had been proposed in [5]. The enhanced DCI formats are designed for indicating the PDCCH monitoring adaptation including PDCCH skipping or PDCCH monitoring periodicity adjustment. The extension of DCI format 0_1/1_1 is used without introducing additional information field, in which the existing information field, i.e. SCell dormancy indication bits in case 1 or case 2, can be repurposed or joint encoding for mapping or grouping to indicate the PCell and/or SCell.

For the SCell dormancy indication in case 1, SCell dormancy indication in the DCI format 0_1 and 1_1 with the scheduling information can be repurposed to indicate PDCCH skipping. If UE does not detect a carrier indicator field or detects a carrier indicator field with value equal to 0, the SCell dormancy indication field is a bitmap for indicating the dormancy behavior of SCell groups in Rel-16. However, when UE detects a carrier indicator field with value not equal to 0, the SCell dormancy indication field, i.e. up to 5 bits, would be discarded or ignored in UE detection. In the Rel-17 UE power saving enhancement, this field can be reused for joint indication including the PDCCH skipping in PCell and/or SCell dormancy. For instance, if the SCell dormancy group(s) is configured, e.g. 2 bits, the interpretation of SCell dormancy indication field in DCI format 0_1/1_1 for the joint indication for PDCCH skipping and/or SCell dormancy is provided in Table 1.

Table 1. The interpretation of information field in DCI format 0_1/1_1 (Example 1)

| SCell dormancy indication field in DCI | Interpretation (CIF≠0) |
| --- | --- |
| 00 | Monitor each monitoring occasion (MO) & SCell group 1(“active BWP”)SCell group 2(“active BWP”) |
| 01 | Monitor each monitoring occasion (MO) & SCell group 1(“active BWP”)SCell group 2(“dormancy BWP”) |
| 10 | Skip one MO/monitoring periodicity 1 & SCell group 1(“dormancy BWP”)SCell group 2(“active BWP”) |
| 11 | Skip following MOs until the next DRX on-duration & SCell group 1(“dormancy BWP”)SCell group 2(“dormancy BWP”) |

For the case 2: SCell dormancy indication using non-scheduling DCI format 1_1, the scheduling related information fields in DCI format 1_1 without scheduling information can be used to indicate SCell dormancy, including modulation and coding scheme, new data indicator, redundancy version, HARQ process number, antenna port(s), DMRS sequence initialization etc. There are up to 19 bits used for SCell dormancy indication.  The 19 bits can be repurposed for joint indication including the PDCCH monitoring adaptation in PCell and/or SCell dormancy when SCell dormancy is configured. Taking 2 out of 19 bits information field in DCI format 1_1 as an example, the 2 bits can be reused for PDCCH skipping indication for PCell and the remaining 17 bits information field can also be used for SCell dormancy indication, which is shown in Table 2.

Table 2. The interpretation of 2bits information field in DCI (Example 2)

| Information field in DCI | Interpretation |
| --- | --- |
| 00 | Monitor each monitoring occasion (MO) |
| 01 | Skip one MO/monitoring periodicity 1 |
| 10 | Skip two MOs/monitoring periodicity 2 |
| 11 | Skip following MOs until the next DRX on-duration |

The extension of DCI format designs could reuse the existing DCI format 0_1 and 1_1 in Rel-16 and not increase the size of DCI format with additional function in extending the PDCCH monitoring adaptation in PCell to achieve more power saving gain without introducing additional information field. Otherwise, if the DCI format 2_6 can also be used in active time, it can introduce a specific information field, e.g. 2bits, for the PDCCH adaptation indication.

Proposal 3: The PDCCH monitoring adaptation can dynamically indicate UE to reduce the PDCCH monitoring without any changes of SearchSpace configuration.

The power saving gain of the Search Space Set Group switching and the PDCCH skipping

In this section, the power saving gain of the SSSG switching and PDCCH skipping are evaluated. Based on the agreements in RAN1#102e, the Rel-15/Rel-16 features such as RLM, RRM and DRX wake-up indication etc., are all considered in the baseline of power consumption. The evaluation parameters of the baseline in this subsection are listed in the Table 3. In order to assess the power saving gain under the different traffic models, FTP3 and Instant Message (IM) traffic are both considered in the evaluations, in which the reference DRX configurations associated with the traffic model are chosen from TR38.840.

Several typical schemes for PDCCH monitoring adaptation are considered in the evaluation, whose details are as following:

- The PDCCH skipping scheme 1(Go-to-sleep indication): When UE finishes the packet transmission, it would be indicated to go to sleep until next DRX-On.

- The PDCCH skipping scheme 2 (Step={1,2,4,8}): When UE finishes the packet transmission, it would be indicated to skip the PDCCH monitoring up to 8 steps, i.e. 1, 2, 4, 8 steps.

- The Search Space Set Group Switching (Periodicity = 2slots): When UE finishes the packet transmission, it would be indicated to switch to the SSSG with the 2slots PDCCH monitoring periodicity. The maximum process of the HARQ-ACK is 3 and the HARQ probability is 10%. The 2 slots switching delay is also considered.

Table 3. Simulation assumptions for the baseline of PDCCH monitoring adaptation

| Parameters | Assumptions/Values |
| --- | --- |
| DRX configuration | FTP 3: Period = 160ms, Inactive timer = 100ms, on-duration = 8ms IM traffic: Period = 320ms, Inactive timer = 80ms, on-duration = 10ms |
| BWP | 100 MHz |
| RLM | maximum {the shortest periodicity for RLM resources，the DRX period} 15*160ms = 2400ms |
| RRM | 150 for RRM power consumption SMTC Period = 20ms 2 SSBs per slot, only intra-frequency measurements considered, synchronies FR1 scenario RRM period = 15* 160ms |
| DRX adaptation indication | UE detects Power saving signal which will trigger UE to wake-up when the data is arrival. When there is no data transmission in the DRX cycle, UE could go to sleep by indication in DCP. |
| Power consumption for DCP | 100, the same as that of PDCCH-only DCP offset to DRX ON = 2 ms |
| Channel tracking and CSI measurement | Based on 5ms CSI-RS period; Based on SSB with 20ms periodicity. |
| TDD configuration | DSUUD |
| SCS | 30kHz |
| Transmission rate | Transmission data rate is based on ITU-DU SE. 10UE is dropped within one TRP. |
| Traffic model | FTP3 with 0.5Mbytes packet, 200ms inter-arrival time; IM with 0.1M bytes packet,2sec inter-arrival time |

The power saving gains of the schemes above are provided in the Table 4. According to the evaluation results, we find that the different power saving gain for the different traffic model with the corresponding DRX configuration. It is shown that the PDCCH skipping scheme 1(Go-to-sleep indication) and the PDCCH skipping scheme 2 (Step={1,2,4,8}) can obtain 71.77% and 33.96% power saving gain under the FTP traffic, respectively. The 41.2% and 19.78% power saving gain can be observed for scheme 1 and scheme 2 under the IM traffic, respectively. However, the Search Space Set Group Switching (Periodicity = 2slots) can obtain 24.64% and 14.15% power saving gain under the FTP traffic and the IM traffic, respectively.

Table 4. The power saving gain for the PDCCH monitoring adaptation schemes

| Power consumption / Power saving gain | FTP traffic | IM traffic |
| --- | --- | --- |
| Power consumption of baseline | 28.67 | 4.62 |
| Power consumption of the PDCCH skipping scheme 1 (Go-to-sleep) | 8.09 | 2.72 |
| Power consumption of the PDCCH skipping scheme 2 (Step={1,2,4,8}) | 18.93 | 3.76 |
| Power consumption of the Search Space Set Group Switching (Periodicity = 2slots) | 21.49 | 3.99 |
| Power saving gain for the PDCCH skipping scheme 1 (Go-to-sleep) | 71.77% | 41.2% |
| Power saving gain for the PDCCH skipping scheme 2 (Step={1,2,4,8}) | 33.96% | 19.78% |
| Power saving gain for Search Space Set Group Switching (Periodicity = 2slots) | 24.64% | 14.15% |

According to the evaluation results, the PDCCH skipping can obtain the higher power saving gain than the SSSG switching for the nearly zero application delay and the flexible skipping steps. The performance of power saving gain for the SSSG switching is mainly affected by the additional switching delay and the acknowledgement of HARQ-ACK for switching indication.

Observation 4: The PDCCH skipping can obtain the higher power saving gain than the SSSG switching.

Conclusion

We have the following observations and proposals:

Observation 1: SSSG switching has the additional application delay during search space switching, which can decrease the power saving gain.

Observation 2: For SSSG switching, the flexible PDCCH adaptation would need large Search Space Set overhead.

Observation 3: Although the SSSG switching is enhanced by using UE-specific indication, e.g. scheduling DCI, the catastrophe could be created when DCI is miss-detected and large search space set overhead.

Observation 4: The PDCCH skipping can obtain the higher power saving gain than the SSSG switching.

Proposal 1: The SSSG switching cannot create the exact function of the PDCCH skipping to achieve UE power saving for its non-negligible drawbacks, especially for the additional switching delay, miss detection of HARQ-ACK and large search space set overhead.

Proposal 2: The Alt 2: PDCCH schedules data and also indicates PDCCH monitoring adaptation by PDCCH skipping for a duration should be supported.

Proposal 3: The PDCCH monitoring adaptation can dynamically indicate UE to reduce the PDCCH monitoring without any changes of SearchSpace configuration.

References

  - Chairman’s Notes, RAN1#105-e, Agreements for UE Power Saving Enhancements.

  - R1-2100455, Discussion on DCI-based power saving adaptation in connected mode, vivo

  - R1-2100593, On enhancements to DCI-based UE power saving during DRX active time, MediaTek Inc.

  - R1-2101476, DCI-based power saving adaptation during DRX ActiveTime, Qualcomm Incorporated

  - 3GPP TS 38.213 V16.2.0, “NR; Physical layer procedures for control”.