3GPP TSG RAN WG1 #106-e	R1-2107019

e-Meeting, Aug 16th - Aug 27th, 2021

Agenda item:		8.3.1.2

Source:		Nokia, Nokia Shanghai Bell

Title:	CSI feedback enhancements for URLLC/IIoT use cases

Document for:		Discussion and Decision

# 1	Introduction

The work item on enhanced Industrial Internet of Things (IIoT) and URLLC support initiated on RAN1#102-e, agenda item 8.3. Since then the work item has been discussed widely, but the progress in reaching agreements has been limited. RAN#92 gave guidance to RAN1 on this WI and the endorsed recommendations were captured in [RP-211546]:

## Outcome of RAN#92 GTW session (Wednesday)

During the GTW session the following recommendations with further revisions were endorsed.

- Revised Recommendation1: Provide the following RAN guidance on CSI feedback enhancement [RAN1]

  - Focus subsequent working group discussions on the schemes proposed in RP-211297.

Details (e.g. how to calculate delta-MCS) are up to further working group discussions.

and the corresponding [RP-211297] proposal reads:

Proposal: RAN confirms the following as a guidance to RAN1 for CSI enhancement in Enhanced URLLC/IIoT WI:

RAN1 to further investigate the following for CSI enhancements for IIoT/URLLC:

- Increasing the number of bits used for the reported subband CQI (3-bits differential subband CQI or 4-bits CQI)

- Reporting of delta-MCS:

  - Report consists of delta-MCS for a TB received with MCS index IMCS:

    - delta-MCS is calculated from the difference between IMCS_tgt and IMCS, where IMCS_tgt is the largest MCS index such that the estimated BLER for a TB received with this MCS index would be smaller than or equal to a BLER target, and IMCS is the MCS index of the received TB.

In this contribution, we further elaborate on the CSI enhancements that have been recommended by RAN#92.

# 2	CSI Reporting Enhancements for more accurate link adaptation – 3bit/4bit differential/absolute CQI

3-bit/4-bit differential/absolute CQI has been regarded by a majority of companies as a straightforward enhancement with little impact on specifications, and the proposal was also endorsed by RAN#92. In the following we will assume that the discussion is about 3-bit differential CQI and 4-bit absolute CQI. Table 1 gives a high-level comparison between these alternatives.

Table 1: High level comparison of 3- and 4-bit SB-CQI reporting schemes.

| Scheme | Technical merits | Technical drawbacks | Standardization complexity |
| --- | --- | --- | --- |
| 3-bit differential SB-CQI | Provides some additional information on subbands as well as frequency-domain channel variations. | Limited visibility to SBs. High reporting overhead. | Low (in case of fixed offsets) |
| 4-bit absolute SB-CQI | Provides additional information on SBs as well as frequency-domain channel variations. Allows identifying worst SBs and/or CQI statistics in frequency domain. Enables URLLC service. | High reporting overhead. | Low |

Reporting overhead

Assuming e.g. SCS=30kHz, 2 ms reporting period, 100 PRBs and 13 subbands we can observe the following report sizes and overhead increases:

- 2 bit differential SB-CQI feedback (ref. case):	
 									4 bits (WB CQI) + 13SBs*2bits/SB = 30bits

- 3 bit differential SB-CQI: 	4 bits (WB CQI) + 13SBs*3bits/SB = 43bits  à +43% resource usage

- 4 bit absolute SB-CQI: 		4 bits (WB CQI) + 13SBs*4bits/SB = 56bits  à +87% resource usage

Considering that the WI is aiming for URLLC, and considering the fact that proper frequency-domain information is needed to support URLLC, we prefer the 4 bit absolute SB-CQI alternative from the options above. With 3 bits there will always be the problem of the limited information value of the report, which in practice makes it extremely difficult or impossible to serve URLLC traffic based on such limited information.

In case 3-bit differential SB-CQI will be specified, considering the limited remaining time for the WI, we prefer fixed pre-defined offsets which can be specified in 38.214 in the same way as 2-bit differential SB-CQI has been specified. At most two of the eight entries should indicate positive offset and most offset values should indicate negative offsets, simply because the feature is aimed for URLLC traffic and the reference resource size for SB-CQI is smaller than the reference resource size for WB-CQI, which already alone means that there is more use for negative offsets.

In order to reduce the reporting overhead, RAN1 may further consider options how the reporting overhead could be reduced e.g. by reporting only a subset of subbands. Currently, CSI-ReportConfig contains csi-ReportingBand (bit string) which can be used to specify which subbands are reported. Since it is generally not possible to know in advance which subbands will be the most interfered ones, UE should by default be configured to report all subbands, which results in excessive reporting overhead. Therefore, RAN1 may consider an alternative where UE reports only the CQI of the worst subbands. It should be noted that subband index information is not necessary (since the location of the worst subbands cannot be predicted in advance) and the number of subbands to be reported can be configured by gNB.

Proposal 1: In order to enable proper support for URLLC services, RAN1 should specify at least 4-bit absolute SB-CQI reporting.

Proposal 2: In case 3-bit differential SB-CQI will be specified,  RAN1 should specify fixed offsets in 38.214 and most offset values should be used for negative offsets.

Observation 1: In order to reduce reporting overhead, RAN1 can use the enhanced granularity of SB-CQI reporting to specify an option where UE reports the CQI from the worst subbands only.  Number of reported subband CQIs can be configured by gNB.

# 3	Additional UE measurements to improve (outer-loop) link adaptation accuracy – reporting delta-MCS

## 3.1 Scope of delta-MCS feedback

Legacy OLLA is based on binary ACK/NACK feedback for each scheduled PDSCH. Within the current WI discussions several companies have acknowledged that legacy OLLA is not a feasible solution for URLLC service, but some form of soft feedback from the UE is necessary. In this context, the introduction of a delta-MCS reporting based on PDSCH decoding for OLLA performance enhancement received significant support in the last RAN1 meetings and it was endorsed by RAN#92 meeting.

At the same time some companies were supporting delta-MCS feedback with the aim to improve the performance of HARQ-retransmissions, i.e. delta-MCS-feedback is given after HARQ-NACK. This approach is feasible only in a limited set of use cases, where there is time for retransmissions. While the motivation is easy to understand, retransmissions may be extremely rare depending on the service requirements, thus leading to very limited gains, and more importantly: we should first enable techniques which can bring the system performance to the point where HARQ retransmissions are extremely rare. This can only be done with more frequent feedback which is given after HARQ-ACK.

Proposal 3: RAN1 shall support delta-MCS new reporting at least based on HARQ-ACKs after initial transmission.

## 3.2 Delta-MCS feedback quantity

### 3.2.1 BLERtarget for delta-MCS-reporting

The proposal endorsed by RAN#92 reads “…largest MCS index such that the estimated BLER for a TB received with this MCS index would be smaller than or equal to a BLER target...“.  At the UE receiver a PDSCH reception has no BLERtarget associated to it (*), and therefore RAN1 needs to define what BLERtarget UE should assume for reporting delta-MCS.

(*) CQI-reporting has an associated BLERtarget, which is defined implicitly by the configured CQI-table.  In this case however, we are talking about delta-MCS, not delta-CQI.

When defining the BLERtarget that UE should assume, it should be kept in mind that 
a) different services have different PER targets at SDAP layer, and b) PHY layer BLERtarget changes dynamically between consecutive PDSCH transmissions, even if the PDSCHs belong to the same service flow. This can happen because of differences in the latency budget, segmentation of data, PDCP duplication etc.

One simple option is that gNB explicitly configures the UE with a BLER target that shall be used for determining the delta-MCS feedback to be reported, e.g. BLERTarget = 1E-5. As discussed earlier, BLER target for different TBs or repetitions of the same TB may be changing over time, and this information is not known at the UE. This means that the feedback received at the gNB may not be useful, e.g. the UE suggesting a delta-MCS to reach 1E-5, while gNB may require the delta-MCS for the BLER target 1E-2.

Some alternative ways to define PDSCH BLERtarget assumption for the UE could be e.g.

- gNB configures the BLERtarget that UE should assume e.g. together with MCS-table configuration.

- BLERtarget is indicated dynamically in the DCI.

- BLERtarget is implied by e.g. the selected DCI format, selected RNTI for the PDSCH, selected HARQ process ID, etc.

For each of the implied BLERtarget-alternatives, an association between the indicated information and BLERtarget can be defined in the specification, or gNB can configure a mapping between them.

Observation 2: A PDSCH transmission has no BLERtarget associated to it at the receiving UE.

Proposal 4: RAN1 needs to define what BLERtarget UE should assume for reporting delta-MCS taking into account variations of the actual BLER target selected by the gNB for PDSCH transmissions.

### 3.2.2 Which TBs should be used for deriving delta-MCS-feedback?

In order to reduce the feedback overhead and to reduce ambiguity of the payload size of the reported UCI, only one delta-MCS could be reported for multiple scheduled PDSCHs. Therefore, specific configuration rules are required to report delta-MCS feedback from a subset of scheduled PDSCH or to combine the information from multiple PDSCH receptions. The criteria to determine how to derive delta-MCS from several PDSCHs can be configured by the network, e.g.  based on PHY-priority, HARQ processes ID, a configured TBS or MCS threshold, etc.

Proposal 5: To support delta-MCS with reduced reporting overhead, a subset of the scheduled PDSCHs shall be selected to generate the delta-MCS feedback.

- To enable the interpreting of the UCI, both gNB and the UE shall be aware of the subset for which the reporting is applied.

### 3.2.3 Delta-MCS feedback quantity

R1-2105472 showed that a single-bit feedback after ACK provides good support for OLLA. Further, the same paper also considers NACK-based delta-MCS feedback for improved retransmission performance and concludes that multi-bit feedback does not provide meaningful additional gain. Thus we can conclude that regardless of the scope of the feedback (OLLA / retransmission), one-bit feedback is the most efficient option. One-bit feedback could be defined e.g. as

- delta-MCS=0 if  ( HARQ-NACK OR delta-MCS ≤ 0 ) è OLLA will be adjusted towards more robust (lower) MCSes,

- delta-MCS=1 if  ( HARQ-ACK AND delta-MCS > 0 ) è OLLA will be adjusted towards more efficient (higher) MCSes.

Proposal 6: RAN1 shall agree that delta-MCS feedback is given by providing 1-bit feedback from UE to gNB.

## 3.3 Delta-MCS reporting framework: HARQ or CSI

The new delta-MCS information reporting and periodicity needs to be properly handled. For instance, this new metric could be directly reported per PDSCH reception and can be directly included as an extension of the HARQ-ACK information (e.g. defined as a maximum, minimum or average of the MCS/CQI values of scheduled PDSCHs mapped to the HARQ-ACK codebook), or it can be jointly coded with the HARQ information or included in the CSI reporting feedback.

Aspects related to the reporting framework:

- support for OLLA and/or retransmissions:

  - feedback within CSI framework can support OLLA, but not HARQ retransmissions

  - feedback within HARQ framework can support both OLLA and HARQ retransmissions

- feedback overhead:  CSI framework provides potentially lower overhead if assuming that delta-MCS feedback is e.g. only reported from a subset of scheduled PDSCH and/or some compression like min/max or averaging of the delta-MCS from multiple PDSCH receptions is applied.

- reaction speed: HARQ-framework provides faster response to dynamically changing radio conditions.

Since NACKs may be nearly non-existent with URLLC, primary focus of the work should be on OLLA support. For that purpose delta-MCS feedback within CSI framework can do the job with very low overhead, even with just one bit. Reaction time to changing conditions is in this case limited by the CSI reporting periodicity.  Usage of HARQ-framework for delta-MCS feedback would allow faster reaction times and that could also support HARQ retransmissions, but a drawback would be the higher overhead.  Both frameworks for delta-MCS reporting seem to be feasible.

Considering the recent RAN1 discussions about enhancing SB-CQI feedback resolution & range, we take it that RAN1 does not consider the reporting overhead to be a serious problem. Having observed that and considering that using HARQ framework for delta-MCS feedback is somewhat more flexible than using CSI framework for the same purpose, we suggest that HARQ-framework should be used. In case RAN1 decides to proceed with HARQ-framework based delta-MCS feedback, it should consider ways to reduce the overhead.

Proposal 7: HARQ-ACK reporting framework should be used for delta-MCS feedback due to its flexibility.

# 4	Conclusions

In this contribution, we have discussed potential enhancements related to CSI feedback for URLLC/IIoT use cases, as summarized in the following proposals and observations:

Proposal 1: In order to enable proper support for URLLC services, RAN1 should specify at least 4-bit absolute SB-CQI reporting.

Proposal 2: In case 3-bit differential SB-CQI will be specified,  RAN1 should specify fixed offsets in 38.214 and most offset values should be used for negative offsets.

Observation 1: In order to reduce reporting overhead, RAN1 can use the enhanced granularity of SB-CQI reporting to specify an option where UE reports the CQI from the worst subbands only.  Number of reported subband CQIs can be configured by gNB.

Proposal 3: RAN1 shall support delta-MCS new reporting at least based on ACKs after initial transmission.

Observation 2: A PDSCH transmission has no BLERtarget associated to it at the receiving UE.

Proposal 4: RAN1 needs to define what BLERtarget UE should assume for reporting delta-MCS taking into account variations of the actual BLER target selected by the gNB for PDSCH transmissions.

Proposal 5: To support delta-MCS with reduced reporting overhead, a subset of the scheduled PDSCHs shall be selected to generate the delta-MCS feedback.

- To enable the interpreting of the UCI, both gNB and the UE shall be aware of the subset for which the reporting is applied.

Proposal 6: RAN1 shall agree that delta-MCS feedback is given by providing 1-bit feedback from UE to gNB.

Proposal 7: HARQ framework should be used for delta-MCS feedback due to it’s flexibility.