3GPP TSG RAN WG1 #106-e	R1-2107020

E-meeting, August 16th – August 27th, 2021

Source:	Panasonic

Title: 	Enhancements on UL time and frequency synchronization

Agenda Item:		8.4.2

Document for:	Discussion and decision

# Introduction

The work item on Non Terrestrial Networks (NTN) is considered. The detailed description of the WID [1] and the list of past agreements related to UL time and frequency synchronization in RAN1 can be found in the Appendix. In this document, we discuss issues on UL timing and frequency synchronization for NTN.

# UL timing synchronization

It was agreed in RAN1 that a non-terrestrial network may broadcast a common timing offset value. The rationale behind the common timing offset is to implicitly define the point where DL/UL frame alignment (i.e., the reference point for timing synchronization) can be observed. The exact location and definition of the reference point is a matter internal to the network. We describe aspects and details related to the common timing offset in the following sections.

## Feeder link timing drift handling

The purpose of a common timing drift rate, in addition to the base common timing offset value, is to adjust the feeder’s link common timing offset in accordance with LEO satellite movement, such that the RTD drift between the reference point and the satellite can be accounted for. Moreover, it seems that the common timing advance drift rate (and its variation) are envisaged as tools to enable tight UL/DL frame timing at the gNB. It requires potentially frequent SIB reading especially in RRC_CONNECTED mode for larger subcarrier spacing. Figure 1 shows exemplarily the drift rate of the feeder link RTD for a fixed-Earth beam. The change rates assume value between roughly +/-40ms/s during the visibility window of the satellite.

Figure 1: Change rates of feeder link RTD for a circular orbit (altitude 600km) and fixed-Earth beam with beam diameter 1000km

The assumptions underlying Figure 1 are as follows: The orbit is circular. Initially at time t=0s the satellite (here, altitude 600km) is visible from the gateway under the minimum elevation angle of 10°. The satellite then moves on its orbit through the zenith point of the GW (with elevation angle 90°) until it is no longer visible by the gateway (at t~=370s) from the opposite cell edge, for which a fixed Earth-beam is assumed with a beam diameter of 1000km. This implies that the elevation angle under which the satellite is seen will be time-variant. The feeder link RTD values between the gateway and the satellite are computed for various points in time. Earth’s rotation is modelled as well. From the feeder link RTD values the timing drift rate is obtained by numerical derivation.

It is evident that the changing rate of the propagation delay between LEO satellite and ground GW depends on the satellite position and, thus, is itself time-varying. Furthermore, the approximation of the actual TA value via linear interpolation will lead to a certain approximation error. In Figure 2, we have exemplarily for one case assessed the maximum error max.error=maxfor all ie(i) which occurs over various update intervals DT whereby

ei=maxtTAt0+TA't0*t-t0-TA(t) for i∙∆T≤t<i∙∆T+∆T and t0=i∙∆T

The rationale is as follows. The actual, true TA value is denoted as TA(t). The time axis t is partitioned into segments of duration ∆T during which the linear interpolation of the TA is evaluated as TAt0+TA't0*t-t0. The maximum error during the i-th segment is given by e(i). The maximum error show in  Figure 2 is the maximum taken over all segments. It is clear that the error due to a linear approximation increases with a larger update interval DT.

Figure 2: Maximum error between the true feeder link RTD value and its linear approximation for various update intervals T, circular orbit, satellite altitude 600km, beam diameter 1000km, Earth-fixed beam with minimum elevation angle of 10°

It is also worth noting that the approximation is modelled here as going outwards from the starting point of a segment, i.e., it is an extrapolation. With relaxed causal assumptions, i.e., not extrapolation, but interpolation for i∙∆T-∆T2 and i∙∆T+∆T2, the error could be smaller by a factor of 4 than currently shown.

The Cyclic Prefix duration for PUSCH is 4.7ms for SCS=15kHz and 0.57ms for SCS=120kHz. From Figure 2, it can be said that in order to confine the error within the CP length, timing drift rate indication is not sufficient and the update of the Common TA in a SIB is necessary. A SIB update every 4.5 seconds for SCS=15kHz and roughly every second for SCS=120kHz is required (Roughly twice these numbers for interpolation). Having a SIB update in NTN utilize all paging resources in a cell for the indication of SIB update requires too large of an overhead. And to newly specify periodic SIB reception is also a possible design but it requires large power consumption. Such operation should be avoided.

In Rel.15/16, gradual timing adjustment and TA command are used to adjust the UE’s transmission timing. The gradual timing adjustment specified in TS 38.133 section 7.1.2.1 says, “When the transmission timing error between the UE and the reference timing exceeds +/-Te, then the is required to adjust its timing to within +/-Te. The reference timing shall be (NTA+NTA,offset)*Tc before the downlink timing of the reference cell.”

Excerpt from TS 38.133 section 7.1.2

In accordance with LEO satellite movement, the first path (earliest arrived radio signal at UE) arrival timing changes, i.e., the reference timing described in TS38.133 changes as shown in Figure 1. According to the current specification (TS38.133), when the difference between UE transmission timing and the reference timing exceeds ±Te, UE is required to adjust its timing to within ±Te. Based on Rel.15/16 specifications the currently admissible values for the timing error limit are shown below (Table 7.1.2-1 from TS 38.133 Sec. 7.1.2) and it is clear that the timing advanced error due to the timing drift rate based linear approximation (shown in Figure 2) would be much larger than Te.

The use of a timing drift rate is an attempt to replicate the rather rigid UL/DL frame timing occurring in a terrestrial deployment, where UE uplink transmissions arrive at the gNB within the Cyclic Prefix duration compared to the DL timing. For the right decision in RAN1, it is necessary to investigate if the timing drift rate is a practical solution and whether and how timing drift rate indication can work considering that the timing drift rate is time-variant as shown in the Figure 1 and considering the inherent approximation error show in Figure 2.

We also noted that other companies in RAN1 recognize the deficiencies of linear approximation and propose to signal up to the third order derivative of the TA function. For example, from R1-2106142 (Ericsson) the following Figure 7 (left hand side) has been copied. The result of our attempt to replicate these findings is shown on the right-hand side in Figure 3. Please note that we consider a quarter of the cylic prefix for SCS 120kHz as the worst case error for PUSCH/PUCCH on the feeder link segment, which the system should not exceed (i.e., the horizontal magenta line in Figure 3). While the order of magnitude of the various approximations are in good alignment, the specifics of the results are not. In particular, from our results larger update periods lead to much worse maximum approximation errors then presented in R1-2106142. We believe that there is quite some variability in these results depending on the assumptions initially made.

| Figure 7: Feeder link RTT approximation error. (Copied from R1-2106142 by Ericsson.) | Figure 3: Feeder link RTT approximation error for comparison |
| --- | --- |

Rather than compensating the feeder link delay by the TA in such a very accurate manner, an alternative approach would be to allow for a DL-UL timing offset which is managed by the network implementation. Our view is that the feeder link switch operation (i.e. gateway change but keeping the same satellite to the UE) should be supported in a manner transparent to the UE. The DL-UL timing offset, including the variation of drift rate, is managed by the network implementation would allow such feeder link switch operation even if it is not supported in Release 17. If the timing drift rate is used, the drift rate may be required to be differentiated between two gateways before and after the feeder link switch.

Proposal 1: We propose to allow for a non-zero DL-UL timing difference managed by the network.

- FFS: The need and feasibility of a common timing drift rate to accurately compensate the feeder link delay including feeder link switch operation.

Proposal 2: Residual feeder link timing drift is compensated by the network.

An unclear issue is still the case when the common TA value is zero, which corresponds to a position of the timing reference point at the satellite. Although it was agreed that the value 0 is supported, it is not clear what that actually means. We see two options. Firstly, value 0 is assumed when NTA,common is not indicated to the device. Secondly, value 0 is explicitly signalled, which also implies that NTA,common is always signalled. The overhead reduction by omitting the common TA in SIB would not be significant, especially if the granularity is coarse as proposed above. In addition, NTA,common can contain TA margin amongst other margins which are potentially necessary for the network implementation. Therefore, in our view NTA,common should be always included in SIB.

Proposal 3: The common TA value NTA,common is always indicated in SIB.

- FFS: the value range of NTA,common

## Granularity and signalling of Common TA

According to [2], the maximum RTD including feeder link and service link for GEO is 541.46ms and for LEO (1200km) 41.77ms. If the reference point is assumed at the gNB, NTA,common compensates the full feeder link delay, and then NTA,common has to assume values which are approximately half of these durations, i.e., 270.73ms and 20.89ms, respectively. With Tc=0.509ns, NTA,common would have to assume relatively large integer values, e.g., 531886051 (29bits) and 41041257 (26bits). To avoid such large values, the granularity of the Timing Offset should be dependent on the Cyclic Prefix of the employed PRACH preamble.

Granularity is here understood as the step size in time by which the Timing Offset can be adjusted. Format 0 in FR1 has the smallest CP with 103us (3168*k), and Format B1 has the smallest CP with 7us (216*k*2- m). The granularity of the Timing Offset would have to be chosen sufficiently small if precise UL/DL frame timing alignment is necessary.

As outlined in the previous section, rather than enforcing a rigid UL/DL timing, an alternative approach is to allow for a timing offset between UL and DL frame timing which is managed by the network. Such an offset will appear naturally if all UEs in a cell are using the same common TA which is not necessarily identical to the true feeder link RTD. Since it will identically apply to all uplink transmissions in a cell, ISI among UEs is avoided at the expense of an UE common offset between UL and DL frames at the network. The network can adjust its reception timing (which is, e.g., in the order of slots of half-slots, and which is known to the network).

An additional benefit of this approach is less signalling overhead since the resolution (granularity) of NTA,common can be relatively coarse. Relatively coarse granularity in the order of slot or half slot granularity would be sufficient because the network would be able to adjust the UE common reception timing by its implementation.

Proposal 4: We propose a granularity of the common TA value NTA,common in the order of slot or half slot duration.

## The need and the indication of TA margin

A TA margin is envisaged in RAN1 to compensate for uncertainties in the estimation of TA related parameters. A non-zero DL-UL timing difference managed by the network offers the benefit that it can also manage such uncertainties by its implementation. Hence, we propose to include the TA margin transparently to the UE in the common TA.

Proposal 5: Include the TA margin transparently to the UE in the common TA.

## TA update in RRC_CONNECTED state

The UE specific TA compensates the RTD on the service link. A GNSS-capable UE can autonomously determine the UE-specific TA based on GNSS-acquired UE position and satellite ephemeris. In particular, we are supportive of the discussion during the last RAN1-meeting #105-e and the disposition that the UE specific TA is computed as the sum of the service link delay on UL and DL and that it is autonomously added on the TA command.

Proposal 6: The UE computes the UE specific TA as the sum of the service link delay on UL and DL. In RRC_CONNECTED state, the UE adds it autonomously on the TA command.

An open issue is the system’s behaviour in RRC_CONNECTED mode after the timing advance timer has expired. In that case, when TA timer is expired, a UE triggers the RACH transmission to establish the synchronization between UE and network again. TA based on GNSS location and satellite ephemeris should be used similar to RACH transmission for initial access.

Proposal 7: In RRC_CONNECTED mode and after expiration of the TA timer, a UE triggers the random-access procedure based on GNSS-acquired TA similar to RRC_IDLE with the same timing advance equation.

## NTN UE Validity Timers

We acknowledge the likely need for introducing validity timers for Common TA and satellite ephemeris for calculation of UE specific TA in order to maintain UL synchronization in NTN. It seems to be obvious that the UE requires some means to decide the time when these assistance information is valid and when it must be newly acquired. However, at this stage the details of the TA mechanism are still in flux, and any detailed discussion seems to be premature.

Proposal 8: Wait for progress on the TA procedure before introducing validity timers for satellite ephemeris and common TA to maintain UL synchronization in NTN. Decide early if this issue may better be handled in RAN2.

# UL frequency synchronization

## Indication of common frequency pre-compensation/post-compensation offset on the service link

The disposition after the RAN1#104bis-e meeting was to wait for progress on the common Doppler shift pre/post compensation on the feeder link before discussing the necessity of indicating a common compensation frequency offset to be applied by the UEs on the UL.

In the recent RAN1#105-e meeting it was concluded that the Doppler shift over the feeder link and any transponder frequency error for both Downlink and Uplink is transparent for the UEs, i.e., it is compensated by the GW and satellite-payload without any specification impacts in Release 17.

The actual pre-compensation value applied at UE side depends on the network implementation, e.g., pre/post-compensation of DL/UL doppler shift as discussed in [3]. Rather than indicating the amount of frequency compensation performed at the network side, the indication of the common frequency offset, that a UE should apply, would be useful to enable a flexible network implementation.

Proposal 9: Support the indication of the frequency offset that the UE shall apply for uplink transmissions at least for Earth moving cells. The offset is up to the network implementation and can comprise both DL pre-compensation and UL post-compensation at the network.

- FFS for Earth fixed cell

# Conclusion

In this contribution, we discussed issues on UL timing advance and frequency synchronization for NTN. We propose the following:

Proposal 1: We propose to allow for a non-zero DL-UL timing difference managed by the network.

- FFS: The need and feasibility of a common timing drift rate to accurately compensate the feeder link delay including feeder link switch operation.

Proposal 2: Residual feeder link timing drift is compensated by the network.

Proposal 3: The common TA value NTA,common is always indicated in SIB.

- FFS: the value range of NTA,common

Proposal 4: We propose a granularity of the common TA value NTA,common in the order of slot or half slot duration.

Proposal 5: Include the TA margin transparently to the UE in the common TA.

Proposal 6: The UE computes the UE specific TA as the sum of the service link delay on UL and DL. In RRC_CONNECTED state, the UE adds it autonomously on the TA command.

Proposal 7: In RRC_CONNECTED mode and after expiration of the TA timer, a UE triggers the random-access procedure based on GNSS-acquired TA similar to RRC_IDLE with the same timing advance equation.

Proposal 8: Wait for progress on the TA procedure before introducing validity timers for satellite ephemeris and common TA to maintain UL synchronization in NTN. Decide early if this issue may better be handled in RAN2.

Proposal 9: Support the indication of the frequency offset that the UE shall apply for uplink transmissions at least for Earth moving cells. The offset is up to the network implementation and can comprise both DL pre-compensation and UL post-compensation at the network.

- FFS for Earth fixed cell

# References

- RP-201256, “WID: Solutions for NR to support non-terrestrial networks (NTN)”, Thales

- TR38.821 V16.0.0, “Solutions for NR to support non-terrestrial networks (NTN)”

- R1-2106290, “FL Summary on enhancements on UL time and frequency synchronization for NR NTN”, RAN1#105-e

- R1-2106142, “On UL time and frequency synchronization enhancements for NTN”, Ericsson, RAN1#105-e

# Appendix

## WID

The complete WID [1] in RAN1 reads:

Enhancing features to address the identified issues due to long propagation delays, large Doppler effects, and moving cells in NTN, the following should be specified (see TR 38.821): - Timing relationship enhancements [RAN1, RAN2] - Enhancements on UL time and frequency synchronization [RAN1, RAN2] - HARQ - Number of HARQ process [RAN1] - Enabling / disabling of HARQ feedback as described in the TR 38.821 [RAN1&2] In addition, the following topics should be specified if beneficial and needed - Enhancement on the PRACH sequence and/or format and extension of the ra-ResponseWindow duration (in the case of UE with GNSS capability but without pre-compensation of timing and frequency offset capabilities) [RAN1/2]. - Feeder link switch [RAN2, RAN1] - Beam management and Bandwidth Parts (BWP) operation for NTN with frequency reuse [RAN1/2] Including signalling of polarization mode

## RAN1#102-e agreements

In the RAN1#102-e meeting, the following was agreed.

Agreement: - In Rel-17 NR NTN, at least support UE which can derive based on its GNSS implementation one or more of: - its position - a reference time and frequency - And, based on one or more of these elements together with additional information (e.g., serving satellite ephemeris or timestamp) signalled by the network, can compute timing and frequency, and apply timing advance and frequency adjustment at least for UE in RRC idle/inactive mode. - FFS: Details on additional information signalled from network Agreement: In case of GNSS-assisted TA acquisition in RRC idle/inactive mode, the UE calculates its TA based on the following potential contributions: - The User specific TA which is estimated by the UE: - Option 1: The User specific TA is estimated by the UE based on its GNSS acquired position together with the serving satellite ephemeris indicated by the network: - FFS: Details on serving satellite ephemeris indication - Option 2: The User specific TA is estimated by the UE based on the GNSS acquired reference time at UE together with reference time as indicated by the network - The Common TA if indicated by the network: - FFS: The need and details of Common TA indication - FFS: The TA margin, if needed and indicated by the network (in order to account for the TA estimation uncertainty)

## RAN1#103-e agreements

In the RAN1#103-e meeting, the following was agreed.

Agreement: An NTN UE in RRC_IDLE and RRC_INACTIVE states is required to at least support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris. Agreement: An NR NTN UE in RRC_IDLE and RRC_INACTIVE states shall be capable of at least using its acquired GNSS position and satellite ephemeris to calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. Agreement: An NR NTN UE in RRC_CONNECTED states shall be capable of at least using its acquired GNSS position and satellite ephemeris to perform frequency pre-compensation to counter shift the Doppler experienced on the service link. Agreement: In NTN, the network may broadcast - A common timing offset value - FFS details of the common timing offset - FFS: A common timing drift rate Before Msg1/MsgA transmission, the NR NTN UE in idle/inactive mode calculates its TA as follows: TA= NTA+NTA, offset[+X]×Tc[+X] Where: NTA is derived from the User specific TA self-estimation X is derived at least from the common timing offset value if broadcasted by the network. The granularity of X and whether X is indicated as a Timing Advance or as a Timing Offset value [unit] are FFS. Upon resolving the FFS, one of the X in the equation will be removed. NTA, offset depends on band and LTE/NR coexistence and is specified in TS 38.213 section 4.2. Tc is specified in TS 38.211 section 4.1. Note: UE will not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A. Working assumption: It is assumed that the requirement on UL time pre-compensation for Msg1/MsgA transmission of an NR NTN UE in idle/inactive mode will be defined such that the existing TAC 12-bit field in msg2 (or msgB) can be reused without any extension.

## RAN1#104-e agreements

In the RAN1#104-e meeting, the following was agreed.

Agreement: An NTN UE in RRC_CONNECTED state is required to support UE specific TA calculation based at least on its GNSS-acquired position and the serving satellite ephemeris. FFS: Operation of closed loop and open loop TA control Agreement: For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops shall be supported for NTN. FFS: Details of the combination of open and closed loop TA control Conclusion: It is up to RAN4 to decide whether interruptions or measurement gaps are required for GNSS measurements during NTN operation Agreement: RAN1 should send an LS to RAN4 with the following questions: Question 1: RAN1 would like to ask RAN4, to indicate what are the NTN UL time synchronization requirements? - For initial access (i.e. PRACH transmission) - For UL transmissions in RRC Connected State Question 2: RAN1 would like to ask RAN4, to indicate what are the NTN UL frequency synchronization requirements? - For initial access (i.e. PRACH transmission) - For UL transmissions in RRC Connected State Conclusion: If DL frequency compensation for the service link Doppler is applied, indication of the amount of frequency compensation is necessary. - FFS: support of DL frequency compensation for the service link Doppler. Agreement: - RAN1 to support satellite ephemeris broadcast based at least on one of the following format options: - Option 1: Ephemeris format based on satellite position and velocity state vectors - FFS: Details on state vectors formats - FFS: Details on time reference provisioning/format - Option 2: Ephemeris format based on orbital elements - FFS: Details on orbital elements formats - FFS: Details on time reference provisioning/format FFS: Whether down-selection is needed or both options are supported

## RAN1#104bis-e agreements

In the RAN1#104bis-e meeting, the following was agreed.

Agreement: The Timing Advance applied by an NR NTN UE in RRC_IDLE/INACTIVE and RRC_CONNECTED is given by: TTA=NTA+NTA,UE-specific+NTA,common+NTA,offset×Tc Where: - NTA is defined as 0 for PRACH and updated based on TA Command field in msg2/msgB and MAC CE TA command. - FFS: details of NTA update/accumulation. - NTA,UE-specific is UE self-estimated TA to pre-compensate for the service link delay. - NTA,common is network-controlled common TA, and may include any timing offset considered necessary by the network. - NTA,common with value of 0 is supported. - FFS: details of signaling including granularity. - NTA,offset is a fixed offset used to calculate the timing advance. Note-1: Definition of NTA is different from that in RAN1#103-e agreement. Note-2: UE might not assume that the RTT between UE and gNB is equal to the calculated TA for Msg1/Msg A. Note-3: NTA,common is the common timing offset X as agreed in RAN1 #103-e. Agreement: Support serving-satellite ephemeris broadcast based on one or more of the following: - Set 1: Satellite position and velocity state vectors: - position X, Y, Z in ECEF (m) - velocity VX, VY, VZ in ECEF (m/s) - Set 2: At least the following parameters in orbital parameter ephemeris format: - Semi-major axis α [m] - Eccentricity e - Argument of periapsis ω [rad] - Longitude of ascending node Ω [rad] - Inclination i [rad] - Mean anomaly M [rad] at epoch time to - FFS: Whether pre-provisioned ephemeris based on orbital elements can be used as reference. Thereby, only delta corrections can be broadcast in order to reduce the overhead - FFS: The field size for each parameter - FFS: The impact on signaling due to the required accuracy of serving-satellite ephemeris - FFS: Whether down-selection is needed or both sets are supported Conclusion: The orbital propagator model to be used at UE side can be left to implementation.

## RAN1#105-e agreements

Finally, in the recent RAN1#105-e meeting, the following was agreed.

Agreement: - Specifications should support delivery of ephemeris information using both ephemeris formats, i.e., state vectors and orbital elements. Agreement: RAN1 should send an LS to SA3, SA1 and possibly SA3-LI to get more inputs regarding the security/regulatory aspects if the NTN GW/gNB position is broadcast or possible to be derived by the UE with assistance information from the network, and on any aspects related to accuracy of the position. Conclusion: The Doppler shift over the feeder link and any transponder frequency error for both Downlink and Uplink is compensated by the GW and satellite-payload without any specification impacts in Release 17.