3GPP TSG RAN WG1 #106-e 	                                                         	    R1-2107272

e-Meeting, August 16th – 27th, 2021

Source:	OPPO

Title:	HARQ-ACK enhancements for Rel-17 URLLC/IIoT

Agenda Item:	8.3.1.1

Document for:	Discussion and Decision

Introduction

In RAN1 #104b-e and 105-e, HARQ-ACK feedback enhancements for enhanced IoT and URLLC were discussed with following agreements and working assumption:

Agreements:

- Support deferring SPS HARQ-ACK dropped due to TDD specific collisions until a next available PUCCH in Rel-17 based on semi-static configuration of slot format

  - FFS: Details (including possible conditions for such a deferring, whether or not to consider semi-statically configured flexible symbols for PUCCH availability, etc.)

  - Aim for minimal standardization efforts and UE complexity in implementation

Agreements: For SPS HARQ-ACK, the deferral from the initial slot/sub-slot determined by k1 in the activation DCI to the target slot/sub-slot determined by k1+ k1def, the UE will check the validity (TBD) of a target slot/sub-slot evaluating from one slot/sub-slot to the next sub/sub-slot (i.e. in principle k1def granularity is 1 slot/sub-slot)

- FFS: if there is a limit on the minimum deferral considered the required UE processing (k1def ≥0 >1)

- FFS: if there is a limit on the maximum deferral given in terms of e.g. k1def <=X or k1+k1def <=X

Agreement: Support Type-1 HARQ-ACK codebook for sub-slot based PUCCH configuration in Rel-17.

- The properties of the Type-1 HARQ-ACK codebook for sub-slot PUCCH at least includes that a PDSCH TDRA is associated with a UL /PUCCH sub-slot if the end of the PDSCH overlaps with the associated sub-slot determined by a k1 in the set of sub-slot timing values K1.

- FFS: whether the PDSCH TDRA grouping is performed per DL slot or sub-slot

  - Decide between PDSCH TDRA grouping per DL slot and sub-slot during RAN1#105-e

Working Assumption: For at least HARQ-ACK re-transmission:

- Support at least one enhanced Type 3 HARQ-ACK CB with smaller size (compared to Rel-16) in Rel-17

  - Definition of enhanced Type 3 CB:

    - The codebook size of a single triggered enhanced Type 3 HARQ-ACK codebook at least determined by RRC configuration

    - The codebook construction uses HARQ processes as a bases (i.e. ordered according to HARQ-IDs and serving cells)

- Support one-shot triggering (by a DL assignment) of HARQ-ACK re-transmission on a PUCCH resource other than enhanced Type 2 or (enhanced) Type 3 HARQ-ACK CB (i.e. Alt. 3) in Rel-17

  - Details are FFS

- Enhanced Type 3 HARQ-ACK CB and/or one-shot triggering (by a DL assignment) of HARQ-ACK re-transmission on a PUCCH resource other than enhanced Type 2 or (enhanced) Type 3 HARQ-ACK CB are subject to separate UE capabilities

Agreement: Support PUCCH carrier switching based on dynamic indication in DCI scheduling a PUCCH and semi-static configuration

- Details are FFS (including applicability of dynamic and/or semi-static means)

- Aim for minimum specification impact

- Dynamic indication and/or semi-static configuration are subject to separate UE capabilities

- The semi-static PUCCH carrier switching configuration operation is based on RRC configured PUCCH cell timing pattern of applicable PUCCH cells and supports PUCCH carrier switching across cells with different numerologies.

  - FFS whether additional rules are needed to support PUCCH carrier switching across cells with different numerologies

- FFS the maximum number of PUCCH cells

- FFS whether and how to support joint operation of dynamic and semi-static carrier switching for a UE

- FFS whether and how to support joint operation of PUCCH carrier switching and SPS HARQ-ACK deferral

In this contribution, we show our views on the HARQ-ACK enhancements for Rel-17 IoT and URLLC.

SPS HARQ-ACK deferral

Configuration of SPS HARQ-ACK Deferral

In RAN1 #104b-e, the configuration of the deferring of SPS HARQ-ACK dropped due to TDD was discussed and the following two options were proposed:

    - Option 1: per PUCCH cell group.

    - Option 2: per SPS configuration.

Different SPS configurations can have different latency requirements. For the SPS configuration with extremely short delay, “HARQ-ACK disabling /skipping for certain SPS configurations” was proposed. For some other SPS configuration, if HARQ-ACK can be transmitted with small k1, the feedback is useful; otherwise, redundant HARQ-ACK bits are transmitted. For some SPS configurations with lager latency requirements, SPS HARQ-ACK deferring can effectively improve the retransmission efficiency. Therefore, we support that the deferring of SPS HARQ-ACK dropped due to TDD specific collisions is configured by RRC per SPS configuration.

Proposal 1: SPS HARQ-ACK deferral should be configured by RRC per SPS configuration.

Limitations on k1def

The maximum timing between SPS PDSCH and corresponding HARQ-ACK should be limited, in order to avoid the ineffective HARQ-ACK feedback, i.e. the transmission of SPS HARQ-ACK exceeds the maximum latency. One approach is the maximum value of k1+ k1def of one SPS configuration is same as the maximum value of K1 set corresponding to the DCI format used to activate the SPS configuration. On the other hand, the minimum value of k1def is not needed.

Proposal 2: The maximum value of the total PDSCH to HARQ-ACK delay/offset, i.e. k1+k1def, of one SPS configuration is same as the maximum value of K1 set corresponding to the DCI format used to activate the SPS configuration.

Furthermore, if both SPS HARQ-ACK deferral and PUCCH repetition are configured, for a SPS PDSCH reception ending in slot/subslot n, the following three options can be considered:

    - Option a: If the first PUCCH occasion is no later than slot/subslot n+k1+k1def, a UE transmits the PUCCH as Rel-15/16.

    - Option b: If the last PUCCH occasion is no later than slot/subslot n+k1+k1def, a UE transmits the PUCCH as Rel-15/16.

    - Option c: If the first PUCCH occasion is no later than slot/subslot n+k1+k1def,

  - if the last PUCCH occasion is no later than slot/subslot n+k1+k1def, a UE transmits the PUCCH as Rel-15/16;

  - otherwise, the UE transmits the PUCCH repetition(s) no later than slot/subslot n+k1+k1def and cancels the PUCCH repetition(s) after slot/subslot n+k1+k1def.

We slightly prefer option c, since the feedback latency can be kept, and the UE can have more PUCCH transmission opportunities.

Proposal 3: When both SPS HARQ-ACK deferral and PUCCH repetition are configured, for a SPS PDSCH reception ending in slot/subslot n, if the first PUCCH occasion is no later than slot/subslot n+k1+k1def,

- if the last PUCCH occasion is no later than slot/subslot n+k1+k1def, a UE transmits the PUCCH as Rel-15/16;

- otherwise, the UE transmits the PUCCH repetition(s) no later than slot/subslot n+k1+k1def and cancels the PUCCH repetition(s) after slot/subslot n+k1+k1def.

Conditions for Deferral from /within the Initial Slot

The following conditions to determine the deferral of SPS HARQ-ACK were proposed in RAN1 #104b:

    - Alt. 1: Deferral only, if the SPS HARQ-ACK in the initial slot/sub-slot cannot be transmitted as the resulting PUCCH resource for transmission using the PUCCH by SPS-PUCCH-AN-List-r16 or n1PUCCH-AN is not valid.

    - Alt. 2: Deferral only, if the SPS HARQ-ACK in the initial slot/sub-slot cannot be transmitted as the resulting PUCCH resource for transmission using SPS-PUCCH-AN-List-r16, n1PUCCH-AN or other configured PUCCH resource(s) is not valid.

    - Alt.3: Defer if there is no available symbol for an UL transmission in the initial slot/sub-slot.

To simplify the terminal implementation, we suggest that:

    - The deferral of SPS HARQ-ACK should be semi-statically determined, i.e. the deferral result should not be changed by the reception of a dynamic PDSCH. In other words, deferral should be before multiplexing decision.

    - The UE using one uniform condition to check the validity of a target slot/subslot starting from the initial slot. In other words, the condition used to determine the deferral from the initial slot/subslot should also be used to check the validity of a target slot/subslot. From perspective of spec., clear definition of target slot/subslot is enough.

    - Only PUCCH resources configured to SPS PDSCH is used to check the validity of a target slot/sub-slot. Reasonable SPS-PUCCH-AN-List-r16 or n1PUCCH-AN can alleviate SPS HARQ-ACK dropping to some extent. Increasing PUCCH resource for SPS HARQ-ACK leads complex spec and implementation work.

Specifically, after receiving k1 from the activation DCI, a set of SPS-PDSCH-to-HARQ feedback timing values should be determined by the UE for one SPS configuration. We support the above Alt.1 is used as the uniform condition to check the validity of a target slot/subslot starting from the initial slot.

Proposal 4: Deferral should be before multiplexing decision.

Proposal 5: To determine the target slot/subslot for deferring SPS HARQ-ACK, starting from the initial slot/subslot,

- Deferral only, if the SPS HARQ-ACK in one slot/sub-slot cannot be transmitted as the resulting PUCCH resource for transmission using the PUCCH by SPS-PUCCH-AN-List-r16 or n1PUCCH-AN is not valid.

Check the Validity of a SPS PUCCH Resource

Take the example shown in Figure 1, the HARQ feedback timing (k1) of both SPS configuration 1 and SPS configuration 2 is 1, and PUCCH resource 1 ~3 are configured to transmit SPS HARQ-ACK:

    - PUCCH 1 is for up to 2 bits;

    - PUCCH 2 is for 3~9 bits;

    - PUCCH 3 is for more than 9 bits.

In slot n+2, based on the total payload of SPS HARQ-ACKs for PDSCH 0~3, PUCCH resource 2 is determined, but PUCCH 2 is not valid in slot n+2. However, PUCCH 3 is valid to transmit SPS HARQ-ACK with some padding bits. To avoid SPS deferral, we support that SPS PUCCH resources carrying N bits can be used as the candidates to check the validity of a SPS PUCCH resource for M-bit SPS HARQ-ACK associated to a slot, where N≥M.

Figure 1: One example of the candidates to check the validity of a SPS PUCCH resource

Proposal 6: SPS PUCCH resources carrying N bits can be used as the candidates to check the validity of a SPS PUCCH resource for M-bit SPS HARQ-ACK associated to a slot, where N≥M.

Check the Validity of a Slot for Deferral of SPS HARQ-ACK

One example is shown in Figure 2, the HARQ feedback timing (k1) of both SPS configuration 1 and SPS configuration 2 is 1, and PUCCH resource 1 is configured for up to 2 UCI bits and PUCCH 2 is configured for more than 2 UCI bit. To check the validity of a slot for deferral of SPS HARQ-ACK, the following options can be considered:

    - Option 1: A PUCCH resource is determined based on the total number of SPS HARQ-ACK associated to the slot. Specifically, for slot n+2 in Figure 2, PUCCH 2 is determined based the total HARQ-ACKs of SPS PDSCH 0~3. Since PUCCH 2 is not valid in slot n+2, HARQ-ACKs of SPS PDSCH 0~3 are deferred into slot n+4.

    - Option 2: If a PUCCH resource A, determined based on both the deferred SPS HARQ-ACKs and the non-deferred SPS HARQ-ACKs associated to the slot, is not valid, and

  - Option 2-1:  If a PUCCH resource B determined based on the non-deferred SPS HARQ-ACKs associated to the slot is valid, the UE transmits the non-deferred SPS HARQ-ACKs in the slot and the deferred SPS HARQ-ACK is dropped.

  - Option 2-2:  If a PUCCH resource B, determined based on urgent SPS HARQ-ACKs, is valid, the UE transmits the urgent SPS HARQ-ACKs in the slot. The urgent SPS HARQ-ACKs include:

    - HARQ-ACK for the SPS PDSCH with the maximum value of k1+ k1def;

    - HARQ-ACK for the SPS PDSCH not configured with SPS deferral.

Otherwise, check the validity of next slot for deferral.

Figure 2: One example of Option 2

Comparing with option 1, option 2-2 may achieve lower feedback latency and reduce SPS HARQ-ACK drop due to over-limit.

Proposal 7: To check the validity of a slot for deferral of SPS HARQ-ACK,

- If a PUCCH resource A, determined based on all SPS HARQ-ACKs associated to the slot, is not valid, and a PUCCH resource B, determined based on urgent SPS HARQ-ACKs (i.e. HARQ-ACKs for the SPS PDSCH with the maximum value of k1+ k1def and/or SPS PDSCH not configured with SPS deferral), is valid, the UE transmits the urgent SPS HARQ-ACKs in the slot;

- Otherwise, check the validity of next slot for deferral.

Retransmission of Cancelled HARQ-ACK

- Enhanced Type 3 HARQ-ACK CB

For enhanced type 3 HARQ-ACK CB, a new HARQ-ACK codebook should be constructed for HARQ-ACK retransmission. Redundant HARQ-ACK bits will be transmitted together with the cancelled HARQ-ACK, especially when multiple HARQ processes and/or serving cells are configured to the UE. To reduce the redundant HARQ-ACK bits, we support an enhanced Type 3 HARQ-ACK CB contains the HARQ-ACKs for the HARQ processes on a subset of serving cells. At least one subset of serving cells is configured by RRC, and DCI indicates one subset for HARQ-ACK feedback. Different subsets can be configured to different serving cells or DCI formats.

Self-carrier triggering can be supported as a special case, i.e. a DCI used to schedule cell X can only trigger an enhanced Type 3 HARQ-ACK CB contains the HARQ-ACKs for the HARQ processes on cell X. Comparing with R16 Type-3 HARQ-ACK CB, no additional DCI overhead is needed for enhanced type 3 HARQ-ACK CB with self-carrier triggering. Take Figure 3 as an example, a DCI on the self-scheduling cell 1 can trigger an enhanced Type 3 HARQ-ACK CB only contains the HARQ-ACKs for the HARQ processes on cell 1. A DCI, transmitted on cell 3 and used to schedule cell 2, can trigger an enhanced Type 3 HARQ-ACK CB only contains the HARQ-ACKs for the HARQ processes on cell 2.

Figure 3: Enhanced type 3 HARQ-ACK CB with self-carrier triggering

Furthermore, if multiple enhanced Type 3 HARQ-ACK CBs constructed based on different subsets are triggered in one slot/subslot, an enhanced Type 3 HARQ-ACK CB containing the HARQ-ACKs constructed based on the union of multiple subsets should be transmitted in the slot/subslot. One example is shown in Figure 4, in slot n+3, a first enhanced Type 3 HARQ-ACK CB constructed based on cell 1 and 2 is triggered by DCI 1, and a second enhanced Type 3 HARQ-ACK CB constructed based on cell 2 and 3 is triggered by DCI 2. UE transmits an enhanced Type 3 HARQ-ACK CB constructed based on cell 1, 2 and 3 in slot n+3.

Figure 4: Multiplexing of multiple enhanced Type 3 HARQ-ACK CBs

Proposal 8: At least one subset of serving cells is configured by RRC, and DCI indicates one subset for enhanced Type 3 HARQ-ACK feedback.

Proposal 9: Self-carrier triggering for enhanced Type 3 HARQ-ACK CB can be supported as a special case, i.e. a DCI used to schedule cell X can only trigger an enhanced Type 3 HARQ-ACK CB contains the HARQ-ACKs for the HARQ processes on cell X.

Proposal 10: If multiple enhanced Type 3 HARQ-ACK CBs constructed based on different subsets are triggered in one slot/subslot, an enhanced Type 3 HARQ-ACK CB containing the HARQ-ACKs constructed based on the union of multiple subsets should be transmitted in the slot/subslot.

- One-shot triggering of dropped HARQ-ACK

The cancelled HARQ-ACK codebook with/without other UCI can be directly retransmitted on a new PUCCH resource based on a DL assignment. The following options were proposed to identify the dropped HARQ-ACK that is to be retransmitted:

    - Option 1: Explicit indication. For example, 1-bit explicit indication to trigger the last dropped PUCCH.

    - Option 2: Implicit indication. A DCI, indicating a given HARQ process which is included into the dropped HARQ ACK codebook without NDI toggle, triggers a retransmission of the dropped HARQ ACK codebook, as shown in Figure 5. A LP PUCCH, including the HARQ-ACK for HARQ process X, is cancelled by a HP PUCCH. UE receives a DCI indicating HARQ process X without NDI toggle, then LP PUCCH should be retransmitted based on the PRI and k1 indicated by the DCI.

Figure 5. Implicit indication of dropped HARQ-ACK

Proposal 11: Implicit indication of a retransmitted HARQ-ACK CB for one-shot triggering of HARQ-ACK retransmission on a PUCCH resource can be considered.

Subslot-based Type-1 HARQ-ACK Codebook

To achieve lower overhead of subslot-based Type-1 HARQ-ACK CB, the following procedure for determining a set of occasions for candidate PDSCH receptions can be supported:

Step 1: Determine candidate DL subslots corresponding to one UL subslot based on the K1 set.

Step 2: If the last symbol of a PDSCH TDRA row r is not in the candidate DL subslots, row r is removed from the candidates of TDRA rows.

Step 3: Determine occasions for candidate PDSCH receptions based on the remaining PDSCH TDRA rows.

Proposal 12: To determine a set of occasions for candidate PDSCH receptions for subslot-based Type-1 HARQ-ACK CB:

- Step 1: Determine candidate DL subslots corresponding to one UL subslot based on the K1 set.

- Step 2: If the last symbol of a PDSCH TDRA row r is not in the candidate DL subslots, row r is removed from the candidates of TDRA rows.

- Step 3: Determine occasions for candidate PDSCH receptions based on the remaining PDSCH TDRA rows.

PUCCH Carrier Switching

To minimize the specification modification and implementation complexity, we support that:

- At most 2 PUCCH cells for PUCCH carrier switching can be configured to one UE.

- No additional rules are needed to support PUCCH carrier switching across cells with different numerologies.

- Joint operation of dynamic and semi-static carrier switching is not supported, since dynamic switching is flexible enough.

- Deferral of SPS HARQ-ACK without carrier switching should be firstly determined. Additional enhancement for joint operation of PUCCH carrier switching and SPS HARQ-ACK deferral is not supported.

Proposal 13: To support PUCCH carrier switching

- At most 2 PUCCH cells for PUCCH carrier switching can be configured to one UE.

- Dynamic carrier switching or semi-static carrier switching can be configured,

    - Additional enhancement on joint operation, including joint operation between dynamic carrier switching and semi-static carrier switching, and joint operation between carrier switching and SPS HARQ-ACK deferral is not supported.

Conclusions

In this contribution, we show our views on the HARQ-ACK enhancements for Rel-17 IoT and URLLC with following proposals:

Proposal 1: SPS HARQ-ACK deferral should be configured by RRC per SPS configuration.

Proposal 2: The maximum value of the total PDSCH to HARQ-ACK delay/offset, i.e. k1+k1def, of one SPS configuration is same as the maximum value of K1 set corresponding to the DCI format used to activate the SPS configuration.

Proposal 3: When both SPS HARQ-ACK deferral and PUCCH repetition are configured, for a SPS PDSCH reception ending in slot/subslot n, if the first PUCCH occasion is no later than slot/subslot n+k1+k1def,

- if the last PUCCH occasion is no later than slot/subslot n+k1+k1def, a UE transmits the PUCCH as Rel-15/16;

- otherwise, the UE transmits the PUCCH repetition(s) no later than slot/subslot n+k1+k1def and cancels the PUCCH repetition(s) after slot/subslot n+k1+k1def.

Proposal 4: Deferral should be before multiplexing decision.

Proposal 5: To determine the target slot/subslot for deferring SPS HARQ-ACK, starting from the initial slot/subslot,

- Deferral only, if the SPS HARQ-ACK in one slot/sub-slot cannot be transmitted as the resulting PUCCH resource for transmission using the PUCCH by SPS-PUCCH-AN-List-r16 or n1PUCCH-AN is not valid.

Proposal 6: SPS PUCCH resources carrying N bits can be used as the candidates to check the validity of a SPS PUCCH resource for M-bit SPS HARQ-ACK associated to a slot, where N≥M.

Proposal 7: To check the validity of a slot for deferral of SPS HARQ-ACK,

- If a PUCCH resource A, determined based on all SPS HARQ-ACKs associated to the slot, is not valid, and a PUCCH resource B, determined based on urgent SPS HARQ-ACKs (i.e. HARQ-ACKs for the SPS PDSCH with the maximum value of k1+ k1def and/or SPS PDSCH not configured with SPS deferral), is valid, the UE transmits the urgent SPS HARQ-ACKs in the slot;

- Otherwise, check the validity of next slot for deferral.

Proposal 8: At least one subset of serving cells is configured by RRC, and DCI indicates one subset for enhanced Type 3 HARQ-ACK feedback.

Proposal 9: Self-carrier triggering for enhanced Type 3 HARQ-ACK CB can be supported as a special case, i.e. a DCI used to schedule cell X can only trigger an enhanced Type 3 HARQ-ACK CB contains the HARQ-ACKs for the HARQ processes on cell X.

Proposal 10: If multiple enhanced Type 3 HARQ-ACK CBs constructed based on different subsets are triggered in one slot/subslot, an enhanced Type 3 HARQ-ACK CB containing the HARQ-ACKs constructed based on the union of multiple subsets should be transmitted in the slot/subslot.

Proposal 11: Implicit indication of a retransmitted HARQ-ACK CB for one-shot triggering of HARQ-ACK retransmission on a PUCCH resource can be considered.

Proposal 12: To determine a set of occasions for candidate PDSCH receptions for subslot-based Type-1 HARQ-ACK CB:

- Step 1: Determine candidate DL subslots corresponding to one UL subslot based on the K1 set.

- Step 2: If the last symbol of a PDSCH TDRA row r is not in the candidate DL subslots, row r is removed from the candidates of TDRA rows.

- Step 3: Determine occasions for candidate PDSCH receptions based on the remaining PDSCH TDRA rows.

Proposal 13: To support PUCCH carrier switching

- At most 2 PUCCH cells for PUCCH carrier switching can be configured to one UE.

- Dynamic carrier switching or semi-static carrier switching can be configured,

    - Additional enhancement on joint operation, including joint operation between dynamic carrier switching and semi-static carrier switching, and joint operation between carrier switching and SPS HARQ-ACK deferral is not supported.

References

- R1-2101818, “Moderator summary #2 on HARQ-ACK feedback enhancements for NR Rel-17 URLLC/IIoT”, Nokia

- R1-2106249, “Final moderator summary on HARQ-ACK feedback enhancements for NR Rel-17 URLLC/IIoT”, Nokia