3GPP TSG RAN WG1 #106-e		R1-2107285

e-Meeting, August 16th – 27th, 2021

Source:	OPPO

Title:	Discussion on Reply LS on L1/L2 centric inter-cell mobility

Agenda Item:	5

Document for:	Discussion and Decision

- Introduction

RAN4 replied the RAN1 LS on TCI state update for L1/L2-centric inter-cell mobility in [2]. In the Reply LS [2], RAN4 asks RAN1 to clarify a couple questions as follows:

RAN4 reply to Question 5: RAN4 would like to ask the following clarification questions for RAN4 to further respond to the question related to RAN4. - It is RAN4 common understanding that for the intra-band CA scenario the primary serving cell and secondary serving cell(s) belong to the same frequency band, rather than the serving and non-serving cells belong to the same frequency band as described in the question. Likewise intra-band CA explanation, for inter-band CA case a serving cell and secondary serving cell(s) belong to different frequency bands rather than the serving and non-serving cells. Can RAN1 please further clarify the intra-band/inter-band scenarios based on above RAN4 common understanding? - RAN4 would like RAN1 to clarify whether “the operation” in question 5 refers to only L1/L2-centric inter-cell mobility or both inter-cell mobility and inter-cell mTRP operations. Does RAN4 need to consider inter-cell mTRP operation into the “CA scenario” of Question 5? - RAN4 also would like RAN1 and RAN2 to further clarify on the definition of non-serving cell especially whether UE shall support data/control channel reception and transmission from non-serving cell including simultaneous reception and transmission capabilities under CA scenarios. It is RAN4 understanding that the response to question 5 are related to above clarification questions. RAN4 will further respond to question 5 after receiving the feedback from RAN1 and RAN2 on above questions.

In this contribution, we will provide our views for the aforementioned question from RAN4.

# Discussions

As in the revised FeMIMO WID [1], the work scope for ‘L1/L2-centric inter-cell mobility’ has been updated as follows:

- Enhancement on multi-beam operation, mainly targeting FR2 while also applicable to FR1: - Identify and specify features to facilitate more efficient (lower latency and overhead) DL/UL beam management for intra-cell and inter-cell scenarios to support higher UE speed and/or a larger number of configured TCI states: - Common beam for data and control transmission/reception for DL and UL, especially for intra-band CA - Unified TCI framework for DL and UL beam indication - Enhancement on signaling mechanisms for the above features to improve latency and efficiency with more usage of dynamic control signaling (as opposed to RRC) - For inter-cell beam management, a UE can transmit to or receive from only a single cell (i.e. serving cell does not change when beam selection is done). This includes L1-only measurement/reporting (i.e. no L3 impact) and beam indication associated with cell(s) with any Physical Cell ID(s) - The beam indication is based on Rel-17 unified TCI framework - The same beam measurement/reporting mechanism will be reused for inter-cell mTRP - This work shall only consider intra-DU and intra-frequency cases - Identify and specify features to facilitate UL beam selection for UEs equipped with multiple panels, considering UL coverage loss mitigation due to MPE, based on UL beam indication with the unified TCI framework for UL fast panel selection

The work scope of “L1/L2-centric inter-cell mobility” is changed to inter-cell beam management. Specifically, the inter-cell beam management does not involve serving cell change. The UE can be indicated with a beam that is associated with a physical cell ID that is different from that of the current serving cell.  To support this updated feature, in our view, the followings can be supported:

- The UE can be configured to conduct L1 beam measurement and reporting on SSBs associated with one physical cell ID that is different from that of the current serving cell.

- The UE can be configured with TCI state(s) where the QCL source CSI-RS resource can be QCLed with a SSB associated with one physical cell ID that is different from that of the current serving cell.

- The UE can be indicated with one TCI state that is indirectly associated with a SSB associated with one physical cell ID that is different from that of the current serving cell.

Since there is no serving cell change, the UE shall not be requested to read the system information (e.g., MIB) associated with the physical cell ID that different from that of the current serving cell. Furthermore, as specified in the revised WID, the inter-cell beam management assumes intra-DU and intra-frequency. This work focuses on FR2 where the cell range is small. The TCI state changing to different physical cell ID only happens in the overlapping area of two TRPs, therefore, the timing difference between TRPs is not significant in practice and we shall assume TRPs are synchronized and the timing advance is maintained so that extra RACH is not needed.

Regarding the question from RAN4 on intra-band/inter-band scenario, as stated in revised WID, only intra-DU and intra-frequency scenario are considered in inter-cell beam management and there is no serving cell change, therefore we only need to consider intra-frequency SSBs associated with a physical cell ID that is different from that of the serving cell.

Regarding the question from RAN4 on inter-cell mTRP operation: in our understanding, the inter-cell mTRP operation is still operated in each CC, as the mTRP operation specified in release 16.

Regarding the question from RAN4 on non-serving cell: in the latest work scope of inter-cell beam management, we assume there is no serving cell change. The UE still only support data/channel reception/transmission to the serving cell. That will only includes L1-based beam measurement/reporting on SSBs associated with a physical cell ID that is different from that of the serving cell and beam indication of a TCI state that is indirectly QCLed to a SSB associated with a physical cell ID that is different from that of the serving cell.

Based on the above discussion, we propose to clarify the questions from RAN4 as follows:

Proposal 1: Answer the questions from RAN4 as follows

- As in the revised FeMIMO WID, the work scope is inter-cell beam management. In this work, no serving cell change is assumed and only the case of intra-DU and intra-frequency is considered. Furthermore, it shall be assumed that TRPs are synchronized and time advance is maintained so that RACH is not needed. The only work involved is the L1-based beam measurement/reporting on some SSBs associated with one physical cell ID that is different from that of the serving cell and beam indication of one TCI state that is indirectly QCLed with a SSB associated with one physical cell ID that is different from that of the serving cell.

- Conclusion

In this contribution, we presented our views on the work of inter-cell beam management and proposals on answers to the questions of RAN4 are provided:

Proposal 1: Answer the questions from RAN4 as follows

- As in the revised FeMIMO WID, the work scope is inter-cell beam management. In this work, no serving cell change is assumed and only the case of intra-DU and intra-frequency is considered. Furthermore, it shall be assumed that TRPs are synchronized and time advance is maintained so that RACH is not needed. The only work involved is the L1-based beam measurement/reporting on some SSBs associated with one physical cell ID that is different from that of the serving cell and beam indication of one TCI state that is indirectly QCLed with a SSB associated with one physical cell ID that is different from that of the serving cell.

- Reference

RP-211586 Revised WID: Further ehancements on MIMO for NR

R1-2106426 Reply LS on L1/L2 centric inter-cell mobility