| 3GPP TSG-RAN WG1 Meeting #106-e e-Meeting, August 16th –27th, 2021 | R1-2107328 |
| --- | --- |

Agenda item:	8.1.3

Source: 	Qualcomm Incorporated

Title: 	Discussion on SRS enhancement

Document for:	Discussion/Decision

# Introduction

Rel-17 RAN1 work-item on further enhanced MIMO (FeMIMO) [1] has scoped the following on SRS enhancement for both FR1 and FR2.

Enhancement on SRS, targeting both FR1 and FR2: - Identify and specify enhancements on aperiodic SRS triggering to facilitate more flexible triggering and/or DCI overhead/usage reduction - Specify SRS switching for up to 8 antennas (e.g., xTyR, x = {1, 2, 4} and y = {6, 8}) - Evaluate and, if needed, specify the following mechanism(s) to enhance SRS capacity and/or coverage: SRS time bundling, increased SRS repetition, partial sounding across frequency

In the previous RAN1 meetings, few agreements were made to enable A-SRS flexible triggering, specify SRS antenna switching for 6Rx and 8Rx devices and enhance SRS coverage and capacity by increase repetition and partial frequency sounding. In this contribution, we continue the discussion of the remaining issues and share our views on the different designs’ alternative.

# Aperiodic SRS Triggering

## Flexible triggering

The definition of the available slot was specified as a slot with enough UL or flexible symbols that accommodate the transmission of all SRS resources within the set while meeting UE capabilities of minimum timeline requirement. This framework provides flexibility for triggering A-SRS while making sure that there is perfect alignment between gNB and UE behaviour for determining the available slot that rely on RRC configuration while not considering any dynamic events such as SFI and/or ULCI events. This makes it simple where the UE considers the configured UL-DL TDD patterns and the SRS time resource allocation of the triggered aperiodic SRS resource set to determine which slot for sending the aperiodic SRS. Collision handling between triggered SRS resource set(s) and other UL signal/channels is performed after determination of the available slot.

Two items are left for further discussion; one to down select between option-1 and option-2 for reference slot determination from which UE starts counting towards available slot. The other item is how to handle the scenario where multiple triggered SRS resource sets with overlapping symbols.

In this section, we first discuss the remaining two issues of selection of the reference slot and multiple sets triggering followed by the discussion on the DCI indication of the available slot.

Agreement An “available slot” is a slot satisfying there are UL or flexible symbol(s) for the time-domain location(s) for all the SRS resources in the resource set and it satisfies UE capability on the minimum timing requirement between triggering PDCCH and all the SRS resources in the resource set. - From the first symbol carrying the SRS request DCI and the last symbol of the triggered SRS resource set, UE does not expect to receive SFI indication, UL cancellation indication or dynamic scheduling of DL channel/signal(s) on flexible symbol(s) that may change the determination of “available slot”. - Note: Collision handling between the triggered SRS and any other UL channel/signal is performed after the determination of available slot. - FFS: Rules to handle the case of multiple SRS resource sets with overlapping symbols and/or triggered by a same DCI

### Reference slot

For down selection between both options, UE complexity should be considered and legacy back compatibility as the new triggering mechanism should be incremental and build on top of current framework. Also, Rel-17 UE should support both legacy schemes based on fixed slot as well as the new triggering mechanism based on available slot indication. From UE complexity point of view and timeline perspective, it is preferred to have the reference slot based on legacy Slot Offset. This enables simple UE implementation to handle both scenarios of triggering mechanisms in which the UE either transmits the SRS on legacy slot offset for legacy triggering scheme or at a later slot determined by DCI indication of available slot.

Figure 2-1: Reference slot based on legacy Slot.

Observation 1: A Rel-17 UE supporting enhanced A-SRS triggering and transmission based on available slot indication should support legacy triggering mechanism based on slot offset. Reference slot based on slot offset is an incremental addition on legacy scheme and does not impact UE complexity neither timeline.

Also, option 2 of the reference slot is more general and can accommodate option 1 if the slotOffset is set to zero or not configured. Accordingly, the reference slot can be determined by the slot of the legacy triggering offset if configured, otherwise is determined by slot indicated by the legacy triggering offset.

Observation 2: Option 2 of reference slot can accommodate option 1 by NW implementation of slofOffset set to zero or not configured.

Proposal 1: Support option 2 where reference slot is indicated by the legacy triggering offset.

### Multiple SRS resource Sets Triggering

A single DCI may trigger multiple SRS resource sets, and in some scenario, the triggered SRS resource sets may have overlapping symbol(s). Depending on how available slot is determined for each set, in some scenario a collision of the triggered SRS resource sets my happen. In general, when single DCI trigger multiple SRS resource sets, there could be two approaches for determination of available slot.

- The available slot is determined independently per each set, then a collision between the two sets may happen.

- The available slot is determined sequentially, and the UE considers the UL/flexible symbols for the first determined set as unavailable for the second set.

To further illustrate, we consider an example where a DCI triggers two SRS resource sets and indicate t=0 for the first SRS resource set and t=1 for the second SRS resource set. Assuming the slot offset is configured to be the same (slot offset = 1) for both sets, then both sets will collide in the U slot which represents the available slot for each set when determined independently.

Figure 2-2: Collision between triggered SRS resource sets

Such collision can be avoided with a sequential approach for determining the available slot for each set. This mechanism is illustrated in Figure 2-3 where the available slot is determined for the SRS resource set #1 first and then the U slot is considered as non-available for SRS set #1.  However, this require redefinition of the available slot and may complicate UE processing.

Figure 2-3: sequential determination of available slot per each triggered SRS resource set

Observation 3: For multiple triggered SRS resource sets with overlapping symbol(s), there are two options to handle this scenario:

- Option 1: The available slot is determined independently for the triggered sets and a dropping rule is introduced when a collision happens.

- Option 2: The available slot is determined sequentially for the triggered SRS resource sets.

Proposal 2: Support Option 1 to introduce dropping rule when collision happen between A-SRS resource sets.

## DCI-based indication of available slot indication

In the RAN1 #104b-e meeting, it was agreed that a list of up to four ‘t’ values is RRC configured per each SRS resource set and DCI indicates one of the t values. Also, as working assumption, a new configurable DCI bitfield (up to 2bits) is added to indicate one of the t values. This applies to the scheduling DCI format 0_1, 0_2, 1_1 and 1_2 and also to non-scheduling DCI format 0_1 and 0_2 if that RRC parameter is not configured.

Agreement - Up to 4 “t” values can be configured per SRS resource set. Working Assumption For DCI indication of “t” in Rel-17 SRS triggering offset enhancement - For both DCI that schedules a PDSCH/PUSCH and DCI 0_1/0_2 without data and without CSI request - t is indicated by adding a new configurable DCI field (up to 2 bits) - Applies only when there are multiple candidate values of t configured - No further enhancement to indicate “t” for DCI 0_1/0_2 without data and without CSI request at least when the new DCI field is configured

For the scenario when the new DCI field is not configured and SRS is triggered with non-scheduling DCI format 0_1 or 0_2, unused DCI bitfields could be utilized to explicitly indicate the available slot ‘t’. This will give the flexibility and may enable finer granularity to indicate the available slot per triggered SRS resource set separately. However, this results into two different indication methods of available slot depending on whether the DCI filed is configured or not. From UE perspective, it comes at a cost as the UE should support by implementation both indication methods. Based on this, we prefer to confirm the working assumption to keep a unified framework for available slot indication using the configurable DCI bitfield.

Proposal 3: Confirm the working assumption with the following updates:

For DCI indication of “t” in Rel-17 SRS triggering offset enhancement

- For both DCI that schedules a PDSCH/PUSCH and DCI 0_1/0_2 without data and without CSI request

  - t is indicated by adding a new configurable DCI field (up to 2 bits)

    - Applies only when there are multiple candidate values of t configured

  - No further enhancement to indicate “t” for DCI 0_1/0_2 without data and without CSI request at least when the new DCI field is configured

## MAC-CE for flexible SRS triggering

To enable flexible and faster timeline for the reconfiguration of the list of the available slot ‘t’ of an SRS resource and the reference slot (i.e legacy slot offset), a MAC CE command can be introduced in Rel-17 to update the slot offset of the aperiodic SRS resource. This mechanism may be very helpful for an SRS resource set with many numbers of SRS resources, such that the dynamic update of the slot offset may enable a better multiplexing of the SRS resource set with other UL channels and a variety of slot formats.

To further enable more flexibility, in addition to updating the slot offset of the aperiodic SRS resource set, the same MAC CE command can update the number of the active SRS resources of the associated SRS resource set. This solution enables flexible activation or deactivation of the SRS resources within each SRS resource. In the current 3GPP framework when a specific A-SRS set is triggered, then all the SRS resources within the set should be transmitted. The proposed scheme enables finer granularity and flexibility to select which resources within the set to be transmitted.

Proposal 4: Support the introduction of new MAC-CE to reconfigure the available slot and legacy slot offset of SRS resource set.

- In addition, the MAC-CE can include a bitmap that can indicate the activate or inactivate SRS resources within the set.

This mechanism may be beneficial for some scenarios as ‘antenna Switching’ SRS resource set which has more than 4 SRS resources or a ‘beam management’ SRS resource set with many SRS resources with different beams. This gives the flexibility to the network to control which resources are transmitted when this resource set is triggered. For ‘Antenna Switching, this is useful for dynamic adaption of SRS switching configuration within the BWP. More detailed discussion of the reconfiguration of the SRS resources within a set is presented at subsection 3.8.

## Enhanced A-SRS trigging by non-scheduling DCI

In the RAN1 103e meeting, it was agreed to support a non-scheduling UL DCI format 0_1 and 0_2 to trigger aperiodic SRS without data and without CSI request. And in RAN1 meeting #104e, it was agreed to further study how to achieve further enhancement to aperiodic SRS triggering by repurposing some of the unused bitfields to enable some SRS related functionality. In this section, we discuss the functionalities of the non-scheduling, which bit fields to be repurposed and the support of other DCI formats.

Agreement Further study whether and if needed, how to achieve further enhancements on aperiodic SRS triggering and resource management based on repurposing unused fields in DCI format 0_1/0_2 without data and without CSI. Consider the following examples - CAT A: Time-domain parameters - A-1: Indication of available slot position, i.e., the t values - A-2: Indication of slot offset - A-3: Indication of SRS symbol-level offset - A-4: Indication of time-domain behavior for SRS transmission over multiple OFDM symbols, e.g., repetition, hopping, and/or splitting - CAT B: Frequency-domain parameters - B-1: Indication of a group of CCs for SRS transmission - B-2: Indication of frequency domain resource in a BWP for SRS transmission - B-3: Indication of whether DL/UL BWP is applied for SRS transmission - CAT C: Power control parameters - C-1: Re-purpose ‘TPC command for PUSCH’ as ‘TPC command for SRS’ - FFS impact on power control, impact from triggering a group of CCs for SRS - C-2: Indication of open loop power control parameter e.g., p0. - CAT D: Spatial-domain parameters, i.e., indication of SRS port and beamforming - CAT E: Extend the number of DCI codepoints for aperiodic SRS trigger states Other examples are not precluded

### CAT-B: Frequency domain (group of CC SRS triggering)

One of the main objectives to adopt of non-scheduling DCI is to enable more flexible triggering of the A-SRS resource set(s) and reduce the DCI overhead where single DCI can trigger A-SRS on one or more component carriers. The DCI should enable A-SRS triggering on single CC (either the scheduling CC or a different CC) or can enable multiple CCs (a group of CCs) A-SRS triggering.

### CAT-C: Power control

The last needed functionality is to include a TPC command to enable closed loop power control for the triggered SRS resource set.

Proposal 5: Support the following functionalities for DCI 0_1 and 0_2 triggering aperiodic SRS without data and without CSI:

- CAT-B: Enable A-SRS on a single CC or a group of CCs

- CAT-C: Indicate the TPC command for each CC.

## A-SRS triggering xCC using scheduling DCI

To reduce the DCI overhead for UL-CA, a single DCI can be used to trigger multiple aperiodic SRS resource sets jointly on different UL carrier which have PUSCH/PUCCH configured. An example is shown in Figure 2-4 where the UE receives a single DCI on one of the component carriers that triggers SRS transmission on CC1 + CC2 + CC4 where all the component carriers are configured for UL CA and there is no need for carrier switching. The component carrier configuration can be intra-band CA as CC1 and CC2 or inter-band CA as CC4 and CC1 or CC2 and could correspond to FR1 and/or FR2. Each of the triggered SRS resource sets have their own configured slot offset and could be transmitted on the same OFDM symbols or not.

Figure 2-4 Aperiodic SRS triggering for cross component carriers

The A-SRS transmission of the above UL CA can be enabled without extra DCI overhead. For example, when the UE receives an SRS request filed in the DCI from a CC1 with codepoint value for example ‘01’, then the UE is expected to transmit all SRS resource sets in the respective active BWP of all CCs that are associated with codepoint ‘01’ in each of the perspective CCs. To simplify the procedure on which CC where DCI can be received and trigger SRS transmission across the other component carriers, a new RRC parameter ‘CC index field’ could be added at the AP-SRS resource set level that indicates the CC index from which a DCI can be received that would trigger this specific AP-SRS resource set. This fields may have multiple CC indices.

Proposal 6: Support single scheduling DCI to trigger simultaneous A-SRS transmission across multiple component carriers.

- RAN1 further discussion on the specific configuration of the RRC configuration and triggering mechanism.

## Group common DCI

NR Rel-15 introduced group common DCI format 2_3 for SRS carrier switching. It is used to trigger aperiodic SRS resource set(s) with ‘antenna switching’ usage for UL carriers without PUSCH / PUCCH configured. The group common DCI format 2_3 is used to enable group of UEs for transmitting SRS signals. The DCI contents provide the TPC command for SRS transmission along with an optional SRS request. However, it is limited to the transmission of SRS resource sets with ‘antenna switching’ usage only. And it is only used for carrier switching where UE switches from the serving cell to another cell without PUSCH/PUCCH configured or another UL on which SRS power control is noted tied with PUSCH power control.

Observation 4: The group common DCI format 2_3 has some limitation where it triggers only SRS transmission for ‘antenna Switching’ use cases and is limited to UL carrier switching.

To enable aperiodic-SRS triggering across multiple UEs and across multiple serving cells per each UE, multiple PDCCHs need to be scheduled by the network which incurs large DCI overhead. Two alternatives have been proposed to either enhance UE-specific DCI or group-common. The first alternative may enable a single DCI to trigger A-SRS across multiple serving cells as discussed in section 2.4.1. The second alternative may enable SRS triggering across multiple UE.

To enable more flexible design and reduce the DCI overhead, the current DCI format 2_3 can be enhanced, or a new DCI format can be introduced to mitigate the limitation of the current SRS carrier switching mechanism. In our views, the SRS triggering should be flexible and should enable the SRS transmission for all SRS usages and should not be limited to antenna switching usage. Also, the framework should enable multiple CCs SRS triggering and simultaneous transmission without the need of carrier switching. An example of the DCI contents with multiple blocks is shown in Figure 2-5 where the block may indicate to the UE which CC the group and corresponding trigger codepoint and the TPC command.

Figure 2-5: Example of group common DCI for multiple UEs and xCC SRS triggering

Proposal 7: Support the enhancement of DCI formant 2_3 to enable multiple UE SRS transmission across multiple serving cells per UE.

# SRS antenna switching for 6 and 8 Rx

In the previous meeting, it was agreed to support SRS resource configuration for 1T6R, 1T8R, 2T6R, 2T8R and 4T8R while 4T6R is FFS. In RAN1 meeting #104bis-e, more progress was made, and the maximum number of SRS resource sets for each antenna switching configurated was agreed where N_max = 4 for 1T8R/2T8R, N_max = 3 for 1T6R/2T6R and N_max = 2 for 2T8R. Three different alternatives were listed as different design methodologies for the support of the number of SRS resources less than the maximum.

Agreement On aperiodic SRS configuration for antenna switching with > 4Rx, support the following N_max values - 1T6R: N_max = 3 - 1T8R: N_max = 4 - 2T6R: N_max = 3 - 2T8R: N_max = 4 - 4T8R: N_max = 2 - The support of N_max value does not imply the support of N value that is smaller than N_max. This is FFS. - FFS whether further enhancement for single-DCI or multi-DCI based MTRP is needed Agreement On supported values of N for Rel-17 aperiodic SRS antenna switching with >4Rx, down-select at least one of the following alternatives in RAN1#105e - Alt 1: All the non-zero integer values <= N_max are supported for N - Alt 2: Support N=N_max only - Alt 3: Support specific N values <= N_max - FFS whether different alternatives may be selected for the same xTyR configuration subject to the UE capability on maximum number of symbols that can be used for SRS in a slot - FFS: whether different alternatives may be selected for different xTyR configuration

In our views, to determine the different configurations of number of resource sets for each antenna switching configuration for A-SRS, three design factors should be taken into considerations. First, A-SRS for antenna switching is meant to get an updated DL CSI in timely manner to enable proper DL scheduling. Therefore, if possible, UE should sound all its antenna ports within short amount of time, preferably in one slot. Secondly, depending on UE capability whether SRS transmission is limited to last 6 OFDM symbols within the slot or not. The third factor is the guard symbol between antenna ports which depends on SCS where a minimum of 1 guard symbol is required for 15/30/60 KHz and two guard symbols are needed for 120 KHz SCS.

Observation 5: DL CSI through A-SRS antenna switching should be captured within short period of time (i.e., least number of slots) otherwise DL scheduling may be impacted due to CSI decorrelation.

## Aperiodic SRS antenna switching configuration.

The following subsections show the proposed configuration of aperiodic SRS antenna switching.

### SRS configuration for 1T6R

In general, 6 SRS resources are needed where each resource has single SRS port. For aperiodic SRS antenna switching, one or two guard symbols are required between the two SRS resources within the set. Therefore, considering single SRS resource set with 6 SRS resources, 12 OFDM symbols are needed for 30KHz SCS which can fit into one slot if the UE supports SRS transmission across all OFDM symbols within the set. Otherwise, single slot will not accommodate if the UE is limited of SRS transmission to last 6 SRS symbols within the slot. In that case two SRS resource sets are required; one per each slot. The examples for single SRS resource set with 6 resources is shown in Figure 3-1.

Figure 3-1: 1T6R using single SRS set with 6 resources (30KHz SCS – 1 guard symbol)

Proposal 8: For 1T6R, support N = 1, 2 SRS resource sets.

- For N = 1, support mapping of 6 SRS resources per the SRS resource set.

- For N = 2 sets, support mapping of SRS resources of (3,3) across the two SRS resource sets.

- For N_max = 3 sets, support mapping of SRS resources of (2,2,2) across the three SRS resource sets.

### SRS configuration for 1T8R

For 1T8R sounding, 8 SRS resources are needed where each resource has single SRS port. For aperiodic SRS resource set, one or two guard symbols are required between the two different antenna ports. Therefore, single slot can’t accommodate single SRS resource set with 8 SRS resources. That is why for aperiodic SRS, at least two SRS resource sets can be used for 1T8R, with four SRS resources within each SRS resource set. For UE limited to last 6 SRS symbols, three SRS resource sets are required.

Figure 3-2: 1T8R SRS switching configuration using 2 sets and 4 SRS resources per set.

Proposal 9: For 1T8R, support N = 2, 3 of SRS resource sets.

- For N = 2 sets, support at least mapping of SRS resources of (4,4) across the two SRS resource sets

- For N = 3 sets, support at least mapping of SRS resources of (3,3,2) across the three SRS resource sets

- For N_max = 4 sets, support at least mapping of SRS resources of (2,2,2,2) across the four SRS resource sets

### SRS configuration for 2T6R

For 2T6R, three SRS resources are needed; each resource with two SRS ports. A single SRS resource set with three SRS resources may be sufficient and can fit into single slot. For UEs limited to last 6 SRS symbols within slot and operating at 120 KHz SCS, two SRS resource sets are need. The examples using 2 SRS resource sets, one set with two resources and other set with single resource is shown in  Figure 3-3. Similarly, the scenario with single set and three SRS resources is shown in Figure 3-4.

Figure 3-3:SRS configuration for 2T6R using two sets and 2 + 1 SRS resources among both sets.

Figure 3-4 SRS configuration for 2T6R using single set and 3 SRS resources.

Proposal 10: For 2T6R, support N = 1,2 of SRS resource sets.

- For N = 1, support mapping of 3 SRS resources per the SRS resource set.

- For N = 2 sets, support mapping of SRS resources of (2,1) across the two SRS resource sets.

- For N_max = 3 sets, support mapping of SRS resources of (1,1,1) across the three SRS resource sets.

### SRS configuration for 2T8R

For 2T8R, four SRS resources are needed; each resource with two SRS ports. Single SRS resource set with 4 SRS resources can be accommodated in single slot. SRS antenna switching configuration using two sets and one set are shown in Figure 3-5and Figure 3-6 respectively.

Figure 3-5:SRS configuration for 2T8R using two sets with 2 SRS resources per set

Figure 3-6: SRS configuration for 2T8R using single set with 3 SRS resources.

Proposal 11: For 2T8R, support N = 1, 2 of SRS resource sets

- For N = 1, support mapping of 4 SRS resources per the SRS resource set.

- For N = 2 sets, support mapping of SRS resources of (2,2) across the two SRS resource sets.

- For N_max = 4 sets, support mapping of SRS resources of (1,1,1,1) across the four SRS resource sets.

### SRS configuration for 4T8R

The configurations of 4T8R antenna switching requires two SRS resources with 4 SRS port per SRS resource. Single SRS resource set with two SRS resources can be used for the configuration of 4T8R antenna switching as

Figure 3-7:SRS configuration for 4T8R

Proposal 12: For 4T8R, support N = 1 of SRS resource set with two SRS resources per set.

- For N_max = 2 sets, support mapping of SRS resources of (1,1) across the two SRS resource sets

## P/SP SRS antenna switching configuration

The current spec allows flexible configuration of periodicity and slot offset for each SRS resource. That is why we believe there is no need to configure multiple SRS resource sets for P/SP antenna switching. A single SRS resource set is sufficient to accommodate multiple SRS resources where each resource can be configured independently.

Agreement For antenna switching, support one of the following - Alt 1: Support maximum one SRS resource set for periodic SRS and maximum one SRS resource set for semi-persistent SRS - Alt 2: Support up to two semi-persistent SRS resource sets in addition to a periodic SRS resource set - Note: the two SP-SRS resource sets are not activated at the same time. - FFS whether further enhancement for single-DCI or multi-DCI based MTRP is needed - FFS whether configurations on SRS repetitions have impact - FFS relevant UE capability design

Proposal 13: Support Alt 1 of single SRS resource set configuration for P/SP SRS antenna switching.

- Each set contains K resources where K = 8 for 1T8R, K = 6 for 1T6R, K = 4 for 2T8, K = 3 for 2T6R and K = 2 for 4T8R.

For UE with <= 4Rx antennas, Rel-15 specified that UE is configured with maximum of two SRS resource sets with different time domain behaviour: periodic, aperiodic, or semi-persistent. This means, in addition to aperiodic SRS set, the UE can be configured with either one additional periodic or semi-persistent set. Similar design methodology should be adopted for UE with > 4Rx antennas. The specification can be flexible to specify support of configuration of both semi-persistent and periodic sets. However, it should be subject to UE capability to support maximum of two or three time-domain behaviour: periodic, semi-persistent and aperiodic.

Proposal 14: To support configuration of one periodic and one semi-persistent set for SRS antenna switching of is subject to UE capability.

- UEs that don’t indicate the support this capability should be configured with either one periodic or one semi-persistent SRS resource set.

## Guard period between two SRS sets

Rel-15 specification introduced symbol(s) band between SRS resources within the SRS resource set. In addition, the UE shall not expect to be configured or triggered with more than one SRS resource set with higher layer parameter usage set as 'antennaSwitching' in the same slot.

Now, for the for the support of antenna switching with >4Rx, multiple SRS resource sets could be configured on consecutive slots or within the same slot if Rel-15 restriction of single set per slot is lifted. In our views, we should keep similar design mythology of Rel-15, where guard symbol is also needed between SRS resource sets across consecutive while keeping same restriction of one configured or triggered SRS resource set per slot.

Proposal 15: Support the minimum guard symbol(s) requirement as defined in Rel-15 between two consecutive SRS resources mapped to two SRS resource sets across multiple slots.

Proposal 16: For 1T6R, 1T8R, 2T6R, 2T8R and 4T4R, the UE shall not expect to be configured or triggered with more than one SRS resource set with higher layer parameter usage set as 'antennaSwitching' in the same slot.

## Extension of SRS resource sets to legacy xT4R/xT2R

It was proposed in the last meeting to consider extension of number of SRS resource sets for legacy xT2R and xT4R configurations for aperiodic, periodic, and semi-persistent SRS. For P/SP, we do not see any need to increase the number of SRS resource sets as it is already flexible to configure each SRS resource with different periodicity and slot offset. For aperiodic SRS antenna switching, we believe this should be discussed after the completion of RAN1 work on SRS antenna switching for 6Rx and 8Rx UE.

Proposal 17: RAN1 to discuss the extension of increasing number of SRS resources sets for aperiodic SRS antenna switching after the completion for RAN1 work on xTyR for 6/8Rx UE.

## UE reporting of power offset

For 6Rx and 8Rx UE, to improve DL CSI acquisition, the UE needs to sound all receive antenna ports. This comes at the cost of extra RF switching circuitry that introduces some insertion loss. For some UE implementation, the insertion loss may be large which causes big mismatch between the actual DL channel and the one estimated by the gNB from UL sounding. This results into different Tx power between antenna ports or equivalent power offset between antenna ports.

Considering the 4Rx UE architecture as a baseline and scaling the number of Rx antennas to 8Rx while still have single Tx chain, an extra RF switching circuitry is needed to route the Tx path to the extra Rx antennas. An example of such architecture is shown in the Figure 3-8 below. It is important to note that there is no extra insertion loss for the original 4Rx antenna while there will be X1 to X2 dB power offset for the added antennas.

Figure 3-8 Example of 8Rx UE RF architecture with single Tx chain

To overcome this mismatch, the UE can report to the network the power offset between the antenna ports which can help the network to compensate the UL/DL channel mismatch. We did a link-level evaluation for 8Rx UE with 1T8R SRS antenna switching using CDL-C 300ns, 3km/hr and 20MHz DL/UL BW. The results are shown in Figure X in which we compare three different schemes: legacy 4Rx with 1T4R, 8Rx UE with 1T4R and 8Rx with 1T8R for both scenarios of power offset compensation enabled and disabled at the gNB. The power offset across the extra four antenna ports is assumed to be [ 1 1 2 2] dB. For the case of 1T8R with gNB reporting of power offset, the throughput performance is close to the ideal scenario of no power offset across the antenna ports.

Figure 3-9: Power offset reporting and compensation

Observation 6: UE reporting of power offset can help the gNB to compensate of the power offset between the UL and DL channels and improve the DL throughput.

Proposal 18: Support UE capability reporting of power offset across antenna ports for SRS DL CSI acquisitions.

## Maximum number of DL MIMO layers for 6Rx and 8Rx

In current NR specification, the UE reports the maximum number of supported DL MIMO layers through the capability parameter ‘maxNumberMIMO-LayersPDSCH’ as part of downlink feature set per CC (FSPC) [38.306]. The values that are allowed for the UE to report is either two, four or eight layers as indicated by the ‘MIMO-LayersDL’ in 38.33. It is now allowed for 6Rx or 8Rx UE to report a value of sixLayers.

Observation 7: A 6Rx or 8Rx UE can not report a capability of six DL MMO layers as maximum number of DL MIMO layers.

A 6Rx UE should be able to report capability of two, four or six layers depending on the band as some NR bands (e.g n7, n38, n41, n77, n78, n79) requires a minimum of four receive antenna ports and mandatory four DL MIMO layers. Similarly, eight Rx UE should be able to report capability of two, four, six or eight layers.

Proposal 19: A 6Rx can report a capability of two, four or six layers of maximum number of DL MMO layers. And 8Rx UE can report a capability of two, four, six or eight layers of maximum number of DL MMO layer.

## 4T6R antenna switching

In most current NR deployment, the UE is equipped with 4Rx antenna and up to two transmit chains. In the near future, it is expected that 6Rx and 8Rx will be more common and possibly with up to four transmit chains. Therefore, it is important for 3GPP specification to be flexible and allow such configuration of antenna switching with 4T6R. Few companies raised some concerns on the RF switching implementation and the insertion loss. One possible implantation for the RF front end showing the switching between each the four transmit chains and corresponding antennas is shown in Figure 3-10. In such implementation, each two transmit chain can route to three antennas which gives flexibility to support different sounding configuration and similar insertion loss across the antennas.

Figure 3-10: RF front end switching for 4T6R

This architecture enables sounding 4 ports or two ports per each sounding opportunity which makes it flexible to support SRS resource with 2port or 4 ports. To sound the 6 ports, there are two alternatives. In the first alternative is shown in Table 1 where 4 ports are sounded in the first SRS resource while the other 2 SRS ports are sounded in the second resource.  In the second alternative which is shown in Table 2, 2 or 3 SRS resources with 4 SRS ports per resource is used. In the later scheme, some or all ports may be sounded twice.

Table 1: 4T6R antenna switching with Alt 1

| Alt 1 (4,2) | Antenna ports-SRS resource #1 | Antenna ports-SRS resource #2 |
| --- | --- | --- |
| Tx0 | AP0 | AP2 |
| Tx1 | AP1 |  |
| Tx2 | AP3 | AP5 |
| Tx3 | AP4 |  |

Table 2: 4T6R antenna switching with Alt 2

| Alt 2 (4,4,4) or (4,4) | Antenna ports-SRS resource #1 | Antenna ports-SRS resource #2 | Antenna ports-SRS resource #3 |
| --- | --- | --- | --- |
| Tx0 | AP0 | AP0 | AP1 |
| Tx1 | AP1 | AP2 | AP2 |
| Tx2 | AP3 | AP3 | AP4 |
| Tx3 | AP4 | AP5 | AP5 |

To compare the performance of each Alternative, we did a link-level study comparing the performance of each alternative using CDL-B 30ns channel and different mobility scenario.

Figure 3-11: 4T6R LLS evaluation with different sounding configuration

At low mobility, the two sounding alternatives have similar performance while at mid and high mobility, Alt 1 has some performance degradation compared to Alt 1.

Observation 8: For 4T6R antenna switching, configuring two or three SRS resources with 4 SRS ports per resource outperform two SRS resource configuration with 4 and 2 ports per resource, respectively.

A similar study on the system level is done comparing 4T6R SRS configuration against 1T6R and 2T6R at different mobility and using different sounding periodicity.

Observation 9: For the low mobility scenarios, when UEs with limited transmit power are assigned with lower SRS sounding periodicity, it is preferred to have low-dimensionality antenna switching (1T6R and 2T6R) to improve SRS coverage and also for UE power saving purposes.

Observation 10: For both of the low mobility and the high mobility scenarios, when UEs are assigned with longer SRS sounding periodicity, it is preferred to have high-dimensionality antenna switching (4T6R) to get accurate channel states and the overcome the fast aging of the channel.

Based on all evaluation analysis, we believe it is essential to support 4T6R antenna switching to overcome limitation of 1T6R and 2T6R limitation when NW configure long sounding channel periodicity regardless of UE mobility.

Proposal 20: Support 4T6R SRS antenna switching.

In last RAN1 meeting #104e, couple of concerns related to the support 4T6R were discussed by Ericsson at [3]. First, similar discussion on the flexibility and multiplexing exists when comparing 2T8R vs 4T8R. Following same logic, then RAN1 should deprioritize 4T8R given similar concerns on 4T6R. Additionally, when comparing 2T6R to 4T6R, it should not be blindly looked at from energy efficiency perspective while ignoring performance aspects. Finally, related to the PA sizing and UE architecture, a 17 dBm PA is assumed. It is surprising and rather counter intuitive that UE would downsize the PA when adding more Tx chains. It is well known that UL is link budget limited and it is always desired to increase Tx power when scaling up the number of Tx chains. Intuitively, UE with 4 Tx chains should be targeting at least 20+20+20+20 to achieve 26 dBm or even higher Tx power.

### SRS configuration for 4T6R

Based on the earlier discussion in section 0, two alternatives of SRS configurations are discussed where first configuration is based on two or three SRS resources with 4 SRS ports per resource, and second configuration is based on two SRS resource configuration with 4 and 2 ports per resource respectively. Based on our LLS, the first configuration has slight better performance. However, we are supportive for both configurations with preference of Alt 2 as it has same number of SRS ports across SRS resources.

Figure 3-12 shows the SRS configuration of single SRS resource set and three SRS resources within each SRS resource set, with 4 ports per SRS resource. This configuration allows each port to be sounded twice with two port cycling across the SRS resources.

Figure 3-12:SRS configuration for 4T6R

For more details on the configuration of 4T6R, one example with antenna cycling is shown in Figure 3-13 .

Figure 3-13:SRS configuration for 4T6R with two antennas cycling.

Proposal 21: For 4T6R SRS antenna switching, support single SRS resource set with two or three SRS resources, each with 4 ports per SRS resource.

Proposal 22: For 4T6R SRS antenna switching, support single SRS resource set with two SRS resource. The first resource with 4 SRS port and second resource with 2 SRS ports.

## Flexible reconfiguration of SRS resource set

To support SRS antenna switching with 8 receive antennas, the SRS resource set may be configured with multiple SRS resources. For example, 1T8R can be configured with one SRS resource set that contains 8 SRS resources each with one SRS port. In some scenarios, the network may not need the UE to sound all the 8 SRS resources and the RRC reconfiguration can be time consuming plus the signalling overhead. A flexible and faster method through MAC-CE has been discussed in section 2.1.1 which may be beneficial for the reconfiguration of SRS antenna switching. The MAC-CE can indicate which SRS resources within the set is activated or deactivated. This can be used a tool to enable UE fallback to lower SRS switching configuration, e.g. from 1T8R to 1T4R. In Rel-16, UE can report combo SRS switching capability and gNB can configure different switching capability per each BWP. So, the current mechanism to change or fallback of SRS antenna switching requires BWP switching. The proposed MAC-CE can adapt the SRS configuration without the need of BWP switching.

Proposal 23: To enable flexible reconfiguration of the number of active resources within SRS resource set, MAC-CE command can be used to activate/deactivate the individual resources with the aperiodic SRS set.

Figure 3-14: Antenna switching re-configuration via MAC-CE.

# SRS coverage and capacity enhancement

In the last RAN1 104bis-e meeting, more details on the design of the supported SRS coverage/capacity schemes were agreed. In this section, we address some details regarding the supported schemes.

## SRS increased repetition

Agreement For increased repetition in Rel-17, support the following N_symbol (number of OFDM symbols in one SRS resource) and R (repetition factor) values - N_symbol = 8, R = {1, 2, 4, 8} - N_symbol = 12, R = {1, 2, [3], 4, 6, 12} - FFS the following configurations - N_symbol = 10, R = {1, 2, 5, 10} - N_symbol = 14, R = {1, 2, 7, 14} - FFS options to reduce SRS BW for R>1

In the last meeting it was agreed to support increase the number of repetition symbols per SRS resource per slot to 8 and 12 symbols respectively. Within the SRS resource configuration, there are two parameters for resource mapping namely: Ns which represents the number of symbols per resource and R which represents the repetition factor. Depending on the values of R and Ns, frequency hopping within repetition can be either enabled or disabled. For the scenario where R is equal to Ns, then frequency hopping is disabled, otherwise it is enabled.

In our views, we believe it is beneficial to support all listed values of increased number symbols which means to expand the value of Ns to additionally support (12 and 14) to allow full flexibility in configuration and scheduling. Also, the agreement made in the last meeting should not be limited to just repetition scenario when (R=Ns). However, it should allow for extension of frequency hopping within the increased number of symbols. Following same principles, it is proposed to adopt the repetition factor for each Ns configuration shown in table below.

| Number of SRS symbols per SRS resource (Ns) | Repetition factor (R) |
| --- | --- |
| 8 | 1, 2, 4, 8 |
| 10 | 1, 2, 10 |
| 12 | 1, 2, 4, 6, 12 |
| 14 | 1, 2,14 |

Proposal 24: In addition to Ns = 8, 12, support the following configuration of number of symbols (Ns) and repetition factor (R) per SRS resource:

- Ns = 10, R ∈1, 2,[5],10

- Ns = 14, R ∈1, 2,7,14

## Partial frequency sounding

### Partial frequency (PF) value

Agreement - For RPFS in Rel-17, support PF = {2, 4}. - FFS 3, 8, 12, 16 or fractional numbers - Support at least one of the following alternatives (to be decided in RAN1#105-e) - Alt 1: 1PFmSRS, BSRS1PFmSRS, BSRS1PFmSRS,BSRS is an integer value - Alt 2: 1PFmSRS, BSRS1PFmSRS, BSRS1PFmSRS,BSRS is an integer value with minimum value 4 - Alt 3: 1PFmSRS, BSRS is a multiple of 4 - Alt 4: Round 1PFmSRS, BSRS1PFmSRS,BSRS to a multiple of 4 in case of Alt 1 or Alt 2

Based on the last meeting, PF values = {2, 4} was agreed. Regarding the number of RBs with PFS, several alternatives were listed for down selection. In our views, to enable SRS multiplying with legacy UEs which only support RB allocation as integer multiple of 4, the PFS RB allocation should be multiple of 4 as well. Also, keeping such configuration will not impose any change to UE behavior or implementation (e.g., MPR, sequence generation) as compared to legacy behavior.

Observation 11: For partial frequency sounding scheme, it is preferred to 1PFmSRS, BSRS1PFmSRS,BSRS a multiple of 4 RBs to enable multiplexing with Rel-15/16 UEs and ensure legacy UE behavior.

Among Alt 3 and Alt 4, Alt 3 is a bit restriction where the mSRS, BSRS should be multiple of 8 or 16 to ensure that the partial RBs is multiple of 4 assuming PF = 2 and 4 respectively.

Proposal 25: Support Alt 4: Round 1PFmSRS, BSRS1PFmSRS,BSRS to a multiple of 4 RBs

Another aspect is whether partial frequency is applicable only to frequency hopping scenario only or it can support the scenario of non-frequency hopping. In our views, we believe partial frequency sounding should be supported at least to frequency hopping scenario such that both coverage and capacity benefits are achieved.

Proposal 26: Support partial frequency sounding at least when frequency hopping is configured.

### Subhop location and pattern for partial frequency sounding

Agreement For RB-level partial frequency sounding (RPFS) in Rel-17 - The start RB index of the 1PFmSRS, BSRS RBs in the mSRS, BSRS RBs is Noffset=kFPFmSRS, BSRS, where kF = {0, …, PF-1} - FFS support start RB location (Noffset) hopping in different SRS occasions, symbols or frequency hopping periods, and if supported, detailed hopping pattern - Support to determine PF and Noffset at least via RRC configuration per SRS resource. - FFS whether to introduce DCI and/or MAC CE in addition

In partial frequency sounding, there is a need to indicate to the UE start RB index within the mSRS, BSRS . The simplest method is to evenly divide the mSRS, BSRS into PF chunks of contiguous RBs, let’s call it subhop. Then, the index of each sub-hop is Kf= 0, 1,.. PF – 1. Then, the RB start index within the hop is Noffset= mSRS,b×KfPF.  This is illustrated in the example in Figure 4-1 for intra-band frequency hopping with two SRS symbols (Ns = 2 and R=1) and PF = 4. The mSRS,b per hop is divided into 4 sub-hops and the example show the second sub-hop is selected (sub-hop index = 1). This allows the network to multiplex different UE ports on different subhop within the original hop which improves both SRS capacity and SRS coverage.

Figure 4-1: Subhop index (Kf)

To determine start RB index, it is needed to RRC configure the UE with the value of Pf and Kf (subHop index) on the SRS resource level. Also, RRC configuration should be sufficient and there is no obvious need to introduce L1 or L2 adaptation

Proposal 27: Introduce RRC parameters of Pf and Kf to enable partial frequency sounding and indicate to the UE which RBs for SRS sounding.

After determining the RB start of the first hop, the next question is how to determine the start RB of the sub-sequent hops. There could be two approaches. The first one is fixed hopping pattern, in which the same start RB is used for the same hop at different time instances. This explained in Figure 4-2. Although being a simple approach, it doesn’t enable sounding different sub-hops and extracts the frequency characteristic of the channel.

Figure 4-2: Fixed hopping pattern

The other approach is to enable hopping within the original hop frequency allocation of  mSRS,BSRS (i.e. sub-hop pattern). Different sub-hop pattern could be further discussed. The simplest one is to allow sub-hop cycling (0,1, 2,..PF-1) across the hops as show in Figure 4-3. Other sub-hop patterns could be considered as well.

Figure 4-3: Cycling sub-hop pattern.

Proposal 28: RAN1 to discuss the sub-hopping pattern across multiple hops whether adopting fixed hopping pattern or sub-hop cycling.

### SRS sequence generation

Agreement For RPFS SRS in Rel-17, adopt one of the following alternatives for sequence generation, where no new sequence length other than the ones supported in the current spec is introduced (to be decided in RAN1#105-e) - Alt 1: Generate length-12PFmSRS, BSRSComb12PFmSRS, BSRSComb12PFmSRS,BSRSComb ZC sequence - Alt 2: Truncate from legacy length-12⋅mSRS, BSRSComb sequence according to the location of RPFS SRS

Once the number of RBs is determined, the ZC sequence length should be determined following Rel-15 design methodology where ZC length is determined based on  mSRS, BSRSPF RBs. There was other alternative in which the SRS sequence is determined based on original non-PFS mSRS, BSRS RBs and then truncated based on location of RFSS SRS. In our views, there are two concerns on that approach. The first one is the increase of PAPR of the SRS signal which will impact the SRS coverage gain. As an example, Figure 4-4 shows PAPR-CDF plot comparing both alternative where up to 2.4 dB PAPR increase when Alt-2 is used. The other concern, the truncated sequence is a new SRS sequence which conflicts with the RAN1 agreement that no new SRS sequence is introduced in Rel-17.

Observation 12: Truncated SRS sequence will result into increase of PAPR which reduce PFS SRS coverage gain. In addition, it conflicts with RAN1 agreement stating that no new SRS sequence is introduced in Rel-17.

Proposal 29:  Support Alt1 where the length of the ZC SRS sequence is determined as -12PFmSRS, BSRSComb12PFmSRS, BSRSComb

Figure 4-4: PAPR CDF comparison between Alt-1 and Alt-2

## Comb-8

Agreement Study the maximum number of cyclic shifts for Comb-8 in Rel-17, with the following alternatives as starting points - Alt 1: The maximum number of CSs for Comb-8 is 6 - Alt 2: The maximum number of CSs for Comb-8 is 12, and introduce a rule to restrict applicable CSs when SRS sequence is shorter than the maximum number of CSs

In Rel-16, Comb-8 was introduced for SRS-positioning and had a maximum number of CS of 6. The discussion of maximum number of CSs for comb8 was brought up last meeting. Two alternatives were proposed, either keeping maximum number of CSs for comb-8 as 6 or increase it to 12 CSs.

From SRS capacity perspective, Comb-8 should enhance SRS multiplexing compared to Comb-4 which supports up to 48 ports. From SRS coverage perspective, comb-8 provides 3dB power boost compared to comb-4 which improve SRS coverages [2].  One issue with maximum number of CS of 6 is that it requires to change the equation for SRS CS when number of antenna ports = 4 as the maximum number of CS of 6 is not integer multiple 4.

nSRSCS,i=nSRSCS+ nSRSCS,maxpi-1000NapSRS mod nSRSCS,max

Observation 13: Setting the maximum number of CS for Comb-8 to 6 (nSRSCS,max=6) for Comb-8 has similar SRS capacity gain as Comb-4.

- It requires spec change to adopt proper per-port CS calculation when total number of antenna ports is 4.

On the other hand, increasing the maximum number of CSs to >6 will improve SRS capacity beyond the one for comb 4, however, this depends on the delay spread of the multiplexed antenna ports. For example, setting nSRSCS,max=12 for comb-8 will result into 50% capacity gain, however, maximum resolvable delay spread should be less than 347 ns. However, it may require some special handling of CSs when the length of the SRS is less than 12. For example, when 1PFmSRS, BSRS=4 for comb 8, the SRS sequence length is 6 which requires to restrict some of CSs (e.g. only even CSs are allowable).

Observation 14: Setting the maximum number of CS for Comb-8 to 12 (nSRSCS,max=12) for Comb-8 has theoretically 50% SRS capacity gain as Comb-4.

- It requires to restrict some CSs when the length sequence is less than 12.

We did a link-level evaluation using CDL-C channel with two different delay spreads (CDL-C 100ns, 300ns) and different values of maximum number of CS = 6, 8 and 12. The results are shown in Figure 4-4. Similar performance observed for nSRSCS,max=6, 8 while nSRSCS,max=12 has some performance degradation which gets larger with CDL-C 300ns as compared to 100ns.

Figure 4-5 Link level evaluation results for Comb-8 with different options of maximum CSs

Observation 15: Setting maximum number of CS to 8 is good compromise between nSRSCS,max=6, 12 to improve SRS capacity while delivering good performance.

Proposal 30:  Support nSRSCS,max>6 to improve SRS capacity gains

- it is up to gNB to proper configure the CSs (e.g., restrict some CSs configuration or use subset of CSs)

# Conclusion

In this contribution, we presented our views on the SRS enhancements techniques. Below is the summary of our conclusion and proposals:

Observation 1: A Rel-17 UE supporting enhanced A-SRS triggering and transmission based on available slot indication should support legacy triggering mechanism based on slot offset. Reference slot based on slot offset is an incremental addition on legacy scheme and does not impact UE complexity neither timeline.

Observation 2: Option 2 of reference slot can accommodate option 1 by NW implementation of slofOffset set to zero or not configured.

Observation 3: For multiple triggered SRS resource sets with overlapping symbol(s), there are two options to handle this scenario:

- Option 1: The available slot is determined independently for the triggered sets and a dropping rule is introduced when a collision happens.

- Option 2: The available slot is determined sequentially for the triggered SRS resource sets.

Observation 4: The group common DCI format 2_3 has some limitation where it triggers only SRS transmission for ‘antenna Switching’ use cases and is limited to UL carrier switching.

Observation 5: DL CSI through A-SRS antenna switching should be captured within short period of time (i.e., least number of slots) otherwise DL scheduling may be impacted due to CSI decorrelation.

Observation 6: UE reporting of power offset can help the gNB to compensate of the power offset between the UL and DL channels and improve the DL throughput.

Observation 7: A 6Rx or 8Rx UE can not report a capability of six DL MMO layers as maximum number of DL MIMO layers.

Observation 8: For 4T6R antenna switching, configuring two or three SRS resources with 4 SRS ports per resource outperform two SRS resource configuration with 4 and 2 ports per resource, respectively.

Observation 9: For the low mobility scenarios, when UEs with limited transmit power are assigned with lower SRS sounding periodicity, it is preferred to have low-dimensionality antenna switching (1T6R and 2T6R) to improve SRS coverage and also for UE power saving purposes.

Observation 10: For both of the low mobility and the high mobility scenarios, when UEs are assigned with longer SRS sounding periodicity, it is preferred to have high-dimensionality antenna switching (4T6R) to get accurate channel states and the overcome the fast aging of the channel.

Observation 11: For partial frequency sounding scheme, it is preferred to 1PFmSRS, BSRS1PFmSRS,BSRS a multiple of 4 RBs to enable multiplexing with Rel-15/16 UEs and ensure legacy UE behavior.

Observation12: Truncated SRS sequence will result into increase of PAPR which reduce PFS SRS coverage gain. In addition, it conflicts with RAN1 agreement stating that no new SRS sequence is introduced in Rel-17.

Observation 13: Setting the maximum number of CS for Comb-8 to 6 (nSRSCS,max=6) for Comb-8 has similar SRS capacity gain as Comb-4.

- It requires spec change to adopt proper per-port CS calculation when total number of antenna ports is 4.

Observation 14: Setting the maximum number of CS for Comb-8 to 12 (nSRSCS,max=12) for Comb-8 has theoretically 50% SRS capacity gain as Comb-4.

- It requires to restrict some CSs when the length sequence is less than 12.

Observation 15: Setting maximum number of CS to 8 is good compromise between nSRSCS,max=6, 12 to improve SRS capacity while delivering good performance.

Proposal 1: Support option 2 where reference slot is indicated by the legacy triggering offset.

Proposal 2: Support Option 1 to introduce dropping rule when collision happen between A-SRS resource sets.

Proposal 3: Confirm the working assumption with the following updates:

For DCI indication of “t” in Rel-17 SRS triggering offset enhancement

- For both DCI that schedules a PDSCH/PUSCH and DCI 0_1/0_2 without data and without CSI request

  - t is indicated by adding a new configurable DCI field (up to 2 bits)

    - Applies only when there are multiple candidate values of t configured

  - No further enhancement to indicate “t” for DCI 0_1/0_2 without data and without CSI request at least when the new DCI field is configured

Proposal 4: Support the introduction of new MAC-CE to reconfigure the available slot and legacy slot offset of SRS resource set.

- In addition, the MAC-CE can include a bitmap that can indicate the activate or inactivate SRS resources within the set.

Proposal 5: Support the following functionalities for DCI 0_1 and 0_2 triggering aperiodic SRS without data and without CSI:

- CAT-B: Enable A-SRS on a single CC or a group of CCs

- CAT-C: Indicate the TPC command for each CC.

Proposal 6: Support single scheduling DCI to trigger simultaneous A-SRS transmission across multiple component carriers.

- RAN1 further discussion on the specific configuration of the RRC configuration and triggering mechanism.

Proposal 7: Support the enhancement of DCI formant 2_3 to enable multiple UE SRS transmission across multiple serving cells per UE.

Proposal 8: For 1T6R, support N = 1, 2 SRS resource sets.

- For N = 1, support mapping of 6 SRS resources per the SRS resource set.

- For N = 2 sets, support mapping of SRS resources of (3,3) across the two SRS resource sets.

- For N_max = 3 sets, support mapping of SRS resources of (2,2,2) across the two SRS resource sets.

Proposal 9: For 1T8R, support N = 2, 3 of SRS resource sets.

- For N = 2 sets, support at least mapping of SRS resources of (4,4) across the two SRS resource sets

- For N = 3 sets, support at least mapping of SRS resources of (3,3,2) across the three SRS resource sets

- For N_max = 4 sets, support at least mapping of SRS resources of (2,2,2,2) across the four SRS resource sets

Proposal 10: For 2T6R, support N = 1,2 of SRS resource sets.

- For N = 1, support mapping of 3 SRS resources per the SRS resource set.

- For N = 2 sets, support mapping of SRS resources of (2,1) across the two SRS resource sets.

- For N_max = 3 sets, support mapping of SRS resources of (1,1,1) across the three SRS resource sets.

Proposal 11: For 2T8R, support N = 1, 2 of SRS resource sets

- For N = 1, support mapping of 4 SRS resources per the SRS resource set.

- For N = 2 sets, support mapping of SRS resources of (2,2) across the two SRS resource sets.

- For N_max = 4 sets, support mapping of SRS resources of (1,1,1,1) across the four SRS resource sets.

Proposal 12: For 4T8R, support N = 1 of SRS resource set with two SRS resources per set.

- For N_max = 2 sets, support mapping of SRS resources of (1,1) across the two SRS resource sets

Proposal 13: Support Alt 1 of single SRS resource set configuration for P/SP SRS antenna switching.

- Each set contains K resources where K = 8 for 1T8R, K = 6 for 1T6R, K = 4 for 2T8, K = 3 for 2T6R and K = 2 for 4T8R.

Proposal 14: To support configuration of one periodic and one semi-persistent set for SRS antenna switching of is subject to UE capability.

- UEs that don’t indicate the support this capability should be configured with either one periodic or one semi-persistent SRS resource set.

Proposal 15: Support the minimum guard symbol(s) requirement as defined in Rel-15 between two consecutive SRS resources mapped to two SRS resource sets across multiple slots.

Proposal 16: For 1T6R, 1T8R, 2T6R, 2T8R and 4T4R, the UE shall not expect to be configured or triggered with more than one SRS resource set with higher layer parameter usage set as 'antennaSwitching' in the same slot.

Proposal 17: RAN1 to discuss the extension of increasing number of SRS resources sets for aperiodic SRS antenna switching after the completion for RAN1 work on xTyR for 6/8Rx UE.

Proposal 18: Support UE capability reporting of power offset across antenna ports for SRS DL CSI acquisitions.

Proposal 19: A 6Rx can report a capability of two, four or six layers of maximum number of DL MMO layers. And 8Rx UE can report a capability of two, four, six or eight layers of maximum number of DL MMO layer.

Proposal 20: Support 4T6R SRS antenna switching.

Proposal 21: For 4T6R SRS antenna switching, support single SRS resource set with two or three SRS resources, each with 4 ports per SRS resource.

Proposal 22: For 4T6R SRS antenna switching, support single SRS resource set with two SRS resource. The first resource with 4 SRS port and second resource with 2 SRS ports.

Proposal 23: To enable flexible reconfiguration of the number of active resources within SRS resource set, MAC-CE command can be used to activate/deactivate the individual resources with the aperiodic SRS set

Proposal 24: In addition to Ns = 8, 12, support the following configuration of number of symbols (Ns) and repetition factor (R) per SRS resource:

- Ns = 10, R ∈1, 2,[5],10

- Ns = 14, R ∈1, 2,7,14

Proposal 25: Support Alt 4: Round 1PFmSRS, BSRS1PFmSRS,BSRS to a multiple of 4 RBs

Proposal 26: Support partial frequency sounding at least when frequency hopping is configured.

Proposal 27: Introduce RRC parameters of Pf and Kf to enable partial frequency sounding and indicate to the UE the RB allocation for SRS sounding.

Proposal 28: RAN1 to discuss the sub-hopping pattern across multiple hops whether adopting fixed hopping pattern or sub-hop cycling.

Proposal 29:  Support Alt1 where the length of the ZC SRS sequence is determined as -12PFmSRS, BSRSComb12PFmSRS, BSRSComb

Proposal 30:  Support nSRSCS,max>6 to improve SRS capacity gains

- it is up to gNB to proper configure the CSs (e.g., restrict some CSs configuration or use subset of CSs)

# Reference

- RP-193133, “WID proposal FeMIMO”, Samsung.

- R1-2006795, Enhancements on SRS flexibility, coverage and capacity, Qualcomm

- R1-2101519, SRS Performance and Potential Enhancements, Ericsson

# Appendix

## 4T6R SLS evaluation

### Low mobility

Figure 7-1 and Figure 7-2Error! Reference source not found. show the SU-MIMO and MU-MIMO DL throughput performance comparison of 1T6R, 2T6R, and 4T6R with low mobility (Indoor UE v =3 km/h. outdoor UE v=30 km/h).  The system level setup and simulation assumptions can be found at Table 3 in the appendix. The SRS target SNR is applied to reflect the UL power control. For lower sounding periodicity, i.e., 2ms, 1T6R shows the best performance due to per-port power efficiency as compared to other antenna switching mechanism.

Figure 7-1: DL throughput performance of 6 Rx UE for indoor(outdoor) 3km/h (30km/h), 2ms sounding periodicity scenarios; SU and MU MIMO system performance.

Figure 7-2: DL throughput performance of 6 Rx UE for indoor(outdoor) 3km/h (30km/h), 32ms sounding periodicity scenarios; SU and MU MIMO system performance.

Figure 7-2 shows the SU-MIMO and MU-MIMO DL throughput performance with higher sounding periodicity, i.e., 32ms, As expected, 4T6R shows the best performance due to the faster channel updating as compared to other antenna switching mechanisms. For longer sounding periodicity, 4T6R achieves more instant uplink channel estimations to derive the DL precoder because the CSI for more number of ports can be simultaneously updated in each SRS occurrence.

### High mobility

Figure 7-3  and Figure 7-4  show the SU-MIMO and MU-MIMO DL throughput performance comparison of 1T6R, 2T6R and 4T6R with higher mobility (Outdoor UE v =30 km/h). The SRS target SNR is also applied to reflect the UL power control.  For lower sounding periodicity, i.e., 2ms, 1T6R still shows the best performance due to per-port power efficiency as compared to other antenna switching mechanism.

Figure 7-3: DL throughput  performance of 6 Rx UE  with speed 30km/h, 2ms sounding periodicity scenarios: SU and MU MIMO system performance.

Figure 7-4 shows the SU-MIMO and MU-MIMO DL throughput performance with higher sounding periodicity, i.e., 32ms, As expected, high dimensionality antenna switching of 4T6R achieves best performance as the gNB can obtain the accurate channel state information more frequently as compared to 1T6R and 2T6R which suffers from channel decorrelation. Furthermore, in comparison with low mobility cases, more significant gains can be achieved by 4T6R, especially for the MU-MIMO scenario. The intuition behind this observation is that faster channel updating in each SRS occurrence also helps the network to obtain the instant UL channel estimations for each UE and implement a better UE scheduling.

Figure 7-4: DL throughput performance of 6 Rx UE  with speed 30km/h, 32ms sounding periodicity scenarios: SU and MU MIMO system performance.

Similarly, in Figure 7-5 and Figure 7-6Error! Reference source not found., we show the DL throughput performance for 100% outdoor UE with speed of 120km/h for both of the SU-MIMO and MU-MIMO scenarios. The results are similar with the ones of UE with speed of 30 km/h . It should be noted that there should be a tradeoff between the power boosting benefit and the channel updating frequency benefit.  For lower sounding periodicity and power-limited UEs, power boosting results in less channel estimation error and dominates the quality of uplink channel estimations On the other hand, for higher sounding periodicity, antenna switching with higher dimensionality, achieves instant channel updating for more number of antenna ports within each available SRS symbol, which actually results in better uplink channel estimation to implement UE scheduling and to derive DL precoders.

Figure 7-5: DL throughput performance of 6 Rx UE  with speed 120km/h, 2ms sounding periodicity scenarios: SU and MU MIMO system performance.

Figure 7-6: DL throughput performance of 6 Rx UE  with speed 120km/h, 32ms sounding periodicity scenarios: SU and MU MIMO system performance.

## SLS setup and simulation assumption for antenna switching

Table 3: SRS Antenna Switching SLS Assumptions

| Parameters | UMi ISD 200m |
| --- | --- |
| Carrier Frequency | 3.5 GHz |
| Simulation BW | 20 MHz |
| Tx Power | 44dBm/20MHz |
| DownTilt | 14 deg |
| Horizontal antenna element spacing (dH) @ Both gNB, UE | 0.5λ |
| Vertical antenna element spacing | 0.8λ |
| gNB antenna height | 25m |
| gNB Ant | 64 RX/TX (+/- 45deg) (M, N, P) = (4,8,2) |
| UE Ant | 4 Rx: (M, N, P) = (1, 2, 2), 6Rx: (M, N, P) = (1,3,2), 8 Rx: (M, N, P) = (2, 2, 2); 0° / 90° x-pol |
| UE Max Tx Power SRS | 23 dBm / simulation BW of 20 MHz |
| UE distribution | 20% UEs are outdoor and 80% UEs are indoor/100 % outdoor |
| UE speed | indoor (3km/h) and outdoor (30km/h)/30 km/h for 100 % outdoor UEs/ 120 km/h for 100% outdoor UEs |
| UEs/cell | 10 |
| Traffic Models | Full-buffer |
| SRS Target SNR | -10 dB/-20 dB |
| DL precoder derivation | Type II Beamformed CSIRS with eigen beams |