3GPP TSG RAN WG1 #106-e						R1-2107434

e-Meeting, August 16th – 27th, 2021

Agenda Item:	7.2.5

Source: 	LG Electronics

Title: 	PHR issues related to URLLC/IIOT WI

Document for:	Discussion and decision

Introduction

In this contribution, we provide our view on the remaining issue on power headroom reporting for NR URLLC. It is re-submission of R1-2105417.

PHR handling with multiple configured grant

  - Discussion

In the previous meetings, It has been discussed what the current UE behavior is. Since configured grant can be skipped at UE side as default, it seems ambiguous how UE calculates PHR value when related CG PUSCH is cancelled. Based on RAN2#103bis Agreement and related RAN1 CR on TS 38.213 (R1-1903743), it is clear that PHR for CG PUSCH can be calculated as actual transmission. Specifically, once a CG PUSCH is configured to transmit before PHR calculation timing specified in TS 38.213, PHR for the CG PUSCH is based on configured CG PUSCH even if CG PUSCH is not actually transmitted (skipped).

RAN2#103bis Agreements: 1. At the time of determination of PH value for a serving cell, the UE MAC assumes real transmissions for all cells with grant even if any grant is skipped.

Current specification only consider non-overlapped PUSCH resource since there is no overlapped UL grant in Rel-15. In Rel-16, UE can be configured with multiple CG configurations. When multiple CG configurations are configured, they can be overlapped each other to support different services. Based on the above observation, when CG PUSCH is configured to transmit, UE would take the CG PUSCH transmission into account for PHR calculation regardless of whether it is actually transmitted or not. However, it is not specified yet how UE choose one PUSCH resource for PHR calculation among overlapped CG PUSCH having same starting symbol.

The ambiguity become obvious when PUSCH carries multiple PHR values from different cells. If CG PUSCH carries PHR itself, there could be no ambiguity. However, if a PUSCH in a cell carries PHR for other cell, and if there are overlapped multiple CG PUSCHs with same starting symbol, it is necessary to specify which PUSCH would be involved in PHR calculation.

In our perspective, to remove this ambiguity as simple as possible, we would like to specify simple rule to choose one CG among overlapped CGs for PHR calculation. For example, UE can choose higher priority CG and/or lowest configuration index if there are two or more higher priority CG PUSCHs. Alternatively UE may chooses CG has largest value of PUSCH power offset (P0-PUSCH-AlphaSet) if multiple CGs with the same priority have different open-loop power control parameter. By doing so, it is possible to avoid the saturation of PHR values.

Considering this late stage, we propose to use configuration index as the simplest rule. By using CG configuration index, gNB can control UE behavior fully and expect which PUSCH is used for PHR calculation.

Proposal 1: Adopt following TP to remove ambiguity when multiple CG PUSCH are configured and overlapped.

----------------------------------------------------- Start of text proposal ----------------------------------------------------- ### 7.7.1 Type 1 PH report If a UE determines that a Type 1 power headroom report for an activated serving cell is based on an actual PUSCH transmission then, for PUSCH transmission occasion on active UL BWP of carrier of serving cell , the UE computes the Type 1 power headroom report as [dB] where , , , , , and are defined in Clause 7.1.1. If a UE is configured with multiple cells for PUSCH transmissions, where a SCS configuration on active UL BWP of carrier of serving cell is smaller than a SCS configuration on active UL BWP of carrier of serving cell , and if the UE provides a Type 1 power headroom report in a PUSCH transmission in a slot on active UL BWP that overlaps with multiple slots on active UL BWP , the UE provides a Type 1 power headroom report for the earliest PUSCH, if any, on the first slot of the multiple slots on active UL BWP that fully overlaps with the slot on active UL BWP . If there are multiple earliest PUSCH using configured grant on the slot, the UE provided a Type 1 power headroom report for the PUSCH with higher priority index or with lowest Configuredgrantconfig-index value if priorities are same. If a UE is configured with multiple cells for PUSCH transmissions, where a same SCS configuration on active UL BWP of carrier of serving cell and active UL BWP of carrier of serving cell , and if the UE provides a Type 1 power headroom report in a PUSCH transmission in a slot on active UL BWP , the UE provides a Type 1 power headroom report for the earliest PUSCH, if any, on the slot on active UL BWP that overlaps with the slot on active UL BWP . If there are multiple earliest PUSCH using configured grant on the slot, the UE provided a Type 1 power headroom report for the PUSCH with higher priority index or with lowest Configuredgrantconfig-index value if priorities are same. ----------------------------------------------------- End of text proposal -----------------------------------------------------

Reason for changes
According to current specification, it is unclear how UE selects PUSCH for PHR calculation when multiple PUSCH is configured with same starting symbol. It is valid scenario in order to support various service with multiple configured grant configuration.

Summary of changes
For UE configured with multiple cells for PUSCH transmission, with same or different SCS, 
Add a sentence for each case of PHR for PUSCH transmission in multiple cells, with same or different SCS for UE to select PUSCH having lowest configuredgrantindex if there are multiple earliest PUSCH using configured grant.

Specs/Sections impacted
TS 38.213 Clause 7.7.1

Consequences if not approved
It remains unclear which PUSCH is used for the PHR calculation if multiple configured grant are overlapped with same starting symbol.

Conclusion

In this contribution, we discuss possible way to enhance UL transmission with configured grant for URLLC. . Our proposals are as follows:

Proposal 1: Adopt following TP to remove ambiguity when multiple CG PUSCH are configured and overlapped.