3GPP TSG RAN1 WG1 Meeting #106-e 	R1-2107475

e-Meeting, Aug 16 – Aug 27, 2021

Source:	ETRI

Title:	Discussion on HARQ Enhancements for NTN

Agenda Item:	8.4.3 Enhancements on HARQ

Document for:	Discussion/Decision

1	Introduction

The following agreements were made in RAN1#105-e [1].

Agreement: For enhancement on the HARQ process indication, extend the HARQ process ID field up to 5 bits for DCI 0-2/1-2 Agreement: For Type-2 HARQ codebook in NTN, - For the DCI of PDSCH with feedback-enabled HARQ processes, the C-DAI and T-DAI are the count of only feedback-enabled processes - FFS: Whether DCI for SPS release and any other DCIs are included in counting of C-DAI and T-DAI Agreement: Confirm the previous working assumption for X = T_proc,1 where X is defined from the end of the reception of the last PDSCH or slot-aggregated PDSCH for a given HARQ process with disabled feedback to the start of the PDCCH carrying the DCI scheduling another PDSCH or set of slot-aggregated PDSCH for the given HARQ process. Agreement: For enhancement on the HARQ process indication at least for DCI 0-1/1-1, the Option-1 and Option-1a are lower priority for further discussion. Agreement: Discussion of enhancement(s) on the aggregated transmission (including repetition) is prioritized to improve the performance in NTN.

In this contribution, the enhancement issues on the transmission would be discussed and the proposals would be justified by the simulation results.

2	Discussion on transmission enhancement

In addition to the above, 5 methods for enhancing the transmission performance with disabled HARQ feedback are highlighted in the section 6 of [2], Among those potential solutions, the details focusing on the aggregated transmission (larger aggregation factor/indication of aggregation factor) and the optimal aggregation factor guidance via UL feedback (including UCI/MAC-CE/RRC) for adaptive aggregated transmission would be discussed.

2.1	Background

2.1.1	Operating Range

In [3], the system level simulations are calibrated for 30 study cases.

Regarding the DL SINR, the minimum SINR (-5.9 dB) would be observed at SC19 (GEO satellite, handheld, satellite set 2, frequency reuse option 1), and the maximum SINR (9.2 dB) would be observed at SC3 (GEO, VSAT, satellite set 1, reuse option 3). In a word, the DL geometry SINR for NTN would range from -5.9 dB to 9.2 dB. Additionally, regarding the DL SINR difference between 5% SINR and 95% SINR, the minimum SINR difference (1 dB), would correspond to both SC8 (LEO-600, VSAT, satellite set 1, reuse option 3) and SC13 (LEO-1200, VSAT, satellite set 1, reuse option 3). The maximum SINR difference (8.4 dB) would correspond to SC28 (LEO-1200, VSAT, satellite set 2, reuse option 3). In a word, the DL SINR difference for NTN would be range from 1 dB to 8.4 dB.

Observation 1 : For NTN, DL geometry SINR might range from -5.9 dB to 9.2 dB

- For SC19 (worst case), DL geometry SINR might range from -5.9 dB (5%) to -2.1 dB (95%).

- For SC3 (best case), DL geometry SINR might range from 6 dB (5%) to 9.2 dB (95%).

Observation 2 : For NTN, DL geometry SINR difference between 5% and 95% might range from 1 dB to 8.4 dB

- For SC{8,13} (narrowest), DL geometry SINR might range from 7.5 dB (5%) to 8.5 dB (95%). (1dB=8.5-7.5)

- For SC28 (widest), DL geometry SINR might range from -1.4 dB (5%) to 7 dB (95%). (8.4dB=7+1.4)

Regarding the UL SINR, the minimum SINR (-13.9 dB) would be observed at SC19 (GEO, handheld, satellite set 2, reuse option 1), and the maximum SINR (14.8 dB) would be observed at SC23 (LEO-600, VSAT, satellite 2, reuse option 3). In a word, the UL geometry SINR for NTN would range from -13.9 dB to 14.8 dB. Additionally, regarding the UL SINR difference between 5% SINR and 95% SINR, the minimum SINR difference (3.4 dB) would correspond to SC5 (GEO, handheld, satellite set 1, resuse option 2). The maximum difference (13.4 dB) would correspond to SC26 (LEO-1200, VSAT, satellite set 2, reuse option 1). In a word, the UL SINR difference for NTN would be range from 3.4 dB to 13.4 dB.

Observation 3 : For NTN, UL geometry SINR might range from -13.9 dB to 14.8 dB.

- For SC19 (worst case), the UL geometry SINR might range from -13.9 dB (5%) to -9.3 dB (95%).

- For SC23 (best case), the UL geometry SINR might range from 3 dB (5%) to 14.8 dB (95%).

Observation 4 : For NTN, UL geometry SINR difference might range from 3.4 dB to 13.4 dB.

- For SC5 (narrowest), the UL geometry SINR might range from 1.5 dB (5%) to 4.9 dB (95%). (3.4dB=4.9-1.5)

- For SC26 (widest), the UL geometry SINR might range from -8.5 dB (5%) to 4.9 dB (95%). (13.4dB=4.9+8.5)

In summary, in order to cover all the NTN study cases, the performance should be checked with the operating ranges from -5.9 dB to 9.2 dB for DL, and from -13.9 dB to 14.8 dB for UL, respectively. Additionally, if possible, the potential solution needs to be applicable and effective across all the SINR ranges (or, at least, across widest SINR difference, which are 8.4 dB for DL, 13.4 dB for UL, respectively).

2.1.2 Legacy NR's Aggregated Transmission

In NR, HARQ retransmission might mitigate the performance degradation caused by low S(I)NR. On the other hand, in NTN (especially GEO), HARQ retransmission might be not possible because HARQ feedback would be likely to be disabled in NTN and it would determine whether HARQ retransmission is performed or new transmission is performed.

Accordingly, the important aspect for considering transmission in NTN would be an ability to compensate performance degradation without HARQ retransmission in low S(I)NR. Fortunately, legacy NR has another retransmission scheme as known as “slot aggregation”. The slot aggregation could be considered as retransmissions without feedback. The “retransmission” aspect of the slot aggregation could mitigate the impact owing to lower S(I)NR in NTN and the “without feedback” aspect might contribute to compensate longer delay in NTN.

From Figure 1 to Figure 4, the LLS performances, which are BLER and spectral efficiency, are evaluated by changing aggregation factor (AF) according to the legacy NR specification. For a certain AF value, the 6 IMCS are chosen, as described in Table 3, of which values are minimum and maximum per each modulation order.

For BLER, as a rule of thumb, target BLER is assumed to be 10-2 (i.e., target BLER=1%). If slot aggregation is not used (AF=1), as described in Figure 1, nothing could achieve target BLER at worst DL SINR. However, if slot aggregation is used (AF>1), the BLER would be enhanced as AF increases. Accordingly, it could be observed that even higher IMCS than 0 would achieve target BLER as described in Figure 2 (AF=2), Figure 3 (AF=4), and Figure 4 (AF=8).

For spectral efficiency (SE), regarding a certain IMCS, it could be observed that the maximum value of SE curves are limited into lower values as AF increases. However, simultaneously, it could be also observed that SE curves are shifted into low S(I)NR. Consequently, as AF increases, SE within high S(I)NR region would be degraded, but SE within low S(I)NR range  could be enhanced. Lastly, slot aggregation could contribute to lowering the latency because all the (re)transmissions could be issued in a bundle, which have shorter duration than HARQ (re)transmission. Consequently, the optimal AF value for both guaranteeing target BLER=1% and maximizing SE might be varying. for example, AF=2 might be optimal around 1 dB, but AF=8 might be optimal around -6 dB. Thus, for maximizing SE, the change of aggregation factor might be needed.

Figure 1 PDSCH LLS Evaluation (NTN-TDL-B, AF=1) (L: BLER, R: Spectral Efficiency)

Figure 2 PDSCH LLS Evaluation (NTN-TDL-B, AF=2) (L: BLER, R: Spectral Efficiency)

Figure 3 PDSCH LLS Evaluation (NTN-TDL-B, AF=4) (L: BLER, R: Spectral Efficiency)

Figure 4 PDSCH LLS Evaluation (NTN-TDL-B, AF=8) (L: BLER, R: Spectral Efficiency)

In conclusion, the slot aggregation could be beneficial for obtaining not only enhanced BLER but also improved SE in low S(I)NR region. Moreover, it could reduce latency. Based on these, in NTN, it might be inevitable to be used for guaranteeing BLER and SE simultaneously with lowering latency. Moreover, the slot aggregation could be used without specification impact.

Observation 5 : The slot aggregation (aggregation factor>1) could enhance BLER and SE simultaneously within low S(I)NR ranges.

Observation 6 : The slot aggregation (aggregation factor> 1) might be inevitable for achieving target BLER.

Observation 7 : The change of aggregation factor might be needed for achieving optimal SE performance.

2.2 Enhancement on the aggregated transmission

2.2.1 Enhancement via Larger Aggregation Factor (Value of Aggregation Factor)

Typically, the legacy NR “with HARQ retransmission” might assume that the target BLER might be around 10-1 (target BLER=10-1). However, under NTN, HARQ retransmission may not be possible. In order to maintain the similar reliability under NTN without HARQ retransmissions, target BLER might be set to the lower value achievable by HARQ retransmission during Radio Link Control (RLC) Round Trip Time (RTT) with NR under legacy NR. For 15 kHz subcarrier spacing, average 3.125 HARQ retransmissions would be possible during RLC RTT=50 ms if 16 HARQ processes are used. Thus, aggregation factor (AF)=2 could be chosen for target BLER under NTN because AF=3 is impossible. With IMCS=0, BLER of “AF=2” is roughly 2*10−2 at -3.5 dB (the required SNR for BLER=10−1 of “AF=1”). Thus, the target BLER under NTN might be assumed to be 10−2 (target BLER=1%) roughly.

The BLER of NTN transmission using normal spectral efficiency (SE) MCS table are illustrated in Figure 5 (PDSCH with MCS index table 1) and Figure 6 (PUSCH with transform precoding MCS index table). If target BLER is assumed to be 1%, 8 aggregated transmission (AF=8) might be enough for PDSCH, as described in Figure 5. However, as depicted in Figure 6, 8 aggregated transmission (AF=8) would be not enough for PUSCH. Additionally, if more challenging target BLER is required (i.e., target BLER=0.1%, 0.01%, …), 8 aggregated transmission (AF=8) might be insufficient for all the cases except PDSCH NTN-TDL-D.

Figure 5 Normal SE MCS PDSCH BLER (IMCS=0 (R=0.12), AF={1,2,4,8}) (L: NTN-TDL-B, R: NTN-TDL-D)

Figure 6 Normal SE MCS PUSCH BLER (IMCS=0 (R=0.12), AF={1,2,4,8}) (L: NTN-TDL-B, R: NTN-TDL-D)

Actually, for achieving aggressive target BLER, there is another way in legacy NR, which uses low SE index table, instead of normal SE MCS index table. In other words, low SE MCS index table might be applied to PDSCH and PUSCH for getting better BLER and the corresponding BLER performances are described in Figure 7 (PDSCH with MCS index table 3) and Figure 8 (PUSCH with transform precoding MCS index table 2). Compared to normal SE MCS index table, as illustrated in Figure 8, “AF=8” is still insufficient for PUSCH NTN-TDL-B with target BLER=1% assumption. Unfortunately, the only difference is that 8 aggregated transmission (AF=8) is now enough for PDSCH NTN-TDL-B and PUSCH NTN-TDL-D. If target BLER<0.1%, 8 aggregated transmission (AF=8) is not enough for PUSCH as presented in Figure 8.

Figure 7 Low SE MCS PDSCH BLER (IMCS=0 (R=0.03), AF={1,2,4,8}) (L: NTN-TDL-B, R: NTN-TDL-D)

Figure 8 Low SE MCS PUSCH BLER (IMCS=0 (R=0.03), AF={1,2,4,8}) (L: NTN-TDL-B, R: NTN-TDL-D)

In summary, to cover all the NTN study cases, larger AF might be inevitable for PUSCH, even if low SE MCS index table is applied instead of normal SE MCS index table. In the case of PDSCH, if more aggressive target BLER is required (i.e., target BLER=0.1%, 0.01%, …), either (or both) larger AF or/and low SE MCS index table might be inevitable. Additionally, larger AF has advantage over low SE MCS index since it could be applied simply by introducing new AF value and it could be fortified in easy way. i.e, as depicted in Figure 8, if more challenging target BLER is required (i.e., target BLER=0.1%, 0.01%, …), low SE MCS index table in legacy NR is not enough. Thus, for achieving target BLER, in case of low SE MCS index table, a new low SE MCS index table (or some modification in legacy low SE MCS index table) would be needed. Meanwhile, larger AF would need just new AF value. Consequently, larger AF might be preferable to low SE MCS index table because it has less specification impact.

Observation 8 :BLER improvement by legacy low SE MCS index table is not enough to cover all NTN study cases

- for PUSCH : low SE MCS index table in legacy NR might be insufficient

- for PDSCH :

  - low SE MCS index table in legacy NR might be enough only if target BLER ≥  0.1%

  - otherwise, low SE MCS index table in legacy NR might be insufficient

Observation 9 :larger aggregation factor might be inevitable for NTN.

- for PUSCH : 8 aggregated transmission might be insufficient even if low SE MCS index is applied.

- for PDSCH :

  - if target BLER ≥  0.1%, 8 aggregated transmission might be enough.

  - otherwise, 8 aggregated transmission might be insufficient

Observation 10 : larger aggregation factor method has less specification impact and could be more effective than low SE MCS index table method.

Proposal 1 : Consider the enhancement via “larger aggregation factor” as the one of the NTN’s transmission enhancement solutions to achieve target BLER performance.

- for PUSCH : 8 aggregated transmission might be insufficient even if low SE MCS table is applied.

- for PDSCH :

  - 8 aggregated transmission might be enough only if both condition#1 and condition#2 are met.

    - condition#1 : target BLER ≥  0.1%

    - condition#2 : low SE MCS table is applied,

  - otherwise, 8 aggregated transmission might be insufficient

2.2.2 Enhancement via Different Aggregation Factors (Indication of Aggregation Factor)

Generally, assuming that S(I)NR was determined to a certain value, the performances such as SE/throughput and BLER depend on (almost only) the value of the transmission parameter. Moreover, assuming that the AF was also determined to a specific value, then, it might not be optimal for all the transmission parameters because it would be effective for the certain transmission parameters only. For example, Assuming that the situation with SNR=-4 dB and AF=4 is given, the SE performance with IMCS=9 is optimal as shown in Figure 3. In other words, AF=4 would not be optimal for the other IMCS far from IMCS=9. Consequently, if the aggregation factor is determined, available parameters might be limited. In other words, it might be concluded that each parameter has its own optimal AF value, and different AF per each parameter might be needed for achieving optimal performance.

Additionally, in NR, various kinds of transport channels are multiplexed into the PDSCH/PUSCH. The transmission purpose might be different from each transport channel. Consequently, depending on the purpose, the different target performance might be needed. Normally, in NR, the purpose can be distinguished through RNTI (or search space type). For example, the following RNTIs such as {P,SI,RA,MSGB,TC,C,MCS-C,CS}-RNTI are used in PDSCH [4]. On the other hand, the following RNTIs such as {TC,C,MCS-C,CS}-RNTI are used in PUSCH [4]. Additionally, for NTN, the required performance might be different from the HARQ feedback availability (enabled/disabled).

Consequently, for optimal adaptation, different AF should be applied depending on the target performance, which are distinguished by RNTI (or search space)/HARQ feedback availability. Specifically, separate AF might be introduced and defined per IMCS/RNTI/HARQ feedback availability/HARQ process number. Regarding the HARQ process number, the inclusion might be needed because the HARQ feedback availability might be configurable per a HARQ process number.

Observation 11 : For optimal adaptation, different aggregation factor might be applied depending on the parameter (especially IMCS).

Observation 12 : For optimal adaptation, different aggregation factor should be applied depending on the target performance.

Observation 13 : In NR, various kinds of transport channels are multiplexed into PDSCH/PUSCH.

- Target performance of each transport channel might be distinguishable by checking the RNTI

  - PDSCH related RNTI : {P,SI,RA,MSGB,TC,C,MCS-C,CS}-RNTI

  - PUSCH related RNTI : {TC,C,MCS-C,CS}-RNTI

Observation 14 : In NTN, different target performance might be defined by the HARQ feedback availability.

Proposal 2 : Consider the enhancement via “different aggregation factors” as the one of the NTN’s transmission enhancement solutions.

- the followings might be a start point for configuring different aggregation factors

  - (a group of) MCS index

  - (a group of) RNTI type (or search space)

    - PDSCH related RNTI : {P,SI,RA,MSGB,TC,C,MCS-C,CS}-RNTI

    - PUSCH related RNTI : {TC,C,MCS-C,CS}-RNTI

  - HARQ feedback availability (enabled/disabled)

    - (a group of) HARQ process number

  - combinations of the above

  - subsets of the above

2.2.3 Enhancement via UL Feedback (Guidance of Aggregation Factor)

From Figure 1 to Figure 4, it could be observed that the adaptation between BLER and SE could be possible with the slot aggregation by changing AF. Accordingly, if AF is determined properly, the optimal channel adaptation might be possible. Additionally, the appropriate AF value might be varying depending on the channel condition and target performance. Thus, in order to replace HARQ with slot aggregation, the adaptation in both directions should be possible when the slot aggregation is used. For reference, there are 3 adaptation statuses, which are proper (optimal), too reliable (biased towards BLER), too un-reliable (biased towards SE). When the number of CRC OK in a bundle is greater than 1 (# of CRC OK in a bundle > 1) consecutively, it might corresponds to too reliable status. It would be a waste of resources (RBs & slots) because multiple CRC OK would occur in a bundle. Consequently, it would result in SE/throughput loss. On the other hands, when (# of CRC OK in a bundle = 0) occurs continuously, it might correspond to too un-reliable status and it would result in reliability/latency loss. Finally, when (# of CRC OK in a bundle = 1) occurs consistently, it might correspond to proper status. For the reference, the statistical majority among 3 cases (# of CRC OK in a bundle{>1,=1,=0}) might be used for corresponding each adaptation status, instead of consistency.

However, as described in Figure 9, if slot aggregation is used, all the CRC results for the multiple retransmissions in a bundle are lumped into a HARQ feedback depending on the number of CRC OK in a bundle. If the number of CRC OK in a bundle is greater than or equal to 1 (# of CRC OK in a bundle ≥1), the HARQ feedback is determined as “ACK”. Otherwise (# of CRC OK in a bundle = 0), the HARQ feedback is determined as “NACK”. If the transmitter receives NACK after a bundle, the optimal transmitter might apply larger aggregation factor to the next slot aggregated transmission. However, if the transmitter receives ACK after a bundle, it could not distinguish between too reliable status and proper status because the same HARQ feedback is received in both cases. Thus, transmitter cannot determine whether to keep the same AF value or to apply smaller value for the next slot aggregated transmission.

Figure 9 adaptation feasibility issue in the slot aggregation

In summary, with the slot aggregation, the adaptation toward enhancing reliability could be possible, but the adaptation towards enhancing throughput would be impossible because the mechanism to report multiple CRC OK in a bundle does not exist in NR. Assuming a situation where the AF value is increased because of the bad channel situation in the past, the AF value may not be recovered (reduced) again even if the channel situation is improved in the future.

Moreover, in NTN, HARQ feedback might be disabled and would not be sent to the transmitter. Especially, if all the HARQ feedback are disabled, all the HARQ feedbacks for DL might not be reported to gNB. In this case, it is impossible for gNB to know whether the transmission parameter is appropriate for the current situation or not. Consequently, gNB cannot determine whether to keep the same AF or to apply different (larger/smaller) AF.

Table 1 summary of adaptation feasibility issue with slot aggregation

| Adaptation status | Transmission parameter status | # of CRC OK in a bundle @ receiver | HARQ feedback | Optimal Reaction @ transmitter | Could transmitter do optimal reaction? |
| --- | --- | --- | --- | --- | --- |
| reliability loss/ latency loss | Too un-reliable | 0 | NACK | adjust AF to larger (adjust parameter towards better BLER) | possible (if NACKs occur quite consistently) |
| optimal adaptation | Proper | 1 | ACK | maintain AF (maintain parameter) | possible (otherwise) (in other words, if ACKs occur quite consistently) |
| throughput loss | Too Reliable | >1 | ACK | Adjust AF to lower (adjust parameter towards better SE) | impossible |

Observation 15 : The value of aggregation factor should be determined properly if slot aggregation is used.

- Too un-reliable parameter : reliability/latency loss (might be unable to communicate)

- Proper parameter : optimal adaptation

- Too reliable parameter : throughput loss

Observation 16 : NR gNB cannot distinguish between just proper parameter and too reliable parameter, if the slot aggregation is used.

- 0 CRC OK in a bundle (too un-reliable parameter) : NACK

- only 1 CRC OK in a bundle (proper parameter) : ACK

- multiple(>1) CRC OK in a bundle (too reliable parameter) : ACK

Observation 17 : NR gNB cannot optimally react to some cases, if the slot aggregation is used.

- toward better reliability : possible (reaction for receiving NACK quite consistently)

- maintain : possible (reaction for receiving ACKs quite consistently)

- toward better throughput : (seems to be )impossible

Observation 18 : In NR, there is no feedback mechanism to guide aggregation factor into lower value for better throughput

- Once the aggregation factor value gets larger, it may be impossible to be reduced again

Observation 19 : If all the HARQ feedback are disabled, gNB cannot optimally react to all cases

- toward better reliability : (seems to be )impossible

- maintain : (seems to be )impossible

- toward better throughput : (seems to be )impossible

From Figure 10 to Figure 13, the spectral efficiency (SE) of the proposed method are evaluated and compared with respect to the legacy NR. Each figure consists of 3 rows (1st row : achievable SE, 2nd row : optimal AF value, 3rd row : SE ratio=fixed AF SE/variable AF). The evaluation was performed with choosing only the MCS indexes and aggregation factor such that the target BLER(=1%) is guaranteed. As described above, if slot aggregation is used, NR gNB cannot distinguish the difference between optimal and too reliable aggregation factor. Moreover, once the aggregation factor value gets larger, it may be impossible to be reduced again in NR because there is no feedback mechanism in NR to guide aggregation factor into lower value for better throughput. Based on these reasons, the aggregation factor might be likely to be set into non-optimal value. The effect of non-optimal aggregation factor might be observed as the followings. As described from Figure 10 to Figure 13, the SE ratio for fixed aggregation factor cannot reach to 100 % over whole operating range for NTN. In other words, variable aggregation factor is superior than fixed aggregation factor. Moreover, if aggregation factor is set to too large value, it might be impossible to reach the value achievable by lower aggregation factor because the maximum SE might be limited to the lower value as the aggregation factor increases. The optimal values could be listed such as, in the NTN-TDL-B case of Figure 10, AF=8 (-6 ~ -4 dB), AF=4 (-3 ~ -2 dB), AF=2 (-1 ~ 1 dB), AF=1 (>1 dB). In addition, it could be also found that the optimal value would be affected by the channel condition (TDL-B, TDL-D). In NTN-TDL-D, the different optimal values might be given like AF=4 (-6 dB), AF=2 (-5 dB), AF=1 (> -4 dB). Furthermore, the optimal value would be also affected by the type of MCS index table because optimal AF is different between Figure 10 (or Figure 11) and Figure 12 (or Figure 13), respectively.

Consequently, according to the legacy NR, the non-optimal aggregation factor might be set and it results in the throughput loss. To estimate those throughput loss, SE ratio will be averaged over operating range and it would be converted to average loss by (100 % - average SE ratio [%]). The result is presented in Table 2. The average throughput loss caused by fixed aggregation factor might be from 20.8% to 45.7% for PDSCH, from 16.2% to 51% for PUSCH, respectively. On the other hand, non-optimal aggregation factor might be corrected by the proposed method (Enhancement via UL feedback), and it would be helpful to control aggregation factor properly. Additionally, UL feedback is used to maximizing SE in the above example. However, it also might be used to maximizing reliability.

Figure 10 PDSCH SE using only Normal SE MCS table
{fixed AF={1,2,4,8} (legacy NR) vs. variable AF (Enhancement via UL feedback)} 
(L: NTN-TDL-B, R: NTN-TDL-D)

Figure 11 PUSCH SE using only Normal SE MCS table
{fixed AF={1,2,4,8} (legacy NR) vs. variable AF (Enhancement via UL feedback)} 
(L: NTN-TDL-B, R: NTN-TDL-D)

Figure 12 PDSCH SE using both Normal SE MCS table and Low SE MCS table
{fixed AF={1,2,4,8} (legacy NR) vs. variable AF (Enhancement via UL feedback)} 
(L: NTN-TDL-B, R: NTN-TDL-D)

Figure 13 PUSCH SE using both Normal SE MCS table and Low SE MCS table
{fixed AF={1,2,4,8} (legacy NR) vs. variable AF (Enhancement via UL feedback)} 
(L: NTN-TDL-B, R: NTN-TDL-D)

Table 2 Average Throughput Loss caused by fixed aggregation factor

|  | Normal SE MCS Table only |  |  |  | Normal +Low SE MCS Table |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PDSCH |  | PUSCH |  | PDSCH |  | PUSCH |  |
|  | TDL-B | TDL-D | TDL-B | TDL-D | TDL-B | TDL-D | TDL-B | TDL-D |
| AF=1 | 37.1 | 26.1 | 28.2 | 23.3 | 27.4 | 20.8 | 21.8 | 16.2 |
| AF=2 | 27.0 | 30.1 | 23.7 | 25.8 | 22.7 | 23.1 | 22.9 | 21.8 |
| AF=4 | 25.8 | 34.8 | 29.0 | 38.4 | 22.0 | 28.1 | 26.3 | 35.5 |
| AF=8 | 36.8 | 45.7 | 42.4 | 51.0 | 31.2 | 41.7 | 37.2 | 46.7 |

In conclusion, for achieving the optimal adaptation, the new UL feedback to guide the aggregation factor change might be introduced. Especially, if the slot aggregation is used, the request to decrease AF should be essential for achieving optimal adaptation. Moreover, if all the HARQ feedbacks are disabled in NTN, the request that could indicate both directions, should be provided for achieving optimal adaptation. For reference, it might be configured to maintain the AF value by not sending UL feedback. CRC statistics might be used for the same purpose as the request for changing AF. For the reference, the UL feedback via RRC/MAC-CE might be preferred rather than UL feedback via UCI because it has less specification changes. Additionally, considering lower S(I)NR in NTN, UL feedback via MAC-CE/RRC could be more helpful than UL feedback via UCI because it could use retransmission mechanism in PUSCH, which supports soft combining of retransmissions.

Observation 20 : UL feedback might be helpful to guide aggregation factor into optimal value

- Non-optimal value (fixed aggregation factor) might lead the throughput loss

  - for PDSCH: from 20.8% to 45.7%.

  - for PUSCH: from 16.2% to 51%

Observation 21 : UL feedback via MAC-CE/RRC might be preferred rather than UL feedback via UCI.

- specification impact would be minimized

- soft combinable retransmission mechanism on PUSCH might be beneficial for compensating in low S(I)NR under NTN

Proposal 3 : Consider the enhancement on the aggregated transmission guidance via “UL feedback” for adaptive aggregated transmission as the one of the NTN’s transmission enhancement solutions to achieve better adaptation performance.

- UL feedback can include information such as

  - request for guiding pdsch-AggregationFactor

  - decoding statistics

  - combination of the above

- MAC-CE/RRC might be also acceptable, instead of UCI.

  - for minimizing specification impact.

  - for compensating low S(I)NR in NTN by using soft combinable retransmissions on PUSCH

- if aggregation factor is not guided by UL feedback, fixed aggregation factor (non-optimal value) might cause the throughput loss

  - for PDSCH: from 20.8% to 45.7%.

  - for PUSCH: from 16.2% to 51%

3	Conclusion

In this contribution, the following observations and proposals are made.

Observation 1 : For NTN, DL geometry SINR might range from -5.9 dB to 9.2 dB

- For SC19 (worst case), DL geometry SINR might range from -5.9 dB (5%) to -2.1 dB (95%).

- For SC3 (best case), DL geometry SINR might range from 6 dB (5%) to 9.2 dB (95%).

Observation 2 : For NTN, DL geometry SINR difference between 5% and 95% might range from 1 dB to 8.4 dB

- For SC{8,13} (narrowest), DL geometry SINR might range from 7.5 dB (5%) to 8.5 dB (95%). (1dB=8.5-7.5)

- For SC28 (widest), DL geometry SINR might range from -1.4 dB (5%) to 7 dB (95%). (8.4dB=7+1.4)

Observation 3 : For NTN, UL geometry SINR might range from -13.9 dB to 14.8 dB.

- For SC19 (worst case), the UL geometry SINR might range from -13.9 dB (5%) to -9.3 dB (95%).

- For SC23 (best case), the UL geometry SINR might range from 3 dB (5%) to 14.8 dB (95%).

Observation 4 : For NTN, UL geometry SINR difference might range from 3.4 dB to 13.4 dB.

- For SC5 (narrowest), the UL geometry SINR might range from 1.5 dB (5%) to 4.9 dB (95%). (3.4dB=4.9-1.5)

- For SC26 (widest), the UL geometry SINR might range from -8.5 dB (5%) to 4.9 dB (95%). (13.4dB=4.9+8.5)

Observation 5 : The slot aggregation (aggregation factor>1) could enhance BLER and SE simultaneously within low S(I)NR ranges.

Observation 6 : The slot aggregation (aggregation factor> 1) might be inevitable for achieving target BLER.

Observation 7 : The change of aggregation factor might be needed for achieving optimal SE performance.

Observation 8 :BLER improvement by legacy low SE MCS index table is not enough to cover all NTN study cases

- for PUSCH : low SE MCS index table in legacy NR might be insufficient

- for PDSCH :

  - low SE MCS index table in legacy NR might be enough only if target BLER ≥  0.1%

  - otherwise, low SE MCS index table in legacy NR might be insufficient

Observation 9 :larger aggregation factor might be inevitable for NTN.

- for PUSCH : 8 aggregated transmission might be insufficient even if low SE MCS index is applied.

- for PDSCH :

  - if target BLER ≥  0.1%, 8 aggregated transmission might be enough.

  - otherwise, 8 aggregated transmission might be insufficient

Observation 10 : larger aggregation factor method has less specification impact and could be more effective than low SE MCS index table method.

Observation 11 : For optimal adaptation, different aggregation factor might be applied depending on the parameter (especially IMCS).

Observation 12 : For optimal adaptation, different aggregation factor should be applied depending on the target performance.

Observation 13 : In NR, various kinds of transport channels are multiplexed into PDSCH/PUSCH.

- Target performance of each transport channel might be distinguishable by checking the RNTI

  - PDSCH related RNTI : {P,SI,RA,MSGB,TC,C,MCS-C,CS}-RNTI

  - PUSCH related RNTI : {TC,C,MCS-C,CS}-RNTI

Observation 14 : In NTN, different target performance might be defined by the HARQ feedback availability.

Observation 15 : The value of aggregation factor should be determined properly if slot aggregation is used.

- Too un-reliable parameter : reliability/latency loss (might be unable to communicate)

- Proper parameter : optimal adaptation

- Too reliable parameter : throughput loss

Observation 16 : NR gNB cannot distinguish between just proper parameter and too reliable parameter, if the slot aggregation is used.

- 0 CRC OK in a bundle (too un-reliable parameter) : NACK

- only 1 CRC OK in a bundle (proper parameter) : ACK

- multiple(>1) CRC OK in a bundle (too reliable parameter) : ACK

Observation 17 : NR gNB cannot optimally react to some cases, if the slot aggregation is used.

- toward better reliability : possible (reaction for receiving NACK quite consistently)

- maintain : possible (reaction for receiving ACKs quite consistently)

- toward better throughput : (seems to be )impossible

Observation 18 : In NR, there is no feedback mechanism to guide aggregation factor into lower value for better throughput

- Once the aggregation factor value gets larger, it may be impossible to be reduced again

Observation 19 : If all the HARQ feedback are disabled, gNB cannot optimally react to all cases

- toward better reliability : (seems to be )impossible

- maintain : (seems to be )impossible

- toward better throughput : (seems to be )impossible

Observation 20 : UL feedback might be helpful to guide aggregation factor into optimal value

- Non-optimal value (fixed aggregation factor) might lead the throughput loss

  - for PDSCH: from 20.8% to 45.7%.

  - for PUSCH: from 16.2% to 51%

Observation 21 : UL feedback via MAC-CE/RRC might be preferred rather than UL feedback via UCI.

- specification impact would be minimized

- soft combinable retransmission mechanism on PUSCH might be beneficial for compensating in low S(I)NR under NTN

Proposal 1 : Consider the enhancement via “larger aggregation factor” as the one of the NTN’s transmission enhancement solutions to achieve target BLER performance.

- for PUSCH : 8 aggregated transmission might be insufficient even if low SE MCS table is applied.

- for PDSCH :

  - 8 aggregated transmission might be enough only if both condition#1 and condition#2 are met.

    - condition#1 : target BLER ≥  0.1%

    - condition#2 : low SE MCS table is applied,

  - otherwise, 8 aggregated transmission might be insufficient

Proposal 2 : Consider the enhancement via “different aggregation factors” as the one of the NTN’s transmission enhancement solutions.

- the followings might be a start point for configuring different aggregation factors

  - (a group of) MCS index

  - (a group of) RNTI type (or search space)

  - HARQ feedback availability (enabled/disabled)

  - combinations of the above

  - subsets of the above

Proposal 3 : Consider the enhancement on the aggregated transmission guidance via “UL feedback” for adaptive aggregated transmission as the one of the NTN’s transmission enhancement solutions to achieve better adaptation performance.

- UL feedback can include information such as

  - request for guiding pdsch-AggregationFactor

  - decoding statistics

  - combination of the above

- MAC-CE/RRC might be also acceptable, instead of UCI.

  - for minimizing specification impact.

  - for compensating low S(I)NR in NTN by using soft combinable retransmissions on PUSCH

- if aggregation factor is not guided by UL feedback, fixed aggregation factor (non-optimal value) might cause the throughput loss

  - for PDSCH: from 20.8% to 45.7% loss

  - for PUSCH: from 16.2% to 51% loss

4	Reference

- Chairman’s notes RAN1#105-e

- R1-2106269, Summary#4 of AI 8.4.3 for HARQ in NTN

- 3GPP TR 38.821 v16.0.0, Solutions for NR to support non-terrestrial networks (NTN)

- 3GPP TS 38.321 v16.2.1, Medium Access Control(MAC) protocol speciﬁcation

5	Appendix

The simulation parameters from Figure 1 to Figure 4 are listed in Table 3.

Table 3 LLS parameter

| Parameters | Value |  |  |
| --- | --- | --- | --- |
| Desired Delay Spread | 300 ns |  |  |
| Subcarrier Spacing | 15 kHz (Slot duration = 1 ms) |  |  |
| Bandwidth | 20 MHz (106 RB) |  |  |
| Slot Aggregation Factor | 1,2,4,8 |  |  |
| Redundancy Version sequence | AF=1 | {0} |  |
|  | AF=2 | {0,2} |  |
|  | AF=4 | {0,2,3,1} |  |
|  | AF=8 | {0,2,3,1,0,2,3,1} |  |
| Modulation and coding scheme | QPSK | 0 (R=0.12) | 9 (R=0.66) |
|  | 16QAM | 10 (R=0.33) | 16 (R=0.64) |
|  | 64QAM | 17 (R=0.43) | 28 (R=0.93) |
| Decoder Algorithm | Layered Belief Propagation |  |  |