3GPP TSG RAN WG1 Meeting #106-e 	R1-2107504

e-Meeting, Aug. 16th – Aug. 27th, 2021

Agenda Item: 7.1

Source: MediaTek Inc.

Title: On two DCIs in the same slot for BWP switch

Document for: Discussion and decision

Introduction

In this contribution, we discuss the issue about when there are two DCIs (Ex. DCI 1_1 and DCI 0_1) in the same slot both indicating BWP switch for the same cell. A recommended conclusion to clarify RAN1’s understanding for this issue is provided.

Two DCIs in the same slot for BWP switch

In current spec, it is not clear what the expected UE behaviour is when NW sends two DCIs, Ex. DCI 1_1 and DCI 0_1, in the same slot for BWP change for the same cell. An example is shown in Fig. 1 below.

Figure 1. An example of two DCIs in the same slot for BWP switch for the same cell

In Fig. 1, UE receives DCI 1_1 with k0=5 and DCI 0_1 with k2=7 at the same slot both indicating BWP change for the same cell. It is not clear what the expected UE behaviour is for the following cases:

Table I: Cases for UE receiving two DCIs in the same slot both indicating BWP switch for the same cell

| Duplex | Both DCIs indicating the same or different DL/UL BWP pairs | K0 = K2? (Same SCS for DL/UL) | Violate Spec? |
| --- | --- | --- | --- |
| Case 1: TDD | Same | Yes | Not specified |
| Case 2: TDD | Same | No | Not specified |
| Case 3: TDD | Different | Yes | Not specified |
| Case 4: TDD | Different | No | Not specified |
| Case 5: FDD | N/A (no DL/UL BWP pairing in FDD) | Yes | Not specified |
| Case 6: FDD | N/A (no DL/UL BWP pairing in FDD) | No | Not specified |

Observation 1: In current spec, it is not clear what the expected UE behaviour is when UE receives two DCIs, Ex. DCI 1_1 and DCI 0_1, in the same slot for BWP change for the same carrier. An example is shown in Fig. 1 and 6 possible cases are shown in Table I.

In current spec 38.213 [1] Chapter 12, the following UE behaviours related to DCI-triggered BWP switch are specified:

A UE does not expect to detect a DCI format with a BWP indicator field that indicates an active DL BWP or an active UL BWP change with the corresponding time domain resource assignment field providing a slot offset value for a PDSCH reception or PUSCH transmission that is smaller than a delay required by the UE for an active DL BWP change or UL BWP change, respectively [10, TS 38.133].

If a UE detects a DCI format with a BWP indicator field that indicates an active DL BWP change for a cell, the UE is not required to receive or transmit in the cell during a time duration from the end of the third symbol of a slot where the UE receives the PDCCH that includes the DCI format in a scheduling cell until the beginning of a slot indicated by the slot offset value of the time domain resource assignment field in the DCI format.

If a UE detects a DCI format indicating an active UL BWP change for a cell, the UE is not required to receive or transmit in the cell during a time duration from the end of the third symbol of a slot where the UE receives the PDCCH that includes the DCI format in the scheduling cell until the beginning of a slot indicated by the slot offset value of the time domain resource assignment field in the DCI format.

With the 38.213 [1] Chapter 12 spec quoted above, we have the following observation:

Observation 2: In current spec 38.213 [1] Chapter 12, UE is not required to receive or transmit in the BWP-changing cell from the end of the third symbol of a slot where UE receives the DL/UL BWP change indication DCI to the beginning of a slot indicated by k0/k2. Hence, take Fig. 1 as an example, the PDSCH in slot (N+5) should be dropped.

To clarify the UE behaviour for cases specified in Table I, we have the following proposal:

Proposal 1: To clarify the UE behaviour for cases specified in Table I, RAN1 to draw a conclusion based on the table shown below, where Case 1,2, 5, & 6 are allowed in Rel-15 NR with specified UE behaviour, while Case 3 & 4 are precluded in Rel-15 NR.

When UE receives two DCIs in the same slot for BWP change for the same cell:

| Duplex | Both DCIs indicating the same or different DL/UL BWP pairs | K0 = K2? (Same SCS for DL/UL) | Allowed or not | Expected UE behaviour |
| --- | --- | --- | --- | --- |
| Case 1: TDD | Same | Yes | Yes | - If single DCI is detected, UE switches to the designated DL/UL BWP pair no matter which DCI is detected and receive/transmit the corresponding PDSCH/PUSCH - If both DCIs are detected, UE switches to the designated DL/UL BWP pair based on the detected DCIs - Both PDSCH and PUSCH scheduled by the DCIs are not dropped |
| Case 2: TDD | Same | No | Yes | - If single DCI is detected, UE switches to the designated DL/UL BWP pair no matter which DCI is detected and receive/transmit the corresponding PDSCH/PUSCH - If both DCIs are detected, UE switches to the designated DL/UL BWP pair based on the detected DCIs; earlier PDSCH or PUSCH scheduled by the 1st DCI indicating DL/UL BWP pair change will be dropped - If PDSCH is dropped, PUSCH Tx scheduled by the 2nd DCI is kept and can be viewed as an ACK from a UE to gNB for the DL/UL BWP pair change - If PUSCH is dropped, HARQ-ACK to the PDSCH scheduled by the 2nd DCI can be viewed as an ACK from a UE to gNB for the DL/UL BWP pair change |
| Case 3: TDD | Different | Yes | No | UE does not expect this case |
| Case 4: TDD | Different | No | No | UE does not expect this case |
| Case 5: FDD | N/A (no DL/UL BWP pairing in FDD) | Yes | Yes | - If single DCI is detected, UE switches its DL BWP or UL BWP to the designated one based on the detected DCI and receive/transmit the corresponding PDSCH/PUSCH - If both DCIs are detected, UE switches its DL BWP and UL BWP to the designated ones based on the detected DCIs - Both PDSCH and PUSCH scheduled by the DCIs are not dropped |
| Case 6: FDD | N/A (no DL/UL BWP pairing in FDD) | No | Yes | - If single DCI is detected, UE switches its DL BWP or UL BWP based on the detected DCI and receive/transmit the corresponding PDSCH/PUSCH - If both DCIs are detected, UE switches its DL BWP and UL BWP to the designated ones based on the detected DCIs; earlier PDSCH or PUSCH scheduled by the 1st DCI indicating DL BWP or UL BWP change will be dropped; later PDSCH or PUSCH scheduled by the 2nd DCI indicating DL BWP or UL BWP change will be kept - There is no ACK from a UE to gNB for the DL BWP or UL BWP change indicated by the 1st DCI - There is ACK from a UE to gNB for the DL BWP or UL BWP change indicated by the 2nd DCI |

Conclusion

In this contribution, we discuss the issue about when there are two DCIs (Ex. DCI 1_1 and DCI 0_1) in the same slot both indicating BWP switch for the same cell with the following observations and proposals:

Observation 1: In current spec, it is not clear what the expected UE behaviour is when UE receives two DCIs, Ex. DCI 1_1 and DCI 0_1, in the same slot for BWP change for the same carrier. An example is shown in Fig. 1 and 6 possible cases are shown in Table I.

Observation 2: In current spec 38.213 [1] Chapter 12, UE is not required to receive or transmit in the BWP-changing cell from the end of the third symbol of a slot where UE receives the DL/UL BWP change indication DCI to the beginning of a slot indicated by k0/k2. Hence, take Fig. 1 as an example, the PDSCH in slot (N+5) should be dropped.

Proposal 1: To clarify the UE behaviour for cases specified in Table I, RAN1 to draw a conclusion based on the table shown below, where Case 1,2, 5, & 6 are allowed in Rel-15 NR with specified UE behaviour, while Case 3 & 4 are precluded in Rel-15 NR.

When UE receives two DCIs in the same slot for BWP change for the same cell:

| Duplex | Both DCIs indicating the same or different DL/UL BWP pairs | K0 = K2? (Same SCS for DL/UL) | Allowed or not | Expected UE behaviour |
| --- | --- | --- | --- | --- |
| Case 1: TDD | Same | Yes | Yes | - If single DCI is detected, UE switches to the designated DL/UL BWP pair no matter which DCI is detected and receive/transmit the corresponding PDSCH/PUSCH - If both DCIs are detected, UE switches to the designated DL/UL BWP pair based on the detected DCIs - Both PDSCH and PUSCH scheduled by the DCIs are not dropped |
| Case 2: TDD | Same | No | Yes | - If single DCI is detected, UE switches to the designated DL/UL BWP pair no matter which DCI is detected and receive/transmit the corresponding PDSCH/PUSCH - If both DCIs are detected, UE switches to the designated DL/UL BWP pair based on the detected DCIs; earlier PDSCH or PUSCH scheduled by the 1st DCI indicating DL/UL BWP pair change will be dropped - If PDSCH is dropped, PUSCH Tx scheduled by the 2nd DCI is kept and can be viewed as an ACK from a UE to gNB for the DL/UL BWP pair change - If PUSCH is dropped, HARQ-ACK to the PDSCH scheduled by the 2nd DCI can be viewed as an ACK from a UE to gNB for the DL/UL BWP pair change |
| Case 3: TDD | Different | Yes | No | UE does not expect this case |
| Case 4: TDD | Different | No | No | UE does not expect this case |
| Case 5: FDD | N/A (no DL/UL BWP pairing in FDD) | Yes | Yes | - If single DCI is detected, UE switches its DL BWP or UL BWP to the designated one based on the detected DCI and receive/transmit the corresponding PDSCH/PUSCH - If both DCIs are detected, UE switches its DL BWP and UL BWP to the designated ones based on the detected DCIs - Both PDSCH and PUSCH scheduled by the DCIs are not dropped |
| Case 6: FDD | N/A (no DL/UL BWP pairing in FDD) | No | Yes | - If single DCI is detected, UE switches its DL BWP or UL BWP based on the detected DCI and receive/transmit the corresponding PDSCH/PUSCH - If both DCIs are detected, UE switches its DL BWP and UL BWP to the designated ones based on the detected DCIs; earlier PDSCH or PUSCH scheduled by the 1st DCI indicating DL BWP or UL BWP change will be dropped; later PDSCH or PUSCH scheduled by the 2nd DCI indicating DL BWP or UL BWP change will be kept - There is no ACK from a UE to gNB for the DL BWP or UL BWP change indicated by the 1st DCI - There is ACK from a UE to gNB for the DL BWP or UL BWP change indicated by the 2nd DCI |

Reference

3GPP TS 38.213 g60, ”NR; Physical layer procedures for control”