3GPP TSG RAN WG1 #106-e		    	R1-2107511

e-Meeting, August 16th – 27th, 2021

Agenda item:	8.2.4

Title:	Beam management discussion for 52.6-71 GHz NR operation

Source:	MediaTek Inc.

Document for:	Discussion

- Introduction

The following objectives are included in the updated WID [1] for the discussion related to beam management

  - Specify timing associated with beam-based operation to new SCS (i.e., 480kHz and/or 960kHz), study, and specify if needed, potential enhancement for shared spectrum operation

    - Study which beam management will be used as a basis: R15/16 or R17 in RAN #91-e

In RAN1 #104-e and RAN1 #104bis-e meetings, several topics have been discussed including the baseline of beam management in >52.6GHz WI, beam management related UE capability, multi-beam enhancement for multi-PDSCH scheduling, and periodic reference signal enhancement for beam management in unlicensed spectrum. The following agreements for the above discussion are reached.

Agreement: - For NR operation in 52.6-71GHz with new SCSs, new parameter values for at least the following timing parameters are needed: - timeDurationForQCL - beamSwitchTiming - beamReportTiming - Companies are encouraged to provide preferred values on timeDurationForQCL, beamSwitchTiming and beamReportTiming Agreement: Rel-15/16 and any Rel-17 beam management enhancements can be considered for 52.6-71 GHz. Whether particular features should be excluded for 52.6-71 GHz can be further discussed. - Note: As per usual procedure, duplication of work between work items in Rel-17 should be avoided Agreement: - Further study new parameter values for at least the following parameters: - maxNumberRxTxBeamSwitchDL - Additional beam switching time delay d for triggering AP-CSI-RS when triggering PDCCH with 120kHz or 480kHz has a smaller subcarrier spacing than AP-CSI-RS - Study whether/how to introduce a beam switching gap between signals/channels - FFS: condition to apply including potential UE capability definition - Study should account for inputs from RAN4 Agreement: Further study the following: - For multi-PDSCH scheduling with a single DCI, study the QCL assumption(s) the UE should apply for each PDSCH for the case when some of the scheduled PDSCHs have scheduling offset less than timeDurationForQCL while some have scheduling offset equal to or greater than timeDurationForQCL. - For multi-PDSCH scheduling with a single DCI, study the QCL assumption(s) the UE should apply for each PDSCH for the case when all of the scheduled PDSCHs have scheduling offset less than timeDurationForQCL - Note: If the current Rel-16 behavior would be extended to multiple-PDSCH scheduling, it could result in a different QCL assumption for each PDSCH due to the fact the that the CORESET with the lowest ID can be different for different slots, resulting in a potentially different TCI state for each slot - Note: Applicability to multi-TRP can be discussed further Agreement: Further study the following: - For multi-PDSCH scheduling with a single DCI, study whether or not it is needed to indicate a separate TCI state for each scheduled PDSCH - For multi-PUSCH scheduling with a single DCI, study whether or not it is needed to indicate a separate SRI (indication of TCI can be further discussed) for each scheduled PUSCH - Note: the study should take into account DCI overhead aspects - Note: Applicability to multi-TRP can be discussed further Agreement: Introduce new parameter values for additional beam switching time delay d, when triggering PDCCH with 120kHz or 480kHz has a smaller subcarrier spacing than AP-CSI-RS or PDSCH Agreement: For timeDurationForQCL, beamSwitchTiming and beamReportTiming, - Following candidate values of FR2 are reused for 120 kHz: - timeDurationForQCL: 14 and 28 symbols - beamSwitchTiming: 14, 28, 48, 224 and 336 symbols - beamReportTiming: 14, 28 and 56 symbols - For 480 kHz - Support at least the candidate values for 120 kHz scaled by 4x - FFS: Support for additional candidate value(s) less than maximum of 4x (candidate values for 120 kHz) - For 960 kHz - Support at least the candidate values for 120 kHz scaled by 8x - FFS: Support for additional candidate values(s) less than maximum of 8x (candidate values for 120 kHz) - FFS: UE capability signaling details - Note: The scaled values 224 and 336 symbols for beamSwitchTiming are used as in Rel-16 (defined in Rel-15 with updates in Rel-16). Agreement: For multiple PDSCHs/PUSCHs scheduled by a single DCI, at least for single TRP, support indication of only a single TCI state/SRI in DCI FFS: number of TCI states/SRIs in a single DCI scheduling multiple PDSCHs/PUSCHs for multi-TRP

Couple of enhancements are decided for further study, especially on the need of multi-beam enhancement for multi-PDSCH scheduling and beam management related UE capabilities. Our views on those aspects are as follows.

- Beam management related UE capability

In #104bis-e meetings, the candidate values of timeDurationForQCL, beamSwitchTiming, and beamReportTiming for 480 kHz and 960kHz were discussed and at least the absolute time durations used in 120kHz are agreed to be supported for the new SCSs. To proceed the discussion of additional candidate values to address the FFS discussion, the BD/CCE limit should be taken into account. Therefore, we suggest to evaluate those additional candidate values after the BD/CCE limits are specified for 480kHz and 960kHz.

Proposal 1: Defer the discussion of additional values of timeDurationForQCL, beamSwitchTiming and beamReportTiming for 480 kHz and 960kHz till the BD/CCE limits are specified for 480kHz and 960kHz.

There is another discussion on the candidate values of maxNumberRxTxBeamSwitchDL for 120kHz, 480kHz, and 960kHz but no agreed values yet. The definition of maxNumberRxTxBeamSwitchDL is the number of Tx and Rx beam changes UE can perform within a slot and the candidate values are {4, 7, 14} for 120kHz in FR2-1. In our view, the same candidate values should be reused in FR2-2.

Proposal 2: For maxNumberRxTxBeamSwitchDL, the candidate values of {4, 7, 14} supported in FR2-1 are reused for 120kHz in FR2-2.

As for the candidate values of maxNumberRxTxBeamSwitchDL for 480kHz and 960kHz, the discussion should acknowledge the outcome of RAN4 discussion on the beam switching gap. Based on the LS from RAN4 [2], the agreements from RAN4 are summarized in Table 1, where beam switching related discussions are still pending for 480kHz and 960kHz. On the other hand, based on the supported candidate values {4, 7, 14} of 120 kHz in FR2-1, the most advanced UE capability is to switch beams in every symbol of 120kHz, which is equivalent to switch beams in every 4 symbols of 480kHz or every 8 symbols of 960kHz. Therefore, maxNumberRxTxBeamSwitchDL candidate values {<=3} for 480kHz and {1} for 960kHz should be supported and all additional values should be discussed after the RAN4 decision on the beam switching time.

Proposal 3: For maxNumberRxTxBeamSwitchDL, the UE should support at least {≤3} for 480kHz and at least {1} for 960kHz in FR2-2. Additional values should be discussed after the RAN4 decision on the beam switching time in 480kHz and 960kHz.

Table 1 Summary of 60 GHz time-related issues and agreements

| Issue | Agreement |
| --- | --- |
| RX-TX and TX-RX beam switching | - For NR operation in the 52.6 – 71 GHz range, the Rx-Tx and Tx-Rx transition time shall reuse the FR2 value of 13792 Tc. (7.015 usec) for 120 kHz SCS - FFS for Rx-Tx and Tx-Rx transition time for 480/960 kHz SCS |
| Minimum duration between beam switches | - RAN4 will further discuss the definition of beam switch scenario(s) related to this proposed requirement and whether a requirement on the minimum duration between beam switches is needed - If the requirement is needed, then RAN4 will further discuss how to decide the value |
| UE Beam switch time (beam direction switch only) | - RAN4 will further discuss based on the following alternatives: (1) simulation study to quantify impact of beam switch time on network performance, (2) further discussion of UE feasibility, (3) analysis of the system impact (by some other means than sim study) |
| UE Inter-panel Beam switch time (beam direction switch only) | - Depends on conclusion of the intra-panel beam switch time and analysis of delays in addition to intra-panel, if any, associated with inter-panel beam switch time |
| gNB Beam switch time (beam direction switch only) | - RAN4 tentatively agrees [59 ns] with the understanding that the value can be confirmed once open issues related to BS output power are resolved |
| TX ON-ON and TX ON-OFF transient period | - Re-use UE transient time from current FR2 for 120 kHz SCS - FFS on UE transient time for 480/960 kHz SCS |

- Beam management enhancement on multi-PDSCH

In RAN1 #104-e and #104bis-e meetings, default beam assumption for multi-PDSCH scheduling has been discussed. In Rel-15/16, when the gap between scheduled PDSCH(s) and the scheduling DCI is less than a duration specified by timeDurationForQCL, the PDSCH(s) is received with a default PDSCH beam defined as the beam for the lowest CORESET ID in latest monitored slot. Based on our understanding, this default beam assumption is also applied to multi-slot PDSCH repetition scheduling and it is allowed that the scheduled multi-slot PDSCH repetition are received by different beams when the lowest CORESET ID is changing through the slots for the multi-slot PDSCH. Therefore, we don’t see a clear motivation to discharge the same default QCL assumption for multi-PDSCH scheduling unless a substantial benefit is identified. Moreover, for the reception of a PDSCH within the duration specified by timeDurationForQCL, when the beam for PDSCH DM-RS is different from that of the PDCCH DM-RS, UE should prioritize the reception of the PDCCH based on the current Rel-15/16 QCL assumption. Therefore, the QCL assumption within duration specified by timeDurationForQCL has been clearly specified by prioritizing the beam for receiving CORESET and the same default beam assumption should be applicable to multi-PDSCH scheduling as well.

Proposal 4: For the reception of multi-PDSCHs scheduled by a single DCI within the duration specified by timeDurationForQCL, current Rel-15/16 default beam assumption should be applied.

For the beam management enhancement on multi-PDSCH scheduling, there was another discussion on the number of TCI states in the DCI for multi-TRP. In Rel-16, reception of PDSCH repetition from multi-TRP has been introduced and only one TCI field is included in the scheduling DCI. Therefore, the motivation to introduce more TCI fields in the scheduling DCI is not clear to us. On the other hand, this enhancement may increase DCI overhead, which is not preferred without distinct benefit.

Proposal 5: For multi-TRP, support only single TCI field in the DCI scheduling multi-PDSCHs..

- Periodic RS enhancement for unlicensed spectrum

In unlicensed spectrum, periodic RS transmission is subjected to LBT in the regions where LBT is required. Consequently, the beam management procedure relying on the measurement of periodic RS may be impacted when the periodic RS transmission is not guaranteed. The same issue has been discussed in sub 6 GHz NRU and the validation rules of the period CSI-RS are specified as the discussion outcome. For the >52.6GHz band, the transmission is tend to be more directional and LBT failure is generally rare based on the analysis in SI. Therefore, applying validation rule is not efficient to address the reliability issue of periodic RS transmission. Consequently, instead of proposing the validation rule of periodic RS, companies proposed to enhance the reliability of reception, e.g., considering AP-CSI-RS or increase the transmission opportunity of period RS, to accommodate the failure of LBT.

Although the AP-CSI-RS triggering when periodic CSI-RS transmission is prevented by LBT failure can improve the CSI-RS transmission reliability and may inform UE the periodic CSI-RS was not transmitted, the feasibility of such feature is not clear when considering the scheduling latency between DCI and the triggered AP-CSI-RS. For example, to serve the purpose of amending the missing periodic CSI-RS, the DCI and the triggered AP-CSI-RS should be sent immediately after the gNB occupied the channel and before the next periodic CSI-RS transmission, which is not guaranteed. Moreover, when the gap between DCI and the triggered AP-CSI-RS is smaller than the threshold beamSwitchTiming, a default beam to receive the AP-CSI-RS is specified in the current specification, which might not be QCLed with the missing periodic CSI-RS.

Proposal 6: The feasibility of AP-CSI-RS triggering for accommodating the periodic CSI-RS transmission prevented by LBT failure needs to be studied when the gap between DCI and the triggered AP-CSI-RS is smaller than the threshold beamSwitchTiming.

Regarding the proposal of increasing the transmission opportunity of period RS to alleviate the impact of LBT, the same principle is adopted in sub 6 GHz SSB transmission and the same enhancement on SSB transmission >52.6GHz unlicensed spectrum is discussed in this WI as well. However, there is no detection mechanism for CSI-RS transmission and increasing the transmission opportunity of period RS might lead to more measurement opportunities on missing periodic CSI-RS due to LBT failure. Also, the increasing transmission opportunity reduces the resource usage efficiency. Hence, we don’t see the clear advantage to adopt such enhancement to address the rare LBT failure cases in this frequency range.

Proposal 7: The feasibility of increasing periodic RS transmission occasion for accommodating the periodic CSI-RS transmission prevented by LBT failure needs to be studied with respect to the minimization of resource and impact on measurement procedure when more measurements on missing RS due to LBT failure.

- Conclusion

In summary, we have the following proposals:

Proposal 1: Defer the discussion of additional values of timeDurationForQCL, beamSwitchTiming and beamReportTiming for 480 kHz and 960kHz till the BD/CCE limits are specified for 480kHz and 960kHz.

Proposal 2: For maxNumberRxTxBeamSwitchDL, the candidate values of {4, 7, 14} supported in FR2-1 are reused for 120kHz in FR2-2.

Proposal 3: For maxNumberRxTxBeamSwitchDL, the UE should support at least {≤3} for 480kHz and at least {1} for 960kHz in FR2-2. Additional values should be discussed after the RAN4 decision on the beam switching time in 480kHz and 960kHz.

Proposal 4: For the reception of multi-PDSCHs scheduled by a single DCI within the duration specified by timeDurationForQCL, current Rel-15/16 default beam assumption should be applied.

Proposal 5: For multi-TRP, support only single TCI field in the DCI scheduling multi-PDSCHs..

Proposal 6: The feasibility of AP-CSI-RS triggering for accommodating the periodic CSI-RS transmission prevented by LBT failure needs to be studied when the gap between DCI and the triggered AP-CSI-RS is smaller than the threshold beamSwitchTiming.

Proposal 7: The feasibility of increasing periodic RS transmission occasion for accommodating the periodic CSI-RS transmission prevented by LBT failure needs to be studied with respect to the minimization of resource and impact on measurement procedure when more measurements on missing RS due to LBT failure.

- References

- RP-202925, “Revised WID:  Extending current NR operation to 71 GHz”, December, 2020

- R1-2106425, “R4-2104451 (R1-2102202) LS on beam switching gap for 60 GHz band”, August, 2021