3GPP TSG RAN WG1 #106-e		                                    		R1-2107535

e-Meeting, Aug 16 – Aug 27, 2021

Agenda Item:	8.14.2

Source:	InterDigital Inc.

Title:	Remaining Issues on XR Evaluation Methodologies

Document for:	Discussion and Decision

# Introduction

In RAN#105-e meeting, the methodology for coverage evaluations was discussed and included in chair’s notes as follows [1].

For companies to further study and if necessary, discuss in RAN1#106-e (Coverage evaluation methodology) For XR/CG in DL or UL, coverage is defined to be the A-percentile point in CDF of Coupling gain for the “satisfied” UEs, with #UEs per cell = B, for a given XR application (AR/VR/CG) in a given deployment scenario (DU/InH/UMa) - A = [5], other value can also be reported - FFS: Value of B, e.g. B = 1, capacity, etc. - Note: Coupling gain for coverage evaluation is defined as the ratio of received and transmitted power measured in dB, and includes antenna gains, path loss, shadowing, indoor- or body loss, etc. Example of coupling gain can refer to TR 37.910. An alternate method could be to use the “traditional” method such as what is used in the CE study/work item.

In this contribution, we provide our view on remaining issues for evaluation of coverage for XR and evaluation of multi-stream XR traffic.

# RAN Evaluation for XR

2.1 Coverage Evaluation for XR

Coverage evaluation has so far been given lower priority and majority of discussion in RAN1#104-e - RAN1#105-e focused capacity and UE power evaluation employing system level simulation based on the agreed assumptions for each XR application. Coverage for XR can be defined as the probability that a UE is satisfied in a low load condition. For NR, maximum coupling gain/loss is used as a measure of coverage performance for a network service. All the assumptions that have been agreed upon to evaluate capacity for XR services can also be used to evaluate coverage performance. That is, the same methodologies and assumptions agreed for system level simulation to evaluate capacity of XR in DL/UL can be reused for coverage evaluation.

Proposal 1:  	For XR coverage evaluation, reuse the methodologies and simulation assumptions agreed for system level simulation to evaluate capacity.

Assuming a single UE per cell, the coverage performance would be the maximum coupling gain, which is the 100th percentile point in CDF of coupling gain, in the absence of interference noise. For XR, capacity is the maximum number of UEs with at least X% of UEs being satisfied. A UE is declared satisfied if more than Y (%) of packets are successfully transmitted within a given air interface PDB. The same satisfaction criteria should be used to evaluate coverage for XR.

Proposal 2:  	For XR coverage evaluation, apply the same per-UE satisfaction criteria (e.g. X%, PDB) used for capacity evaluation.

For XR, it is reasonable to look at satisfied UEs only, which is a relatively low load cell with limited interference, when looking into coverage performance. In low load cell with a given number of UEs, coupling gain/loss evaluation for a UE would still have to consider the interference experienced by the UE. In such a case, coverage can be defined as a percentile in CDF of coupling gain where 100th percentile refers to interference free scenario. Therefore, the formal definition of coverage for XR services given in [1] can be adopted. That is, for XR/CG in DL/UL, coverage is defined to be the A-percentile point in CDF of coupling gain for the “satisfied” UEs, with #UEs per cell = B, for a given XR application (AR/VR/CG) in a given deployment scenario (DU/InH/UMa).

Proposal 3:  	Adopt the definition for DL/UL coverage evaluation in XR/CG as stated in [1].

The values for A can be reported by companies whereas the number of UEs per cell, B, can be equal to 1 as a baseline. Optionally, B can also be chosen to be equal to the number of satisfied UEs.

Proposal 4:  	The value for A (90, 95, 99) percentile, can be reported by companies and as a baseline, B can be set to 1 while optionally, B can be chosen to be equal to the number of satisfied UEs.

2.2 Multiple data streams for XR

For a UE supporting multiple data streams, each stream represents data from and intended to a single application which is subjected to a corresponding QoS (e.g., PDP, PER, …) requirement and KPI. For multiple data streams, it was agreed that a UE can be declared satisfied if more than Xm (%) of packets in each of the M flows are successfully transmitted within their respective air interface PDB values. However, in majority of AR/VR services with multiple data streams, the requirement on the QoE can force some interdependency between streams such that, in addition to the QoS (e.g. PER, PDB, etc) each stream is required to fulfil independently, there needs to be coordination between the streams to ensure that the data in different streams arrive within a bounded time delay between them.

Observation 1:  In majority of AR/VR services with multiple data streams, the requirement on the QoE can force some interdependency between streams such that, in addition to the QoS (eg. PER, PDB, etc…) each stream is required to fulfil independently, there needs to be coordination between the streams to ensure that the data in different streams arrive within a bounded time delay between them.

This implies that in addition to the individual QoS requirement assigned to each stream, there needs to be a joint QoS requirement that all streams associated with an application must fulfil jointly. Therefore, RAN1 should study to determine and agree on a joint QoS requirement for multi-stream XR applications.

Proposal 5:  	RAN1 to study to determine a joint QoS requirement that all streams must jointly fulfil when supporting multi-stream XR application

# Conclusion

In this contribution, remaining issues on XR evaluation methodology are discussed and the following observation were made:

Observation 1:  In majority of AR/VR services with multiple data streams, the requirement on the QoE can force some interdependency between streams such that, in addition to the QoS (eg. PER, PDB, etc…) each stream is required to fulfil independently, there needs to be coordination between the streams to ensure that the data in different streams arrive within a bounded time delay between them.

Based on the observation, the following conclusions were made:

Proposal 1:  	For XR coverage evaluation, reuse the methodologies and simulation assumptions agreed for system level simulation to evaluate capacity.

Proposal 2:  	For XR coverage evaluation, apply the same per-UE satisfaction criteria (e.g. X%, PDB) used for capacity evaluation.

Proposal 3:  	Adopt the definition for DL/UL coverage evaluation in XR/CG as stated in [1].

Proposal 4:  	The value for A (90, 95, 99) percentile, can be reported by companies and as a baseline, B can be set to 1 while optionally, B can be chosen to be equal to the number of satisfied UEs.

Proposal 5:  	RAN1 to study to determine a joint QoS requirement that all streams must jointly fulfil when supporting multi-stream XR application

# References

RAN1 Chairman notes, RAN1#105-e, May 2021