3GPP TSG RAN WG1 #106-e		                     R1-2107540

e-Meeting, August 16th – 27th, 2021

Agenda item:	8.4.3

Source:	LG Electronics

Title:	Discussions on HARQ enhancements in NTN

Document for:	   Discussion and Decision

- Introduction

In the RAN1#105-e meeting, following working assumption and agreements were made and captured in the chairman’s note as:

Agreement:

For enhancement on the HARQ process indication, extend the HARQ process ID field up to 5 bits for DCI 0-2/1-2

Agreement:

For Type-2 HARQ codebook in NTN,

- For the DCI of PDSCH with feedback-enabled HARQ processes, the C-DAI and T-DAI are the count of only feedback-enabled processes

- FFS: Whether DCI for SPS release and any other DCIs are included in counting of C-DAI and T-DAI

Agreement:

Confirm the previous working assumption for  X = T_proc,1 where X is defined from the end of the reception of the last PDSCH or slot-aggregated PDSCH for a given HARQ process with disabled feedback to the start of the PDCCH carrying the DCI scheduling another PDSCH or set of slot-aggregated PDSCH for the given HARQ process.

Agreement:

For enhancement on the HARQ process indication at least for DCI 0-1/1-1, the Option-1 and Option-1a are lower priority for further discussion.

Agreement:

Discussion of enhancement(s) on the aggregated transmission (including repetition) is prioritized to improve the performance in NTN.

In this contribution, we further discuss on the HARQ enhancements in NTN.

- Discussions

HARQ process id identification

In RAN1#105-e meeting, it was intensively discussed on the HARQ process id identification, and made agreement on the DCI format 0-2/1-2 and some progress on the DCI format 0-1/1-1. The remaining issues are how to handle DCI format 0-0/1-0 and 0-1/1-1. For HARQ process id identification, following options are listed during the discussion.

- Option 1: Slot index as the MSB

- Option 1-a:Slot index as the LSB

- Option 2: Reusing one bit from other bit field

- Option 3: Extending the HARQ process ID field up to 5 bits

Option 1 or 1-a can provide additional 1 bit based on the slot index at the expense of scheduling flexibility or additional UE complexity. For example, in the slot based solution, additional 1bit can be obtained by the scheduled slot index (e.g., according to the even slot index or odd slot index), but the scheduling flexibility at gNB can be restricted compared to other options. In addition, this option is de-prioritized for DCI format 0-1/1-1 in the last meeting.

In option 2, other field can be reused for obtaining additional 1 bit. In case of HARQ feedback is disabled, some fields such as NDI, RV, PRI, etc., can be a candidate for option 2. However, it is not clear how this option work when HARQ feedback is enabled.

Option 3 can be a straightforward and clean way for supporting up to 32 HARQ processes. For non-fallback DCI (DCI format 0-1/1-1) cases, increasing 1bit payload in DCI can be acceptable for explicit HARQ process indication. However, in case of fall back DCI (DCI format 0-0/1-0), it is not desirable to increase the payload, since fall back DCI is designed for robustness and supporting basic NR operations. Thus, in this case, it is desirable to limit the number of HARQ processes up to 16 for fallback operation. Based on above discussion, for enhanced HARQ process id identification, our preference is supporting option 3 for DCI format DCI 0-1/1-1 and no enhancement for DCI format 0-0/1-0.

Proposal 1: For enhanced HARQ process id identification in NTN, support followings enhancement.

- For DCI format 0-1/1-1, increase the HARQ process ID field up to 5 bits

- For DCI format 0-0/1-0, support no enhancement

Enabling/disabling HARQ feedback

Performance enhancements

In order to increase the link reliability performance, some enhancement on the transmission can be considered when the HARQ feedback is disabled. In the last meeting, among the candidate enhancements, aggregated transmission (including repetition) is agreed to prioritize in Rel-17 NTN.  For NTN scenario, larger number of repetition may provide good DL BLER performance at the expense of spectral efficiency. Thus, it is desired for gNB to select proper repetition number according to SINR and channel condition. One way to gNB know the channel condition of UE is relying on CSI and/or HARQ feedback. In conventional CSI reporting, UE measures DL channels based on the reference signal (e.g., CSI-RS), and calculates its preferred channel state information (e.g., RI, CQI, PMI) and reports it to gNB. This implicit CSI reporting may be helpful for gNB to select MCS and beamforming, but it is not directly coupled with proper repetition number. To this end, repetition number can be recommended by UE.

Proposal 2. For transmission enhancement when HARQ feedback is disabled, consider following enhancements:

- Repetition number is recommended/reported by UE

PDSCH scheduling restriction between different HARQ process ids

In a given scheduled cell, the UE is not expected to receive a first PDSCH and a second PDSCH, starting later than the first PDSCH, with its corresponding HARQ-ACK assigned to be transmitted on a resource ending before the start of a different resource for the HARQ-ACK assigned to be transmitted for the first PDSCH, where the two resources are in different slots for the associated HARQ-ACK transmissions, each slot is composed of symbols [4] or a number of symbols indicated by subslotLengthForPUCCH if provided, and the HARQ-ACK for the two PDSCHs are associated with the HARQ-ACK codebook of the same priority. In a given scheduled cell, the UE is not expected to receive a first PDSCH, and a second PDSCH, starting later than the first PDSCH, with its corresponding HARQ-ACK assigned to be transmitted on a resource ending before the start of a different resource for the HARQ-ACK assigned to be transmitted for the first PDSCH if the HARQ-ACK for the two PDSCHs are associated with HARQ-ACK codebooks of different priorities.

When two PDSCHs are associated with different HARQ process id, PDSCH scheduling restriction can also be considered and corresponding UE behavior is captured in TS 38.214 as below.

According to above specification, if there are two PDSCHs where PDSCH1 is scheduled in slot n and PDSCH2 is scheduled in slot n+k, ACK/NACK feedback timing for PDSCH1 should be before that of PDSCH2. In Rel-17 NTN, it is agreed to support HARQ feedback disabling and thus, if one of HARQ process is feedback-disabled, there might be some ambiguity on out-of-order (OOO) constraint check. For example, if PDSCH1 is feedback-enabled and PDSCH2 is feedback-disabled, there is some ambiguity for PDSCH2 OOO check timing. To this end, one simple solution can be introducing default/virtual ACK/NACK timing or timing offset for feedback-disabled HARQ process.

Proposal 3. In case of two PDSCHs are associated with different HARQ process ids, introduce default/virtual ACK/NACK timing or timing offset for feedback-disabled HARQ process.

- HARQ-ACK codebook enhancement

Type-1 HARQ-ACK codebook

In Type-1 HARQ-ACK codebook, the location of ACK/NACK feedback bit is determined based on the SLIV of the scheduling DCI and the codebook size is determined by semi-static configuration. In NTN, some HARQ processes can be configured as HARQ feedback disabled. In this case, the UE is not required to read some fields (e.g., K1 and PRI field) used for ACK/NACK feedback even though those fields are presented. Then, Type-1 HARQ-ACK codebook could be constructed only for the serving cells having at least one feedback-enabled HARQ process, by excluding the serving cells not having any feedback-enabled HARQ process. For example, in case of all HARQ processes in a certain CC are disabled for feedback, ACK/NACK feedback corresponding to this CC is not needed. If only one HARQ process is enabled for HARQ feedback in a certain CC, the UE is only required to feedback the information of ACK/NACK for the enabled HARQ process, i.e., 1bit feedback. It is quite desirable since Type-1 codebook size would be large compared to Type 2 codebook, and even this reduction, obviously there would be no ambiguity on codebook size between UE and gNB.

Proposal 4. For Type-1 HARQ-ACK codebook, codebook is generated by excluding HARQ-ACK for serving cells not having any feedback-enabled HARQ process.

Type-2 HARQ-ACK codebook

In the last meeting, it is agreed that the C-DAI and T-DAI are the count of only feedback-enabled processes for the DCI of PDSCH with feedback-enabled HARQ processes. The remaining issue is details of C-DAI and T-DAI counting for DCI of PDSCH with feedback-disabled HARQ processes. In this regard, several detailed alternatives on C-DAI and T-DAI counting for DCI of PDSCH with disabled HARQ processes were listed during the discussion as below

- Option-1: The C-DAI and T-DAI are the count of feedback-enabled processes, despite they are not incremented

- Option-2: The C-DAI and T-DAI are a reserved value that can be ignored by the UE

- Option-3: The C-DAI/T-DAI are the count of feedback-disabled processes

- Option-4: The C-DAI is a reserved value that can be ignored by the UE and T-DAI is the count of feedback-enabled processes despite it not incremented.

Among the alternatives, in order to ensure the reliability for HARQ feedback-disabled processes, it is preferred that the C-DAI and T-DAI are given their true values (i.e., the count of feedback-enabled processes), despite they are not incremented. If all of DCIs scheduling PDSCHs are with disabled HARQ processes or the first DCI scheduling PDSCH is associated with disabled HARQ process, there might be some issues such as 1) whether or not to read the C-DAI value in scheduling DCI, 2) what is the initial C-DAI value if UE reads the C-DAI in scheduling DCI, and 3) how to construct corresponding ACK/NACK payload and transmit ACK/NACK feedback. One solution can be to set initial C-DAI as 0, if all of DCIs scheduling PDSCHs are with disabled HARQ processes or the first DCI scheduling PDSCH is with disabled HARQ process.

Proposal 5. For Type-2 HARQ-ACK codebook, for the DCI of PDSCH with feedback-disabled HARQ processes, the C-DAI and T-DAI are the count of feedback-enabled processes, despite they are not incremented.

Type-3 HARQ-ACK codebook

Type-3 HARQ-ACK codebook was introduced for NR-U (unlicensed band), and it was further agreed to be supported in licensed bands. In Type-3 HARQ-ACK codebook, HARQ-ACK feedback for all HARQ processes is provided in one shot manner. Therefore, if Type-3 HARQ-ACK codebook is constructed based only on enabled HARQ processes, the overhead can be reduced effectively. Compared to Type-2 HARQ-ACK codebook, Type-3 HARQ-ACK codebook has a merit in terms of DCI overhead and no codebook ambiguity caused by DCI missing, while it requires larger UCI overhead. In that sense, support of enhancement on Type-3 HARQ-ACK codebook is beneficial when HARQ feedback is disabled.

Proposal 6. For Type-3 HARQ codebook, the codebook is generated based only on feedback-disabled HARQ processes.

SPS PDSCH activation/release

In NR, HARQ-ACK feedback is used to indicate whether the UE is correctly received SPS PDSCH activation/release command or not. However, if HARQ feedback is disabled, gNB cannot know that information, and thus it may cause redundant DL resource reservation or DL buffer corruption. To resolve these issues, if the first PDSCH after reception of the SPS activation DCI is with disabled HARQ process, UE feedbacks acknowledgement for the reception of SPS activation DCI. If the first PDSCH after reception of the SPS activation DCI is with enabled HARQ process, UE transmits ACK/NACK information according to the decoding results of that PDSCH. Similarly, for SPS PDSCH release, UE feedbacks acknowledgement for the reception of SPS release DCI even though HARQ feedback is disabled. As another solution, UE does not expect the SPS PDSCH configuration with any disabled HARQ process. However, it may restrict the gNB scheduling flexibility.

Proposal 7. UE feedbacks acknowledgement for the reception of SPS activation DCI, if the first PDSCH after reception of the SPS activation DCI is associated with disabled HARQ process.

- Conclusion

In this contribution, we discussed on the HARQ enhancements in NTN. Based on the above discussion, we have following proposals:

Proposal 1: For enhanced HARQ process id identification in NTN, support followings enhancement.

- For DCI format 0-1/1-1, increase the HARQ process ID field up to 5 bits

- For DCI format 0-0/1-0, support no enhancement

Proposal 2. For transmission enhancement when HARQ feedback is disabled, consider following enhancements:

- Repetition number is recommended/reported by UE

Proposal 3. In case of two PDSCHs are associated with different HARQ process ids, introduce default/virtual ACK/NACK timing or timing offset for feedback-disabled HARQ process.

Proposal 4. For Type-1 HARQ-ACK codebook, codebook is generated by excluding HARQ-ACK for serving cells not having any feedback-enabled HARQ process.

Proposal 5. For Type-2 HARQ-ACK codebook, for the DCI of PDSCH with feedback-disabled HARQ processes, the C-DAI and T-DAI are the count of feedback-enabled processes, despite they are not incremented.

Proposal 6. For Type-3 HARQ codebook, the codebook is generated based only on feedback-disabled HARQ processes.

Proposal 7. UE feedbacks acknowledgement for the reception of SPS activation DCI, if the first PDSCH after reception of the SPS activation DCI is associated with disabled HARQ process.