3GPP TSG RAN WG1 Meeting #106-e	    			                                R1-2107586

e-Meeting, August 16th – August 27th, 2021

Source:	Intel Corporation

Title:	Further details of intra-UE uplink channel multiplexing and prioritization

Agenda item:	8.3.3

Document for:	Discussion and Decision

Introduction

In RAN1-105-e, the following was agreed in connection to intra-UE multiplexing/prioritization:

Agreement: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is more than 2, - For HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s), support separate coding. Down-select from the two options: - Option 1: Reuse R15 TS 38.212 Clause 5.3.3.1 for 1-bit. Reuse R15 TS 38.212 Clause 5.3.3.2 for 2-bit. - Option 2: Reuse R15 TS 38.212 Clause 5.3.3.3, i.e., padding to 3 bits and using RM coding. - For HP HARQ-ACK or LP HARQ-ACK >2 bit(s), HP HARQ-ACK and LP HARQ-ACK are separately encoded according to R15 TS 38.212 Clause 5.3.3.3 or Clause 5.3.1. - FFS rate matching equation and RE mapping rules for PF2/3/4. Rel-15 is baseline if available. Agreement: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is 2, treat the two bits as HARQ-ACK bits with High priority. · Rel-15 design (for PF0 and PF1) is baseline. · Note: QC has strong concern on above scheme. The scheme cannot provide unequal error protection between the HP bit and LP bit hence could suffer from performance degradation for the HP bit. QC accept the scheme for the sake of progress in RAN 1 with the concern on the performance reserved.

In RAN1-104-bis-e, the following was agreed in relation to intra-UE multiplexing/prioritization:

Agreements: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is more than 2, support separate coding for the two HARQ-ACKs. - FFS for HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s). - (working assumption) Drop CSI (including part 1 and part2, if exist) if CSI would multiplex on a PUCCH which has HP A/N. - FFS Strive to let HP A/N reuse the encoder, rate matching equation, and RE mapping rules in Rel-15 for A/N+CSI-1. - FFS Strive to let LP A/N reuse the encoder, rate matching equation, and mapping rules in Rel-15 for CSI-2. Agreement: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, support separate coding for the two HARQ-ACKs. - It is understood that it is intended that the number of encoding chains for all UCI multiplexing combinations in Rel-17 should not exceed that in Rel-15/16.

In RAN1-104-e [2], the following was agreed in relation to intra-UE multiplexing/prioritization:

Agreements: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, - Use a PUCCH resource in the second PUCCH-Config (the PUCCH-config containing the PUCCH resource of the HP HARQ-ACK) at least in case the total number of LP and HP HARQ-ACK bits is more than 2. - FFS: The PUCCH resource is configured dedicated for multiplexing of HP HARQ-ACK and LP HARQ-ACK. - FFS in case the total number of LP and HP HARQ-ACK bits is 2. FFS details Working assumption: Reuse Rel-15 intra-UE PUCCH/PUSCH multiplexing timeline requirements for Rel-17 intra-UE PUCCH/PUSCH multiplexing with different priorities - FFS whether or not to specify a different behavior than Rel-15 when the timeline requirements are not met Agreements: When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, further study the following options (proponents are encouraged to provide more details and analysis):  Opt.1: The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource.  Opt.1a: The UE does not transmit negative SR.  Opt.1b: For negative SR, the UE transmit only HARQ-ACK on the HARQ-ACK resource.  Opt.1c: For negative SR, the UE transmits SR and HARQ-ACK on the SR resource  FFS: whether with power boost to transmit multiplexed payload or not.  Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource.  Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource.  Opt.2b: Using 4 CS values as for SR+1-bit HARQ-ACK in Rel-15/16. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit.  Opt.2c: If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.  Opt.3: No enhancement over Rel-16.  Other options not excluded.  FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK? Agreements: When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF1, further study the following options (proponents are encouraged to provide more details and analysis):  Opt.1: The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource.  Opt.1a: The UE does not transmit negative SR.  Opt.1b: For negative SR, the UE transmit only HARQ-ACK on the HARQ-ACK resource.  Opt.1c: For negative SR, the UE transmits SR and HARQ-ACK on the SR resource  FFS: whether with power boost to transmit multiplexed payload or not.  Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource.  Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource.  Opt.2b: Applying QPSK for SR+1-bit HARQ-ACK. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit.  FFS on conditions of multiplexing.  Opt.3: For positive SR, transmit HARQ-ACK on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource.  Opt.4: For positive SR, transmit SR on the SR resource and drop HARQ-ACK. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource.  Opt.5: No enhancement over Rel-16.  Other options not excluded.  FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK? Agreements: When a PUCCH carrying HP SR with PF1 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, further study the following options (proponents are encouraged to provide more details and analysis):  Opt.1: The SR and HARQ-ACK are multiplexed and transmitted on the SR resource.  Opt.1a: For positive SR, the UE transmits the PUCCH in the resource using PUCCH format 1 for SR. The value of cyclic shift of sequence, i.e., , of this PUCCH format 1 is determined by HARQ-ACK, and the bit, i.e., b(0), of this PUCCH format 1 is determined by SR. For negative SR, the UE transmits only a PUCCH with HARQ-ACK information and drops the PUCCH with negative SR.  Opt.1b: SR and HARQ-ACK are multiplexed and modulated to be transmitted on the SR resource  Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource.  Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource.  Opt.2b: Using 4 CS values as for SR+1-bit HARQ-ACK in Rel-15/16. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit.  Opt.2c: If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.  Opt.2d: HP SR and LP HARQ-ACK are multiplexed by the Rel-15 cyclic shift only if latency requirement for HP SR is met. Otherwise, drop the LP HARQ-ACK and only transmit the HP SR on its resource.  Opt.3: For positive SR, transmit HARQ-ACK on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource.  Opt.4: No enhancement over Rel-16.  Other options not excluded.  FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK? Agreements: For multiplexing LP HARQ-ACK in a HP PUSCH, support 0< beta-offset <1. - FFS value(s) - FFS to additionally support beta-offset =0 or a value disabling the multiplexing - Aim to NOT increase the corresponding bitwidth in the DCI (compared to Rel-16) Agreements: Per UE with the capability of inter-band CA, simultaneous PUCCH/PUSCH transmission of different PHY priorities over different cells can be RRC configured within the same PUCCH group - FFS: dynamic indication

In this contribution, we express our views on Intra-UE prioritization and multiplexing based on the agreements reached in last meeting.

# DG and CG PUSCH collision of different priorities

In this section, we discuss collision handling when resources of DG and CG PUSCH of different priorities overlap.

## Collision of Low priority DG PUSCH and high priority CG PUSCH

It has been agreed in RAN1 # 102-e that PHY prioritization for the case where low-priority DG-PUSCH collides with high-priority CG-PUSCH is supported in Rel-17. In our view, the cancelation timelines we have agreed so far may not apply for this case since the triggering of the transmission of the HP CG PUSCH would be dependent on when the MAC delivers the corresponding MAC PDU to PHY for the HP CG PUSCH. The UE may be expected to cancel the LP DG PUSCH at least starting from the first overlapping symbol with the HP CG PUSCH as long as the MAC PDU for the HP CG PUSCH is received Tproc,2+d1 before the first overlapping symbol.

While the above could be specified in PHY specifications, it may not be testable.

Observation 1: It may not be feasible to define a proper cancellation timeline that is testable since it may not be feasible to externally determine the exact timing when the MAC layer delivers the corresponding MAC PDU to PHY for the HP CG PUSCH.

Proposal 1: UE is expected to transmit the CG PUSCH and cancel the overlapping DG PUSCH at the latest from the first symbol that is overlapping with the CG PUSCH when collision between HP CG PUSCH and LP DG PUSCH occurs.

- Sufficient to capture the above in RAN1 specification.

## Collision of Low priority CG PUSCH and high priority DG PUSCH

In RAN1 103e, PHY prioritization of overlapping high-priority dynamic grant PUSCH and low-priority configured grant PUSCH on a BWP of a serving cell was agreed. In our view, consideration should be limited to overlap of two channels in Rel-17 since this is the most common and expected use case.

Regarding timeline, the end of the PDCCH carrying the UL grant can be used as the cancelation triggering point. Thus, as long as the Rel-16 timeline for the time between end of the PDCCH with the UL grant and start of the DG PUSCH (HP) is at least Tproc,2 +min (d1, d2), PHY prioritization can be performed.

Proposal 2.  Define a new UE capability for collision handling between the LP CG and HP DG PUSCH in PHY layer.

- If UE supports the capability, the UE is expected to cancel the overlapping low priority CG PUSCH by the first overlapping symbol at the latest. Further, the UE expects that the first symbol of the high priority DG PUSCH is not earlier than Tproc,2+min(d1,d2) after the last symbol of the PDCCH with the DCI format scheduling the high priority DG PUSCH, where d1 and d2 can be from {0, 1, 2} symbols, and correspond to the additional margins for cancelation and preparation times respectively in case of intra-UE prioritization and reported as UE capability.

- Otherwise, the UE can only cancel the entire PUSCH transmission corresponding to the configured grant starting in a symbol 𝑗, if the end of symbol 𝑖 for PDCCH scheduling the PUSCH is at least Tproc,2 before the beginning of symbol 𝑗.

Since both PDUs are forwarded to PHY, PHY is handling the prioritization and canceling a low priority transmission. Hence, we do not see an issue or impact for Rel-17 behaviors depending on whether UL skipping is enabled or not.

# HARQ-ACK and HARQ-ACK collisions of different priorities

In RAN1 #104e it was agreed that if LP and HP HARQ-ACKs are to be multiplexed, resultant PUCCH resource would be based on PUCCH configuration of HP HARQ-ACK codebook, i.e., only PUCCH resource IDs associated with PUCCH-config of HP codebook are considered for multiplexing, at least when total HARQ-ACK payload is more than 2 bits, so that multiplexed HARQ-ACKs are not delayed. For example, if LP HARQ-ACK is slot based and HP HARQ-ACK is sub-slot based, multiplexed HARQ-ACKs would be transmitted in a sub-slot based resource.  In RAN1 #104bis-e, separate coding was agreed to be supported for multiplexing of LP HARQ-ACK and HP HARQ-ACK bits, when combined payload is more than 2 bits. Below, we discuss the remaining aspects of multiplexing LP HARQ-ACK and HP HARQ-ACK bits.

In RAN1-105e, it was agreed that for multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is 2, the two bits would be treated as HARQ-ACK bits with high priority. It is reasonable to assume a unified solution for PUCCH resource selection when total payload is 2 bits, i.e., we suggest using a PUCCH resource in the second PUCCH-Config for multiplexing HARQ-ACKs of low and high priority when payload is 2 bits.

Proposal 3: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, in case the total number of LP and HP HARQ-ACK bits is 2:

- Use a PUCCH resource in the second PUCCH-Config (the PUCCH-config containing the PUCCH resource of the HP HARQ-ACK).

## Separate Coding Procedure

For total HARQ-ACK payload of more than 2 bits, it was agreed in RAN1-105e that for HP HARQ-ACK or LP HARQ-ACK >2 bit(s), HP HARQ-ACK and LP HARQ-ACK are separately encoded according to R15 procedure in TS 38.212 Clause 5.3.3.3 or Clause 5.3.1. For HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s), we also suggest reusing R15 procedures of 1-2 UCI bits encoding methods, i.e., repetition code/simplex code. We do not see strong need for padding to 3 bits and use the Rel-16 coding method of Type-2 CSI report, i.e., padding to 3 bits and use RM coding, and increase specification efforts in this regard.

Proposal 4: For HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s), support separate coding by reusing R15 TS 38.212 Clause 5.3.3.1 for 1-bit. Reuse R15 TS 38.212 Clause 5.3.3.2 for 2-bit.

Following Rel-15 procedure, separate encoding of LP and HP HARQ-ACK bits can be achieved if HP HARQ-ACK uses an encoder similar way as HARQ-ACK and LP HARQ-ACK uses an encoder similar way when CSI part 2 is multiplexed with HARQ-ACK in Rel-15. However, in Rel-15, a UE is configured by maxCodeRate a code rate for multiplexing HARQ-ACK, SR, and CSI report(s) in a PUCCH transmission using PUCCH format 2, PUCCH format 3, or PUCCH format 4. For a UE following Rel-17, following scenarios may occur:

- HARQ-ACK with or without multiplexing with SR is transmitted in a PUCCH, where both HARQ-ACK and SR is priority index 1.

- HARQ-ACK with or without SR and CSI reports (if present) are multiplexed and transmitted in a PUCCH, where both HARQ-ACK and SR is priority index 0.

- HARQ-ACK with or without multiplexing with SR, where both HARQ-ACK and SR is of priority index 1, and HARQ-ACK of priority index 0 are separately encoded and multiplexed onto a PUCCH.

- HARQ-ACK with or without multiplexing with SR, where both HARQ-ACK and SR is of priority index 0, and HARQ-ACK of priority index 1 are separately encoded and multiplexed onto a PUCCH.

A single value of maxCodeRate per PUCCH format may not result in resource efficient multiplexing for the above scenarios. Since the reliability of HARQ-ACK with/without SR of priority index 1 needs to be protected, appropriate amount of resource may need to be budgeted, e.g., by choosing a low maxCodeRate for the UE along with proper selection of PUCCH resource. On the other hand, always choosing a conservative maxCodeRate may cause HARQ-ACK bits of priority index 0 to be dropped (partially or fully) on most occasions, since it is expected that payload of HARQ-ACK bits of priority index 0 is larger than that of HARQ-ACK bits of priority index 1.

Observation 2: A single value of maxCodeRate per PUCCH format may not result in resource efficient multiplexing for different UCI multiplexing scenarios in Rel-17 and may cause excessive dropping of LP HARQ-ACK bits.

To facilitate UCI multiplexing of same or different priorities into a PUCCH in Rel-17, a pair of maxCodeRate values per PUCCH format can be configured to the UE. UE may report certain values via capability signaling.

Proposal 5: A pair of maxCodeRate values per PUCCH format can be configured to UE for UCI multiplexing of different priorities into PUCCH.

In Rel-15, separate coding is only supported for PUCCH format 3 and 4. Only joint coding is supported for PUCCH format 2. Hence, extending support of separate coding to PUCCH format 2 may require more specification efforts, such as RE mapping pattern for HP and LP HARQ-ACK bits. For PUCCH format 3 and 4, Rel-15 mechanisms for RE mapping when CSI part 2 is multiplexed can be considered.

Proposal 6: At least PUCCH format 3 and 4 are supported for LP and HP HARQ-ACK multiplexing

- FFS: PUCCH format 2

Moreover, PUCCH resource determination depends on UCI payload. In RAN1-105e, the following two options were discussed regarding combined UCI payload:

- Option 1: UCI payload size = the number of HP UCI bits + the number of LP UCI bits

- Option 2: UCI payload size = the number of HP UCI bits + the number of LP UCI bits * scaling factor.

We are not convinced that scaling factor for LP UCI bits is needed. The code rate is obtained during the encoding stage and we can have different code rate for HP UCI and LP UCI. It is not clear why the scaling done at the information bits level. Moreover, directly using the number of HP UCI bits + LP UCI bits to determine the PUCCH resource set would not cause resource waste, as after the encoding according to the real coding rate for HP UCI and LP UCI, the unused PUCCH resources will be automatically released for other UE’s usage since the UE would only use a required number of RBs within the resource.

Based on the above, UE would determine PUCCH resource with PUCCH format 3 or 4 when UE would multiplex HP and LP HARQ-ACK bits with corresponding maxCodeRates rc1 and rc0, respectively, as

- UE determines the PUCCH resource using the PUCCH resource indicator field in the DCI that triggered the HP HARQ-ACK from a PUCCH resource set

- UE determines the PUCCH resource set based on UCI payload OACK, 1+OACK, 0, i.e., the number of HP UCI bits + the number of LP UCI bits

- UE determines/selects minimum number MRB,minPUCCH of PRBs from MRBPUCCH PRBs that satisfy

(OACK, 1+ OCRC,1)/rc1+(OACK, 0+ OCRC,0)/rc0≤MRB,minPUCCH∙Nsc,ctrlRB∙Nsymb_UCIPUCCH∙Qm

where, MRBPUCCH is the total number of PRBs, Nsc,ctrlRB indicates number of sub-carriers used per PRB, Nsymb_UCIPUCCH indicates number of PUCCH symbols, and Qm is the modulation order.

Proposal 7: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, support the following for determining the UCI payload size for PUCCH resource determination:

- UCI payload size = the number of HP UCI bits + the number of LP UCI bits

## LP HARQ-ACK Payload Control

If sufficient resource is not available for accommodating LP HARQ-ACK payload bits, LP HARQ-ACK bits can be dropped. However, full dropping may severely impact throughput and resource efficiency. Hence, LP HARQ-ACK payload control can be considered. In our view, compression may not work well when LP HARQ-ACK payload tends to increase. Partial dropping is preferrable since gNB would have a clear idea on which bits are actually transmitted and their corresponding status. Depending on the compression factor, gNB maybe unaware of bit status for a significant number of LP HARQ-ACK bits. For example, partial dropping can be at the CBG level or TB-level (e.g., if CA is used). If gNB thinks UCI multiplexing with or without LP HARQ-ACK payload reduction is not suitable, it can always enable/disable multiplexing dynamically, e.g., by the DCI triggering HP HARQ-ACK.

Proposal 8: LP HARQ-ACK payload bits can be partially dropped

- FFS: How to partition LP HARQ-ACK payload bits

Moreover, it was agreed in RAN1 #102e that for multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, a mechanism for gNB to enable/disable the multiplexing is supported. In our view, a dynamic indication in DCI to enable or disable multiplexing is more flexible and allow for scenario specific handling of different overlaps. DCI triggering the HARQ-ACK may include an indication. Moreover, we think the enable/disable trigger may not only be applicable for LP and HP HARQ-ACK multiplexing but also for handling overlap between HARQ-ACK and SR of different priorities.

Proposal 9: DCI triggering HARQ-ACK may include an indication for enabling or disabling multiplexing.

- The indication may be applicable to both HARQ-ACK/HARQ-ACK and HARQ-ACK/SR multiplexing.

## Issue of miss-detection of DCI triggering LP HARQ-ACK

DCI triggering LP HARQ-ACK may be less reliable than the DCI triggering HP HARQ-ACK. Hence, chance of missed detection of the DCI corresponding to LP HARQ-ACK is higher, e.g., 1%, compared to that (e.g., 0.001%) of DCI triggering HP HARQ-ACK. If dynamic or type 2 codebook is used and DAI field only includes C-DAI bits, the problem of ambiguity due to missed detection of DCI on LP HARQ-ACK codebook size could impact the reliability of HP HARQ-ACK transmission, since assumption on number of LP HARQ-ACK bits, PUCCH resource and/or rate matching determination can be different between gNB and UE. To this end, we think a set of reference sizes can be identified for LP HARQ-ACK codebook sizes and DCI triggering HP HARQ-ACK may include an identifier regarding the reference size to be assumed for LP HARQ-ACK codebook size. Only semi-static reservation of a certain reference size for LP type 2 codebook may not be efficient.

Proposal 10: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, combination of the following options can be considered to resolve the ambiguity on LP HARQ-ACK type-2 codebook size due to DCI miss-detection

- A set of reference sizes are configured for LP type 2 codebook

- Additional DCI field in the DCI corresponding to HP HARQ-ACK includes an identifier regarding the reference size for LP type codebook to assume for multiplexing.

# HARQ-ACK and PUSCH collisions of different priorities

In RAN1 102-e, separate configurations of beta-offset values for multiplexing with different priority combinations were agreed, such as for the following

- Multiplexing a low-priority HARQ-ACK in a high-priority PUSCH (conveying UL-SCH only).

- Multiplexing a high-priority HARQ-ACK in a low-priority PUSCH (conveying UL-SCH only)

- Multiplexing a low-priority HARQ-ACK, a high-priority PUSCH conveying UL-SCH, a high-priority HARQ-ACK and/or CSI.

- Multiplexing a high-priority HARQ-ACK, a low-priority PUSCH conveying UL-SCH, a low-priority HARQ-ACK and/or CSI.

For more flexible multiplexing and protecting reliability of HP HARQ-ACK or HP PUSCH, separate beta offset configurations can be used. If the UL grant includes both priority indicator and beta offset indicator field, UE identifies the correct set of beta offset values for indication in the beta offset indicator field depending on the priority indicated. Moreover, this may also depend on the priority of the HARQ-ACK being multiplexed, such as whether HP and/or LP HARQ-ACK are multiplexed. Separate encoding and beta offset values should be used for HP and LP HARQ-ACKs if they multiplexed onto the same PUSCH. In the following figure 3, beta offset selection based on different combinations are illustrated:

Figure 3: Beta offset selection for different priority combinations.

For payload control, LP HARQ-ACK bits can be partitioned, such as Part 1 and Part 2, where Part 2 can be dropped if sufficient resource is not available. The procedure can be similar to CSI Part 1 and Part 2 handling. Dropped LP HARQ-ACK bits can be retransmitted.

Proposal 11: Separate encoding is used for multiplexing LP and HP HARQ-ACK bits onto the PUSCH.

- Separate beta offset configuration can be used depending on the combination of PUSCH and HARQ-ACK to be multiplexed

- LP HARQ-ACK payload bits can be partially dropped if needed.

In our view, following combinations are possible regarding HARQ-ACK multiplexing onto PUSCH in R17:

    - Multiplexing LP HARQ-ACK/UCI on LP PUSCH

    - Multiplexing LP HARQ-ACK/UCI on HP PUSCH

    - Multiplexing HP HARQ-ACK/UCI on LP PUSCH

    - Multiplexing HP HARQ-ACK/UCI on HP PUSCH

    - Multiplexing LP HARQ-ACK/UCI, HP HARQ-ACK/UCI on LP PUSCH

    - Multiplexing HP HARQ-ACK/UCI, LP HARQ-ACK/UCI on HP PUSCH

In a straightforward approach, 4 sets of beta-offset configuration can be provided to the UE for the Scenario 1 to 4. Whether beta offset configuration of scenario 1 to 4 can be directly applied to the multiplexing scenario or some reordering of the entries of the sets would be useful needs further discussion. This is because the amount and/or % of resource allocation, i.e., potentially selection of beta offset, for multiplexing LP HARQ-ACK onto LP PUSCH (e.g., in Scenario 5) could change depending on whether there is also HP HARQ-ACK to multiplex onto the same LP PUSCH or not. In other words, mapping of codepoint in the beta offset indicator field to the entries (e.g., four indices or beta-offset values if 2-bit field is used) in the set of beta offsets can be different depending on the multiplexing scenario.

Proposal 12:

- Support 4 sets of beta-offset configuration for the following scenarios

    - Multiplexing LP HARQ-ACK/UCI on LP PUSCH

    - Multiplexing LP HARQ-ACK/UCI on HP PUSCH

    - Multiplexing HP HARQ-ACK/UCI on LP PUSCH

    - Multiplexing HP HARQ-ACK/UCI on HP PUSCH

- FFS: whether and how to apply the above beta offset configurations for the following multiplexing scenario

    - Multiplexing LP HARQ-ACK/UCI, HP HARQ-ACK/UCI on LP PUSCH

    - Multiplexing HP HARQ-ACK/UCI, LP HARQ-ACK/UCI on HP PUSCH

On the other hand, CG-PUSCH may include CG-UCI. However, in NR-U design, no priority of CG-UCI was considered with respect to HARQ-ACK and CG-UCI and HARQ-ACK are jointly encoded with same beta offset. In our view, CG-UCI contains important information such as HARQ-ID, etc., and is critical to the performance of CG-PUSCH in NR-U setup. Hence, CG-UCI should be considered as high priority and can be multiplexed in a similar manner as HP HARQ-ACK onto PUSCH. If HP and LP HARQ-ACKs are to be multiplexed onto CG-PUSCH which also includes CG-UCI, CG-UCI can be jointly encoded with HP HARQ-ACK with same beta offset.

Proposal 13: CG-UCI is regarded as high priority and can be multiplexed in a similar manner as HP HARQ-ACK onto PUSCH.

Proposal 14: If both HP and LP HARQ-ACK are to be multiplexed onto CG-PUSCH that includes CG-UCI, CG-UCI is jointly encoded with HP HARQ-ACK with same beta offset.

Moreover, for HARQ-ACK multiplexing on PUSCH of different priority in R17, it was agreed in RAN1 103e that a mechanism for gNB is supported to enable/disable the multiplexing. In our view, for DG PUSCH scheduling DCI may include an indication to indicate whether multiplexing of UCI is enabled or not. For CG PUSCH, a higher layer parameter can be provided to enable/disable multiplexing.

Proposal 15: DCI and higher layer indication can be provided to enable multiplexing of UCI onto DG PUSCH and CG PUSCH, respectively.

# SR, HARQ-ACK and/or CSI collisions of different priorities

In last meeting, several options were listed for multiplexing a low-priority HARQ-ACK and a high-priority SR into a PUCCH. In our view, HP HARQ-ACK (HP SR) and LP SR (LP HARQ-ACK) can be multiplexed onto a PUCCH resource only if the PUCCH resource carrying the multiplexed HARQ-ACKs and SR ends no later than the last symbol of the PUCCH resource of HP HARQ-ACK or HP SR. We have the following preference for HP SR (HP HARQ-ACK) and LP HARQ-ACK (LP SR) multiplexing for different PF combinations:

Proposal 16:

HP SR PF0, LP HARQ PF0:

- If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.

HP SR PF0, LP HARQ PF1:

- For positive SR, transmit SR on the SR resource and drop HARQ-ACK. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource

HP SR PF1, LP HARQ PF0:

- If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.

Collision handling LP SR and HP HARQ-ACKs

|  | HARQ-ACK with PF0 | HARQ-ACK with PF1 | HARQ-ACK with PF2 | HARQ-ACK with PF3 or PF4 |
| --- | --- | --- | --- | --- |
| SR with PF0 | Multiplexed UCI is transmitted using PF0 on HARQ-ACK resource | Drop SR and transmit HARQ-ACK on HARQ-ACK resource | Multiplexed UCI is transmitted using PF 2 on HARQ-ACK resource if SR is with PF 0. SR is dropped if it is PF 1 | Multiplex HARQ-ACK and SR according to Rel-15 procedure. |
| SR with PF1 | SR is dropped | Multiplexed UCI is transmitted using PF 1 on HARQ-ACK resource |  |  |

On the other hand, in Rel-16, two-step approach was adopted when channels of different priorities overlap. UE would resolve collision of same priority first and then handle collision between the outcome of resolution in first step and channel of different priority. However, if UE supports intra-UE multiplexing across different priorities, UE may jointly consider multiplexing the channels of different priorities whenever applicable and two step approach may not be needed. Such as in the following Figure 4, where overlap of HP SR, HP HARQ-ACK, and LP HARQ-ACK are shown and this scenario was agreed to be supported in RAN1-102e. UE could consider all the overlapping UCIs together for multiplexing onto a resultant PUCCH resource, if timeline and resource constraints are satisfied, whereas according to Rel-16 two-step approach, UE would find PUCCH resource for multiplexing HP SR and HARQ-ACKs first.

Figure 4: Resources of HP HARQ-ACK, HP SR, and LP HARQ-ACK are overlapping in a slot.

Proposal 17: Instead of two-step approach, consider a single checking step among all channels for  multiplexing of UCIs of different priorities into a PUCCH resource if UE supports intra-UE multiplexing across different priorities.

Timeline conditions

In Rel-15, UE does not expect different UCIs overlap when timeline condition is not satisfied. In Rel-17, this can be allowed to occur for greater flexibility in UCI scheduling and UE may just drop the low priority UCI and transmit the high priority UCI.

Proposal 18: When UCIs of different priorities overlap and if at least one is based on a DCI, UE may drop the low priority UCI and transmit the high priority UCI, when timeline conditions are not satisfied.

# PUCCH (PUSCH) overlapping with multiple PUSCHs (PUCCHs)

In Rel16, UE does not expect a PUSCH to be scheduled overlapping
with two sub-slot based PUCCHs in a slot, such as shown in Figure 5. However, this may be unavoidable in some cases, such as PUSCH or PUCCH may be high priority and need to be transmitted as soon as possible. In Rel17, it can be considered that UCIs from both PUCCHs are multiplexed onto the PUSCH if timeline conditions are met. Alternatively, UE could apply Rel16 prioritization rule such as UE multiplexes UCI from PUCCH onto PUSCH if they are of same priority and if there is another PUCCH that is of different priority, UE only transmits the channel with high priority and drops the channel with low priority.

Figure 5. A PUSCH is overlapping with two sub-slot based PUCCHs in a slot.

Proposal 19: If a PUSCH overlaps with two sub-slot based PUCCHs, multiplex the UCIs from the PUCCHs onto the PUSCH if timeline conditions are met. If timeline conditions are not met, drop the low priority channel and transmit the high priority channel.

- FFS: whether to apply Rel16 intra-UE prioritization in this case.

Moreover, if a PUCCH overlaps with more than one PUSCHs in a slot, Rel15 specified that UCI from PUCCH is multiplexed onto the first PUSCH in the slot, that has starting symbol earliest in the slot, cf. Figure 6. In Rel17, a more flexible approach can be considered. UCI can be multiplexed onto either first or second PUSCH, depending on the priority of the involved channels. If PUCCH is of high priority, PUCCH is multiplexed onto first PUSCH if timeline conditions are met, and second PUSCH is transmitted as usual. If first PUSCH is high priority and second PUSCH is low priority, UCI from PUCCH is multiplexed onto second PUSCH if the PUCCH is of low priority.

Figure 6: Rel15 solutions: UCI is multiplexed onto the earliest PUSCH (first PUSCH) in the slot. Second PUSCH is the one with starting symbol after first PUSCH.

Proposal 20: If a PUCCH overlaps with two PUSCHs, following behaviors can be considered, assuming timeline conditions are met:

- If PUCCH is of high priority, PUCCH is multiplexed onto first PUSCH.

- If first (second) PUSCH is of high (low) priority, UCI from PUCCH is multiplexed onto second PUSCH if the PUCCH is of low priority.

# Simultaneous transmission of PUCCH and PUSCH over x-CCs

In Rel-15, for UL carrier aggregation, when UL control channel (PUCCH) overlaps with a UL data channel (PUSCH) on a different carrier, the PUCCH is multiplexed on the PUSCH. This may not always be desirable, such as PUCCH duration can be long and PUSCH duration can be short, and consequently coverage of PUCCH can be impacted. Moreover, if both PUCCH and PUSCHs are of high priority, e.g., both requiring high reliability and/or low latency, multiplexing them in a carrier may not beneficial for PUCCH reliability in some occasions, since a limited resource within the PUSCH can be allocated for piggybacking UL control information (UCI). 
To this end, in RAN1 102-e, it was agreed that simultaneous PUCCH and PUSCH transmission over multiple carriers is supported at least for inter-band CA. This feature may also facilitate reduced dropping of low priority transmission, in the event of overlap of low and high priority transmission across multiple carriers. In RAN1-104e, it was agreed that with simultaneous PUCCH/PUSCH transmission of different PHY priorities over different cells can be RRC configured within the same PUCCH group for inter-band CA.

In our view, if UE is configured with both simultaneous PUSCH and PUCCH transmissions over different carriers and Rel16 or Rel17 intra-UE prioritization, option of simultaneous transmissions should take precedence over the intra-UE prioritization. On the other hand, UE may support the feature for intra-band CA based on capability signaling, the scope may be limited such as simultaneous transmissions may only be possible when their durations are aligned so that phase distortions are avoided.

Proposal 21: If a PUSCH overlaps with a PUCCH repetition in a slot, multiplex the UCI onto the PUSCH and drop the PUCCH repetition.

- FFS whether this is only applicable if PUSCH is of high priority and/or PUCCH is of low priority.

Proposal 22: If UE is configured with both simultaneous PUSCH and PUCCH transmissions over different carriers and Rel16 or Rel17 intra-UE prioritization, option of simultaneous transmissions should take precedence over the intra-UE prioritization.

Observation 3: Although UE may support simultaneous transmission over different carriers for intra-band CA based on capability signaling, the scope may be limited such as simultaneous transmissions may only be possible when their durations are aligned.

Proposal 23: Discussion on support of simultaneous transmission of PUSCH and PUCCH over different carriers for intra-band CA is deprioritized.

Conclusions

In summary, we have the following list of proposals and observations:

Observation 1: It may not be feasible to define a proper cancellation timeline that is testable since it may not be feasible to externally determine the exact timing when the MAC layer delivers the corresponding MAC PDU to PHY for the HP CG PUSCH.

Observation 2: A single value of maxCodeRate per PUCCH format may not result in resource efficient multiplexing for different UCI multiplexing scenarios in Rel-17 and may cause excessive dropping of LP HARQ-ACK bits.

Observation 3: Although UE may support simultaneous transmission over different carriers for intra-band CA based on capability signaling, the scope may be limited such as simultaneous transmissions may only be possible when their durations are aligned.

Proposal 1: UE is expected to transmit the CG PUSCH and cancel the overlapping DG PUSCH at the latest from the first symbol that is overlapping with the CG PUSCH when collision between HP CG PUSCH and LP DG PUSCH occurs.

- Sufficient to capture the above in RAN1 specification.

Proposal 2.  Define a new UE capability for collision handling between the LP CG and HP DG PUSCH in PHY layer.

- If UE supports the capability, the UE is expected to cancel the overlapping low priority CG PUSCH by the first overlapping symbol at the latest. Further, the UE expects that the first symbol of the high priority DG PUSCH is not earlier than Tproc,2+min(d1,d2) after the last symbol of the PDCCH with the DCI format scheduling the high priority DG PUSCH, where d1 and d2 can be from {0, 1, 2} symbols, and correspond to the additional margins for cancelation and preparation times respectively in case of intra-UE prioritization and reported as UE capability.

- Otherwise, the UE can only cancel the entire PUSCH transmission corresponding to the configured grant starting in a symbol 𝑗, if the end of symbol 𝑖 for PDCCH scheduling the PUSCH is at least Tproc,2 before the beginning of symbol 𝑗.

Proposal 3: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, in case the total number of LP and HP HARQ-ACK bits is 2:

- Use a PUCCH resource in the second PUCCH-Config (the PUCCH-config containing the PUCCH resource of the HP HARQ-ACK).

Proposal 4: For HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s), support separate coding by reusing R15 TS 38.212 Clause 5.3.3.1 for 1-bit. Reuse R15 TS 38.212 Clause 5.3.3.2 for 2-bit.

Proposal 5: A pair of maxCodeRate values per PUCCH format can be configured to UE for UCI multiplexing of different priorities into PUCCH.

Proposal 6: At least PUCCH format 3 and 4 are supported for LP and HP HARQ-ACK multiplexing

- FFS: PUCCH format 2

Proposal 7: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, support the following for determining the UCI payload size for PUCCH resource determination:

- UCI payload size = the number of HP UCI bits + the number of LP UCI bits

Proposal 8: LP HARQ-ACK payload bits can be partially dropped

- FFS: How to partition LP HARQ-ACK payload bits

Proposal 9: DCI triggering HARQ-ACK may include an indication for enabling or disabling multiplexing.

- The indication may be applicable to both HARQ-ACK/HARQ-ACK and HARQ-ACK/SR multiplexing.

Proposal 10: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, combination of the following options can be considered to resolve the ambiguity on LP HARQ-ACK type-2 codebook size due to DCI miss-detection

- A set of reference sizes are configured for LP type 2 codebook

- Additional DCI field in the DCI corresponding to HP HARQ-ACK includes an identifier regarding the reference size for LP type codebook to assume for multiplexing.

Proposal 11: Separate encoding is used for multiplexing LP and HP HARQ-ACK bits onto the PUSCH.

- Separate beta offset configuration can be used depending on the combination of PUSCH and HARQ-ACK to be multiplexed

- LP HARQ-ACK payload bits can be partially dropped if needed.

Proposal 12:

- Support 4 sets of beta-offset configuration for the following scenarios

    - Multiplexing LP HARQ-ACK/UCI on LP PUSCH

    - Multiplexing LP HARQ-ACK/UCI on HP PUSCH

    - Multiplexing HP HARQ-ACK/UCI on LP PUSCH

    - Multiplexing HP HARQ-ACK/UCI on HP PUSCH

- FFS: whether and how to apply the above beta offset configurations for the following multiplexing scenario

    - Multiplexing LP HARQ-ACK/UCI, HP HARQ-ACK/UCI on LP PUSCH

    - Multiplexing HP HARQ-ACK/UCI, LP HARQ-ACK/UCI on HP PUSCH

Proposal 13: CG-UCI is regarded as high priority and can be multiplexed in a similar manner as HP HARQ-ACK onto PUSCH.

Proposal 14: If both HP and LP HARQ-ACK are to be multiplexed onto CG-PUSCH that includes CG-UCI, CG-UCI is jointly encoded with HP HARQ-ACK with same beta offset.

Proposal 15: DCI and higher layer indication can be provided to enable multiplexing of UCI onto DG PUSCH and CG PUSCH, respectively.

Proposal 16:

HP SR PF0, LP HARQ PF0:

- If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.

HP SR PF0, LP HARQ PF1:

- For positive SR, transmit SR on the SR resource and drop HARQ-ACK. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource

HP SR PF1, LP HARQ PF0:

- If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource.

Collision handling LP SR and HP HARQ-ACKs

|  | HARQ-ACK with PF0 | HARQ-ACK with PF1 | HARQ-ACK with PF2 | HARQ-ACK with PF3 or PF4 |
| --- | --- | --- | --- | --- |
| SR with PF0 | Multiplexed UCI is transmitted using PF0 on HARQ-ACK resource | Drop SR and transmit HARQ-ACK on HARQ-ACK resource | Multiplexed UCI is transmitted using PF 2 on HARQ-ACK resource if SR is with PF 0. SR is dropped if it is PF 1 | Multiplex HARQ-ACK and SR according to Rel-15 procedure. |
| SR with PF1 | SR is dropped | Multiplexed UCI is transmitted using PF 1 on HARQ-ACK resource |  |  |

Proposal 17: Instead of two-step approach, consider a single checking step among all channels for multiplexing of UCIs of different priorities into a PUCCH resource if UE supports intra-UE multiplexing across different priorities.

Proposal 18: When UCIs of different priorities overlap and if at least one is based on a DCI, UE may drop the low priority UCI and transmit the high priority UCI, when timeline conditions are not satisfied.

Proposal 19: If a PUSCH overlaps with two sub-slot based PUCCHs, multiplex the UCIs from the PUCCHs onto the PUSCH if timeline conditions are met. If timeline conditions are not met, drop the low priority channel and transmit the high priority channel.

- FFS: whether to apply Rel16 intra-UE prioritization in this case.

Proposal 20: If a PUCCH overlaps with two PUSCHs, following behaviors can be considered, assuming timeline conditions are met:

- If PUCCH is of high priority, PUCCH is multiplexed onto first PUSCH.

- If first (second) PUSCH is of high (low) priority, UCI from PUCCH is multiplexed onto second PUSCH if the PUCCH is of low priority.

Proposal 21: If a PUSCH overlaps with a PUCCH repetition in a slot, multiplex the UCI onto the PUSCH and drop the PUCCH repetition.

- FFS whether this is only applicable if PUSCH is of high priority and/or PUCCH is of low priority.

Proposal 22: If UE is configured with both simultaneous PUSCH and PUCCH transmissions over different carriers and Rel16 or Rel17 intra-UE prioritization, option of simultaneous transmissions should take precedence over the intra-UE prioritization.

Proposal 23: Discussion on support of simultaneous transmission of PUSCH and PUCCH over different carriers for intra-band CA is deprioritized.

References

- 3GPP RAN1# 105-e Chairman’s Notes.

- 3GPP RAN1# 104bis-e Chairman’s Notes.

- 3GPP RAN1# 104e Chairman’s Notes.