3GPP TSG RAN WG1 #106-e	R1-2107608

e-Meeting, August 16th – 27th, 2021

Agenda item:	8.10.2

Source: 	Intel Corporation

Title: 	Other Enhancements for Simultaneous Operations

Document for:	Discussion and decision

# Introduction

Regarding timing alignment and synchronization, RAN1#104-105 has made the following agreements.

- Case 6 timing mode operation at an IAB-node is controlled by the parent node to which the UL transmission is intended for.

- Case 7 timing is supported with symbol level alignment without explicit support for slot level alignment

- Switching between Case 1, Case 6, and Case 7 timing is supported.

  - FFS whether Case 6 and Case 7 timing shall be restricted to certain resources, e.g. excluding resources used for access or TDM backhaul

  - FFS details on switching including the switching conditions

  - FFS relationship between switching timing modes with the usage/indication of different resource multiplexing modes

  - FFS whether Rel-16 OTA synchronization shall be enhanced to support switching timing modes

- RAN1 to downselect how the IAB-MT Tx timing is set for Case 6 timing at a given IAB-node:

- Alt1: the IAB-MT Tx timing is obtained by the node via the legacy TA loop plus an offset from the parent node.

    - FFS details of the required offset.

- Alt2: the IAB-MT Tx timing is set by the node to the timing obtained for the node’s DL Tx.

- Alt3: the IAB-MT Tx timing is obtained by the node jointly with the IAB-DU Tx timing via a common offset from the parent node.

Downselection to consider at least the following aspects:

- Dependency of DL synchronization schemes at the IAB-DU

- Potential additional signaling overhead.

- Achievable DU Tx / MT Tx alignment error tolerance.

- Suitability for switching between timing modes.

- RAN1 to downselect how the IAB-MT Tx timing is set at an IAB-node for Case 7 timing at the parent node:

- Alt1: the IAB-MT Tx timing of the node is obtained via the legacy TA loop plus an offset from the parent node.

  - FFS details of the required offset

- Alt2: the IAB-MT Tx timing of the node is obtained via the legacy TA loop from the parent node.

- Alt3: the IAB-MT Tx timing of the node is obtained via a Case 7 specific TA loop from the parent node.

Downselection to consider at least the following aspects:

- Potential impact to OTA synchronization availability for DU Tx at the IAB-node.

- Potential additional signaling overhead.

- Suitability for switching between timing modes.

- An IAB-node is indicated when Case 6 timing is performed at the IAB-node.

- FFS details of the indication (e.g. semi-static and/or dynamic, implicit and/or explicit, linkage to multiplexing capability, etc.).

FFS whether an IAB-node is also indicated when Case 7 timing is performed at the IAB-node.

Regarding power control for simultaneous operation of IAB-node’s child and parent links, RAN1#105 has made the following agreements.

- The information to assist DL power allocation of the parent-node is indicated by the IAB-MT to the parent node DU in terms of desired power adjustment.

  - FFS applicability of assistance information, e.g. per multiplexing scenario, per resource, etc.

- Decide in RAN1#106-e whether to support an IAB-node indicating assistance information to help with its MT’s UL TX power control. The assistance information can be:

- FFS: Desired TX power

- FFS: Offset to a baseline PHR

- FFS: Desired dynamic range

FFS: whether this information is provided to the parent-node, the CU, or both.

FFS: whether the MT’s UL TX power control formula needs to be changed

Regarding interference management, RAN1#105 has made the following agreements and conclusions.

- Rel-16 CLI coordination signaling (Intended TDD DL-UL Configuration) is extended to support IAB specific UFD patterns.

  - FFS: Support the exchange of IAB-DU H/S/NA resource configuration information among neighbouring IAB-nodes/IAB-donors for CLI management purposes.

In this contribution, based on those discussion and proposal, we further elaborate on power control mechanisms to support IAB-node’s simultaneous operation.

# Support for Case#6 and Case#7 Timing Alignment

We will focus on OTA-based timing alignment since non-OTA-based timing alignment (e.g., GNSS based) is a special case and we should always provide an OTA-based solution.

## Case#6 Timing for Simultaneous MT-TX/DU-TX

In Figure 2.1, we show the Case#6 timing relationship with some notations explained as follows.

- TAcase1: The original Case#1 TA at IAB-MT to make IAB node operating at Case#1 timing mode

- TAcase6: The absolute Case#6 TA at IAB-MT to make IAB node operating at Case#6 timing mode

- Tp: parent link propagation delay

Figure 2.1: Case#6 for MT TX/DU TX timing alignment illustration

To achieve Case#6 timing for simultaneous MT-TX/DU-TX at an IAB node, the IAB-MT’s TX timing needs to be aligned with IAB-DU’s TX timing. The absolute Case#6 TA at the IAB-MT is the same as parent link propagation delay Tp.

TACase6=Tp                                                                        (1)

According to TAcase1/Tp/Tg relationship for Case#1 timing in Rel-16 IAB discussion [1] as in Figure 2.2, we have

TAcase1=2Tp+Tg                                                               (2)

The MT TX shifting offset from Case#1 timing to Case#6 timing can be expressed as

TAoffset, Case6=TACase1-TACase6

=Tp+Tg                                                     (3)

Figure 2.2: TAcase1/Tp/Tg relationship for Case#1 timing [1]

In RAN1#105, three alternatives are summarized to down-select for Case#6 IAB-MT TX timing control:

- Alt1: the IAB-MT Tx timing is obtained by the node via the legacy TA loop plus an offset from the parent node.

    - FFS details of the required offset.

- Alt2: the IAB-MT Tx timing is set by the node to the timing obtained for the node’s DL Tx.

- Alt3: the IAB-MT Tx timing is obtained by the node jointly with the IAB-DU Tx timing via a common offset from the parent node.

Alt.1 Discussion

In Alt. 1, TACase1 and Tdelta,Case1 are remained as legacy TA loop, i.e., they keep the same values to calculate IAB-DU DL TX timing for Case#1 and Case#6. IAB-MT TX timing is obtained with TACase1 and TAoffset, Case6.

- Case#6 IAB-DU TX timing:               TAcase12+Tdelta,case1

where   TAcase1=2Tp+Tg and Tdelta,case1=-Tg/2

- Case#6 IAB-MT TX timing:               TACase1-TAoffset, Case6

where   TAcase1=2Tp+Tg and  TAoffset, Case6=Tp+Tg

In RAN1#105-e discussion, the main concern of Alt. 1 is regarding the “precise timing alignment” for DU-TX/MT-TX simultaneous operation.

Firstly, Case#6 IAB-DU TX timing is according to TACase1/Tdelta,case1 transmission from parent node and related to Tp/Tg estimation at parent node;

Secondly, Case#6 IAB-MT TX timing is according to  TACase1/Toffset,case6 transmission from parent node and also related to Tp/Tg estimation at parent node.

Hence, TACase1/Tdelta,case1/Toffset,case6 are all transmitted/controlled by the parent node according to Tp/Tg estimation at parent node. With proper granularity control, precise DU-TX/MT-TX timing alignment can be achieved.

Alt.2 Discussion

In Alt. 2, it is not clear how IAB-DU DL TX timing is calculated for OTA-based IAB nodes. In our understanding of Alt. 2, TACase1 and Tdelta,Case1 should also be remained as legacy TA loop, i.e., they keep the same values to calculate IAB-DU DL TX timing for Case#1 and Case#6. Hence, both Alt. 1 and Alt. 2 have legacy TACase1 and Tdelta,Case1.

In addition, since it has been agreed that Case#6 timing at an IAB-node is controlled by the parent node, additional signalling from parent node is needed to inform the IAB-node when to set IAB-MT UL TX timing to IAB-DU DL TX timing.

Hence, comparing Alt. 1 and Alt. 2, they both have legacy TACase1 and Tdelta,Case1 for OTA-based IAB-DU DL TX timing. The difference is the content of additional signalling from parent node.

- In Alt. 1, the additional signalling is TAoffset, Case6 to control IAB-MT UL TX timing as TACase1-TAoffset, Case6.

- In Alt. 2, the additional signalling is to inform the IAB-node when to set IAB-MT UL TX timing to IAB-DU DL TX timing.

Alt.3 Discussion

In Alt. 3, it is not clear how IAB-DU DL TX timing and IAB-MT TX timing are calculated and what the “common offset” means. More details are needed for this alternative.

Comparing all three alternatives for Case#6 timing:

- Both Alt. 1 and Alt. 2 have legacy TACase1 and Tdelta,Case1 for OTA alignment.

- Both Alt. 1 and Alt. 2 need additional signaling from parent node.

  - In Alt. 1, the additional signaling is TAoffset, Case6 to control IAB-MT UL TX timing as TACase1-TAoffset, Case6.

  - In Alt. 2, the additional signaling is to inform the IAB-node when to set IAB-MT UL TX timing to IAB-DU DL TX timing.

- Details of Alt. 3 are not clear.

- Alt. 1 can provide a unified solution for both Case#6 and Case#7.

Hence, we support Alt. 1 for Case#6 timing.

Observation 1: In Alt. 1, TACase1 and Tdelta,Case1 are remained as legacy TA loop, i.e., they keep the same values to calculate IAB-DU DL TX timing for Case#1 and Case#6. IAB-MT TX timing is obtained with TACase1 and TAoffset, Case6.

- Case#6 IAB-DU TX timing:               TAcase12+Tdelta,case1

where   TAcase1=2Tp+Tg and Tdelta,case1=-Tg/2

- Case#6 IAB-MT TX timing:               TACase1-TAoffset, Case6

where   TAcase1=2Tp+Tg and  TAoffset, Case6=Tp+Tg

Observation 2: In Alt. 1, TACase1/Tdelta,case1/Toffset,case6 are all transmitted/controlled by the parent node according to Tp/Tg estimation at parent node.  With proper granularity control, precise DU-TX/MT-TX timing alignment can be achieved.

Observation 3: Comparing all three alternatives for Case#6 timing:

- Both Alt. 1 and Alt. 2 have legacy TACase1 and Tdelta,Case1 for OTA alignment.

- Both Alt. 1 and Alt. 2 need additional signaling from parent node.

  - In Alt. 1, the additional signaling is TAoffset, Case6 to control IAB-MT UL TX timing as TACase1-TAoffset, Case6.

  - In Alt. 2, the additional signaling is to inform the IAB-node when to set IAB-MT UL TX timing to IAB-DU DL TX timing.

- Details of Alt. 3 are not clear.

- Alt. 1 can provide a unified solution for both Case#6 and Case#7.

Proposal 1: Support Alt. 1 for Case#6 timing, i.e., the IAB-MT TX timing is obtained via the legacy TA loop plus an offset from the parent node. The offset value is  TAoffset, Case6=Tp+Tg, where Tp is parent link propagation delay and  Tg is the switching gap between UL RX and DL TX at the parent node.

## Case#7 Timing for Simultaneous MT-RX/DU-RX

In Figure 2.3, we show an IAB-node’s timing relationship with parent node operating in Case#7 timing:

- TAcase1: The original Case#1 TA at IAB-MT to make parent node operating at Case#1 timing mode

- TAcase7: The absolute Case#7 TA at IAB-MT to make parent node operating at Case#7 timing mode

The shifting offset from Case#1 TA to absolute Case#7 TA is defined as:

TAoffset, Case7=TACase1-TACase7

Figure 2.3: IAB-node timing relationship when parent node operating at Case#7 timing

In RAN1#105, three alternatives are summarized to down-select for IAB-MT TX timing control when parent node under Case#7 timing:

- Alt1: the IAB-MT Tx timing of the node is obtained via the legacy TA loop plus an offset from the parent node.

  - FFS details of the required offset

- Alt2: the IAB-MT Tx timing of the node is obtained via the legacy TA loop from the parent node.

- Alt3: the IAB-MT Tx timing of the node is obtained via a Case 7 specific TA loop from the parent node.

Alt.1 Discussion

In Alt. 1, TACase1 and Tdelta,Case1 are remained as legacy TA loop, i.e., they keep the same values to calculate IAB-DU DL TX timing for Case#1 and Case#7. IAB-MT TX timing is obtained with TACase1 and TAoffset, Case7.

In Figure 2.4, we show more details to investigate the required offset value of TAoffset, Case7 with two additional parameters.

- Tp0: parent-MT RX propagation delay

- Tp: IAB-MT RX propagation delay

We derive TAoffset, Case7 value with the following steps.

        - IAB-MT RX timing is Tp behind DL TX timing

IAB-MT TX Case#1 timing is TACase1 ahead of IAB-MT RX timing

        - IAB-MT TX Case#1 timing is TACase1-Tp ahead of DL TX timing

        - Parent-DU RX Case#1 timing is TACase1-2Tp ahead of DL TX timing

        - Parent-MT RX timing is Tp0 behind DL TX timing

Parent-DU RX Case#7 timing is the same as parent-MT RX timing

        - Parent-DU RX Case#7 timing is Tp0 behind DL TX timing

        - Parent-DU RX Case#1 timing is TACase1-2Tp ahead of DL TX timing (from Step1)

Parent-DU RX Case#7 timing is Tp0 behind DL TX timing (from Step 2)

        - Parent-DU RX Case#1 timing is TACase1-2Tp+Tp0 ahead of Parent-DU RX Case#7 timing

        - IAB-MT TX Case#1 timing is TACase1-2Tp+Tp0 ahead of IAB-MT TX Case#7 timing

        - TAoffset, Case7=TACase1-2Tp+Tp0

        - TAcase1=2Tp+Tg                               from Rel-16 IAB discussion [1] as in Figure 2.2

TAoffset, Case7=TACase1-2Tp+Tp0      from Step 3

        - TAoffset, Case7=Tp0+Tg

Figure 2.4: IAB-node timing relationship details with parent node operating at Case#7 timing

Hence, in Alt.1, IAB-DU TX timing and IAB-MT TX timing when parent node operating on Case#7 mode are:

- IAB-DU TX timing when parent node in Case#7:               TAcase12+Tdelta,case1

where   TAcase1=2Tp+Tg and Tdelta,case1=-Tg/2

- IAB-MT TX timing when parent node in Case#7:               TACase1-TAoffset, Case7

where   TAcase1=2Tp+Tg and  TAoffset, Case7=Tp0+Tg

Note that the Case#7 offset  TAoffset, Case7=Tp0+Tg is always positive.

Alt.2 Discussion

In Alt. 2, the TA loop is used to transmit absolute Case#7 TA to control IAB-MT TX timing when parent node in Case#7 timing mode. According to Alt. 1 discussion, we have

TAcase1=2Tp+Tg                                               (4)

TAoffset, Case7=Tp0+Tg                                    (5)

Hence, we can express the absolute Case#7 TA in terms of parent-MT RX propagation delay (Tp0) and IAB-MT RX propagation delay (Tp) as

TACase7=TACase1-TAoffset,Case7

=2Tp-Tp0                                            (6)

As there is no fixed relationship between the two propagation delays (Tp0 and Tp), the absolute Case#7 TA in Eq. (6) may be negative.

Hence, in Alt.2, IAB-DU TX timing and IAB-MT TX timing when parent node operating on Case#7 mode are:

- IAB-DU TX timing when parent node in Case#7:               TAcase72+Tdelta,case7

where   TAcase7=2Tp-Tp0 (may be negative) and Tdelta,case1≠-Tg/2.

- IAB-MT TX timing when parent node in Case#7:               TACase7

where   TAcase7=2Tp-Tp0 and may be negative.

Note that since absolute Case#7 TA is transmitted instead of Case 1 TA, the IAB-DU DL TX timing for OTA-based IAB node is calculated as  TAcase72+Tdelta,case7.  (Note that TA and Tdelta are coupled to make TA2+Tdelta=Tp.) When an IAB parent node switches from Case#1 timing to Case#7 timing, there will be a sudden change/big gap of TA and Tdelta values to the IAB-node. The TA averaging scheme based on averaging across a time window may fail under this kind of TA sudden change.

In addition, TA and Tdelta values are not guaranteed to be transmitted together. The IAB-DU’s DL TX timing needs to be calculated either based on (TAcase12+Tdelta,case1) or based on (TAcase72+Tdelta,case7). If (TA,Tdelta) pair gets mis-matched for Case#1 and Case#7, there will be severe error of IAB-DU’s DL TX timing.

Therefore, there will be the following issues regarding Alt. 2 for Case#7 timing:

- TACase7 may be negative and the current TA/Tdelta range need to be modified.

- There will be a sudden change/big gap of TA and Tdelta values when parent node switches between Case#1 timing and Case#7 timing.

- TA and Tdelta values are not guaranteed to be transmitted together. If (TA,Tdelta) pair gets mis-matched for Case#1 and Case#7, there will be severe error of IAB-DU’s DL TX timing

Alt.3 Discussion

In Alt. 3, TACase1 and Tdelta,Case1 are remained as legacy TA loop, i.e., they keep the same values to calculate IAB-DU DL TX timing for Case#1 and Case#7. IAB-MT TX timing is obtained with a Case 7 specific TA loop to transmit absolute Case#7 TA (TACase7) from the parent node.

Hence, in Alt.3, IAB-DU TX timing and IAB-MT TX timing when parent node operating on Case#7 mode are:

- IAB-DU TX timing when parent node in Case#7:               TAcase12+Tdelta,case1

where   TAcase1=2Tp+Tg and Tdelta,case1=-Tg/2

- IAB-MT TX timing when parent node in Case#7:               TACase7

where   TAcase7=2Tp-Tp0 and may be negative.

Comparing all three alternatives for Case#7 timing:

- Both Alt. 1 and Alt. 3 have legacy TACase1 and Tdelta,Case1 for OTA alignment.

- Alt. 2 will transmit absolute Case#7 TA (TACase7) and may have the following issues

  - TACase7 may be negative and the current TA/Tdelta range need to be modified.

  - There will be a sudden change/big gap of TA and Tdelta values when parent node switches between Case#1 timing and Case#7 timing.

  - TA and Tdelta values are not guaranteed to be transmitted together. If (TA,Tdelta) pair gets mis-matched for Case#1 and Case#7, there will be severe error of IAB-DU’s DL TX timing

- Alt. 3 will transmit both legacy Case#1 TA (TACase1) and absolute Case#7 TA (TACase7)

  - TACase7 may be negative and the range is different from TACase1.

  - IAB-node does not always apply Case#7 timing at parent node. Alt.1 is better in terms of signaling overhead.

- Alt. 1 will transmit an offset of  TAoffset, Case7=Tp0+Tg, which is always positive.

- Alt. 1 can provide a unified solution for both Case#6 and Case#7.

Hence, we support Alt. 1 for Case#7 timing.

Observation 4: In Alt.1, IAB-DU TX timing and IAB-MT TX timing when parent node operating on Case#7 mode are:

- IAB-DU TX timing when parent node in Case#7:               TAcase12+Tdelta,case1

where   TAcase1=2Tp+Tg and Tdelta,case1=-Tg/2

- IAB-MT TX timing when parent node in Case#7:               TACase1-TAoffset, Case7

where   TAcase1=2Tp+Tg and  TAoffset, Case7=Tp0+Tg

Observation 5: In Alt.2, IAB-DU TX timing and IAB-MT TX timing when parent node operating on Case#7 mode are:

- IAB-DU TX timing when parent node in Case#7:               TAcase72+Tdelta,case7

where   TAcase7=2Tp-Tp0 (may be negative) and Tdelta,case1≠-Tg/2.

- IAB-MT TX timing when parent node in Case#7:               TACase7

where   TAcase7=2Tp-Tp0 and may be negative.

Observation 6: In Alt.3, IAB-DU TX timing and IAB-MT TX timing when parent node operating on Case#7 mode are:

- IAB-DU TX timing when parent node in Case#7:               TAcase12+Tdelta,case1

where   TAcase1=2Tp+Tg and Tdelta,case1=-Tg/2

- IAB-MT TX timing when parent node in Case#7:               TACase7

where   TAcase7=2Tp-Tp0 and may be negative.

Observation 7: Comparing all three alternatives for Case#7 timing:

- Both Alt. 1 and Alt. 3 have legacy TACase1 and Tdelta,Case1 for OTA alignment.

- Alt. 2 will transmit absolute Case#7 TA (TACase7) and may have the following issues

  - TACase7 may be negative and the current TA/Tdelta range need to be modified.

  - There will be a sudden change/big gap of TA and Tdelta values when parent node switches between Case#1 timing and Case#7 timing.

  - TA and Tdelta values are not guaranteed to be transmitted together. If (TA,Tdelta) pair gets mis-matched for Case#1 and Case#7, there will be severe error of IAB-DU’s DL TX timing

- Alt. 3 will transmit both legacy Case#1 TA (TACase1) and absolute Case#7 TA (TACase7)

  - TACase7 may be negative and the range is different from TACase1.

  - IAB-node does not always apply Case#7 timing at parent node. Alt.1 is better in terms of signaling overhead.

- Alt. 1 will transmit an offset of  TAoffset, Case7=Tp0+Tg, which is always positive.

- Alt. 1 can provide a unified solution for both Case#6 and Case#7.

Proposal 2: Support Alt. 1 for Case#7 timing, i.e., the IAB-MT TX timing is obtained via the legacy TA loop plus an offset from the parent node. The offset value is  TAoffset, Case7=Tp0+Tg, where Tp0 is parent-MT RX propagation delay and  Tg is the switching gap between UL RX and DL TX at the parent node.

Proposal 3: A unified TA transmission scheme (always transmitting legacy Case#1 TA with additional positive TA offset) can be applied for both Case#6 and Case#7 timing.

## Switching Between Case#1/Case#6/Case#7 timing

RAN1#104 has agreed to support switching between Case#1, Case#6 and Case#7 timing.

###### 2.3.1 Case#1Case#6Case#1

In Figure 2.5, we show the transition of Case#1Case#6Case#1 in three consecutive slots. The IAB-node operating in Case#6 timing and its parent IAB-node will be affected and included in the figure.

Figure 2.5: Case#1Case#6Case#1 transition

For the switching of Case#1 Case#6 timing at an IAB-node, as IAB-MT TX timing being shifted backwards to align with IAB-DU TX timing, there is no conflict in IAB-MT TX or parent-DU RX from slot n to slot (n+1). In addition, in our previous contribution [2], we have shown that when one of the multiplexing modes is supported, the corresponding transition guard symbols are not needed. In other words, for Case#1 Case#6 switching, the guard symbols for the following two transitions defined in Rel-16 IAB can be removed as simultaneous DU TX/MT TX is allowed in Case#6 timing:

- IAB-DU TX in slot n to IAB-MT TX in slot (n+1)

- IAB-MT TX in slot n to IAB-DU TX in slot (n+1)

For the switching of Case#6 Case#1 timing at an IAB-node, as IAB-MT TX timing moving in advance of IAB-DU TX timing, there will be conflict in IAB-MT TX or parent-DU RX from slot (n+1) to slot (n+2). Hence, in addition to MT/DU transition guard symbols defined in Rel-16 IAB, new guard symbols for the following scenario are needed for Case#6 Case#1 switching.

- IAB-MT Case#6 TX in slot (n+1) to IAB-MT Case#1 TX in slot (n+2)

Observation 8: For the switching of Case#1 Case#6 timing at an IAB-node, the guard symbols for the following two transitions defined in Rel-16 IAB can be removed as simultaneous DU TX/MT TX is allowed in Case#6 timing:

- IAB-DU TX in slot n to IAB-MT TX in slot (n+1)

- IAB-MT TX in slot n to IAB-DU TX in slot (n+1)

Observation 9: For the switching of Case#6 Case#1 timing at an IAB-node, new guard symbols are needed for IAB-MT Case#6 TX to IAB-MT Case#1 TX transition.

###### 2.3.2 Case#1Case#7Case#1

In Figure 2.6, we show the transition of Case#1Case#7Case#1 in three consecutive slots. The IAB-node operating in Case#7 timing and its child IAB-node will be affected and included in the figure.

Figure 2.6: Case#1Case#7Case#1 transition

For the switching of Case#1 Case#7 timing at an IAB-node, as its child-MT TX timing being shifted backwards to make IAB-DU RX timing align with IAB-MT RX timing, there is no conflict in child-MT TX or IAB-DU RX from slot n to slot (n+1). In addition, in our previous contribution [2], we have shown that when one of the multiplexing modes is supported, the corresponding transition guard symbols are not needed. In other words, for Case#1 Case#7 switching, the guard symbols for the following two transitions defined in Rel-16 IAB can be removed as simultaneous DU RX/MT RX is allowed in Case#7 timing:

- IAB-DU RX in slot n to IAB-MT RX in slot (n+1)

- IAB-MT RX in slot n to IAB-DU RX in slot (n+1)

For the switching of Case#7 Case#1 timing, as its child-MT TX timing moving forwards for IAB-DU RX Case#1 timing, there will be conflict in child-MT TX or IAB-DU RX from slot (n+1) to slot (n+2). Hence, in addition to MT/DU transition guard symbols defined in Rel-16 IAB, guard symbols for the following new scenario are needed for Case#7 Case#1 switching.

- child-MT Case#7 TX in slot (n+1) to child-MT Case#1 TX in slot (n+2)

Observation 10: For the switching of Case#1 Case#7 timing, the guard symbols for the following two transitions defined in Rel-16 IAB can be removed as simultaneous DU RX/MT RX is allowed in Case#7 timing:

- IAB-DU RX in slot n to IAB-MT RX in slot (n+1)

- IAB-MT RX in slot n to IAB-DU RX in slot (n+1)

Observation 11: For the switching of Case#7 Case#1 timing at an IAB-node, guard symbols are needed for child-MT Case#7 TX to child-MT Case#1 TX transition.

###### 2.3.3 Case#6Case#7Case#6

In Figure 2.7, we show the transition of Case#6Case#7Case#6 in three consecutive slots. The IAB-node operating in Case#6 or Case#7 timing, its parent IAB-node and its child IAB-node will be affected and included in the figure.

Figure 2.7: Case#6Case#7Case#6 transition

For the switching of Case#6 Case#7 timing, it is equivalent to the combination of Case#6 Case#1 switching and Case#1 Case#7 switching. Refer to the discussion in Section 2.3.1 and 2.3.2, new guard symbols are needed for switching of Case#6 Case#1 timing, which will affect Case#6 Case#7 switching.

For the switching of Case#7 Case#6 timing, it is equivalent to the combination of Case#7 Case#1 switching and Case#1 Case#6 switching. Refer to the discussion in Section 2.3.1 and 2.3.2, new guard symbols are needed for switching of Case#7 Case#1 timing, which will affect Case#7 Case#6 switching.

In summary for new guard symbols, we provide the following Table 1 in case of switching between Case#1/Case#6/Case#7 timing as below, in addition to MT/DU resource transition guard symbol table defined in Rel-16 IAB.

Table 1: new guard symbols for switching between Case#1/Case#6/Case#7 timing

| IAB-node | To: Case#1 | To: Case#6 | To: Case#7 |
| --- | --- | --- | --- |
| From: Case#1 |  |  |  |
| From: Case#6 | IAB-MT Case#6 TX to IAB-MT Case#1 TX |  | IAB-MT Case#6 TX to IAB-MT Case#1 TX |
| From: Case#7 | child-MT Case#7 TX to child-MT Case#1 TX | child-MT Case#7 TX to child-MT Case#1 TX |  |

In summary for Rel-16 IAB guard symbols removal, we provide the following Table 2 when simultaneous operation(s) are supported [2].

Table 2: Guard symbols removal with different simultaneous operations

| MT to DU | DL Tx | UL Rx |
| --- | --- | --- |
| DL Rx | Remove Ng if MT RX/DU TX is supported | Remove Ng if MT RX/DU RX is supported |
| UL Tx | Remove Ng if MT TX/DU TX is supported | Remove Ng if MT TX/DU RX is supported |
| DU to MT | DL Rx | UL Tx |
| DL Tx | Remove Ng if MT RX/DU TX is supported | Remove Ng if MT TX/DU TX is supported |
| UL Rx | Remove Ng if MT RX/DU RX is supported | Remove Ng if MT TX/DU RX is supported |

Observation 12: For the switching of Case#6 Case#7 timing at an IAB-node, it is equivalent to the combination of Case#6 Case#1 switching and Case#1 Case#7 switching. New guard symbols are needed for IAB-MT Case#6 TX to IAB-MT Case#1 TX transition.

Observation 13: For the switching of Case#7 Case#6 timing at an IAB-node, it is equivalent to the combination of Case#7 Case#1 switching and Case#1 Case#6 switching. New guard symbols are needed for child-MT Case#7 TX to child-MT Case#1 TX transition.

Proposal 4: New guard symbols are needed for switching between Case#1/Case#6/Case#7 timing as in Table 1.

Proposal 5: When simultaneous operation(s) are supported, the corresponding transition guard symbols defined in Rel-16 IAB are not needed as in Table 2.

# Power Control to Support IAB-node’s Simultaneous Operation

For the four FFS power control mechanisms agreed in RAN1#103e (DL/UL power control with assistance information from the child node; DL/UL power control with assistance information from the parent node), we rewrite those FFS mechanism as P1/P2/P3/P4, which are illustrated in Figure 3.1.

  - P1: DL power control with assistance information from the child node

  - P2: DL power control with assistance information from the parent node

  - P3: UL power control with assistance information from the child node

  - P4: UL power control with assistance information from the parent node

Figure 3.1: Illustration of four power control (PC) mechanism for further study in RAN1#103e

## P1: child-node assisted DL power control

There can be three alternatives to fulfil P1:

- P1-Alt.1: Open-loop DL power control

- P1-Alt.2: Closed-loop DL power control

- P1-Alt.3: UL TPC for DU

For P1-Alt. 1, we illustrate the open-loop DL power control mechanism in Figure 3.2. An IAB DU’s DL transmission power setting algorithm will be based on some configured parameters and UL reference signal measurement for path-loss calculation. The parameter configurations can include UL reference signal power, target DL reception power, fractional path-loss compensation factor, etc.

Figure 3.2: Open-loop DL power control illustration (P1-Alt.1)

For P1-Alt. 2, we illustrate the closed-loop DL power control mechanism in Figure 3.3. There will be additional transmit power control (TPC) command transmitted from child MT to IAB DU according to IAB DU’s DL transmission measurements. The IAB-DU’s DL transmission power setting algorithm will be based on the configured parameters, UL reference signal measurement for path-loss calculation and TPC command.

Figure 3.3: Closed-loop DL power control illustration (P1-Alt.2)

For P1-Alt.3, we illustrate the UL TPC for DU power control mechanism in Figure 3.4. This can be considered as simplified DL power control mechanism without parameter configuration at IAB-DU (e.g., UL reference signal power, target DL reception power, fractional path-loss compensation factor). UL TPC for DU is introduced from child IAB-MT to IAB-DU to dynamically inform the IAB-DU to increase or decrease DL transmission power.

Figure 3.4: UL TPC for DU illustration (P1-Alt.3)

Proposal 6: For child-node assisted DL power control, further discuss the following three alternatives:

- P1-Alt.1: Open-loop DL power control

- P1-Alt.2: Closed-loop DL power control

- P1-Alt.3: UL TPC for DU

## P2: parent-node assisted DL power control

We further investigate P2 (parent node assisted DL power control) for at least TX imbalance mitigation at IAB node.  In MT TX/DU TX simultaneous operation, the received signal at parent DU may have interference coming from DU’s DL transmission. With current UL power control, parent DU can send dynamic TPC for MT to increase its UL transmission power. However, the UL transmission power is limited by the UE/MT capability and may not fully solve this issue. Hence, besides DL TPC for MT, DL TPC for DU can be introduced from parent DU to IAB MT to inform the IAB DU to adjust its DL transmission power, as shown in Figure 3.5.

Figure 3.5: DL TPC for DU illustration

Proposal 7: For parent-node assisted DL power control, support DL TPC for DU.

## P3/P4: child-node or parent-node assisted UL power control

We illustrate the UL power control mechanisms (open-loop and closed-loop) supported in current specification for an IAB MT or UE in Figure 3.6 and Figure 3.7. In open-loop UL power control, the parent DU send DL RS to an IAB MT or UE. With parameter configuration including DL RS signal power, target UL reception power and fractional path-loss compensation factor, the IAB MT or UE can figure out its UL transmission power accordingly. In closed-loop UL power control, additional dynamic TPC command will be transmitted according to the IAB MT or UE’s UL transmission measurements.

Figure 3.5: Open-loop UL power control illustration

Figure 3.6: Closed-loop UL power control illustration

For P3 (child-node assisted UL power control in Figure 3.1-c), with MT TX/DU TX imbalance, the received signal at IAB- DU may have interference coming from child-DU’s DL transmission. This will be reflected in the child-MT’s UL transmission measurements naturally and the IAB DU can send TPC command to adjust the child-MT’s UL transmission power. The UL transmission measurements and dynamic TPC command are already included in existing closed-loop UL power control mechanism.

For P4 (parent node assisted UL power control in Figure 3.1-d), with MT RX/DU RX imbalance, the received signal at IAB-DU may have interference coming from parent-DU’s DL transmission. This will also be reflected in the child-MT’s UL transmission measurements naturally and the IAB-DU can send TPC command to adjust the child-MT’s UL transmission.

Proposal 8: Child-node assisted or parent-node assisted UL power control can be fulfilled with existing UL power control mechanisms.

# Interference Management

For inter-IAB scenarios: MT-to-MT, DU-to-DU, DU-to-MT, and MT-to-DU are included, which also corresponds to the four cases agreed in RAN1#93 and illustrated in Figure 4.1.

- Case 1 (MT-to-MT): Victim IAB node is receiving in DL in the backhaul link, interfering IAB node is transmitting in UL in the backhaul link.

- Case 2 (DU-to-MT): Victim IAB node is receiving in DL in the backhaul link, interfering IAB node is transmitting in DL in the access link.

- Case 3 (MT-to-DU): Victim IAB node is receiving in UL in the access link, interfering IAB node is transmitting in UL in the backhaul link.

- Case 4 (DU-to-DU): Victim IAB node is receiving in UL in the access link, interfering IAB node is transmitting in DL in the access link.

Figure 4.1: Inter-IAB scenarios illustration

For Case 1 (MT-to-MT) interference management, current CLI measurements (e.g., CLI-RSSI and SRS-RSRP) in Rel-16 NR to address UE-to-UE interference can be the starting point. On the other hand, Rel-16 CLI measurements and reports are through RRC signalling (e.g., CU configures UE/MT measurements and UE/MT reports to CU). For dynamic measurement configuration and scheduling efficiency in IAB network, we can enhance measurements/reporting with L1/L2 signalling (e.g., parent-DU configures IAB-MT’s measurements and IAB-MT reports to parent-DU).

For Case 2 (DU-to-MT) interference management, current interference management methods, e.g., NZP CSI-RS and CSI-IM based methods in Rel-16 NR can be starting point. Similar to MT-to-MT scenario, L1/L2 signalling enhancements can be introduced.

For Case 3 (MT-to-DU) interference management, since there are no BS measurements defined in current specifications for UE to BS interference, we cannot reuse Rel-16 CLI or interference management methods and additional enhancements are needed. There can be the following two options.

- MT-to-DU-Option.1: DU-based measurement and report procedure

In this option, the victim IAB-DU can be informed with interfering IAB-MT’s SRS/DM-RS configuration and perform measurements/report accordingly

- MT-to-DU-Option.2: MT-based measurement and report procedure

In this option, the co-located IAB-MT can perform MT-based CLI measurement/report.

For Case 4 (DU-to-DU) interference management, RAN1#104e has made the following agreement.

RAN1 to select among the following options to support DU-to-DU measurement and report.

- For DU-to-DU CLI measurement:

- Option 1.1. no specific mechanism is specified (e.g., it is handled by the implementation, or the available techniques)

- Option 1.2. enhanced legacy DU-based measurement procedures (e.g., enhanced Rel-16 RIM)

- Option 1.3. enhanced MT-based measurements (e.g., MT-based CLI, MT RRM measurements)

- For DU-to-DU CLI report:

- Option 2.1. no specific mechanism is specified (e.g., it is handled by the implementation, or the available techniques)

- Option 2.2. enhanced legacy DU-based report (e.g., enhanced Rel-16 RIM)

- Option 2.3. enhanced MT-based report (e.g., MT-based CLI, MT RRM measurements)

As Rel-16 RIM is designed for remote BS-to-BS interference with atmospheric conditions, Option 1.2/Option 2.2 based on legacy Rel-16 RIM are not suitable for IAB DU-to-DU interference management.

Proposal 9: For MT-to-MT interference management, current CLI measurements (e.g., CLI-RSSI and SRS-RSRP) in Rel-16 NR to address UE-to-UE interference can be the starting point. L1/L2 signalling enhancements can be introduced.

Proposal 10: For DU-to-MT interference management, current interference management methods, e.g., NZP CSI-RS and CSI-IM based methods in Rel-16 NR can be the starting point. L1/L2 signalling enhancements can be introduced.

Proposal 11: For MT-to-DU interference management, further discuss the following options.

- MT-to-DU-Option.1: DU-based measurement and report procedure

- MT-to-DU-Option.2: MT-based measurement and report procedure

Proposal 12: For DU-to-DU interference management, Option 1.2/Option 2.2 based on legacy Rel-16 RIM are not suitable.

# Conclusion

In this contribution, we discussed mechanisms to support Case#6 and Case#7 timing, power control for IAB-node’s simultaneous operation and interference management issues.  It is summarized by the following observations and proposals.

Observation 1: In Alt. 1, TACase1 and Tdelta,Case1 are remained as legacy TA loop, i.e., they keep the same values to calculate IAB-DU DL TX timing for Case#1 and Case#6. IAB-MT TX timing is obtained with TACase1 and TAoffset, Case6.

- Case#6 IAB-DU TX timing:               TAcase12+Tdelta,case1

where   TAcase1=2Tp+Tg and Tdelta,case1=-Tg/2

- Case#6 IAB-MT TX timing:               TACase1-TAoffset, Case6

where   TAcase1=2Tp+Tg and  TAoffset, Case6=Tp+Tg

Observation 2: In Alt. 1, TACase1/Tdelta,case1/Toffset,case6 are all transmitted/controlled by the parent node according to Tp/Tg estimation at parent node.  With proper granularity control, precise DU-TX/MT-TX timing alignment can be achieved.

Observation 3: Comparing all three alternatives for Case#6 timing:

- Both Alt. 1 and Alt. 2 have legacy TACase1 and Tdelta,Case1 for OTA alignment.

- Both Alt. 1 and Alt. 2 need additional signaling from parent node.

  - In Alt. 1, the additional signaling is TAoffset, Case6 to control IAB-MT UL TX timing as TACase1-TAoffset, Case6.

  - In Alt. 2, the additional signaling is to inform the IAB-node when to set IAB-MT UL TX timing to IAB-DU DL TX timing.

- Details of Alt. 3 are not clear.

- Alt. 1 can provide a unified solution for both Case#6 and Case#7.

Observation 4: In Alt.1, IAB-DU TX timing and IAB-MT TX timing when parent node operating on Case#7 mode are:

- IAB-DU TX timing when parent node in Case#7:               TAcase12+Tdelta,case1

where   TAcase1=2Tp+Tg and Tdelta,case1=-Tg/2

- IAB-MT TX timing when parent node in Case#7:               TACase1-TAoffset, Case7

where   TAcase1=2Tp+Tg and  TAoffset, Case7=Tp0+Tg

Observation 5: In Alt.2, IAB-DU TX timing and IAB-MT TX timing when parent node operating on Case#7 mode are:

- IAB-DU TX timing when parent node in Case#7:               TAcase72+Tdelta,case7

where   TAcase7=2Tp-Tp0 (may be negative) and Tdelta,case1≠-Tg/2.

- IAB-MT TX timing when parent node in Case#7:               TACase7

where   TAcase7=2Tp-Tp0 and may be negative.

Observation 6: In Alt.3, IAB-DU TX timing and IAB-MT TX timing when parent node operating on Case#7 mode are:

- IAB-DU TX timing when parent node in Case#7:               TAcase12+Tdelta,case1

where   TAcase1=2Tp+Tg and Tdelta,case1=-Tg/2

- IAB-MT TX timing when parent node in Case#7:               TACase7

where   TAcase7=2Tp-Tp0 and may be negative.

Observation 7: Comparing all three alternatives for Case#7 timing:

- Both Alt. 1 and Alt. 3 have legacy TACase1 and Tdelta,Case1 for OTA alignment.

- Alt. 2 will transmit absolute Case#7 TA (TACase7) and may have the following issues

  - TACase7 may be negative and the current TA/Tdelta range need to be modified.

  - There will be a sudden change/big gap of TA and Tdelta values when parent node switches between Case#1 timing and Case#7 timing.

  - TA and Tdelta values are not guaranteed to be transmitted together. If (TA,Tdelta) pair gets mis-matched for Case#1 and Case#7, there will be severe error of IAB-DU’s DL TX timing

- Alt. 3 will transmit both legacy Case#1 TA (TACase1) and absolute Case#7 TA (TACase7)

  - TACase7 may be negative and the range is different from TACase1.

  - IAB-node does not always apply Case#7 timing at parent node. Alt.1 is better in terms of signaling overhead.

- Alt. 1 will transmit an offset of  TAoffset, Case7=Tp0+Tg, which is always positive.

- Alt. 1 can provide a unified solution for both Case#6 and Case#7.

Observation 8: For the switching of Case#1 Case#6 timing at an IAB-node, the guard symbols for the following two transitions defined in Rel-16 IAB can be removed as simultaneous DU TX/MT TX is allowed in Case#6 timing:

- IAB-DU TX in slot n to IAB-MT TX in slot (n+1)

- IAB-MT TX in slot n to IAB-DU TX in slot (n+1)

Observation 9: For the switching of Case#6 Case#1 timing at an IAB-node, new guard symbols are needed for IAB-MT Case#6 TX to IAB-MT Case#1 TX transition.

Observation 10: For the switching of Case#1 Case#7 timing, the guard symbols for the following two transitions defined in Rel-16 IAB can be removed as simultaneous DU RX/MT RX is allowed in Case#7 timing:

- IAB-DU RX in slot n to IAB-MT RX in slot (n+1)

- IAB-MT RX in slot n to IAB-DU RX in slot (n+1)

Observation 11: For the switching of Case#7 Case#1 timing at an IAB-node, guard symbols are needed for child-MT Case#7 TX to child-MT Case#1 TX transition.

Observation 12: For the switching of Case#6 Case#7 timing at an IAB-node, it is equivalent to the combination of Case#6 Case#1 switching and Case#1 Case#7 switching. New guard symbols are needed for IAB-MT Case#6 TX to IAB-MT Case#1 TX transition.

Observation 13: For the switching of Case#7 Case#6 timing at an IAB-node, it is equivalent to the combination of Case#7 Case#1 switching and Case#1 Case#6 switching. New guard symbols are needed for child-MT Case#7 TX to child-MT Case#1 TX transition.

Proposal 1: Support Alt. 1 for Case#6 timing, i.e., the IAB-MT TX timing is obtained via the legacy TA loop plus an offset from the parent node. The offset value is  TAoffset, Case6=Tp+Tg, where Tp is parent link propagation delay and  Tg is the switching gap between UL RX and DL TX at the parent node.

Proposal 2: Support Alt. 1 for Case#7 timing, i.e., the IAB-MT TX timing is obtained via the legacy TA loop plus an offset from the parent node. The offset value is  TAoffset, Case7=Tp0+Tg, where Tp0 is parent-MT RX propagation delay and  Tg is the switching gap between UL RX and DL TX at the parent node.

Proposal 3: A unified TA transmission scheme (always transmitting legacy Case#1 TA with additional positive TA offset) can be applied for both Case#6 and Case#7 timing.

Proposal 4: New guard symbols are needed for switching between Case#1/Case#6/Case#7 timing as in Table 1.

Proposal 5: When simultaneous operation(s) are supported, the corresponding transition guard symbols defined in Rel-16 IAB are not needed as in Table 2.

Proposal 6: For child-node assisted DL power control, further discuss the following three alternatives:

- P1-Alt.1: Open-loop DL power control

- P1-Alt.2: Closed-loop DL power control

- P1-Alt.3: UL TPC for DU

Proposal 7: For parent-node assisted DL power control, support DL TPC for DU.

Proposal 8: Child-node assisted or parent-node assisted UL power control can be fulfilled with existing UL power control mechanisms.

Proposal 9: For MT-to-MT interference management, current CLI measurements (e.g., CLI-RSSI and SRS-RSRP) in Rel-16 NR to address UE-to-UE interference can be the starting point. L1/L2 signalling enhancements can be introduced.

Proposal 10: For DU-to-MT interference management, current interference management methods, e.g., NZP CSI-RS and CSI-IM based methods in Rel-16 NR can be the starting point. L1/L2 signalling enhancements can be introduced.

Proposal 11: For MT-to-DU interference management, further discuss the following options.

- MT-to-DU-Option.1: DU-based measurement and report procedure

- MT-to-DU-Option.2: MT-based measurement and report procedure

Proposal 12: For DU-to-DU interference management, Option 1.2/Option 2.2 based on legacy Rel-16 RIM are not suitable.

# References

[1] R1-1905842, “LS on clarification of OTA timing alignment for IAB”, Xi’an, China, April 2019.

[2] R1-2005893, “Enhancements to resource multiplexing between child and parent links of an IAB node”, Intel Corporation, RAN1#102e, Aug. 2020.