3GPP TSG RAN WG1 #106-e                                                                  	              R1-2107617

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.14.2

Source: 	Intel Corporation

Title:	On evaluation methodology for XR

Document for: 	Discussion/Decision

# Introduction

In this contribution we provide our views on coverage evaluation methodology for XR SID.

# Coverage Evaluation

In RAN1#105-e meeting the following was captured in the Chairman’s notes with respect to EVM for coverage for NR XR:

For companies to further study and if necessary, discuss in RAN1#106-e (Coverage evaluation methodology) For XR/CG in DL or UL, coverage is defined to be the A-percentile point in CDF of Coupling gain for the “satisfied” UEs, with #UEs per cell = B, for a given XR application (AR/VR/CG) in a given deployment scenario (DU/InH/UMa) - A = [5], other value can also be reported - FFS: Value of B, e.g. B = 1, capacity, etc. - Note: Coupling gain for coverage evaluation is defined as the ratio of received and transmitted power measured in dB, and includes antenna gains, path loss, shadowing, indoor- or body loss, etc. Example of coupling gain can refer to TR 37.910. An alternate method could be to use the “traditional” method such as what is used in the CE study/work item.

For coverage evaluation, our view is generally aligned with the proposal captured above. We believe that SLS based coverage evaluation is useful. However, to isolate the impact of coverage, B =1 is more useful. This is due to the fact that in SLS interference (both intra- and inter-cell) can play a major role in overall cell coverage. In order to identify the coverage bottleneck irrespective of cell loading, it is better to evaluate with many drops of B=1 UE and consider only those UEs which successfully satisfy the PDB. If we consider the number of UEs from capacity evaluation, the interference impact will not allow for accurate evaluation of coverage bottleneck

Furthermore, the entire CDF can be evaluated without concentrating on a single A-percentile point. The CDF can be captured in the TR.

Another major consideration is for UL coverage, where power control parameters play a major role. The UPLC is a function of load due to MU-MIMO as well inter-cell interference. Additionally, the coupling gain is also a function of ULPC parameters. SLS based evaluation can account for both load and power control and therefore more realistic than traditional methods for UL. With a single UE drop a more aggressive power control can be adopted and optimization for ULPC parameters is required to evaluate the coverage bottleneck for the ULPC that provides the best coverage. Companies can report the power control values used and the CDF of the coupling gain under this assumption.

Based on the above discussion, SLS appears to be a much more robust tool for coverage evaluation for XR accounting for both DL and UL coverage. Therefore, we have the following proposal:

Proposal: For XR/CG in DL or UL, for coverage evaluation, companies can report the CDF of Coupling gain for the “satisfied” UEs, with #UEs per cell = 1, for a given XR application (AR/VR/CG) in a given deployment scenario (DU/InH/UMa)

- Note: Coupling gain for coverage evaluation is defined as the ratio of received and transmitted power measured in dB, and includes antenna gains, path loss, shadowing, indoor- or body loss, etc. Example of coupling gain can refer to TR 37.910.

# References

| [1] | 3GPP RAN1, "TR 38.824, Study on physical layer enhancements for NR ultra-reliable and low latency case (URLLC)," 27 03 2019. [Online]. |
| --- | --- |
| [2] | Qualcomm, "S4-201399, FS_XR Traffic: Permanent document v0.3.1," [Online]. |
| [3] | 3GPP SA4, "TR 22.842, Study on Network Controlled Interactive Service (NCIS) in the 5G System (5GS)," 20 12 2019. [Online]. |
| [4] | 3GPP SA4, "TR 22.925, Quality of Service (QoS) and network performance 3.1.1," 23 04 1999. [Online]. |
| [5] [6] [7] [8] [9] | 3GPP SA4, "TR26.928, Extended Reality (XR) in 5G 16.1.0," 23 12 2020. [Online]. 3GPP RAN1 TR 38.840, " Study on User Equipment (UE) power saving in NR (Release 16)" R1-070674, "LTE Physical Layer framework for performance verification", RAN WG1 #48, February 2007, St. Louis, MI, USA. R1-2101765, LS to on XR-Traffic Models, SA4 R1-2007151, "Moderator Summary for multi-beam enhancement: EVM", Moderator (Samsung), RAN1 WG1 #102-e, August 2020 |