3GPP TSG RAN WG1 Meeting #106-e	R1-2107668

E-meeting, August 16th – 27th, 2021

Agenda Item:	8.6.1.4

Source:	Huawei, HiSilicon

Title:	Reduced maximum MIMO layers and reduced maximum modulation order for RedCap

Document for:	Discussion and Decision

# Introduction

In RAN1 #104-e and #105-e meeting, the UE complexity reduction and related issues caused were discussed. The following agreements related to the reduced number of DL MIMO layers were made:

Agreements: - For relaxed maximum number of DL MIMO layers: - FFS: need for modification of DCI fields/formats - FFS: need for modification of CSI measurement/reporting Conclusion: - For a RedCap UE, when motivated by reduced max number of DL MIMO layers modifications to CSI measurement and/or reporting mechanisms are not pursued in Rel-17.

The following agreements related to the reduced maximum modulation order were made:

Agreements: - For a RedCap UE, 64QAM MCS tables (Table 5.1.3.1-1 in TS 38.214 for DL and UL OFDM and Table 6.1.4.1-1 in TS 38.214 for UL w/ transform precoding respectively) are the “default” ones and are mandatory. - The following is optionally supported by RedCap UEs: - 256QAM MCS tables (Table 5.1.3.1-2 in TS 38.214 for DL and UL OFDM) - 64QAM low SE MCS tables (Table 5.1.3.1-3 in TS 38.214 for DL and UL OFDM and Table 6.1.4.1-2 in TS 38.214 for UL w/ transform precoding respectively) Agreements: - For a RedCap UE, “CQI table 1” (Table 5.2.2.1-2 in TS 38.214), that corresponds to MCS Table 5.1.3.1-1 in TS 38.214, is mandatory. - The following is optionally supported by a RedCap UE: - “CQI table 2” (Table 5.2.2.1-3 in TS 38.214) that corresponds to MCS Table 5.1.3.1-2 in TS 38.214 (256QAM MCS table) - “CQI table 3” (Table 5.2.2.1-4 in TS 38.214) that corresponds to MCS Table 5.1.3.1-3 in TS 38.214 (64QAM low SE MCS table) Agreements: - Both 256QAM MCS table for PDSCH and “CQI table 2” (Table 5.2.2.1-3 in TS 38.214) are supported by a RedCap UE indicating support of 256QAM for PDSCH. Agreements: - For a RedCap UE, support of 64QAM low SE MCS table for PDSCH and support of “CQI table 3” (Table 5.2.2.1-4 in TS 38.214) are not coupled and capability of each can be reported independent of the other. Agreements: - For a RedCap UE, support of 64QAM low SE MCS table for PDSCH (Table 5.1.3.1-3 in TS 38.214) and support of 64QAM low SE MCS tables for PUSCH (Table 5.1.3.1-3 in TS 38.214 for UL OFDM and Table 6.1.4.1-2 in TS 38.214 for UL w/ transform precoding respectively) are not coupled and capability of each can be reported independent of the other.

As the number of MIMO layers directly corresponds to the number of Rx branches, relevant discussion is provided in our companion contribution [2]. In this contribution, some other aspects related to modulation order reduction are analyzed.

# On reduced maximum modulation order

In NR, the 64QAM low SE MCS table can be configured by RRC signalling, or by DCI signalling if the MCS-C-RNTI is configured. The dynamic indication of MCS table with MCS-C-RNTI for PDSCH or PUSCH are optionally supported by R15 UEs. According to the agreements in RAN1 #105-e, the 64QAM low SE MCS tables (Table 5.1.3.1-3 in TS 38.214 for DL and UL OFDM and Table 6.1.4.1-2 in TS 38.214 for UL w/ transform precoding respectively) are optionally supported by RedCap UEs. Therefore, the support of MCS-C-RNTI shall continue to be optional for RedCap UEs.

Proposal 1: The dynamic indication of MCS table with MCS-C-RNTI for PDSCH or PUSCH shall continue to be optional for RedCap UEs.

In NR R17, support of 1024QAM is discussed for eMBB UEs. As the peak data rate requirement for RedCap UEs can be met with 64QAM (20MHz and 1Tx2Rx) and 256QAM is optional for RedCap UEs, there is no necessity to support 1024QAM for RedCap UEs at least for R17.

Proposal 2: Do not support DL 1024QAM for Rel-17 RedCap UEs.

# Conclusions

Proposal 1: The dynamic indication of MCS table with MCS-C-RNTI for PDSCH or PUSCH shall continue to be optional for RedCap UEs.

Proposal 2: Do not support DL 1024QAM for Rel-17 RedCap UEs.

# References

RP-211574, “Revised WID on support of reduced capability NR devices”, RAN#92e.

R1-2106460, “Reduced number of Rx branches for RedCap”, RAN1 #106e