3GPP TSG RAN WG1 #106-e	R1- 2107710

e-Meeting, August 16th – 27th, 2021

Agenda Item:	7.1

Source:	Apple Inc.

Title:	Timeline Considerations for SRS Carrier Switching and Dropping Procedure

Document for:	Discussion and Decision

# Introduction

For SRS carrier switching when UE switches to a carrier of the serving cell without configured PUCCH/PUSCH transmission to transmit SRS, some priority rules are applied between source and target carriers, if SRS transmission (including any interruption due to uplink or downlink RF retuning time) overlaps in time with simultaneous uplink transmission on source carrier. Out of this prioritization rules, UE shall cancel the low priority UL transmission, where the cancellation involves a timeline that shall be met by the scheduler. In RAN1 #105e, it was discussed how/whether to extend the cancellation timeline for low priority UL transmissions on CCs other than the source and target carriers [1]. The question is on which other UL CCs the priority rules, as defined in 38.214 6.2.1.3, are applicable. In general, there are two views, as below:

- The priority rules are applied between source/CCs intra-band with source, and target carrier. Given that UE shall drop the low priority transmission (either SRS or UL transmission on source/CCs intra-band with source), no UE capability indication is needed. In other words, the dropping procedure does not depend on the indicated UE’s UL-CA capability and the current specification, like in 38.214 6.2.1.3, which refers to “cancellation if simulations transmission is beyond UE’s UL-CA capability” shall be corrected accordingly. For the case that UE cannot keep simultaneous transmission between SRS on target carrier and UL transmissions on CCs inter-band with the source carrier, UE is allowed to interrupt transmission on inter-band CCs, where number of interruption slots is specified in 38.133.

- Like view 1, dropping rules are applied between source/CCs intra-band with source, and target carrier. Simultaneous transmission between SRS on target and UL transmissions on source and CCs intra-band with source is considered beyond UE’s UL-CA capability, similar to view 1. In addition, prioritization rules shall be applied between target carrier and any CC inter-band with source carrier if simultaneous SRS transmission and UL transmission on that CC is beyond the indicated UE’s UL-CA capability.

In the following, the drawbacks with each of these views are discussed and then we explain our desired solution.

# Prioritization Rules for SRS switching

Let’s start with the first view where to us it is not sufficient to leave cancellation requirements to interruption slots specified in RAN4 spec. Indeed from 38.133, “when SRS carrier based switching is performed between carriers, the UE is allowed interruptions on any active serving cell in SCG if UE is not capable of Per-FR gap”, or on active serving cell(s) in SCG in the same FR as target CC if UE is capable of Per-FR gap, during the switching to/from the target carrier. Having said that, from 38.133 viewpoint, there is no difference between source CC and any other UL carrier (same FR with target for perFR), where UE anyway is allowed to interrupt low priority transmission. If we take view-1, a dual policy is defined for the same UE behavior: while for a low priority transmission on a CC intra-band with source, gNB shall meet cancellation timeline and gNB knows UE’s behavior once timeline is met, UE behavior is left undefined if for any reason UE has to drop a low priority transmission on a CC inter-band with source, as in the latter case no cancellation timeline is defined. For instance, if by the time UE is supposed to transmit SRS (including any interruption due to uplink or downlink RF retuning time), UE is not yet ready to complete cancellation of the concurrent low priority UL channel, it is not known whether SRS can be transmitted, or low priority UL should be continued. Here we should note that under the discussion of SRS carrier switching, while the reason for dropping of an UL transmission on a CC intra-band with source can be different from the reason for dropping of an UL transmission on a CC inter-band with source, if UE has to cancel a low priority UL transmission, the cancellation timeline shall be met by gNB regardless of the cancellation reason.

The drawback of the second view is the lack of precise UE’s capability signaling to indicate which bands within the band combination will be impacted due to SRS carrier switching to the target CC. Consider for example parallelTxSRS-PUCCH-PUSCH within the IE CA-ParametersNR. It is understood that if UE indicates that parallelTxSRS-PUCCH-PUSCH is not supported, then gNB shall make sure SRS transmission on target CC will not collide with a low priority UL PUCCH/PUSCH transmission on a CC inter-band with the source carrier. Now the issue is, if UE indicates by parallelTxSRS-PUCCH-PUSCH to be capable of parallel transmission of SRS and PUCCH/ PUSCH across CCs in an inter-band CA band combination, there is no signalling for UE to indicate whether or not the capability is applicable to the switched band as well.

We think the first view leaves the caveat for simultaneous SRS transmission on target CC and UL transmissions on carriers inter-band with source, if/once UE has to drop the low priority transmission. On the second view, current specification already addresses a similar problem, where UE can indicate capability signalling of antenna switching for the switched band, within the original band combination. We can reuse a similar procedure to address which bands other than the source band are impacted by SRS carrier switching. Thus, we propose the following:

Proposal 1: Support UE to report within a band combination, which bands are (or are not) impacted/interrupted due to SRS carrier switching to a target band.

- In lack of such signalling, gNB shall assume UE is not able to support simultaneous SRS transmission on the target CC and an UL transmission on any another band

# SRS switching and cancellation timeline across carriers

## Cancellation timeline for transmissions on source carrier and intra/inter-band carriers

Dropping an UL transmission is associated with a cancellation timeline which shall be met, otherwise UE cannot cancel the low priority uplink transmission in time. While NR defines cancellation timeline for various scenarios, like intra/inter-UE cancellation for URLLC, cancellation of configured uplink transmission by dynamic SFI or another DCI, etc, current specification lacks the proper definition of cancellation timeline when SRS carrier switching is involved. We should note that without specification of cancellation timeline for the case of SRS carrier switching, gNB does not know the latest time that the UE can drop a low priority channel. Therefore, it cannot schedule the high priority transmission accordingly; and in case UE is not given enough time (which is not specified yet) to cancel the low priority channel, UE behavior becomes undefined.

In RAN1#104b-e, a CR was endorsed in [2] that specifies the required timeline to cancel the low priority transmission on a single carrier c, where c is restricted to be either the source carrier, i.e., the “switching from” carrier, or the target carrier, i.e., “switch to” carrier. Given that with SRS carrier switching, simultaneous UL transmissions on carriers intra (or even inter) band with the source carrier may be impacted, the cancellation timeline needs to be extended to include transmissions on other carriers besides the source carrier. In this contribution, we extend the timeline specification to the general case, where due to SRS switching, carriers that shall be dropped and/or DCIs that schedule high priority transmissions on other carriers, can be more than one.

In the following, we define a cancellation timeline for scenarios where SRS transmission on the target carrier requires simultaneous low priority UL transmissions on some UL carriers (including source CC, and/or carriers intra or inter band with source) to be dropped. Here, the formulation reuses the basics of UCI multiplexing, given by 38.213, Sec. 9.2.5.

Figure 1: Timeline for cancellation due to higher priority SRS transmission on target carrier.

Proposal 2: For the case that aperiodic SRS transmission on the target cell has higher priority than overlapping UL transmissions on other carriers, and the simultaneous transmission is beyond UE’s capability:

- UE does not expect that the gap between the last symbol of DCI indicating A-SRS on target CC and the first symbol of the earliest low priority UL transmission, among a group of overlapping UL transmissions with a priority lower than A-SRS, to be less than Tprocmax, with Tprocmax=max{Tproc1, …,Tproci, …}

where Tproci is based on N2, which itself is determined based on the UE processing capability on the i-th low priority carrier, and the minimum of (µDL, µith-UL), where the µDL corresponds to the SCS of the PDCCH scheduling A-SRS, and µith-UL corresponds to the SCS of the uplink channel on the i-th low priority carrier.

Note: Based on 38.214, Sec. 6.2.1.3, the overlapping includes any interruption due to UL/DL RF retuning time as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR

## Cancellation timeline for SRS on target cell

Following the same line of discussion, we can have a scenario where SRS transmission on the target cell is dropped due to high priority scheduling on other carriers, and simultaneous transmission is beyond UE’s UL-CA capability. An example is given by Figure 2, where UE is configured to switch from C3 (not shown in the figure) to C0 (C0 is DL carrier) for periodic SRS switching, but UE is also dynamically indicated by DCI_1 and DCI_2 to transmit PUSCH on C1 and C2, and simultaneous transmission on (C0, C1, C2) is beyond UE’s capability. Based on current specification, P-SRS on the target cell shall be dropped. Although this cancellation needs a timeline that shall be met, similar to the cancellation timeline for CG PUSCH in current specification.

Figure 2: Timeline for cancellation of SRS transmission on the target carrier.

Proposal 3: For the case that UE is scheduled by a DCI, or a set of DCIs, to transmit a high priority UL transmission on a serving cell overlapping with a low priority SRS transmission on a carrier without configured PUSCH/PUCCH, and simultaneous transmission is beyond UE’s capability:

- UE does not expect the gap between the first symbol of the earliest low priority SRS transmission on the target cell and a last symbol of the last DCI among all DCIs indicating high priority transmissions on another carriers, to be less than Tprocmax, with Tprocmax=max{Tproc1, …,Tproci, …}

where Tproci is based on SRS-SwitchingTime + N2, and N2 is determined based on the UE processing capability on the target carrier, and the minimum of (µith-DL, µUL), where the µith-DL corresponds to the SCS of the PDCCH scheduling a high priority UL transmission on the i-th high priority carrier, and µUL corresponds to the SCS of the SRS on the target cell. SRS-SwitchingTime represents the UL or DL RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR,

## Text proposal for cancellation timeline for SRS Carrier Switching

Based on what we discussed, the following TP is proposed:

[Start of TP, 38.214]

#### 6.2.1.3	UE sounding procedure between component carriers

If an uplink transmission on a carrier configured for PUSCH/PUCCH transmission, is dropped based on prioritization rules defined in this subclause due to overlapping in time with an aperiodic SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on a carrier of a serving cell with slot formats comprised of DL and UL symbols not configured for PUSCH/PUCCH transmission, the UE expects that the first symbol of the earliest of such UL transmissions on carriers configured for PUSCH/PUCCH transmission, is not before a symbol with CP starting after Tprocmax, from a last symbol of the PDCCH indicating aperiodic SRS transmission on a carrier of a serving cell not configured for PUSCH/PUCCH transmission. Tprocmax is given by Tprocmax=max{Tproc1, …,Tproci, …}, where Tproci is determined by N2, and N2 is based on the UE processing capability on the i-th UL carrier with low priority UL transmission, and the minimum of (µDL, µith-UL), where the µDL corresponds to the subcarrier spacing of the PDCCH scheduling aperiodic SRS on a carrier of a serving cell not configured for PUSCH/PUCCH transmission, and µith-UL corresponds to the subcarrier spacing of the low priority uplink channel on the i-th UL carrier.

If an SRS transmission (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR) on a carrier of a serving cell with slot formats comprised of DL and UL symbols not configured for PUSCH/PUCCH transmission, is dropped based on prioritization rules defined in this subclause due to overlapping in time with an uplink transmission on a carrier configured for PUSCH/PUCCH transmission, the UE expects that the first symbol of the low priority SRS transmission on the carrier of the serving cell, is not before a symbol with CP starting after Tprocmax, from a last symbol among all PDCCHs indicating high priority UL transmissions on carriers configured for PUSCH/PUCCH transmission. Tprocmax is given by Tprocmax=max{Tproc1, …,Tproci, …}, where Tproci is based on SRS-SwitchingTime + N2, and N2 is determined based on the UE processing capability on a carrier of a serving cell not configured for PUSCH/PUCCH transmission, and the minimum of (µith-DL, µUL), where the µith-DL corresponds to the subcarrier spacing of the PDCCH scheduling a high priority UL transmission on the i-th UL carrier, and µUL corresponds to the subcarrier spacing of the SRS transmission on a carrier of a serving cell not configured for PUSCH/PUCCH transmission. SRS-SwitchingTime represents the UL or DL RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR,

[text unchanged…]

[End of TP, 38.214]

Proposal 4: Endorse the proposed TP for TS 38.214, 6.2.1.3.

# Conclusions

In this contribution, we shared our views on which bands the priority rules for SRS carrier switching are applied. Next, we discussed why specifying cancellation timeline for sounding between component carriers, that is not limited only to source and target carriers, is necessary. Next, we provided a formulation that is aligned with the rest of specification for timeline cancellation. Based on views shared in this paper, we make the following proposals:

Proposal 1: Support UE to report within a band combination, which bands are (or are not) impacted/interrupted due to SRS carrier switching to a target band.

- In lack of such signalling, gNB shall assume UE is not able to support simultaneous SRS transmission on the target CC and an UL transmission on any another band

Proposal 2: For the case that aperiodic SRS transmission on the target cell has higher priority than overlapping UL transmissions on other carriers, and the simultaneous transmission is beyond UE’s capability:

- UE does not expect that the gap between the last symbol of DCI indicating A-SRS on target CC and the first symbol of the earliest low priority UL transmission, among a group of overlapping UL transmissions with a priority lower than A-SRS, to be less than Tprocmax, with Tprocmax=max{Tproc1, …,Tproci, …}

where Tproci is based on N2, which itself is determined based on the UE processing capability on the i-th low priority carrier, and the minimum of (µDL, µith-UL), where the µDL corresponds to the SCS of the PDCCH scheduling A-SRS, and µith-UL corresponds to the SCS of the uplink channel on the i-th low priority carrier.

Proposal 3: For the case that UE is scheduled by a DCI, or a set of DCIs, to transmit a high priority UL transmission on a serving cell overlapping with a low priority SRS transmission on a carrier without configured PUSCH/PUCCH, and simultaneous transmission is beyond UE’s capability:

- UE does not expect the gap between the first symbol of the earliest low priority SRS transmission on the target cell and a last symbol of the last DCI among all DCIs indicating high priority transmissions on another carriers, to be less than Tprocmax, with Tprocmax=max{Tproc1, …,Tproci, …}

where Tproci is based on SRS-SwitchingTime + N2, and N2 is determined based on the UE processing capability on the target carrier, and the minimum of (µith-DL, µUL), where the µith-DL corresponds to the SCS of the PDCCH scheduling a high priority UL transmission on the i-th high priority carrier, and µUL corresponds to the SCS of the SRS on the target cell. SRS-SwitchingTime represents the UL or DL RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR,

Proposal 4: Endorse the proposed TP for TS 38.214, 6.2.1.3.

# References

- R1-2106100, “Summary of [105-e-NR-7.1CRs-12] Issue#26 SRS carrier switching”, ZTE

- R1-2104043, “Timelines for SRS carrier switching”, Qualcomm, ZTE

- TS 38.306, User Equipment (UE) radio access capabilities, V16.3.0