3GPP TSG RAN WG1 #106-e			R1-2107715

e-Meeting, August 16th – August 27th, 2021

Agenda Item:	7.2.5

Source:	Apple Inc.

Title:	Remaining issues on intra-UE multiplexing/prioritization for eURLLC

Document for:	Discussion/Decision

# 1. Introduction

In this contribution, we discuss the remaining issues on intra-UE multiplexing/prioritization for eURLLC, targeting for a clear common understanding among companies on the procedure.

We had the following working assumption:

Working assumption (RAN1#102-e)

- Multiplexing/overriding/etc. is performed similar to Rel.15 as if HP channels do not exist; this means that LP operations, multiplexing/overriding/etc., are performed before cancellation.

- A UE cancels the transmission of a LP channel including any intermediate scheduled LP transmission that does not overlap with any LP channel, if any DCI schedules an overlapping HP transmission with the LP channel, before performing multiplexing/overriding HP channels if any.

- Multiplexing/overriding of HP channels is performed as if LP channels do not exist.

- A final HP channel is prioritized if it overlaps with a final LP channel, after performing multiplexing of HP channels

In addition, we had the following agreement:

Agreement: (RAN1#101-e)

- If a UE is expected to cancel a scheduled low priority PUCCH/PUSCH due to a first DCI scheduling an overlapping high priority channel, the UE is not expected to transmit the scheduled low priority PUCCH/PUSCH due to a second DCI scheduling PUCCH/PUSCH that is received after the first DCI.

  - Note: The collision between HP PUSCH and LP PUSCH is not covered by this agreement.

However, there are still different understandings regarding which HP channels should be used to cancel LP channels, and there was also concern on the involved UE complexity if the UE needs to perform all the intermediate multiplexing. This was extensively discussed in RAN1#105-e but not concluded [1].

# 2. Intra-UE multiplexing/prioritization

During the email discussion [105-e-NR-L1enh-URLLC-03], the following options were suggested for further consideration [1]:

- Option 2: The UE does not use the outcome of intermediate multiplexing for HP channels to cancel LP channels.

  - Any HP channel that overrides or overlaps with a HP channel that overlaps with a LP channel shall meet the cancellation timeline, namely all HP DCIs must arrive Tproc,2+d1 before the earliest symbol that would be cancelled by the final HP channel.

  - All HP PUCCH/PUSCH channels except the final HP PUCCH/PUSCH that gets transmitted by the UE are intermediate channels.

- Option 3: [No change from the spec is needed.] Clarify that the “before or after” term in Claus 9 in 38.213 is interpreted as:

  - the UE checks overlapping between HP and LP channel for each HP grant it receives, including any intermediate HP channel that results from UCI multiplexing and PUCCH overriding triggered by each of the HP grant.

- Option 3a: [No change from the spec is needed.] Clarify that the “before or after” term in Claus 9 in 38.213 is interpreted as:

  - A UE checks the overlap between a HP channel and a low priority channel before multiplexing. If there is an overlap, the LP channel gets cancelled. If not, a UE performs multiplexing across the HP PUCCH channels.  If then there is an overlap with a LP channel, the LP channel gets cancelled. Then, multiplexing between PUCCH and PUSCH is performed. If then there is an overlap with a LP channel, the LP channel gets cancelled

- Option 4: whether the intermediate HP channels is used to cancel the LP channels is left to UE implementation.

The main concerns about Option 3 are that it causes unnecessary gNB/UE implementation complexity and unnecessary cancellation.

- It complicates both the gNB and the UE implementation in the sense that both the gNB and UE need to do all the intermediate multiplexing (based on HARQ-ACK resource before final overriding) and use the PUCCHs before and after intermediate multiplexing to determine which LP transmissions get cancelled.

- Such a procedure can cause unnecessary cancellation of LP channels by intermediate HP channels (which are not transmitted anyway), resulting in inefficiency from system operation point of view.

For Option 3a, the intention is that the UE only checks the final HARQ-ACK resource after overriding. However, there is a timeline issue associated with it. One such example is shown in Fig. 1. Here HP DCI1 schedules PUCCH1 for HARQ-ACK, which overlaps with LP PUCCH. If there is no more HP DCI later on, HP PUCCH1 should partially cancel LP PUCCH. But there could be a HP DCI2 that schedules PUCCH2 for HARQ-ACK, which overrides PUCCH1 and does not overlap with LP PUCCH any more. According to Option 3a, the UE should transmit LP PUCCH and HP PUCCH2 because it does not consider any intermediate HARQ-ACK PUCCH before the final overriding. This would require the UE to wait until (roughly) N3 before PUCCH1 to determine the final HP PUCCH resource. However, if at this time, the UE determines there is no HP DCI2, the UE would need to transmit HP PUCCH1 and cancel LP PUCCH. But there is no sufficient time for UE to cancel LP PUCCH.

Figure 1 Timeline issue for Option 3a

Option 4 leaves the most flexibility to UE implementation, and the main drawback is that there is ambiguity at the gNB in terms of which LP channels are cancelled.

- Comparing Option 4 to Option 3a, the gNB can implement in the same way as for Option 3a, i.e., using the conservative assumption that all intermediate HP channels can cancel LP channels. With this kind of gNB implementation, the drawback of Option 4 vs Option 3a includes: (1) there may be some extra UL interference caused by not-cancelled LP channels due to UE implementation; (2) the gNB cannot re-schedule other UL transmission on the LP resources because it is not sure whether a LP channel is actually cancelled or not. The impact of both aspects should be minimal and acceptable in our view, because the cancellation is not intended to be a frequent event. If Option 3a is acceptable, we think Option 4 should be (almost) equally acceptable from gNB/network perspective.

- Comparing Option 4 to Option 2, Option 2 still has the clear advantage that the procedure at both gNB and UE is clean, without any unnecessary cancellation. The cost is the additional scheduling constraint in Option 2, which should not cause issues in practice.

For Option 2, the intention is that the gNB/UE are not mandated to perform intermediate multiplexing after receiving each UL DCI. With the timeline in place, one example of how the UE can implement is as follows:

- The LP channels are processed in the same way as in Rel-15.

- The UE performs the multiplexing for HP channels at d1 symbols before the regular HP channel freezing time and checks if any of the HP channels after multiplexing would cancel the LP transmission. If yes, the UE cancels the LP transmission.

  - The additional timeline constraint introduced in Option 2 guarantees that if the UE cancels a LP transmission in this step, there will not be another HP DCI that comes later.

  - If HP cancelling LP occurs in this step, the UE does not need to continue with step 3, because the HP transmission is already final in this case.

  - This is considered as an extra multiplexing step that the UE needs to perform comparing to Rel-15 procedure. However, this step is due to the different cancellation timeline compared to multiplexing timeline, so it is a step that exists in any of the options, not unique to Option 2. (Different UE implementation may perform this step at different time.)

- If in step 2 the UE does not have any HP cancelling LP, the UE continues to process and transmit the HP channels in the same way as in Rel-15.

  - Note that there should not be overlapping HP and LP as the outcome of this step, because any overlapping should have been identified in step 2 already.

This is one example of what UE can do. However, it does not prevent any other UE implementation as long as the outcome is the same. For example, the UE can still perform all the intermediate multiplexing after receiving each DCI. This means that the UE can still keep the existing implementation for multiplexing, such as the time point to check/perform multiplexing (e.g. whether to do it after receiving each DCI, or to do it only at certain time points). The only difference comparing to Option 3 is that the UE does not use all the intermediate HP channels to cancel LP.

In short, Option 2 leaves full flexibility to UE to reuse Rel-15 multiplexing procedure to support Rel-16 intra-UE multiplexing/prioritization feature, and there is no need to change.

Proposal 1: Adopt Option 2 with minor clarification as below:

- Option 2: The UE does not use the outcome of intermediate multiplexing for HP channels to cancel LP channels.

  - Any scheduled HP channel that overrides or overlaps with a HP channel that overlaps with a LP channel shall meet the cancellation timeline, namely all the corresponding HP DCIs must arrive Tproc,2+d1 before the earliest symbol that would be cancelled by the final HP channel.

  - All HP PUCCH/PUSCH channels except the final HP PUCCH/PUSCH that gets transmitted by the UE are intermediate channels.

An example TP is also provided here.

# 9 UE procedure for reporting control information < unchanged part omitted> When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes other than PUCCH transmissions with SL HARQ-ACK reports before considering limitations for UE transmission as described in clause 11.1, including repetitions if any, the UE resolves the overlapping for PUCCH and/or PUSCH transmissions of smaller priority index as described in clauses 9.2.5 and 9.2.6, and resolves the overlapping for PUCCH and/or PUSCH transmissions of larger priority index as described in clauses 9.2.5 and 9.2.6. Then, - if a transmission of a first PUCCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of a transmission of a second PUSCH or a second PUCCH of smaller priority index, the UE cancels the repetition of a transmission of the second PUSCH or the second PUCCH before the first symbol that would overlap with the first PUCCH transmission - if a transmission of a first PUSCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of the transmission of a second PUCCH of smaller priority index, the UE cancels the repetition of the transmission of the second PUCCH before the first symbol that would overlap with the first PUSCH transmission where - - any remaining PUCCH and/or PUSCH transmission after overlapping resolution is subjected to the limitations for UE transmission as described in clause 11.1 - the UE expects that the transmission of the first PUCCH or the first PUSCH, respectively, would not start before Tproc,2 after a last symbol of the corresponding PDCCH reception - Tproc,2 is the PUSCH preparation time for a corresponding UE processing capability assuming d2,1= d1 [6, TS 38.214], based on μ and N2 as subsequently defined in this clause, and d1 is determined by a reported UE capability If a UE - receives a DCI scheduling a first PUCCH of larger priority index that overlaps with a second PUCCH of larger priority index and the second PUCCH overlaps with a third PUCCH or PUSCH of smaller priority index, or - receives a DCI scheduling a first PUCCH of larger priority index that overlaps with a second PUSCH of larger priority index and the second PUSCH overlaps with a third PUCCH of smaller priority index, or - receives a DCI scheduling a first PUSCH of larger priority index that overlaps with a second PUCCH of larger priority index and the second PUCCH overlaps with a third PUCCH or PUSCH of smaller priority index, or - receives a DCI scheduling a first PUCCH for HARQ-ACK of larger priority index that overrides a second PUCCH for HARQ-ACK of larger priority index as described in clause 9.2.3 and the second PUCCH overlaps with a third PUCCH or PUSCH of smaller priority index, the UE expects that the first overlapping symbol of the second PUCCH or PUSCH and the third PUCCH or PUSCH would not start before Tproc,2 after a last symbol of the PDCCH reception for the first PUCCH or PUSCH, where Tproc,2 is the PUSCH preparation time for a corresponding UE processing capability assuming d2,1= d1 [6, TS 38.214], based on μ and N2 as subsequently defined in this clause, and d1 is determined by a reported UE capability. < unchanged part omitted>

# 4. Conclusions

In this contribution, we have discussed the few options for intra-UE multiplexing/prioritization in eURLLC and proposed:

Proposal 1: Adopt Option 2 with minor clarification as below:

- Option 2: The UE does not use the outcome of intermediate multiplexing for HP channels to cancel LP channels.

  - Any scheduled HP channel that overrides or overlaps with a HP channel that overlaps with a LP channel shall meet the cancellation timeline, namely all the corresponding HP DCIs must arrive Tproc,2+d1 before the earliest symbol that would be cancelled by the final HP channel.

  - All HP PUCCH/PUSCH channels except the final HP PUCCH/PUSCH that gets transmitted by the UE are intermediate channels.

# References

[1] R1- 2106359, Summary of [105-e-NR-L1enh-URLLC-03] Email discussion/approval on remaining issues on Scheduling & HARQ enhancements, Moderator (Qualcomm), RAN1#105-e, May 2021.