3GPP TSG- RAN WG1 Meeting #106-e                                                 R1-2107720

# e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.1.2.2

Source:	Apple Inc.

Title:	On Inter-Cell Multi-TRP Enhancement

Document for:	Discussion/Decision

# Introduction

In RAN1 #104b meeting, the following agreement on inter-cell multi-TRP operation have been achieved.

Agreement - For intercell MTRP operation, 1 additional PCI different from the serving cell PCI is supported per CC - The additional PCI is the one associated with one or more TCI states that are activated for [CSI-RS for CSI]/PDSCH/PDCCH, per CC. - Applicable at least for non-cross carrier QCL indication - FFS: Cross carrier scheduling QCL indication - RAN1 to decide on the maximum number of PCIs different from the serving cell PCI per CC and/or across all CCs that can be RRC-configured for multi-DCI based inter-cell multi-TRP - Above should be specified by reusing R15 QCL rules as concluded in RAN1#104-e Conclusion Configuration of CSI-RS for mobility as QCL source for intercell MTRP operation is not supported from Rel-17 specifcation point of view Agreement For intercell MTRP operation, downselect one or more of the following alternatives in RAN1#105-e - Alt1: one PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH can be associated with only one CORESETPoolIndex - Alt2: one PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH can be associated with more than one CORESETPoolIndex - Alt3: one PCI associated with TCI states for [PDSCH]/PDCCH via QCL relationship without association with CORESETPoolIndex Note: This agreement is not related to the down-selection of one of the 5 options from RAN1#104-e Note: Above should be specified by reusing Rel-15/Rel-16 QCL rules as concluded in RAN1#104-e

In this contribution, we provide some discussion on inter-cell multi-TRP enhancement.

# Discussion

## Details on configuration of non-serving cell RS

In RAN1 #104, the following 5 options on TCI and non-serving cell RS association are agreed for further down-selection.

Agreement For inter-cell MTRP operation, further discuss following options and down select in RAN1#104bis-e - Option1: Indicate/associate non-serving cell PCI in the TCI state - FFS other non-serving cell information - Option2: Introduce a flag to indicate whether a TCI state/QCL information is associated with non-serving cell information or serving cell - FFS: how the flag is linked to non-serving cell - Option3: Explicit or implicit grouping of TCI states associated with non-serving cell information corresponding to the serving cell and the non-serving cell respectively. - FFS: Each group is associated with a CORESETPoolIndex value. - FFS: how to link the group of TCI states to non-serving cell. - Option4: Re-index the non-serving cell RS, e.g., in the TCI state/QCL-Info, so that the UE can differentiate between a serving cell RS and a non-serving cell RS - Example: serving cell RSs are indexed from #0, #1, …, #N-1, while non-serving cell RSs are re-indexed from #N, #N+1, … - FFS: detailed re-indexing rule(s) of non-serving cell RSs - Option5: Introduce a new indicator (e.g., re-index the non-serving cell) to indicate the non-serving cell information that a TCI state/QCL information is associated with - FFS: how the indicator is linked to non-serving cell - Note: when there is only one non-serving cell, it means the same as Option2.

Option 1 is to indicate a PCI in the TCI state. But SSB can only be provided in the TCI for CSI-RS, and this may be with higher overhead, since only few cells may be enabled for multi-TRP operation. Option 2, 3 and 5 are quite similar, and the only difference is whether the indication is based on a kind of 1-bit flag or group index or a multi-bit indicator. Option 4 may need additional signalling to maintain the variable N, and may not have a good forward compatibility to support more than 2 TRP.

Proposal 1: For inter-cell multi-TRP operation, support option 2/3/5 to define the association between TCI and non-serving cell information, where an indicator can be used to provide the linkage between non-serving cell information and a TCI

- The TCI with the same indicator should be associated with the same CORESETPoolIndex

## Rate matching

In RAN1 #104, it was agreed that PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around non-serving cell SSB with the same PCI. Thus, the remaining issue is whether the cross-cell rate matching is needed. This should depend on whether UE uses advanced receiver for interference cancellation. Since the non-serving cell SSB may be from different beams and some may create strong interference to the UE. So, it is better to introduce a UE capability on whether the cross-cell rate matching is needed.

Proposal 2: Support to introduce a UE capability to report the following information

- Whether PDSCH /PDCCH from serving cell (PCI) is rate matched around non-serving cell SSB

- Whether PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around serving cell SSB

## PCI and CORESETPoolIndex

In RAN1 #104b, the following options are agreed.

Agreement For intercell MTRP operation, downselect one or more of the following alternatives in RAN1#105-e - Alt1: one PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH can be associated with only one CORESETPoolIndex - Alt2: one PCI associated with one or more of activated TCI states for [PDSCH]/PDCCH can be associated with more than one CORESETPoolIndex - Alt3: one PCI associated with TCI states for [PDSCH]/PDCCH via QCL relationship without association with CORESETPoolIndex Note: This agreement is not related to the down-selection of one of the 5 options from RAN1#104-e Note: Above should be specified by reusing Rel-15/Rel-16 QCL rules as concluded in RAN1#104-e

Among the 3 alternatives, since the inter-cell mTRP is based on Rel-16 mDCI based mTRP, it is straight-forward that each PCI is associated with a CORESETPoolIndex. In Rel-16, CORESETPoolIndex is introduced to identify a TRP, and in inter-cell mTRP, a TRP should be the same as a cell. In addition, since it was agreed that the PCI can be associated with both PDCCH and PDSCH, the bracket for “[PDSCH]” should be removed.

Proposal 3: For PCI and CORESETPoolIndex association, support Alt 1, where one PCI associated with one or more of activated TCI states for PDSCH/PDCCH can be associated with only one CORESETPoolIndex.

## Number of PCIs

In RAN1 #104b, the following options are agreed.

Agreement - For intercell MTRP operation, 1 additional PCI different from the serving cell PCI is supported per CC - The additional PCI is the one associated with one or more TCI states that are activated for [CSI-RS for CSI]/PDSCH/PDCCH, per CC. - Applicable at least for non-cross carrier QCL indication - FFS: Cross carrier scheduling QCL indication - RAN1 to decide on the maximum number of PCIs different from the serving cell PCI per CC and/or across all CCs that can be RRC-configured for multi-DCI based inter-cell multi-TRP - Above should be specified by reusing R15 QCL rules as concluded in RAN1#104-e

In RAN #92, it was agreed that the inter-cell mTRP should focus on Rel-15/Rel-16 TCI framework, and it should be based on the mDCI framework, it is naturally that only 1 additional PCI is needed. Meanwhile, in Rel-16, the SSB from different TRPs can be configured as the QCL source of CSI-RS, the CSI-RS should be included in addition to PDSCH/PDCCH as the target signals to be associated with the non-serving cell PCI.

Proposal 4: Only 1 additional PCI is supported for inter-cell mTRP.

Proposal 5: The additional PCI is associated with the TCI state configured for CSI-RS in addition to PDSCH/PDCCH.

# Conclusion

In this contribution, we discussed inter-cell multi-TRP enhancement. Based on the discussion, the following proposals have been achieved.

Proposal 1: For inter-cell multi-TRP operation, support option 2/3/5 to define the association between TCI and non-serving cell information, where an indicator can be used to provide the linkage between non-serving cell information and a TCI

- The TCI with the same indicator should be associated with the same CORESETPoolIndex

Proposal 2: Support to introduce a UE capability to report the following information

- Whether PDSCH /PDCCH from serving cell (PCI) is rate matched around non-serving cell SSB

- Whether PDSCH/PDCCH from non-serving cell (PCI) associated with TCI state and/or QCL-info is rate matched around serving cell SSB

Proposal 3: For PCI and CORESETPoolIndex association, support Alt 1, where one PCI associated with one or more of activated TCI states for PDSCH/PDCCH can be associated with only one CORESETPoolIndex.

Proposal 4: Only 1 additional PCI is supported for inter-cell mTRP.

Proposal 5: The additional PCI is associated with the TCI state configured for CSI-RS in addition to PDSCH/PDCCH.