3GPP TSG RAN WG1 #106-e	                                         	      R1-2107768

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.14.1

Source:	Apple Inc.

Title:	Remaining issues in XR traffic model

Document for:	Discussion/Decision

# Introduction

At RAN1 #104bis-e, some agreements were made regarding XR traffic models, and there are also some open issues.

At RAN1 #105-e, details for some DL traffic models were agreed, but details for one agreed DL traffic model remains open.

In this contribution we provide our views on those open issues in XR traffic models.

# XR use cases

In SID of Rel-17 XR study, the use cases are enumerated below:

The following applications are to be considered as starting points for this study:

- VR1: “Viewport dependent streaming”

- VR2: “Split Rendering: Viewport rendering with Time Warp in device”

- AR1: “XR Distributed Computing”

- AR2: “XR Conversational”

- CG: Cloud Gaming

Note: Use cases in quotes are from TR26.928.

At RAN1 #103-e, the following was agreed:

Agreement:

XR applications

RAN1 confirms that diverse applications of VR1/2, AR1/2, CG are of interest for study. Potential prioritization/down selection of these applications for evaluation is to be discussed after detailed traffic models and relevant evaluation assumptions are stable.

- FFS: other applications, e.g., XR conferencing

In SA4 traffic modeling details concerning VR2, AR2 and Could gaming are being finalized; and traffic modeling details for VR1 and AR1 will be made available after the SA4 meeting in February 2021. Between AR2 and VR2, much of VR2 modeling details are re-used in AR2, and important modeling details specific for AR2 are also available in SA4’s discussion.

# XR traffic models

## Discussion on traffic models

### Multiple data flows

For XR use cases discussed in SA4, video stream, audio stream, and UE pose/control streams all need to be transmitted or received by the UE.  First thing we note is that those traffics’ periodicities can be different, for example video stream can be generated at 60, 90 or 120 frames per second, but a packet is generated very 20 milliseconds for an audio stream. And their sensitivity to packet loss and latency can be also different, in another word they have different QoS requirements. From that, it is not suitable to lump the traffics for all them in the same data flow, that would force the same treatment for gNB for them. For that reason, it is important to model multiple data flows with different QoS requirements.

In general, the agreements from RAN1 are conducive to the study of multiple flow aspects of XR traffic.

Figure 5 Multiple data flows for XR service

## Review on SA4 traffic models

In SA4 [10][8][9] different approaches are adopted for the modeling of traffic flows, for video streams, trace-based approach has been considered, for other data flows, statistical models or characteristics have been used. SA4 has discussed a number of XR applications, including VR2, AR2 and cloud gaming. For AR2, the overview of SA4 study with many details is provided in Appendix, some salient points of AR2 (XR conversational) are captured below.

### Review on SA4 traffic model for AR2 [10]

| Media | Format and Model | E2E Latency requirement |
| --- | --- | --- |
| 2D Video is split rendering | 1080p or 4K (2 eyes) same model as split rendering for DL | 60ms 100ms |
| 3/6 DOF Pose | Same as for VR2 for UL | UL: 5-10 ms |
| Video + Depth | 1080p, Capped VBR 10/20 Mbit/s for UL | Conversational 100ms, 200ms |
| Front Facing Camera* | 720p, CBR 3 Mbit/s for UL | Conversational 100ms, 200ms |
| Audio (MPEG-H) | 256/512 kbps for both UL/DL | Conversational 100ms, 200ms |
| Data Stream | 0.5 Mbps for both UL/DL | Conversational 100ms, 200ms |

For AR2, in the downlink, there are 3 data flows [8]:

- 2D video (leverage the modeling work of split rendering)

- Audio

- Data stream

For AR2 in the uplink, there are 5 data flows [8]:

- Video + Depth

- Front facing Camera

- 3/6 DOF Pose

- Audio

- Data stream

## Open issues in traffic models

At RAN1 #104bis-e, the following was agreed regarding DL XR traffic models:

Agreement:

In addition to single stream per UE in DL which is baseline, two streams can be optionally evaluated for DL

- Option 1: I-frame + P-frame

  - Option 1A: slice-based traffic model

  - Option 1B: Group-Of-Picture (GOP) based traffic model

- Option 2: video + audio/data

- Option 3: FOV + omnidirectional stream

- Companies should report detailed assumptions in their simulations on packet size distribution for each stream, packet arrival interval (or fps) for each stream, PDB for each stream, PER requirement for each stream, criteria for being satisfied.

- Companies should strive to align the parameter values for the options chosen as much as possible

- FFS: Whether audio stream is separate or aggregated with the data stream in option 2 (Intention of option 2 is not to create a 3 stream option)

At RAN1 #105-e, details on Option 1 were agreed. However, for Option 2, the details remain open.  For Option-2,  the main open issue is the modeling of the audio/data flow. Due to symmetry of audio/data flows for uplink and downlink as communicated by SA4 to RAN1 (Please refer to the table in Section 3.2, also copied below), the model for audio/data flow in the uplink traffic model (Option 3) can be easily reused.

| Audio (MPEG-H) | 256/512 kbps for both UL/DL |
| --- | --- |
| Data Stream | 0.5 Mbps for both UL/DL |

We have

Proposal:

For DL traffic model Option 2, the audio/data flow is modeled with:

- A stream aggregating streams of audio and data

  - Periodicity: 10ms

  - Data rate: 0.756 Mbps/s or 1.12 Mbps

  - Packet size: determined by periodicity and data rate

  - PDB: 30 ms

Also one question was raised during online discussion regarding Option 2: i.e. whether the audio stream is separate or aggregated with the data stream in Option 2. From our evaluation on bundling traffic flows of data/audio stream and the video stream in downlink and uplink [10], visible difference identified in the latency distribution between two transmission strategies:

- With strategy 1, the video stream and the audio/data stream are modelled separately, and a packet with the audio/data stream can be transmitted right away at its arrival at the UE.

- With strategy 2, the transmission of a packet with the audio/data stream always piggybacks on the transmission of a packet of the video stream.

By comparing the simulation results, we would like to see whether merging the audio/data stream and the video stream is feasible or not for DL and UL traffic modeling.  At least between the video stream and the audio/data stream, we see the importance of modelling the video stream and the audio/data stream separately. As can be seen from the SA4 traffic models, for AR2, there are separate flows for audio and data. Due to their similar QoS requirements, we support the merging of the data flow and the audio flow into a single flow in the XR traffic study. We are also open to study capturing the difference between the audio flow and the data flow, as their QoS requirements are similar but not identical.

# Conclusion

In this contribution, we discuss remaining issues in XR traffic models. We have

Proposal:

For DL traffic model Option 2, the audio/data flow is modeled with:

- A stream aggregating streams of audio and data

  - Periodicity: 10ms

  - Data rate: 0.756 Mbps/s or 1.12 Mbps

  - Packet size: determined by periodicity and data rate

  - PDB: 30 ms

# References

- RP-201145, “Revised SID on XR Evaluations for NR”, Qualcomm, RAN #88e, June 29 -July 3, 2020

- TR 38.824, “Study on physical layer enhancements for NR ultra-reliable and low latency case (URLLC)”, March, 2019

- TS 38.300, “NR and NG-RAN Overall Description; Stage 2”, v15.b.0, September 2020

- 3GPP2, “cdma2000 Evaluation Methodology”, 2005

- S4aV200617, “FS_XRTraffic: Parameters for XR conversational”, Sony, Apple, December 2020.

- S4aV200619, “[FS_XRTraffic] Proposed Updated Modelling for AR Conversational”, Qualcomm, December 2020.

- R1-2101765, ”LS to on XR-Traffic Models”, SA4, SA4 #111-e, January, 2021

- S4aV200640, “[FS_XRTraffic] Summary of XR Traffic Models for RAN1 and Open Issues”, Qualcomm Incorporated (Rapporteur), Apple, Sony Mobile Communications, January 2021.

- S4aV200634, “[FS_XRTraffic] 	[FS_XRTraffic] Traces and Configurations for VR2, CG and AR2”, Qualcomm, MediaTek, January 2021.

- R1-2103833, “Initial performance evaluation on XR”, Apple, RAN1 #104b-e, April 2021