3GPP TSG RAN WG1 #106-e	                                         	      R1-2107770

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.14.3

Source:	Apple Inc.

Title:	Performance evaluation on XR

Document for:	Discussion/Decision

# Introduction

At RAN1 #104-e and RAN1 #104bis-e, a number of agreements were reached concerning the evaluation methodology. More specifically, the following were agreed at RAN1 #104bis-e:

Agreement:

In addition to single stream per UE in DL which is baseline, two streams can be optionally evaluated for DL

- Option 1: I-frame + P-frame

  - Option 1A: slice-based traffic model

  - Option 1B: Group-Of-Picture (GOP) based traffic model

- Option 2: video + audio/data

- Option 3: FOV + omnidirectional stream

- Companies should report detailed assumptions in their simulations on packet size distribution for each stream, packet arrival interval (or fps) for each stream, PDB for each stream, PER requirement for each stream, criteria for being satisfied.

- Companies should strive to align the parameter values for the options chosen as much as possible

- FFS: Whether audio stream is separate or aggregated with the data stream in option 2 (Intention of option 2 is not to create a 3 stream option)

Agreement:

For evaluations of AR in UL:

- Option 1 (Baseline for power and capacity evaluations): Two streams as defined below

  - Stream 1: pose/control

    - Traffic model and QoS parameters are same as for pose/control for UL CG/VR.

  - Stream 2: A stream aggregating streams of scene, video, data, and audio.

    - Packet size: Truncated Gaussian distribution with the parameter values same as for DL

    - Periodicity: 60 fps

      - Jitter (optional): same model as for DL

    - Data rate: 10 Mbps (baseline), 20 Mbps (optional)

    - PDB: [60] ms (baseline), [10/15] ms (optional)

- Option 2 (Optional for power evaluation and baseline for capacity evaluation): Single stream as defined below

  - Packet size: Truncated Gaussian distribution with the parameter values same as for DL

  - Periodicity: 60 fps

    - Jitter (optional): same model as for DL

  - Data rate: 10 Mbps (baseline), 20 Mbps (optional)

  - PDB: [60] ms (baseline), [10/15] ms (optional)

- Option 3 (Optional): Three streams as defined below

  - Stream 1: pose/control

    - Traffic model and QoS parameters are same as for pose/control for UL CG/VR.

  - Stream 2: A stream aggregating streams of scene and video

    - Packet size: Truncated Gaussian distribution with the parameter values same as for DL

    - Periodicity: 60 fps

      - Jitter (optional): same model as for DL

    - Data rate: 10 Mbps (baseline), 20 Mbps (optional)

    - PDB: [60] ms (baseline), [10/15] ms (optional)

  - Stream 3: A stream aggregating streams of audio and data

    - Periodicity: 10ms

    - Data rate: 0.756 Mbps/s or 1.12 Mbps

    - Packet size: determined by periodicity and data rate

    - PDB: 30 ms

- Option 4 (Optional): Three streams as defined below

  - Stream 1: pose/control

    - Traffic model and QoS parameters are same as for pose/control for UL CG/VR.

  - Stream 2: I-stream for video

  - Stream 3: P-stream for video

  - Note: For stream 2 and stream 3, the I/P-stream model for DL video can be reused for UL video.  Companies should report detailed assumptions in their simulations on packet size distribution for each stream, packet arrival interval (or fps) for each stream, PDB for each stream, PER requirement for each stream, criteria to be satisfied UE.

  - Companies should strive to align the parameter values for the options chosen as much as possible

- Note: Above PDB values in [ ] for Stream 2 in Option 1 and 3, and Option 2 are to be further discussed and potentially confirmed in RAN1#105-e, where other values can be also discussed if needed.

- In case multiple steams are evaluated for UL AR, a UE is declared as satisfied only when each stream meets the requirement that X (%) of packets are successfully delivered within a given air interface PDB.

  - X value for pose/control: follow X values for pose/control for CG/VR

  - X value for other stream: follow X values for DL video stream.

for XT study, in this contribution we provide initial evaluation on XR.

# Modeling of multiple flows

From SA4’s LS [3][4][5], the following are assumed for XR traffic in the evaluation, other evaluation parameters can be found in Appendix:

Traffic models for DL (Option 2 in DL from RAN1 #104bis-e):

- Video stream:

  - Packet size follows a truncated Gaussian distribution:

    - mean packet size at 30 Mbits /(arrival rate)

    - normalized standard deviation: 0.105

    - normalized maximum packet size: 1.5

    - normalized minimum packet size: 0.5

  - Arrival rate: 60 Hz

  - Arrival offset: randomized

  - Air interface delay budget: 10 ms

- Data/audio stream:

  - Fixed packet size: 1.12 Mbits/(arrival rate)

  - Arrival rate: 100 Hz

  - Air interface delay budget: 30 ms

Traffic models for UL ((Option 3in UL from RAN1 #104bis-e):

- Video stream:

  - Packet size follows a truncated Gaussian distribution:

    - mean packet size at 10 Mbits / (arrival rate),

    - normalized standard deviation: 0.105

    - normalized maximum packet size: 1.5

    - normalized minimum packet size: 0.5

  - Arrival rate: 60 Hz

  - Arrival offset: randomized

  - Air interface delay budget: 10 ms

- Data/audio stream:

  - Fixed packet size: 1.12/(arrival rate) Mbits

  - Arrival rate: 100 Hz

  - Air interface delay budget: 30 ms

- Pose/control stream:

  - Fixed packet size: 800 bits

  - Arrival rate: 250 Hz

  - Air interface delay budget: 4 ms

To model multi-flows of XR traffic, we have considered two approaches.

## Approach 1

- one actual UE is duplicated into multiple virtual UEs (e.g. twin 1, twin 2).

  - Twin 1 and twin 2 are located on the same physical location, and share the same large scale & small-scale channel parameters, and also the same random seeds (e.g.  for polarization modeling), and each virtual UE is associated a single flow.

    - e.g. UE 1 —> UE 1A and UE 1B, UE 1A is associated with a video stream with a delay bound and PER requirement, UE 1B is associated with a data/video stream with a separate delay bound and PER requirement.

  - To avoid simultaneous Tx to the same UE or simultaneous Tx from the same UE, arbitration is applied to UE 1A and UE 1B, so at a given slot, only one of them is scheduled.

  - As long as delay budget is available a packet in a data flow, retransmission will continue. If the delay budget is exceeded, then retransmission stops.

Figure 1 Approach 1 (Twin UEs)

In Figure 2, it shows each PDSCH contains data with a single data flow.

Figure 2 Mapping from traffic flows to respective PDSCHs (Approach 1)

## Approach 2

In Approach 2, multiple data flows can be mapped to the same PDSCH or PUSCH, as shown in Figure 3. In contrast to Approach 1, we don’t need to duplicate UE modeling parameters to create virtual UEs, instead some book-keeping is needed to track the transmission and delay budget for each packet in a PDSCH/PUSCH:

- If multiple packets are mapped to the same PDSCH/PUSCH, as long as one packet’s delay budget is not exceeded, retransmission can continue. If the delay budget for every packet in a PDSCH/PUSCH is exceeded, then retransmission stops.

Figure 3 Mapping multiple SDUs with different data flows to PDSCH (Approach 2)

We have

Observation: multiple flow traffic can be modelled with different approaches.

## Satisfactory UEs

Since for both DL and uplink, there can be multiple data flows, we define a UE is satisfied in DL traffic if packets for each DL data flows are received with at least X% success rate and a UE is satisfied in UL traffic if packets for each UL data flows are received with at least X% success rate.

# Evaluation Results

This section is to be updated upon availability of simulation evaluation.

# References

- RP-201145, “Revised SID on XR Evaluations for NR”, Qualcomm, RAN #88e, June 29 -July 3, 2020

- S4aV200617, “FS_XRTraffic: Parameters for XR conversational”, Sony, Apple, December 2020.

- R1-2101765, ”LS to on XR-Traffic Models”, SA4, SA4 #111-e, January, 2021

- S4aV200640, “[FS_XRTraffic] Summary of XR Traffic Models for RAN1 and Open Issues”, Qualcomm Incorporated (Rapporteur), Apple, Sony Mobile Communications, January 2021.

- S4aV200634, “[FS_XRTraffic] 	[FS_XRTraffic] Traces and Configurations for VR2, CG and AR2”, Qualcomm, MediaTek, January 2021.

# Appendix – simulation assumptions

| Simulation configuration | Value |
| --- | --- |
|  | Dense urban FR1 |
| Layout | 21 cells ISD: 200m |
| Carrier frequency | FR1: 4 GHz |
| Subcarrier spacing | FR1: 30 kHz |
| TDD split | DDDUU |
| BS noise figure | FR1: 5 dB |
| UE noise figure | FR1: 9 dB |
| UE receiver | MMSE-IRC |
| Channel estimation | Realistic |
| UE speed | 3 km/h |
| MCS | Up to 256QAM |
| BS Antenna parameters | Option 2: 32 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,2,2,1,1,8,2) |
| UE antenna parameters | 4 antennas |
| BS Tx Power | FR1: 44dBm per 20 MHz |
| UE Tx Power | FR1: 23 dBm |
| System bandwidth | 100 MHz |
| Transmission Scheme | rank adaptation |
| Scheduler | SU-MIMO PF Scheduler |
| CSI acquisition | Type I single panel codebook |
| UL power control | Alpha =0.6, Po = -75 dBm |
| PHY Processing delay | UE PDSCH/PUSCH processing capability #1 |