3GPP TSG RAN WG1 #106-e                                     R1-2107780

e-Meeting, August 16th – 27th, 2021

Source:	ZTE

Title:	Discussion on timing relationship for IoT-NTN

Agenda Item:	8.15.2

Document for:   Discussion

- Introduction

In RAN#92-e, the work item of NB-IoT/eMTC support for Non-Terrestrial Networks was approved [1], and objective for RAN 1 includes:

Specify the following IoT NTN specific timing relationships enhancements according to Section 8 in TR 36.763:

-	Timing relationships for NB-IoT / eMTC: as listed in Section 6.6.3 in TR 36.763

-	UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD

-	Signalling aspects in UE-specific TA maintenance and reporting, techniques to reduce the signalling load and determination of the UE-specific TA.

In this contribution, timing relationships are discussed based on the study in TR36.763 with detailed views. Furthermore, for IoT-UE, the usage of UE-specific TA and/or K_offset to handle UL-DL collisions is analyzed. Then signalling aspects in UE-specific TA maintenance and reporting are discussed.

- Discussion on remaining time relationships

According to the discussion in IoT-NTN SI, the timing relationships listed in Table 1 in Appendix A have been agreed to be enhanced in normative phase. And following two were discussed without consensus on the necessity for enhancement. Thus, in this section, further discussion is conducted and corresponding signalling mechanism of K_offset, Kmac are also provided.

- PDCCH order to NPRACH (for NB-IoT/eMTC)

According to current specification in the section 16.3.2 and section 6.1.2 of TS 36.213 [2] for NB-IoT/eMTC, no TA impact is assumed for the preamble transmission. However, in IoT-NTN, the large TA needs to be taken into account. Then, in last meeting, enhancement by either introduce K_offset or enlarge the minimum value of parameter of k2 was proposed in SI phase. Given the previous agreement achieved for the identified timing relationships e.g., as listed in Appendix A, introducing K_offset is preferred.

Proposal-1: In NB-IoT/eMTC over NTN, introducing K_offset should be supported to enhance the timing of PDCCH order to PRACH.

- PRACH preamble retransmission (for NB-IoT/eMTC)

For this issue, based on the existing specification in the section 6.1.1 and section 16.3.2 of TS 36.213 [2] for NB-IoT/eMTC, there are two different understandings on the spec based on the discussion of last meeting [3]. For example for NB-IoT, one alternative is assuming that TA is not taken into account and the UE will conduct the preamble retransmission in UE UL n+12 as showed in Figure 1 below with the received RAR ending in subframe n. In this way, the enhancement is needed.

Another alternative is to assume UE receives RAR ending in subframe n. Then, since there is no less than 12 ms in absolute timing line before a new preamble is transmitted in subframe n’+12 as shown in Figure 1.

With consideration that a UE transmits a preamble in actual UL subframe index #, it seems the first understanding is more reasonable. Otherwise, from spec perspective, new definition on slot n’ that align with UE DL slot n should be introduced.

Figure 1 Understanding on timing of PRACH preamble retransmission

Proposal-2: In NB-IoT/eMTC over NTN, enhancement on the timing of PRACH preamble retransmission is needed.

- Signalling of of K_offset, Kmac

For signalling of K_offset, the solution defined in NR-NTN can be reused. As to signalling of Kmac, e.g., for timing of start of RAR window, when eNB DL-UL frame timing is aligned, Kmac = 0. On the other hand, when eNB DL-UL frame timing is not aligned, Kmac is not equal to 0 and signalling of Kmac is needed.

Basically, the initial value of K_offset, Kmac can be used for all timing relationships, if no indication of updating values were received. And the mechanism of updating K_offset in NR-NTN can be reused. From power saving perspective, e.g., for sporadic traffic, it can be handled by configuring a large periodicity as discussed in section 4.

Proposal 3: Reusing the signalling mechanism on indication/updates of K_offset, Kmac defined in NR-NTN should be supported for IoT-NTN.

- UL scheduling for FDD-HD

- 2.1

In this section, it is discussed how to use UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD UE.

In general, if cell/beam specific K_offset is configured for all UEs, the potential collision between the DL reception of scheduling DCI and corresponding UL transmissions can be avoided along with the enabling of other timing relationship of UL scheduling. However, it will lead to the larger latency and lower scheduling efficiency in case of larger beam size, especially for IoT scenario. Then, with enabling of UE-specific TA reporting and configuration of UE specific K_offset, UL-DL collision can be avoided without degradation of performance.

Proposal-4: The UE-specific TA reporting and configuration of UE-specific K_offset to avoid UL-DL collision in FDD-HD UE should be supported in IoT-NTN.

Moreover, the UL-DL collision issue may arise when two HARQ-Processes is configured, according to existing specification in TS 36.213/16.6, it should be considered to adapt impact of introduction of K_offset due to large TA effect. For instance, assume NPUSCH transmission of 1st HARQ process start from subframe n+k+K_offset, for single TB unicast case, a UE can continue receiving 2nd DCI Format N0 before subframe n+k-2+K_offset, and the scheduled NPUSCH of second HARQ process will not exceed UL subframe n+k+255+K_offset. Then with the retained constraint in current specification, the collision between the UL transmission and potential 2nd PDCCH reception can be avoided [4].

Proposal-5: For two HARQ-Processes, introduce K_offset to avoid collision between the UL transmission and potential 2nd PDCCH reception.

- UE-specific TA maintenance and reporting

- TA maintenance

As mentioned in the WID of IoT-NTN [1], TA maintenance mechanism agreed in NR-NTN is the baseline. And it can be observed that the reported TA should be based on the latest applied TA with maintenance, where the maintenance may refer to the full TA and partial TA. For example, it contains UE autonomous TA estimation, common TA indication during a period of time, and TA commands in RRC_CONNECTED state.

- TA reporting

As discussed in companion contribution [5], reporting full TA is preferred. Because if just fractional TA e.g., UE autonomous estimated TA was reported, there would be a problem of mismatch between UE and gNB on the TA calculation, especially with consideration on the TA drift rate. And also if open/closed loop TA maintenance is done, it’s hard to reflect the maintenance influence in a report of partial TA.

Furthermore, as to the frequency of reporting, a TA report before the UL transmission is proper with consideration of typical sporadic traffic. Thus corresponding UE-specific K_offset can be used for the UL scheduling. And unnecessary waste of power consumption due to TA reporting can be avoided.

Proposal-6: Full TA reporting is preferred for the purpose of updating K_offset and avoiding UL-DL collision.

Furthermore, for segment pre-compensation the proper time pre-compensation should be processed for each part of the long transmission, which corresponding to different accurate TA values for each segment. W.r.t TA report in this case, either the first or last TA value applied for corresponding (N)PUSCH segments can be reported [4].

Proposal-7: In case of segment pre-compensation, the value of reported TA can be either the first or last TA values applied at corresponding segment.

- Conclusions

In this contribution, discussion on remaining timing relationship, UL scheduling for FDD-HD, and UE-specific TA maintenance and reporting are conducted, with following proposals:

Proposal-1: In NB-IoT/eMTC over NTN, introducing K_offset should be supported to enhance the timing of PDCCH order to PRACH.

Proposal-2: In NB-IoT/eMTC over NTN, enhancement on the timing of PRACH preamble retransmission is needed.

Proposal 3: Reusing the signalling mechanism on indication/updates of K_offset, Kmac defined in NR-NTN should be supported for IoT-NTN.

Proposal-4: The UE-specific TA reporting and configuration of UE-specific K_offset to avoid UL-DL collision in FDD-HD UE should be supported in IoT-NTN.

Proposal-5: For two HARQ-Processes, introduce K_offset to avoid collision between the UL transmission and potential 2nd PDCCH reception.

Proposal-6: Full TA reporting is preferred for the purpose of updating K_offset and avoiding UL-DL collision.

Proposal-7: In case of segment pre-compensation, the value of reported TA can be either the first or last TA values applied at corresponding segment.

# References

- RP-211601, WID on NB-IoT/eMTC support for NTN, MediaTek Inc., Eutelsat S.A.

- TS 36.213 (E-UTRA); Physical layer procedures.

- R1-2106310, AI8_15_3_FL Summary IoT NTN Timing Relationships, Sony.

- R1-2105195 Discussion on timing relationship for IoT-NTN, ZTE.

- R1-2107775 Discussion on timing relationship for NR-NTN, ZTE.

# Appendix A

Table 1 Time relationships that have identified to need enhancement for IoT-NTN

|  | NB-IoT time relationships | eMTC time relationships |
| --- | --- | --- |
|  | NPDCCH to NPUSCH format 1 | MPDCCH to PUSCH |
|  | RAR grant to NPUSCH format 1 | RAR grant to PUSCH |
|  | NPDSCH to HARQ-ACK on NPUSCH format 2 | PDSCH to HARQ-ACK on PUCCH |
|  | Timing advance command activation | Timing advance command activation |
|  | Start of RAR window | Start of RAR window |
|  |  | MPDCCH to scheduled uplink SPS |
|  |  | CSI reference resource timing |
|  |  | MPDCCH to aperiodic SRS |