3GPP TSG RAN WG1 #106-e	R1-2107802

e-Meeting, August 16th – 27th, 2021

Source:	Sharp

Title:	PUCCH coverage enhancement

Agenda Item:	8.8.2

Document for:	Discussion and Decision

Background

In this paper, we provide views on PUCCH coverage enhancement.

Dynamic PUCCH repetition factor indication

At RAN1#105-e meeting, the following Working assumption was made in AI 8.8.2 for dynamic PUCCH repetition factor indication [1].

Working assumption: In Rel-17, for a PUCCH with associated scheduling DCI, support the following for dynamic PUCCH repetition factor indication. - Enhance RRC signaling to allow configuration of PUCCH repetition factor per PUCCH resource. Reuse Rel-16 PUCCH resource indication mechanism based on “PUCCH resource indicator” (PRI) field and starting CCE index (when applicable based on Rel-16 spec) of DCI to indicate a PUCCH resource and its associated repetition factor. - FFS: RRC signaling enhancement details

At RAN1#104-e meeting, the following agreement was made in AI 8.3.1.1 for IIoT/URLLC [2].

Agreements: Support sub-slot based PUCCH repetition for HARQ-ACK based on the Rel-16 PUCCH procedure for slot-based PUCCH applied to sub-slot based PUCCH - Note: the intention is to take the Rel-16 slot-based PUCCH by replacing with “sub-slot” appropriately, without further optimization unless necessary - FFS whether or not there is any restriction for the applicability of sub-slot based PUCCH repetition for HARQ-ACK - Dynamic repetition indication is supported also for sub-slot based PUCCH in Rel-17 - FFS: if the method to be specified in Cov. Enh WI for slot-based PUCCH repetition can be directly applied to sub-slot PUCCH or if changes are needed

For RRC signaling enhancement details, a new repetition parameter (e.g., repetitionFactor) should be configured per e.g., PUCCH-ResourceTBD-r17 as an additional information of PUCCH-Resource.

Proposal 1: A new repetition parameter (e.g., repetitionFactor) should be configured per e.g., PUCCH-ResourceTBD-r17 as an additional information of PUCCH-Resource.

We need to consider a case that both new repetition parameter and Rel-15/16 repetition parameter (i.e., nrofSlots) are configured. In Rel-16 PUSCH repetition, numerOfRepetitions overrides pusch-AggregationFactor to determine the number of repetitions K. Likewise, we propose the new repetition parameter overrides nrofSlots as shown in Proposal 2.

Proposal 2: the number of repetitions NPUCCHrepeat for a PUCCH transmission corresponding to a PUCCH resource and a PUCCH format is determined as:

    - if the new repetition parameter is configured on the PUCCH resource, NPUCCHrepeat is equal to the new repetition parameter;

    - elseif nrofSlots is configured on the PUCCH format, NPUCCHrepeat is equal to nrofSlots;

    - otherwise, NPUCCHrepeat=1.

For CovEnh, it is enough that the values of the new repetition parameter include 1, 2, 4, and 8, where the value 1 is necessary to cancel repetition by nrofSlots. For IIoT/URLLC, since a sub-slot consists of 2, 6, or 7 symbols, the values of the new repetition parameter should include at least 7, 6, and 2 to align with a slot consisting of 12 or 14 symbols.

Proposal 3: The values of the new repetition parameter should include at least 1, 2, 4, and 8 for the slot-based PUCCH and include at least 2, 6, and 7 for the sub-slot-based PUCCH.

DMRS bundling across PUCCH repetitions

  - Time domain window

At RAN1#105-e meeting, the following agreement was made in AI 8.8.2 for PUCCH enhancement [1].

Agreement: For DMRS bundling for PUCCH repetitions, specify a time domain window during which a UE is expected to maintain power consistency and phase continuity among PUCCH repetitions subject to power consistency and phase continuity requirements. - Strive for common design of the time domain window for PUSCH/PUCCH with DMRS bundling as much as possible.

At RAN1#105-e meeting, the following agreement was made in AI 8.8.1.3 for Joint channel estimation for PUSCH [1].

Agreement: - Definition of the maximum duration: a maximum time duration during which UE is able to maintain power consistency and phase continuity subject to power consistency and phase continuity requirements. - FFS whether or not such a definition is necessary for RAN1 specifications. - Note: whether such a definition is to be specified in RAN4 specifications is up to RAN4. - FFS the maximum duration may be reported by UE. - Note: it is understood that for a UE, the maximum duration is no less than the time domain window duration Agreement: For joint channel estimation for PUSCH repetition type A of PUSCH repetitons of the same TB, down select one of the following alternatives for the time domain window. - Alt 1: All the repetitions are covered by one single time domain window - The start of the window is the first PUSCH transmission - FFS: how to handle non-consecutive physical slots for UL transmission, e.g., due to DL/UL configuration for unpaired spectrum - FFS: frequency hopping and precoder cycling - Alt 2: All the repetitions are covered by one or multiple time domain windows - For the start of each window, - The start of the first window is the first PUSCH transmission. - FFS: how to determine the start of other windows, e.g., whether multiple windows are consecutive or non-consecutive, whether the start of the window depends on DL/UL configuration for unpaired spectrum - For the length of each window, - FFS Each window consists of at least two adjacent physical slots for UL transmission. - The length of each window is no longer than the maximum duration. - FFS: how to determine the length of each window - FFS: whether the length of each window depends on DL/UL configuration for unpaired spectrum - FFS: how to handle non-consecutive physical slots for UL transmission, e.g., due to DL/UL configuration for unpaired spectrum. - FFS: frequency hopping and precoder cycling - Other alternatives are not precluded.

In Alt.1 as in the agreement above, the UE shouldn’t be expected to maintain phase continuity and power consistency within the one single time domain window at least for TDD since it is highly possible that repetitions are allocated in multiple non-continuous uplink resources and frequency hopping cannot be performed within the one single time domain window. For this reason, we support Alt. 2.

Proposal 4: All repetitions should be covered by one or multiple time domain windows.

In Alt.2, several FFSs are remaining as in the following.

- How to determine the maximum length of each window.

- How to handle non-consecutive physical slots for UL transmission.

- How to determine the length of each window.

- How to determine the start of subsequent windows other than the first window.

- Frequency hopping (Section 3.2).

How to determine the maximum length of each window

It was agreed that the length of each window is no longer than the maximum duration. In our view, the maximum duration is a parameter reported by the UE as one of the UE capabilities subject to RAN4.

For a high-speed UE, DMRS bundling gain may be limited if the maximum length of each window equals to the maximum duration. For example, if the maximum length of each window can be set shorter (e.g., 2 or 4 slots), frequency diversity or open-loop spatial diversity (e.g., precoder cycling) can be exploited for the high-speed UE. Therefore, the maximum length of each window should be explicitly indicated by the gNB. Even for a low-speed UE, the maximum length of each window can be set longer to fully exploit DMRS bundling gain.

Proposal 5: Subject to RAN4, the maximum duration is a parameter reported by the UE.

Proposal 6: The maximum length of each window should be explicitly indicated by the gNB.

How to handle non-consecutive physical slots for UL transmission.

Since the number of un-scheduled symbols the UE can keep the phase continuity is still being discussed in RAN4, we shouldn’t conclude how to handle non-consecutive physical slots for UL transmission.

If the non-consecutive slots for UL transmission is handled, the UE has to consider whether DL monitoring occasions exist between the non-consecutive slots. This is because the UE cannot keep phase continuity when DL monitoring occasions exist between the non-consecutive slots for UL transmission. Therefore, the time domain window shouldn’t include DL monitoring occasion. Besides, note that DL monitoring occasions can be dynamically changed by SFI.

Proposal 7: The time domain window shouldn’t include DL monitoring occasion.

How to determine the length of each window.

In a first case that each window cannot include the non-consecutive slots for UL transmission, it is reasonable that each window consists of consecutive physical slots for UL transmission as shown in Figure 1.

In a second case that each window can include the non-consecutive slots for UL transmission, each window can consist of consecutive physical slots for UL transmission and for no DL monitoring as shown in Figure 1. However, whether the UE is required to meet off-power requirement in the consecutive physical slots for no DL monitoring should be discussed in RAN4.

Observation 1: In a first case that each window cannot include the non-consecutive slots for UL transmission, it is reasonable that each window consists of consecutive physical slots for UL transmission.

Observation 2: In a second case that each window can include the non-consecutive slots for UL transmission, each window can consist of consecutive physical slots for UL transmission and for no DL monitoring. However, whether the UE is required to meet off-power requirement in the consecutive physical slots for no DL monitoring should be discussed in RAN4.

How to determine the start of subsequent windows other than the first window.

It was agreed that the start of the first window is the first PUCCH transmission. Likewise, the start of subsequent windows should be the earliest PUCCH transmission after the last window. This is because the UE does not need to keep the phase continuity in a duration PUCCH is not transmitted.

Proposal 8: The start of subsequent windows should be the earliest PUCCH transmission after the last window.

Figure 1: Examples of the one or multiple time domain window when the maximum length is 8 and repetition factor is 8 in TDD operation (DDDSUDDSUU).

  - Frequency hopping

At RAN1#104bis-e meeting, the following agreement was made in AI 8.8.1.3 for joint channel estimation for PUSCH [3].

Agreements: - For inter-slot frequency hopping with inter-slot bundling, down select on the following two options: - Option 1: The bundle size (time domain hopping interval) equals to the time domain window size. - Option 2: The bundle size (time domain hopping interval) can be different from the time domain window size. - FFS: Whether the bundle size (time domain hopping interval) is explicitly configured or implicitly determined. - FFS: Whether/How the bundle size (time domain hopping interval) is defined separately for FDD and TDD. - FFS: relation between the bundle size (time domain hopping interval) and the time domain window size

We discuss Option 1 and Option 2 of the agreement in AI 8.8.1.3 for joint channel estimation for PUSCH of RAN1#104b-e meeting for PUCCH repetition with DMRS bundling.

For comparison of Option 1 and Option 2, Figure 2 shows examples of the relation between the time domain windows and the bundles. In Figure 2, we use a new hopping pattern, not the legacy hopping pattern of Rel-15/16, because the legacy hopping pattern performs hopping per slot (i.e., the bundle size is always 1) and DMRS bundling cannot be applied to two consecutive UL slots.

Figure 2: Examples of the relation between the time domain windows and the bundles when repetition factor is 8 and FDD operation.

In Option 2, the time domain window size can be larger than the bundle size. However, phase continuity and power consistency requirements are not met in a part of the time domain window. To avoid this issue, we support Option 1.

Proposal 9: The bundle size should equal to the time domain window size.

Hopping bundle

Hopping bundle consisting of some slots can be used as the following two options:

- Option 1-a: Alternating hopping pattern which switches hops alternatively at every single bundle (i.e., first hop → second hop → first hop).

- Option 1-b: New hopping pattern other than alternating hopping pattern (e.g., first hop → second hop → second hop).

For TDD operation with a specific TDD pattern (e.g., DDDSUDDSUU), Option 1-a potentially reduces frequency diversity gain as shown in Figure 3. That is, in Option 1-a, each hop has the different number of slots. On the other hand, Option 1-b can maximize frequency diversity gain (i.e., each hop has the same number of slots) by determining the hopping pattern before the window determination. Therefore, to maximize frequency diversity gain, support new hopping pattern other than alternating hopping pattern and support Option 1-b.

Proposal 10: To maximize frequency diversity gain, support new hopping pattern other than alternating hopping pattern.

Figure 3: Hopping patterns for PUCCH repetition with DMRS bundling when a repetition factor is 4 and TDD configuration is DDDSUDDSUU.

Conclusion

In this contribution, we have the following proposal:

Proposal 1: A new repetition parameter (e.g., repetitionFactor) should be configured per e.g., PUCCH-ResourceTBD-r17 as an additional information of PUCCH-Resource.

Proposal 2: the number of repetitions NPUCCHrepeat for a PUCCH transmission corresponding to a PUCCH resource and a PUCCH format is determined as:

    - if the new repetition parameter is configured on the PUCCH resource, NPUCCHrepeat is equal to the new repetition parameter;

    - elseif nrofSlots is configured on the PUCCH format, NPUCCHrepeat is equal to nrofSlots;

    - otherwise, NPUCCHrepeat=1.

Proposal 3: The values of the new repetition parameter should include at least 1, 2, 4, and 8 for the slot-based PUCCH and include at least 2, 6, and 7 for the sub-slot-based PUCCH.

Proposal 4: All repetitions should be covered by one or multiple time domain windows.

Proposal 5: Subject to RAN4, the maximum duration is a parameter reported by the UE.

Proposal 6: The maximum length of each window should be explicitly indicated by the gNB.

Proposal 7: The time domain window shouldn’t include DL monitoring occasion.

Proposal 8: The start of subsequent windows should be the earliest PUCCH transmission after the last window.

Proposal 9: The bundle size should equal to the time domain window size.

Proposal 10: To maximize frequency diversity gain, support new hopping pattern other than alternating hopping pattern.

Observation 1: In a first case that each window cannot include the non-consecutive slots for UL transmission, it is reasonable that each window consists of consecutive physical slots for UL transmission.

Observation 2: In a second case that each window can include the non-consecutive slots for UL transmission, each window can consist of consecutive physical slots for UL transmission and for no DL monitoring. However, whether the UE is required to meet off-power requirement in the consecutive physical slots for no DL monitoring should be discussed in RAN4.

References

- RAN1 chairman’s note, RAN1#105 e-meeting, May 2021

- RAN1 chairman’ note at RAN1#104 e-meeting, February 2021

- RAN1 chairman’s note, RAN1#104bis e-meeting, April 2021