3GPP TSG-RAN WG1 #106-e	R1-2107830

e-Meeting, 16th August – 27th August, 2021

Agenda item:		8.5.6

Source:	MediaTek Inc.

Title:	Potential physical layer impact to the RAN2-led topics

Document for:		Discussion

# Introduction

In this contribution, we provide our views for the following issue,

- The support of SRS transmission under RRC inactive state

# Analysis

The LS from RAN2 states that,

- Note that RAN2 discussed the 2nd priority objectives for UL/UL+DL positioning in RRC_INACTIVE on the configuration in UL positioning and so far, has not reached any conclusion considering it is still open in RAN1 on how positioning SRS should be used for UE in RRC_INACTIVE. RAN2 will continue the work as time permits

In our view, there are several areas requiring RAN1 to make decision. The below gives the list of the potential items,

- SRS usage:

  - Whether SRS could be used in RRC inactive state?

- SRS configuration:

  - Whether SRS could retain same configuration as that in RRC connected state, or needs to be reconfigured when entering RRC inactive state?

  - Which types of SRS resource are supported in RRC inactive state?

  - Both Rel-15 and Rel-16 SRS are supported?

- SRS procedure

  - What is the SRS transmission behaviour when the TAT expires?

In principle, we support SRS transmission in RRC inactive state for the purpose of positioning under potential power saving for UE, since the limited data transmission with frequent positioning operation could be the use case for IIOT scenario. However, to stay in RRC connected state with long DRX is also a feasible option.

RAN2 also agrees that,

- The RRC state of the UE is not exposed to the LMF for INACTIVE UL and DL positioning

- CG-SDT resource can be configured on either initial BWP or separate SDT BWP.  Ask RAN1 to confirm

UE may have started the SRS transmission for positioning in RRC connected state. The serving gNB may send the UE to RRC inactive state when the UE doesn't have frequent and large traffic, and at the same time the UE may continue to transmit SRS for positioning in RRC inactive state.

In our view, the state transition of UE should not disrupt the positioning operation, otherwise UE may simply stay in RRC connected state with longer DRX as a more feasible solution. To keep the flexibility, the indication to the UE may support whether SRS will retain the existing configuration, or will be re-configured after entering RRC inactive state. The RRC release message with SuspendConfig is feasible to contain the indication.

It is also noticed that the RACH resource for RA-SDT and the CG-SDT resource may be configured within the initial BWP in RRC inactive state. If SRS has been transmitted with larger bandwidth than the initial BWP in RRC connected state, it is expected to maintain the same transmission bandwidth in RRC inactive state. As such, the SRS transmission bandwidth and uplink resource bandwidth may need to be decoupled in RRC inactive state.

In RRC inactive state, NW can’t send the DL message to a UE, unless there is UL SDT initialized by the UE. Hence, the aperiodic SRS may not be suitable in inactive state since it requires DCI triggering for each shot. The periodic and semi-persistent transmission is desirable. The activation command for SP-SRS could be considered in RRC release message.

In section 5.2 of 38.321, maintenance of uplink time alignment, MAC identity may notify RRC to release SRS when a TAT expires. The same rule could be followed for SRS transmission in RRC inactive state. Also the anchor gNB which sent the UE to RRC inactive state also knows whether TAT expires, and the anchor gNB may report to LMF about the release of SRS due to TAT expiration of UE.

Proposal 2-1: Support SRS transmission for positioning in RRC inactive state

Proposal 2-2: Support the indication within RRC release with SuspendConfig message to indicate whether to continue using the existing SRS configuration, or to use new SRS configuration in RRC inactive state

Proposal 2-3: Support SRS transmission bandwidth and uplink resource bandwidth to be decoupled in RRC inactive state.

Proposal 2-4: Support periodic and semi-persistent SRS transmission in RRC inactive state

Proposal 2-5: Support Rel-15 and Rel-16 SRS for transmission in RRC inactive state

Proposal 2-6: The activation command of semi-persistent SRS may be contained within the RRC release with SuspendConfig message

Proposal 2-7: The anchor gNB may report to LMF about the release of SRS due to TAT expiration of UE

# Conclusion

Proposal 2-1: Support SRS transmission for positioning in RRC inactive state

Proposal 2-2: Support the indication within RRC release with SuspendConfig message to indicate whether to continue using the existing SRS configuration, or to use new SRS configuration in RRC inactive state

Proposal 2-3: Support SRS transmission bandwidth and uplink resource bandwidth to be decoupled in RRC inactive state.

Proposal 2-4: Support periodic and semi-persistent SRS transmission in RRC inactive state

Proposal 2-5: Support Rel-15 and Rel-16 SRS for transmission in RRC inactive state

Proposal 2-6: The activation command of semi-persistent SRS may be contained within the RRC release with SuspendConfig message

Proposal 2-7: The anchor gNB may report to LMF about the release of SRS due to TAT expiration of UE

# Reference