3GPP TSG RAN WG1 #106-e                                          	       R1-2107923

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.5.4

Source:	Xiaomi

Title:                     Latency improvements for both DL and DL+UL positioning method

Document for:	Discussion and Decision

Introduction

In RAN-91 meeting, the work item of NR Positioning enhancements [1] was approved. The detailed specific objectives on latency improvement can be seen as follows:

- Specify the enhancements of signalling, and procedures for improving positioning latency of the Rel-16 NR positioning methods, for DL and DL+UL positioning methods, including:

  - Latency reduction related to the request and response of location measurements or location estimate and positioning assistance data; [RAN2, RAN3, RAN1]

  - Latency reduction related to the time needed to perform UE measurements; [RAN1, RAN4]

  - Latency reduction related to the measurement gap; [RAN1, RAN4, RAN2]

In RAN1- 105 e-meeting [2], some agreements on latency improvement can be seen as below:

Agreement:

- M-sample (1<=M<4) PRS processing corresponding to measurements performed within M instances of the DL PRS resource set on a PRS resource, subject to UE capability, is beneficial from a RAN1 perspective for latency reduction.

- One sample corresponds to one instance

- Send an LS to RAN4 informing that

  - M-sample (1<=M<4) measurements corresponding to measurements performed within M (1<=M<4) instances of the DL PRS resource set on a PRS resource are beneficial for reduction of measurement latency from RAN1 point of view.

  - RAN4 is requested to check the feasibility of measurements performed within M (1<=M<4) instances of the DL PRS resource set and identify the impact on requirements/side condition.

- RAN1 to further study at least the following aspects for allowing M-sample (1<=M<4) PRS processing

  - Details of UE capability

  - Signaling details, e.g., to indicate whether measurement is based on one or more samples

  - Whether the PRS sample processing time is defined and the relation with (N, T).

    - Note: This may have RAN4 dependency

Agreement:

RAN1 to further study at least the following aspects for MG enhancement with regards to MG requesting and configuration/activation/triggering for the purpose of latency reduction for positioning:

- Preconfiguration of multiple MGs

- Triggering/activation of MG(s) with lower layer signalings (DCI or DL MAC CE)

- Request of MG(s) with lower layer signaling by the UE to the gNB

- Request/determination of MG(s) by LMF indication to the gNB/UE

- Note: The combination of the above items is possible.

Agreement:

- Further study the following options (with the same numerology) to support PRS measurement without MGs for latency reduction in Rel-17

  - Option 1: The PRS is from the serving cell and UE measurement is inside the active DL BWP

  - Option 2: The PRS can be from the serving cell and non-serving cell, and UE measurement is inside the active DL BWP

  - Option 3: The PRS (from the serving cell or non-serving cell) used for UE measurement may extend outside or be completely outside the active DL BWP (including with potentially a different numerology)

  - Note: RAN1 strives not to increase the PRS measurement time compared with Rel-16 MG-based measurement

- The following aspects are FFS

  - PRS processing prioritization window

  - MechanisPRSm to trigger UE DL PRS measurements and report

  - UE/gNB assumptions on processing of DL PRS and other DL physical channels / signals

  - UE DL PRS processing capabilities

- Note: Companies are encouraged to compare the latency benefits of introducing MG-less PRS measurements over MG-based PRS measurements

- Note: Depending on the comparison of latency benefits (and other considerations such as complexity) between introducing MG-less PRS measurements and MG-based PRS measurements, none/one/multiple of the above options should be adopted in Rel-17.

In this contribution, we focus on latency improvements for both DL and DL+UL positioning method.

On-demand PRS transmission and reception

Initiation of on-demand PRS transmission and reception

In order to improve the network efficiency and reduce the latency, on-demand transmission and reception of DL PRS for DL and DL+UL positioning for UE based and UE-assisted positioning solutions will be specified in Rel-17. Different type of on-demand DL PRS should be included for different scenarios, i.e., periodical PRS, Semi-persistent PRS and aperiodic PRS.

Proposal 1: on-demand PRS should support periodical transmission, semi-persistent transmission and aperiodic transmission.

As for the initiation of on-demand PRS transmission, both UE initiated request and LMF (network) initiated request will be specified. For the case of UE initiated, two procedures need to be considered. One is UE initiated request of on-demand DL PRS transmission to LMF, which belongs to RAN2’s work scope. The other one is UE initiated request of on-demand DL PRS transmission to gNB, and some mechanism should be discussed in RAN1. In this contribution, we will mainly focus on the second procedure and discuss on how to initiate the request of on-demand DL PRS transmission.

As for mechanism of UE initiated request to gNB, there are two options.

- Option 1-1: indicate the on-demand PRS transmission request by transmission of random access preamble on PRACH.

  - Both contention free based random access and contention free based random access should be considered. And also both 2-step RACH and 4-step RACH can be used.

  - With this option, UE can further indicate the recommending PRS configuration parameter set ID by PUSCH in Msg 3 or Msg A.

- Option 1-2: indicate the on-demand PRS transmission request by transmission of SR on PUCCH.

  - With this option, UE can further indicate the recommending PRS configuration parameter set ID by PUSCH scheduled by gNB responding to the SR.

While the mapping between PRS configuration parameter set ID and related PRS configuration parameter set need to be known by UE in advance, which can be transmitted to the UE by LMF or serving gNB. And the PRS configuration parameter can include the bandwidth, comsize, number of symbols, periodicity and repetition factor (if periodical PRS and semi-persistent transmission is requested).

But based on the request from UE to gNB, gNB further need to send request to LMF for indicating PRS resource of neighboring TRPs. From this perspective, the latency will be not reduced a lot. Thus we can take it as low priority.

For the DL PRS transmission request initiated by LMF and gNB, here we will focus on the request initiated by gNB for latency reduction. As for the case of initiated by gNB, different type of DL PRS transmission can be initiated by different mechanism.

As for mechanism of gNB initiated request to UE, there are three options.

- Option 2-1: indicate the on-demand DL PRS transmission by RRC signaling.

  - This can be applied for periodical on-demand DL PRS transmission. Configuration parameters for periodical on-demand DL PRS transmission can be configured in RRC signaling.

- Option 2-2: indicate the on-demand DL PRS transmission by MAC CE.

  - This can be applied for semi-persistent on-demand DL PRS transmission. Multiple configuration parameter sets for semi-persistent on-demand DL PRS transmission can be configured in RRC signaling and one of them will be activated by MAC CE.

- Option 2-3: indicate the on-demand DL PRS transmission by DCI.

  - This can be applied for semi-persistent and aperiodic on-demand DL PRS transmission. Multiple configuration parameter sets for semi-persistent and aperiodic on-demand DL PRS transmission can be configured in RRC signaling and one of them will be activated by DCI. If the number of configuration parameter sets is larger than the number of codepoints in DCI, MAC CE can be used to activate some of configuration parameter sets configured in RRC signaling.

As the same time, gNB sends triggering indication of on-demand DL PRS transmission to neighboring gNB directly or through LMF.  After above procedure, measurement for positioning can be started.

Proposal 2: gNB initiated of on-demand PRS transmission can be supported by RRC, MAC CE and DCI.

PRS measurement report transmission

After PRS measurement, UE need to report the measurement results. In order to reduce the latency, we suggest that UE sends measurement report to gNB. Because of the higher overhead of the measurement report, it is better to send it by PUSCH. Both configured grant PUSCH and DCI scheduled PUSCH can be supported. For example, periodical PRS measurement report can be sent by configured grant PUSCH. While semi-persistent and aperiodic PRS measurement report can be sent by DCI scheduled PUSCH. And configured grant PUSCH can be triggered by RRC only or RRC+DCI. While for DCI triggered PRS measurement report, the configuration parameters of PRS can be indicated by the same DCI.

Proposal 3: Support PRS measurement report by PUSCH including configured grant PUSCH and dynamic grant PUSCH.

Measurement gap for positioning

In Rel-16 specification, if the DL PRS resource locates on a BWP which is outside the active DL BWP or with a numerology different from the numerology of the active DL BWP, the UE is not able to measure such PRS resource unless measurement gap is configured. While in current specification, the measurement gap appears with periodicity value from 20ms to 160ms, and with gap length value from 1.5ms to 6ms. And during measurement gap, the UE can only perform measurement of PRS, the data transmission or reception on the active BWP must be interrupted. Such data interruption and latency cannot be tolerable for some service. But performing PRS measurement within active DL BWP may decrease the positioning accuracy because of the narrow bandwidth. In this section, we will discuss on enhancement to reduce latency related to measurement gap.

On-demand measurement gap

Currently the measurement gap can only be configured by RRC signaling and the measurement time appears periodically. In order to reduce the latency, MAC CE or DCI can be used to trigger the on-demand measurement gap. For example, the configuration parameter set(s) are configured through RRC signaling and one of them will be activated or indicated by MAC CE or DCI. As for DCI, paging can be reused as well as DCI format for PUSCH allocation. And the same DCI for PUSCH allocation can be used to indicate the PUSCH resource for measurement report. In order to support the flexibility and reduce the signaling overhead, we suggest to use the CSI report like mechanism, which means to associate a state ID with a PRS configuration, a measurement gap configuration and a PRS measurement report configuration, and MAC CE or DCI can activate/deactivate or triggering the PRS measurement and measurement report by indicating a state ID.

Proposal 4: Support triggering of on-demand measurement gap by MAC CE or DCI.

Proposal 5: Suggest to associate a state ID with a PRS configuration, a measurement gap configuration and a PRS measurement report configuration, and MAC CE or DCI can activate/deactivate or trigger the PRS measurement report by indicating a state ID.

Gap-less measurement for positioning by BWP switching

During the measurement gap, the data transmission or reception on the active BWP must be interrupted. In order to solve this problem, it is possible to switch the active BWP to the positioning BWP. It means to take the positioning BWP as a new active BWP. In this case, the UE can perform both data reception and PRS measurement on the same active BWP. But the disadvantage is that if there is only a small data for that UE, some resource waste will be introduced since the granularity of frequency domain resource assignment in DCI is much larger with large BWP size. In addition, UE need to monitor the larger bandwidth of positioning BWP even on the time of no PRS transmission.

There are three options listed in last RAN1-meeting for further study to support PRS measurement without MGs. As for Option 1 and Option 2, since PRS is inside the active BWP, we think PRS measurement can be supported without MG or BWP switching except some cases such as that: DL PRS Resources on serving or neighboring cells on symbols are indicated as UL by the serving cell or DL PRS in the same OFDM symbol where other DL signals and channels are transmitted to the UE. While for Option 3, since the PRS extend outside or be completely outside the active DL BWP, we think PRS measurement can’t be supported if without MG or BWP switching. Thus BWP switching should be used for Option 3. After BWP switching, there will still be same problem raised in Option 3 as that in Option 1 and Option 2. As for the case of that DL PRS in the same OFDM symbol where other DL signals and channels are transmitted to the UE, it can be solved by priority definition between PRS and other DL signals/channels. For the case of that DL PRS Resources on serving or neighboring cells on symbols are indicated as UL by the serving cell, it is necessary to define the UE behavior to drop the PRS or receive the PRS.

Proposal 6: to discuss the UE behavior when PRS resource on symbols are indicated as UL or used by other DL signals or channels transmission with gap-less measurement for positioning.

Panel specific measurement gap

For UE with more than one panel, it is able to perform data reception and PRS measurement by different UE panels. And different panel works on different DL BWP. In this case, if two BWPs related to a same CC, it need to configure one or two active DL BWPs for a UE on a same CC. if two BWPs related to different CC, there is only one active DL BWP on each CC. If two active DL BWPs on a CC are configured, the DL BWP for data reception is the primary active DL BWP and the PRS BWP is the secondary active BWP. It is necessary to specify the configuration of two active BWPs. With these methods, the PRS can be transmitted/received by different panel with the service data. But the disadvantage is that the panel for data reception cannot be used for PRS measurement at the same time.

Proposal 7: Consider of simultaneous reception of PRS and data by different panel for MPUE by panel specific measurement gap.

Priority definition for PRS

Dynamically switching between data BWP and positioning BWP can reduce the interruption time, But the disadvantage of BWP switching for positioning measurement is that UE need to monitor the larger bandwidth of positioning BWP even at the time of no PRS transmission, which will introduce high power consumption. In addition, if there is only a small data for that UE, some resource waste will be introduced since the granularity of frequency domain resource assignment is much larger with large BWP size. And for panel specific measurement gap, it can only be used for MPUE. It means that for UE with high capability, it can simultaneously support data BWP and positioning BWP with different RF respectively. But for UE with low capability, either data interruption or high power consumption will be experienced.

In Rel-16, UE is not expected to process DL PRS in the same OFDM symbol where other DL signals and channels are transmitted to the UE. Considering the latency of positioning measurement and data transmission, it is better to support transmission of PRS and other signals/channels in the same OFDM symbol. Thus it is necessary to define the priority of data and PRS. If the PRS is configured for positioning with short latency, the PRS should indicated as higher priority. Else, the PRS can be indicated as lower priority. If there is resource collision between PRS and other signals/channels, dropping will be performed according to the priority rules.

Proposal 8: The priority of PRS should be differentiated for different latency requirement.

Beam management for positioning

In order to perform positioning PRS measurement with short latency, it is better to perform beam management before positioning PRS configuration. Considering PRS measurement bandwidth, it may need to perform beam management of non-active BWP, beam management of Multi-TRP and beam management of neighboring cell. These beam managements will result in high signaling overhead, long latency and high power consumption.

For beam management of non-active BWP, it is possible to reuse the beam management results of active BWP within the same TRP. Thus only beam management on active BWP of Multi-TRP and/or neighboring cell is needed.

As for beam management of neighboring cell, both UE based and gNB based beam measurement should be supported for neighboring cell beam management. For UE based beam measurement, it is preferred to configure cell specific reference signal for beam management of neighboring cell, e.g., SSB. In this case, the signaling overhead in neighboring cell and beam management latency can be reduced. And it is also better to configure a cell specific reference signal with smaller transmission period. While for neighboring gNB based beam measurement, UE need to transmit the beam measurement reference signal, such as SRS. It is preferred to reuse the SRS for other use cases if possible, for example, reuse beam management SRS for serving cell. In this case, there will be no more impact on UE side. It is the neighboring gNB that measures the beam management reference signal and send the measurement report to the UE’s serving gNB.

In order to improve the positioning accuracy, it is very important to find the LOS path or the first arrival path. Since only the first arrival path can indicate the accurate distance between the UE and the TRP. But during beam management procedure, it is possible that the beam with the strongest L1-RSRP/L1-SINR is not the one refer to the first arrival path. Thus the beam refer to the first arrival path may not be included in the beam measurement report. If gNB configure PRS resources based on beam measurement report, there may be no PRS resource with beam refer to the first arrival path. That will decrease the positioning accuracy. If gNB configure more PRS resources, e.g., based on gNB Tx sweeping, the signaling overhead and the latency for positioning measurement will be increased. In order to solve this problem, we propose to find the first arrival path during beam management procedure and report to the gNB. For example, the arrival time of each beam can be included in the beam measurement report. Based on the enhanced beam measurement report, gNB can configure PRS resources with early arrival time. Thus the latency for positioning measurement will be reduced.

Proposal 9: To indicate the first arrival path by reporting the arrival time of each beam in beam measurement report.

Conclusion

In this contribution, we discussed about latency improvements for both DL and DL+UL positioning method. Based on above discussion, we provide the following proposals.

Proposal 1: on-demand PRS should support periodical transmission, semi-persistent transmission and aperiodic transmission.

Proposal 2: gNB initiated of on-demand PRS transmission can be supported by RRC, MAC CE and DCI.

Proposal 3: Support PRS measurement report by PUSCH including configured grant PUSCH and dynamic grant PUSCH.

Proposal 4: Support triggering of on-demand measurement gap by MAC CE or DCI.

Proposal 5: Suggest to associate a state ID with a PRS configuration, a measurement gap configuration and a PRS measurement report configuration, and MAC CE or DCI can activate/deactivate or trigger the PRS measurement report by indicating a state ID.

Proposal 6: to discuss the UE behavior when PRS resource on symbols are indicated as UL or used by other DL signals or channels transmission with gap-less measurement for positioning.

Proposal 7: Consider of simultaneous reception of PRS and data by different panel for MPUE by panel specific measurement gap.

Proposal 8: The priority of PRS should be differentiated for different latency requirement.

Proposal 9: To indicate the first arrival path by reporting the arrival time of each beam in beam measurement report.

References

- RP-210903 Revised WID on NR Positioning Enhancements

- RAN1-105 e-meeting chairman notes.