3GPP TSG-RAN WG1 Meeting #106-e		R1-2107956

e-Meeting, August 16th – 27th, 2021

Title:	[DRAFT] Reply LS on synchronous operation between Uu and SL in TDD band

Response to:	R4-2108114

Release:	Rel-17

Work Item:	NR_SL_enh-Core

Source:	vivo [RAN1]

To:	RAN4

Cc:

Contact person:	Zichao Ji

zichao.ji@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

RAN1 thanks RAN4 for the LS on the following two synchronous operation options between Uu and SL in TDD band.

Option 1: To follow the Rel-16 agreement to align SL transmission timing with DL timing.

Option 2: To reconsider SL transmission timing to align with UL timing to mitigate the interference between Uu and SL, i.e.

- For sidelink transmissions,

  - SL transmission timing is aligned with Uplink timing when Uu and sidelink is TDMed/FDMed coexistence in the same band, including TDM coexistence within the same carrier or different carriers.

  - Otherwise, SL transmission timing is aligned with Downlink timing.

From RAN1 perspective, Option 2 has several issues. Firstly, with Option 2, UL timing is only available for in-coverage UEs in RRC connected mode. Consequently, these UEs cannot communicate with UEs in idle or inactive mode. Secondly, it would introduce a backward compatibility issue between Rel-16 and Rel-17 UEs in SL communication because different transmission timings are applied. Therefore, RAN1 recommends RAN4 to accept Option1, i.e., following the Rel-16 agreement to align SL transmission timing with DL timing.

2	Actions

To RAN4

RAN1 respectfully asks RAN4 to take the above information into account in related work.

3	Dates of next TSG-RAN WG1 meetings

TSG RAN WG1 Meeting #106bis-e			October 11th – 19th, 2021 			e-meeting

TSG RAN WG1 Meeting #107-e			November 11th – 19th, 2021   			e-meeting