3GPP TSG RAN WG1 #106		R1-2107973

e-Meeting, August 16th – 27th, 2021

Source:	vivo

Title:	Discussion on CSI-RS multiplexing with PDSCH scheduled by PDCCH with CRC scrambled by P-RNTI and SI-RNTI

Agenda Item:	7.1

Document for:	Discussion and Decision

- Discussion

In 38.211, the following specification defines the UE rate matching behavior when CSI-RS is multiplexed with PDSCH. From the definition, UE does not expect to be rate matched around CSI-RS for PDSCH scheduled by PDCCH scrambled by P-RNTI and SI-RNTI.

The UE shall, for each of the antenna ports used for transmission of the physical channel, assume the block of complex-valued symbols yp0, …, yp(Msymbap-1) conform to the downlink power allocation specified in [6, TS 38.214] and are mapped in sequence starting with yp0 to resource elements k',lp,μ in the virtual resource blocks assigned for transmission which meet all of the following criteria: - they are in the virtual resource blocks assigned for transmission; - the corresponding physical resource blocks are declared as available for PDSCH according to clause 5.1.4 of [6, TS 38.214]; - the corresponding resource elements in the corresponding physical resource blocks are - not used for transmission of the associated DM-RS or DM-RS intended for other co-scheduled UEs as described in clause 7.4.1.1.2; - not used for non-zero-power CSI-RS according to clause 7.4.1.5 if the corresponding physical resource blocks are for PDSCH scheduled by PDCCH with CRC scrambled by C-RNTI, MCS-C-RNTI, CS-RNTI, or PDSCH with SPS, except if the non-zero-power CSI-RS is a CSI-RS configured by the higher-layer parameter CSI-RS-Resource-Mobility in the MeasObjectNR IE or except if the non-zero-power CSI-RS is an aperiodic non-zero-power CSI-RS resource; - not used for PT-RS according to clause 7.4.1.2; - not declared as 'not available for PDSCH according to clause 5.1.4 of [6, TS 38.214].

However in 38.214, it is only clarified in 38.214 that CSI-RS and SIB1 message does not overlap as following.

The UE is not expected to receive CSI-RS and SIB1 message in the overlapping PRBs in the OFDM symbols where SIB1 is transmitted.

Based on the specification, there are following two different kinds of interpretations:

- Interpretation1: the related UE behavior for CSI-RS and Paging (or system information) PDSCH multiplexing was not discussed in Rel-15, thus such multiplexing is unspecified, and UE does not expect such overlapping.

- Interpretation2: the specification does not preclude CSI-RS and Paging (or system information) PDSCH reception in overlapping PRBs, thus UE needs to be prepared for such overlapping.

This may cause the following misaligned behavior:

- If the gNB holds interpretation2, while UE holds interpretation1, UE would be confused on Paging (or system information) PDSCH scheduling in overlapped PRBs with CSI-RS. UE does not know whether to receive the CSI-RS or the PDSCH.

During last meeting discussion, there are indeed two different kinds of interpretations from companies. Thus it is better to clarify the UE behavior regarding CSI-RS multiplexing with PDSCH scheduled by SI-RNTI and P-RNTI. The following conclusion was supported by most companies during email discussion and can be used as the starting point.

Proposal 1: For Rel-16 NR, conclude that the overlapping between broadcast PDSCH and CSI-RS is allowed by the NW and it is up to UE implementation on how to receive CSI-RS and PDSCH when they are overlapped.

- Conclusion

In this contribution, the following is proposed to clarify multiplexing behavior between CSI-RS and PDSCH.

Proposal 1: For Rel-16 NR, conclude that the overlapping between broadcast PDSCH and CSI-RS is allowed by the NW and it is up to UE implementation on how to receive CSI-RS and PDSCH when they are overlapped.

- References