3GPP TSG-RAN WG1 Meeting #106-e	R1-2108045

e-Meeting, August 16th – 27th, 2021

Agenda Item:	8.1.2.3

Source:	Ericsson

Title:	On beam management enhancements for multi-TRP

Document for:	Discussion

# 1	Introduction

The Rel-17 feMIMO WID [1] objective 2c states:

- Evaluate and, if needed, specify beam-management-related enhancements for simultaneous multi-TRP transmission with multi-panel reception

In this paper, we provide Ericsson’s views related to this objective considering the progress made until RAN1#105-e.

# 2	Discussion

RAN1#104-e, the following agreement was made with respect to beam measurement/reporting enhancements:

Agreement

For beam measurement in support of M-TRP simultaneous transmission

- Support a single CSI-report consisting of N beams pairs/groups and M (M>1) beams per pair/group, and different beams within a pair/group can be received simultaneously

  - Support M = 2

  - Support extending the maximum value of N > 1, exact value FFS

  - N=1 and N=2

    - FFS: Other values larger than 2

    - FFS: Whether the UE could report beams are received with different RX beams

- Further study the support of option 1 and option 3

- The above applies at least for L1-RSRP

  - FFS: L1-SINR

One of the issues that is still open is whether to support Option 1 (UE reporting N>1 pair/groups and M>=1 beams per pair/group in a CSI report) and Option 3 (UE reporting M>=1 beams in N >1 CSI-reports corresponding to N report setting) for beam measurement/reporting enhancements.  Given that Option 2 is already agreed, we do not prefer introducing multiple options as that would lead to higher specification impact, potential market fragmentation and more complicated UE capabilities in NR Rel-17.  Hence, we make the following proposal:

For beam measurement/reporting enhancement to facilitate inter-TRP beam pairing, Option 1 (UE reporting N>1 pair/groups and M>=1 beams per pair/group in a CSI report) and Option 3 (UE reporting M>=1 beams in N >1 CSI-reports corresponding to N report setting) are not supported in NR Rel-17.

In the above agreement, it was not clarified if the different beams within a pair/group can be received simultaneously with different Rx spatial filters.  It should be kept in mind that objective 2c in the WID [1] clearly states that this enhancement is for ‘simultaneous multi-TRP transmission with multi-panel reception’.  As shown in the results of our previous contribution [2], significant system-level performance gains are observed from enhancing group-based reporting for the simultaneous multi-TRP transmission with multi-panel reception use case to ensure 4 layers reception in FR2.  However, if the two beams from a pair/group are received with the same Rx spatial filter at the UE, 4 layer reception may not be possible and there may not be peak throughput improvement.  Hence, simultaneous reception of the two beams from a pair/group with different Rx spatial filters is needed in order to see benefits of this enhancement.  In NR Rel-16, the following UE capability was introduced for simultaneous reception with different QCL Type D assumption, which allows the UE to receive with two different receive beams the signals transmitted from two different TRPs:

| simultaneousReceptionDiffTypeD-r16 Indicates whether the UE supports simultaneous reception with different QCL Type D reference signal as specified in TS38.213 [11]. | Band | No | N/A | FR2 only |
| --- | --- | --- | --- | --- |

It is quite natural to assume that a UE that supports the above capability can simultaneously receive the beams transmitted from within a pair/group with different receive beams.  Hence, a new UE capability is not needed in NR Rel-17 to indicate whether a UE could report that the beams within a pair/group are received with different RX beams.

Reuse simultaneousReceptionDiffTypeD-r16 UE capability to indicate if the UE is capable of receiving beams within a beam pair/group with different Rx spatial filters.

With regards to the issue of UE panel/antenna related feedback, the following proposal is captured in [3] although there was no agreement in RAN1#105-e.

Offline proposal 1.2.1:

On reporting of information related to UE Rx panel/antenna group for beam measurement/reporting option 2, further study and decide if any of the following alternatives for additional UE indication in the report is to be supported in Rel.17 in RAN1#106b-e

- Alt-1.0: UE reports UE panel ID per CMR within a group/pair

- Alt-1.1: if reported beams within a beam group/pair are associated to the same or different RX spatial filters

- Alt-1.2: maximum number of supported layers per CMR within a group/pair

- Alt-1.3: whether two beams within a group/pair can be used for spatial multiplexing or diversity

In general, the notion of panel ID is an implementation issue and hence we prefer to avoid specification of panel IDs or antenna-group IDs.  Furthermore, whether to specify panel ID or not is being discussed in the multi-beam agenda, and hence we prefer not to duplicate the discussion in this agenda.  Having said that there is a need for the network to know whether two beams in the group-based beam report were simultaneously received by the UE using different RX spatial filters.  In current specifications TS 38.214, the following is captured with regards to spatial RX filter assumption for group-based beam reporting:

-	if the UE is configured with the higher layer parameter groupBasedBeamReporting set to 'enabled', the UE is not required to update measurements for more than 64 CSI-RS and/or SSB resources, and the UE shall report in a single reporting instance two different CRI or SSBRI for each report setting, where CSI-RS and/or SSB resources can be received simultaneously by the UE either with a single spatial domain receive filter, or with multiple simultaneous spatial domain receive filters.

According to the yellow highlighted part above, the UE may use either a single spatial domain RX filter or multiple simultaneous spatial domain filters to receive the two beams in each group.  If the UE uses a single spatial domain RX filter for both reported beams in each group, it will not be possible for the gNB to simultaneously scheduled PDSCH from two TRPs with rank higher than 2.   If 4 layer reception is not possible, peak throughput improvement cannot be achieved using this feature as demonstrated by our results in [2].  Alt-1.1 in the above proposal provides the information needed by the gNB so that the gNB can decide whether or not to schedule simultaneous PDSCH transmission from two TRPs.  In general, it is sufficient that the UE indicates if the two beams in each group were received with same or different Rx spatial filters.

In case the UE uses the same UE panel for both reported beams within a beam group it will not be possible for the gNB to perform NC-JT from the two TRPs with rank higher than 2 (assuming that each UE panel has maximum two RX chains). However, the gNB is not aware of this, and might therefore try to schedule the UE with further CSI-reporting from both TRPs to facilitate NC-JT, even though it is not possible, which causes unnecessary signaling overhead.

For potential UE panel related information feedback for group-based beam reporting, support the indication of whether the reported beams in a group-based beam report are associated with same or different RX spatial filters (i.e., support Alt-1.1).

In RAN1#105-e, the following offline proposal with regards to simultaneous configuration of RACH based BFR and TRP specific BFR was captured in [3]:

Offline proposal 2.1.1:

- Discuss whether simultaneous configuration of RACH based BFR and TRP-specific BFR on at least the SpCell is supported

  - In case of two TRPs failure for TRP-specific BFR on SpCell, RACH-based BFR can be triggered.

  - Note: if two sets of BFD-RS for TRP-specific BFR are configured on the SpCell, there is no additional configured BFD-RS for RACH-specific BFR on the SpCell.

  - FFS: above RACH-based BFR refers to CFRA-based cell-specific BFR and/or CFRA-based cell-specific BFR on SpCell

In our view, RACH based BFR and TRP-specific BFR are independent configurations, and in case beam failure is detected in both TRPs, the UE can fall back to RACH based BFR.  Hence, we propose the following:

In NR Rel-17, simultaneous configuration of RACH based BFR and TRP-specific BFR in the SpCell is not forbidden.

In RAN1#104bis-e, the following agreement was reached with regards to M-TRP BFD:

Agreement

- Support S-DCI and M-DCI in TRP-specific BFR in Rel.17

  - S-DCI is low priority, M-DCI is high priority

  - Unified design for S-DCI and M-DCI should not be precluded due to the prioritization

Regarding explicit and implicit BFD-RS configuration, both these configurations are supported in NR Rel-15 and Rel-16. If the explicit BFD-RS configuration is extended to M-TRP beam failure recovery operation, multiple BFD-RS sets will need to be configured via higher layer signaling. This extension seems straightforward. On the other hand, with implicit BFD-RS configuration, the BFD-RS sets can be determined implicitly via the activated TCI states of the CORESETs associated with different TRPs. Since the activated TCI states for the CORESETs can be updated via MAC CE, the implicit approach provides more flexibility with regards to updating the RSs in the BFD-RS set.  Our preference is to support both explicit and implicit BFD-RS configuration in NR Rel-17.

In NR Rel-17, support both explicit and implicit BFD-RS configuration in order to support M-TRP beam failure detection.

With regard to supporting explicit and implicit M-TRP BFD-RS configuration, a further open issue is whether to support one or both of multi-DCI based multi-TRP and single-DCI based multi-TRP.  In the case of explicitly configured BFD-RS resource sets, these can be used to support M-TRP beam failure detection for both multi-DCI based M-TRP and single-DCI based M-TRP.  Hence, we propose the following:

In NR Rel-17, support explicit M-TRP BFD-RS configurations for both multi-DCI based M-TRP and single-DCI based M-TRP.

In RAN1#105-e, some companies proposed to support implicit per-TRP BFD-RS configurations only for multi-DCI based multi-TRP.  The reasoning given is that the already specified CORESETPoolIndex (e.g., k = 0 or 1) can be leveraged when determining BFD-RS sets for multi-DCI based multi-TRP.  For instance, the BFD-RSs in BFD-RS sets #1 and #2 can be determined via the activated TCI states of the CORESETs associated with CORESETPoolIndex values 0 and 1, respectively.  On the other hand, for single-DCI based multi-TRP, supporting implicit per-TRP BFD-RS configurations would require introduction of a CORESET group identifier or something similar.  In terms of standardization effort, it appears that supporting implicit per-TRP BFD-RS configurations only for multi-DCI based multi-TRP requires less effort.  Hence, our preference is to support only multi-DCI based multi-TRP for implicit per-TRP BFD-RS configurations in NR Rel-17.

In NR Rel-17, support implicit M-TRP BFD-RS configurations only for multi-DCI based M-TRP.

In NR Rel-16, multi-DCI based multi-TRP operation was limited to intra-cell operation (i.e., both TRPs correspond to the same PCI).  In NR Rel-17, multi-DCI based multi-TRP operation is being extended to inter-cell operation (i.e., both TRPs correspond to the different PCIs).  Hence, when introducing per-TRP BFD-RS configurations in Rel-17, it is beneficial to support both intra-cell and inter-cell multi-DCI based multi-TRP operations.

In NR Rel-17, support per-TRP BFD-RS configurations for both intra-cell and inter-cell multi-DCI based multi-TRP operation.

Agreement

For PDCCH reliability enhancements, support SFN scheme + Alt 1-1.

- FFS: TCI state activation for CORESET, impact on default beam, BFD resource for BFR

In RAN1#103-e, SFN based PDCCH reliability was agreed.

In the above scenario, a CORESET will be activated with two TCI states.  Then, the open question is which TCI state the UE should use for determining the BFD-RS.  Note that the SFN scheme + Alt 1-1 is essentially a single-DCI based multi-TRP scheme since the same DCI is transmitted from two TRPs in SFN fashion.  If implicit per-TRP BFD-RS configuration is not supported for single-DCI based multi-TRP in Rel-17, then there will be only a single BFD-RS set.  A simple solution is to let the UE assume that the reference signals used as QCL sources (if each TCI state contains two reference signal, the UE will utilize the QCL-TypeD RS) in the two activated TCI states for the CORESET can be used as BFD-RSs in the single BFD-RS set.  As shown in the example of Figure 1,  the QCL source reference signal with CSI-RS resource IDx corresponding to the 1st activated TCI state and QCL source reference signal with CSI-RS resource IDy corresponding to the 2nd activated TCI state shall be included by the UE in the beam failure detection resource set q0.

When two TCI states are activated for a CORESET, support inclusion of reference signals used as QCL sources in the two activated TCI states as BFD-RSs in the single BFD-RS set q0. If any of the activated TCI states contains two reference signals, the UE will use the reference signal configured with QCL-TypeD

Figure 1.  An example of BFD resource determination with single BFD resource set when CORESET is configured for SFN-based PDCCH diversity

With regards to PUCCH-SR resource selection rules for TRP specific BFR, the following agreement was reached in RAN1#104bis-e:

Agreement

Select one of the following alternatives with possible modification in RAN1#106-e

- Alt 2.5.2 A:

  - On PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured, there is no consensus to adopt alt-1 or alt-2. PUCCH-SR resource selection is up to UE implementation.

- Alt 2.5.2 B:

  - On the PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured, and at most one BFD RS set fails per CC, adopt alt 2 if all failed BFD RS sets cross CCs are associated with the same PUCCH SR resource, else PUCCH-SR resource selection is up to UE implementation.

- Alt 2.5.2 C:

  - On the PUCCH-SR resource selection rule when SR is triggered and 2 PUCCH-SR resources are configured, and at most one BFD RS set fails per CC, adopt alt 1 if all failed BFD RS sets cross CCs are associated with the same PUCCH SR resource, else PUCCH-SR resource selection is up to UE implementation.

- Alt 2.5.2 D:

  - Revert the past agreement on supporting configuration of up to 2 PUCCH-SR resources. A UE can be configured up to 1 PUCCH-SR resource in a cell group.

Among the alternatives above, Alts 2.5.2B and 2.5.2C define selection rule for case when at most one BFD RS set fails per CC, and for all other cases PUCCH-SR resource selection is up to UE implementation.  We do not see any benefit of leaving PUCCH-SR resource selection up to UE implementation when two PUCCH-SR resources are configured.  Alt 2.5.2A has a similar issue that PUCCH-SR resource selection is left up to UE implementation.  Hence, our preference is to make an agreement on Alt 2.5.2D.

Regarding PUCCH-SR resource selection when SR is triggered when beam failure is detected, revert the past agreement on supporting configuration of up to 2 PUCCH-SR resources. A UE can be configured up to 1 PUCCH-SR resource in a cell group.

# 3	Conclusion

Based on the discussion in the previous section, we make the following observation and proposal:

Proposal 1	For beam measurement/reporting enhancement to facilitate inter-TRP beam pairing, Option 1 (UE reporting N>1 pair/groups and M>=1 beams per pair/group in a CSI report) and Option 3 (UE reporting M>=1 beams in N >1 CSI-reports corresponding to N report setting) are not supported in NR Rel-17.

Proposal 2	Reuse simultaneousReceptionDiffTypeD-r16 UE capability to indicate if the UE is capable of receiving beams within a beam pair/group with different Rx spatial filters.

Proposal 3	For potential UE panel related information feedback for group-based beam reporting, support the indication of whether the reported beams in a group-based beam report are associated with same or different RX spatial filters (i.e., support Alt-1.1).

Proposal 4	In NR Rel-17, simultaneous configuration of RACH based BFR and TRP-specific BFR in the SpCell is not forbidden.

Proposal 5	In NR Rel-17, support both explicit and implicit BFD-RS configuration in order to support M-TRP beam failure detection.

Proposal 6	In NR Rel-17, support explicit M-TRP BFD-RS configurations for both multi-DCI based M-TRP and single-DCI based M-TRP.

Proposal 7	In NR Rel-17, support implicit M-TRP BFD-RS configurations only for multi-DCI based M-TRP.

Proposal 8	In NR Rel-17, support per-TRP BFD-RS configurations for both intra-cell and inter-cell multi-DCI based multi-TRP operation.

Proposal 9	When two TCI states are activated for a CORESET, support inclusion of reference signals used as QCL sources in the two activated TCI states as BFD-RSs in the single BFD-RS set q0. If any of the activated TCI states contains two reference signals, the UE will use the reference signal configured with QCL-TypeD

Proposal 10	Regarding PUCCH-SR resource selection when SR is triggered when beam failure is detected, revert the past agreement on supporting configuration of up to 2 PUCCH-SR resources. A UE can be configured up to 1 PUCCH-SR resource in a cell group.

# References

RP-193133, New WID: Further enhancements on MIMO for NR, Samsung, RAN#86, Sitges, December 2019.

R1-2009225, ‘On beam management enhancements for simultaneous multi-TRP transmission with multi-panel reception’, Ericsson, RAN1#103-e, October 26th – November 13th, 2020.

R1-2106287, Moderator summary #4 on M-TRP simultaneous transmission with multiple Rx panels, RAN1#105-e, May 10th – 27th, 2021.