3GPP TSG RAN WG1 #105-e Meeting	R1-2108056

e-Meeting, August 16th – 27th, 2021

Agenda item:		8.1.2.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Enhancements for HST-SFN deployment

Document for:		Discussion and Decision

# Introduction

The Rel-17 work item for enhancements on MIMO for NR includes an objective to extend specification support for enhancements on multi-TRP/panel transmission. In RAN #105-e, the following agreements were made on the HST-SFN topic [1]:

Agreement

Confirm the following working assumption from RAN1#104b-e:

All QCL source RS resource types as defined in TCI state for Rel-16 multi-TRP are supported for scheme 1.

Agreement

UE is not expected to be indicated by MAC CE with single TCI state per any of TCI codepoint , if UE is configured with scheme 1 PDSCH by RRC , but not capable to support dynamic switching between scheme 1 and single-TRP by TCI state field in DCI Format 1_1/1_2

Agreement

For specification based TRP-based frequency offset pre-compensation scheme

- Support dynamic (DCI -based) switching with single-TRP scheme by TCI state field in DCI format 1_1/1_2

  - This feature is UE optional

  - UE is not expected to be indicated by MAC CE with single TCI state per any of TCI codepoint , if UE is configured with TRP-based frequency PDSCH by RRC , but not capable to support dynamic switching between TRP-based frequency and single-TRP by TCI state field in DCI Format 1_1/1_2

- Support semi-static (RRC based) switching with Rel-16 schemes 1a, 2a, 2b, 3, 4

- Support semi-static (RRC based) switching with Rel-17 scheme 1 (PDSCH)

Agreement

Enhanced MAC CE signaling is not applicable to any of the configured CORESETs in a BWP if the CORESETs are configured with different CORESETPoolindex values in the BWP.

Working Assumption

For TRP-based pre-compensation, Variant A (based on RAN1#103-e meeting agreement) are supported as QCL types/assumption, when the same DMRS port(s) are associated with two TCI states.

- FFS: Additional support of Variant B

Agreement

- For TRP-based pre-compensation QCL assumptions is provided to the UE by using the existing QCL type(s) with certain QCL parameters dropped from the indicted QCL type

  - FFS rule or signalling to determine which TCI state with dropped QCL parameters

- UE does not expect to be configured different SFN schemes (scheme 1 or TRP pre-compensation) for both PDCCH and PDSCH.

  - FFS whether this restriction is per UE or per CC

- UE does not expect to be configured different SFN schemes (scheme 1 or TRP pre-compensation) for different CORESETs.

  - FFS whether this restriction is per UE or per CC

Agreement

Enhanced SFN PDCCH transmission scheme (scheme 1 or TRP-based pre-compensation) is identified by the number of TCI states activated per CORESET and RRC parameter

- FFS: Configuration detail of RRC parameter

  - Including whether the same RRC parameter is used for PDCCH and PDSCH

Agreement

If enhanced SFN PDCCH transmission scheme (scheme 1 or TRP -based pre-compensation) is configured and a CORESET is activated with two TCI states and UE is configured with enableTwoDefaultTCI-States and time offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL, down-select rule to determine default beam(s) for Rel-17 SFN PDSCH reception in RAN1#106-e:

- Alt 1: Reuse rule to determine TCI states as defined for Rel-16 PDSCH scheme-1a

- Alt 2: Introduce new rules to determine TCI states based on two TCI state(s) of the CORESET

Agreement

If enhanced SFN PDCCH transmission scheme (scheme 1 or TRP-based pre-compensation) is configured and two TCI states are activated for at least one CORESET, support the following configuration of RS for BFD

- Down-select one alternative for implicit configuration

  - Alt 1-2: RS of CORESETs with both single and two TCI states are used

  - Alt 1-3: RS of CORESETs with only two TCI states are used

- Down-select one alternative for explicit configuration

  - Alt 2-1: Support defining CSI-RS resource or SSB pairs as BFD RS

    - FFS other details

  - Alt 2-2: Reuse the existing Rel-15/Rel-16 approach for BFD RS configuration

- Note: down-selection can be done separately for Rel-15/16 cell specific BFR and Rel-17 TRP-specific BFR, Rel-17 TRP-specific BFR to be discussed under AI 8.1.2.3

In this contribution, we discuss enhancements to support high speed train SFN scenario and make some observations and proposals.

# Discussion

## HST-SFN transmission

Downlink performance with baseline HST-SFN transmission is degraded from the effects of multiple Doppler shifts experienced by the signals received from the TRPs at the UE. The demodulation performance suffers from the ability of the UE to estimate only a single Doppler shift and perform channel estimation based on U-shaped Doppler spectrum. Doppler shift estimation based on SFN TRS or DMRS in the UE receiver provides a single, composite Doppler shift due to the inability to separate the transmissions from different TRPs. Therefore, UE-based and network-based solutions are considered for specification in Rel-17 for DL HST-SFN performance enhancement.

## UE-based solution for HST-SFN enhancement

### Signalling for indication of scheme 1

It was agreed in RAN1 #104bis-e that scheme 1 for PDSCH is identified by a new RRC parameter and the number of TCI states indicated by DCI. Scheme 1 requires configuration of distributed TRS. As such, if scheme 1 is configured per component carrier, then transmission of TRS in each BWP of the component carrier is necessary to enable the UE to obtain the QCL parameters for the receiver. Likewise, it is also necessary for the UE to track the TRS in all the BWPs. In practice, however, the network may use only a single bandwidth part for scheme 1, which results in unnecessary network overhead and UE complexity. On the other hand, with configuration of scheme 1 per BWP, the network can choose to provide a configuration in only those BWPs in which scheme 1 will be used. Furthermore, this would be similar to the configuration of multi-TRP schemes in Rel-16 and as such should follow the same approach. Any issues related to dynamic switching between scheme 1 and other schemes that may arise due to not configuring scheme 1 in all BWPs can be handled through implementation.

Proposal 1: For RRC configuration of scheme 1, configuration per BWP is supported.

In RAN1 #104bis-e, it was agreed to support semi-static (RRC-based) switching of scheme 1 (for PDSCH) with Rel-16 scheme 1a. In our view, the scenarios in which Rel-16 scheme 1a and Rel-17 scheme 1 would be supported are different. While Rel-16 scheme 1a may be supported in areas with dense TRP deployment, such as in a city or near a train station, scheme 1 is likely to be supported on a high-speed train where TRPs are deployed with a certain separation. As such, when a user transits through a train station and on to train, support of semi-static switching between Rel-16 1a and Rel-17 scheme is sufficient, and we do not see a use case for dynamic switching.

Proposal 2: Do not support dynamic switching of Rel-17 scheme 1 (PDSCH) with Rel-16 scheme 1a.

An open issue related to scheme 1 is whether the same RRC parameter should be used for configuring the scheme for both PDCCH and PDSCH. It is noted that scheme 1 can be configured for PDCCH for URLLC enhancement as well. In this case, the scheme configured for PDSCH may be a scheme other than scheme 1. Thus, it is desirable to be able to configure scheme 1 separately for PDCCH and PDSCH using different RRC parameters.

Proposal 3: Different RRC parameters are used for configuring scheme 1 for PDCCH and PDSCH.

## Network-based frequency pre-compensation scheme

### QCL types/assumptions to support frequency pre-compensation

The following variants were agreed in RAN1 #103-e for consideration to be used with the frequency pre-compensation scheme.

- Variant A: One of the TCI state can be associated with {average delay, delay spread} and another TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA)

- Variant B: One of the TCI state can be associated with {average delay, delay spread} and another TCI state with {Doppler shift, Doppler spread} (i.e., QCL-TypeB)

- Variant C: One of the TCI state can be associated with {delay spread} and another TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA)

- Variant E: Both TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA)

In RAN1 #106-e, there was a working assumption that for TRP-based pre-compensation, Variant A is supported as QCL types/assumption and that additional support of variant B is FFS.

In variant B, one of the TCI states indicates QCL of the DMRS ports with the SFN (non-pre-compensated) TRS with respect to {average delay, delay spread} while the other TCI state is used to indicate QCL of the DMRS ports with the anchor TRS with respect to {Doppler shift, Doppler spread}, which corresponds to QCL-TypeB. The average delay and delay spread parameters for the composite channel from both TRPs can be obtained from the SFN TRS. In practice, however, transmission of the non-anchor TRS is also necessary to enable the UE to track it such that it can apply the QCL appropriate QCL assumption when the network changes the indicated TCI state corresponding to a change in the anchor TRS as the UE crosses the mid-point. Variant B requires the transmission of three TRSs and correspondingly entails additional UE complexity. Our understanding is that the average delay should be based on transmissions from both TRPs, as supported by variant A. In the current specification, QCL-TypeB (with TRS) is indicated only for CSI-RS. Therefore, support of QCL-TypeB for PDSCH with TRS will also require a change to the specifications. Furthermore, if variant B is supported in addition to variant A, it will introduce yet another solution for the same problem and signalling to distinguish between variant A and variant B would be necessary, due to which there is specification impact. In our view, there is no benefit from also supporting variant B.

Proposal 4: Confirm the working assumption that for TRP-based pre-compensation, Variant A (based on RAN1#103-e meeting agreement) is supported as QCL types/assumption, when the same DMRS port(s) are associated with two TCI states.

Proposal 5: Variant B for TRP-based pre-compensation is not supported.

In the above variants A, B, C, the {Doppler shift, Doppler spread} parameters are indicated in one of the two TCI states. One simple rule to indicate the anchor TCI is to associate the anchor TCI to the TCI having {Doppler shift, Doppler spread} parameters.

For TRP-based pre-compensation QCL assumptions are provided to the UE by using the existing QCL type(s) with certain QCL parameters dropped from the indicted QCL type. Based on the supported variants two cases are present:

- Only variant A is supported. In this case, UE expect the TCI state for anchor TRP to be associated with {average delay, delay spread, Doppler shift, Doppler spread}, and the TCI state for non-anchor TRP to be associated with {average delay, delay spread}. No need to specify any rule or signalling for dropped parameters. One possible approach is that when both TCI states are indicated with QCL-TypeA assumption, there is a rule to indicate which of the two TCI states assumes dropped parameters. As an example, the rule can be that the first TCI state indicated by the TCI codepoint in the DCI corresponds to the anchor TRP. Then, the second TCI state corresponds to the non-anchor TRP and hence is the one for which parameters are dropped to associate it with only {average delay, delay spread}.

- Two or more variants are supported. In this case, dropping rule for ignoring some parameters associated to a TCI state, should be scalable rule such that it can be used latter for future releases.

Observation 1: If only variant A is supported, a simple rule can be defined to determine which TCI state is associated with dropped parameters.

Proposal 6: UE assumes that the QCL parameters in the second TCI states of two TCI states are dropped.

### Indication of the carrier frequency for UL

In RAN1 #104bis-e, it was agreed that specification-based TRP Doppler pre-compensation scheme is supported in Rel-17 for FR1 with one or both of (a) UL RS based Doppler estimation by gNB and (b) DL RS based Doppler feedback by UE.

The frequency pre-compensation scheme based on implicit indication uses UL RS such as SRS for estimation of the compensation frequencies. The TRS is usually configured to be periodically transmitted, with typical periodicities of 10 ms to 40 ms. While TRS is used for time and frequency tracking by the UE, there are several alternatives for UL frequency offset estimation by the TRP.

In general, UL frequency offset estimation requires a reference signal and is performed using adjacent DM-RS symbols or PT-RS during UL demodulation, i.e., there is no separate measurement for frequency offset estimation. Therefore, there is additional complexity for the TRP to measure UL frequency offset when no PUSCH transmission is scheduled. Also, PUSCH is only sent to a single TRP; as such, without introducing multi-TRP PUSCH transmission, PUSCH-based frequency offset estimation is not applicable for HST-SFN scheme.

Using SRS is an alternative solution, but SRS is generally used for DL/UL CSI. Hence, to apply the frequency pre-compensation scheme, the UL SRS configuration must be optimized for frequency offset estimation. Because SRS has intra-symbol repetitions for comb-type RE mapping, we can use time-domain estimation. However when SNR of UL SRS signal is low, the frequency offset estimation error is distributed over a wide range. Figure 2 shows simple evaluation results for CDF of frequency offset estimation error when applying (a) one symbol SRS transmission with time-domain estimation (b) and two consecutive SRS symbols with frequency-domain estimation. We see that the frequency offset estimation error variability is lower in the case of two SRS symbols (frequency-domain estimation) but still quite large. In a practical deployment, the SRS link budget is low because of lower UE power than TRP (23 dBm vs. 46 dBm) and wider SRS bandwidth than other UL signal such as PRACH, PUCCH, and so on. Assuming low SNR of SRS signal, the estimation error can be upto 20% or 40% of the real frequency offset. Simulation results indicate that some of the performance gain of the frequency pre-compensation scheme at the mid-point track location is eroded due to large frequency offset estimation error.

(a)

(b)

Figure 2. Frequency offset estimation error (AWGN) with (a)	one SRS symbol (time-domain estimation) (b) two SRS symbols (frequency-domain estimation).

As an example of SRS optimization, the time domain span of SRS resource should be multiple symbols similar to TRS time domain pattern. There are two options for SRS transmission: the first option is using existing SRS structure with symbol repetition. The second option is introducing new SRS pattern composed of two SRS symbols with an interval of 2-3 symbols. The first option can be supported without specification impact while higher SRS overhead is expected. The second option requires additional specification effort such as new SRS type. The feasibility of each option should be studied, and regardless of the option to apply, the SRS overhead is very high.

Observation 2: SRS-based frequency offset estimation introduces higher SRS overhead and complexity in gNB.

If SRS spatial relation is configured with a TRS, it can be both for spatial relation and path loss calculation. However, for supporting SRS-based frequency pre-compensation, the SRS transmission should have different spatial relation RS and pathloss RS, which cannot be supported by the existing SRS transmission. If the SRS is configured with a spatial relation to the primary TRP, received SNR of the SRS signal in the other TRP is expected to be very poor. In addition, for FR2, there is no option to support simultaneous two-panel transmission, so SRS should be sent to two TRPs separately.

Observation 3: Single SRS transmission toward one TRP may degrade the frequency offset estimation in the other TRP.

Proposal 7: For SRS based frequency offset estimation, if supported, UE should be configured with two SRS resources to different TRPs, where each SRS resource is associated with different TRP.

Depending on the rate at which the pre-compensation frequency needs to be updated, the SRS could have different periodicities. Nevertheless, if frequency pre-compensation is separately applied for each UE, the SRS overhead can be significant. In addition, the approach would require association of SRS configuration with TRS along with dedicated triggering. Methods based on prediction at the network can also be used to reduce the overhead, where again triggering of SRS can be based on conditions observed either by the UE or by the network indicating poor prediction.

Proposal 8: To reduce the overhead of SRS-based frequency offset estimation, if supported, efficient triggering method for SRS transmission should be supported.

One of the critical problems of SRS-based frequency offset estimation is the limitation for applicability to FDD system. Though a partial reciprocity is discussed for CSI acquisition, the main idea was more related to delay and doppler spread, and they cannot be used for frequency offset estimation. In general, for supporting HST network with wide coverage, deployment in low band FDD spectrum is very promising, and FDD support is one of the key requirements for HST-SFN deployment.

Observation 4: SRS-based implicit indication has limitation for FDD support, which is one of key deployment scenario for HST-SFN.

On the other hand, the explicit indication would also entail additional overhead for reporting the Doppler shifts estimated for the two TRPs. The approach would require enhancement of reporting to include Doppler shift estimates for two TRPs. However, the accuracy of frequency offset estimation by TRS at UE side is higher than that of UL SRS based measurement. In addition, this CSI-based explicit report only requires UL transmission to a single TRP, which is much easier than sending multiple SRS to two TRPs. Also, there is no restriction for FDD support.

For explicit indication, a new reporting quantity of frequency offset shall be defined. In addition, the triggering of UE’s measuring frequency offset by TRS shall be supported. Other details such as any changes to CSI timeline and CPU occupation rule can also be considered. A potential disadvantage is the impact of frequency offset estimation errors, which would be more dominant at lower SNRs, e.g., for the TRP that is relatively far away from the UE. Mitigation of this impact may therefore be necessary. Thus, if UE can report RSRP of each TRS used for measuring frequency offset, NW can estimate the reliability of the frequency offset estimates.

Proposal 9: Support explicit indication of the Doppler shifts through the CSI framework with following enhancement

- TRSs are configured as CMR

- new report quantity frequency offset

- report of TRS RSRP.

For the implicit approach for carrier frequency indication, the carrier frequency of UL RS transmitted by the UE for frequency offset estimation need not have an exactly known relationship with the carrier frequency at the TRPs. Theoretically, the carrier frequency of the UL RS may have an arbitrary offset since the frequency compensation value for one TRP can be zero while the frequency pre-compensation value for the other TRP can be equal to the difference between the Doppler shifts estimated at the two TRPs. In practice, however, the frequency offset of the UL RS received by either of the TRPs must be within the estimation range, which can be handled through implementation. Therefore, there is no need to specify a QCL-like association between the DL TRS and UL signal used for carrier frequency indication.

Proposal 10: QCL-like association of the TRS with UL signal transmitted for carrier frequency indication is supported by implementation without specification impact.

### Signalling for indication of frequency pre-compensation scheme

It has already been agreed that Rel-17 scheme 1 is identified by a new RRC parameter and the number of TCI states indicated in the DCI. For maintaining consistency between scheme 1 and the frequency pre-compensation scheme and due to similarities in UE operations with respect to the two schemes due to both schemes being based on SFN transmission, it is preferred that identification of the frequency pre-compensation scheme is also based on an RRC parameter and the number of TCI states indicated in the DCI.

Proposal 11: The TRP-based frequency pre-compensation scheme is identified based on an RRC parameter and the number of TCI states indicated in the DCI.

The dropping of QCL parameters for the TCI state associated with the non-anchor TRP for variant A is linked to the RRC configuration. If a dropping rule is adopted, the use of the rule can be based on configuration of TRP-based pre-compensation for PDSCH. Alternatively, if dropping of QCL parameters is indicated using RRC signalling, a separate RRC parameter is not required for configuring the TRP-based pre-compensation scheme for PDSCH.

Similar to scheme 1, it is preferred that TRP-based frequency pre-compensation is also configured per BWP.

Proposal 12: For RRC configuration of TRP-based frequency pre-compensation, configuration per BWP is supported.

Also similar to scheme 1, it is necessary to consider whether TRP-based frequency pre-compensation would need to be configured only for PDCCH or only for PDSCH. For example, it is possible to use TRP-based per-compensation for PDSCH when Rel-17 URLLC enhancement is configured for PDCCH. Further study is needed to determine whether this should be supported. In general, however, it is desirable to configure different schemes for PDCCH and PDSCH, also considering forward compatibility. Therefore, similar to scheme 1, it is preferred to have different RRC parameters to configure TRP-based frequency pre-compensation for PDCCH and PDSCH.

Proposal 13: Different RRC parameters are used for configuring TRP-based frequency pre-compensation for PDCCH and PDSCH.

## SFN transmission of PDCCH

Enhancement of SFN transmission of PDCCH is also supported in Rel-17 with either scheme 1 or TRP-based frequency pre-compensation. It is agreed that PDCCH enhancement for SFN transmission is supported through activation of two TCI states using enhanced MAC-CE signaling. Open issues remain, however, related to the definition of default TCI state(s) for PDSCH if a CORESET is activated with two TCI states. Additionally, default TCI state definition must also be considered when PDCCH in a CORESET activated with a single TCI state (i.e., Rel-15/16 PDCCH) schedules a PDSCH configured with a Rel-17 HST-SFN enhancement scheme (if allowed).

### Default TCI state for PDSCH

We first need to consider the various cases in which it is necessary to define the default QCL assumption for PDSCH. Table 1 shows the combinations of PDCCH and PDSCH schemes that are already supported in Rel-15/16 and those that should now be considered with the specification of new schemes in Rel-17. The rows represent the different PDCCH schemes and the columns represent the different PDSCH schemes. The entry ‘o’ represents that the corresponding PDCCH-PDSCH scheme combination is already support for the schemes in legacy releases or should be supported with the Rel-17 schemes. ‘FFS’ is entered for the combinations since it needs to be discussed whether these combinations schemes should be supported. Table 2 categorizes the cases in the table 1 according to the conditions and shows the possible options for default QCL assumption. In table 2, we didn’t account for Rel-17 non-SFN PDCCH schemes.

In our view, we have to avoid such many variants deployed in a network, and only valid use cases should be supported. The first issue to be determined is if SFN PDCCH + non-SFN PDSCH to be supported. They are case 4/a and 5 in table 2. In our view, there is no justification for SFN PDCCH scheduling PDSCH with single-TRP PDSCH or Rel-16 TDM schemes. When PDCCH contains fallback DCI, CORESET indicated with a single TCI state for non-SFN PDCCH can be used. Otherwise, there is no point of using SFN PDCCH and to schedule non-SFN PDSCH.

Observation 5: There is no need to consider default TCI state for non-SFN PDSCH scheduled by SFN PDCCH.

Proposal 14: when SFN PDCCH is used, RAN1 clarify the support of following scenarios (from the table 2)

- Support of single TRP PDSCH scheme (Case 4/4a)

- Support of Rel-16 M-TRP PDSCH schemes (Case 5)

- Support of Rel-17 SFN PDSCH without enableTwoDefaultTCI-States.

Proposal 15: At least for NR Rel-17, do not support scheduling SFN PDSCH by Rel-17 non-SFN PDCCH.

Table 1. Combinations of PDCCH and PDSCH schemes that are supported in Rel-17.

|  |  | PDSCH |  |  |
| --- | --- | --- | --- | --- |
| PDCCH |  | Rel-15 | Rel-16 | Rel-17 SFN |
|  | Rel-15 | O | O | O |
|  | Rel-17 non-SFN | O | O | FFS |
|  | Rel-17 SFN | FFS | FFS | O |

Table 2. Default QCL assumptions for PDSCH with Rel-15/16/17 schemes.

| Case | PDCCH | PDSCH | enableTwoDefaultTCI-States is configured | TCI present in DCI | At least one TCI codepoint with Two TCI states in PDSCH MAC-CE | Possible options for Default QCL assumption for PDSCH |
| --- | --- | --- | --- | --- | --- | --- |
| 1 (Legacy) | Rel-15 PDCCH | Rel-15 PDSCH | No | Yes/No | NA | The TCI state of the lowest CORESET ID in the latest slot |
|  |  |  | Yes | Yes | No |  |
| 2 (Legacy) |  | Rel-16 PDSCH | Yes | Yes | Yes | The lowest TCI codepoint with two TCI states (Rel-16) |
| 3 (new) |  | Rel-17 SFN PDSCH | Yes | Yes | Yes | The lowest TCI codepoint with two TCI states (Rel-16 rule) |
| 4 (new) | Rel-17 SFN PDCCH | Rel-15 PDSCH | No | Yes/No | NA | Option 1: The (first) TCI state of the lowest CORESET ID in the latest slot (Rel-15 rule) Option 2: Any one of two TCI states of TCI state of the lowest CORESET ID in the latest slot (Rel-17a) |
| 4a (new) |  |  | Yes | Yes/No | No |  |
| 5 (new) |  | Rel-16 PDSCH | Yes | Yes/No | Yes | The lowest TCI codepoint with two TCI states (Rel-16 rule) |
| 6 (new) |  | Rel-17 SFN PDSCH | Yes | Yes/No | Yes | The lowest TCI codepoint with two TCI states (Rel-16 rule) |
| 6a (new) |  |  | No | Yes/No | NA | The TCI states of the lowest CORESET ID with two TCI states in the latest slot (Rel-17) |
| 6b (new) |  |  | Yes | Yes/No | No | The TCI states of the lowest CORESET ID with two TCI states in the latest slot (Rel-17) |

Below we discuss default QCL assumption for PDSCH in various cases in table 2.

Case 3: Rel-15 PDCCH + Rel-17 SFN PDSCH

In this case, Rel-15 PDCCH schedules PDSCH configured with Rel-17 SFN scheme. In order to schedule SFN PDSCH, Rel-16 MAC-CE is used, and this is possible only when enableTwoDefaultTCI-States is configured, If at least one TCI codepoint includes with two TCI states, default TCI states are the lowest TCI codepoint having two TCI states (Rel-16 default QCL assumption). Otherwise, UE shall assume single-TRP operation with Rel-15 default QCL assumption.

Proposal 16: For Rel-17 SFN PDSCH with non-SFN PDCCH (Case 3),

- When enableTwoDefaultTCI-States is configured and at least one TCI codepoint is indicated with two TCI states, UE use default QCL assumption as the lowest TCI codepoint having two TCI states in MAC-CE (Rel-16 default QCL assumption)

- Otherwise, UE assume single TRP operation, and Rel-15 default QCL assumption is applied.

- FFS: when Rel-17 SFN PDSCH is configured, whether “none of the TCI codepoints is indicated with two TCI states in MAC-CE” is supported

Case 4/4a: Rel-17 SFN PDCCH + Rel-15 PDSCH

In this case, Rel-17 SFN PDCCH schedules Rel-15 PDSCH. As proposed before, we don’t see the use case. However, if this case is supported, there is ambiguity on UE’s capability of receiving two beams at the same time. To fully exploit UE’s capability, we can consider different default QCL assumption than Rel-15.

There are two cases for this combination, case 4 and case 4a according to the configuration of enableTwoDefaultTCI-States. If enableTwoDefaultTCI-States is not configured, it is considered that UE doesn’t have any capability to process two PDSCH beam at the same time. In this case, UE can make a simple QCL assumption of the first TCI state of two TCI states for the lowest index CORESET in the latest slot. If enableTwoDefaultTCI-States is configured, it is considered that UE has capability to process two PDSCH beam at the same time. In this case, we can exploit the UE capability for flexible scheduling especially for fast beam switching to overcome blockage. UE can assume default TCI state is any one of two TCI states of the lowest CORESET ID having Two TCI states in the latest slot.

Proposal 17: For Rel-15 PDSCH with SFN PDCCH (Case 4/4a), if supported,

- When enableTwoDefaultTCI-States is configured, and none of the TCI codepoints is indicated with two TCI states, UE uses default QCL assumption as any one of two TCI states of the lowest indexed CORESET having two TCI states (Case 4a)

- Otherwise, UE assumes single TRP operation, and assume the (first) TCI state of the lowest indexed CORESET. (Case 4)

Case 5: Rel-17 SFN PDCCH + Rel-16 PDSCH

In this case, Rel-17 SFN PDCCH schedules Rel-16 PDSCH. As proposed before, we don’t see the use case. However, if this case is supported, we can simply apply Rel-16 default QCL assumption.

Proposal 18: For Rel-16 PDSCH with Rel-17 SFN PDCCH (Case 5), if supported,

- UE assumes default TCI state is the lowest TCI codepoint having two TCI states. (Rel-16 default QCL assumption)

Case 6: Rel-17 SFN PDCCH + Rel-17 SFN PDSCH

In this case, Rel-17 SFN PDCCH schedules Rel-17 SFN PDSCH.

When enableTwoDefaultTCI-States is configured, to support two TCI states for PDSCH, UE shall be indicated by MAC-CE where at least one code point includes two TCI states. In this case, UE shall apply the lowest codepoint of TCI state in the MAC-CE as the default TCI state for PDSCH. We think that HST-SFN shall be used after activation of TCI states by MAC-CE. We don’t see any need for further specification support other than Rel-16 default QCL assumption.

When enableTwoDefaultTCI-States is not configured, or if none of the TCI codepoints of MAC-CE includes two TCI states, or if the UE is scheduled with a DCI format not having TCI field present, UE cannot apply Rel-16 default QCL assumption. In this case, UE may apply similar principle of the Rel-15 default QCL assumption, which is to follow the TCI States or QCL assumption applied to the lowest indexed CORESET. Therefore, if the CORESET is indicated with two TCI states for SFN transmission of PDCCH, PDSCH is also transmitted with the SFN scheme.

Proposal 19: for SFN PDSCH with SFN PDCCH (Case 6/6a/6b),

- When enableTwoDefaultTCI-States is configured, and at least one of TCI codepoints is indicated with two TCI states, UE assumes default TCI states is the lowest TCI codepoint having two TCI states in MAC-CE. (Rel-16 default QCL assumption) (Case 6)

- Otherwise, UE assume default TCI states is The TCI state of the lowest CORESET ID with two TCI states in the latest slot (Case 6a/6b)

- FFS: when Rel-17 SFN PDSCH is configured, whether “none of the TCI codepoints is indicated with two TCI states in MAC-CE” is supported (Case 6a)

### Default TCI state for aperiodic CSI-RS

Another issue is related to default TCI state for AP CSI-RS when it is scheduled by PDCCH in a CORESET indicated with two TCI states, the scheduling offset for the AP CSI-RS is less than the threshold, and there is no other overlapping DL signal with the indicated TCI state. In this case, both the TCI states indicated for the CORESET can be assumed as the default beam by the UE for reception even if the AP CSI-RS transmission is scheduled on only one of the two TCI states. Therefore, it is not necessary assume only of the two TCI states as the default beam. We think this is the identical problem with the Case 4/4a in the previous section.

Observation 6: When AP CSI-RS is scheduled by PDCCH in a CORESET indicated with two TCI states, both the TCI states indicated for the CORESET can be assumed as the default beam by the UE for reception even if the AP CSI-RS transmission is scheduled on only one of the two TCI states.

### BFR support

For BFR, a periodic reference signal is used for BFD-RS. When implicit method is applied for determining BFD-RS, the periodic CSI-RS associated with current CORESET is used as BFD-RS. If scheme 1 is applied for PDCCH, there is an ambiguity for determining BFD-RS for the CORESET.

There are two alternative options. The first option is corresponding to Rel-15 principle, using both CSI-RS as the q0 BFD-RSs and determining beam failure when RSRP of both CSI-RS are below threshold. The second option is applying Rel-17 multi-TRP BFR principle: defining two sets of BFD-RSs for two TRP and deriving BFD-RS per TRP based on further clarification of BFD-RS to TRP association.

Because the multi-TRP BFR is not completed, this issue can be postponed until after completion of multi-TRP BFR.

Observation 7: For BFD-RS for SFN PDCCH, RAN1 only consider for applying to Rel-15/16 BFR.

In RAN1 #104bis-e, the following proposal listed the potential discussion points.

Proposal #3-8a:

- When two TCI states are configured activated for a CORESET, study the following enhancements to BFR procedures

  - Configuration of RS for BFD

    - Implicit configuration

      - Alt 1-1: RS of CORESETs with only single active TCI states are used

      - Alt 1-2: RS of CORESETs with both single and two TCI states are used

      - Alt 1-3: RS of CORESET(s) with only two TCI states are used

    - Explicit configuration

      - Alt 2-1 Support defining CSI-RS resource or SSB pairs

        - FFS other details

      - Alt 2-2 Reuse the existing approach for BFD RS configuration

  - Assumptions for hypothetical BLER calculation for PDCCH

      - Alt 3-1: RS in the two TCI states or CSI-RS / SSB are directly used as the BFD RS

      - Alt 3-2: UE calculates one hypothetical BLER under SFN assumption of BFD RS pairs

  - Configuration of NBI RS

      - Alt 4-1: Reuse the existing Rel-15 NBI configuration based on single CSI-RS resource

      - Alt 4-2: Introduce two new beam identification CSI-RS resource sets or new beam identification CSI-RS resource pairs

  - FFS applicability of the BFR enhancements above to

    - Rel-15 BFR

    - Rel-16 BFR

    - Rel-17 BFR

  - FFS UE behavior on monitoring the PDCCH candidate after BFD

- FFS applicability of some enhancements to RLM procedures

- FFS other aspects

Also, in RAN1 #105-e meeting, RAN1 has made following agreement.

Agreement

If enhanced SFN PDCCH transmission scheme (scheme 1 or TRP-based pre-compensation) is configured and two TCI states are activated for at least one CORESET, support the following configuration of RS for BFD

- Down-select one alternative for implicit configuration

  - Alt 1-2: RS of CORESETs with both single and two TCI states are used

  - Alt 1-3: RS of CORESETs with only two TCI states are used

- Down-select one alternative for explicit configuration

  - Alt 2-1: Support defining CSI-RS resource or SSB pairs as BFD RS

    - FFS other details

  - Alt 2-2: Reuse the existing Rel-15/Rel-16 approach for BFD RS configuration

- Note: down-selection can be done separately for Rel-15/16 cell specific BFR and Rel-17 TRP-specific BFR, Rel-17 TRP-specific BFR to be discussed under AI 8.1.2.3

Since UE can be configured with both SFN and non-SFN PDCCH, the BFD-RS determination shall take into account all cases, and we think BFD-RS shall be supported for both single and two TCI states. Since SFN PDCCH can be supported only by MAC-CE triggering, it is not related with explicit BFD-RS configuration, and no further specification is required.

Proposal 20: For BFD-RS set determination for SFN PDCCH,

- Support Alt 1-2, for implicit BFD-RS set configuration, i.e. RS of CORESETs with both single and two TCI states are used.

- Support Alt 2-2, for explicit BFD-RS set configuration. No further specification is required.

Note: The above is only applicable for Rel-15/16 BFR.

For the issue of hypothetical BLER calculation, both alternatives can be used as UE implementation, and no specification is required.

For NBI-RS configuration, existing specification already supports configuration and indication of multiple NBI-RSs, and the network can schedule one or two TCI states for PDCCH transmission, and no specification support is required.

Observation 8: For NBI-RS configuration, the existing specification can be applied for BFR by implementation. For per-TRP BFR in Rel-17, further study is required.

# Conclusion

In this contribution, we discuss the sub-objective of multi-TRP/panel transmission related to enhancement for HST-SFN. The following observations and proposals are made.

For UE-based solution

Proposal 1: For RRC configuration of scheme 1, configuration per BWP is supported.

Proposal 2: Do not support dynamic switching of Rel-17 scheme 1 (PDSCH) with Rel-16 scheme 1a.

Proposal 3: Different RRC parameters are used for configuring scheme 1 for PDCCH and PDSCH.

For NW-based solution

Proposal 4: Confirm the working assumption that for TRP-based pre-compensation, Variant A (based on RAN1#103-e meeting agreement) is supported as QCL types/assumption, when the same DMRS port(s) are associated with two TCI states.

Proposal 5: Variant B for TRP-based pre-compensation is not supported.

Observation 1: If only variant A is supported, a simple rule can be defined to determine which TCI state is associated with dropped parameters.

Proposal 6: UE assumes that the QCL parameters in the second TCI states of two TCI states are dropped.

Observation 2: SRS-based frequency offset estimation introduces higher SRS overhead and complexity in gNB.

Observation 3: Single SRS transmission toward one TRP may degrade the frequency offset estimation in the other TRP.

Proposal 7: For SRS based frequency offset estimation, if supported, UE should be configured with two SRS resources to different TRPs, where each SRS resource is associated with different TRP.

Proposal 8: To reduce the overhead of SRS-based frequency offset estimation, if supported, efficient triggering method for SRS transmission should be supported.

Observation 4: SRS-based implicit indication has limitation for FDD support, which is one of key deployment scenario for HST-SFN.

Proposal 9: Support explicit indication of the Doppler shifts through the CSI framework with following enhancement

- TRSs are configured as CMR

- new report quantity frequency offset

- report of TRS RSRP.

Proposal 10: QCL-like association of the TRS with UL signal transmitted for carrier frequency indication is supported by implementation without specification impact.

Proposal 11: The TRP-based frequency pre-compensation scheme is identified based on an RRC parameter and the number of TCI states indicated in the DCI.

Proposal 12: For RRC configuration of TRP-based frequency pre-compensation, configuration per BWP is supported.

Proposal 13: Different RRC parameters are used for configuring TRP-based frequency pre-compensation for PDCCH and PDSCH.

For SFN PDCCH transmission

Observation 5: There is no need to consider default TCI state for non-SFN PDSCH scheduled by SFN PDCCH.

Proposal 14: when SFN PDCCH is used, RAN1 clarify the support of following scenarios (from the table 2)

- Support of single TRP PDSCH scheme (Case 4/4a)

- Support of Rel-16 M-TRP PDSCH schemes (Case 5)

- Support of Rel-17 SFN PDSCH without enableTwoDefaultTCI-States.

Proposal 15: At least for NR Rel-17, do not support scheduling SFN PDSCH by Rel-17 non-SFN PDCCH.

Proposal 16: For Rel-17 SFN PDSCH with non-SFN PDCCH (Case 3),

- When enableTwoDefaultTCI-States is configured and at least one TCI codepoint is indicated with two TCI states, UE use default QCL assumption as the lowest TCI codepoint having two TCI states in MAC-CE (Rel-16 default QCL assumption)

- Otherwise, UE assume single TRP operation, and Rel-15 default QCL assumption is applied.

- FFS: when Rel-17 SFN PDSCH is configured, whether “none of the TCI codepoints is indicated with two TCI states in MAC-CE” is supported

Proposal 17: For Rel-15 PDSCH with SFN PDCCH (Case 4/4a), if supported,

- When enableTwoDefaultTCI-States is configured, and none of the TCI codepoints is indicated with two TCI states, UE uses default QCL assumption as any one of two TCI states of the lowest indexed CORESET having two TCI states (Case 4a)

- Otherwise, UE assumes single TRP operation, and assume the (first) TCI state of the lowest indexed CORESET. (Case 4)

Proposal 18: For Rel-16 PDSCH with Rel-17 SFN PDCCH (Case 5), if supported,

- UE assumes default TCI state is the lowest TCI codepoint having two TCI states. (Rel-16 default QCL assumption)

Proposal 19: for SFN PDSCH with SFN PDCCH (Case 6/6a/6b),

- When enableTwoDefaultTCI-States is configured, and at least one of TCI codepoints is indicated with two TCI states, UE assumes default TCI states is the lowest TCI codepoint having two TCI states in MAC-CE. (Rel-16 default QCL assumption) (Case 6)

- Otherwise, UE assume default TCI states is The TCI state of the lowest CORESET ID with two TCI states in the latest slot (Case 6a/6b)

- FFS: when Rel-17 SFN PDSCH is configured, whether “none of the TCI codepoints is indicated with two TCI states in MAC-CE” is supported (Case 6a)

Observation 6: When AP CSI-RS is scheduled by PDCCH in a CORESET indicated with two TCI states, both the TCI states indicated for the CORESET can be assumed as the default beam by the UE for reception even if the AP CSI-RS transmission is scheduled on only one of the two TCI states.

Observation 7: For BFD-RS for SFN PDCCH, RAN1 only consider for applying to Rel-15/16 BFR.

Proposal 20: For BFD-RS set determination for SFN PDCCH,

- Support Alt 1-2, for implicit BFD-RS set configuration, i.e. RS of CORESETs with both single and two TCI states are used.

- Support Alt 2-2, for explicit BFD-RS set configuration. No further specification is required.

Note: The above is only applicable for Rel-15/16 BFR.

Observation 8: For NBI-RS configuration, the existing specification can be applied for BFR by implementation. For per-TRP BFR in Rel-17, further study is required.

# References

- Draft Report of 3GPP TSG RAN WG1 #104bis-e, v0.1.0.