3GPP TSG RAN WG1 Meeting #106-e	R1-2108065

e-Meeting, August 16th – 27th, 2021

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	Views on RAN4 reply LS for L1/L2 mobility

Document for:	Discussion and Decision

# Introduction

RAN1 sent an LS to RAN4 asking questions on L1/L2 mobility [1] and received feedback with questions in [4]. In this paper, we provide our views on RAN4 questions. Note that the scope of L1/L2 mobility was discussed in RAN [5] and the corresponding WID was updated in [6] (copied below), with which the agreements in WGs may need to be revisited.

- Identify and specify features to facilitate more efficient (lower latency and overhead) DL/UL beam management for intra-cell and inter-cell scenarios to support higher UE speed and/or a larger number of configured TCI states: - Common beam for data and control transmission/reception for DL and UL, especially for intra-band CA - Unified TCI framework for DL and UL beam indication - Enhancement on signaling mechanisms for the above features to improve latency and efficiency with more usage of dynamic control signaling (as opposed to RRC) - For inter-cell beam management, a UE can transmit to or receive from only a single cell (i.e. serving cell does not change when beam selection is done). This includes L1-only measurement/reporting (i.e. no L3 impact) and beam indication associated with cell(s) with any Physical Cell ID(s) - The beam indication is based on Rel-17 unified TCI framework - The same beam measurement/reporting mechanism will be reused for inter-cell mTRP - This work shall only consider intra-DU and intra-frequency cases

# Answer to Questions from RAN4

In [4], RAN4 asked the following questions:

RAN4 reply to Question 5: RAN4 would like to ask the following clarification questions for RAN4 to further respond to the question related to RAN4. - It is RAN4 common understanding that for the intra-band CA scenario the primary serving cell and secondary serving cell(s) belong to the same frequency band, rather than the serving and non-serving cells belong to the same frequency band as described in the question. Likewise intra-band CA explanation, for inter-band CA case a serving cell and secondary serving cell(s) belong to different frequency bands rather than the serving and non-serving cells. Can RAN1 please further clarify the intra-band/inter-band scenarios based on above RAN4 common understanding? - RAN4 would like RAN1 to clarify whether “the operation” in question 5 refers to only L1/L2-centric inter-cell mobility or both inter-cell mobility and inter-cell mTRP operations. Does RAN4 need to consider inter-cell mTRP operation into the “CA scenario” of Question 5? - RAN4 also would like RAN1 and RAN2 to further clarify on the definition of non-serving cell especially whether UE shall support data/control channel reception and transmission from non-serving cell including simultaneous reception and transmission capabilities under CA scenarios. It is RAN4 understanding that the response to question 5 are related to above clarification questions. RAN4 will further respond to question 5 after receiving the feedback from RAN1 and RAN2 on above questions.

Regarding RAN4 questions, our understanding are provided as follows.

- RAN1 were assuming the serving cell and non-serving/neighbor cell could be anchored to different frequency points in same or different frequency bands, which could be similar to traditional CA band combination. With the updated WID, only intra-frequency scenario will be considered in R17, with which SSBs of non-serving/neighbor cells would have the same center frequency and SCS as the SSBs of the serving cell.

- “the operation” in question 5 referred to L1/L2-centric inter-cell mobility only (which has been updated as inter-cell beam management without changing serving cell for R17 [6]), i.e., not for inter-cell mTRP operation which is applicable to intra-frequency scenario only. In addition, there is no need for RAN4 to consider inter-cell mTRP operation into the “CA scenario”.

- As mentioned by RAN2 in [2], UE is not expected to support data/control channel reception and transmission from non-serving cell. Given the updated WID which explicitly indicated the serving cell does not change, there is also no need to further discuss the definition of non-serving cell in terms of data/control channel reception and transmission.

To sum up, with the updated WID, RAN1 can simply reply to RAN4 notifying them that inter-cell beam management in R17 is restricted to intra-frequency case, with which further response from RAN4 to Question 5 in R1-2102248 is not required at this point.

Proposal 1: Send a follow-up LS to RAN4 notifying that inter-cell beam management in R17 is restricted to intra-frequency case, with which further response from RAN4 to Question 5 in R1-2102248 is not required at this point.

# Summary and conclusion

In this contribution, we proposed the following.

Proposal 1: Send a follow-up LS to RAN4 notifying that inter-cell beam management in R17 is restricted to intra-frequency case, with which further response from RAN4 to Question 5 in R1-2102248 is not required at this point.

# References

[1] R1-2102248, LS on TCI State Update for L1/L2-Centric Inter-Cell Mobility

[2] R2-2106787, LS Reply on TCI State Update for L1/L2-Centric Inter-Cell Mobility

[3] R3-212879, Reply LS to RAN1 LS on TCI State Update for L1/L2-Centric Inter-Cell Mobility

[4] R4-2108356, Reply to RAN1 LS on L1/L2-Centric Inter-Cell Mobility

[5] RP-211551, Moderator's summary for email discussion [92-e-08-feMIMO-Scope]

[6] RP-211586, Revised WID: Further enhancements on MIMO for NR