3GPP TSG-RAN WG1 Meeting #106-e	R1-2108161

e-Meeting, Aug 16th – Aug 27th, 2021

Agenda Item:	8.10.1

Source:	CEWiT, Tejas Networks, IITM, IITH, IITB

Title:	Discussions on enhancements to resource multiplexing between child and parent links of an IAB node

Document for:	Discussion, Decision

# Introduction

The Rel. 17 IAB WID focuses on enhancements to support simultaneous operation of parent and child links of an IAB node, and dual connectivity scenarios. Several agreements were made during past RAN meetings related to these enhancements. In this contribution, we provide our views on the enhancements required in resource multiplexing between child and parent links of an IAB node to support simultaneous operation and dual connectivity scenarios.

# Enhancements to resource multiplexing between parent and child links

## 2.1 Determining active mode of operation and mode switching

The resource partitioning between the backhaul and access/child links of an IAB node can be done in time/frequency/space domains. In addition, some IAB nodes are subjected to half duplexing constraint (HDC), where the node cannot perform MT-Rx/DU-Tx and MT-Tx /DU-Rx in the same time-frequency resources. Therefore, TDM, FDM, FDM with HDC, SDM, SDM with HDC and IBFD are various possible multiplexing capabilities in an IAB node and the IAB node can use these capabilities for operating in one of the following modes

  - TDM

  - Multiplexing Case A: Simultaneous MT-Tx/DU-Tx

  - Multiplexing Case B: Simultaneous MT-Rx/DU-Rx

  - Multiplexing Case C: Simultaneous MT-Rx/DU-Tx

  - Multiplexing Case D: Simultaneous MT-Tx/DU-Rx

An IAB node cannot always operate in a particular multiplexing mode and it depends on various factors. In RAN1#105-e [1], the following agreement was made:

Agreement

The parent IAB-node is dynamically provided with conditions/parameters to facilitate adaptation between multiplexing operation modes:

- FFS: Required number of guard symbols for switching of multiplexing mode (FFS: per timing mode or per multiplexing mode) for IAB-DU

- FFS: Signalling procedure

- FFS: Required guard band for FDM

- FFS: other conditions, e.g., required timing mode, required power control parameters, and preferred TCI.

The active mode of operation of an IAB node depends on the supported modes of operation of IAB node, supported modes of operation and active mode of operation of parent node, the network conditions and the resource configuration of IAB-MT and IAB-DU. Therefore, capability of parent node is one of the factors determining active mode of operation of IAB node. For e.g., simultaneous Tx from IAB-MT and IAB-DU is possible only when parent-DU schedules UL-Tx from IAB-MT. In case of parent node with TDM capability, the parent-DU will not be active all the time and hence, it cannot schedule UL-Tx from IAB-MT when the MT of parent node is active. Also, a parent node with simultaneous Tx and/or Rx capability leads to lower latency as compared to parent node with only TDM capability. Therefore, if both the IAB node and the parent node are capable of simultaneous operation, there will be reduction in latency in communication between them. Therefore, it is beneficial for IAB node to know capability of various prospective parent nodes before initial association, to identify the best parent node to perform initial access.

Proposal 1: Mechanism to inform multiplexing capability of parent nodes to IAB node is supported

The active mode of operation of an IAB node in a symbol/slot also depends on the resource configuration of IAB-MT and IAB-DU. For e.g., an IAB node in Multiplexing Case A can transmit simultaneously from IAB-MT and IAB-DU, only when CU configures IAB-DU as DL, parent-DU configures IAB-MT as UL and parent-DU schedules UL-Tx from IAB-MT. Otherwise, any one of IAB-MT and IAB-DU will be active at a time, leading to reduction in resource utilization. Therefore, an IAB node need to signal its multiplexing capability and supported modes of operation to CU and parent-DU to enable CU and parent-DU configure resources accordingly.

Proposal 2: Signalling of multiplexing capability and supported modes from IAB node to CU and parent-DU is supported

Once the capability and/or supported mode of operation of IAB node are known, the active mode of operation of IAB node is determined based on the supported modes of operation at IAB node, active mode of operation of parent node and network conditions, and the resources are configured to IAB-MT and IAB-DU accordingly. One of the following alternatives can be used for determining the active mode of operation of IAB node and configuring resources for IAB node

Alt 1: Donor node determining the active mode of operation of IAB node

Alt 2: Parent node determining the active mode of operation of IAB node

In Alt1, the donor determines active mode of operation of IAB node, based on active mode of operation of IAB node, capability of IAB node and the network conditions, and configure resources to IAB-DU accordingly. The parent node can either implicitly derive the active mode of operation of IAB node based on the capability of IAB node and IAB-DU resource configuration or donor node can explicitly signal the active mode of operation of IAB node to parent node. The parent node configures resources to IAB-MT based on the active mode of operation of IAB node. In Alt 2, donor node configures resources to IAB-DU based-on capability of IAB node and network conditions and signal the resource configuration of IAB-DU to parent node. The parent node determines active mode of operation of IAB node based on capability of IAB node and resource configuration of IAB node.

Proposal 3: Following alternatives are proposed for configuring active mode of operation at an IAB node

- Alt1: Donor configuring the active mode of operation of IAB node

- Alt2: Parent node configuring the active mode of operation of IAB node

The timing of various transmission/reception at an IAB node changes with the switch in active mode of operation at IAB node and its parent node. For e.g., an IAB node in Multiplexing Case A use case 6 timing with TA=T1, whereas an IAB node in TDM mode employ case 1 timing with TA=2T1. Similarly, IAB node use TA=2T1+nTs-T0 for UL-Tx, if parent node in multiplexing case B and use case 7 timing. Consequently, the guard requirement at IAB node changes based on the timing of various transmission/reception at IAB node and its parent node. Therefore, an IAB node and its parent node should have same understanding of the active mode of operation of IAB node and the parent node should update TA and guard required at IAB node based on active mode of operation of parent node and IAB nodes.

Observation 1: IAB node and its parent node should have same understanding of the active mode of operation of the IAB node at every time instant

Proposal 4: Parent node updating the parameters like TA and guard required at IAB node based on active mode of operation of parent node and IAB node before mode switching is supported

Further, the active mode of operation is required to dynamically configure F resource at IAB-MT and IAB-DU. For e.g., parent-DU cannot configure F resource at IAB-MT as DL, if the IAB node is operating in Multiplexing Case A. Further, the semi-static configuration of active mode of operation cannot work in all scenarios. For e.g., in case of an IAB node in Multiplexing Case B, the reception in backhaul link may interfere with the reception in child/access link due to ACI in case of FDM and inter beam interference in case of SDM. In some cases, the interference can be high, causing performance degradation. Therefore, the IAB node cannot continue in Multiplexing Case B and need to switch to TDM mode. Mechanism to fall back from simultaneous mode to TDM mode should be supported.

Proposal 5: Dynamic switching between multiplexing modes of IAB node is supported under the control of parent node

Proposal 6: Mechanism to fall back to TDM mode from simultaneous mode at IAB node is supported

Even though dynamic switching and fall back is supported at IAB node, frequent switching between multiplexing modes leads to signaling storm, as parent node has to update the parameters like TA and guard every time before switching. Also, IAB node should update its configuration and signal necessary parameters to its child nodes and access UEs. Therefore, an IAB node should stay in a particular multiplexing mode of operation for minimum duration.

Observation 2: Frequent switching between multiplexing modes leads to signaling storm

Proposal 7: Define minimum duration for active mode of operation

## 2.2 IAB-DU resource configuration

In case of IAB node in simultaneous mode of operation using FDM, the same frequency resource cannot be used both in the parent backhaul and child backhaul/access link. Therefore, semi-static H/S/NA resource configuration for IAB-DU should be extended to frequency. In RAN1#105-e [1], the following agreements were made:

Agreement

For frequency domain multiplexing, H/S/NA configurations for an IAB-node are provided separately in addition to the Rel-16 H/S/NA

Agreement

If an IAB node is configured with a frequency-domain H/S/NA configuration down select between the following options:

- Alt. 1 Either the Rel-16 H/S/NA configuration or frequency domain configuration is applied for a given resource

  - FFS: Whether configurations are switched with per-slot, per-resource type within a slot, or per-symbol granularity

- Alt. 2 The Rel-16 H/S/NA configuration and frequency domain configuration are jointly applied

The H/S/NA attributes for the time and frequency resources at IAB-DU is introduced to avoid simultaneous use of resource by IAB-MT and IAB-DU in TDM and FDM mode of operations, respectively. Therefore, the H/S/NA attribute in time and frequency domain should be interpreted based on the active mode of operation of IAB node. For e.g., DU of an IAB node in TDM mode cannot transmit in a slot configured as S-DL unless it receives IA from parent node, whereas DU of an IAB node in multiplexing case A can transmit in a S-DL slot along with UL-Tx in IAB-MT using a frequency resource configured as H/S+IA. Similarly, DU of an IAB node in multiplexing case A cannot transmit using a RBG within DU cell configured as S, whereas DU of an IAB node in TDM mode can transmit using the entire DU cell. Therefore, either time domain H/S/NA or frequency domain H/S/NA will be active at an IAB node based on the active mode of operation. For e.g., in Fig. 1 illustrated below, an IAB node receives two separate configurations: the time domain H/S/NA resource with a granularity of resource type within a slot (indicated in green), and the frequency domain H/S/NA resource with granularity of RBG (indicated in orange). In case of IAB node operating in TDM mode in slot n, IAB-DU perform DL-Tx using entire cell2 even though RBG1 and RBG3 is configured as S. Similarly, DU of an IAB node in multiplexing mode A perform DL-Tx in slot n+1 using RBG2 and RBG3, even though slot n+1 is configured as S.

Proposal 8: An IAB node apply either the time domain H/S/NA configuration or frequency domain H/S/NA configuration for a given resource based on the active mode of operation

Proposal 9: The switching between time domain H/S/NA configuration and frequency domain H/S/NA configuration occur based on the granularity of active mode of operation

| Cell1 | RBG1 | S, H | S, H | NA, H | S, H |
| --- | --- | --- | --- | --- | --- |
|  | RBG2 | S, S | S, S | NA, S | S, S |
| Cell2 | RBG1 | H, S | H, S | S, S | S, NA |
|  | RBG2 | H, H | H, H | S, H | S, H |
|  | RBG3 | H, S | H, S | S, S | S, H |
|  |  | DL | F | UL | DL |
|  |  | Slot n |  |  | Slot n+1 |

Fig. 1: Semi-static H/S/NA resource indication for time and frequency resources at IAB-DU

For dynamic signalling of IA in frequency domain, following agreement was made in RAN1#105-e [1]:

Agreement

DCI Format 2_5 is reused to support soft resource availability indications for frequency-domain resources

- FFS: If additional enhancements are necessary

Similar to the semi-static indication of H/S/NA for IAB-DU, explained above, explicit signalling of IA for S resource is needed based on active mode of operation of IAB node. For e.g., an IAB node in Multiplexing Case A simultaneously transmits in IAB-MT and IAB-DU using different frequency resources. As a result, DU of an IAB node in Multiplexing Case A can transmit simultaneously with IAB-MT, even if slot/symbol of IAB-DU is configured S. Therefore, explicit signalling of IA for time domain is not mandatory for an IAB node in simultaneous mode of operation. Similarly, for an IAB node in TDM mode either IAB-MT or IAB-DU will be active at a time, and hence, IAB-DU can transmit/receive in slot/symbol configured as H using the entire cell, irrespective of the H/S flavor of RBGs of the cell. Therefore, IAB node in TDM mode does not require explicit IA for a S resource in frequency domain.

Observation 3: Explicit signalling of IA for a S resource is needed based active mode of operation

## 2.3 Cell-specific/semi-static signals and channels

Cell-specific signals/channels has high priority and hence IAB-DU transmits/receives cell-specific signal/channel irrespective of H/S/NA configuration of the time/frequency resource at IAB-DU. RAN1 #104-e [2] made the following agreement:

Agreement

Regardless of simultaneous operation, the same cell-specific/semi-static signals and channels of the IAB-DU considered as hard time/frequency resources in Rel-16 are also considered as hard time/frequency resources in Rel-17.

- FFS: IAB-MT behavior in case of conflicts between cell-specific signals/channels and other resource configurations of the IAB-MT (e.g., dedicated slot configurations)

Regarding the FFS point, to enable simultaneous operation of IAB-MT and IAB-DU in Rel. 17, tdd-UL-DL-ConfigurationDedicated-IAB-MT can overwrite the DL/UL configuration given by tdd-UL-DL-ConfigurationCommon. Therefore, additional conflict can arise when tdd-UL-DL-ConfigurationDedicated-IAB-MT overwrites the slot/symbol at IAB-MT configured for transmission/reception of cell specific signal/channel.

A similar conflict at IAB-MT is already discussed in Rel. 16, when IAB-MT is configured to transmit/receive cell-specific signal/channel in a resource configured as hard for IAB-DU, and the following agreement was made in RAN1-#96b

Agreements:

- In case of Hard or Soft Indicated Available DU resources, no additional exception cases need to be defined for cell specific signals/channels to be transmitted or received by the MT in the same resource (e.g. SS/PBCH blocks, SI reception, RACH).

  - The decision on whether to give priority to the DU or to the MT for the use of the resource (e.g. in case of MT RACH transmission) is left to the IAB node implementation.

  - The IAB shall fulfil its performance requirements in terms of measurement and transmission of cell specific signals / channels.

From the agreement, it is not mandatory for IAB-MT to transmit/receive cell-specific signal/channel at all the configured instants. Therefore, priority can be given to the DL/UL configuration given by tdd-UL-DL-ConfigurationDedicated-IAB-MT, in case of conflict between resource configuration given by tdd-UL-DL-ConfigurationDedicated-IAB-MT and cell specific signal/channel configuration. For e.g., configuring a symbol as UL by tdd-UL-DL-ConfigurationDedicated-IAB-MT overrides the configured SSB reception in that symbol.

Proposal 10: In case of IAB-MT, the resource configuration given by tdd-UL-DL-ConfigurationDedicated-IAB-MT override the configuration of cell specific signal/channel

# Conclusion

In this paper we made the following observations and conclusions:

Proposal 1: Mechanism to inform multiplexing capability of parent nodes to IAB node is supported

Proposal 2: Signalling of multiplexing capability and supported modes from IAB node to CU and parent-DU is supported

Proposal 3: Following alternatives are proposed for configuring active mode of operation at an IAB node

- Alt1: Donor configuring the active mode of operation of IAB node

- Alt2: Parent node configuring the active mode of operation of IAB node

Observation 1: IAB node and its parent node should have same understanding of the active mode of operation of the IAB node at every time instant

Proposal 4: Parent node updating the parameters like TA and guard required at IAB node based on active mode of operation of parent node and IAB node before mode switching is supported

Proposal 5: Dynamic switching between multiplexing modes of IAB node is supported under the control of parent node

Proposal 6: Mechanism to fall back to TDM mode from simultaneous mode at IAB node is supported

Observation 2: Frequent switching between multiplexing modes leads to signaling storm

Proposal 7: Define minimum duration for active mode of operation

Proposal 8: An IAB node apply either the time domain H/S/NA configuration or frequency domain H/S/NA configuration for a given resource based on the active mode of operation

Proposal 9: The switching between time domain H/S/NA configuration and frequency domain H/S/NA configuration occur based on the granularity of active mode of operation

Observation 3: Explicit signalling of IA for a S resource is needed based active mode of operation

Proposal 10: In case of IAB-MT, the resource configuration given by tdd-UL-DL-ConfigurationDedicated-IAB-MT override the configuration of cell specific signal/channel

# References

- RAN1, Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #105-e, May 2021.

- RAN1, Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #104-e, February 2021