3GPP TSG RAN WG1 #106-e	R1-2108179

e-Meeting, Aug 16th - 27th, 2021

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on RAN2 LS on DRX impact

Agenda item:		5

Document for:		Discussion and Decision

# Introduction

In RAN1 #104-e meeting, RAN2 sent an LS on SL DRX working assumption and its design [1]. From the LS, RAN2 indicated the working assumption of sidelink DRX:

Working assumption: SL DRX should take PSCCH monitoring also for sensing (in addition to data reception) into account if SL DRX is used.

RAN2 asks RAN1 to provide feedback if there is any concern on the working assumption and take the above information into their future works.

In RAN1 #104b-e meeting, RAN1 discussed the related SL DRX issue. A related proposal [2] was discussed without agreement. The proposal was:

Proposal 7 (round II) for Thursday GTW session: - It is up to UE implementation to perform SL reception of PSCCH and RSRP measurement for sensing during its SL DRX inactive time. - FFS any conditions on sensing in SL-DRX inactive time.

RAN1 #105-e meeting provided further and intensive discussion without any agreement. At the 4th round of discussion, two wayforward working assumptions were proposed [3]; however, there was no agreement.

Proposed Working Assumption A UE can perform SL reception of PSCCH and RSRP measurement for sensing during its SL DRX inactive time. - FFS whether such reception is performed is subject to specification(including e.g. UE shall perform sensing during its SL DRX inactivity time), or is up to UE implementation - FFS: Other details Proposed Possible Agreement RAN1 aim to down-select between the two alternatives (subject to necessary finetuning for either of the alternative) during RAN1#106-e Alt 1 A UE can perform SL reception of PSCCH and RSRP measurement for sensing during its SL DRX inactive time. - FFS whether such reception is performed is subject to specification(including e.g. UE shall perform sensing during its SL DRX inactivity time), or is up to UE implementation - FFS: Other details Alt 2 A UE cannot perform sensing out of the DRX active time - FFS Other details Note: Companies are encouraged to provide further input including e.g. simulation results to facilitate the down-selection

In this contribution, we continue the discussion and provide our view based on RAN2’s LS. Our proposal for the LS reply is also provided in [4].

# Discussions

SL DRX is configured for UE Rx power saving. The DRX configuration and SL resource allocation shall be discussed together. Based on RAN2’s agreements indicated in the LS [1], there are certain constraints when SL DRX is configured. The Tx side constraint is that the Tx UE shall ensure that the Rx UE can receive the SL data transmission during Rx active time. On the Rx side, RAN2 defines the behavior for monitoring the SCI reception (PSCCH and the 2nd SCI in PSSCH) during the SL Rx active time. The issue is raised when the Rx UE’s sensing window/occasions is overlapped with its Rx inactive time.

One way to avoid the overlap between sensing window and Rx inactive time is through coordination of the SL DRX configuration with UE Rx sensing window. But such approach would be too expensive to be practicable because of the signaling cost between SL Tx UE and SL Rx UE. The overlap between sensing window and Rx inactive time is usually hard to avoid when SL DRX is configured.

When the overlapping happens, the question is whether the sensing is allowed during the Rx inactive time. Apparently there are two choices, as indicated in Table 1. If the sensing is not allowed, the Rx UE would have limited or even no sensing results. When the Rx UE needs to transmit, the UE may only use random selection because it has little information about resource allocation. Random selection will have impact of increased probability of collision, which would result in poor SL transmission performance. If the sensing is allowed, the Rx UE will have sufficient results for its potential SL resource allocation, with the cost of power consuming for sensing in Rx inactive time. The tradeoff between the sensing and power saving can be summarized in Table 1.

Table 1    Comparison between sensing and no-sensing in Rx inactive time, when overlapping between Rx UE sensing window/occasions and Rx inactive time with SL DRX configuration

| Sensing is allowed during the Rx inactive time | Advantage | Disadvantage |
| --- | --- | --- |
| No | Power saving in Rx inactive time | No sensing capability in Rx inactive time |
| Yes | Sufficient sensing results | Power consuming for sensing in Rx inactive time |

During the discussion in RAN1 #105-e meeting, there are various views on whether the UE is allowed for sensing during the Rx inactive time of DRX. There are the general views from companies:

- Sensing during the Rx inactive time is not allowed.

- UE can perform sensing during the Rx inactive time. This capability is up to UE implementation.

- UE performs sensing during its SL DRX inactivity time, based on specified rule(s). FFS the rule(s).

- UE shall perform sensing during its SL DRX inactivity time.

From this list, the first choice prohibits sensing during the Rx inactive time, resulting in no sensing capability in Rx inactive time, as indicated in Table 1. The 3rd choice would need further specification to support possible conditions to enable sensing during Rx inactive time. The 4th choice may imply that the UE will perform sensing, regardless SL DRX. This choice may defy the purpose of DRX design of power saving. Considering the tradeoff between the sensing and the power saving with DRX configuration, at least UE may have the freedom to conduct sensing during the Rx inactive time. The SL UE shall have the choice between power saving and SL transmission performance. Therefore, we support the second choice in the list, indicating that it is up to UE implementation to perform SL reception of PSCCH and RSRP measurement for sensing during its SL DRX inactive time.

Proposal 1: It is up to UE implementation to perform SL reception of PSCCH and RSRP measurement for sensing during its SL DRX inactive time.

We shall indicate this proposal as the LS reply to RAN2. A draft reply-LS is prepared in R1-2108178 [4].

# Conclusion

Regarding to the working assumption indicated in RAN2 LS [1], we demonstrate the tradeoff between the sensing and the power saving when the sensing window/occasions is overlapped with the Rx inactive time when the SL DRX is configured. We have this proposal, following previous discussion in RAN1:

Proposal 1: It is up to UE implementation to perform SL reception of PSCCH and RSRP measurement for sensing during its SL DRX inactive time.

We shall indicate this proposal as the LS reply to RAN2. A draft reply-LS is prepared in R1-2108178 [4].

# References

- R1-2100021		LS to RAN1 on SL DRX design, RAN2 (R2-2010961), RAN1 #104-e, Jan-Feb, 2021

- R1-2104093		FL summary for AI 8.11.1.1 – resource allocation for power saving (final), Moderator (OPPO), RAN1 #104b-e, April 2021

- R1-2106368		Moderator summary of Email Discussion/approval to rely LS in R1-2100021, Moderator (ZTE), RAN1 #105-e, May 2021

- R1-2108178		draft Reply LS on SL DRX design, Nokia, NSB, RAN1 #106-e, August 2021