3GPP TSG RAN WG1 Meeting #106-e	R1-2108194

E-meeting, August 16th - 27th, 2021

Agenda Item:	6

Source:	Huawei, HiSilicon

Title:	Discussion on distinguishing between PUR and SPS PUSCH for eMTC

Document for:	Discussion and Decision

- Introduction

In Rel-16 eMTC, PUR was introduced to support PUSCH transmission in preconfigured uplink resources for UEs in idle mode.

In this paper, we discussed the ambiguity between PUR PUSCH and SPS PUSCH for eMTC, and proposed a TP to resolve the problem.

- Discussion

In current spec, the term “PUSCH transmission without a corresponding MPDCCH” is used to refer to SPS PUSCH, as below:

For BL/CE UEs, and for a PUSCH transmission starting in subframe n+ k0 without a corresponding MPDCCH, the UE shall adjust the PUSCH transmission in subframe(s) n+ki with i = 0, 1, …, N-1, where

-	0≤k0<k1<…,kN-1 and the value of  is determined by the repetition number field in the activation DCI, where are given in Table 8-2b and Table 8-2c; and

-	if the UE is configured with higher layer parameter ce-PUSCH-SubPRB-Config-r15, and the PUSCH resource assignment in the activation DCI is using uplink resource allocation type 5,  where  is defined in [3] and  is determined according to procedure in clause 8.1.6,  otherwise

-	in case N>1, subframe(s) n+ki with i=0,1,…,N-1 are N consecutive BL/CE UL subframe(s), and in case N=1, k0=0;

When PUR was introduced, the term “PUSCH transmission using a preconfigured uplink resource” is used to refer to a PUR PUSCH. However, as there is no corresponding MPDCCH for a PUR PUSCH either, the term “PUSCH without a corresponding MPDCCH” also covers the PUR PUSCH. As a result, there is ambiguity between PUR PUSCH and SPS PUSCH in the spec.

- Text proposal

To resolve the ambiguity discussed in section 2, the following is proposed:

Proposal 1: Endorse the following text proposal to TS 36.213.

=========================Text proposal to TS 36.213==============================

## 8.0	UE procedure for transmitting the physical uplink shared channel

<Unchanged part omitted>

For BL/CE UEs, the set of BL/CE UL subframes is indicated as follows

-	If UL resource reservation is enabled for the UE as specified in [11],

-	for PUSCH transmission associated with C-RNTI or SPS C-RNTI using UE-specific MPDCCH search space including PUSCH transmission without a corresponding MPDCCH or preconfigured uplink resource,

-	if the Resource reservation field in the DCI is set to 0, then the set of BL/CE UL subframes corresponds to all uplink subframes during the PUSCH transmission;

-	if the Resource reservation field in the DCI is set to 1, then the set of BL/CE UL subframes corresponds to all uplink subframes that are not fully reserved according to higher layer parameters (a subframe is considered fully reserved if and only if all SC-FDMA symbols of the PUSCH transmission are reserved in the subframe);

-	for PUCCH transmission associated with C-RNTI or SPS C-RNTI using UE-specific MPDCCH search space including PUSCH transmission without a corresponding MPDCCH,

-	the set of BL/CE UL subframes corresponds to all uplink subframes that are not fully reserved according to higher layer parameters (a subframe is considered fully reserved if and only if all SC-FDMA symbols of the PUCCH transmission are reserved in the subframe).

<Unchanged part omitted>

For BL/CE UEs, and for a PUSCH transmission starting in subframe n+ k0 without a corresponding MPDCCH or preconfigured uplink resource, the UE shall adjust the PUSCH transmission in subframe(s) n+ki with i = 0, 1, …, N-1, where

-	0≤k0<k1<…,kN-1 and the value of  is determined by the repetition number field in the activation DCI, where are given in Table 8-2b and Table 8-2c; and

-	if the UE is configured with higher layer parameter ce-PUSCH-SubPRB-Config-r15, and the PUSCH resource assignment in the activation DCI is using uplink resource allocation type 5,  where  is defined in [3] and  is determined according to procedure in clause 8.1.6,  otherwise

-	in case N>1, subframe(s) n+ki with i=0,1,…,N-1 are N consecutive BL/CE UL subframe(s), and in case N=1, k0=0;

<Unchanged part omitted>

If a UE is configured by higher layers to decode MPDCCHs with the CRC scrambled by the SPS C-RNTI, the UE shall decode the MPDCCH according to the combination defined in Table 8-5B and transmit the corresponding PUSCH if a transport block corresponding to the HARQ process of the PUSCH transmission is generated as described in [8]. 
The scrambling initialization of this PUSCH corresponding to these MPDCCHs and PUSCH retransmission for the same transport block is by SPS C-RNTI. The scrambling initialization of initial transmission of this PUSCH without a corresponding MPDCCH or preconfigured uplink resource and the PUSCH retransmission for the same transport block is by SPS C-RNTI.

======================End of Text proposal to TS 36.213===========================