3GPP TSG RAN WG1 #106	R1-2108196

e-Meeting, 16 – 27 August, 2021

Agenda item:		7.2.11

Source:	Nokia, Nokia Shanghai Bell

Title:	On Rel-16 UE Features

Document for:		Discussion and Decision

# Introduction

In RAN1#105-e it has been discussed how to align FGs 18-1/1a/1b on power sharing for DC with the current RAN4 understanding, in particular regarding differences between operation in FR1 and FR2. In this contribution we present our views on how this aspect should be addressed in RAN1.

# Discussion

### Updated FL proposal #1

- For FG18-1/1a/1b. add following note and ask RAN2 to modify the descriptions in TS38.306 accordingly

  - In case MCG and/or SCG have cells in different frequency ranges, this capability is applicable for power sharing only between those MCG and SCG cells with UL in FR1

The status of the discussion from RAN1#105-e is as follows, as captured in the email discussion summary:

Though most companies seemed to be fine with the proposal above, a decision could not be reached in the meeting because RAN1 was waiting for a RAN4 LS that was on the imminence of being agreed. Unfortunately, RAN4 failed to send the LS in the end, though it is clear from the related RAN4 discussions that the pending issues are not directly impacting RAN1.

In our understanding, this proposal above is very well aligned with the current understanding from RAN4, which is reflected in an earlier LS [1]. In [1] it is clear that RAN4 does not intend to introduce power sharing mechanisms for FR2 in Rel-16, and hence it is not reasonable for RAN1 to redefine the FGs such that UE could indicate power sharing support in FR2. This would cause ASN.1 impacts for a feature that cannot be supported in Rel-16 in any case. Hence, we propose the following:

Proposal: Confirm updated FL proposal#1 above, i.e.

- For FG18-1/1a/1b. add following note and ask RAN2 to modify the descriptions in TS38.306 accordingly

  - In case MCG and/or SCG have cells in different frequency ranges, this capability is applicable for power sharing only between those MCG and SCG cells with UL in FR1

# Conclusion

In this contribution we presented our views on how to align FGs 18-1/1a/1b on power sharing for DC with the RAN4 conclusions so far. We have made the following proposal:

Proposal: Confirm updated FL proposal#1 above, i.e.

- For FG18-1/1a/1b. add following note and ask RAN2 to modify the descriptions in TS38.306 accordingly

  - In case MCG and/or SCG have cells in different frequency ranges, this capability is applicable for power sharing only between those MCG and SCG cells with UL in FR1

# References

- R1-2102303, ”Further Reply LS on power control for NR-DC,” RAN4, Apr. 2021